3GPP TSG-RAN5 Meeting #107	R5-252461r2

# St Julian’s, Malta, 19th May 2025 – 23rd May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1607 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new LTE IMS data channel test case 12.35 - MT Voice call with precondition and BDC success |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_IMS_dataCH-UEConTest |  |  |  |  |  | Date: |  |  | 2025-04-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This new IMS data channel (IMS DC) test case is added to test: SS send SDP offer with IMS DC and MT UE reply SDP answer with IMS DC. The UE can establish voice call with precondition + IMS DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new LTE IMS data channel test case 12.35 IMS Data Channel / DCMTSI MT Voice call / Bootstrap Data Channel success / with preconditions at both originating UE and terminating UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NG_RTC_IMS_dataCH-UEConTest is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.35 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 34.229-2 CR 0369 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: - Update the Comment column in the expected sequence. 2nd revision: - Editorial update. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

12.35	IMS Data Channel / DCMTSI MT Voice call / Bootstrap Data Channel success / with preconditions at both originating UE and terminating UE

12.35.1	Definition

Test to verify that the UE correctly performs IMS mobile terminated voice call setup with preconditions and IMS Bootstrap data channel setup. This process is described in 3GPP TS 24.186 [159].

12.35.2	Conformance requirement

Same as 12.34.2

12.35.3	Test purpose

1)	To verify that UE setups IMS Bootstrap data channel successfully during MT Voice call establishment; and

2)	To verify that UE includes media feature tag +sip.app-subtype="webrtc-datachannel" when it receive INVITE message.

3)	To verify that UE generates the correct SDP answer for IMS data channel media negotiation when it receives the INVITE request with an SDP offer including the IMS data channel media descriptions.

12.35.4	Method of test

Initial conditions

UE contains either SIM application (GIBA), ISIM and USIM applications or only USIM application on UICC. UE has discovered P-CSCF and registered to IMS services, by executing the generic test procedure in Annex C.2 or C.2a (GIBA only) up to the last step.

SS is configured with the shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE. SS has performed AKAv1-MD5 authentication with the UE and accepted the registration (IMS security). UE is configured to use the precondition mechanism. UE is configured to use IMS data channel.

Test procedure applicable for a UE with E-UTRA support (TS 34.229-2 [5] A.18/1)

1-15)	UE executes the procedures described in TS 36.508 [94] table 4.5A.7.3-1 steps 1 to26. The qos flow for IMS bootstrap data channel (specified in TS 36.508[94] table 4.5A.32.3-1) is established after 200OK for INVITE.

Expected sequence

NOTE:	Only the IMS procedure relevant to the test purpose is described below.

| Step | Direction |  | Message | Comment |
| --- | --- | --- | --- | --- |
|  | UE | SS |  |  |
| 1-15 |  |  | Steps defined in annex C.11 | MTSI MT speech call. Referred from 36.508 [94] table 4.5A.7.3-1 for a UE with E-UTRA support. The qos flow for IMS bootstrap data channel is established after 200OK for INVITE message. Referred from 36.508 [94] table 4.5A.32.3-1. |

Specific Message Contents

INVITE (Step 2)

Use the default message “INVITE” in annex C.11 with the following exceptions:

| Header/param | Cond | Value/remark |
| --- | --- | --- |
| Contact feature-param |  | Feature-param is same as Step 1 of C.11 except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |
| Message-body |  | SDP body is same as Step 1 of C.11 except adding one Media description for IMS bootstrap data channel as follow: Media description for IMS data channel: m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel [Note 1] b=AS:500 Attributes for media for IMS data channel: a=max-message-size:64000 a=sctp-port:(sctp-port-value) [Note 2] a=fingerprint:SHA-256 (fingerprint) [Note 3] a=setup:actpass a=tls-id:(tls-id-value) a=dcmap:0 subprotocol="http” Note 1: Transport port is the port number of the SS. Note 2: The sctp-port-value is the SCTP port of the SS. Note 3: Fingerprint is calculated by SS. Note 4: The tls-id-value is random number generated by SS. |

183 Session Progress (Step 4)

Use the default message “183 Session Progress” in annex C.11 with the following exceptions:

| Header/param | Cond | Value/remark |
| --- | --- | --- |
| Contact feature-param |  | Feature-param is same as Step 4 of C.11 except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |
| Message-body |  | SDP body is same as Step 4 of C.11 except it may adding one Media description as follow: Media description for IMS data channel:[Note 4] m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel Attributes for media for IMS data channel: [Note 4] a=max-message-size: (max-message-size-value)[Note 1] a=sctp-port:(sctp-port-value) [Note 2] a=setup: active a=fingerprint: (hash-func) (fingerprint) [Note 3] a=tls-id: (tls-id-value) a=dcmap:0 subprotocol="http" Note 1: Optional attribute. If the SDP”max-message-size” attribute is not present the default value is 64K. Note 2: SCTP port range is between 0 and 65535(both included). Leading zeroes must not be used. Note 3: SHA-1, SHA-224, SHA-256, SHA-384, or SHA-512 is used as hash-func. Note4: The media description and attributes for media for IMS data channel shall present in 183 Session Progress or 200 OK for INVITE. |

UPDATE (Step 7)

Use the default message “UPDATE” in annex C.11 with the following exceptions:

| Header/param | Cond | Value/remark |
| --- | --- | --- |
| Message-body | A1 A2 | SDP body is same as Step 7 of C.11 except adding one Media description as follow: Media description for IMS data channel: Same as it in the INVITE message. Attributes for media for IMS data channel: Same as it in the INVITE message. SDP answer is same as Step 7 of C.11 |

| Condition | Explanation |
| --- | --- |
| A1 | 183 Session Progress includes SDP offer containing media lines both for voice and IMS data channel in Step 4. |
| A2 | 183 Session Progress does not include SDP offer containing media lines for IMS data channel in Step 4. |

200OK for UPDATE (Step 8)

Use the default message “200OK for UPDATE” in annex C.11 with the following exceptions:

| Header/param | Cond | Value/remark |
| --- | --- | --- |
| Contact feature-param |  | Feature-param is same as Step 8 of C11 except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |
| Message-body | A1 A2 | SDP body is same as Step 8 of C.11 except adding one Media description as follow: Media description for IMS data channel: Same as it in the 183 Session Progress message. Attributes for media for IMS data channel: Same as it in the 183 Session Progress message. SDP answer is same as Step 8 of C.11 |

| Condition | Explanation |
| --- | --- |
| A1 | 183 Session Progress includes SDP offer containing media lines both for voice and IMS data channel in Step 4. |
| A2 | 183 Session Progress does not include SDP offer containing media lines for IMS data channel in Step 4. |

200OK for INVITE (Step 12)

Use the default message “200OK for INVITE” in annex C.11 with the following exceptions:

| Header/param | Cond | Value/remark |
| --- | --- | --- |
| Contact feature-param |  | Feature-param is same as Step 12 of C.11 except adding one tag as follow: +sip.app-subtype="webrtc-datachannel" |
| Message-body | A2 A1 | SDP body is same as Step 7 of C.11(UPDATE) except it add one Media description as follow: Media description for IMS data channel: m=application (transport port) UDP/DTLS/SCTP webrtc-datachannel Attributes for media for IMS data channel: a=max-message-size: (max-message-size-value) [Note 1] a=sctp-port:(sctp-port-value) [Note 2] a=setup: active a=fingerprint: (hash-func) (fingerprint) [Note 3] a=tls-id: (tls-id-value) a=dcmap:0 subprotocol="http" Note 1: Optional attribute. If the SDP”max-message-size” attribute is not present the default value is 64K. Note 2: SCTP port range is between 0 and 65535(both included). Leading zeroes must not be used. Note3: SHA-1, SHA-224, SHA-256, SHA-384, or SHA-512 is used as hash-func SDP body is empty. |

| Condition | Explanation |
| --- | --- |
| A1 | 183 Session Progress includes SDP offer containing media lines both for voice and IMS data channel in Step 4. |
| A2 | 183 Session Progress does not include SDP offer containing media lines for IMS data channel in Step 4. |

<End of modified section 1>