3GPP TSG-RAN5 Meeting #108	R5-253974

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4927 | rev | - | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LTE Band 106: Test Model updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_900MHz_US-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. The limitation for TBS 8248 is not applicable to DCI combination 2 for 3 MHz bandwidth because only 7 PRBs are available, this needs to be clarified in clause 7.3.3.4. 2. In test cases using 3 Intra-frequency cells, we have to avoid intra-frequency interferences between PBCH and PDSCH in another cell. Then only 4 PRBs can be used. A new Physical resource allocation bitmap for DCI combination 1 (3 MHz) with 4 PRBs and TBS sheet need to be added. 3. The band 106 using 3MHz bandwidth is applicable to UE category M1 in both full-duplex FDD and half-duplex FDD mode, this need to be added. 4. Others editorial/typo. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Text clarified for TBS 8248 in clause 7.3.3.4. 2. Added new frequency domain resources with 4 PRBs in clauses 7.3.3.4 and 7.3.3.5. 3. For BL/CE UE, added physical resource allocation bitmap for DCI combination 1 for 3 MHz in clause 7.3.3.9.1. Added SIx and SIx-BR scheduling offsets for BL/CE UEs in clause 7.7.2a. Added scheduling pattern for type B half-duplex FDD in clause 7.26. 4. Corrected minor typo. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - TBS usage will remain confusing. - Test cases using 3 Intra-frequency cells will fail in 3MHz BW. - It will not be possible to run test cases in 3MHz BW with a BL/CE UE. - Typos will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.3.4, 7.3.3.5, 7.3.3.9.1, 7.7.2a, 7.26, 11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified sections>

#### 7.3.3.4	Additional rules for UE-dedicated scheduling scheme in normal mode

The UE-dedicated DL scheduling can work in the normal mode or in the explicit mode. The two resource allocation schemes shall be reconfigurable from each other when the UE and SS are not sending and receiving data, for instance, at end of the test preamble and before the beginning of the test body.

The present clause is specified for the use of the normal mode. The explicit mode is referred to clause 7.3.3.6.

The scheme specified in the present clause is applicable for transmission of data dedicated to a UE in a DL subframe, mapped to DL-SCH, PDCCH scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI when spatial multiplexing MIMO mode is not configured. The maximum modulation scheme is restricted to 64QAM. For the DCI combination 1, 20 physical resource blocks (5 to 24), 11 physical resource blocks (4 to 14) for 3 MHz and for the DCI combination 2, 17 physical resource blocks, 7 physical resource blocks for 3MHz are reserved. In the case when three intra frequency cells are applied to the test in the DCI combination 1, for the purpose of interference reduction, only 9 PRBs (16 to 24) are reserved and for 3 MHz bandwidth only 4 PRBs (11 to 14) are reserved.

In TDD normal TBS selection mode, no data is transmitted in DwPTS of the special subframe. For FDD, data can be transmitted in any subframe, except for half-duplex Type B UEs for which scheduling restrictions are specified in clause 7.26. In addition for BL/CE UEs, only valid subframes, according to fdd-DownlinkOrTddSubframeBitmapBR (when configured in the SS), shall be considered as suitable.

The following additional rules are applied for TBS selection:

-	Multiple ASPs can also carry same explicit timing information; indicating different ASP payloads, eventually needs to be transmitted in 1 TTI.

-	For UE Category ≠ 0 the Max TBS is restricted to 10296 bits (Max supported by UE category type 1).

-	For UE Category = 0 and M1, the Max TBS is restricted to 1000 bits. Hence in all TBS sheets only TB combinations with TBSize ≤ 1000 are used.

For 3 MHz bandwidth and the DCI combination 1 with 11 PRBs, the TBS 8248 is blocked as it results in coding rates higher than 0.93.

For 5 MHz bandwidth and the DCI combination 1 with 20 PRBs or DCI combination 2, the TBS 8248, 8760, and 9528 are blocked as they result in coding rates higher than 0.93.

For 5 MHz bandwidth and special DCI combination 1 with 9 PRBs, the TBS 2216, 5992 and 6712 are blocked as they result in coding rates higher than 0.93.

For 10 MHz, 15 MHz and 20 MHz bandwidths none of TBSs are blocked due to coding rates higher than 0.93.

TBS 280 is blocked for all bandwidths as it can be achieved by only Nprb =1 and Itbs=15, requiring 64 QAM modulation scheme. This TBS would normally be used for “RACH procedure MSG4+RRCconnectionSetup”.

The blocked TBS are considered to be not available for selection.

-	Data pending for transmission in a given sub-frame consists of (listed in transmission priority order):

-	MAC Control Elements that the SS needs to send.

-	AMD STATUS PDU(s) that the SS needs to send.

-	Data not sent in previous subframe(s).

-	Fresh Data scheduled for transmission in this subframe for all logical channels.

-	Distinct TBSs and all (NPRB, ITBS) combinations for each distinct TBS are listed in the sheet.

-	If a TBS size can be achieved with more than one combination of IMCS (ITBS) and NPRB:

-	Select combination with lowest delta between NPRB and IMCS.

-	If still more than one combination remain, select combination with highest NPRB.

-	Not more than one RLC Data PDU shall be placed in a MAC PDU per logical channel (i.e. minimize RLC segmentation).

-	In a subframe, in case there is data pending for transmission from more than one logical channel, for each type of data pending for transmission as defined above, priority shall be given to the logical channel with the lowest logical channel priority value. In case of more than one logical channel with the same logical channel priority value, these logical channels should be served equally. Data pending for transmission from more than one logical channel will rarely happen for the signalling and protocol test.

-	Data not transmitted within a subframe is scheduled as pending for transmission in the next available subframe according to the priorities given above. Pending data for transmission will rarely happen for the signalling and protocol test.

-	TBS selected in a context by various platforms shall be within an allowed deterministic tolerance of:

-	2 bytes for potential Timing Advance Command MAC Control Element (1 byte data + 1 byte MAC sub header).

-	4 bytes each for AMD STATUS PDU (2 bytes data + 2 bytes MAC subheader).

-	Therefore in the worst case the SS may add up to (2 + 4 x NAMRB ) bytes to the data scheduled for transmission in a certain subframe, where NAMRB is the number of AM radio bearers (SRB or DRB) actively sending DL data in the test, in any subframe.

-	For DCI combination 1 RIV is calculated based on physical resource blocks corresponding to NPRB of the selected TBS and (NPRB, ITBS) combination. The physical resource blocks that can be allocated are the first NPRB resources of index range

-	4..14 for 3 MHz bandwidth,

-	5..24 for 5 MHz bandwidth,

-	28..49 for 10 MHz bandwidth,

-	9..30 for 15 & 20 MHz bandwidth.

-	For DCI combination 2, RBG assignment is calculated based on physical resource blocks corresponding to NPRB of the selected TBS and (NPRB, ITBS) combination. The size of RBG is 2 for 3 MHz and 5 MHz, 3 for 10 MHz and 4 for 15 & 20 MHz. The available physical resource blocks for allocation are:

-	For 3 MHz bandwidth, RBG1(2,3), RBG3(6,7), RBG5(10,11) and RBG7(14). If NPRB is even, the first NPRB /2 available RBGs are allocated. If NPRB is odd, then first (NPRB -1)/2 RBGs and RBG7 are allocated.

-	For 5 MHz bandwidth, RBG1(2,3), RBG2(4,5), RBG4(8,9), RBG5(10,11), RBG7(14,15), RBG8(16,17), RBG10(20,21), RBG11(22,23) and RBG12(24). If NPRB is even, the first NPRB /2 available RBGs are allocated. If NPRB is odd, then first (NPRB -1)/2 RBGs and RBG 12 are allocated.

-	For 10 MHz bandwidth, RBG1(3,4,5), RBG2(6,7,8), RBG3(9,10,11), RBG5(15,16,17), RBG6(18,19,20), RBG10(30,31,32), RBG11(33,34,35), RBG12(36,37,38) and RBG16(48,49). If NPRB mod 3 is 0, the first NPRB /3 RBGs are allocated. If NPRB mod 3 is 2, then first (NPRB -2)/3 available RBGs and RBG 16 are allocated.

-	For 15 MHz bandwidth, RBG1(4,5,6,7), RBG2(8,9,10,11), RBG3(12,13,14,15), RBG5(20,21,22,23), RBG6(24,25,26,27), RBG7(28,29,30,31), RBG11(44,45,46,47), RBG13(52,53,54,55), RBG14(56,57,58,59), RBG15(60,61,62,63), RBG17(68,69,70,71), RBG18(72.73.74). If NPRB mod 4 is 0, the first NPRB /4 RBGs are allocated. If NPRB mod 4 is 3, then first (NPRB -3)/4 available RBGs and RBG 18 are allocated.

-	For 20 MHz bandwidth, RBG1(4,5,6,7), RBG2(8,9,10,11), RBG3(12,13,14,15), RBG4(16,17,18,19), RBG5(20,21,22,23), RBG7(28,29,30,31), RBG8(32,33,34,35), RBG9(36,37,38,39), RBG10(40,41,42,43), RBG14(56,57,58,59), RBG15(60,61,62,63), RBG16(64,65,66,67), RBG17(68,69,70,71), RBG19(76.77.78.79) and RBG20(80,81,82,83). The first NPRB /4 RBGs are allocated.

#### 7.3.3.5	DL Resource allocation bitmaps

##### 7.3.3.5.1	DCI combination 1

Table 7.3.3.5.1-1: Physical resource allocation bitmap
for DCI combination 1 (5 MHz) with 20 PRBs

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.1-2: Physical resource allocation bitmap
for DCI combination 1 (5 MHz) with 9 PRBs

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.1-3 (columns 0–34): Physical resource allocation bitmap
for DCI combination 1 (10 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9..22 | 22..27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  | Not Used | Used for PBCH and other common signals |  |  |  |  |  |  |  |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.1-3 (columns 35–49): Physical resource allocation bitmap
for DCI combination 1 (10 MHz)

| NPRB | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.1-3A (columns 0–20): Physical resource allocation bitmap
for DCI combination 1 (15 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.1-3B (columns 21-74): Physical resource allocation bitmap
for DCI combination 1 (15 MHz)

| NPRB | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31..33 | 34..41 | 42..74 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  | Not Used | Used for PBCH and other common signals | Not Used |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.1-4 (columns 0–20): Physical resource allocation bitmap
for DCI combination 1 (20 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.1-5 (columns 21-99): Physical resource allocation bitmap
for DCI combination 1 (20 MHz)

| NPRB | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31..46 | 47..52 | 53..99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  | Not Used | Used for PBCH and other common signals | Not Used |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.1-6: Physical resource allocation bitmap
for DCI combination 1 (3 MHz) with 11 PRBs

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PBCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.1-7: Physical resource allocation bitmap
for DCI combination 1 (3 MHz) with 4 PRBs

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PBCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 7.3.3.5.2	DCI combination 2

Table 7.3.3.5.2-1: Physical resource allocation bitmap
for DCI combination 2 (5 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even | 0 |  |  |  |  |  | 1 |  |  |  |  |  | 2 |  |  |  |  |  | 3 |  |  |  |  |  |  |
| BCCH-Odd | 2 |  |  |  |  |  | 3 |  |  |  |  |  | 0 |  |  |  |  |  | 1 |  |  |  |  |  |  |
| PCCH-Even |  | 4 |  |  |  |  |  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 |  |  |  |  |  | 5 |  |  |  |  |  |
| RAR-Even |  |  | 8 |  |  |  |  |  | 9 |  |  |  |  | 6 |  |  |  |  |  | 7 |  |  |  |  |  |
| RAR-Odd |  | 6 |  |  |  |  |  | 7 |  |  |  |  |  |  | 8 |  |  |  |  |  | 9 |  |  |  |  |
| UE-Dedicated | x | x |  |  |  |  | x | x |  |  |  |  | x | x |  |  |  |  | x | x |  |  |  |  |  |
| RBGs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.2-2 (columns 0-20): Physical resource allocation bitmap
for DCI combination 2 (10 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even | 0 |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |
| BCCH-Odd | 2 |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |  |
| PCCH-Even |  | 4 |  |  |  |  |  |  |  |  |  |  |  | 5 |  |  |  |  |  |  |  |
| PCCH-Odd |  | 6 |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |
| RAR-Even |  |  | 8 |  |  |  |  |  |  |  |  |  |  |  | 9 |  |  |  |  |  |  |
| RAR-Odd |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  | 11 |  |  |  |  |  |  |
| UE-Dedicated | x | x |  |  |  |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |
| RBGs | 0 |  |  | 1 |  |  | 2 |  |  | 3 |  |  | 4 |  |  | 5 |  |  | 6 |  |  |

Table 7.3.3.5.2-2 (columns 21-41): Physical resource allocation bitmap
for DCI combination 2 (10 MHz)

| NPRB | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |
| BCCH-Odd |  |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |
| PCCH-Even |  |  |  |  |  |  |  | 6 |  |  |  |  |  |  |  |  |  |  |  | 7 |  |
| PCCH-Odd |  |  |  |  |  |  |  | 4 |  |  |  |  |  |  |  |  |  |  |  | 5 |  |
| RAR-Even |  |  |  |  |  |  |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  | 11 |
| RAR-Odd |  |  |  |  |  |  |  |  | 8 |  |  |  |  |  |  |  |  |  |  |  | 9 |
| UE-Dedicated |  | x | x | x | x | x | x | x |  |  |  |  |  |  |  |  |  |  | x | x |  |
| RBGs | 7 |  |  | 8 |  |  | 9 |  |  | 10 |  |  | 11 |  |  | 12 |  |  | 13 |  |  |

Table 7.3.3.5.2-2 (columns 42-49): Physical resource allocation bitmap
for DCI combination 2 (10 MHz)

| NPRB | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  | Not Used |  |  |  |
| BCCH-Odd |  |  |  |  |  |  |  |  |
| PCCH-Even |  |  |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  |  |  |  |
| RAR-Even |  |  |  |  |  |  |  |  |
| RAR-Odd |  |  |  |  |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  |
| RBG's | 14 |  |  | 15 |  |  | 16 |  |

Table 7.3.3.5.2-2a (columns 0-19): Physical resource allocation bitmap
for DCI combination 2 (15 MHz)

| nprb | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |
| BCCH-Odd | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |
| PCCH-Even |  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |  |  |
| PCCH-Odd |  | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |
| RAR-Even |  |  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9 |  |
| RAR-Odd |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 11 |  |
| UE-Dedicated | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x | x |  |  |
| RBG's | 0 |  |  |  | 1 |  |  |  | 2 |  |  |  | 3 |  |  |  | 4 |  |  |  |

Table 7.3.3.5.2-2a (columns 20-39): Physical resource allocation bitmap
for DCI combination 2 (15 MHz)

| nprb | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BCCH-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Even |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR-Even |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RBG's | 5 |  |  |  | 6 |  |  |  | 7 |  |  |  | 8 |  |  |  | 9 |  |  |  |

Table 7.3.3.5.2-2a (columns 40-59): Physical resource allocation bitmap
for DCI combination 2 (15 MHz)

| nprb | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |
| BCCH-Odd |  |  |  |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Even |  |  |  |  |  |  |  |  |  | 6 |  |  |  |  |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  |  |  |  |  | 4 |  |  |  |  |  |  |  |  |  |  |
| RAR-Even |  |  |  |  |  |  |  |  |  |  | 10 |  |  |  |  |  |  |  |  |  |
| RAR-Odd |  |  |  |  |  |  |  |  |  |  | 8 |  |  |  |  |  |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |  |  |
| RBG's | 10 |  |  |  | 11 |  |  |  | 12 |  |  |  | 13 |  |  |  | 14 |  |  |  |

Table 7.3.3.5.2-2a (columns 60-74): Physical resource allocation bitmap
for DCI combination 2 (15 MHz)

| nprb | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  | 3 |  |  |  |  |  |  |  |  |  |  |
| BCCH-Odd |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |
| PCCH-Even |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  | 5 |  |  |  |  |  |  |  |  |  |
| RAR-Even |  |  |  |  |  |  | 11 |  |  |  |  |  |  |  |  |
| RAR-Odd |  |  |  |  |  |  | 9 |  |  |  |  |  |  |  |  |
| UE-Dedicated |  |  |  |  | x | x |  |  |  |  |  |  |  |  |  |
| RBG's | 15 |  |  |  | 16 |  |  |  | 17 |  |  |  | 18 |  |  |

Table 7.3.3.5.2-3 (columns 0-19): Physical resource allocation bitmap
for DCI combination 2 (20 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BCCH-Odd | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Even |  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Odd |  | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR-Even |  |  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR-Odd |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Dedicated | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RBGs | 0 |  |  |  | 1 |  |  |  | 2 |  |  |  | 3 |  |  |  | 4 |  |  |  |

Table 7.3.3.5.2-3 (columns 20-39): Physical resource allocation bitmap
for DCI combination 2 (20 MHz)

| NPRB | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BCCH-Odd |  |  |  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Even |  |  |  |  |  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR-Even |  |  |  |  |  |  | 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR-Odd |  |  |  |  |  |  | 11 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Dedicated |  |  |  |  | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RBGs | 5 |  |  |  | 6 |  |  |  | 7 |  |  |  | 8 |  |  |  | 9 |  |  |  |

Table 7.3.3.5.2-3 (columns 40-59): Physical resource allocation bitmap
for DCI combination 2 (20 MHz)

| NPRB | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |
| BCCH-Odd |  |  |  |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Even |  |  |  |  |  |  |  |  |  | 6 |  |  |  |  |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  |  |  |  |  | 4 |  |  |  |  |  |  |  |  |  |  |
| RAR-Even |  |  |  |  |  |  |  |  |  |  | 10 |  |  |  |  |  |  |  |  |  |
| RAR-Odd |  |  |  |  |  |  |  |  |  |  | 8 |  |  |  |  |  |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  | x | x | x |  |  |  |  |  |  |  |  |  |
| RBG's | 10 |  |  |  | 11 |  |  |  | 12 |  |  |  | 13 |  |  |  | 14 |  |  |  |

Table 7.3.3.5.2-3 (columns 60-79): Physical resource allocation bitmap
for DCI combination 2 (20 MHz)

| NPRB | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |
| BCCH-Odd |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |
| PCCH-Even |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |  |  |  |  |  |  |
| RAR-Even |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 11 |  |  |  |  |  |
| RAR-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9 |  |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |
| RBGs | 15 |  |  |  | 16 |  |  |  | 17 |  |  |  | 18 |  |  |  | 19 |  |  |  |

Table 7.3.3.5.2-3 (columns 80-99): Physical resource allocation bitmap
for DCI combination 2 (20 MHz)

| NPRB | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 | 96 | 97 | 98 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Not Used |  |  |  |
| BCCH-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Even |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR-Even |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RBGs | 20 |  |  |  | 21 |  |  |  | 22 |  |  |  | 23 |  |  |  | 24 |  |  |  |

Table 7.3.3.5.2-4: Physical resource allocation bitmap
for DCI combination 2 (3 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even | 0 |  |  |  | 1 |  |  |  | 2 |  |  |  | 3 |  |  |
| BCCH-Odd | 2 |  |  |  | 3 |  |  |  | 0 |  |  |  | 1 |  |  |
| PCCH-Even |  | 4 |  |  |  | 5 |  |  |  |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  |  |  |  |  | 4 |  |  |  | 5 |  |
| RAR-Even |  |  | 8 |  |  |  | 9 |  |  | 6 |  |  |  | 7 |  |
| RAR-Odd |  | 6 |  |  |  | 7 |  |  |  |  | 8 |  |  |  | 9 |
| UE-Dedicated | x | x |  |  | x | x |  |  | x | x |  |  | x | x |  |
| RBGs | 0 |  | 1 |  | 2 |  | 3 |  | 4 |  | 5 |  | 6 |  | 7 |

NOTE:	Odd and even refer to slots.

#### 7.3.3.6	UE-dedicated scheduling scheme in explicit mode

This scheme applies to:

1.	spatial multiplexing MIMO configurations or

2.	transmit diversity MIMO configurations and non-MIMO configuration where the normal mode scheduling scheme is inappropriate.

SS is configured with an exact TBS (modulation and coding scheme, Imcs, and number of resource blocks, Nprb) to use. Other parameters, such as the HARQ process number and redundancy version to use for each transmission, are also configured by the TTCN. SS shall use TBS sheets with matching DCI format and Resource allocation Type. If the parameter 'FirstRbIndex' is configured different than specified in respective TBS sheet, the resource block bit maps in TBS sheets are shifted by ' FirstRbIndex' and applied, with an exception for Resource allocation type 0 where only the full size 'Resource block groups' are shifted by ' FirstRbIndex'; if the last Resource block group is not full size, and is part of resource block bitmap, it is applied without any shift.

All data scheduled for a certain subframe shall be transmitted in the single indicated subframe, using configured parameters. The TTCN shall ensure that the configured parameters are consistent, in particular that the scheduled data size and the configured TBS match each other. Data scheduled by the prose, and hence also by the TTCN, provides possible space for the Timing Advance MAC control element and the RLC Status PDU. The SS shall include one of these if so triggered, else the bits reserved for these are filled by MAC padding.

Additionally, in the case of MIMO data scheduled for transmission in a given sub-frame, this consists of (listed in transmission priority order):

-	MAC Control Elements that the SS needs to send (if triggered).

-	AMD STATUS PDU(s) that the SS needs to send (if triggered).

-	Fresh data scheduled for transmission in this subframe for one or more logical channels, as per logical channel priority [lower value = higher priority]; if data is available for more than one logical channel with the same priority, then the logical channel corresponding to the DRB-ID with the lower value has the higher priority.

-	MAC padding.

The following additional rules need to be applied on data scheduled for transmission to be mapped on two transport blocks corresponding to two code words:

-	Higher priority data (as stated above) maps on to Transport Block 1 and lower priority data maps on Transport Block 2 (if Transport Block 1 gets full); and

-	Minimum MAC padding is performed in Transport Block 1; and

-	If data from one logical channel needs to be mapped on to two transport blocks, the PDCP PDUs with lower PDCP sequence numbers get mapped on to Transport Block 1.

By default no data is scheduled in TDD special subframes (i.e. subframes 1 and 6 for default TDD subframe configuration). For testing DL data reception in TDD special subframe, explicit mode TBS selection shall be used. The TTCN shall ensure that the configured parameters are consistent, in particular that the scheduled data size and the configured TBS match each other. As per TS36.213 [30] clause 7.1.7, for special subframe configuration 9 with normal cyclic prefix or special subframe configuration 7 with extended cyclic prefix, the actual Nprb used for TB size calculation will be a max( floor{ Nprb in DCI command *0.375},1). Tables 7.3.3.6-1/2 give the mapping for Nprb in DCI command and Nprb used for TB size determination.

Table 7.3.3.6-1: Nprb DCI to Nprb TBS mapping for 20 Mhz

| Nprb in DCI command (configured by TTCN) | 4 | 8 | 12 | 16 | 20 | 24 | 28 | 32 | 36 | 40 | 44 | 48 | 54 | 60 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nprb used for TBS determination | 1 | 3 | 4 | 6 | 7 | 9 | 10 | 12 | 13 | 15 | 16 | 18 | 20 | 22 |

Table 7.3.3.6-2: Nprb DCI to Nprb TBS mapping for 5 Mhz

| Nprb in DCI command (configured by TTCN) | 4 | 6 | 8 | 12 | 14 | 16 |
| --- | --- | --- | --- | --- | --- | --- |
| Nprb used for TBS determination | 1 | 2 | 3 | 4 | 5 | 6 |

##### 7.3.3.6.1	DL Scheduling in Transport Block Size Selection Test Cases

The MAC transport block size selection test cases defined in clause 7.1.7 of TS 36.523-1 [1], use bandwidths of 3/5/10/15/20MHz. For the preamble and post amble in these tests, the default scheduling rules defined in clauses 7.3.3.1 to 7.3.3.4 for 3/5/10/15/20 MHz and DCI combination 1A are applied respectively. During the test body, when the actual TB sizes with appropriate DCI and resource allocation formats needed are to be tested, the SS is configured in explicit mode for UE-dedicated scheduling.

#### 7.3.3.7	Resource allocation sheets

Attached with this Technical Specification, the DL resource allocation tables can be found, providing physical resource allocations for various transport block sizes, developed as per rules specified in clause 7.3.3, in Microsoft Excel format. Each individual sheet in the workbook represents various scheduling schemes as per table 7.3.3.7-1.

Table 7.3.3.7-1: DL resource allocation sheets

| S. No | Sheet Name | Description |
| --- | --- | --- |
| 1 | DCI-1A-PCCH | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by P-RNTI (3 MHz, 5 MHz, 10 MHz, 15 MHz and 20 MHz) |
| 2 | DCI-1A-BCCH | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by SI-RNTI (3 MHz, 5 MHz, 10 MHz, 15 MHz and 20 MHz) |
| 3 | DCI-1A-RAR | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by RA-RNTI (3 MHz, 5 MHz, 10 MHz, 15 MHz and 20 MHz) |
| 4 | DCI-1A-UE-Dedicated | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (5 MHz) |
| 5 | DCI-1A-3-IntraFreq-UE-Dedicated | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI and three Intra Freq cells are configured (5 MHz) |
| 6 | DCI-1A-UE-Dedicated-10MHz | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (10 MHz) |
| 6A | DCI-1A-UE-Dedicated-15MHz | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (15 MHz) |
| 7 | DCI-1A-UE-Dedicated-20MHz | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (20 MHz) Also in preamble/postamble phase of MAC TBS test cases with 15 MHz bandwidth configured |
| 7A | DCI-1A-UE-Dedicated-3MHz | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (3 MHz) |
| 7B | DCI-1A-3-IntraFreq-UE-Ded-3MHz | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI and three Intra Freq cells are configured (3 MHz) |
| 8 | DCI-1C-PCCH | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by P-RNTI (5 MHz) |
| 8A | DCI-1C-PCCH-3MHz | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by P-RNTI (3 MHz) |
| 9 | DCI-1C-BCCH | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by SI-RNTI (5 MHz) |
| 9A | DCI-1C-BCCH-3MHz | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by SI-RNTI (3 MHz) |
| 10 | DCI-1C-RAR | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by RA-RNTI (5 MHz) |
| 10A | DCI-1C-RAR-3MHz | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by RA-RNTI (3 MHz) |
| 11 | DCI-1-UE-Dedicated | DL Resource scheduling for DCI format 1, Resource allocation 0 and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (5 MHz) |
| 11A | DCI-1-UE-Dedicated-3MHz | DL Resource scheduling for DCI format 1, Resource allocation 0 and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (5 MHz) |
| 12 | DCI-1C-PCCH-10MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by P-RNTI (10 MHz) |
| 13 | DCI-1C-BCCH-10MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by SI-RNTI (10 MHz) |
| 14 | DCI-1C-RAR-10MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by RA-RNTI (10 MHz) |
| 15 | DCI-1-UE-Dedicated-10MHz-Gap1 | DL Resource scheduling for DCI format 1, Resource allocation 0 and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (10 MHz) |
| 15A | DCI-1C-PCCH-15MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by P-RNTI (15 MHz) |
| 15B | DCI-1C-BCCH-15MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by SI-RNTI (15 MHz) |
| 15C | DCI-1C-RAR-15MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by RA-RNTI (15 MHz) |
| 15D | DCI-1-UE-Dedicated-15MHz-Gap1 | DL Resource scheduling for DCI format 1, Resource allocation 0 and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (15 MHz) |
| 16 | DCI-1C-PCCH-20MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by P-RNTI (20 MHz) |
| 17 | DCI-1C-BCCH-20MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by SI-RNTI (20 MHz) |
| 18 | DCI-1C-RAR-20MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by RA-RNTI (20 MHz) |
| 19 | DCI-1-UE-Dedicated-20MHz-Gap1 | DL Resource scheduling for DCI format 1, Resource allocation 0 and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (20 MHz) |
| 20 | DCI-1-RA0-ExplicitConfig | DL Resource scheduling for DCI format 1, Resource allocation 0 and PDCCH is scrambled by C-RNTI |
| 21 | DCI-1-RA1-ExplicitConfig | DL Resource scheduling for DCI format 1, Resource allocation 1 and PDCCH is scrambled by C-RNTI |
| 22 | DCI1A-ExplicitConfig | DL Resource scheduling for DCI format 1A, Resource allocation 2(localised & distributed) and PDCCH is scrambled by C-RNTI |
| 23 | DCI-2A-RA0-ExplicitConfig | DL Resource scheduling for DCI format 2A, Resource allocation 0 and PDCCH is scrambled by C-RNTI |
| 24 | DCI-2A-RA1-ExplicitConfig | DL Resource scheduling for DCI format 2A, Resource allocation 1 and PDCCH is scrambled by C-RNTI |
| 25 | DCI-6-1A-RAR | DL Resource scheduling for DCI format 6-1A and PDCCH is scrambled by RA-RNTI |
| 26 | UE-Dedicated-DCI-6-1A | DL Resource scheduling for DCI format 6-1A, Resource allocation 0 and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI |
| 27 | UE-Dedicated-DCI-6-1B | DL Resource scheduling for DCI format 6-1B, Resource allocation 0 and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (15 MHz) |

<End of modified sections>

<Start of modified section>

##### 7.3.3.9.1	DCI combination 1

For simplicity of scheduling, it is proposed to restrict DCI combination 1 for WB-E-UTRA operation in test cases also applicable for BL/CE UE and executed against BL/CE UE.

UE-Dedicated below includes all other transmission C-RNTI/T-CRNTI/P-RNTI/RA-RNTI on PDSCH.

Table 7.3.3.9.1-1: Physical resource allocation bitmap
for DCI combination 1 (5 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9-15 |  |  |  |  | 16 | 17 | 18 | 19-24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  | Used for PBCH and other common signals |  |  |  |  |  |  |  |  |
| SIB1-BR | NB 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NB 3 |
| SIx-BR | NB 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NB 3 |
| MPDCCH/P-RNTI | NB 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NB 3 |
| MPDCCH |  |  |  |  |  |  | NB 1 |  |  |  |  |  |  |  |  |  |  |  |

NOTE:	The allocation assumes that SIB1-BR/SIx-BR/SIx are not transmitted in same subframe.

It is unavoidable to use NB 3 for both SIB1-BR and UE-Dedicated. This means that UE-Dedicated transmission cannot be used in (frame number mod 2=0 in even physical layer cell Id and frame number mod 2 =1 in odd physical layer cell 1d, subframe =4). NB 1 and 2 need to be avoided so as to avoid central 72 carriers carrying MIB and reduce interference between intra frequency cells.

Table 7.3.3.9.1-2: Narrow band allocation per cell (5 MHz)

| Physical cell ID mod 2 | SIB1-BR set | SIx-BR | UE-Dedicated | MPDCCH | MPDCCH associated with the P-RNTI |
| --- | --- | --- | --- | --- | --- |
| 0 | NB 0,NB 3 | NB 0 | NB 3 | NB 1 | NB 0 |
| 1 | NB 3,NB 0 | NB 0 | NB 3 | NB 1 | NB 3 |

Table 7.3.3.9.1-3 (columns 0–30): Physical resource allocation bitmap
for DCI combination 1 (10 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7-12 | 13-18 | 19..21 | 22..27 | 28-30 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  | Not Used | Used for PBCH and other common signals | Not Used |
| SIB1-BR |  | NB 0 |  |  |  |  |  | NB 1 | NB 2 |  |  |  |
| SIx-BR |  | NB 0 |  |  |  |  |  |  |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  |  |  |  |  |
| MPDCCH/P-RNTI |  | NB 0 |  |  |  |  |  | NB 1 | NB 2 |  |  |  |
| MPDCCH |  |  |  |  |  |  |  | NB1 | NB2 |  |  |  |

Table 7.3.3.9.1-3 (columns 31–49): Physical resource allocation bitmap
for DCI combination 1 (10 MHz)

| NPRB | 31-36 | 37-42 | 43-48 | 49 |
| --- | --- | --- | --- | --- |
| BCCH |  |  |  | Not Used |
| SIB1-BR | NB 5 | NB 6 | NB 7 |  |
| SIx-BR |  |  |  |  |
| UE-Dedicated |  | NB 6 | NB 7 |  |
| MPDCCH/P-RNTI | NB 5 | NB 6 | NB 7 |  |
| MPDCCH |  |  |  |  |

Table 7.3.3.9.1-4: Narrow band allocation per cell (10 MHz)

| Physical cell ID mod 6 | SIB1-BR set | SIx-BR | UE-Dedicated | MPDCCH | MPDCCH associated with the P-RNTI |
| --- | --- | --- | --- | --- | --- |
| 0 | NB 0,NB 5 | NB 0 | NB 6 | NB 1 | NB 0 |
| 1 | NB 1,NB 6 | NB 0 | NB 7 | NB 2 | NB 1 |
| 2 | NB 2,NB 7 | NB 0 | NB 6 | NB 1 | NB 2 |
| 3 | NB 5,NB 0 | NB 0 | NB 6 | NB 1 | NB 5 |
| 4 | NB 6,NB 1 | NB 0 | NB 7 | NB 2 | NB 6 |
| 5 | NB 7,NB 2 | NB 0 | NB 6 | NB 1 | NB 7 |

NOTE 1:	The allocation assumes that SIB1-BR/SIx-BR and SIB1-BR/SIx are not transmitted in same subframe.

NOTE 2:	The setting of NBs does not consider the case of bandwidth 10MHz for TDD.

Table 7.3.3.9.1-5 (columns 0–43): Physical resource allocation bitmap
for DCI combination 1 (20 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8-13 | 14-19 | 20-25 | 26-31 | 32-37 | 38-43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB1-BR |  |  | NB 0 |  |  |  |  |  | NB 1 | NB 2 | NB 3 | NB 4 | NB 5 | NB 6 |
| SIx-BR |  |  |  |  |  |  |  |  | NB 1 |  |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MPDCCH/P-RNTI |  |  | NB 0 |  |  |  |  |  | NB 1 | NB 2 | NB 3 | NB 4 | NB 5 | NB 6 |

Table 7.3.3.9.1-5 (columns 44-99): Physical resource allocation bitmap
for DCI combination 1 (20 MHz)

| NPRB | 44-46 | 47..52 |  |  |  | 53-54 | 56-61 | 62-67 | 68-73 | 64-79 | 80-85 | 86-91 | 92-97 | 98..99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH | Not Used | Used for PBCH and other common signals |  |  |  | Not Used |  |  |  |  |  |  |  | Not Used |
| SIB1-BR |  |  |  |  |  |  | NB 9 | NB 10 | NB11 | NB 12 | NB 13 | NB14 | NB 15 |  |
| SIx-BR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  |  |  |  | NB 14 |  |  |
| MPDCCH/P-RNTI |  |  |  |  |  |  | NB 9 | NB 10 | NB11 | NB 12 | NB 13 | NB 14 | NB 15 |  |
| MPDCCH | NB 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.9.1-6: Narrow band allocation per cell (20 MHz)

| Physical cell ID mod 14 | SIB1-BR set | SIx-BR | UE-Dedicated | MPDCCH | MPDCCH associated with the P-RNTI |
| --- | --- | --- | --- | --- | --- |
| 0 | NB 0, NB 3, NB 6, NB 11 | NB 1 | NB 14 | NB 7 | NB 0 |
| 1 | NB 1, NB 4, NB 9, NB 12 | NB 1 | NB 14 | NB 7 | NB 1 |
| 2 | NB 2, NB 5, NB 10, NB 13 | NB 1 | NB 14 | NB 7 | NB 2 |
| 3 | NB 3, NB 6, NB 11, NB 14 | NB 1 | NB 15 | NB 7 | NB 3 |
| 4 | NB 4, NB 9, NB 12, NB 15 | NB 1 | NB 14 | NB 7 | NB 4 |
| 5 | NB 5, NB 10, NB 13, NB 0 | NB 1 | NB 14 | NB 7 | NB 5 |
| 6 | NB 6, NB 11, NB 14, NB 1 | NB 1 | NB 15 | NB 7 | NB 6 |
| 7 | NB 9, NB 12, NB 15, NB 2 | NB 1 | NB 14 | NB 7 | NB 9 |
| 8 | NB 10, NB13, NB 0, NB 3 | NB 1 | NB 14 | NB 7 | NB 10 |
| 9 | NB 11, NB 14, NB 1, NB 4 | NB 1 | NB 15 | NB 7 | NB 11 |
| 10 | NB 12, NB 15, NB 2, NB 5 | NB 1 | NB 14 | NB 7 | NB 12 |
| 11 | NB 13, NB 0, NB 3, NB 6 | NB 1 | NB 14 | NB 7 | NB 13 |
| 12 | NB 14, NB 1, NB 4, NB 9 | NB 1 | NB 15 | NB 7 | NB 14 |
| 13 | NB 15, NB 2, NB 5, NB 10 | NB 1 | NB 14 | NB 7 | NB 15 |

NOTE:	The allocation assumes that SIB1-BR/SIx and SIB1-BR/SIx-BR are not transmitted in same subframe.

Table 7.3.3.9.1-7: Physical resource allocation bitmap for DCI combination 1 (3 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PBCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB1-BR |  | NB 0 |  |  |  |  |  |  | NB 1 |  |  |  |  |  |  |
| SIx-BR |  | NB 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Dedicated |  | NB 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MPDCCH/P-RNTI |  | NB 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MPDCCH |  |  |  |  |  |  |  |  | NB 1 |  |  |  |  |  |  |

NOTE:	The allocation assumes that SIB1/SIB1-BR/SIx/SIx-BR are not transmitted in the same subframe.

It is unavoidable to use NB 0 for both SIB1-BR and UE-Dedicated, this means that UE-Dedicated transmission cannot be used in subframe 4 in even SFN in even physical layer cell Id and in odd SFN in odd physical layer cell 1d. NB 0 and NB 1 in subframe 0 need to be avoided for UE-Dedicated transmission so as to avoid central 72 carriers carrying MIB and reduce interference between intra frequency cells. UE-Dedicated transmission cannot be done in subframes used to broadcast SIx-BR.

Table 7.3.3.9.1-8: Narrow band allocation per cell (3 MHz)

| Physical cell ID mod 2 | SIB1-BR set | SIx-BR | UE-Dedicated | MPDCCH | MPDCCH associated with the P-RNTI |
| --- | --- | --- | --- | --- | --- |
| 0 | NB 0,NB 1 | NB 0 | NB 0 | NB 1 | NB 0 |
| 1 | NB 1,NB 0 | NB 0 | NB 0 | NB 1 | NB 0 |

<End of modified section>

<Start of modified section>

### 7.7.2a	Scheduling information for BR System information

The scheduling of BR System Information is based upon the following conditions ( TS 36.211 [35] clause 6.4.1):

-	 =4 i.e. the schedulingInfoSIB1-BR-r13 values are selected from {1,4,7,10,13,16} which as per TS 36.213 [30] table 7.1.6-1 would result in 4 repetitions.

-	si-WindowLength-BR-r13 =20 ms (same value as si-WindowLength)

-	si-RepetitionPattern-r13 = every2ndRF (hence results eventually in only transmission in first frame of window)

-	Except for SIB1-BR (mandated for SIB1-BR by TS 36.211 [35]) hopping is assumed to be not enabled for anything else.

-	The current test plan excludes inter frequency and inter rat test cases due to limitations in RAN4. Inter frequency, inter RAT system information combinations are not considered.

-	The typical size of SIB1/2/3/4 is less than 700 bits, the Max TB size for SIx-BR is 936 bits, TS 36.213 [30] table 7.1.7.2.7-1, hence no segmentation is assumed. For SIB1-BR schedulingInfoSIB1-BR-r13 =10, and for SIB 2,3,4 the si-TBS-r13 is set as b712.

-	TTCN may configure the SS to repeat the MIB. In that case the SS shall repeat the MIB as per TS 36.331[19] clause 5.2.1.2 in subframe#9 of the previous radio frame for FDD and subframe #5 of the same radio frame for TDD, the same can be achieved in the below schedule.

-	System information broadcasting rules defined in clause 7.7.1 apply with the exception that SIB1 is replaced with SIB1 & SIB1-BR and SI is replaced with SI & SI-BR.

The subframe offset values used for SI messages are according to table 7.7.2a-1.

Table 7.7.2a-1: SubframeOffset values

| Scheduling Information No. Acc to TS 36.508 [3], clause 4.4.3.1.2 | subframeOffset (FDD) | subframeOffset (FDD DL bandwidth = 3 MHz) | subframeOffset (TDD DL bandwidth > 5 MHz) | subframeOffset (TDD DL bandwidth = 5 MHz) |
| --- | --- | --- | --- | --- |
| SI1 | 1 | 0 | 0 | 10 |
| SI2 | 1 | 0 | 0 | 10 |
| SI3 | 3 | 0 | 0 | 10 |
| SI4 | 3 | 0 | 0 | 10 |
| SI1-BR | 0 | 7 | 0 | 0 |
| SI2-BR | 0 | 7 | 0 | 0 |
| SI3-BR | 0 | 7 | 0 | 0 |
| SI4-BR | 0 | 7 | 0 | 0 |
| NOTE: The subframe offsets for SIx/SIx-BR of 0 and 10 are suitable for all TDD uplink-downlink configurations. |  |  |  |  |

The following tables give the SFN and subframe numbers in which MIB, SIs and SI-BRs are actually scheduled as per default parameters defined in TS 36.508 [3] for:

Table 7.7.2a-2 for FDD with even physical layer cell IDs

Table 7.7.2a-4 for FDD with odd physical layer cell IDs

Table 7.7.2a -3 for TDD for DL bandwidth > 5 MHz and even physical layer cell Id

Table 7.7.2a-5 for TDD for DL bandwidth > 5 MHz and odd physical layer cell Id

Table 7.7.2a-6 for TDD for DL bandwidth = 5 MHz

Table 7.7.2a-7 for FDD for DL bandwidth = 3 MHz with even physical layer cell IDs

Table 7.7.2a-8 for FDD for DL bandwidth = 3 MHz with odd physical layer cell IDs.

Table 7.7.2a-2: BR System Information Scheduling (FDD) for even physical layer cell IDs[=0]

| SFN\SubFrame | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | MIB SI1-BR | SI1 |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 1 | MIB |  |  |  |  |  |  |  |  |  |
| 2 | MIB SI2-BR | SI2 |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 3 | MIB |  |  |  |  |  |  |  |  |  |
| 4 | MIB SI3-BR |  |  | SI3 | SIB1-BR | SIB1 |  |  |  |  |
| 5 | MIB |  |  |  |  |  |  |  |  |  |
| 6 | MIB SI4-BR |  |  | SI4 | SIB1-BR | SIB1 |  |  |  |  |
| 7 | MIB |  |  |  |  |  |  |  |  |  |
| 8 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 9 | MIB |  |  |  |  |  |  |  |  |  |
| 10 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 11 | MIB |  |  |  |  |  |  |  |  |  |
| 12 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 13 | MIB |  |  |  |  |  |  |  |  |  |
| 14 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 15 | MIB |  |  |  |  |  |  |  |  |  |
| 16 | MIB SI1-BR | SI1 |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 17 | MIB |  |  |  |  |  |  |  |  |  |
| 18 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 19 | MIB |  |  |  |  |  |  |  |  |  |
| 20 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 21 | MIB |  |  |  |  |  |  |  |  |  |
| 22 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 23 | MIB |  |  |  |  |  |  |  |  |  |
| 24 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 25 | MIB |  |  |  |  |  |  |  |  |  |
| 26 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 27 | MIB |  |  |  |  |  |  |  |  |  |
| 28 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 29 | MIB |  |  |  |  |  |  |  |  |  |
| 30 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 31 | MIB |  |  |  |  |  |  |  |  |  |
| 32 | MIB SI1-BR | SI1 |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 33 | MIB |  |  |  |  |  |  |  |  |  |
| 34 | MIB SI2-BR | SI2 |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 35 | MIB |  |  |  |  |  |  |  |  |  |
| 36 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 37 | MIB |  |  |  |  |  |  |  |  |  |
| 38 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 39 | MIB |  |  |  |  |  |  |  |  |  |
| 40 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 41 | MIB |  |  |  |  |  |  |  |  |  |
| 42 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 43 | MIB |  |  |  |  |  |  |  |  |  |
| 44 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 45 | MIB |  |  |  |  |  |  |  |  |  |
| 46 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 47 | MIB |  |  |  |  |  |  |  |  |  |
| 48 | MIB SI1-BR | SI1 |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 49 | MIB |  |  |  |  |  |  |  |  |  |
| 50 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 51 | MIB |  |  |  |  |  |  |  |  |  |
| 52 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 53 | MIB |  |  |  |  |  |  |  |  |  |
| 54 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 55 | MIB |  |  |  |  |  |  |  |  |  |
| 56 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 57 | MIB |  |  |  |  |  |  |  |  |  |
| 58 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 59 | MIB |  |  |  |  |  |  |  |  |  |
| 60 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 61 | MIB |  |  |  |  |  |  |  |  |  |
| 62 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 63 | MIB |  |  |  |  |  |  |  |  |  |
| 64 | MIB SI1-BR | SI1 |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 65 | MIB |  |  |  |  |  |  |  |  |  |
| 66 | MIB SI2-BR | SI2 |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 67 | MIB |  |  |  |  |  |  |  |  |  |
| 68 | MIB SI3-BR |  |  | SI3 | SIB1-BR | SIB1 |  |  |  |  |
| 69 | MIB |  |  |  |  |  |  |  |  |  |
| 70 | MIB SI4-BR |  |  | SI4 | SIB1-BR | SIB1 |  |  |  |  |
| 71 | MIB |  |  |  |  |  |  |  |  |  |
| 72 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |

Table 7.7.2a-3: BR System Information Scheduling (TDD) for DL bandwidth > 5 MHz [] and even physical layer cell Id [=0]

| SFN\SubFrame | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | MIB, SI1, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 1 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 2 | MIB, SI2, SI2-BR |  |  |  |  | SIB1 |  |  |  |  |
| 3 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 4 | MIB, SI3, SI3-BR |  |  |  |  | SIB1 |  |  |  |  |
| 5 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 6 | MIB, SI4, SI4-BR |  |  |  |  | SIB1 |  |  |  |  |
| 7 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 8 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 9 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 10 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 11 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 12 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 13 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 14 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 15 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 16 | MIB, SI1 SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 17 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 18 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 19 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 20 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 21 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 22 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 23 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 24 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 25 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 26 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 27 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 28 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 29 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 30 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 31 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 32 | MIB, SI1, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 33 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 34 | MIB, SI2, SI2-BR |  |  |  |  | SIB1 |  |  |  |  |
| 35 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 36 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 37 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 38 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 39 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 40 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 41 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 42 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 43 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 44 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 45 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 46 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 47 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 48 | MIB, SI1, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 49 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 50 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 51 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 52 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 53 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 54 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 55 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 56 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 57 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 58 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 59 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 60 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 61 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 62 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 63 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 64 | MIB, SI1, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 65 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 66 | MIB, SI2, SI2-BR |  |  |  |  | SIB1 |  |  |  |  |
| 67 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 68 | MIB, SI3, SI3-BR |  |  |  |  | SIB1 |  |  |  |  |
| 69 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 70 | MIB, SI4, SI4-BR |  |  |  |  | SIB1 |  |  |  |  |
| 71 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 72 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 73 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |

Table 7.7.2a-4: BR System Information Scheduling (FDD) for odd physical layer cell IDs[=1]

| SFN\SubFrame | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | MIB SI1-BR | SI1 |  |  |  | SIB1 |  |  |  |  |
| 1 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 2 | MIB SI2-BR | SI2 |  |  |  | SIB1 |  |  |  |  |
| 3 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 4 | MIB SI3-BR |  |  | SI3 |  | SIB1 |  |  |  |  |
| 5 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 6 | MIB SI4-BR |  |  | SI4 |  | SIB1 |  |  |  |  |
| 7 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 8 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 9 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 10 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 11 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 12 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 13 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 14 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 15 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 16 | MIB SI1-BR | SI1 |  |  |  | SIB1 |  |  |  |  |
| 17 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 18 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 19 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 20 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 21 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 22 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 23 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 24 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 25 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 26 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 27 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 28 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 29 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 30 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 31 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 32 | MIB SI1-BR | SI1 |  |  |  | SIB1 |  |  |  |  |
| 33 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 34 | MIB SI2-BR | SI2 |  |  |  | SIB1 |  |  |  |  |
| 35 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 36 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 37 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 38 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 39 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 40 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 41 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 42 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 43 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 44 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 45 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 46 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 47 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 48 | MIB SI1-BR | SI1 |  |  |  | SIB1 |  |  |  |  |
| 49 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 50 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 51 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 52 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 53 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 54 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 55 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 56 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 57 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 58 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 59 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 60 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 61 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 62 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 63 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 64 | MIB SI1-BR | SI1 |  |  |  | SIB1 |  |  |  |  |
| 65 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 66 | MIB SI2-BR | SI2 |  |  |  | SIB1 |  |  |  |  |
| 67 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 68 | MIB SI3-BR |  |  | SI3 |  | SIB1 |  |  |  |  |
| 69 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 70 | MIB SI4-BR |  |  | SI4 |  | SIB1 |  |  |  |  |
| 71 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 72 | MIB |  |  |  |  | SIB1 |  |  |  |  |

Table 7.7.2a-5: BR System Information Scheduling (TDD) for DL bandwidth > 5 MHz [] and odd physical layer cell Id [=1]

| SFN\SubFrame | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | MIB, SI1, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 1 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 2 | MIB, SI2, SI2-BR |  |  |  |  | SIB1 |  |  |  |  |
| 3 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 4 | MIB, SI3, SI3-BR |  |  |  |  | SIB1 |  |  |  |  |
| 5 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 6 | MIB, SI4, SI4-BR |  |  |  |  | SIB1 |  |  |  |  |
| 7 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 8 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 9 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 10 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 11 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 12 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 13 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 14 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 15 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 16 | MIB, SI1, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 17 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 18 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 19 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 20 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 21 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 22 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 23 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 24 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 25 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 26 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 27 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 28 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 29 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 30 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 31 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 32 | MIB, SI1, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 33 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 34 | MIB, SI2, SI2-BR |  |  |  |  | SIB1 |  |  |  |  |
| 35 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 36 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 37 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 38 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 39 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 40 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 41 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 42 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 43 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 44 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 45 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 46 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 47 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 48 | MIB, SI1, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 49 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 50 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 51 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 52 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 53 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 54 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 55 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 56 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 57 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 58 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 59 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 60 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 61 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 62 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 63 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 64 | MIB, SI1, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 65 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 66 | MIB, SI2, SI2-BR |  |  |  |  | SIB1 |  |  |  |  |
| 67 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 68 | MIB, SI3, SI3-BR |  |  |  |  | SIB1 |  |  |  |  |
| 69 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 70 | MIB, SI4, SI4-BR |  |  |  |  | SIB1 |  |  |  |  |
| 71 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 72 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 73 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |

Table 7.7.2a-6: BR System Information Scheduling (TDD) for DL bandwidth = 5 MHz

| SFN\SubFrame | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | MIB, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 1 | MIB, SI1 |  |  |  |  | SIB1-BR |  |  |  |  |
| 2 | MIB, SI2-BR |  |  |  |  | SIB1 |  |  |  |  |
| 3 | MIB, SI2 |  |  |  |  | SIB1-BR |  |  |  |  |
| 4 | MIB, SI3-BR |  |  |  |  | SIB1 |  |  |  |  |
| 5 | MIB, SI3 |  |  |  |  | SIB1-BR |  |  |  |  |
| 6 | MIB, SI4-BR |  |  |  |  | SIB1 |  |  |  |  |
| 7 | MIB, SI4 |  |  |  |  | SIB1-BR |  |  |  |  |
| 8 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 9 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 10 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 11 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 12 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 13 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 14 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 15 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 16 | MIB, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 17 | MIB, SI1 |  |  |  |  | SIB1-BR |  |  |  |  |
| 18 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 19 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 20 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 21 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 22 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 23 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 24 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 25 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 26 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 27 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 28 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 29 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 30 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 31 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 32 | MIB, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 33 | MIB, SI1 |  |  |  |  | SIB1-BR |  |  |  |  |
| 34 | MIB, SI2-BR |  |  |  |  | SIB1 |  |  |  |  |
| 35 | MIB, SI2 |  |  |  |  | SIB1-BR |  |  |  |  |
| 36 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 37 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 38 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 39 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 40 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 41 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 42 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 43 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 44 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 45 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 46 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 47 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 48 | MIB, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 49 | MIB, SI1 |  |  |  |  | SIB1-BR |  |  |  |  |
| 50 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 51 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 52 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 53 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 54 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 55 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 56 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 57 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 58 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 59 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 60 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 61 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 62 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 63 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 64 | MIB, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 65 | MIB, SI1 |  |  |  |  | SIB1-BR |  |  |  |  |
| 66 | MIB, SI2-BR |  |  |  |  | SIB1 |  |  |  |  |
| 67 | MIB, SI2 |  |  |  |  | SIB1-BR |  |  |  |  |
| 68 | MIB, SI3-BR |  |  |  |  | SIB1 |  |  |  |  |
| 69 | MIB, SI3 |  |  |  |  | SIB1-BR |  |  |  |  |
| 70 | MIB, SI4-BR |  |  |  |  | SIB1 |  |  |  |  |
| 71 | MIB, SI4 |  |  |  |  | SIB1-BR |  |  |  |  |
| 72 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 73 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |

Table 7.7.2a-7: BR System Information Scheduling (FDD DL Bandwidth = 3 MHz) for even physical layer cell IDs[=0]

| SFN\SubFrame | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | MIB SI1 |  |  |  | SIB1-BR | SIB1 |  | SI1-BR |  |  |
| 1 | MIB |  |  |  |  |  |  |  |  |  |
| 2 | MIB SI2 |  |  |  | SIB1-BR | SIB1 |  | SI2-BR |  |  |
| 3 | MIB |  |  |  |  |  |  |  |  |  |
| 4 | MIB SI3 |  |  |  | SIB1-BR | SIB1 |  | SI3-BR |  |  |
| 5 | MIB |  |  |  |  |  |  |  |  |  |
| 6 | MIB SI4 |  |  |  | SIB1-BR | SIB1 |  | SI4-BR |  |  |
| 7 | MIB |  |  |  |  |  |  |  |  |  |
| 8 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 9 | MIB |  |  |  |  |  |  |  |  |  |
| 10 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 11 | MIB |  |  |  |  |  |  |  |  |  |
| 12 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 13 | MIB |  |  |  |  |  |  |  |  |  |
| 14 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 15 | MIB |  |  |  |  |  |  |  |  |  |
| 16 | MIB SI1 |  |  |  | SIB1-BR | SIB1 |  | SI1-BR |  |  |
| 17 | MIB |  |  |  |  |  |  |  |  |  |
| 18 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 19 | MIB |  |  |  |  |  |  |  |  |  |
| 20 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 21 | MIB |  |  |  |  |  |  |  |  |  |
| 22 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 23 | MIB |  |  |  |  |  |  |  |  |  |
| 24 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 25 | MIB |  |  |  |  |  |  |  |  |  |
| 26 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 27 | MIB |  |  |  |  |  |  |  |  |  |
| 28 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 29 | MIB |  |  |  |  |  |  |  |  |  |
| 30 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 31 | MIB |  |  |  |  |  |  |  |  |  |
| 32 | MIB SI1 |  |  |  | SIB1-BR | SIB1 |  | SI1-BR |  |  |
| 33 | MIB |  |  |  |  |  |  |  |  |  |
| 34 | MIB SI2 |  |  |  | SIB1-BR | SIB1 |  | SI2-BR |  |  |
| 35 | MIB |  |  |  |  |  |  |  |  |  |
| 36 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 37 | MIB |  |  |  |  |  |  |  |  |  |
| 38 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 39 | MIB |  |  |  |  |  |  |  |  |  |
| 40 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 41 | MIB |  |  |  |  |  |  |  |  |  |
| 42 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 43 | MIB |  |  |  |  |  |  |  |  |  |
| 44 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 45 | MIB |  |  |  |  |  |  |  |  |  |
| 46 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 47 | MIB |  |  |  |  |  |  |  |  |  |
| 48 | MIB SI1 |  |  |  | SIB1-BR | SIB1 |  | SI1-BR |  |  |
| 49 | MIB |  |  |  |  |  |  |  |  |  |
| 50 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 51 | MIB |  |  |  |  |  |  |  |  |  |
| 52 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 53 | MIB |  |  |  |  |  |  |  |  |  |
| 54 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 55 | MIB |  |  |  |  |  |  |  |  |  |
| 56 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 57 | MIB |  |  |  |  |  |  |  |  |  |
| 58 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 59 | MIB |  |  |  |  |  |  |  |  |  |
| 60 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 61 | MIB |  |  |  |  |  |  |  |  |  |
| 62 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 63 | MIB |  |  |  |  |  |  |  |  |  |
| 64 | MIB SI1 |  |  |  | SIB1-BR | SIB1 |  | SI1-BR |  |  |
| 65 | MIB |  |  |  |  |  |  |  |  |  |
| 66 | MIB SI2 |  |  |  | SIB1-BR | SIB1 |  | SI2-BR |  |  |
| 67 | MIB |  |  |  |  |  |  |  |  |  |
| 68 | MIB SI3 |  |  |  | SIB1-BR | SIB1 |  | SI3-BR |  |  |
| 69 | MIB |  |  |  |  |  |  |  |  |  |
| 70 | MIB SI4 |  |  |  | SIB1-BR | SIB1 |  | SI4-BR |  |  |
| 71 | MIB |  |  |  |  |  |  |  |  |  |
| 72 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |

Table 7.7.2a-8: BR System Information Scheduling (FDD DL Bandwidth = 3 MHz) for odd physical layer cell IDs[=1]

| SFN\SubFrame | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | MIB SI1 |  |  |  |  | SIB1 |  | SI1-BR |  |  |
| 1 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 2 | MIB SI2 |  |  |  |  | SIB1 |  | SI2-BR |  |  |
| 3 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 4 | MIB SI3 |  |  |  |  | SIB1 |  | SI3-BR |  |  |
| 5 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 6 | MIB SI4 |  |  |  |  | SIB1 |  | SI4-BR |  |  |
| 7 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 8 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 9 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 10 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 11 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 12 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 13 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 14 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 15 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 16 | MIB SI1 |  |  |  |  | SIB1 |  | SI1-BR |  |  |
| 17 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 18 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 19 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 20 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 21 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 22 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 23 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 24 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 25 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 26 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 27 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 28 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 29 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 30 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 31 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 32 | MIB SI1 |  |  |  |  | SIB1 |  | SI1-BR |  |  |
| 33 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 34 | MIB SI2 |  |  |  |  | SIB1 |  | SI2-BR |  |  |
| 35 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 36 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 37 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 38 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 39 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 40 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 41 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 42 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 43 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 44 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 45 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 46 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 47 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 48 | MIB SI1 |  |  |  |  | SIB1 |  | SI1-BR |  |  |
| 49 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 50 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 51 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 52 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 53 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 54 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 55 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 56 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 57 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 58 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 59 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 60 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 61 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 62 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 63 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 64 | MIB SI1 |  |  |  |  | SIB1 |  | SI1-BR |  |  |
| 65 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 66 | MIB SI2 |  |  |  |  | SIB1 |  | SI2-BR |  |  |
| 67 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 68 | MIB SI3 |  |  |  |  | SIB1 |  | SI3-BR |  |  |
| 69 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 70 | MIB SI4 |  |  |  |  | SIB1 |  | SI4-BR |  |  |
| 71 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 72 | MIB |  |  |  |  | SIB1 |  |  |  |  |

<End of modified section>

<Start of modified section>

## 7.26	Type B FDD Half-Duplex Considerations

Type B FDD half-duplex UE has limitations in receiving a DL subframe preceding or following a UL subframe for the same UE hence a blank subframe is needed. In [non TTI bundling considerations] max 3 contiguous subframes UL or DL can be scheduled. And after and before any contiguous DL/UL subframes block a blank subframe is needed.

If the UE under test is a Type B FDD half-duplex UE:

-	SS will be configured per cell by providing UE category info and FDD half-duplex operation.

-	When explicit timing info is provided in any DL transmission, the SS shall ignore the UE capability and shall follow the explicit scheduling. It will be TTCN responsibility to schedule UL/DL transmissions based on UE capability. Paging is always explicitly scheduled and hence SS follows explicit scheduling for paging.

-	For UE Category 0, SI scheduling will be configured as per TDD SI scheduling (Table 7.7.2-4) restricting system info transmissions in sub frames 0 and 5. For non-explicitly scheduled new DL transmissions/ new UL grant allocations, the SS shall restrict UL/DL operations as per table below:

Table 7.26-1: Scheduling pattern for type B half-duplex FDD Cat.0 UE (non-explicitly scheduled)

| SFN\ SubFrame | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X (any) | MIB/SI | Blank | New UL Data/ HARQ | Blank | Blank | SIB1/SI | DL HARQ | Blank | New DL Data/ UL Grant | Blank |

- For BL/CE UE, SI scheduling is specified in clause 7.7.2a. BL/CE UE acquires sysinfo in RRC_IDLE. In RRC_CONNECTED BL/CE UE acquires MIB/SIB1-BR only when T311 is running. For non-explicitly scheduled new DL transmissions/ new UL grant allocations:

-	No subframe restriction using fdd-DownlinkOrTddSubframeBitmapBR nor fdd-UplinkSubframeBitmapBR are provided by TTCN to SS,

-	The SS shall restrict UL/DL operations as per table below:

Table 7.26-2: Scheduling pattern for type B half-duplex FDD BL/CE UE (non-explicitly scheduled)

| SFN\ SubFrame | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X (any) | MIB/SI-BR/New DL Data | Blank | New UL Data/ HARQ | Blank | SIB1-BR | Blank | DL HARQ | Blank | DL Grant/ UL Grant | Blank |

Table 7.26-3: Scheduling pattern for type B half-duplex FDD BL/CE UE (non-explicitly scheduled) DL Bandwidth = 3 MHz

| SFN\ SubFrame | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X (any) | MIB | Blank | New UL Data/ HARQ | Blank | SIB1-BR | Blank | DL HARQ/DL Grant | SI-BR | New DL Data/UL Grant | Blank |

NOTE 1:	As per TS 36.213[30] clause 7.1.11, the DL data will be 2 subframes after the subframe containing DL Grant on MPDCCH. As per TS 36.213[30] clause 8.0, the UL data will be 4 subframes after the subframe containing UL Grant on MPDCCH.

NOTE 2:	For new DL data transmission, SS transmits DL DCI in subframe 8 and PDSCH data in subframe 0 of the next subframe.

-	If HARQ retransmissions are required:

-	DL retransmissions being asynchronous in nature shall be scheduled by the SS as per above schedule (i.e. subframe 8).

-	UL retransmissions being synchronous will not be limited to subframe 2. i.e. the first UL retransmission will be in frame X+1, subframe 0, second retransmission in frame X+1, subframe 8 and so on.

-	HARQ retransmissions shall be prioritised over new transmissions.

<End of modified section>

<Start of modified section>

# 11	Guidelines on test execution

This clause provides the guidelines on test executions.

The restriction on test case execution as listed in this clause is due to the restriction of bandwidth to accommodate the necessary number of radio frequencies for the specific operating Band as used by the test cases.

## 11.1	E-UTRA single technology

This clause provides the guidelines for the test cases to be executed on the pure E-UTRA test configuration.

A test case using more than one radio frequency, i.e. using the radio frequencies f2 or f3 or f4 specified in TS 36.508 [3], shall avoid to be executed on operating:

Band 13,

Band 18,

Band 31,

Band 54

Band 72,

Band 73,

Band 87,

Band 88,

Band 106.

The list containing such test cases is given below:

6.1.1.1, 6.1.1.2, 6.1.1.3, 6.1.1.6, 6.1.1.7, 6.1.1.8, 6.1.1.12, 6.1.1.13, 6.1.1.14, 6.1.2.6b, 6.1.2.7, 6.1.2.8, 6.1.2.9, 6.1.2.11, 6.1.2.11a, 6.1.2.13, 6.1.2.15, 6.1.2.17, 6.1.2.18, 6.3.1, 6.3.5, 6.3.6, 6.3.9, 6.4.1, 6.4.2,

8.1.3.4, 8.1.3.5, 8.1.3.5a, 8.2.4.6, 8.2.4.26, 8.2.4.28, 8.3.1.3, 8.3.1.3a, 8.3.1.4, 8.3.1.6, 8.3.1.9, 8.3.1.10, 8.3.1.11, 8.3.1.26, 8.3.1.27, 8.3.1.31, 8.3.4.2, 8.3.4.3, 8.3.4.5, 8.6.2.2, 8.6.2.3a, 8.6.2.8, 8.6.2.10, 8.6.2.11, 8.6.2.12, 8.6.4.2, 8.6.4.3, 8.6.4.4, 8.6.4.8, 8.6.4.9, 8.6.4.10, 8.6.6.2, 8.6.6.5, 8.6.6.6, 8.6.6.7, 8.6.8.2, 8.6.8.6, 8.9.4a, 8.9.5a,

9.2.1.1.1a, 9.2.1.1.7, 9.2.1.1.13, 9.2.1.1.15, 9.2.1.1.16, 9.2.1.2.1, 9.2.1.2.10, 9.2.1.2.12, 9.2.1.2.14, 9.2.3.2.1, 9.2.3.2.12, 9.2.3.2.15, 9.2.3.2.16, 9.2.5.1,

11.2.6, 11.2.7,

13.4.1.2,

17.4.1, 17.4.2, 17.4.3,

21.3.1, 21.3.2, 21.3.3, 21.3.7,

24.1.3.

A test case using more than two radio frequencies, i.e. using the radio frequencies f3 or f4 specified in TS 36.508 [3], shall avoid to be executed on operating:

Band 6,

Band 11,

Band 14,

Band 17,

Band 23 with 10MHz bandwidth,

Band 24,

Band 30,

Band 38,

Band 39

Band 53

The list containing such test cases is given below:

6.1.1.1, 6.1.1.2, 6.1.1.3, 6.1.1.6, 6.1.1.7, 6.1.1.8, 6.1.1.12, 6.1.1.13, 6.1.1.14, 6.1.2.7, 6.1.2.8, 6.1.2.9, 6.1.2.11a, 6.1.2.15, 6.3.5

8.1.3.5, 8.1.3.5a, 8.3.1.4, 8.6.4.3, 8.9.4a, 8.9.5a,

9.2.1.1.1a, 9.2.1.1.7, 9.2.1.1.15,

11.2.7.

A test case using more than three radio frequencies, i.e. using the radio frequency f4 specified in TS 36.508 [3], shall avoid to be executed on operating:

Band 12,

Band 19,

Band 20,

Band 21,

Band 27,

Band 34,

Band 70,

Band 85.

The list containing such test cases is given below:

6.1.1.1, 6.1.1.2, 6.1.1.6, 6.1.1.8,

8.1.3.5a,

9.2.1.1.7.

### 11.1.1	Replacement of test case execution

In case of bandwidth limitation for accommodation of more frequencies, a number of test cases can be replaced with the corresponding mirror test cases without affecting the test coverage. The table 11.1.1-1 shows the possible replacements. Only one of the paired test cases is required for execution.

Table 11.1.1-1: Replacement of test cases

| Original test case | Replacing test case |
| --- | --- |
| 6.1.1.1 | 6.1.1.1b |
| 6.1.1.2 | 6.1.1.2a |
| 6.1.1.3 | 6.1.1.3b |
| 6.1.1.6 | 6.1.1.6a or 6.1.1.6b |
| 6.1.1.7 | 6.1.1.7a |
| 6.1.2.7 | 6.1.2.7a |
| 6.1.2.8 | 6.1.2.8a |
| 6.1.2.9 | 6.1.2.9a |
| 8.1.3.12 | 8.1.3.12b |
| 8.3.1.9 | 8.3.1.9a |
| 8.3.1.11 | 8.3.1.11a |
| 9.2.1.1.1a | 9.2.1.1.1b |
| 9.2.1.1.7 | 9.2.1.1.7a |
| 9.2.1.1.13 | 9.2.1.1.13a |
| 9.2.1.1.15 | 9.2.1.1.15a |
| 9.2.1.1.16 | 9.2.1.1.16a |
| 9.2.3.1.15 | 9.2.3.1.15a |
| 9.2.3.1.18 | 9.2.3.1.18a |
| 17.4.1 | 17.4.1a |

## 11.2	E-UTRA - UTRA - GERAN

This clause contains the guidelines for the E-UTRA interRAT test cases to be executed on the different test configurations: with only UTRA configured, with only GERAN configured or with UTRA-GERAN both configured. Whether or not an E-UTRA frequency band overlaps the UTRA band, the dependency will affect the restrictions of the test execution on this band.

NOTE:	The case where an E-UTRA band overlaps a GSM band is not considered.

### 11.2.1	UTRA configured – GERAN not configured

This clause provides the guidelines for the E-UTRA interRAT test cases where UTRA is configured, while GERAN is either not needed or not configured.

#### 11.2.1.1	E-UTRA band overlapping UTRA band

The restriction on test case execution as listed in this clause is due to the bandwidth of an E-UTRA Band accommodating the necessary number of E-UTRA or E-UTRA, UTRA radio frequencies if an E-UTRA band overlaps the UTRA Band. A test case using more than one radio frequency, on the same E-UTRA and UTRA band, shall avoid to be executed on operating

Band 13,

Band 18,

Band 31.

The list containing such test cases is given below:

6.2.1.2, 6.2.1.3, 6.2.2.1, 6.2.2.5, 6.2.2.8, 6.2.3.3, 6.2.3.3a, 6.2.3.4, 6.2.3.4a, 6.2.3.5, 6.2.3.5a, 6.2.3.6, 6.2.3.13, 6.2.3.31, 6.2.3.32, 6.2.3.33, 6.2.4.1, 6.2.4.2, 6.2.4.3, 6.2.4.4, 6.2.4.5, 6.2.4.6, 6.2.4.7, 6.3.3, 6.3.4, 6.3.7, 6.4.3, 6.4.4, 6.4.5, 6.4.6,

8.1.3.6, 8.1.3.6a, 8.1.3.6b, 8.1.3.7, 8.3.2.3, 8.3.2.3a, 8.3.2.4, 8.3.3.2, 8.3.4.4, 8.4.1.2, 8.4.1.4, 8.4.1.5, 8.4.2.2, 8.4.2.4, 8.5.2.1, 8.6.3.1, 8.6.3.4, 8.6.5.1, 8.6.5.1a, 8.6.7.1, 8.6.7.4, 8.6.9.1, 8.6.9.2, 8.6.10.1, 8.7.1,

9.2.1.1.11, 9.2.1.1.12, 9.2.1.2.1b, 9.2.1.2.1c, 9.2.1.2.1d, 9.2.1.2.5, 9.2.1.2.8, 9.2.1.2.9, 9.2.1.2.11, 9.2.1.2.13, 9.2.1.2.15, 9.2.2.1.3, 9.2.2.1.10, 9.2.3.1.6, 9.2.3.1.10, 9.2.3.1.11, 9.2.3.1.12, 9.2.3.1.15, 9.2.3.1.15a, 9.2.3.1.17, 9.2.3.1.18, 9.2.3.1.18a, 9.2.3.2.1a, 9.2.3.2.1b, 9.2.3.2.1c, 9.2.3.2.3, 9.2.3.2.5, 9.2.3.2.6, 9.2.3.2.7, 9.2.3.2.8, 9.2.3.2.9, 9.2.3.2.11, 9.2.3.2.13, 9.2.3.2.14, 9.2.3.3.1, 9.2.3.3.2, 9.2.3.3.3, 9.2.3.3.4, 9.2.3.3.5, 9.2.3.3.5a, 9.3.1.4, 9.3.1.5, 9.3.1.6,

11.2.10, 11.2.11, 11.3.3, 11.3.5, 11.3.7,

13.1.2, 13.1.2a, 13.1.3, 13.1.4, 13.1.5, 13.1.15, 13.1.16, 13.3.2.1, 13.4.2.1, 13.4.2.4, 13.4.3.1, 13.4.3.2, 13.4.3.4, 13.4.3.6, 13.4.3.7, 13.4.3.8, 13.4.3.9, 13.4.3.10, 13.4.3.11, 13.4.3.13, 13.4.3.14, 13.4.3.15, 13.4.3.16, 13.4.3.18, 13.4.3.19, 13.4.3.20.

A test case using more than two radio frequencies on the same E-UTRA and UTRA band shall avoid to be executed on operating:

Band 6,

Band 11,

Band 14,

Band 17,

Band 23 with 10MHz bandwidth,

Band 38,

Band 39.

The list containing such test cases is given below:

6.2.1.2, 6.2.1.3,

8.6.3.4, 8.6.5.1a, 8.6.7.4,

9.2.1.2.9, 9.2.1.2.11, 9.2.1.2.13, 9.2.3.1.15, 9.2.3.1.18, 9.2.3.2.5, 9.2.3.2.6, 9.2.3.2.7, 9.2.3.2.8, 9.2.3.2.11, 9.2.3.2.13, 9.2.3.2.14.

A test case using more than three radio frequencies, on the same E-UTRA and UTRA band shall avoid to be executed on operating

Band 12,

Band 19,

Band 20,

Band 21,

Band 27,

Band 34,

Band 70,

Band 85.

The list containing such test cases is given below:

9.2.1.2.13, 9.2.3.2.13.

#### 11.2.1.2	E-UTRA band not overlapping UTRA band

The restriction on test case execution as listed in this clause is due to the bandwidth of an E-UTRA Band accommodating the necessary number of E-UTRA radio frequencies. A test case using more than one radio frequency shall avoid to be executed on E-UTRA operating

Band 13,

Band 18,

Band 31,

Band 54,

Band 72,

Band 73,

Band 87,

Band 88,

Band 106.

The list containing such test cases is given below:

6.2.1.2, 6.2.1.3,

8.6.3.4, 8.6.5.1a, 8.6.7.4,

9.2.1.2.9, 9.2.1.2.11, 9.2.1.2.13, 9.2.3.1.15, 9.2.3.1.18, 9.2.3.2.5, 9.2.3.2.6, 9.2.3.2.7, 9.2.3.2.8, 9.2.3.2.11, 9.2.3.2.13, 9.2.3.2.14,

A test case using more than two radio frequencies shall avoid to be executed on E-UTRA operating

Band 6,

Band 11,

Band 14,

Band 17,

Band 23 with 10MHz bandwidth,

Band 24,

Band 30,

Band 38,

Band 39.

The list containing such test cases is given below:

9.2.1.2.13, 9.2.3.2.13.

### 11.2.2	GERAN configured - UTRA not configured

This clause provides the guidelines for the E-UTRA/GERAN test cases where UTRA is either not needed or not configured. The restriction on test case execution as listed in this clause is due to the restriction of bandwidth of an E-UTRA Band accommodating the necessary number of E-UTRA radio frequencies.

A test case using more than one radio frequency shall avoid to be executed on E-UTRA operating

Band 13,

Band 18,

Band 31,

Band 54,

Band 72,

Band 73,

Band 87,

Band 88,

Band 106.

The list containing such test cases is given below:

6.2.1.4, 6.2.3.17, 6.2.3.18,

8.3.2.2,

9.2.1.2.9, 9.2.1.2.11, 9.2.1.2.13, 9.2.3.1.15, 9.2.3.1.18, 9.2.3.2.5, 9.2.3.2.6, 9.2.3.2.7, 9.2.3.2.8, 9.2.3.2.11, 9.2.3.2.13, 9.2.3.2.14.

A test case using more than two radio frequencies, i.e. using the radio frequencies f3 or f4 specified in TS 36.508 [3], shall avoid to be executed on operating:

Band 6,

Band 11,

Band 14,

Band 17,

Band 23 with 10MHz bandwidth,

Band 24,

Band 30,

Band 38,

Band 39

The list containing such test cases is given below:

9.2.1.2.13, 9.2.3.2.13.

### 11.2.3	Neither UTRA nor GERAN configured

Certain EMM test cases can be executed as EUTRA_Only configuration despite of UTRA or GERAN test branches included in the test cases. The restriction on test case execution as listed in this clause is due to the bandwidth of an E-UTRA Band accommodating the necessary number of E-UTRA radio frequencies.

A test case using more than one radio frequency shall avoid to be executed on E-UTRA operating

Band 13,

Band 18,

Band 31,

Band 54,

Band 72,

Band 73,

Band 87,

Band 88,

Band 106.

The list containing such test cases is given below:

9.2.3.1.15, 9.2.3.1.18.

### 11.2.4	Both UTRA and GERAN configured

This clause provides the guidelines for the E-UTRA - UTRA - GERAN test cases where three RAT technologies are simultaneously configured.

#### 11.2.4.1	E-UTRA band overlapping UTRA band

The restriction on test case execution as listed in this clause is due to the bandwidth of an E-UTRA Band accommodating the necessary number of E-UTRA or E-UTRA, UTRA radio frequencies if an E-UTRA band overlaps the UTRA Band.

A test case using more than one radio frequency shall avoid to be executed on E-UTRA operating

Band 13,

Band 18,

Band 31.

The list containing such test cases is given below:

6.2.1.1, 8.3.2.5, 8.3.2.6, 13.1.21,

9.2.1.2.6, 9.2.1.2.7,

11.2.8

A test case using more than two radio frequencies, i.e. using the radio frequencies f3 or f4 specified in TS 36.508 [3], shall avoid to be executed on operating:

Band 6,

Band 11,

Band 14,

Band 17,

Band 23 with 10MHz bandwidth,

Band 38

Band 39

The list containing such test cases is given below:

6.2.1.1.

#### 11.2.4.2	E-UTRA band not overlapping UTRA band

The restriction on test case execution as listed in this clause is due to the restriction of bandwidth of an E-UTRA Band accommodating the necessary number of E-UTRA radio frequencies.

A test case using more than one radio frequency shall avoid to be executed on E-UTRA operating

Band 13,

Band 18,

Band 31,

Band 54

Band 72,

Band 73,

Band 87,

Band 88,

Band 106.

The list containing such test cases is given below:

6.2.1.1

### 11.2.5	Replacement of test case execution

In case of bandwidth limitation for accommodation of more frequencies, a number of test cases can be replaced with the corresponding mirror test cases without affecting the test coverage. The table 11.2.5-1 shows the possible replacements. Only one of the paired test cases is required for execution.

Table 11.2.5-1: Replacement of interRAT test cases

| Original test case | Replacing test case |
| --- | --- |
| 9.2.3.1.15 | 9.2.3.1.15a |
| 9.2.3.1.18 | 9.2.3.1.18a |

## 11.3	E-UTRA inter-band

The restriction on test case execution as listed in this clause is due to the restriction of bandwidth of an E-UTRA band accommodating the necessary number of E-UTRA radio frequencies. The inter-band test includes also E-UTRA FDD-TDD and inter-band carrier aggregation tests.

### 11.3.1	Primary operating band

A test case using more than one radio frequency on the first operating band, shall avoid to be executed on operating

Band 13,

Band 18,

Band 31,

Band 54,

Band 72,

Band 73,

Band 87,

Band 88,

Band 106.

The list containing such test cases is given below:

6.1.2.15a, 8.1.3.12.

### 11.3.2	Secondary operating band for inter-band cells

Test case using more than one radio frequency, on the second operating band, shall avoid to be executed on operating

Band 13,

Band 18,

Band 31,

Band 54,

Band 72,

Band 73,

Band 87,

Band 88,

Band 106.

The list containing such test cases is given below:

6.1.1.1a, 6.1.1.3a, 6.1.1.4a, 6.1.2.5c, 6.1.2.16, 8.1.3.11a, 8.1.3.12b, 8.2.4.13a, 8.2.4.14a, 8.3.1.12a, 8.3.1.14a

### 11.3.3	Replacement of test case execution

In case of bandwidth limitation for accommodation of more frequencies, a number of test cases can be replaced with the corresponding mirror test cases without affecting the test coverage. The table 11.3.3-1 shows the possible replacements. Only one of the paired test cases is required for execution.

Table 11.3.3-1: Replacement of test cases

| Original test case | Replacing test case |
| --- | --- |
| 8.1.3.12 | 8.1.3.12b |

### 11.3.4	Inter-band MBMS

MBMS service continuity test cases shall avoid to be executed on operating bands with different bandwidth.

The list containing such test cases is given below:

17.4.3a

## 11.4	E-UTRA CA

The restriction on CA test case execution as listed in this clause is due to the restriction of bandwidth of an E-UTRA CA band accommodating the necessary number of E-UTRA radio frequencies.

### 11.4.1	CA contiguous Intra-band operation

Test case using more than two radio frequencies, i.e. using the radio frequencies f3 or f4 specified in TS 36.508 [3], shall avoid to be executed on E-UTRA CA Configuration:

CA_5B,

CA_23B,

CA_38C,

CA_39C,

CA_70C.

The list containing such test cases is given below:

8.2.4.19.1, 8.2.4.21.1

The following test cases with UL transmission on f2 are not applicable to be run when executed in the band combinations CA_70C:

7.1.2.10.1,

7.1.2.11.1,

7.1.4.20.1,

8.2.2.7.1,

8.2.4.17.1,

8.2.4.20.1,

8.2.4.23.1,

8.3.1.17.1,

8.3.1.18.1

### 11.4.2	CA Inter-band operation

Test case using more than one radio frequency on the primary band, i.e. using the radio frequency f2 specified in TS 36.508 [3], shall avoid to be executed on E-UTRA CA Configuration:

CA_3A-11A,

CA_5A-29A,

CA_8A-11A,

CA_12A-25A,

CA_12A-30A,

CA_12A-66A,

CA_13A-66A,

CA_14A-30A,

CA_14A-66A,

CA_18A-28A,

CA_19A-21A,

CA_19A-28A,

CA_19A-42A,

CA_19A-42C,

CA_21A-42A,

CA_21A-42C,

CA_30A-48A,

CA_30A-66A,

CA_70A-71A,

CA_70C-71A

and on E-UTRA CA switched configuration

CA_1A-18A,

CA_1A-19A,

CA_1A-21A,

CA_2A-12A,

CA_2A-13A,

CA_2A-14A,

CA_2A-17A,

CA_2A-30A,

CA_3A-11A,

CA_3A-19A,

CA_4A-12A,

CA_4A-13A,

CA_4A-17A,

CA_4A-30A,

CA_5A-12A,

CA_5A-13A,

CA_5A-17A,

CA_5A-30A,

CA_7A-12A,

CA_8A-11A,

CA_11A-18A,

CA_12A-30A,

CA_14A-30A,

CA_19A-21A,

CA_20A-31A,

CA_29A-30A,

CA_29A-70C,

CA_46A-70A,

CA_46A-70A,

CA_66A-70A,

CA_66A-70C,

CA_66C-70A,

CA_66A-70C

The list containing such test cases is given below:

8.2.4.19.2

Test case using more than one radio frequency on the primary or secondary band, i.e. using the radio frequency f2 or f6 (depending upon the Cell being used) specified in TS 36.508 [3], shall avoid to be executed on E-UTRA CA Configuration:

CA_3A-11A,

CA_8A-11A,

CA_19A-21A,

CA_12A-30A

CA_14A-30A,

and on E-UTRA CA switched configuration

CA_3A-11A,

CA_8A-11A,

CA_19A-21A,

CA_12A-30A,

CA_14A-30A,

CA_29A-30A

The list containing such test cases is given below:

8.2.4.21.2

These test cases can be run with switched allocation of PCell and SCell as specified in TS 36.508 [3] clause 6.2.3.2 (NOTE 3 and 4).

These test cases with switched allocation of PCell and SCell or with UL transmission on SCell shall avoid to be executed on E-UTRA CA Configuration

with a band having no UL frequency

CA_2A-29A,

CA_4A-29A,

CA_5A-29A,

CA_3A-32A,

CA_3A-69A,

CA_20A-32A,

CA_23A-29A,

CA_20A-67A

and on E-UTRA CA switched configuration

CA_29A-30A,

CA_29A-66A,

CA_29A-70A,

CA_29A-70C

with an LAA unlicenced band

CA_1A-46A,

CA_2A-46A,

CA_3A-46A,

CA_4A-46A,

CA_7A-46A,

CA_41A-46A,

CA_42A-46A

and with an LAA unlicenced band-switched configuration

CA_46A-66A,

CA_46C-66A,

CA_46A-70A,

8.2.4.17.2,

8.2.4.20.2,

8.2.4.23.2,

17.4.11.2,

21.3.12.2

The PICS for UL CA 36.523-2 table A.4.3.3.3-2 shall be set in accordance with the UL CA support in the Band combination under test.

The 3DL CA test cases can be run on 3CCs FDD-TDD band combinations, they are using more than one radio frequency on the primary band, i.e. using the radio frequency f2 specified in TS 36.508 [3], shall avoid to be executed on E-UTRA CA Configuration:

CA_19A-42C,

CA_21A-42A,

CA_21A-42C,

CA_70A-71A

The list containing such test cases is given below:

7.1.2.11.4

### 11.4.3	CA non-contiguous Intra-band operation

Test case using more than two radio frequencies, i.e. using the radio frequencies f3 specified in TS 36.508 [3], shall avoid to be executed on E-UTRA CA Configuration:

CA_5A-5A,

CA_23A-23A

The list containing such test cases is given below:

8.2.4.19.3

8.2.4.21.3

## 11.5	E-UTRA MFBI

The following E-UTRA MFBI test cases shall be executed using the combinations specified in Table 11.5-1 for px_OverlappingNotSupportedFrequencyBand_MFBI and px_MFBI_FrequencyBand:

6.1.2.19, 6.1.2.20, 6.1.2.21, 6.1.2.22, 6.2.3.34, 6.2.3.35

Table 11.5-1: Operating and MFBI E-UTRA bands combinations

| px_OverlappingNotSupportedFrequencyBand_MFBI | px_MFBI_FrequencyBand (Note) |
| --- | --- |
| 2 | 25 |
| 3 | 9 |
| 4 | 10, 66 |
| 5 | 18, 19, 26 |
| 9 | 3 |
| 10 | 4, 66 |
| 12 | 17 |
| 17 | 12, 85 |
| 18 | 5, 26, 27 |
| 19 | 5, 26 |
| 25 | 2 |
| 26 | 5, 18, 19, 27 |
| 27 | 18, 26 |
| 33 | 39 |
| 38 | 41 |
| 39 | 33 |
| 41 | 38 |
| 66 | 4, 10 |
| 85 | 17 |
| 106 | 8 |
| Note: The UE supports one or more of the listed MFBI bands and does not support at least one overlapping band. If the UE supports all overlapping bands, these test cases are not applicable. |  |

Test case 6.1.2.20 is not applicable to be run when executed in the band combination 5 & 18.

For test case 6.2.3.34 additionally the UTRA band combinations specified in Table 11.5-2 for px_UTRAN_OverlappingNotSupportedFrequencyBandMFBI and px_MFBI_UTRAN_FrequencyBand shall also be applicable

Table 11.5-2: Operating and MFBI UTRA bands combinations

| px_UTRAN_OverlappingNotSupportedFrequencyBandMFBI | px_MFBI_UTRAN_FrequencyBand (Note) |
| --- | --- |
| 2 | 25 |
| 3 | 9 |
| 4 | 10 |
| 5 | 18, 19, 26 |
| 9 | 3 |
| 10 | 4 |
| 18 | 5, 26 |
| 19 | 5, 26 |
| 25 | 2 |
| 26 | 5, 18, 19 |
| Note: The UE supports one or more of the listed MFBI bands and does not support at least one overlapping band. If the UE supports all overlapping bands, these test cases are not applicable. |  |

The MFBI test case 6.1.2.23 shall be executed using the combinations specified in Table 11.5-3 for px_ePrimaryFrequencyBand, px_MFBI_FrequencyBand and px_OverlappingSupportedFrequencyBand_MFBI

Table 11.5-3: Operating and MFBI E-UTRA bands combinations

| px_ePrimaryFrequencyBand | px_MFBI_FrequencyBand | px_OverlappingSupportedFrequencyBand_MFBI |
| --- | --- | --- |
| 39 | 38 | 41 |

The following NB-IoT MFBI test cases shall be executed using the combinations specified in Table 11.5-4 for px_OverlappingNotSupportedFrequencyBand_MFBI and px_MFBI_FrequencyBand:

22.2.10

Table 11.5-4: Operating and MFBI E-UTRA bands combinations

| px_OverlappingNotSupportedFrequencyBand_MFBI | px_MFBI_FrequencyBand (Note) |
| --- | --- |
| 2 | 25 |
| 5 | 18, 19, 26 |
| 8 | 106 |
| 12 | 17 |
| 17 | 12, 85 |
| 19 | 5, 26 |
| 25 | 2 |
| 26 | 5, 18, 19 |
| 85 | 17 |
| 106 | 8 |
| Note: The UE supports one or more of the listed MFBI bands and does not support at least one overlapping band. If the UE supports all overlapping bands, these test cases are not applicable. |  |

<End of modified section>