3GPP TSG-RAN5 Meeting #108	R5-254044

# Bengaluru, India, 25th - 29th August, 2025

Agenda item:		5.4.20

Source:				Keysight Technologies UK Ltd

Title:					On Re-Positioning Concept for 3 AoA FR2 RRM TCs

Document for:		Discussion and Endorsement

# Introduction

The re-positioning concept has been adopted for FR2 UE RF TCs, and FR2 RRM TCs up to 2 AoAs. This contribution is exploring the applicability for 3 AoA RRM TCs.

# Discussion

The re-positioning concept, Clause N.3 of [1], has been firmly adopted for FR2 UE RF test cases, especially in Annex K of [1]. Last year, the re-positioning concept was discussed for 1 AoA and 2 AoA RRM TCs [2] and consequently adopted [3][4].

The reference measurement uncertainty (MU) [5] and corresponding Maximum Test System Uncertainty (MTSU) [1] for UE RF test cases was defined based on the re-positioning concept adopted, i.e., the Quality of Quiet Zone (QoQZ) MU elements in the measurement and calibration stage are based on an abbreviated test procedure that yields a lower MU due to the lack of positioner blocking, e.g., Clause O.2.6.2 of [1].

If the device re-positioning approach outlined in Annex N is adopted for all EIRP/EIS/TRP based conformance test cases, the quality of quiet zone analysis is sufficient only for  = 0°, 45°, 90°, 270°, and 315°.

Observation 1: FR2 UE RF test cases take the re-positioning approach into account and the reference MU/MTSU are based on the re-positioning approach adopted.

For 2 AoA RRM TCs, the reference MU/MTSU was also defined with the re-positioning concept applied [4][5] even though some restrictions in positioner placements were demonstrated, e.g., Figure 4 of [2] showed areas in red that the positioner, if the mast is placed within these areas, will yield blocking to at least one probe with two probes (blue and red) active.

Figure 4: Ranges of positioner locations for 2 AoA TCs that are applicable with re-position concept (separately for each 2 AoA combination with P0 active)

Observation 2: 1 AoA and 2 AoA RRM test cases take the re-positioning approach into account and the reference MU/MTSU are based on the re-positioning approach adopted.

When 3 probes are active for traditional RRM TCs, e.g., Setup 7 defined in Clause A.9.7 of [4], even more restrictions in terms of viable positioner ranges that do not introduce blocking effects are evident. This is further illustrated in Figure 1 where three different 3 active probe scenarios are shown with maximum relative angular separation of 60° (top left) through 150° (bottom right). Again, only when the positioner mast is placed within the highlighted green range, no positioner blocking is experienced; however, when the positioner mast is placed within the red area, at least one of the active probes will experience potentially severe blocking.

Figure 1: Ranges of positioner locations for 3 AoA TCs that are applicable (shown in green) with re-position concept with no positioner blocking effects for three probes (1 x blue, 2 x red) active

Clearly, since RRM test cases for 3 AoAs require changing the relative angular separations between iterations, as outlined in Clause A.9.7 [4],

A.9.7 Setup 7: 3 AoAs There are 3 active probes in the test. The DL signals, and noise if applicable, transmitted from the three active probes, align to directions (AoAs) which are from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [3] for each UE power class. The relative angular offset between the directions (AoAs) of the 3 active probes, shall be changed for each test iteration. Any combinations of two relative angular offsets between 2 active probes specified in Table A.9.3-1, is considered as a valid applicable relative angular offsets between 3 active probes for the respective power class.

applying the re-positioning concept would result in a very limited set of ranges for the positioner to avoid blocking effects. Additionally, most commercial Multi-AoA RRM test systems do not rely on 6 discrete probes but rather 4 probes which have additional constraints on the selection of active probes and thus positioner blocking.

Observation 3: 3 AoA RRM Test Cases yield very limited set of ranges for the positioner to avoid blocking effects with the re-positioner concept applied

It is therefore proposed that for 3 AoA RRM TCs, the re-positioning concept is not considered the default approach given the very limited positioner ranges, i.e., the reference MU/MTSU for these test cases are defined with the QoQZ MU that does not take re-positioning into account.

Proposal 1: For 3 AoA RRM TCs, the re-positioning concept is not considered the default approach given the very limited positioner ranges, i.e., the reference MU/MTSU for these test cases are defined without the optimized QoQZ MU.

To determine the 3 AoA RRM reference MU/MTSU, test system vendors will have to assess their chambers for QoQZ MU without the re-positioning concept applied. Until this QoQZ MU is defined, the MTSU for TCs using 3 AoAs cannot be finalized.

Proposal 2: Test system vendors to assess their chambers for QoQZ MU without the re-positioning concept applied.

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: FR2 UE RF test cases take the re-positioning approach into account and the reference MU/MTSU are based on the re-positioning approach adopted.

Observation 2: 1 AoA and 2 AoA RRM test cases take the re-positioning approach into account and the reference MU/MTSU are based on the re-positioning approach adopted.

Observation 3: 3 AoA RRM Test Cases yield very limited set of ranges for the positioner to avoid blocking effects with the re-positioner concept applied

Proposal 1: For 3 AoA RRM TCs, the re-positioning concept is not considered the default approach given the very limited positioner ranges, i.e., the reference MU/MTSU for these test cases are defined without the optimized QoQZ MU.

Proposal 2: Test system vendors to assess their chambers for QoQZ MU without the re-positioning concept applied.

# References

- TS 38.521-2, User Equipment (UE) conformance specification; Radio transmission and reception; Part 2: Range 2 Standalone

- R5-240601, On Re-Positioning Concept for FR2 RRM TCs, Keysight Technologies, 3GPP TSG-RAN5 Meeting #102, March 2024

- R5-241862, Introducing the Re-Positioning Concept for FR2 RRM TCs, Keysight Technologies, Rohde & Schwarz, 3GPP TSG-RAN5 Meeting #102, March 2024

- TS 38.533, User Equipment (UE) conformance specification; Radio Resource Management (RRM)

- TR 38.903, Derivation of test tolerances and measurement uncertainty for User Equipment (UE) conformance test cases