3GPP TSG-RAN5 Meeting #108	R5-254056

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4029 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of ATG random access test case 19.2.3.2.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The ATG RRM test case 19.2.3.2.4 “NR SA FR1 2-step RA type non-contention based random access test for ATG.” needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NR ATG RRM test case 19.2.3.2.4 has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The work plan will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 19.2.3.2.4(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

19.2.3.2.4	NR SA FR1 2-step RA type non-contention based random access test for ATG

Editor's Note: This test case is incomplete in following aspects:

- TT analysis is missing.

- AT commands for UE positioning and UE speed setting are FFS.

- UE specific positioning and the specific gNB reference location emulated by test system are FFS.

- Test procedures for providing network assistance on ATG cells reference locations are FFS.

19.2.3.2.4.1	Test purpose

The purpose of this test is to verify that the behavior of the random access procedure is according to the requirements and that the MsgA PRACH, MsgA PUSCH power settings and timing are within specified limits. This test will verify the requirements in TS 38.133 [6] clause 6.2D.2.3 and clause 7.1D.2 in an AWGN with constant residual doppler model.

19.2.3.2.4.2	Test applicability

This test applies to all types of NR ATG UE supporting 2-step RACH from Release 18 onwards.

19.2.3.2.4.3	Minimum conformance requirement

The minimum conformance requirements are specified in clause 19.2.3.2.0.2.

The normative reference for this requirement is TS 38.133 [6] clauses A.19.2.3.2.4.

19.2.3.2.4.4	Test description

19.2.3.2.4.4.1	Initial conditions

Same as the initial conditions given in clause 6.3.2.2.4.4.1 with following exceptions:

-	Table 6.3.2.2.4.4.1-1 is replaced by Table 19.2.3.2.4.4.1-1.

-	Table 6.3.2.2.4.4.1-2 is replaced by Table 19.2.3.2.4.4.1-2.

-	During the test, UE positioning and UE speed are set by AT command. UE speed is 0km/h, UE specific positioning is emulated by test system.

-	During the test, the specific gNB reference location is emulated by test system.

Table 19.2.3.2.4.4.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 19.2.3.2.4-1 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations depending on UE capability |  |

Table 19.2.3.2.4.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.17-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 19.2.3.2.4.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.7.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

19.2.3.2.4.4.2	Test procedure

Same as the test procedure given in clause 6.3.2.2.4.4.2 with following exceptions:

-	Table 6.3.2.2.1.5-1 is replaced by Table 19.2.3.2.4.5-1.

19.2.3.2.4.4.3	Message contents

Same as the message contents given in 6.3.2.2.4.4.3.

19.2.3.2.4.5	Test requirement

Same as the test requirements given in clause 6.3.2.2.4.5 with following exceptions:

-	Table 6.3.2.2.4.5-1 is replaced by Table 19.2.3.2.4.5-1.

Table 19.2.3.2.4.5-1: General test parameters (Same as Table 6.3.2.2.4.5-1 with following exceptions)

| Parameter | Test configuration | Unit | Test 1 | Comments |
| --- | --- | --- | --- | --- |
| Propagation Condition | Config 1 | - | AWGN+220 Hz |  |
|  | Config 2 | - | AWGN+500 Hz |  |