3GPP TSG-RAN5 Meeting #108	R5-254173

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5097 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new LTM Test Case 7.1.1.3.29 for LTM cell switch 4 step contention free random access procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Sweden |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of new LTM Test Case for LTM cell switch 4 step contention free random access procedure as per NR_Mob_enh2-UEConTest |  |  |  |  |  |  |  |  |
| Summary of change: |  | New test case is added as per NR_Mob_enh2-UEConTest |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NR_Mob_enh2-UEConTest will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.29 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0613 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of modified section]

#### 7.1.1.3.29	Correct selection of RACH parameters / LTM cell switch / 4-step RACH / contention free random access procedure

7.1.1.3.29.1	Test Purpose (TP)

(1)

with { UE in RRC_Connected state with candidate configuration for LTM }

ensure that {

when { UE receives LTM Cell Switch Command MAC CE with contention free random access resources }

then { UE selects the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected SS block and initiates the contention free Random Access Procedure }

}

(2)

with { UE in RRC_Connected state with candidate configuration for LTM }

ensure that {

when { UE receives LTM Cell Switch Command MAC CE with contention free random access resources }

then { UE triggers 4-step RACH procedure on LTM Cell }

}

7.1.1.3.29.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.321 clause 5.1.2 and 5.1.1 and TS 38.331 clause 5.3.7.3. Unless otherwise stated these are Rel-18 requirements.

[TS 38.321, clause 5.1.2]

If the selected RA_TYPE is set to 4-stepRA, the MAC entity shall:

1>	else if contention-free Random Access Resources have been explicitly provided by an LTM Cell Switch Command MAC CE and the SS-RSRP of the SSB signalled by the LTM Cell Switch Command MAC CE is above rsrp-ThresholdSSB:

2>	set the PREAMBLE_INDEX to the Random Access Preamble index signalled by the LTM Cell Switch Command MAC CE;

2>	select the SSB signalled by the LTM Cell Switch Command MAC CE.

[TS 38.321, clause 5.1.1]

The Random Access procedure described in this clause is initiated by a PDCCH order, by the MAC entity itself, or by RRC for the events in accordance with TS 38.300 [2]. There is only one Random Access procedure ongoing at any point in time in a MAC entity. The Random Access procedure on an SCell or an LTM candidate cell shall only be initiated by a PDCCH order with ra-PreambleIndex different from 0b000000.

NOTE 1:	If a new Random Access procedure is triggered while another is already ongoing in the MAC entity, it is up to UE implementation whether to continue with the ongoing procedure or start with the new procedure (e.g. for SI request).

NOTE 2:	If there was an ongoing Random Access procedure that is triggered by a PDCCH order while the UE receives another PDCCH order indicating the same Random Access Preamble, PRACH mask index and uplink carrier, the Random Access procedure is considered as the same Random Access procedure as the ongoing one and not initialized again.

When a Random Access procedure is initiated, UE selects a set of Random Access resources as specified in clause 5.1.1b and initialises the following parameters for the Random Access procedure according to the values configured by RRC for the selected set of Random Access resources:

-	prach-ConfigurationIndex: the available set of PRACH occasions for the transmission of the Random Access Preamble for Msg1. These are also applicable to the MSGA PRACH if the PRACH occasions are shared between 2-step and 4-step RA types;

-	prach-ConfigurationPeriodScaling-IAB: the scaling factor defined in TS 38.211 [8] and applicable to IAB-MTs, extending the periodicity of the PRACH occasions baseline configuration indicated by prach-ConfigurationIndex;

-	prach-ConfigurationFrameOffset-IAB: the frame offset defined in TS 38.211 [8] and applicable to IAB-MTs, altering the ROs frame defined in the baseline configuration indicated by prach-ConfigurationIndex;

-	prach-ConfigurationSOffset-IAB: the subframe/slot offset defined in TS 38.211 [8] and applicable to IAB-MTs, altering the ROs subframe or slot defined in the baseline configuration indicated by prach-ConfigurationIndex;

-	msgA-PRACH-ConfigurationIndex: the available set of PRACH occasions for the transmission of the Random Access Preamble for MSGA in 2-step RA type;

-	preambleReceivedTargetPower: initial Random Access Preamble power for 4-step RA type;

-	msgA-PreambleReceivedTargetPower: initial Random Access Preamble power for 2-step RA type;

-	rsrp-ThresholdSSB: an RSRP threshold for the selection of the SSB for 4-step RA type. If the Random Access procedure is initiated for beam failure recovery, rsrp-ThresholdSSB used for the selection of the SSB within candidateBeamRSList refers to rsrp-ThresholdSSB in BeamFailureRecoveryConfig IE;

-	rsrp-ThresholdCSI-RS: an RSRP threshold for the selection of CSI-RS for 4-step RA type. If the Random Access procedure is initiated for beam failure recovery, rsrp-ThresholdCSI-RS is equal to rsrp-ThresholdSSB in BeamFailureRecoveryConfig IE;

-	msgA-RSRP-ThresholdSSB: an RSRP threshold for the selection of the SSB for 2-step RA type;

-	rsrp-ThresholdSSB-SUL: an RSRP threshold for the selection between the NUL carrier and the SUL carrier;

-	msgA-RSRP-Threshold: an RSRP threshold for selection between 2-step RA type and 4-step RA type when both 2-step and 4-step RA type Random Access Resources are configured in the UL BWP;

-	rsrp-ThresholdMsg1-RepetitionNum2: an RSRP threshold for Msg1 repetition with repetition number 2 (see clause 5.1.1b);

-	rsrp-ThresholdMsg1-RepetitionNum4: an RSRP threshold for Msg1 repetition with repetition number 4 (see clause 5.1.1b);

-	rsrp-ThresholdMsg1-RepetitionNum8: an RSRP threshold for Msg1 repetition with repetition number 8 (see clause 5.1.1b);

-	rsrp-ThresholdMsg3: an RSRP threshold for Msg3 repetition (see clause 5.1.1b);

-	FeatureCombination: feature or a combination of features associated with a set of Random Access resources;

-	featurePriorities: priorities for features, such as (e)RedCap, Slicing, etc. (see clause 5.1.1d);

-	msgA-TransMax: The maximum number of MSGA transmissions when both 4-step and 2-step RA type Random Access Resources are configured;

-	candidateBeamRSList: a list of reference signals (CSI-RS and/or SSB) identifying the candidate beams for recovery and the associated Random Access parameters;

-	recoverySearchSpaceId: the search space identity for monitoring the response of the beam failure recovery request;

-	powerRampingStep: the power-ramping factor;

-	msgA-PreamblePowerRampingStep: the power ramping factor for MSGA preamble;

-	powerRampingStepHighPriority: the power-ramping factor in case of prioritized Random Access procedure;

-	scalingFactorBI: a scaling factor for prioritized Random Access procedure;

-	ra-PreambleIndex: Random Access Preamble;

-	ra-ssb-OccasionMaskIndex: defines PRACH occasion(s) associated with an SSB in which the MAC entity may transmit a Random Access Preamble (see clause 7.4);

-	msgA-SSB-SharedRO-MaskIndex: Indicates the subset of 4-step RA type PRACH occasions shared with 2-step RA type PRACH occasions for each SSB. If 2-step RA type PRACH occasions are shared with 4-step RA type PRACH occasions and msgA-SSB-SharedRO-MaskIndex is not configured, then all 4-step RA type PRACH occasions are available for 2-step RA type (see clause 7.4);

-	ssb-SharedRO-MaskIndex: defines PRACH occasions, on which preambles are allocated for a feature or a combination of features, associated with an SSB in which the MAC entity may transmit a Random Access Preamble (see clause 7.4);

-	ra-OccasionList: defines PRACH occasion(s) associated with a CSI-RS in which the MAC entity may transmit a Random Access Preamble;

-	ra-PreambleStartIndex: the starting index of Random Access Preamble(s) for on-demand SI request;

-	startPreambleForThisPartition: the first preamble associated with the set of Random Access Resources applicable to the Random Access procedure;

-	preambleTransMax: the maximum number of Random Access Preamble transmission;

-	preambleTransMax-Msg1-Repetition: the maximum number of Random Access Preamble transmissions with a given Msg1 repetition number before switching to Msg1 repetition with the next available higher Msg1 repetition number;

-	ssb-perRACH-OccasionAndCB-PreamblesPerSSB: defines the number of SSBs mapped to each PRACH occasion for 4-step RA type and the number of contention-based Random Access Preambles mapped to each SSB;

-	msgA-CB-PreamblesPerSSB-PerSharedRO: defines the number of contention-based Random Access Preambles for 2-step RA type mapped to each SSB when the PRACH occasions are shared between 2-step and 4-step RA types;

-	msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB: defines the number of SSBs mapped to each PRACH occasion for 2-step RA type and the number of contention-based Random Access Preambles mapped to each SSB;

-	numberOfPreamblesPerSSB-ForThisPartition: defines the number of consecutive preambles for a feature or a combination of features mapped to each SSB;

-	msgA-PUSCH-ResourceGroupA: defines MSGA PUSCH resources that the UE shall use when performing MSGA transmission using Random Access Preambles group A;

-	msgA-PUSCH-ResourceGroupB: defines MSGA PUSCH resources that the UE shall use when performing MSGA transmission using Random Access Preambles group B;

-	msgA-PUSCH-Resource-Index: identifies the index of the PUSCH resource used for MSGA in case of contention-free Random Access with 2-step RA type;

-	if groupBconfigured is configured, then Random Access Preambles group B is configured for 4-step RA type.

-	Amongst the contention-based Random Access Preambles associated with an SSB (as defined in TS 38.213 [6]), the first numberOfRA-PreamblesGroupA included in groupBconfigured Random Access Preambles belong to Random Access Preambles group A. The remaining Random Access Preambles associated with the SSB belong to Random Access Preambles group B (if configured).

-	if groupB-ConfiguredTwoStepRA is configured, then Random Access Preambles group B is configured for 2-step RA type.

-	Amongst the contention-based Random Access Preambles for 2-step RA type associated with an SSB (as defined in TS 38.213 [6]), the first numberOfRA-PreamblesGroupA included in GroupB-ConfiguredTwoStepRA Random Access Preambles belong to Random Access Preambles group A. The remaining Random Access Preambles associated with the SSB belong to Random Access Preambles group B (if configured).

NOTE 3:	If Random Access Preambles group B is supported by the cell Random Access Preambles group B is included for each SSB.

-	if Random Access Preambles group B is configured for 4-step RA type:

-	ra-Msg3SizeGroupA: the threshold to determine the groups of Random Access Preambles for 4-step RA type;

-	msg3-DeltaPreamble: ∆PREAMBLE_Msg3 in TS 38.213 [6];

-	messagePowerOffsetGroupB: the power offset for preamble selection included in groupBconfigured;

-	numberOfRA-PreamblesGroupA: defines the number of Random Access Preambles in Random Access Preamble group A for each SSB included in groupBconfigured.

-	if Random Access Preambles group B is configured for 2-step RA type:

-	msgA-DeltaPreamble: ∆MsgA_PUSCH in TS 38.213 [6];

-	messagePowerOffsetGroupB: the power offset for preamble selection included in GroupB-ConfiguredTwoStepRA;

-	numberOfRA-PreamblesGroupA: defines the number of Random Access Preambles in Random Access Preamble group A for each SSB included in GroupB-ConfiguredTwoStepRA;

-	ra-MsgA-SizeGroupA: the threshold to determine the groups of Random Access Preambles for 2-step RA type.

-	the set of Random Access Preambles and/or PRACH occasions for SI request, if any;

-	the set of Random Access Preambles and/or PRACH occasions for beam failure recovery request, if any;

-	the set of Random Access Preambles and/or PRACH occasions for reconfiguration with sync, if any;

-	ra-ResponseWindow: the time window to monitor RA response(s) (SpCell only);

-	ra-ContentionResolutionTimer: the Contention Resolution Timer (SpCell only);

-	msgB-ResponseWindow: the time window to monitor RA response(s) for 2-step RA type (SpCell only).

In addition, the following information for related Serving Cell is assumed to be available for UEs:

-	if Random Access Preambles group B is configured:

-	if the Serving Cell for the Random Access procedure is configured with supplementary uplink as specified in TS 38.331 [5], and SUL carrier is selected for performing Random Access Procedure:

-	PCMAX,f,c of the SUL carrier as specified in TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16].

-	else:

-	PCMAX,f,c of the NUL carrier as specified in TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16].

The following UE variables are used for the Random Access procedure:

-	PREAMBLE_INDEX;

-	PREAMBLE_TRANSMISSION_COUNTER;

-	PREAMBLE_POWER_RAMPING_COUNTER;

-	PREAMBLE_POWER_RAMPING_STEP;

-	PREAMBLE_RECEIVED_TARGET_POWER;

-	PREAMBLE_BACKOFF;

-	PCMAX;

-	SCALING_FACTOR_BI;

-	TEMPORARY_C-RNTI;

-	RA_TYPE;

-	POWER_OFFSET_2STEP_RA;

-	MSGA_PREAMBLE_POWER_RAMPING_STEP.

When the Random Access procedure is initiated on a Serving Cell or for an LTM candidate cell, the MAC entity shall:

1>	if the Random Access procedure was initiated for reconfiguration with sync not initiated for recovery using an LTM candidate configuration as specified in TS 38.331 [5] clause 5.3.7.3 and if the contention-free Random Access Resources for 4-step RA type have been explicitly provided in rach-ConfigDedicated for the BWP selected for Random Access procedure; or

1>	if the contention-free Random Access Resources have been explicitly provided in the LTM Cell Switch Command MAC CE:

2>	set the RA_TYPE to 4-stepRA.

1>	if the Random Access procedure was initiated for reconfiguration with sync not initiated for recovery using an LTM candidate configuration as specified in TS 38.331 [5] clause 5.3.7.3 and if the contention-free Random Access Resources for 2-step RA type have been explicitly provided in rach-ConfigDedicated for the BWP selected for Random Access procedure:

2>	set the RA_TYPE to 2-stepRA.

1>	else:

2>	set the RA_TYPE to 4-stepRA.

1>	if RA_TYPE is set to 2-stepRA:

2>	perform the Random Access Resource selection procedure for 2-step RA type (see clause 5.1.2a).

1>	else:

2>	perform the Random Access Resource selection procedure (see clause 5.1.2).

[TS 38.331, clause 5.3.7.3]

Upon selecting a suitable NR cell, the UE shall:

1>	if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG, except for an LTM cell switch procedure following cell selection performed while timer T311 was running, as specified in 5.3.7.3, or mobility from NR failure, and

1>	if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers as specified in clause 5.3.5.18.6; and

1>	if attemptLTM-Switch is configured; and

1>	if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG:

2>	perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6;

NOTE 2:	In case both attemptCondReconfig and attemptLTM-Switch are configured, it is left to the UE implementation which procedure to execute.

7.1.1.3.29.3	Test description

7.1.1.3.29.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0 except the following:

-	2 NR cells (NR Cell 1 and NR Cell 2) are configured.

-	Test loop function(Off)

7.1.1.3.29.3.2	Test procedure sequence

Table 7.1.1.3.29.3.2-1 and 7.1.1.3.29.3.2-2 illustrates the downlink power levels to be applied for NR Cell 1 and NR Cell 2 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while the configuration marked "T1" is applied at the point indicated in the Main behaviour description in Table 7.1.1.3.29.3.2-3.

Table 7.1.1.3.29.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -91 | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -85 | -79 | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell |
|  | Noc | dBm/ SCS | -85 | - |  |

Table 7.1.1.3.29.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/ SCS | -91 | -100 | Power levels are such that entry condition for event A3 is not satisfied for the neighbour NR cell |
| T1 | SS/PBCH SSS EPRE | dBm/ SCS | -91 | -82 | Power levels are such that entry condition for event A3 is satisfied for intra-frequency neighbour NR cell |
|  | Noc | dBm/ SCS | -91 | - |  |

Table 7.1.1.3.29.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message including MeasConfig to setup NR measurement and reporting for inter-frequency event A3 on NR Cell 1. | <-- | RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |
| 3 | SS re-adjusts the cell-specific reference signal level according to row "T1" in table 7.1.1.3.29.3.2-1/2 | - | - | - | - |
| 4 | The UE transmits a MeasurementReport message to report event A3 with the measured RSRP value for NR Cell 2 in NR Cell 1. | --> | MeasurementReport | - | - |
| 5 | The SS transmits an RRCReconfiguration message with LTM-Candidate-r18 for NR Cell 2 on NR Cell 1. | <-- | RRCReconfiguration | - | - |
| 6 | The UE transmits RRCReconfigurationComplete message. | --> | RRCReconfigurationComplete | - | - |
| 7 | The SS transmits the candidate cell TCI activation MAC CE with Candidate Cell ID set to 0 for NR Cell 2 on NR Cell 1. | <-- | MAC PDU (Candidate Cell TCI Activation/Deactivation MAC CE) | - | - |
| 8 | The SS transmits LTM cell switch command MAC CE with CFRA resources | <-- | MAC PDU (LTM Cell Switch Command MAC CE) | - | - |
| 9 | Check: Does the UE transmit a preamble on PRACH using a preamble with PREAMBLE_INDEX to a ra-PreambleIndex signalled by the LTM Cell Switch Command MAC CE | --> | PRACH Preamble | 1 | P |
| 10 | The SS transmits Random Access Response on NR cell 2, with RAPID corresponding to ra-PreambleIndex in step 9 | <-- | Random Access Response | - | - |
| 11 | The SS transmits an UL Grant, allowing the UE to transmit the RRCReconfigurationComplete message on NR Cell 2. | <-- | (UL Grant (C-RNTI)) | - | - |
| 12 | Check: Does the UE transmit RRCReconfigurationComplete message on NR Cell 2? | --> | RRCReconfigurationComplete | 2 | P |

7.1.1.3.29.3.3	Specific message contents

Table 7.1.1.3.29.3.3-1: RRCReconfiguration (step 1, Table 7.1.1.3.29.3.2-3)

Derivation Path: TS 38.508-1 [4] Table 4.6.1-13 with condition NR_MEAS

Table 7.1.1.3.29.3.3-2: MeasConfig (step 1, Table 7.1.1.3.29.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 1 entry |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR | Table 7.1.1.3.29.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-EventA3 | Table 7.1.1.3.29.3.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 1 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.29.3.3-3: MeasObjectNR (Table 7.1.1.3.29.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 7.1.1.3.29.3.3-4: ReportConfigNR-EventA3 (Table 7.1.1.3.29.3.3-2)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-142 with condition EVENT_A3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR ::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| eventTriggered SEQUENCE { |  |  |  |
| eventId CHOICE { |  |  |  |
| eventA3 SEQUENCE { |  |  |  |
| a3-Offset CHOICE { |  |  |  |
| rsrp | 2 | 1 dB(2*0.5 dB) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportAmount | infinity |  |  |
| maxNrofRS-IndexesToReport | 1 |  |  |
| includeBeamMeasurements | true |  |  |
| reportQuantityCell SEQUENCE { |  |  |  |
| rsrp | true |  |  |
| rsrq | false |  |  |
| sinr | false |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.29.3.3-5: MeasurementReport (step 4, Table 7.1.1.3.29.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-5A |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasurementReport ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| measurementReport SEQUENCE { |  |  |  |
| measResults SEQUENCE { |  |  |  |
| measId | 1 |  |  |
| measResultServingMOList SEQUENCE (SIZE (1..maxNrofServingCells)) OF MeasResultServMO { | 1 entry |  |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | ServCellIndex of NR Cell 1 |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | PCI of NR Cell 1 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | (0..127) |  |  |
| sinr | Not present |  |  |
|  | Not checked |  | pc_ss_SINR_Meas |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR SEQUENCE (SIZE (1.. maxCellReport)) OF MeasResultNR { | 1 entry |  |  |
| MeasResultNR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | Physical layer cell identity of NR Cell 2 |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | (0..127) |  |  |
| rsrq | Not present |  |  |
| sinr | Not present |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell | Not present |  |  |
| } |  |  |  |
| rsIndexResults | Not present |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.29.1.3.3-6: RRCReconfiguration (step 5, Table 7.1.1.3.29.1.3.2-3)

| Derivation path: TS 38.508-1 [4], Table 4.6.1-13 with condition LTM |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ltm-Config-r18 CHOICE { |  |  |  |
| setup | LTM-Config-r18 | Table 7.1.1.3.29.3.3-7 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.29.3.3-7: LTM-Config-r18(Table 7.1.1.3.29.3.3-6)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-67D |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-Config-r18 ::= SEQUENCE { |  |  |  |
| ltm-CandidateToAddModList-r18 SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18 { | 1 entry |  |  |
| LTM-Candidate-r18[1] | LTM-Candidate-r18 | Table 7.1.1.3.29.3.3-8 |  |
| } |  |  |  |
| ltm-CSI-ResourceConfig-r18 | Not present |  |  |
| } |  |  |  |

Table 7.1.1.3.29.3.3-8: LTM-Candidate-r18(Table 7.1.1.3.29.3.3-7)

| Derivation path: TS 38.508-1 [4], Table 4.6.3-67C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LTM-Candidate-r18 ::= SEQUENCE { |  |  |  |
| ltm-CandidateId-r18 | 1 |  |  |
| ltm-CandidatePCI-r18 | Physical layer cell identity of NR Cell 2 |  |  |
| ltm-CandidateConfig-r18 OCTET STRING (CONTAINING RRCReconfiguration) { |  |  |  |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig specified in TS 38.508-1 [4] Table 4.6.3-19 with condition PCell_change |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| ltm-TCI-Info-r18 | LTM-TCI-Info specified in TS 38.508-1 [4] Table 4.6.3-67I with condition Joint |  |  |
| } |  |  |  |

[End of modified section]