3GPP TSG-RAN5 Meeting #108	R5-254506

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5484 | rev |  | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to SSAC test case 13.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | If Attach is accepted with combined EPS/IMSI attach and an MTSI MO speech call attempt fails on EPS, UE may subsequently try to place the call over CS RATs causing test case failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added Table 13.5.1.3.3-0 for Specific message contents in Attach Accept message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may unfairly fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

### 13.5.1	MTSI MO speech call / SSAC / 0% access probability for MTSI MO speech call

13.5.1.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state having received a SysteminformationBlockType2 message including  a ssac-BarringForMMTEL-Voice-r9 set to 0% accessibility }

ensure that {
  when { the user initiates a MTSI MO voice call }

then { the UE does not establish RRC connection }

}

(2)

with { UE in E-UTRA RRC_IDLE state having received a SysteminformationBlockType2 message including a ssac-BarringForMMTEL-Voice-r9 set to 0% accessibility and including back-off timer Ty and having failed to initiate a voice call }

ensure that {
  when { the user initiates a MTSI MO voice call during back-off timer Ty is running }

then { the UE does not establish RRC connection }

}

13.5.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.173 clause J.2.1.2 and TS 36.331 clause 5.3.3.10. Unless otherwise stated these are Rel-9 requirements.

[TS 24.173, clause J.2.1.1]:

The following information is provided by lower layer:

-	BarringFactorForMMTEL-Voice: barring rate for MMTEL voice;

-	BarringTimeForMMTEL-Voice: barring timer for MMTEL voice;

…

Upon request from a user to establish a multimedia telephony communication session as described in subclause 5.2, the UE shall:

1)	if the multimedia telephony communication session to be established is an emergency session, then skip the rest of steps below and continue with session establishment as described in subclause 5.2;

2)	retrieve SSAC related information mentioned above from lower layers;

3)	if video is offered in the multimedia telephony communication session:

…

4)	if audio is offered in the multimedia telephony communication session:

A)	if back-off timer Ty is running, reject the multimedia telephony communication session establishment and skip the rest of steps below; or

B)	else, then;

I)	draw a new random number "rand3" that is uniformly distributed in the range 0 ≤ rand3 < 1; and

II)	if the random number "rand3" is lower than BarringFactorForMMTEL-Voice, then skip the rest of steps below and continue with session establishment as described in subclause 5.2;

III)	else, then;

i)	draw a new random number "rand4" that is uniformly distributed in the range 0 ≤ rand4 < 1; and

ii)	start timer Ty with the timer value calculated using the formula:

Ty = (0,7 + 0,6*rand4) * BarringTimeForMMTEL-Voice; and

iii)	reject the multimedia telephony communication session establishment;

[TS 36.331 clause 5.3.3.10]:

Upon request from the upper layers, the UE shall:

1>	set the local variables BarringFactorForMMTEL-Voice and BarringTimeForMMTEL-Voice as follows:

2>	if ssac-BarringForMMTEL-Voice is present:

3>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11], and

NOTE:	ACs 12, 13, 14 are only valid for use in the home country and ACs 11, 15 are only valid for use in the HPLMN/ EHPLMN.

3>	if, for at least one of these Access Classes, the corresponding bit in the ac-BarringForSpecialAC contained in ssac-BarringForMMTEL-Voice is set to zero:

4>	set BarringFactorForMMTEL-Voice to one and BarringTimeForMMTEL-Voice to zero;

3>	else:

4>	set BarringFactorForMMTEL-Voice and BarringTimeForMMTEL-Voice to the value of ac-BarringFactor and ac-BarringTime included in ssac-BarringForMMTEL-Voice, respectively;

2>	else set BarringFactorForMMTEL-Voice to one and BarringTimeForMMTEL-Voice to zero;

1>	set the local variables BarringFactorForMMTEL-Video and BarringTimeForMMTEL-Video as follows:

2>	if ssac-BarringForMMTEL-Video is present:

3>	if the UE has one or more Access Classes, as stored on the USIM, with a value in the range 11..15, which is valid for the UE to use according to TS 22.011 [10] and TS 23.122 [11], and

3>	if, for at least one of these Access Classes, the corresponding bit in the ac-BarringForSpecialAC contained in ssac-BarringForMMTEL-Video is set to zero:

4>	set BarringFactorForMMTEL-Video to one and BarringTimeForMMTEL-Video to zero;

3>	else:

4>	set BarringFactorForMMTEL-Video and BarringTimeForMMTEL-Video to the value of ac-BarringFactor and ac-BarringTime included in ssac-BarringForMMTEL-Video, respectively;

2>	else set BarringFactorForMMTEL-Video to one and BarringTimeForMMTEL-Video to zero;

1>	forward the variables BarringFactorForMMTEL-Voice, BarringTimeForMMTEL-Voice, BarringFactorForMMTEL-Video and BarringTimeForMMTEL-Video to the upper layers;

13.5.1.3	Test description

13.5.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1.

UE:

None.

Preamble:

-	The UE is in state Registered, Idle mode (state 2) according to [18].

13.5.1.3.2	Test procedure sequence

Table 13.5.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE attempt an MTSI MO Speech call | - | - | - | - |
| 2 | Check: Does the UE transmit a RRCConnectionRequest message within 10 s?. | --> | RRCConnectionRequest | 1 | F |
| 3 | SS changes SIB2 according to TS 36.508, table 4.4.3.3-1 and transmits a Paging message including systemInfoModification. The systemInfoValueTag in the SystemInformationBlockType1 is increased. | <-- | Paging | - | - |
| 4 | Wait for 15 s (Note 1) to allow the new system information to take effect. | - | - | - | - |
| 5 | Make the UE attempt an MTSI MO Speech call | - | - | - | - |
| 6 | Check: Does the UE transmit an RRCConnectionRequest message with in 10s? | --> | RRCConnectionRequest | 2 | F |
| 7 | Wait for 49 s (Note 2) to make the timer Ty expire. |  |  | - | - |
| 8-21 | Steps 1 to 14 of the generic test procedure for MTSI MO Speech call establishment in EUTRA: Normal Service (TS 36.508 4.5A.6.3-1). | - | - | - | - |
| Note 1: The wait time of 15 s in step 3 is to allow for the network to page the system information change during the next modification period, and update the system information at the subsequent modification period. UE should acquire the updated system information within 100ms of the start of modification period. Note 2: The UE starts timer Ty in step2. Maximum time of timer Ty is 83.2 sec ((0.7 + 0.6 * 1) * s64). At the end of step 7, 35 sec elapses from step 2. Therefore 49 sec (84s - 35s) is enough to wait timer Ty expiry. |  |  |  |  |  |

13.5.1.3.3	Specific message contents

Table 13.5.1.3.3-0: Message ATTACH ACCEPT (preamble)

| Derivation Path: TS 36.508 Table 4.7.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EPS attach result | 001 | "EPS only" |  |
| Location area identification | Not present | SS doesn’t provide LAI |  |
| MS identity | Not Present | SS doesn’t provide TMSI |  |
| EMM cause | 00010010 | #18 "CS domain not available" | combined_EPS_IMSI |

| Condition | Explanation |
| --- | --- |
| combined_EPS_IMSI | This condition applies if the UE is configured to initiate combined EPS/IMSI attach or if explicitly specified. |

Table 13.5.1.3.3-1: SystemInformationBlockType2 for Cell 1 (preamble and step 1, Table 13.5.1.3.2-1)

| Derivation Path: 36.508, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2 ::= SEQUENCE { |  |  |  |
| ssac-BarringForMMTEL-Voice-r9 SEQUENCE { |  |  |  |
| ac-BarringFactor | p00 |  |  |
| ac-BarringTime | s64 |  |  |
| ac-BarringForSpecialAC | '11111'B |  |  |
| } |  |  |  |
| } |  |  |  |

Table 13.5.1.3.3-2: Paging (step3, Table 13.5.1.3.2-1)

| Derivation Path: 36.508 Table 4.6.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Paging ::= SEQUENCE { |  |  |  |
| pagingRecordList | Not present |  |  |
| systemInfoModification | TRUE |  |  |
| } |  |  |  |

<End of modified section>