3GPP TSG-RAN5 Meeting #108	R5-254542

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5121 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new UAV Test Case 8.1.3.5.11 Test Case for event A4H1 reporting for Inter-Band Neighbour |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UAV_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The measurement report trigger event A4H1 for Aerial UE has been proposed in TS 38.331, but it is absent of measurement configuration control and reporting for the event A4H1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added measurement configuration control and reporting of Event A4H1 for Aerial UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The event A4H1 for Aerial UE can not be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.5.11 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-2 CR 0625 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

8.1.3.5.11	NR UAV / Measurement configuration control and reporting / Event A4H1/ Measurement of Neighbour NR cell / Inter-band measurements

8.1.3.5.11.1	Test Purpose (TP)

Same as test case 8.1.3.5.9 but applied to inter-band case.

8.1.3.5.11.2	Conformance requirements

Same as test case 8.1.3.5.9 with the following differences:

[TS 38.331, clause 5.5.2.9]

The UE shall:

…

1>	if gapUE is set to setup:

2>	if a per UE measurement gap configuration is already setup, release the per UE measurement gap configuration;

2>	setup the per UE measurement gap configuration indicated by the measGapConfig in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition:

SFN mod T = FLOOR(gapOffset/10);

subframe = gapOffset mod 10;

with T = MGRP/10 as defined in TS 38.133 [14];

2>	if mgta is configured, apply the specified timing advance to the gap occurrences calculated above (i.e. the UE starts the measurement mgta ms before the gap subframe occurrences);

8.1.3.5.11.3	Test description

8.1.3.5.11.3.1	Pre-test conditions

Same as test case 8.1.3.5.9 with the following differences:

-	Cells configuration: NR Cell 10 replaces NR Cell 2.

-	System information combination: NR-4 repleaces NR-2.

8.1.3.5.11.3.2	Test procedure sequence

Same as test case 8.1.3.5.9 with the following differences:

-	Cells configuration: NR Cell 10 replaces NR Cell 2.

8.1.3.5.11.3.3	Specific message contents

Same as test case 8.1.3.5.9 with the following difference:

-	Cells configuration: NR Cell 10 replaces NR Cell 2.

Table 8.1.3.5.11.3.3-1: MeasConfig (Table 8.1.3.5.9.3.3-1)

| Derivation Path: TS 38.508-1 [4] Table 4.6.3-69 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasConfig ::= SEQUENCE { |  |  |  |
| measObjectToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasObjectToAddMod { | 2 entries |  |  |
| MeasObjectToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measObjectId | 1 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-f1 | Table 8.1.3.5.11.3.3-2 |  |
| } |  |  |  |
| } |  |  |  |
| MeasObjectToAddMod[2] SEQUENCE { |  | entry 2 |  |
| measObjectId | 2 |  |  |
| measObject CHOICE { |  |  |  |
| measObjectNR | MeasObjectNR-f2 | Table 8.1.3.5.11.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportConfigToAddModList SEQUENCE(SIZE (1..maxReportConfigId)) OF ReportConfigToAddMod { | 1 entry |  |  |
| ReportConfigToAddMod[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 1 |  |  |
| reportConfig CHOICE { |  |  |  |
| reportConfigNR | ReportConfigNR-EventA4H1 | Table 8.1.3.5.9.3.3-3 |  |
| } |  |  |  |
| measIdToAddModList SEQUENCE (SIZE (1..maxNrofMeasId)) OF MeasIdToAddMod { | 1 entry |  |  |
| MeasIdToAddMod[1] SEQUENCE { |  | entry 1 |  |
| measId | 1 |  |  |
| measObjectId | 2 |  |  |
| reportConfigId | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| measGapConfig | MeasGapConfig |  |  |
| } |  |  |  |

Table 8.1.3.5.11.3.3-2: MeasObjectNR-f1 (Table 8.1.3.5.11.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 1 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

Table 8.1.3.5.11.3.3-3: MeasObjectNR-f2 (Table 8.1.3.5.11.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-76 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectNR::= SEQUENCE { |  |  |  |
| ssbFrequency | ARFCN-ValueNR for SSB of NR Cell 10 |  |  |
| absThreshSS-BlocksConsolidation | Not present |  |  |
| nrofSS-BlocksToAverage | Not present |  |  |
| } |  |  |  |

<End of changes >