3GPP TSG-RAN5 Meeting #108	R5-254990

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5068 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to ATG TC 7.1.1.10.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, CMCC, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - 1.According to the ReferenceLocation updates of R5-254024 in 38.508-1, the location message contents need to be updated. 2.According to the system information combination updates of R5-254023 in 38.508-1, the pre-test conditions needs to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - 1.The ReferenceLocation in Table 7.1.1.10.5.3.3-2 has been updated. 2.The pre-test conditions has been updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The errors will exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.10.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.508-1 CR 3529 TS/TR 38.508-1 CR 3530 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-254026r1. |  |  |  |  |  |  |  |  |

7.1.1.10.5	ATG / TA report / ta-ReportATG / offsetThresholdTA

7.1.1.10.5.1	Test Purpose (TP)

(1)

with { UE in NR RRC_IDLE state on a cell which provides ATG access and ta-ReportATG-r18 is configured with “enabled” in SIB22 }

ensure that {

when { UE initiates the Random Access procedure }

then { UE performs the Timing Advance reporting procedure }

}

(2)

with { UE in NR RRC_IDLE state on a cell which provides ATG access and ta-ReportATG-r18 is not configured in SIB22 }

ensure that {

when { UE receives an RRCSetup message and offsetThresholdTA-r18 is configured }

then { UE performs the Timing Advance reporting procedure }

}

7.1.1.10.5.2	Conformance requirements

References: The conformance requirements covered in the present test case are specified in: TS 38.300 clause 16.19.2.2, TS 38.321 clauses 5.4.8 and 6.1.3.56, TS 38.331 clause 6.3.2 . Unless otherwise stated these are Rel-18 requirements.

[TS 38.300 clause 16.19.2.2]

For the serving cell, the network broadcasts coarse gNB location information. The UE shall have valid GNSS position as well as gNB location before connecting to an ATG cell. To achieve synchronisation, before and during connection to an ATG cell, the UE shall compute the RTT between UE and the gNB based on the GNSS position and the gNB location parameters, and autonomously pre-compensate the TTA for the RTT between the UE and the gNB.

In connected mode, the UE shall be able to continuously update the Timing Advance.

The UE may be configured to report Timing Advance during Random Access procedures or in connected mode. For an RRC_CONNECTED UE, event-triggered reporting of the Timing Advance is supported.

[TS 38.321 clause 5.4.8]

The Timing Advance reporting procedure is used in a non-terrestrial network or an air to ground network to provide the gNB with an estimate of the UE's Timing Advance value (i.e., TTA as defined in the UE's TA formula, see TS 38.211 [8] clause 4.3.1).

RRC controls Timing Advance reporting by configuring the following parameters:

-	offsetThresholdTA;

-	timingAdvanceSR.

A Timing Advance report (TAR) shall be triggered if any of the following events occur:

-	upon indication from upper layers to trigger a Timing Advance report;

-	upon configuration of offsetThresholdTA by upper layers, if the UE has not previously reported Timing Advance value to current Serving Cell;

-	if the variation between the current estimate of the Timing Advance value and the last reported Timing Advance value is equal to or larger than offsetThresholdTA, if configured.

The MAC entity shall:

1>	if the Timing Advance reporting procedure determines that at least one TAR has been triggered and not cancelled:

2>	if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the Timing Advance Report MAC CE plus its subheader as a result of logical channel prioritization:

3>	instruct the Multiplexing and Assembly procedure to generate the Timing Advance Report MAC CE as defined in clause 6.1.3.56.

2>	else

3>	if timingAdvanceSR is configured with value enabled:

4>	trigger a Scheduling Request.

NOTE:	UL-SCH resources are considered available if the MAC entity has been configured with, receives, or determines an uplink grant. If the MAC entity has determined at a given point in time that UL-SCH resources are available, this need not imply that UL-SCH resources are available for use at that point in time.

A MAC PDU shall contain at most one Timing Advance Report MAC CE, even when multiple events have triggered a Timing Advance report. The Timing Advance Report MAC CE shall be generated based on the latest available estimate of the UE's Timing Advance value prior to the MAC PDU assembly.

All triggered Timing Advance reports shall be cancelled when a MAC PDU is transmitted and this PDU includes a Timing Advance Report MAC CE.

[TS 38.321 clause 6.1.3.56]

The Timing Advance Report MAC CE is identified by MAC subheader with LCID as specified in Table 6.2.1-2. It has a fixed size and consists of two octets defined as follows (Figure 6.1.3.56-1):

-	R: Reserved bit, set to 0;

-	Timing Advance: Except for ATG in FR1, the Timing Advance field indicates the least integer number of slots, using subcarrier spacing of 15 kHz, greater than or equal to the Timing Advance value (see TS 38.211 [8], clause 4.3.1).

For ATG in FR1, the Timing Advance field indicates the least integer number of symbols greater than or equal to the Timing Advance value (see TS 38.211 [8], clause 4.3.1). The symbol duration is based on the subcarrier spacing of the active BWP the UE is currently configured with. In this release of the specification, only 15 kHz and 30 kHz SCS are applicable and only values 1 … 56 are used.

The length of the field is 14 bits.

Figure 6.1.3.56-1: Timing Advance Report MAC CE

[TS 38.331 clause 6.3.2]

offsetThresholdTA

Offset for TA reporting as specified in TS 38.321 [3]. Network only configures this parameter for MCG. For offsetThresholdTA-r17, value ms0dot5 corresponds to 0.5 ms, value ms1 corresponds to 1 ms, and so on.

Network only configures offsetThresholdTA-r18 for ATG, which is in unit of symbols based on the subcarrier spacing of the BWP indicated by firstActiveDownlinkBWP-Id if received in the same RRC message, otherwise, it is based on the subcarrier spacing of the active DL BWP of the PCell when offsetThresholdTA-r18 is received. Once configured, the value of TA offset threshold does not change autonomously by the UE with BWP switching.

...

ta-ReportATG

When this field is included in SIB22, it indicates reporting of timing advanced is enabled during Random Access due to RRC connection establishment or RRC connection resume, and during RRC connection reestablishment. When this field is included in ServingCellConfigCommon within dedicated signalling, it indicates TA reporting is enabled during Random Access due to reconfiguration with sync (see TS 38.321 [3], clause 5.4.8).

7.1.1.10.5.3	Test description

7.1.1.10.5.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 as defined in TS 38.508-1 [4] Table 4.4.2-2 is used.

-	System information combination NR-34 as defined in TS 38.508-1 [4] clause 4.4.3.1 is used for NR cell.

UE:

-	The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5D.

Preamble:

-	The UE is in Switched OFF (State 0N-B) as defined in TS 38.508-1 [4] Table 4.4A.2-0.

7.1.1.10.5.3.2	Test procedure sequence

Table 7.1.1.10.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Power on the UE. | - | - | - | - |
| 2 | The UE transmit a preamble on PRACH | --> | PRACH Preamble | - | - |
| 3 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 2. | <-- | Random Access Response | - | - |
| 4 | Check: Does the UE transmit an MAC PDU with Timing Advance Report MAC CE included? | --> | MAC PDU (RRCSetupRequest, Timing Advance Report MAC CE) | 1 | P |
| 5-17 | Steps 3 to 15 of the procedure described in TS 38.508-1 [4] subclause 4.5.2.2-2 are performed. | - | - | - | - |
| 18 | The SS transmits an RRCRelease message. | <-- | RRCRelease | - | - |
| 19 | Power Off the UE. | - | - | - | - |
| 20 | The SS broadcasts the SIB22 with ta-Report-r18 set to not present. | <-- | RRC: SIB22 | - | - |
| 21 | Power on the UE. | - | - | - | - |
| 22 | The UE transmits a preamble on PRACH. | --> | PRACH Preamble | - | - |
| 23 | The SS transmits Random Access Response with RAPID corresponding to the transmitted Preamble in step 22. | <-- | Random Access Response | - | - |
| 24 | The UE transmits an RRCSetupRequest message with no Timing Advance Report MAC CE included. | --> | RRCSetupRequest | - | - |
| 25 | The SS transmits an RRCSetup message with offsetThresholdTA-r18 configured. | <-- | RRCSetup | - | - |
| 26 | Check: Does the UE transmit an MAC PDU with Timing Advance Report MAC CE included? | --> | MAC PDU (RRCSetupComplete, Timing Advance Report MAC CE) | 2 | P |
| 27-39 | Steps 3 to 15 of the procedure described in TS 38.508-1 [4] subclause 4.5.2.2-2 are performed. | - | - | - | - |
| 40 | The SS transmits an RRCRelease message. | <-- | RRCRelease | - | - |

7.1.1.10.5.3.3	Specific message contents

Table 7.1.1.10.5.3.3-1: SIB22 (Preamble and all steps)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB22-r18 ::= SEQUENCE { |  |  |  |  |
| atg-Config-r18 |  | ATG-Config | Table 7.1.1.10.5.3.3-2 |  |
| } |  |  |  |  |

Table 7.1.1.10.5.3.3-2: ATG-Config (Table 7.1.1.10.5.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-5BA |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| ATG-Config-r18 ::= SEQUENCE { |  |  |  |  |
| atg-gNB-Location-r18 |  | ReferenceLocation | TS 38.508-1 [4], Table 4.6.3-138AA with condition RefLocation_1 |  |
| ta-ReportATG-r18 |  | enabled |  | preamble |
|  |  | Not present |  | Step 20 |
| } |  |  |  |  |

Table 7.1.1.10.5.3.3-3: RRCSetup (Step 25, Table 7.1.1.10.5.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCSetup ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcSetup SEQUENCE { |  |  |  |  |
| masterCellGroup |  | CellGroupConfig | Table 7.1.1.10.5.3.3-4 |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 7.1.1.10.5.3.3-4: CellGroupConfig (Table 7.1.1.10.5.3.3-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| mac-CellGroupConfig |  | MAC-CellGroupConfig | Table 7.1.1.10.5.3.3-5 |  |
| } |  |  |  |  |

Table 7.1.1.10.5.3.3-5: MAC-CellGroupConfig (Table 7.1.1.10.5.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-68 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MAC-CellGroupConfig ::= SEQUENCE { |  |  |  |  |
| tar-Config-r18 SEQUENCE { |  |  |  |  |
| offsetThresholdTA-r18 |  | 5 |  |  |
| timingAdvanceSR-r18 |  | enabled |  |  |
| } |  |  |  |  |
| } |  |  |  |  |