3GPP TSG-RAN5 Meeting #108	R5-255415

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 1016 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of missing aperiodic CQI triggering parameters for PDSCH abs ph layer tput test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM Europe Inc. - Italy |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh3-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current test specifications lack sufficient information for the FDD tests related to the CSI configurations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update FDD test specifications with additional clarifications for CSI request, reportTriggerSize and CSI-AperiodicTriggerStateList. Added message contents exception for the CSI IEs introduced in the test parameter table for both FDD and TDD tests |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FDD test specificaitons will remain unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.2, 5.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.101-4 CR 0806 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Dependent on RAN4 CR R4-2509993 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-254879r1 |  |  |  |  |  |  |  |  |

<< Start of Changes >>

### 5.6.2	2RX requirements

#### 5.6.2.1	FDD

##### 5.6.2.1.1	2Rx FDD FR1 PDSCH absolute physical layer throughput performance with link adaptation

5.6.2.1.1.0	Minimum conformance requirements

The purpose of the requirements is to verify the PDSCH absolute physical layer throughput performance with link adaptation under 2 receive antenna conditions.

The performance requirements are specified in Table 5.6.2.1.1.0-2, with additional test parameters in Table 5.6.2.1.1.0-1 and the downlink physical channel setup according to Annex C.2.1.

Table 5.6.2.1.1.0-1: Test parameters

| Parameter |  |  | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 |
| Subcarrier spacing |  |  | kHz | 15 |
| Duplex Mode |  |  |  | FDD |
| Propagation channel |  |  |  | TDLA30-5 |
| Antenna configuration |  |  |  | 2×2 |
| Correlation configuration |  |  |  | ULA Low |
| Beamforming Model |  |  |  | As defined in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5,(4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | (9) |
|  | CSI-RS periodicity and offset |  | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 3 (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | (13) |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |
|  | CSI-IM RE pattern |  |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4,9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 |
| ReportConfigType |  |  |  | Aperiodic |
| CQI-table |  |  |  | Table 2 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | not configured |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband |
| Sub-band Size |  |  | RB | 8 |
| csi-ReportingBand |  |  |  | 1111111 |
| CSI-Report periodicity and offset |  |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  |  | 5 |
| CSI request |  |  |  | 1 in slots i, where mod(i, 5) = 0, otherwise it is equal to 0 |
| reportTriggerSize |  |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A |
|  |  | CodebookSubsetRestriction |  | 111111 |
|  |  | RI Restriction |  | N/A |
| Physical channel for CSI report |  |  |  | PUSCH |
| CQI/RI/PMI delay |  |  | ms | 6 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | 2 |
| Measurement channel |  |  |  | As specified in Table A.4-2 Rank 1: TBS.2-1 Rank 2: TBS.2-2 |
| Note1: For retransmission including initial transmission, RV {0,2,3,1} with same MCS and rank as initial transmission; follow the latest UE reported PMI whose rank is same as the initial transmission |  |  |  |  |

Table 5.6.2.1.1.0-2: Minimum performance requirement

| Test num. | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- |
| Test 1 | 35% | 19.3 |
| Note 1: The maximum throughout is defined as with TBS corresponding to CQI index 15 with rank 2. |  |  |

The normative reference for this requirement is TS 38.101-4 [5], clause 5.6.2.1.1.

###### 5.6.2.1.1_1	2Rx FDD FR1 PDSCH performance with link adaptation - 2x2 MIMO for both SA and NSA

5.6.2.1.1_1.1	Test purpose

Verify the PDSCH absolute physical layer throughput performance with link adaptation under 2 receive antenna conditions.

5.6.2.1.1_1.2	Test applicability

This test applies to all types of release 17 NR UEs and E-UTRAN UEs supporting EN-DC supporting link adaptation.

This test applies to all types of release 18 and forward NR UEs and E-UTRAN UEs supporting EN-DC.

5.6.2.1.1_1.3	Test description

5.6.2.1.1_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the serving cell are set up according to Table 5.2-1 and Table 5.6.2.1.1.0-1 as appropriate.

3.	Downlink signals for NR serving cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.6.2.1.1_1.3.3.

5.6.2.1.1_1.3.2	Test procedure

1.	Set the parameters of the bandwidth, the propagation channel, the correlation matrix and the SNR for the serving cell according to Tables 5.6.2.1.1_1.4-1 and 5.6.2.1.1_1.4-2.

2.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI as per the measurement channel according to Table 5.6.2.1.1_1.4-1. The SS sends downlink MAC padding bits.

3.	Measure the average throughput on the serving cell for a duration sufficient to achieve statistical significance according to Annex G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Annex G.1.4.

5.6.2.1.1_1.3.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.6.2.1.1_1.3.3_1	Message exceptions for SA

Table 5.6.2.1.1_1.3.3_1-1: CSI request

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-1A |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| CSI request | Dependent on test parameters 1 bit |  |  |

Table 5.6.2.1.1_1.3.3_1-2: CSI-MeasConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-38 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| reportTriggerSize | 1 |  |  |
| } |  |  |  |

Table 5.6.2.1.1_1.3.3_1-3: CSI-AperiodicTriggerStateList

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.5-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-AperiodicTriggerStateList ::= SEQUENCE (SIZE (1..maxNrOfCSI-AperiodicTriggers)) OF CSI-AperiodicTriggerState { | 1 entry |  |  |
| CSI-AperiodicTriggerState[1] SEQUENCE { |  | entry 1 |  |
| associatedReportConfigInfoList SEQUENCE (SIZE(1..maxNrofReportConfigPerAperiodicTrigger)) OF CSI-AssociatedReportConfigInfo { | 1 entry |  |  |
| CSI-AssociatedReportConfigInfo[1] SEQUENCE { |  | entry 1 |  |
| reportConfigId | 0 |  |  |
| resourcesForChannel CHOICE { |  |  |  |
| nzp-CSI-RS SEQUENCE { |  |  |  |
| resourceSet | 1 |  |  |
| qcl-info SEQUENCE (SIZE(1..maxNrofAP-CSI-RS-ResourcesPerSet)) OF TCI-StateId { | 1 entry |  |  |
| TCI-StateId[1] | 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| csi-IM-ResourcesforInteference | 1 |  |  |
| nzp-CSI-RS-ResourcesforInterference | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

5.6.2.1.1_1.3.3_2	Message exceptions for NSA

Same as 5.6.2.1.1_1.3.3_1

5.6.2.1.1_1.4	Test requirement

Table 5.6.2.1.1_1.4-1 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink shall meet or exceed the specified value in Table 5.6.2.1.1_1.4-2 for the specified SNR including test tolerances for all throughput tests.

Table 5.6.2.1.1_1.4-2: Test requirements

| Test num. | Fraction of maximum throughput (%) | SNR (dB) |
| --- | --- | --- |
| Test 1 | 35% | 20.3 |
| Note 1: The maximum throughout is defined as with TBS corresponding to CQI index 15 with rank 2. |  |  |

#### 5.6.2.2	TDD

<< Skipped sections >>

5.6.2.2.1_1.3.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.6.2.2.1_1.3.3_1	Message exceptions for SA

Same as 5.6.2.1.1_1.3.3_1

5.6.2.2.1_1.3.3_2	Message exceptions for NSA

Same as 5.6.2.1.1_1.3.3_1

<< Skipped sections >>

#### 5.6.3.1	FDD

##### 5.6.3.1.1	4Rx FDD FR1 PDSCH absolute physical layer throughput performance with link adaptation

5.6.3.1.1.0	Minimum conformance requirements

The purpose of the requirements is to verify the PDSCH absolute physical layer throughput performance with link adaptation under 4 receive antenna conditions.

The performance requirements are specified in Table 5.6.3.1.1.0-2, with additional test parameters in Table 5.6.3.1.1.0-1 and the downlink physical channel setup according to Annex C.2.1.

Table 5.6.3.1.1.0-1: Test parameters

| Parameter |  |  | Unit | Test 1 |
| --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 10 |
| Subcarrier spacing |  |  | kHz | 15 |
| Duplex Mode |  |  |  | FDD |
| Propagation channel |  |  |  | TDLA30-5 |
| Antenna configuration |  |  |  | 2×4 |
| Correlation configuration |  |  |  | ULA Low |
| Beamforming Model |  |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 5, (4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 9 |
|  | CSI-RS periodicity and offset |  | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |
|  | Number of CSI-RS ports (X) |  |  | 2 |
|  | CDM Type |  |  | FD-CDM2 |
|  | Density (ρ) |  |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | Row 3, (6) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | 13 |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 |
| CSI-IM configuration | CSI-IM resource Type |  |  | Periodic |
|  | CSI-IM RE pattern |  |  | 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  |  | (4, 9) |
|  | CSI-IM timeConfig periodicity and offset |  | slot | 5/1 |
| ReportConfigType |  |  |  | Aperiodic |
| CQI-table |  |  |  | Table 2 |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |
| cqi-FormatIndicator |  |  |  | Wideband |
| pmi-FormatIndicator |  |  |  | Wideband |
| Sub-band Size |  |  | RB | 8 |
| Csi-ReportingBand |  |  |  | 1111111 |
| CSI-Report periodicity and offset |  |  | slot | Not configured |
| aperiodicTriggeringOffset |  |  |  | 5 |
| CSI request |  |  |  | 1 in slots i, where mod(i, 5) = 0, otherwise it is equal to 0 |
| reportTriggerSize |  |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |
|  |  | Codebook Mode |  | 1 |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | N/A |
|  |  | CodebookSubsetRestriction |  | 111111 |
|  |  | RI Restriction |  | N/A |
| Physical channel for CSI report |  |  |  | PUSCH |
| CQI/RI/PMI delay |  |  | ms | 6 |
| Maximum number of HARQ transmission (Note 1) |  |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | 2 |
| Measurement channel |  |  |  | As specified in Table A.4-2 Rank 1: TBS.2-1 Rank 2: TBS.2-2 |
| Note 1: For retransmission including initial transmission, RV {0,2,3,1} with same MCS and rank as initial transmission; follow the latest UE reported PMI whose rank is same as the initial transmission. |  |  |  |  |

Table 5.6.3.1.1.0-2: Minimum requirements

|  | Fraction of maximum throughput (Note 1) | SNR (dB) |
| --- | --- | --- |
| Test 1 | 50% | 18.0 |
| Note 1: The maximum throughput is defined as the throughput using the TBS corresponding to CQI index 15 with rank 2. |  |  |

The normative reference for this requirement is TS 38.101-4 [5], clause 5.6.3.1.1.

###### 5.6.3.1.1_1	4Rx FDD FR1 PDSCH performance with link adaptation - 2x4 MIMO for both SA and NSA

5.6.3.1.1_1.1	Test purpose

Verify the PDSCH absolute physical layer throughput performance with link adaptation under 4 receive antenna conditions.

5.6.3.1.1_1.2	Test applicability

This test applies to all types of release 17 NR UEs and E-UTRAN UEs supporting EN-DC supporting link adaptation.

This test applies to all types of release 18 and forward NR UEs and E-UTRAN UEs supporting EN-DC.

5.6.3.1.1_1.3	Test description

5.6.3.1.1_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the serving cell are set up according to Table 5.2-1 and Table 5.6.3.1.1.0-1 as appropriate.

3.	Downlink signals for NR serving cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.6.3.1.1_1.3.3.

5.6.3.1.1_1.3.2	Test procedure

1.	Set the parameters of the bandwidth, the propagation channel, the correlation matrix and the SNR for the serving cell according to Tables 5.6.3.1.1_1.4-1 and 5.6.3.1.1_1.4-2.

2.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI as per the measurement channel according to Table 5.6.3.1.1_1.4-1. The SS sends downlink MAC padding bits.

3.	Measure the average throughput on the serving cell for a duration sufficient to achieve statistical significance according to Annex G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Annex G.1.4.

5.6.3.1.1_1.3.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.6.3.1.1_1.3.3_1	Message exceptions for SA

Same as 5.6.2.1.1_1.3.3_1

5.6.3.1.1_1.3.3_2	Message exceptions for NSA

Same as 5.6.2.1.1_1.3.3_1

5.6.3.1.1_1.4	Test requirement

Table 5.6.3.1.1_1.4-1 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink shall meet or exceed the specified value in Table 5.6.3.1.1_1.4-2 for the specified SNR including test tolerances for all throughput tests.

Table 5.6.3.1.1_1.4-2: Test requirements

|  | Fraction of maximum throughput (Note 1) | SNR (dB) |
| --- | --- | --- |
| Test 1 | 50% | 19 |
| Note 1: The maximum throughput is defined as the throughput using the TBS corresponding to CQI index 15 with rank 2. |  |  |

#### 5.6.3.2	TDD

<< Skipped sections >>

5.6.3.2.1_1.3.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.6.3.2.1_1.3.3_1	Message exceptions for SA

Same as 5.6.2.1.1_1.3.3_1

5.6.3.2.1_1.3.3_2	Message exceptions for NSA

Same as 5.6.2.1.1_1.3.3_1

<< End of changes >>