3GPP TSG RAN WG5 Meeting #108	R5-253951r1

# Bangalore, India, 25-29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0986 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of new test case 6.3.2.1.10 2Rx FDD Single PMI with 4TX TypeI-SinglePanel Codebook for eRedCap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 6.3.2.1.10 2Rx FDD Single PMI with 4TX TypeI-SinglePanel Codebook for eRedCap is still missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 6.3.2.1.10 2Rx FDD Single PMI with 4TX TypeI-SinglePanel Codebook for eRedCap was introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 6.3.2.1.10 2Rx FDD Single PMI with 4TX TypeI-SinglePanel Codebook for eRedCap will remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.1.10 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1:The select of CR impact tick box for ME and Test specifications was cancelled. |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

6.3.2.1.9.4.3.1	Message exceptions for SA

Table 6.3.2.1.9.4.3.1-1: CSI-MeasConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.5-12 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-MeasConfig::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { | n entries | n=7 |  |
| NZP-CSI-RS-Resource[1] | NZP-CSI-RS-Resource for TRS (1) | entry 1 CSI-RS resource 1 |  |
| NZP-CSI-RS-Resource[2] | NZP-CSI-RS-Resource for TRS (2) | entry 2 CSI-RS resource 2 |  |
| NZP-CSI-RS-Resource[3] | NZP-CSI-RS-Resource for TRS (3) | entry 3 CSI-RS resource 3 |  |
| NZP-CSI-RS-Resource[4] | NZP-CSI-RS-Resource for TRS (4) | entry 4 CSI-RS resource 4 |  |
| NZP-CSI-RS-Resource[5] | NZP-CSI-RS-Resource for CSI Acquisition | entry 5 CSI-RS resource 5 |  |
| NZP-CSI-RS-Resource[6] | NZP-CSI-RS-Resource for CSI Acquisition | entry 6 CSI-RS resource 6 |  |
| } |  |  |  |
| nzp-CSI-RS-ResourceSetToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet { | n entries | n=2 |  |
| NZP-CSI-RS-ResourceSet[1] | NZP-CSI-RS-ResourceSet for TRS | entry 1, Table 5.4.2.0-12 in TS 38.508-1 [6] |  |
| NZP-CSI-RS-ResourceSet[2] | NZP-CSI-RS-ResourceSet for CSI Acquisition | entry 2, Table 6.3.2.1.9.4.3.1-2 |  |
| } |  |  |  |
| csi-ReportConfigToAddModList SEQUENCE (SIZE (1..maxNrofCSI-ReportConfigurations)) OF CSI-ReportConfig { |  | 1 entry |  |
| CSI-ReportConfig[1] | CSI-ReportConfig | Table 6.3.2.1.9.4.3.1-9 |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.2.1.9.4.3.1-2: NZP-CSI-RS-ResourceSet for CSI Acquisition

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-18 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-ResourceSet ::= SEQUENCE { |  |  |  |
| nzp_CSI_ResourceSetId | 1 |  |  |
| nzp-CSI-RS-Resources SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourcesPerSet)) OF NZP-CSI-RS-ResourceId { | 2 entries |  |  |
| NZP-CSI-RS-ResourceId[1] | 4 | entry 1 CSI-RS resource 5 |  |
| NZP-CSI-RS-ResourceId[2] | 5 | entry 2 CSI-RS resource 6 |  |
| } |  |  |  |
| repetition | off |  |  |
| aperiodicTriggeringOffset | 0 |  |  |
| trs-Info | Not present |  |  |
| } |  |  |  |

Table 6.3.2.1.9.4.3.1-3: NZP-CSI-RS-Resource for CSI Acquisition

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-14 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| nzp-CSI-RS-ResourceId | 4 | CSI-RS resource 5 |  |
|  | 5 | CSI-RS resource 6 |  |
| resourceMapping | CSI-RS-ResourceMapping for CSI Acquisition | Table 6.3.2.1.9.4.3.1-4 |  |
| powerControlOffset | 0 |  |  |
| periodicityAndOffset | Not present |  |  |
| qcl-InfoPeriodicCSI-RS | Not present |  |  |
| } |  |  |  |

Table 6.3.2.1.9.4.3.1-4: CSI-RS-ResourceMapping for CSI Acquisition

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.5-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 001100 | k0 = 4, k1 = 6, row8 |  |
| } |  |  |  |
| nrofPorts | p8 |  |  |
| firstOFDMSymbolInTimeDomain | 5 | l0 = 5 for CSI-RS resource 5 |  |
|  | 9 | l0 = 9 for CSI-RS resource 6 |  |
| cdm-Type | cdm4-FD2-TD2 |  |  |
| density CHOICE { |  |  |  |
| one | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.2.1.9.4.3.1-4A: CSI-RS-ResourceMapping for TRS

Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-9

Table 6.3.2.1.9.4.3.1-5: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType1 |  |  |
| ZP-CSI-RS-ResourceToAddModList SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource { |  | 1 entry |  |
| ZP-CSI-RS-Resource[1] | ZP-CSI-RS-Resource | Table 6.3.2.1.9.4.3.1-6 |  |
| } |  |  |  |
| p-ZP-CSI-RS-ResourceSet | p-ZP-CSI-RS-ResourceSet | Table 5.4.2.5-5A in TS 38.508-1 [6] |  |
| } |  |  |  |

Table 6.3.2.1.9.4.3.1-6: ZP-CSI-RS-Resource

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-204 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ZP-CSI-RS-Resource ::= SEQUENCE { |  |  |  |
| zp-CSI-RS-ResourceId | 0 |  |  |
| resourceMapping | CSI-RS-ResourceMapping for ZP CSI-RS | Table 6.3.2.1.9.4.3.1-7 |  |
| periodicityAndOffset | CSI-ResourcePeriodicityAndOffset for ZP CSI-RS | Table 6.3.2.1.9.4.3.1-8 |  |
| } |  |  |  |

Table 6.3.2.1.9.4.3.1-7: CSI-RS-ResourceMapping for ZP CSI-RS

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-21 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 000100 | k0 = 4, row5 |  |
| } |  |  |  |
| nrofPorts | p4 |  |  |
| firstOFDMSymbolInTimeDomain | 9 |  |  |
| firstOFDMSymbolInTimeDomain2 | Not present |  |  |
| cdm-Type | fd-CDM2 |  |  |
| density CHOICE { |  |  |  |
| one | NULL |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.2.1.9.4.3.1-8: CSI-ResourcePeriodicityAndOffset for ZP CSI-RS

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots5 | 1 |  |  |
| } |  |  |  |

Table 6.3.2.1.9.4.3.1-9: CSI-ReportConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.5-13 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ReportConfig ::= SEQUENCE { |  |  |  |
| resourcesForChannelMeasurement | 1 | CSI-ResourceConfigId for CSI Acquisition |  |
| csi-IM-ResourcesForInterference | 2 | CSI-ResourceConfigId for CSI-IM |  |
| reportConfigType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| reportSlotOffsetList SEQUENCE { | 2 entries |  |  |
| reportSlotOffsetList[1] | 7 | entry 1 |  |
| reportSlotOffsetList[2] | 7 | entry 2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| reportFreqConfiguration SEQUENCE { |  |  |  |
| pmi-FormatIndicator | Not present |  |  |
| } |  |  |  |
| csi-ReportMode-r17 | mode2 |  |  |
| codebookConfig-r18 | CodebookConfig-r18 | Table 6.3.2.1.9.4.3.1-10 |  |
| } |  |  |  |

Table 6.3.2.1.9.4.3.1-10: CodebookConfig-r18

| Derivation Path: TS 38.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CodebookConfig-r18 ::= SEQUENCE { |  |  |  |
| codebookType CHOICE { |  |  |  |
| type2 CHOICE { |  |  |  |
| typeII-CJT-r18 SEQUENCE { |  |  |  |
| n1-n2-codebookSubsetRestrictionList-r18 | N1-N2-CBSR-List-r18 | Table 6.3.2.1.9.4.3.1-11 |  |
| paramCombination-CJT-r18 | 4 |  |  |
| paramCombination-CJT-L-r18 SEQUENCE { |  | 1 entry |  |
|  | 7 |  |  |
| } |  |  |  |
| restrictedCMR-Selection-r18 | enable |  |  |
| valueOfO3-r18 | Not present |  |  |
| numberOfPMI-SubbandsPerCQI-Subband-r18 | 1 |  |  |
| typeII-RI-Restriction-r18 | 0010 |  |  |
| codebookMode-r18 | 2 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.2.1.9.4.3.1-11: N1-N2-CBSR-List-r18

| Derivation Path: TS 38.331 clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| N1-N2-CBSR-List-r18 ::= CHOICE { |  |  |  |
| four-one-r18 CHOICE { |  |  |  |
| cbsr-list-r18 SEQUENCE { |  | 2 entries |  |
|  | 0x FFFF | entry 1 |  |
|  | 0x FFFF | entry 2 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.3.2.1.9.4.3.1-12: Physical layer parameters for DCI format 1_1

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.5-16 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| PDSCH-to-HARQ_feedback timing indicator | K1 = 2 | “010” |  |
| Antenna port(s) | DMRS port 0 and DMRS port 1 as per Table 7.3.1.2.2-1 in TS 38.212 (dmrs-Type = DMRS type 1 and maxLength = len1 as per Table 4.6.3-50 in 38.508-1) | “0010” |  |

Table 6.3.2.1.9.4.3.1-13: Physical layer parameters for DCI format 0_1

| Derivation Path: TS 38.508-1 [6], Table 4.3.6.1.1.2-1 |  |  |  |
| --- | --- | --- | --- |
| Parameter | Value | Value in binary | Condition |
| CSI request | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |  |

6.3.2.1.9.4.3.2	Message exceptions for NSA

Same as in clause 6.3.2.1.9.4.3.1.

6.3.2.1.9.5	Test requirement

Table 6.3.2.1.9.5-1: Test requirement

| Parameter | Test 1 |
| --- | --- |
| g | 1.79 |

##### 6.3.2.1.10	2Rx FDD FR1 Single PMI with 4TX TypeI-SinglePanel codebook for eRedCap

6.3.2.1.10.1	Test purpose

The purpose of this test is to test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.2.1.10.2	Test applicability

This test applies to all types of NR eRedCap UE with 2Rx from Release 18 and onwards.

6.3.2.1.10.3	Minimum conformance requirements

For the parameters specified in Table 6.3.2.1.10.3-1, and using the downlink physical channels specified in Annex C.3.1, the minimum requirements are specified in Table 6.3.2.1.10.3-2.

Table 6.3.2.1.10.3-1: Test parameters (single layer)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Duplex Mode |  |  | FDD |
| Propagation channel |  |  | TDLA30-5 |
| Antenna configuration |  |  | High XP 4 x 2 (N1,N2) = (2,1) |
| Beamforming Model |  |  | As specified in Annex B.4.1 |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(4) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 5,(9) |
|  | Frequency Occupation | RB | 0 to 27 |
|  | CSI-RS periodicity and offset | slot | 5/1 |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 4 |
|  | CDM Type |  | FD-CDM2 |
|  | Density (ρ) |  | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 4,(0) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 4,(13) |
|  | Frequency Occupation | RB | 0 to 27 |
|  | CSI-RS periodicity and offset |  | Not configured |
|  | aperiodicTriggeringOffset |  | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |
|  | CSI-IM RE pattern |  | Pattern 0 |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,9) |
|  | CSI-IM timeConfig periodicity and offset | slot | Not configured |
| ReportConfigType |  |  | Aperiodic |
| CQI-table |  |  | Table 1 |
| reportQuantity |  |  | cri-RI-PMI-CQI |
| timeRestrictionForChannelMeasurements |  |  | Not configured |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |
| cqi-FormatIndicator |  |  | Wideband |
| pmi-FormatIndicator |  |  | Wideband |
| Sub-band Size |  | RB | 8 |
| csi-ReportingBand |  |  | 1111111 |
| CSI-Report periodicity and offset |  | slot | Not configured |
| Aperiodic Report Slot Offset |  |  | 4 for FDD 3 for HD-FDD |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |
| reportTriggerSize |  |  | 1 |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel |
|  | Codebook Mode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
|  | CodebookSubsetRestriction |  | 11111111 |
|  | RI Restriction |  | 00000001 |
| Physical channel for CSI report |  |  | PUSCH |
| CQI/RI/PMI delay |  | ms | 7 |
| Maximum number of HARQ transmission |  |  | 4 |
| Measurement channel (Note 4) |  |  | R.PDSCH.1-6.5 FDD R.PDSCH.1-3.2 HD-FDD |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-3), this reported PMI cannot be applied at the gNB downlink before slot#(n+3). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. Note 4: Applied reference channel depends on the supported operation mode: FDD or HD-FDD. |  |  |  |

Table 6.3.2.1.10.3-2: Minimum requirement

| Parameter | Test 1 |
| --- | --- |
| g | 1.3 |

The normative reference for this requirement is TS 38.101-4 [5] clause 6.3.2.1.10.

6.3.2.1.10.4	Test description

6.3.2.1.10.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1.

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.8 for TE diagram and section A.3.2.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 6.1.2-1 and Table 6.3.2.1.10.3-1 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2, C.3.1 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 6.3.2.1.10.4.3.

6.3.2.1.10.4.2	Test procedure

1.	Set the parameters of bandwidth, the propagation condition, antenna configuration and measurement channel according to Table 6.3.2.1.10.3-1 as appropriate.

2.	The SS shall transmit PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC with precoding matrix according to PMI report from the UE. The SS sends downlink MAC padding bits on the DL RMC. SS schedules the UL transmission with an UL RMC for CP-OFDM QPSK with 5 RBs allocated according to A.2.2.6 of TS 38.521-1 [21] to carry the PUSCH CQI feedback via PDCCH DCI format 0_1 with aperiodic CSI request triggered. No transport block is sent in parallel to the CQI feedback. Establish and  according to Annex G.3.2.

3.	Set SNR to. The SS shall transmit PDSCH with randomly selected precoding matrix from codebook (Table 5.2.2.2.1-5 in TS 38.214 [12]) every slot regardless of PMI reports from the UE. Note that each precoding matrix shall be selected in equal probabilities. The SS sends downlink MAC padding bits on the DL RMC. SS schedules the UL transmission to carry the PUSCH CSI feedback via PDCCH DCI format 0_1 with aperiodic CSI request triggered. Measure according to Annex G.3.3.

4.	Calculate. If the ratio  g which is specified in Table 6.3.2.1.10.5-1, then the test is pass. Otherwise, the test is fail.

6.3.2.1.10.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1.

6.3.2.1.10.4.3.1	Message exceptions

Table 6.3.2.1.10.4.3.1-1: CSI-ResourceConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-41 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourceConfig ::= SEQUENCE { |  |  |  |
| resourceType | Aperiodic |  |  |
| } |  |  |  |

Table 6.3.2.1.10.4.3.1-2: CSI-RS-ResourceMapping for NZP-CSI-RS

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| Row4 | 001 |  |  |
| } |  |  |  |
| nrofPorts | p4 |  |  |
| firstOFDMSymbolInTimeDomain | 13 |  |  |
| } |  |  |  |

Table 6.3.2.1.10.4.3.1-3: CSI-RS-ResourceMapping for ZP-CSI-RS

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| Row5 | 000100 |  |  |
| } |  |  |  |
| nrofPorts | p4 |  |  |
| firstOFDMSymbolInTimeDomain | 9 |  |  |
| } |  |  |  |

Table 6.3.2.1.10.4.3.1-4: CSI-IM-Resource

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-34 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| csi-IM-ResourceElementPattern |  |  |  |
| pattern0 SEQUENCE { |  |  |  |
| subcarrierLocation-p0 | s4 |  |  |
| symbolLocation-p0 | 9 |  |  |
| } |  |  |  |
| periodicityAndOffset | Not present |  |  |

Table 6.3.2.1.10.4.3.1-5: CSI-ResourcePeriodicityAndOffset

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset CHOICE { |  |  |  |
| Slots5 | 1 |  |  |
| } |  |  |  |

Table 6.3.2.1.10.4.3.1-6: CodebookConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| nrOfAntennaPorts CHOICE { |  |  |  |
| moreThanTwo SEQUENCE { |  |  |  |
| n1-n2 CHOICE { |  |  |  |
| two-one-TypeI-SinglePanel-Restriction | 11111111 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| typeI-SinglePanel-ri-Restriction | 00000001 |  |  |

Table 6.3.2.1.10.4.3.1-7: CSI-ReportConfig

| Derivation Path: TS 38.508-1 [6], clause 4.6.3, Table 4.6.3-39 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| reportConfigType CHOICE { |  |  |  |
| aperiodic SEQUENCE { |  |  |  |
| reportSlotOffsetList | 0 |  |  |
| } |  |  |  |
| reportFreqConfiguration SEQUENCE { |  |  |  |
| csi-ReportingBand CHOICE { |  |  |  |
| subbands7 | 1111111 |  |  |
| } |  |  |  |
| } |  |  |  |
| subbandSize | 8 |  |  |
| } |  |  |  |

6.3.2.1.10.5	Test requirement

Table 6.3.2.1.10.5-1: Test requirement

| Parameter | Test 1 |
| --- | --- |
| g | 1.29 |

#### 6.3.2.2	TDD

##### 6.3.2.2.1	2Rx TDD FR1 Single PMI with 4TX TypeI-SinglePanel codebook for both SA and NSA

6.3.2.2.1.1	Test Purpose

The purpose of this test is to test the accuracy of the Precoding Matrix Indicator (PMI) reporting such that the system throughput is maximized based on the precoders configured according to the UE reports.

6.3.2.2.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

<< End of changes >>