3GPP TSG-RAN5 Meeting #108	R5-253962r1

# Bengaluru, IN, 25 - 29 Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5055 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 6.5.3.12 SNPN Selection in Manual Mode / Automatically Selecting equivalent SNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN_Ph2-NGRAN_plus_CT1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-8-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of new test case 6.5.3.12 SNPN Selection in Manual Mode / Automatically Selecting equivalent SNPN |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new test case 6.5.3.12 SNPN Selection in Manual Mode / Automatically Selecting equivalent SNPN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case cannot run normally |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.3.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0621 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: adding a timer at step 24. |  |  |  |  |  |  |  |  |

<Start of Changed Section>

#### 6.5.3.12	SNPN Selection in Manual Mode / Automatically Selecting equivalent SNPN

6.5.3.12.1	Test Purpose (TP)

(1)

with { UE in Manual SNPN selection mode and has registered on an SNPN }

ensure that {

when { an SNPN is available which is declared as an equivalent SNPN by the registered SNPN }

then { UE automatically register on the equivalent SNPN }

}

6.5.3.12.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 23.122 clauses 4.9.3.1.2. Unless otherwise stated these are Rel-18 requirements.

[TS 23.122, clause 4.9.3.1.2]

The MS indicates to the user any available SNPNs which meet the criteria specified in bullets a) and b). If the MS does not support access to an SNPN using credentials from a credentials holder, this includes SNPNs in the list of "permanently forbidden SNPNs", and the list of "temporarily forbidden SNPNs". The MS may indicate to the user whether the available SNPNs are present in the list of "temporarily forbidden SNPNs" or the list of "permanently forbidden SNPNs". If the MS supports access to an SNPN using credentials from a credentials holder, this includes SNPNs in the lists of "permanently forbidden SNPNs", and the lists of "temporarily forbidden SNPNs", each list associated with an entry of the "list of subscriber data" or the PLMN subscription. If the MS supports equivalent SNPNs, this includes SNPNs in the lists of "permanently forbidden SNPNs", and the lists of "temporarily forbidden SNPNs", each list associated with an entry of the "list of subscriber data". The MS may indicate to the user whether the available SNPNs are present in a list of "temporarily forbidden SNPNs" or a list of "permanently forbidden SNPNs" for an entry of the "list of subscriber data" or the PLMN subscription. If the MS supports access to an SNPN providing access for localized services in SNPN, this includes SNPNs in the lists of "permanently forbidden SNPNs for access for localized services in SNPN", and the lists of "temporarily forbidden SNPNs for access for localized services in SNPN", each list associated with an entry of the "list of subscriber data" or the PLMN subscription. If the MS supports access to an SNPN providing access for localized services in SNPN, the MS may indicate to the user whether an available SNPN is identified by an SNPN identity contained in an entry of one of the "credentials holder controlled prioritized list of preferred SNPNs for access for localized services in SNPN" and whether the validity information of the entry is met, whether an available SNPN is broadcasting a GIN contained in an entry of one of the "credentials holder controlled prioritized list of preferred GINs for access for localized services in SNPN" and whether the validity information of the entry is met and whether an available SNPNs are present in a list of "temporarily forbidden SNPNs for access for localized services in SNPN" or a list of "permanently forbidden SNPNs for access for localized services in SNPN" for an entry of the "list of subscriber data" or the PLMN subscription.

…

Once the MS has registered on an SNPN selected by the user, the MS shall not automatically register on a different SNPN unless:

a)	the user selects automatic SNPN selection mode;

b)	the user initiates an emergency call while the MS is in limited service state and either the SNPN does not broadcast the indication of support of emergency calls in limited service state or the registration request for emergency services is rejected by the network; or

c)	the new SNPN is declared as an equivalent SNPN by the registered SNPN.

NOTE 2:	If case b) occurs, the MS can provide an indication to the upper layers that the MS has exited manual network SNPN selection mode.

If the user does not select an SNPN, the selected SNPN shall be the one that was selected either automatically or manually before the SNPN selection procedure started. If no such SNPN was selected or that SNPN is no longer available, then the MS shall attempt to camp on any acceptable cell and enter the limited service state.

6.5.3.12.3	Test description

6.5.3.12.3.1	Pre-test conditions

System Simulator:

-	3 SNPN cells, NR Cell 1, NR Cell 2 and NR Cell 3, are configured broadcasting default SNPN IDs as indicated in TS 38.508-1 [4] Table 4.4.2-4.

-	System information combination NR-27 as defined in TS 38.508-1 [4] clause 4.4.3.1.2 is used in NR cells.

UE:

-	The UE is in Manual SNPN selection mode.

-	The UE is provisioned with a “list of subscriber data” to allow access to SNPN identified by NR cell 3.

Preamble:

-	Ensure that the UE has cleared the Registered SNPN. And the UE is in state Switched OFF (state 0-A) as defined in TS 38.508-1 [4] Table 4.4A.2-0.

6.5.3.12.3.2	Test procedure sequence

Table 6.5.3.12.3.2-1/2 shows the cell configurations used during the test. Subsequent configurations marked “T1” and “T2” are applied at the points indicated in the Main behaviour description in Table 6.5.3.12.3.2-3. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 38.508-1 [4] Table 6.2.2.1-3 for FR1 and Table 6.2.2.2-2 for FR2.

Table 6.5.3.12.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 3 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T1 |  | SS/PBCH SSS EPRE |  | dBm/SCS |  | “Off” | “Off” |  | -88 |  | Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3. |  |
| T2 |  | SS/PBCH SSS EPRE |  | dBm/SCS |  | “Off” | -88 |  | -96 |  | Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3. |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -88 | -96 |  |

Table 6.5.3.12.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 3 | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | “Off” | -82 | Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3. |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | “Off” | -82 | -91 | Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3. |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -82 | -91 |  |

Table 6.5.3.12.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS adjusts cell levels according to row T1 of table 6.5.3.12.3.2-1/2. | - | - | - | - |
| 2 | The UE is Switched ON. | - | - | - | - |
| 3 | Manual selection of SNPN identified by NR Cell 3 is performed (NOTE 1). | - | - | - | - |
| 4-22a1 | Steps 2 to 20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 3. | - | - | - | - |
| 23 | SS adjusts cell levels according to row T2 of table 6.5.3.12.3.2-1/2. | - | - | - | - |
| 24 | Check: Does the UE send a RRCSetupRequest on NR Cell 2 within 60s? | --> | NR RRC: RRCSetupRequest | 1 | F |
| 25 | SS adjusts cell levels according to row T3 of table 6.5.3.12.3.2-1/2. | - | - | - | - |
| 26 | Check: Does the UE send a RRCSetupRequest on NR Cell 1? | --> | NR RRC: RRCSetupRequest | 1 | P |
| 27-44a1 | Steps 3 to 20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed on NR Cell 1. | - | - | - | - |
| NOTE 1: This is performed via MMI or AT command. |  |  |  |  |  |

6.5.3.12.3.3	Specific message contents

Table 6.5.3.12.3.3-1: REGISTRATION ACCEPT (step 15, Table 6.5.3.12.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Equivalent SNPNs |  |  |  |
| Length of SNPN list contents | '0000 1001'B | 9 octets |  |
| SNPN identity 1 |  |  |  |
| MCC | 001 |  |  |
| MNC | 01 |  |  |
| NID |  |  |  |
| Length of NID contents | '0000 0110'B | 6 octets |  |
| NID |  |  |  |
| Assignment mode | '0001'B |  |  |
| NID value | 1 |  |  |

Table 6.5.3.12.3.3-2: REGISTRATION REQUEST (step 6 and step 28, Table 6.5.3.12.3.2-3)

| Derivation path: TS 38.508-1 Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GS registration type value | ‘001’B | Initial registration |  |

<End of Changed Section>