3GPP TSG-RAN5 Meeting #108	R5-253996r1

# Bengaluru, India, 25th August 2025 – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5472 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to NB-IoT CP PUR test case 22.1.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17_Test |  |  |  |  |  | Date: |  |  | 2025-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The TPs in the test case are defined incompletely and have to be updated. - The TP to test if UE releases the pur-Config after the cell it is camping on disables PUR functionality is missing. Also corresponding conformance requirements were missing. - In the conformance requirements referenced contents of TS 36.321 cl. 5.4.7.1 are outdated. - The delay of a start of the pur-ResponseWindowTimer depends on the UE capability and should be taken into account throughout the test case execution. - Since UE is not expected to release pur-Config after each PUR transmission there is no need to reconfigure UE each time when the RRC Connection is released. - Some TP verification steps appear to be insufficiently strict. - Steps defining TP-specific SS configuration are missing. - Due to change 4 the Exception and step 26b1 became redundant. - Due to change 2 new steps are to be added to Table 22.1.5.3.2-2. Also, specific message contents for the NCell 2 have to be defined. - Due to new steps more clarify about on which NCell the communication with UE takes place is needed. - Editorial changes are needed to result in a compliant 3GPP prose. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated TPs definitions. - The contents of original TP5 are reflected in the modified definitions of TP1 and TP4. Contents of TP5 are updated to verify the release of the pur-Config after cell-reselection to a cell that does not support PUR functionality. Added corresponding conformance requirements. - Updated referenced contents of TS 36.321 cl. 5.4.7.1. - Updated System Simulator pre-test conditions to consider the UE capability to delay the pur-ResponseWindowTimer delay when scheduling the response to a message received over PUR. - Removed redundant UE configurations with pur-Config in Steps 11, 33, 61 in Table 22.1.5.3.2-2. - Updated wording in steps 43, 53, 63 and 82 of Table 22.1.5.3.2-2 to straighten TP verification. - Added steps 3a and 71a in Table 22.1.5.3.2-2 to configure SS for a corresponding TP. - Removed Exception and step 26b1, updated step 26a1 in Table 22.1.5.3.2-2. - Added steps 85-94 to Table 22.1.5.3.2-2 to verify updated TP5. Consequently, added Table 22.1.5.3.3-7 to allow the system information modification for NCell 2. - Added a clarification that after step 81 the messages are exchanged between UE and NCell 2. Furthermore, the note about the PUR configuration of NCell 2 has been modified to be included in Note 3 of the Table 22.1.5.3.2-2. - Applied editorial changes, where necessary. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Test case will not cover all conformance requirements relevant for PUR verification. - The SS may not be configured according to the needs of the TP. - A non-conformant UE may pass the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision1: In Table 22.1.5.3.2-2 updated the wording of step 92 to better align with the description of the TP5. In Table 22.1.5.3.3-3 replaced the absence of the pur-Config by the absence of nonCriticalExtensions for steps 21, 61. Removed irrelevant and added missing IEs, added comments for the types of nontCriticalExtensions for a better readability, fixed inclination. |  |  |  |  |  |  |  |  |

### 22.1.5	NB-IoT / NTN / Control Plane CIoT EPS Optimization /  PUR

##### 22.1.5.1	Test Purpose (TP)

(1)

with { UE in E-UTRA RRC_IDLE state with a valid pur-Config }

ensure that {

when { UE has data to be sent in the UL }

then { UE sends RRCEarlyDataRequest-NB using the PUR uplink grant and starts to monitor NPDCCH identified by PUR-RNTI during the correct pur-ResponseWindowTimer }

}

(2)

with { UE has previously sent RRCEarlyDataRequest-NB using the PUR uplink grant and pur-ResponseWindowTimer is running }

ensure that {

when { UE receives L1 ACK for transmission using PUR }

then { UE retains in E-UTRA RRC_IDLE state }

}

(3)

with { UE has previously sent RRCEarlyDataRequest-NB using the PUR uplink grant and the pur-ResponseWindowTimer is running }

ensure that {

when { UE receives a PDCCH transmission addressed to the PUR-RNTI and the MAC PDU contains only Timing Advance Command MAC control element }

then { UE retains in E-UTRA RRC_IDLE state }

}

(4)

with { UE in E-UTRA RRC_IDLE state having previously sent RRCEarlyDataRequest-NB using the PUR uplink grant and the  pur-ResponseWindowTimer is running }

ensure that {

when { UE receives a PDCCH transmission addressed to the PUR-RNTI and contains an UL grant for a retransmission }

then { UE retransmits the RRCEarlyDataRequest-NB message using the UL grant provided by the SS and restarts pur-ResponseWindowTimer }

}

(5)

with { UE in E-UTRA RRC_IDLE state and with a valid pur-Config }

ensure that {

when { UE has data to be sent in the UL and detects that cp-PUR-EPC is not included in the SystemInformationBlockType2-NB broadcasted in the cell }

then { UE releases the pur-Config and sends RRCConnectionRequest-NB using legacy RRC connection establishment procedure

}

}

(6)

with { UE in E-UTRA RRC_IDLE state having previously sent RRCEarlyDataRequest-NB using the PUR uplink grant and the  pur-ResponseWindowTimer is running }

ensure that {

when { UE receives RRCConnectionSetup-NB }

then { UE responds with RRCConnectionSetupComplete-NB including CONTROL PLANE SERVICE REQUEST and ESM DATA TRANSPORT }

}

(7)

with { UE in E-UTRA RRC_IDLE state with a valid pur-Config and PUR-timeAlignmentTimer has expired }

ensure that {

when { The UE has UL data to send}

then { UE does not attempt to use the PUR uplink grant and sends RRCConnectionRequest-NB using legacy RRC connection establishment procedure }

}

(8)

with { UE in E-UTRA RRC_IDLE state with a valid pur-Config and the serving cell NRSRP has  decreased by more than decreaseThresh }

ensure that {

when { The UE has UL data to send}

then { UE does not attempt to use the PUR uplink grant and sends RRCConnectionRequest-NB using legacy RRC connection establishment procedure }

}

(9)

with { UE in E-UTRA RRC_IDLE state with a valid pur-Config and the serving cell NRSRP has increased by more than increaseThresh }

ensure that {

when { The UE has UL data to send}

then { UE does not attempt to use the PUR uplink grant and sends RRCConnectionRequest-NB using legacy RRC connection establishment procedure }

}

(10)

with { UE in E-UTRA RRC_IDLE state with a valid pur-Config }

ensure that {

when { The UE has UL data to send and detects that pur-ImplicitReleaseAfter number of consecutive PUR occasions have been skipped }

then { UE releases the pur-Config and sends RRCConnectionRequest-NB using legacy RRC connection establishment procedure }

}

(11)

with { UE in E-UTRA RRC_IDLE state with a valid pur-Config }

ensure that {

when { The UE has UL data to send and re-selects to an Ncell that cp-PUR-EPC is included in the SystemInformationBlockType2-NB }

then { UE releases the pur-Config and sends consecutive RRCConnectionRequest-NB using legacy RRC connection establishment procedure }

}

(12)

with { UE has previously sent RRCEarlyDataRequest-NB using the PUR uplink grant and the pur-ResponseWindowTimer is running }

ensure that {

when { UE receives RRCEarlyDataComplete-NB for transmission using PUR }

then { UE retains in E-UTRA RRC_IDLE state }

}

##### 22.1.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: 3GPP TS 36.300[10] clause 7.3d.2, TS 36.321[14] clauses 5.4.7.1, 5.4.7.2 and TS 36.331[17] clauses 5.3.1.1, 5.3.3.1c, 5.6.23.1, 5.6.23.2, 5.6.23.3 and 5.2.2.9. Unless otherwise stated these are Rel-17 requirements

[TS 36.300[10], clause 7.3d.2]

The procedure for PUR configuration request and PUR configuration is common to the Control Plane CIoT EPS/5GS optimisations and the User Plane CIoT EPS/5GS optimisations and is illustrated in Figure 7.3d-1.

Figure 7.3d-1: PUR Configuration Request and PUR Configuration

0.	The UE is in RRC_CONNECTED and PUR is enabled in the cell.

1.	The UE may indicate to the (ng-)eNB that it is interested in being configured with PUR by sending PURConfigurationRequest message providing information about the requested resource (e.g. No. of occurences, periodicity, time offset, TBS, RRC Ack, etc.). Alternatively, the UE may indicate to the (ng-)eNB in the PURConfigurationRequest message that it is interested in the configured PUR to be released.

2.	When the (ng-)eNB moves the UE to RRC_IDLE, based on a precedent UE PUR configuration request, subscription information and/or local policies, the (ng-)eNB may decide to provide a PUR resource to the UE or to release an existing PUR resource. The (ng-)eNB includes the details of the PUR configuration or a PUR release indication in the RRCConnectionRelease message.

[TS 36.321[14], clause 5.4.7.1]

Transmission using PUR is initiated by the RRC layer. When transmission using PUR is initiated, RRC layer provides MAC with the following information:

-	PUR-RNTI;

-	Duration of PUR response window pur-ResponseWindowTimer;

-	UL grant information.

If the MAC entity has a PUR-RNTI, the MAC entity shall for each TTI for which RRC layer has provided uplink grant for transmission using PUR:

-	deliver the uplink grant, and the associated HARQ information to the HARQ entity for this TTI.

After transmission using PUR, the MAC entity shall monitor PDCCH identified by PUR-RNTI in the PUR response window using timer pur-ResponseWindowTimer:

-	if PUR was transmitted in a non-terrestrial network and UE supports delaying the start of the pur-ResponseWindowTimer:

-	the MAC entity shall start pur-ResponseWindowTimer at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes plus UE-eNB RTT.

-	else:

-	the MAC entity shall start pur-ResponseWindowTimer at the subframe that contains the end of the corresponding PUSCH transmission plus 4 subframes.

While pur-ResponseWindowTimer is running, the MAC entity shall:

-	if the PDCCH transmission is addressed to the PUR-RNTI and contains an UL grant for a retransmission:

-	if PUR was transmitted in a non-terrestrial network and UE supports delaying the start of the pur-ResponseWindowTimer:

-	restart pur-ResponseWindowTimer at the last subframe of a PUSCH transmission corresponding to the retransmission indicated by the UL grant plus 4 subframes plus UE-eNB RTT.

-	else:

-	restart pur-ResponseWindowTimer at the last subframe of a PUSCH transmission corresponding to the retransmission indicated by the UL grant plus 4 subframes.

-	if L1 ACK for transmission using PUR is received from lower layers; or

-	if PDCCH transmission is addressed to the PUR-RNTI and the MAC PDU is successfully decoded:

-	stop pur-ResponseWindowTimer;

-	if L1 ACK for transmission using PUR is received from lower layers or the MAC PDU contains only Timing Advance Command MAC control element:

-	indicate to upper layers the transmission using PUR was successful;

-	if repetition adjustment for transmission using PUR is received from lower layers:

-	indicate the value of the repetition adjustment to upper layers.

-	discard the PUR-RNTI.

-	else if fallback indication for PUR is received from lower layers:

-	stop pur-ResponseWindowTimer;

-	indicate to upper layers PUR fallback indication is received;

-	if repetition adjustment for transmission using PUR is received from lower layers:

-	indicate the value of the repetition adjustment to upper layers.

-	discard the PUR-RNTI.

-	if the pur-ResponseWindowTimer expires:

-	if PUR was transmitted in a non-terrestrial network:

-	if no notification of a reception of a PDCCH transmission addressed to the PUR-RNTI containing an UL grant for a retransmission was received after the start of pur-ResponseWindowTimer:

-	indicate to upper layers the transmission using PUR has failed;

-	discard the PUR-RNTI.

-	else:

-	indicate to upper layers the transmission using PUR has failed;

-	discard the PUR-RNTI.

[TS 36.321[14], clause 5.4.7.2]

MAC entity may be configured with timer pur-TimeAlignmentTimer by upper layers as specified in TS 36.331 [8], clause 5.3.8.3.

The MAC entity shall:

-	when pur-TimeAlignmentTimer configuration is received from upper layers:

-	start or restart pur-TimeAlignmentTimer.

-	when pur-TimeAlignmentTimer is released by upper layers:

-	stop the pur-TimeAlignmentTimer, if running.

-	when a Timing Advance Command MAC control element is received or PDCCH indicates timing advance adjustment as specified in TS 36.212 [5] and if a NTA has been stored or maintained:

-	if the Timing Advance Command MAC control element or PDCCH indicating timing advance adjustment is addressed with a PUR-RNTI:

-	apply the Timing Advance Command or the timing advance adjustment;

-	start or restart the pur-TimeAlignmentTimer, if configured;

-	indicate to upper layers that the Timing Advance value has been adjusted.

-	upon considering a Random Access procedure successfully completed:

-	start or restart the pur-TimeAlignmentTimer, if configured;

-	indicate to upper layers that the Timing Advance value has been adjusted;

-	if a temporary NTA has been stored, delete the stored temporary NTA.

-	upon considering a Random Access procedure unsuccessfully completed, if a temporary NTA has been stored:

-	set the NTA to the stored temporary NTA;

-	delete the stored temporary NTA.

Upon request from upper layers, MAC entity shall indicate whether pur-TimeAlignmentTimer is running.

[TS 36.331[17], clause 5.3.1.1]

In case of CP-EDT or CP transmission using PUR, the data are appended in the RRCEarlyDataRequest and RRCEarlyDataComplete messages, if available, and sent over SRB0. In case of UP-EDT or UP transmission using PUR, security is re-activated prior to transmission of RRC message using the nextHopChainingCount provided in the RRCConnectionRelease message with suspend indication during the preceding suspend procedure and the radio bearers are re-established. The uplink data are transmitted ciphered on DTCH multiplexed with the RRCConnectionResumeRequest message on CCCH. In the downlink, the data, if available, are transmitted on DTCH multiplexed with the RRCConnectionRelease message on DCCH. In response to a request for EDT or transmission using PUR, E-UTRA/EPC or E-UTRA/5GC may also choose to establish or resume the RRC connection.

[TS 36.331[17], clause 5.3.3.1]

Figure 5.3.3.1-7: CP-EDT or CP transmission using PUR, successful

Figure 5.3.3.1-7a: CP transmission using PUR, successful

Figure 5.3.3.1-8: CP-EDT fallback or fallback from CP transmission using PUR to RRC connection establishment, successful

Figure 5.3.3.1-9: CP-EDT or CP transmission using PUR, network reject

The purpose of this procedure is to establish an RRC connection, to resume a suspended RRC connection, to move the UE from RRC_INACTIVE to RRC_CONNECTED, to perform EDT or to perform transmission using PUR. RRC connection establishment involves SRB1 (and SRB1bis for NB-IoT) establishment. The procedure is also used to transfer the initial NAS dedicated information/ message from the UE to E-UTRAN.

E-UTRAN applies the procedure as follows:

-	When establishing an RRC connection:

-	to establish SRB1 and, for NB-IoT, SRB1bis;

-	When resuming an RRC connection from a suspended RRC connection or from RRC_INACTIVE:

-	to restore the AS configuration from a stored context including resuming SRB(s) and DRB(s);

-	When performing EDT;

-	When performing transmission using PUR.

[TS 36.331[17], clause 5.3.3.1c]

A BL UE, UE in CE or NB-IoT UE can initiate transmission using PUR when all of the following conditions are fulfilled:

1>	the UE has a valid PUR configuration for the serving cell as specified in 5.3.3.20;

1>	the UE has a valid timing alignment value as specified in 5.3.3.19;

1>	the upper layers request establishment of an RRC connection; or the upper layers request resumption of an RRC connection and the UE has a stored value of the nextHopChainingCount provided in the RRCConnectionRelease message with suspend indication during the preceding suspend procedure;

1>	the establishment or resumption request is for mobile originating calls and the establishment cause is mo-Data or mo-ExceptionData or delayTolerantAccess;

1>	for CP transmission using PUR, the size of the resulting MAC PDU including the total UL data is expected to be smaller than or equal to the TBS configured for PUR.

NOTE 1:	Upper layers request or resume an RRC connection. The interaction with NAS is up to UE implementation.

NOTE 2:	It is up to UE implementation how the UE determines whether the establishment or resumption request is suitable for transmission using PUR.

[TS 36.331[17] clause 5.3.3.20]

The UE configured with pur-Config shall:

1>	consider that the first PUR occasion occurs at the H-SFN/SFN/subframe given by:

-	H-SFN = (H-SFNRef + offset) mod 1024 occurring after FLOOR (offset/1024) H-SFN cycles;

-	SFN and subframe indicated by startSFN and startSubframe;

where:

-	offset is given by periodicityAndOffset;

-	H-SFNRef corresponds to the last subframe of the first transmission of RRCConnectionRelease message containing pur-Config, taking into account hsfn-LSB-Info;

-	H-SFN cycle corresponds to the duration of 1024 H-SFNs;

1>	if the pur-NumOccasions is set to one, for the first PUR occasion:

2>	if transmission using PUR in accordance with conditions in 5.3.3.1c is not initiated; or

2>	if transmission using PUR in accordance with conditions in 5.3.3.1c has been initiated, after the completion of the transmission using PUR:

3>	if pur-TimeAlignmentTimer is configured, indicate to lower layers that pur-TimeAlignmentTimer is released;

3>	release pur-Config;

3>	discard previously stored pur-Config;

1>	else:

2>	consider that the subsequent PUR occasions occur periodically after the occurence of the first PUR occasion at the SFN/subframe indicated by startSubframe and startSFN and periodicity given by periodicityAndOffset;

2>	if the pur-ImplicitReleaseAfter is configured, for each PUR occasion occurring while the UE is in RRC_IDLE:

3>	if transmission using PUR in accordance with conditions in 5.3.3.1c is not initiated; or

3>	if PUR failure indication is received from lower layers:

4>	consider the PUR occasion as skipped;

4>	if pur-ImplicitReleaseAfter number of consecutive PUR occasions have been skipped:

5>	if pur-TimeAlignmentTimer is configured, indicate to lower layers that pur-TimeAlignmentTimer is released;

5>	release pur-Config;

5>	discard previously stored pur-Config.

[TS 36.331, clause 5.6.23.1]

Figure 5.6.23.1-1: PUR Configuration Request

The purpose of this procedure is to indicate to the E-UTRAN that the UE is interested to be configured with PUR and provide PUR related information to E-UTRAN, or that the UE is no longer interested to be configured with PUR.

The procedure is applicable only for BL UEs, UEs in CE or NB-IoT UEs.

[TS 36.331, clause 5.6.23.2]

A UE in RRC_CONNECTED may initiate the procedure when all of the following conditions are fulfilled:

1>	if the UE is connected to EPC:

2>	for CP transmission using PUR, SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) includes cp-PUR-EPC; or

2>	for UP transmission using PUR, SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) includes up-PUR-EPC;

1>	else if the UE is connected to 5GC:

2>	for CP transmission using PUR, SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) includes cp-PUR-5GC; or

2>	for UP transmission using PUR, SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT) includes up-PUR-5GC;

1>	the size of the resulting MAC PDU including the total UL data size of the traffic is smaller than or equal to the maximum supported TBS based on the UE category.

NOTE 1:	It is up to UE implementation how the UE determines whether the size of UL data is suitable for transmission using PUR.

Upon initiating the procedure, the UE shall:

1>	initiate transmission of the PURConfigurationRequest message in accordance with 5.6.23.3;

[TS 36.331[17], clause 5.6.23.3]

When initiating the procedure according to 5.6.23.2, the UE shall set the contents of the PURConfigurationRequest message as follows:

1>	if the UE is interested to be configured with PUR, include pur-SetupRequest and set the contents of pur-SetupRequest as follows:

2>	set requestedNumOccasions to the requested number of PUR occasions requested;

2>	set requestedPeriodicityAndOffset according to the requested periodicity between consecutive PUR occasions and the requested time offset with respect to current time until the first PUR occasion;

2>	set requestedTBS to the requested TBS for the PUR occasion(s);

2>	if RRC response message is preferred by the UE for acknowledging the reception of a transmission using PUR, include rrc-ACK;

1>	if the UE is no longer interested to be configured with PUR:

2>	include pur-ReleaseRequest;

The UE shall submit the PURConfigurationRequest message to lower layers for transmission.

[TS 36.331[17], clause 5.2.2.9]

…

Upon receiving SystemInformationBlockType2 (SystemInformationBlockType2-NB in NB-IoT), the UE shall:

…

1>	if cp-PUR-EPC is not included and the UE connected to EPC in RRC_IDLE without a suspended RRC connection is configured with pur-Config:

2> if pur-TimeAlignmentTimer is configured, indicate to lower layers that pur-TimeAlignmentTimer is released;

2>	release pur-Config;

2>	discard previously stored pur-Config.

##### 22.1.5.3	Test description

22.1.5.3.1	Pre-test conditions

System Simulator:

-	Ncell 1. and Ncell 2 as specified in TS 36.508 clause 8.1.4.1.1 are configured

-	System information combination 8 as defined in TS 36.508 [18] clause 8.1.4.3.1.1 is used on both cells.

-	If UE supports delaying the start of the pur-ResponseWindowTimer for NTN operation as specified in TS 36.306 [13] (pc_NB_ntn_PUR_TimerEnhancement), the SS is configured to delay the response to over the PUR received UL message by UE-eNB RTT according to TS 36.321 [14].

UE:

-	The UE is configured to use PUR to establish or resume the RRC connection.

-	The UE is configured to utilise PUR services as transport mechanism for user data of 1 octet.

Preamble

-	UE is in state NB-IoT UE Attach, Connected Mode, UE Test Loopback Activated (State 2B-NB) with test loop mode G according to TS 36.508 [18] in Ncell 1.

22.1.5.3.2	Test procedure sequence

Table 22.1.5.3.2-1 shows the cell configurations used during the test. The configuration T0 indicates the initial conditions. Subsequent configurations marked “T1”,“T2” etc are applied at the points indicated in the Main behaviour description in Table 22.1.5.3.2-2. Cell powers are chosen for a serving cell and a non-suitable “Off” cell as defined in TS 36.508[18].

Table 22.1.5.3.2-1: Ncell configuration changes over time

|  | Parameter | Unit | Ncell 1 | Ncell 2 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T0 | NRS EPRE | dBm/15kHz | -70 | “Off” | Power level “Off” is defined in TS 36.508[18] Table 6.2.2.1-1 |
| T1 | NRS EPRE | dBm/15kHz | -85 | “Off” | Power level “Off” is defined in TS 36.508[18] Table 6.2.2.1-1 |
| T2 | NRS EPRE | dBm/15kHz | -55 | “Off” | Power level “Off” is defined in TS 36.508[18] Table 6.2.2.1-1 |
| T3 | NRS EPRE | dBm/15kHz | “Off” | -70 | Power level “Off” is defined in TS 36.508[18] Table 6.2.2.1-1 |

Table 22.1.5.3.2-2: Main behaviour

| St |  | Procedure |  | Message Sequence |  |  |  | TP |  | Verdict |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | U - S |  | Message |  |  |  |  |  |  |
| - |  | EXCEPTION: In parallel with below step 1 and steps 74-80the parallel behaviour defined in table 22.1.5.3.2-3 might happen. |  | - |  | - |  | - |  | - |  |  |
| - | Unless otherwise stated all below messages are sent and received on Ncell1 | - | - | - | - |
| 1 | SS transmits an ESM DATA TRANSPORT message containing downlink user data of 1 octet. | <-- | RRC: DLInformationTransfer-NB NAS: ESM DATA TRANSPORT | - | - |  |
| 2 | The SS releases the RRC connection including PUR-Config with pur-TimeAlignmentTimer > IP PDU delay. (Note 2) | <-- | RRC: RRCConnectionRelease-NB | - | - |
| 3 | Check: Does the UE send RRCEarlyDataRequest-NB including CONTROL PLANE SERVICE REQUEST and ESM DATA TRANSPORT containing the data received in step 1 using PUR? | --> | RRC: RRCEarlyDataRequest-NB NAS: CONTROL PLANE SERVICE REQUEST NAS: ESM DATA TRANSPORT | 1 | P |
| 3A | The SS is configured to schedule the DL DCI transmission in the last slot of the pur-ResponseWindowTimer | - | - | - | - |
| 4 | The SS sends RRCEarlyDataComplete-NB for the transmission in step 3. | <-- | RRC: RRCEarlyDataComplete-NB | - | - |
| 5 | Check: The test procedure in TS 36.508[18] table 8.1.5A.2.3-1 is executed to check that the UE has released the RRC connection. | - | - | 1,12 | P |
| 6 | The SS sends OPEN UE TEST LOOP message | <-- | TC: OPEN UE TEST LOOP | - | - |
| 7 | The UE sends OPEN UE TEST LOOP COMPLETE message | --> | TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 8 | The SS sends CLOSE UE TEST LOOP message | <-- | TC: CLOSE UE TEST LOOP | - | - |
| 9 | The UE sends CLOSE UE TEST LOOP COMPLETE message | --> | TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 10 | SS transmits an ESM DATA TRANSPORT message containing downlink user data of 1 octet. | <-- | RRC: DLInformationTransfer-NB NAS: ESM DATA TRANSPORT | - | - |
| 11 | The SS releases the RRC connection. with PUR-Config (pur-TimeAlignmentTimer> IP PDU delay). (Note 1) | <-- | RRC: RRCConnectionRelease-NB | - | - |
| 12 | The SS is configured to schedule the Timing Advance MAC CE in a response for the PUR transmission within the pur-ResponseWindowTimer. | - | - | - | - |
| 13 | Check: Does the UE send RRCEarlyDataRequest-NB including CONTROL PLANE SERVICE REQUEST and ESM DATA TRANSPORT containing the data received in step 10 using PUR? | --> | RRC: RRCEarlyDataRequest-NB NAS: CONTROL PLANE SERVICE REQUEST NAS: ESM DATA TRANSPORT | 1 | P |
| 14 | The SS transmits a Timing Advance Command addressed with the PUR-RNTI without any additional padding (i.e. TBS=16). Start Timer_1 = Time Alignment timer value. The Timing Advance Command indicates a TA index of 31 resulting in no change of timing advance (see TS 36.213 clause 4.2.3) | <-- | NPDCCH (DCI N1)) MAC Control PDU (Timing Advance) | - | - |
| 15 | Check: The test procedure in TS 36.508[18] table 8.1.5A.2.3-1 is executed to check that the UE has released the RRC connection. | - | - | 1,3 | P |
| 16 | The SS sends OPEN UE TEST LOOP message | <-- | TC: OPEN UE TEST LOOP | - | - |
| 17 | The UE sends OPEN UE TEST LOOP COMPLETE message | --> | TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 18 | The SS sends CLOSE UE TEST LOOP message | <-- | TC: CLOSE UE TEST LOOP | - | - |
| 19 | The UE sends CLOSE UE TEST LOOP COMPLETE message | --> | TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 20 | SS transmits an ESM DATA TRANSPORT message containing downlink user data of 1 octet. | <-- | RRC: DLInformationTransfer-NB ESM DATA TRANSPORT | - | - |
| 21 | The SS releases the RRC connection. | <-- | RRC: RRCConnectionRelease-NB | - | - |
| 22 | The SS is configured to schedule an UL grant in a response for the PUR transmission. The UL grant Is scheduled at a time that exceeds the value of the pur-TimeAlignementTimer (Note 3)r | - | - | - | - |
| 23 | The UE sends RRCEarlyDataRequest-NB including CONTROL PLANE SERVICE REQUEST and ESM DATA TRANSPORT containing the data received in step 21 using PUR. | --> | RRC: RRCEarlyDataRequest-NB NAS: CONTROL PLANE SERVICE REQUEST NAS: ESM DATA TRANSPORT | - | - |
| 24 | The SS sends a PDCCH transmission addressed to the PUR-RNTI and contains an UL grant for a retransmission. | <-- | (UL Grant, Timing Advance Command=1) | - | - |
| 25 | Check: Does the UE re-transmit RRCEarlyDataRequest-NB including CONTROL PLANE SERVICE REQUEST and ESM DATA TRANSPORT containing the data received in step 21 using the grant provided in step 24? | --> | RRC: RRCEarlyDataRequest-NB NAS: CONTROL PLANE SERVICE REQUEST NAS: ESM DATA TRANSPORT | 4 | P |
| 26 | The SS sends an L1 ACK in the subframe that corresponds a running pur-ResponseWindowTimer. | <-- | L1 ACK | - | - |
| 26b1 |  | Void |  |  |  |  |  |  |  |  |  |
| 27 | The test procedure in TS 36.508[18] table 8.1.5A.2.3-1 is executed to check that the UE has released the RRC connection. | - | - | 2 | P |
| 28 | The SS sends OPEN UE TEST LOOP message | <-- | TC: OPEN UE TEST LOOP | - | - |
| 29 | The UE sends OPEN UE TEST LOOP COMPLETE message | --> | TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 30 | The SS sends CLOSE UE TEST LOOP message | <-- | TC: CLOSE UE TEST LOOP | - | - |
| 31 | The UE sends CLOSE UE TEST LOOP COMPLETE message | --> | TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 32 | SS transmits an ESM DATA TRANSPORT message containing downlink user data of 1 octet. | <-- | RRC: DLInformationTransfer-NB ESM DATA TRANSPORT | - | - |
| 33 | The SS releases the RRC connection using PUR-Config. | <-- | RRC: RRCConnectionRelease-NB | - | - |
| 34 | The UE sends RRCEarlyDataRequest-NB including ESM DATA TRANSPORT containing CONTROL PLANE SERVICE REQUEST and the data received in step 32 using PUR. | --> | RRC: RRCEarlyDataRequest-NB NAS: CONTROL PLANE SERVICE REQUEST NAS: ESM DATA TRANSPORT | - | - |
| 35 | The SS sends RRCConnectionSetup-NB | <-- | RRC: RRCConnectionSetup-NB | - | - |
| 36 | Check: Does the UE send RRCConnectionSetupComplete-NB message including CONTROL PLANE SERVICE REQUEST and ESM DATA TRANSPORT containing the data received in step 32 using PUR? | --> | RRC: RRCConnectionSetupComplete-NB NAS: CONTROL PLANE SERVICE REQUEST NAS: ESM DATA TRANSPORT | 6 | P |
| 37 | The SS sends OPEN UE TEST LOOP message | <-- | TC: OPEN UE TEST LOOP | - | - |
| 38 | The UE sends OPEN UE TEST LOOP COMPLETE message | --> | TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 39 | The SS sends CLOSE UE TEST LOOP message | <-- | TC: CLOSE UE TEST LOOP | - | - |
| 40 | The UE sends CLOSE UE TEST LOOP COMPLETE message | --> | TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 41 | SS transmits an ESM DATA TRANSPORT message containing downlink user data of 1 octet. | <-- | RRC: DLInformationTransfer-NB ESM DATA TRANSPORT | - | - |
| 42 | The SS releases the RRC connection including PUR-Config with pur-TimeAlignmentTimer< IP PDU delay. (Note 1) | <-- | RRC: RRCConnectionRelease-NB | - | - |
| 43 | Check: Does the UE send RRCConnectionRequest-NB initiating the RRC connection setup procedure by sending a Random Access Preamble on RACH and does not attempt to send RRCEarlyDataRequest-NB using PUR? | --> | RRC: RRCConnectionRequest-NB | 7 | P |
| 44 | The SS sends RRCConnectionSetup-NB | <-- | RRC: RRCConnectionSetup-NB | - | - |
| 45 | The UE sends RRCConnectionSetupComplete-NB message including CONTROL PLANE SERVICE REQUEST and ESM DATA TRANSPORT containing the DL data received in Step 41. | --> | RRC: RRCConnectionSetupComplete-NB NAS: CONTROL PLANE SERVICE REQUEST NAS: ESM DATA TRANSPORT | - | - |
| 46 | The SS sends OPEN UE TEST LOOP message | <-- | TC: OPEN UE TEST LOOP | - | - |
| 47 | The UE sends OPEN UE TEST LOOP COMPLETE message | --> | TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 48 | The SS sends CLOSE UE TEST LOOP message | <-- | TC: CLOSE UE TEST LOOP | - | - |
| 49 | The UE sends CLOSE UE TEST LOOP COMPLETE message | --> | TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 50 | SS transmits an ESM DATA TRANSPORT message containing downlink user data of 1 octet. | <-- | RRC: DLInformationTransfer-NB ESM DATA TRANSPORT | - | - |
| 51 | The SS releases the RRC connection including PUR-Config with pur-TimeAlignmentTimer >IP PDU delay. (Note 2) | <-- | RRC: RRCConnectionRelease-NB | - | - |
| 52 | SS adjusts Ncell levels according to row T1 of table 22.1.5.3.2-1. | - | - | - | - |
| 53 | Check: Does the UE send RRCConnectionRequest-NB initiating the RRC connection setup procedure by sending a Random Access Preamble on RACH and does not attempt to send RRCEarlyDataRequest-NB using PUR? | --> | RRC: RRCConnectionRequest-NB | 8 | P |
| 54 | The SS sends RRCConnectionSetup-NB | <-- | RRC: RRCConnectionSetup-NB | - | - |
| 55 | The UE sends RRCConnectionSetupComplete-NB message including CONTROL PLANE SERVICE REQUEST and ESM DATA TRANSPORT containing thedata received in Step 50. | --> | RRC: RRCConnectionSetupComplete-NB NAS: CONTROL PLANE SERVICE REQUEST NAS: ESM DATA TRANSPORT | - | - |
| 56 | The SS sends OPEN UE TEST LOOP message | <-- | TC: OPEN UE TEST LOOP | - | - |
| 57 | The UE sends OPEN UE TEST LOOP COMPLETE message | --> | TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 58 | The SS sends CLOSE UE TEST LOOP message | <-- | TC: CLOSE UE TEST LOOP | - | - |
| 59 | The UE sends CLOSE UE TEST LOOP COMPLETE message | --> | TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 60 | SS transmits an ESM DATA TRANSPORT message containing downlink user data of 1 octet. | <-- | RRC: DLInformationTransfer-NB ESM DATA TRANSPORT | - | - |
| 61 | The SS releases the RRC connection. | <-- | RRC: RRCConnectionRelease-NB | - | - |
| 62 | SS adjusts Ncell levels according to row T2 of table 22.1.5.3.2-1. | - | - | - | - |
| 63 | Check: Does the UE send RRCConnectionRequest-NB initiating the RRC connection setup procedure by sending a Random Access Preamble on RACH and does not attempt to send RRCEarlyDataRequest-NB using PUR? | --> | RRC: RRCConnectionRequest-NB | 9 | P |
| 64 | The SS sends RRCConnectionSetup-NB | <-- | RRC: RRCConnectionSetup-NB | - | - |
| 65 | The UE sends RRCConnectionSetupComplete-NB message including CONTROL PLANE SERVICE REQUEST and ESM DATA TRANSPORT containing the data received in Step 60. | --> | RRC: RRCConnectionSetupComplete-NB NAS: CONTROL PLANE SERVICE REQUEST NAS: ESM DATA TRANSPORT | - | - |
| 66 | The SS sends OPEN UE TEST LOOP message | <-- | TC: OPEN UE TEST LOOP | - | - |
| 67 | The UE sends OPEN UE TEST LOOP COMPLETE message | --> | TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 68 | The SS sends CLOSE UE TEST LOOP message | <-- | TC: CLOSE UE TEST LOOP | - | - |
| 69 | The UE sends CLOSE UE TEST LOOP COMPLETE message | --> | TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 70 | SS transmits an ESM DATA TRANSPORT message containing downlink user data of 1 octet. | <-- | RRC: DLInformationTransfer-NB ESM DATA TRANSPORT | - | - |
| 71 | The SS releases the RRC connection including PUR-Config with pur-TimeAlignmentTimer > IP PDU delay. (Note 2) | <-- | RRC: RRCConnectionRelease-NB | - | - |
| 72 | Check: Does the UE send RRCConnectionRequest-NB? | --> | RRC: RRCConnectionRequest-NB | 10 | P |
| 73 | The SS sends RRCConnectionSetup-NB | <-- | RRC: RRCConnectionSetup-NB | - | - |
| 74 | The UE sends RRCConnectionSetupComplete-NB message including CONTROL PLANE SERVICE REQUEST and ESM DATA TRANSPORT containing the data received in Step 70. | --> | RRC: RRCConnectionSetupComplete-NB NAS: CONTROL PLANE SERVICE REQUEST NAS: ESM DATA TRANSPORT | - | - |
| 75 | The SS sends OPEN UE TEST LOOP message | <-- | TC: OPEN UE TEST LOOP | - | - |
| 76 | The UE sends OPEN UE TEST LOOP COMPLETE message | --> | TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 77 | The SS sends CLOSE UE TEST LOOP message | <-- | TC: CLOSE UE TEST LOOP | - | - |
| 78 | The UE sends CLOSE UE TEST LOOP COMPLETE message | --> | TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 79 | SS transmits an ESM DATA TRANSPORT message containing downlink user data of 1 octet. | <-- | RRC: DLInformationTransfer-NB ESM DATA TRANSPORT | - | - |
| 80 | The SS releases the RRC connection including PUR-Config withpur-TimeAlignmentTimer < IP PDU delay. (Note 2) | <-- | RRC: RRCConnectionRelease-NB | - | - |
| 81 | SS adjusts Ncell levels according to row T3 of table 22.1.5.3.2-1. | - | - | - | - |  |
| - | Unless otherwise stated, all below messages are sent and received on Ncell2. (Note 4) | - | - | - | - |  |
| 82 | Check : Does the UE send RRCConnectionRequest-NB and does not attempt to send RRCEarlyDataRequest-NB using PUR on Ncell2? | --> | RRC: RRCConnectionRequest-NB | 11 | P |
| 83 | The SS sends RRCConnectionSetup-NB. | <-- | RRC: RRCConnectionSetup-NB | - | - |
| 84 | The UE sends RRCConnectionSetupComplete-NB message including CONTROL PLANE SERVICE REQUEST and ESM DATA TRANSPORT containing the data received in Step 79 on Ncell2. | --> | RRC: RRCConnectionSetupComplete-NB NAS: CONTROL PLANE SERVICE REQUEST NAS: ESM DATA TRANSPORT | - | - |
| 85 | The SS sends OPEN UE TEST LOOP message | <-- | TC: OPEN UE TEST LOOP | - | - |
| 86 | The UE sends OPEN UE TEST LOOP COMPLETE message | --> | TC: OPEN UE TEST LOOP COMPLETE | - | - |
| 87 | The SS sends CLOSE UE TEST LOOP message | <-- | TC: CLOSE UE TEST LOOP | - | - |
| 88 | The UE sends CLOSE UE TEST LOOP COMPLETE message | --> | TC: CLOSE UE TEST LOOP COMPLETE | - | - |
| 89 | SS transmits an ESM DATA TRANSPORT message containing downlink user data of 1 octet. | <-- | RRC: DLInformationTransfer-NB ESM DATA TRANSPORT | - | - |
| 90 | The SS releases the RRC connection including PUR-Config with pur-TimeAlignmentTimer > IP PDU delay. (Note 2) | <-- | RRC: RRCConnectionRelease-NB | - | - |
| 91 | SS adjusts SystemInformationBlockType2-NB to indicate the cp-PUR-EPC-r16 being set to omit and notifies the UE about the change of System Information | <-- | Paging-NB | - | - |
| 92 | Check : Does the UE initiate legacy RRC connection establishment procedure sending RRCConnectionRequest-NB and does not attempt to send RRCEarlyDataRequest-NB using PUR? | --> | RRC: RRCConnectionRequest-NB | 5 | P |
| 93 | The SS sends RRCConnectionSetup-NB | <-- | RRC: RRCConnectionSetup-NB | - | - |
| 94 | The UE sends RRCConnectionSetupComplete-NB message including CONTROL PLANE SERVICE REQUEST and ESM DATA TRANSPORT containing the data received in Step 89. | --> | RRC: RRCConnectionSetupComplete-NB NAS: CONTROL PLANE SERVICE REQUEST NAS: ESM DATA TRANSPORT | - | - |
| Note 1: pur-TimeAlignmentTimer = pur-TimeAlignmentTimer-r16 * periodicityAndOffset-r16 * H-SFN duration (10.24s) = 81.92s Note 2: pur-TimeAlignmentTimer = pur-TimeAlignmentTimer-r16 * periodicityAndOffset-r16 * H-SFN duration (10.24s) = 655.36s Note 3: Considering pur-TimeAlignementTimer being reset at step 15 with a value of 81.92s and IP PDU delay of 81 s, the pur-TimeAlignementTimer should expire shortly after the corresponding transmission of the IP PDU. Note 4: Ncell 2 has locally configured PUR, same as the one configured for Ncell 1 in Step 80. |  |  |  |  |  |

Table 22.1.5.3.2-3: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |  |
| 1 | The UE sends PURConfigurationRequest | --> | RRC: PURConfigurationRequest | - | - |

22.1.5.3.3	Specific message contents

Table 22.1.5.3.3-1: SystemInformationBlockType2-NB for NCell1 (preamble and all steps of Table 22.1.5.3.2-1)

| Derivation Path: TS 36.508[18] Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| cp-PUR-EPC-r16 | true |  |  |
| } |  |  |  |

Table 22.1.5.3.3-2: CLOSE UE TEST LOOP (preamble and steps 8, 18, 30, 39, 48, 58, 68 and 77, Table 22.1.5.3.2-1)

| Derivation path: TS 36.508[18] Table 8.1.5.2B.4-1, Condition: TL_MODE_G, return_via_EMM_SMC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| IP PDU delay | '00000010'B | T_delay_modeG timer=2 sec |  |
|  | ‘01010001’B | T_delay_modeG timer=81 sec | Step 18 |
|  | '01011010'B | T_delay_modeG timer=90 sec | Step 39, 68 |

Table 22.1.5.3.3-3: RRCConnectionRelease-NB (steps 2, 11, 21, 33, 42, 51, 61, 71 and 80, Table 22.1.5.3.2-1)

| Derivation path: TS 36.508[18] Table 8.1.6.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCConnectionRelease-NB ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| rrcConnectionRelease-r13 SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionRelease-NB-v1430-IEs |  |
| redirectedCarrierInfo-v1430 | Not present |  |  |
| extendedWaitTime-CPdata-r14 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionRelease-NB-v1530-IEs |  |
| drb-ContinueROHC-r15 | Not present |  |  |
| nextHopChainingCount-r15 | Not present |  |  |
| nonCriticalExtension | Not present |  | Step 21, 61 |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionRelease-NB-v1550-IEs | Step 2, 11, 33, 42, 51, 71 and 80 |
| redirectedCarrierInfo-v1550 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionRelease-NB-v15b0-IEs |  |
| noLastCellUpdate-r15 | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  | RRCConnectionRelease-NB-v1610-IEs |  |
| resumeIdentity-r16 | Not present |  |  |
| anr-MeasConfig-r16 | Not present |  |  |
| pur-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  | PUR-Config-NB-r16 |  |
| pur-ConfigID-r16 | 000000000000000000 1’B |  |  |
| pur-TimeAlignmentTimer-r16 | 8 |  |  |
|  | 1 |  | Step 11, 42 |
| pur-NRSRP-ChangeThreshold-r16 | Not present |  |  |
| pur-NRSRP-ChangeThreshold-r16 SetupRelease ::= SEQUENCE { |  |  | Step 51 |
| increaseThresh-r16 | dB8 |  |  |
| decreaseThresh-r16 | dB8 |  |  |
| } |  |  |  |  |  |  |  |
| pur-ImplicitReleaseAfter-r16 |  | Not present |  |  |  |  |  |
|  |  | n2 |  |  |  | Step 71 |  |
| pur-RNTI-r16 | ’0001 0001 0001 0001’B |  |  |
| pur-ResponseWindowTimer-r16 | pp32 |  |  |
| pur-StartTimeParameters-r16 SEQUENCE { |  |  |  |
| periodicityAndOffset-r16 CHOICE { |  |  |  |
| periodicity8 | 1 |  |  |
| } |  |  |  |
| startSFN-r16 | 0 |  |  |
| startSubframe-r16 | 0 |  |  |
| hsfn-LSB-Info-r16 | 0 |  |  |
| } |  |  |  |
| pur-NumOccasions-r16 | Infinite |  |  |
| pur-PhysicalConfig-r16 SEQUENCE { |  |  |  |
| carrierConfig-r16 SEQUENCE { |  |  |  |
| dl-CarrierConfig-r13 SEQUENCE { |  |  |  |
| dl-CarrierFreq-r13 | Downlink EARFCN of NCell 1 |  |  |
| downlinkBitmapNonAnchor-r13 | Not present |  |  |
| dl-GapNonAnchor-r13 | Not present |  |  |
| inbandCarrierInfo-r13 | Not present |  |  |
| nrs-PowerOffsetNonAnchor-v1330 | Not present |  |  |
| dl-GapNonAnchor-v1530 | Not present |  |  |
| dl-CarrierFreq-v1550 | Not present |  |  |
| } |  |  |  |
| ul-CarrierConfig-r13 SEQUENCE { |  |  |  |
| ul-CarrierFreq-r13 | Uplink EARFCN of NCell 1 |  |  |
| tdd-UL-DL-AlignmentOffset-r15 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| npusch-NumRUsIndex-r16 | 0 |  |  |
| npusch-NumRepetitionsIndex-r16 | 0 |  |  |
| npusch-SubCarrierSetIndex-r16 CHOICE { |  |  |  |
| khz3dot75 | 0 |  |  |
| } |  |  |  |
| npusch-MCS-r16 CHOICE { |  |  |  |
| singleTone | 1 |  |  |
| } |  |  |  |
| p0-UE-NPUSCH-r16 | 0 |  |  |
| alpha-r16 | al0 |  |  |
| npusch-CyclicShift-r16 | p0 |  |  |
| npdcch-Config-r16 | Table 8.1.6.3-3 in TS 36.508[18]. | NPDCCH-ConfigDedicated-NB-DEFAULT |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.1.5.3.3-4: PURConfigurationRequest-NB (parallel behaviour, Table 22.1.5.3.2-3)

| Derivation path: TS 36.508[18] Table 8.1.6.1-2a |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| PURConfigurationRequest-NB-r16 ::= SEQUENCE { |  |  |  |
| pur-ConfigRequest-r16 CHOICE { |  |  |  |
| pur-SetupRequest | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.1.5.3.3-5: RRCEarlyDataRequest-NB (steps 3, 13, 23, 25 and 34, Table 22.1.5.3.2-1)

| Derivation Path: TS 36.508[18] Table 8.1.6.1-15ab |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCEarlyDataRequest-NB-r15 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcEarlyDataRequest-r15 SEQUENCE { |  |  |  |
| s-TMSI-r15 | Set to the value of the S-TMSI of the UE |  |  |
| establishmentCause-r15 | mo-Data |  |  |
| dedicatedInfoNAS-r15 | CONTROL PLANE SERVICE REQUEST | Table 22.1.5.3.3-6 |  |
| nonCriticalExtension | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 22.1.5.3.3-6: CONTROL PLANE SERVICE REQUEST (steps 3, 13, 23, 25, 34, 36, 45, 55, 65, 74 and 84, Table 22.1.5.3.3-2)

| Derivation Path: TS 36.508[18] Table 4.7.2-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Control plane service type |  |  |  |
| Control plane service type value | Any allowed value |  |  |
| ESM message container | ESM DATA TRANSPORT |  |  |
| NAS message container | Not present |  |  |

Table 22.1.5.3.3-7: SystemInformationBlockType2-NB for NCell2 (preamble and step 91,Table 22.1.5.3.2-1)

| Derivation Path: TS 36.508[18] Table 8.1.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SystemInformationBlockType2-NB-r13 ::= SEQUENCE { |  |  |  |
| cp-PUR-EPC-r16 | true |  |  |
|  | Not present |  | Step 91 |
| } |  |  |  |