3GPP TSG-RAN5 Meeting #108	R5-254138r1

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4034 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updated the Test case 14.1.11 in TS 38.533 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN4 dependent R4-2510637 has introduced some updates of reselection in A.14.1.11: - Removing the entire time interval T3 for LTE to NR NTN reselection is not supported in Rel-18. - Updating test configuration. Therefore, RAN5 need to align with RAN4 core spec updates. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updating the Editor’s Note. - Removing the entire time interval T3 in Table 14.1.11.4.1-3 and Test procedure - Updating test configuration in Table 14.1.11.4.1-1, Table 14.1.11.4.1-3 Table 14.1.11.5-1 and Table 14.1.11.5-2. - Updating test procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case can’t be tested correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.1.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1：Correcting the annex E reference in Table 14.1.11.4.1-2. |  |  |  |  |  |  |  |  |

<< Start of Changes >>

14.1.11	NR SA FR1 inter-RAT Cell reselection with NTN Carrier for Satellite Access

Editor’s Note:

- MU and TT analysis is incomplete.

14.1.11.1	Test purpose

This test is to verify the requirement for the NR NTN to E-UTRAN TN inter-RAT cell reselection requirements specified in TS 38.133 [6] clause 4.2C.2.11 when the E-UTRAN cell is of higher priority.

14.1.11.2	Test applicability

This test applies to all types of NR UE release 18 and forward supporting satellite access.

14.1.11.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.1.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.14.1.11.

14.1.11.4	Test description

The test scenario comprises of one NR cell and one E-UTRAN cell as given in Tables 14.1.11.4.1-1, 14.1.11.4.1-3, 14.1.11.5-1 and 14.1.11.5-2. The test consists of two successive time periods, with time duration of T1 and T2 respectively. NR Cell 1 is already identified by the UE prior to the start of the test. E-UTRAN Cell 2 is of higher priority than Cell 1.

14.1.11.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.1.11.4.1-1.

Table 14.1.11.4.1-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | LTE 10 MHz bandwidth, TDD duplex mode |
| 2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | LTE 10 MHz bandwidth, TDD duplex mode |
| 3 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | LTE 10 MHz bandwidth, FDD duplex mode |
| 4 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | LTE 10 MHz bandwidth, FDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations. NOTE 2: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. |  |  |

Configure the test equipment and the DUT according to the parameters in Table 14.1.11.4.1-2.

Table 14.1.11.4.1-2: Initial conditions for NR NTN to E-UTRAN cell re-selection

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.12-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.1.11.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.6.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 14.1.11.4.1-3.

2.	Message contents are defined in clause 14.1.11.4.3.

3.	The test scenario comprises of one NR NTN cell and one E-UTRAN cell. Cell 1 is the NR NTN cell and Cell 2 is the E-UTRA neighbour cell. Cell 1 is configured according to Annex C.1.1 and C.1.2, Cell 2 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

4.	The initial test environment conditions are setup according to section 14.0.5.

Table 14.1.11.4.1-3:  General test parameters for NR NTN to E-UTRAN cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1-4 | Cell 1 | The UE camps on Cell 1 in the initial phase and during T2 period the UE reselects to Cell 2. |
| T2 end | Active cell |  | 1-4 | Cell 2 | The UE shall perform reselection to cell 2 during T2. |
| condition | Neighbour cell |  | 1-4 | Cell 1 |  |
| T3 end | Active cell |  | 1-4 | Cell 1 | The UE shall perform reselection to cell 1 during T3 for iteration of the tests. |
| condition | Neighbour cell |  | 1-4 | Cell 2 |  |
| Access Barring Information |  | - | 1-4 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1-4 | 1.28 | The value shall be used for all cells in the test. |
| NR PRACH configuration index |  |  | 1-4 | 102 | The detailed configuration is specified in TS 38.211 [6] clause 6.3.3.2 |
| E-UTRAN PRACH configuration index |  |  | 1-4 | 53 | As specified in Table 5.7.1-2 in TS 36.211 [23] |
| E-UTRAN PRACH |  |  | 1-4 | 53 | As specified in Table 5.7.1-2 in |
| T1 |  | s | 1-4 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2. |
| T2 |  | s | 1-4 | 70 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

14.1.11.4.2	Test procedure

Two cells are deployed in the test, which are one FR1 NR NTN Cell (Cell 1) and an E-UTRA TN neighbour cell (Cell 2). The test consists of two successive time periods, with time duration of T1 and T2 respectively. NR cell 1 is already identified by the UE prior to the start of the test. E-UTRAN cell 2 is of higher priority than cell 1.

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRC CONNECTION REQUEST message to perform a Tracking Area Update procedure.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR with Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 1.

2.	Set the parameters according to T1 in Table 14.1.11.5-1 and 14.1.11.5-2. T1 starts.

3.	During T1, Cell 2 shall be powered off and set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for one iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.1.11.5-1 and 14.1.11.5-2. T2 starts.

5.	The SS waits for random access requests information from the UE to perform cell re-selection to a higher priority cell, Cell 2.

6.	If the UE responds on Cell 2 during time duration T2 within 68 seconds from the beginning of time period T2, then the number of successful tests is increased by one. Otherwise, the number of failure tests is increased by one.

7.	The TE shall switch off and on the UE and set the parameters according to T1 in Table 14.1.11.5-1 and 14.1.11.5-2

.

8.	Ensure the UE is in state RRC_IDLE with generic procedure parameters connectivity NR according to TS 38.508-1 [14] clause 4.5.

9.	Repeat step 2-8 until a test verdict has been achieved.

14.1.11.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 14.1.11.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.3-1 Table H.2.3-2 with Condition SMTC 2 and higher priority Table H.2.3-3 with Condition higher priority |
| Default RRC messages and information elements contents exceptions |  |

14.1.11.5	Test requirement

Tables 14.1.11.5-1 and 14.1.11.5-2 define the primary level settings including test tolerances for higher priority E-UTRA cell re-selection test case.

Table 14.1.11.5-1: Cell specific test parameters for NR Cell 1

| Parameter | Unit | Test configuration | Cell 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 |
| Satellite information |  | 1,3 | SSC.1 |  |
|  |  | 2,4 | SSC.2 |  |
| TDD configuration |  | 1-4 | N/A |  |
| PDSCH parameters |  | 1-4 | SR.1.1 FDD |  |
| RMSI CORESET parameters |  | 1-4 | CR.1.1 FDD |  |
| Dedicated CORESET parameters |  | 1-4 | CCR.1.1 FDD |  |
| SSB parameters |  | 1-4 | SSB.1 FR1 |  |
| NR SMTC parameters |  | 1-4 | SMTC.2 |  |
| OCNG Pattern |  | 1-4 | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1-4 | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1-4 | ULBWP.0.1 |  |
| RLM-RS |  | 1-4 | SSB |  |
| Qrxlevmin | dBm/SCS | 1-4 | -140 |  |
|  | dBm/SCS | 1-4 | -98 |  |
|  | dBm/15 kHz | 1-4 | -98 |  |
| SS-RSRP | dBm/SCS | 1-4 | -84 | -84 |
|  | dB | 1-4 | 14+TT | 14+TT |
|  | dB | 1-4 | 14+TT | 14+TT |
| Io | dBm/9.36 MHz | 1-4 | -55.88+TT | -55.88+TT |
| Treselection | s | 1-4 | 0+TT |  |
| SnonintrasearchP | dB | 1-4 | 50+TT |  |
| Threshx, highP (Note 2) | dB | 1-4 | 48+TT |  |
| Threshserving, lowP | dB | 1-4 | 44+TT |  |
| Threshx, lowP | dB | 1-4 | 50+TT |  |
| Propagation Condition |  | 1-4 | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: This refers to the value of Threshx, high which is included in NR system information, and is a threshold for the E-UTRA target cell |  |  |  |  |

Table 14.1.11.5-2: Cell specific test parameters for E-UTRA Cell 2

| Parameter | Unit | Cell 2 |  |
| --- | --- | --- | --- |
|  |  | T1 | T2 |
| E-UTRA RF Channel number |  | 1 |  |
| BWchannel | MHz | 10 |  |
| OCNG Patterns defined in TS 36.133 [15] clause A.3.2 |  | OP.2 TDD for test configuration 1, 2 OP.2 FDD for test configuration 3, 4 |  |
| PBCH_RA | dB | 0 |  |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote 1 | dB |  |  |
| OCNG_RBNote 1 | dB |  |  |
| Qrxlevmin | dBm | -140 |  |
|  | dBm/15 kHz | -98 |  |
| RSRP | dBm/15 KHz | -infinity | -86 |
|  | dB | -infinity | 12+TT |
|  | dB | -infinity | 12+TT |
| TreselectionEUTRAN | s | 0+TT |  |
| SnonintrasearchP | dB | Not sent |  |
| Threshx, highP | dB | 48+TT |  |
| Threshserving, lowP | dB | 44+TT |  |
| Threshx, lowP (Note 2) | dB | 50+TT |  |
| Propagation Condition |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: This refers to the value of Threshx, Low which is included in E-UTRA system information, and is a threshold for the NR target cell |  |  |  |

The cell reselection delay to a higher priority E-UTRAN cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to a higher priority cell shall be less than 68 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90 %.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, E-UTRAN + TSI-E-UTRA,

Where:

Thigher_priority_search	See clause  4.2C.2.9

Tevaluate, NR_inter	See clause 4.2C.2.11

TSI-E-UTRA	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 67.68 s, allow 68 s for the cell re-selection delay to a higher priority E-UTRAN cell.

<<End of changes>>