3GPP TSG RAN WG5 Meeting #108	R5-254193r1

# Bangalore, India, 25-29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4041 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for TT analysis for RRM TC 6.5.12.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-8-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Update TT analysis for RRM TC 6.5.12.1 6.5.12.0 title is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated TT analysis results Updated Cell configuration mapping and Measurement of RRM requirements Updated title of 6.5.12.0 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC cannot complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.12.1 E.4, F.1.1.2,F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: added the missing F.1.3.2 part |  |  |  |  |  |  |  |  |

<Start of change 1>

6.5.12	Subsequent conditional PSCell addition/change

6.5.12.0	Minimum conformance requirements

6.5.12.0.1	Minimum conformance requirements for Intra-frequency subsequent CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC

<End of change 1>

<Start of change 2>

6.5.12.1	Intra-frequency subsequent CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC

Editor’s Note: This test case is incomplete in following aspects:

- Message contents is TBD.

- Test applicability is missing

6.5.12.1.1	Test purpose

The purpose of this test is to verify that the subsequent conditional NR PSCell change under NR-DC is within the requirements stated in clause 8.11E.2 of TS 38.133.

6.5.12.1.2	Test applicability

This test applies to all types of NR UE release 18 onwards.

6.5.12.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.12.0.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.12.1

6.5.12.1.4	Test description

6.5.12.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.5.12.1.4.1-1.

Table 6.5.12.1.4.1-1: Supported test configurations for Intra-frequency CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC

| Configuration | Description |
| --- | --- |
| 1 | PCell: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode. PSCell: FDD |
| 2 | PCell: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode. PSCell: FDD |
| 3 | PCell: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode. PSCell: FDD |
| 4 | PCell: 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode. PSCell: TDD |
| 5 | PCell: 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode. PSCell: TDD |
| 6 | PCell: 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode. PSCell: TDD |
| NOTE: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 6.5.12.1.4.1-2.

Table 6.5.12.1.4.1-2: Initial conditions for DL interruptions at switching between two uplink bands with two transmit antenna connectors in FDD-TDD CA

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.12.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2 |
| Connection Diagram | TE Part | A.3.1.8.2a | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | - For 4Rx capable UEs without any 2Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE part. - Without LTE link - Without Faders |  |  |

1. The general test parameter settings are set up according to Table 6.5.12.1.4.1-3.

2. Message contents are defined in clause 6.5.12.1.4.3.

3. The test scenario comprises of three NR Cells. (Cell 1, Cell 2 and Cell 3). Cell 1, Cell 2 and Cell 3 are configured according to Annex C.1.2 and C.1.3.

Table 6.5.12.1.4.1-3: General Test Parameters for Intra-frequency CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  |  | 1, 2 | Two radio channels are used for this test. One for PCell and second for NR PSCell |
| Initial | Active PCell |  | Cell 1 | PCell on RF channel number 1. |
|  | Neighbour cell |  | Cell2, Cell 3 | Neighbour cells on RF channel number 2. |
| Final | Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Condition | active PSCell |  | Cell 3 | PSCell on RF channel number 2. |
| A4 | Hysteresis | dB | 0 | Used to trigger conditional PSCell addition of Cell 2 |
|  | Threshold RSRP | dBm | -118 |  |
|  | Time to Trigger | s | 0 |  |
| A3 |  | dB | 0 | Used to trigger conditional PSCell change from Cell 2 to Cell 3 |
| DRX |  |  | OFF | Continuous monitoring of primary cell |
| Measurement gap pattern Id |  |  | 0 | Gaps are configured during T1, T2 and released upon T3 starts. |
| PRACH configuration on cell2 and cell3 |  |  | FR1 PRACH configuration 1 | Captured in clause A.3.8.2.1 |
| Cell-individual offset for cells on RF channel number 1 |  | dB | 0 | Individual offset for cells on primary component carrier. |
| Cell-individual offset for cells on RF channel number 2 |  | dB | 0 | Individual offset for cells on carrier frequency of cell2. |
| T1 |  | s | 1 | During this time the PCell is known and Cell 2 is unknown. |
| T2 |  | s | 5 | During this time Cell 2 meets the PSCell addition condition and UE adds this PSCell. |
| T3 |  | s | 5 | During this time Cell 3 meets the PSCell change condition and UE sends PRACH to Cell 3. |

6.5.12.1.4.2	Test procedure

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	The SS transmits a SN RRCReconfiguration message to the UE to configure Cell 2 and Cell 3 as SCells according to Table 6.5.12.4.5-1.

3.	The UE shall transmit an RRCReconfigurationComplete message.

4.	The SS activates Cell 2 and Cell 3

5.	Set the parameters according to T1 in Table 6.5.12.4.4.1-3 and Table 6.5.12.4.5-1. Propagation conditions are set according to Annex C clauses C.2.2. T1 starts.

6.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 6.5.12.4.5-1. T2 starts.

7. When T2 expires, the SS shall switch the power settings at T2 to power setting at T3 as per Tables 6.5.12.4.5-1 and 6.5.12.4.5-2. T3 starts.

8.	If

a)  The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_DU + 2 ms = 920+10+62 ms=992 ms from the start of T2.

b)	The UE shall start to transmit the PRACH to Cell 3 less than TEvent_DU + Tmeasure + TUE_preparation + processing + T∆ + TPSCell_DU + 2 ms = 0+920+10+62 ms=992 ms from the start of T3.

and

c)	no longer than X consecutive ACK/NACK DTXs are observed by the SS from the start of T2 to the instant the UE transmits the first PRACH preamble, where

-	X = Tinterrupt +k1 for test configuration 1 in Table 6.5.3.12.4.1-1

-	X = Tinterrupt for test configuration 2 in Table 6.5.3.12.4.1-1

-	X = 2∙Tinterrupt for test configuration 3 in Table 6.5.3.12.4.1-1.

9.	When T3 expires the UE shall be capable to transmit PRACH preamble towards the Cell3.

10.	If UE is not in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 on Cell 1, switch off and on the UE. Then ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On according to TS 38.508-1 [14] clause 4.5 on Cell 1.

11.	Repeat steps 2-10 until the confidence is achieved.

6.5.12.1.4.3	Message contents

TBD

6.5.12.1.5	Test requirements

Table A.6.5.12.1.2-3: Cell Specific Parameters for Intra-frequency CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC

| Parameter | Unit | Config | Cell 2 |  |  |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 |  | T2 |  | T3 | T1 | T2 | T3 |
| NR RF Channel Number |  | 1,2,3,4,5,6 | 1 |  |  |  |  | 1 |  |  |
| NR RF Channel Number |  | 1,2,3,4,5,6 | 1 |  |  |  |  | 1 |  |  |
| TDD |  | 1,4 | Not Applicable |  |  |  |  | Not Applicable |  |  |
| configuration |  | 2,5 | TDDConf.1.1 |  |  |  |  | TDDConf.1.1 |  |  |
|  |  | 3,6 | TDDConf.2.1 |  |  |  |  | TDDConf.2.1 |  |  |
| BWchannel | MHz | 1,4 | 10: NPRB,c = 52 |  |  |  |  | 10: NPRB,c = 52 |  |  |
|  |  | 2,5 | 10: NPRB,c = 52 |  |  |  |  | 10: NPRB,c = 52 |  |  |
|  |  | 3,6 | 40: NPRB,c = 106 |  |  |  |  | 40: NPRB,c = 106 |  |  |
| Initial BWP Configuration |  | 1,2,3 | DLBWP.0.1 ULBWP.0.1 |  |  |  |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP Configuration |  | 1,2,3 | DLBWP.1.1 ULBWP.1.1 |  |  |  |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| PDSCH Reference |  | 1,4 | SR.1.1 FDD |  |  |  |  | SR.1.1 FDD |  |  |
| measurement |  | 2,5 | SR.1.1 TDD |  |  |  |  | SR.1.1 TDD |  |  |
| channel |  | 3,6 | SR.2.1 TDD |  |  |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET Reference |  | 1,4 | CR.1.1 FDD |  |  |  |  | CR.1.1 FDD |  |  |
| Channel |  | 2,5 | CR.1.1 TDD |  |  |  |  | CR.1.1 TDD |  |  |
|  |  | 3,6 | CR.2.1 TDD |  |  |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET Reference |  | 1,4 | CCR.1.1 FDD |  |  |  |  | CCR.1.1 FDD |  |  |
| Channel |  | 2,5 | CCR.1.1 TDD |  |  |  |  | CCR.1.1 TDD |  |  |
|  |  | 3,6 | CCR.2.1 TDD |  |  |  |  | CCR.2.1 TDD |  |  |
| OCNG Patterns |  | 1,2,3,4,5,6 | OP.1 |  |  |  |  | OP.1 |  |  |
| SSB configuration |  | 1,2,4,5 | SSB.1 FR1 |  |  |  |  | SSB.1 FR1 |  |  |
|  |  | 3,6 | SSB.2 FR1 |  |  |  |  | SSB.2 FR1 |  |  |
| SMTC configuration |  | 1,2,4,5 | SMTC.1 |  |  |  |  | SMTC.1 |  |  |
|  |  | 3,6 | SMTC.1 |  |  |  |  | SMTC.1 |  |  |
| TRS Configuration |  | 1,4 | TRS.1.1 FDD |  |  |  |  | TRS.1.1 FDD |  |  |
|  |  | 2,5 | TRS.1.1 TDD |  |  |  |  | TRS.1.1 TDD |  |  |
|  |  | 3,6 | TRS.1.2 TDD |  |  |  |  | TRS.1.2 TDD |  |  |
| CSI-RS configuration for CSI reporting |  | 1,4 | CSI-RS.1.1 FDD |  |  |  |  | CSI-RS.1.1 FDD |  |  |
|  |  | 2,5 | CSI-RS.1.1 TDD |  |  |  |  | CSI-RS.1.1 TDD |  |  |
|  |  | 3,6 | CSI-RS.2.1 TDD |  |  |  |  | CSI-RS.2.1 TDD |  |  |
| reportConfigType |  | 1,2,3,4,5,6 | Periodic |  |  |  |  | periodic |  |  |
| reportQuantity |  | 1,2,3,4,5,6 | cri-RI-PMI-CQI |  |  |  |  | cri-RI-PMI-CQI |  |  |
| CSI reporting periodicity | slot | 1,2,4,5 | 5 |  |  |  |  | 5 |  |  |
|  |  | 3,6 | 10 |  |  |  |  | 10 |  |  |
| CSI reporting offset | slot | 1,2,4,5 | 2 |  |  |  |  | 2 |  |  |
|  |  | 3,6 | 4 |  |  |  |  | 4 |  |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 1,2,3,4,5,6 | 0 |  |  |  |  | 0 |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1,2,3,4,5,6 | -98 |  |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1,2,4,5 | -98 |  |  |  |  |  |  |  |
|  |  | 3,6 | -95 |  |  |  |  |  |  |  |
|  |  | 1,2,3,4,5,6 | -infinity | 8 |  | -3.3 |  | -infinity | -infinity | 1.36 |
|  |  | 1,2,3,4,5,6 | -infinity | 8 |  | 8 |  | -infinity | -infinity | 10 |
| SS-RSRPNote3 | dBm/SCS | 1,2,4,5 | -infinity | -90 |  | -90 |  | -infinity | -infinity | -88 |
|  |  | 3,6 | -infinity | -87 |  | -87 |  | -infinity | -infinity | -85 |
| IoNote3 | dBm/9.36 MHz | 1,2,4,5 | -70.05 | -61.41 |  | -57.06 |  | -70.05 | -61.41 | -58.06 |
|  | dBm/38.1 MHz | 3,6 | -63.94 | -55.31 |  | -50.96 |  | -63.94 | -55.31 | -51.96 |
| Propagation condition |  | 1,2,3,4,5,6 | AWGN |  |  |  |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |

TRRC_delay + TEvent_DU for PSCell addition (Cell 2) occurs during T1 as the addition condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_DU + 2 ms = 920+10+62 ms=992 ms from the start of T2.

The UE shall start to transmit the PRACH to Cell 3 less than TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_DU + 2 ms = 0+920+10+62 ms=992 ms from the start of T3.

All of the above test requirements shall be fulfilled in order for the observed conditional PSCell addition and release delay to be counted as correct. The rate of correct events observed during repeated tests shall be at least 90 %.

<end of change 2>

<skip unchange part>

<start of change 3>

E.4	Cell configuration mapping for SA FR1 test cases in Chapter 6

Table E.4-1 defines the cell configuration mapping for NR/5GC FR1 test cases in chapter 6 of this test specification.

Table E.4-1: Cell configuration mapping for SA FR1 RRM testing

| TC | Description | 38.533 NR Cell1 | 38.533 NR Cell2 | 38.533 NR Cell3 | 38.533 NR Cell4 | CA Type |
| --- | --- | --- | --- | --- | --- | --- |
| 6.1.1.1 | NR SA FR1 cell re-selection | NR Cell 1 | NR Cell 11 |  |  |  |
| <skip unchange part> |  |  |  |  |  |  |
| 6.5.8.1 | UE specific CBW change on PCell in FR1 in non-DRX | NR Cell 1 |  |  |  |  |
| 6.5.12.1 | Intra-frequency subsequent CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC | NR Cell 1 | NR Cell 2 | NR Cell 4 |  |  |
| 6.6.1.1 | SA event triggered reporting tests without gap under non-DRX | NR Cell 1 | NR Cell 2 |  |  |  |

<end of change 3>

<skip unchange part>

<start of change 4>

F.1	Measurement uncertainties and test tolerances for FR1 and FR2

<skip unchange part>

F.1.1.2	Measurement of RRM requirements

<skip unchange part>

Table F.1.1.2-4: Maximum test system uncertainty for RRM requirements for SA FR1 test cases

| Subclause | Maximum Test System Uncertainty1 | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.1.1.1 NR SA FR1 cell re-selection | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |
| <skip unchange part> |  |  |
| 6.5.8.1 UE specific CBW change on PCell in FR1 in non-DRX | Test System uncertainty on Cell1: ±0.6 dB | Overall system uncertainty for AWGN conditions comprises two quantities: 1. Signal-to-noise ratio uncertainty: ±0.3 dB, 2. Effect of AWGN flatness and signal flatness: ±2.0 dB. Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (Signal-to-noise ratio uncertainty^2 + (0.25 x AWGN flatness and signal flatness)^2) = 0.58dB, round up to 0.6dB. |
| 6.5.12.1 Intra-frequency subsequent CPC from FR1-FR1 NR-DC to FR1-FR1 NR-DC | Noc2 ±1.5 dB Noc3 ±1.5 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB | Note: Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Ês3 / Noc3 is the ratio of cell 3 signal / AWGN |
| 6.6.1.1 SA event triggered reporting tests without gap under non-DRX | Noc ±1.5 dB Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB | Note: Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN |

<end of change 4>

F.1.3.2	Measurement of RRM requirements

<skip unchanged part>

Table F.1.3.2-2: Derivation of test requirements for NR SA FR1 RRM tests

| Test | Minimum requirement in TS 38.133 [6] | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- |
| 6.1.1.1 NR SA FR1 cell re-selection | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: +13dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0.45dB During T3: 0dB 0.45dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +16dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +13dB Ês2 / Noc: +16.45dB During T3: Noc: -98dBm/15kHz Ês1 / Noc: +16.45dB Ês2 / Noc: +13dB |
| <skip unchanged part> |  |  |  |
| 6.5.7D.2 NR SA FR1 DL interruptions at switching across four uplink bands in FDD-TDD CA for single TAG | Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Noc4: -104dBm/15kHz Ês1 / Noc: +17dB Ês2 / Noc: +17dB Ês3 / Noc: +14dB Ês3 / Noc: +14dB Reported CSI-RSRP values ± 10dB | 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB Via mapping | Noc1: -104dBm/15kHz Noc2: -104dBm/15kHz Noc3: -104dBm/15kHz Noc4: -104dBm/15kHz Ês1 / Noc: +17dB Ês2 / Noc: +17dB Ês3 / Noc: +14dB Ês3 / Noc: +14dB CSI-RS#0: RSRP_65 to RSRP_86 |
| 6.5.7D.3 NR SA FR1 DL interruptions at switching across three uplink bands in FDD-TDD CA for two TAGs | Same as 6.5.7B.1 | Same as 6.5.7B.1 | Same as 6.5.7B.1 |
| 6.5.7D.4 NR SA FR1 DL interruptions at switching across four uplink bands in TDD-TDD CA for two TAGs | Same as 6.5.7D.2 | Same as 6.5.7D.2 | Same as 6.5.7D.2 |
| 6.5.8.1 UE specific CBW change on PCell in FR1 in non-DRX | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB | During T1: 0dB 0dB | During T1: Noc1: -104dBm/15kHz Ês1 / Noc1: +17dB |
| 6.5.12.2 Inter-frequency subsequent CPA from FR1-FR1 NR-DC to FR1-FR1 NR-DC | During T2: Noc1: -98dBm/15kHz Ês2 / Noc: +8dB Ês3 / Noc: +3.3dB During T4: Noc1: -98dBm/15kHz Ês2 / Noc: -infinity Ês3 / Noc: +2.36dB | During T2: 0dB 0dB During T4: 0dB 1dB | During T2: Noc1: -98dBm/15kHz Ês2 / Noc: +8dB Ês3 / Noc: +3.3dB During T4: Noc1: -98dBm/15kHz Ês2 / Noc: -infinity Ês3 / Noc: +1.36dB |
| 6.6.1.1 SA event triggered reporting tests without gap under non-DRX | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |

<end of change 4>