3GPP TSG-RAN5 Meeting #108	R5-254203r2

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5100 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 9.3.1.7 related to Inter-system mobility registration update / Rejected / No suitable cells in tracking area / 5GC to EPC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As discussion paper(R5-254208) mentioned, according to TS38.523-1, there is no test case related to UE mobility in inter-system upon receiving the 5GMM cause value #15 in the REGISTRATION REJECT message. So we need to introduce test case related to Inter-system mobility registration update / Rejected / No suitable cells in tracking area / 5GC to EPC for stability of connectivity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new test case 9.3.1.7 related to Inter-system mobility registration update / Rejected / No suitable cells in tracking area / 5GC to EPC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE mobility in inter-system upon receiving the 5GMM cause value #15 in the REGISTRATION REJECT message will not be guaranteed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.7 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/38.523-2 CR 0614 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-254203r1: Correction of the Work item code on the cover sheet, as the original Work item code was incorrect. R5-254203r2: Correction of the font and the typo. Removal of old TP (1) and TP (2). Removal of “and is unable to find a suitable NR cell in another tracking area” in new TP (1) corresponding to old TP (3). Removal of NGC Cell B and E-UTRA Cell A and addtion of E-UTRA Cell B. Removal of old steps 12 to 14. Modification of new step 13 corresponding to old steps 16 to 28 to check that the registration procedure described in TS 36.508 [7] subclause 4.5.2.3 is performed on E-UTRA Cell B and removal of “P” verdict. |  |  |  |  |  |  |  |  |

<Start of modified section>

9.3.1.7	Inter-system mobility registration update / Rejected / No suitable cells in tracking area / 5GC to EPC

9.3.1.7.1	Test Purpose (TP)

(1)

with { the UE has sent initial REGISTRATION REQUEST message }

ensure that {

when { the UE receives a REGISTRATION REJECT with cause #15 (No suitable cells in tracking area) }

then { the UE searches for a suitable cell in another tracking area in another RAT }

}

9.3.1.7.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clauses 5.5.1.2.5 and TS 38.304, clauses 5.2.1. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501, clause 5.5.1.2.5]

If the initial registration request cannot be accepted by the network, the AMF shall send a REGISTRATION REJECT message to the UE including an appropriate 5GMM cause value.

If the initial registration request is rejected due to general NAS level mobility management congestion control, the network shall set the 5GMM cause value to #22 "congestion" and assign a back-off timer T3346.

The UE shall take the following actions depending on the 5GMM cause value received in the REGISTRATION REJECT message.

#3	(Illegal UE); or

#6	(Illegal ME).

….

#15	(No suitable cells in tracking area);

The UE shall set the 5GS update status to 5U3 ROAMING NOT ALLOWED (and shall store it according to subclause 5.1.3.2.2) and shall delete any 5G-GUTI, TAI list and ngKSI. Additionally, the UE shall reset the registration attempt counter.

The UE shall store the current TAI in the list of "5GS forbidden tracking areas for roaming" and enter the state 5GMM-DEREGISTERED.LIMITED-SERVICE. The UE shall search for a suitable cell in another tracking area according to 3GPP TS 38.304 [15].

[TS 38.304, clause 5.2.1]

UE shall perform measurements for cell selection and reselection purposes as specified in TS 38.133 [8].

When evaluating Srxlev and Squal of non-serving cells for reselection evaluation purposes, the UE shall use parameters provided by the serving cell and for the final check on cell selection criterion, the UE shall use parameters provided by the target cell for cell reselection.

The NAS can control the RAT(s) in which the cell selection should be performed, for instance by indicating RAT(s) associated with the selected PLMN, and by maintaining a list of forbidden registration area(s) and a list of equivalent PLMNs. The UE shall select a suitable cell based on RRC_IDLE or RRC_INACTIVE state measurements and cell selection criteria.

In order to expedite the cell selection process, stored information for several RATs, if available, may be used by the UE.

When camped on a cell, the UE shall regularly search for a better cell according to the cell reselection criteria. If a better cell is found, that cell is selected. The change of cell may imply a change of RAT. Details on performance requirements for cell reselection can be found in TS 38.133 [8].

The NAS is informed if the cell selection and reselection result in changes in the received system information relevant for NAS.

For normal service, the UE shall camp on a suitable cell, monitor control channel(s) of that cell so that the UE can:

-	receive system information from the PLMN or SNPN; and

-	receive registration area information from the PLMN or SNPN, e.g., tracking area information; and

-	receive other AS and NAS Information; and

-	if registered:

-	receive paging and notification messages from the PLMN or SNPN; and

-	initiate transfer to Connected mode.

For cell selection in multi-beam operations, measurement quantity of a cell is up to UE implementation.

For cell reselection in multi-beam operations, including inter-RAT reselection from E-UTRA to NR, the measurement quantity of this cell is derived amongst the beams corresponding to the same cell based on SS/PBCH block as follows:

-	if nrofSS-BlocksToAverage (maxRS-IndexCellQual in E-UTRA) is not configured in SIB2/SIB4 (SIB24 in E-UTRA); or

-	if absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA) is not configured in SIB2/SIB4 (SIB24 in E-UTRA); or

-	if the highest beam measurement quantity value is below or equal to absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA):

-	derive a cell measurement quantity as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [11].

-	else:

-	derive a cell measurement quantity as the linear average of the power values of up to nrofSS-BlocksToAverage (maxRS-IndexCellQual in E-UTRA) of highest beam measurement quantity values above absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA).

9.3.1.7.3	Test description

9.3.1.7.3.1	Pre-test conditions

System Simulator:

-	2 cells

-	NGC Cell A (belongs to TAI-1) is configured according to TS 38.508-1 [4] Table 6.3.2.2-1.

-	E-UTRA Cell B (belongs to TAI-2) is configured to TS 36.508 [7] Table 6.3.2.2-1.

UE:

-	None.

Preamble:

-	The UE is in state Switched OFF (State 0-A) as per TS 38.508-1 [4] Table 4.4A.2-0.

-	NGC Cell A is set to “Serving Cell”.

-	E-UTRA Cell B is set to “Non-suitable "Off" cell”.

9.3.1.7.3.2	Test procedure sequence

Table 9.3.1.7.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on | - | - | - | - |
| 2-9 | Steps 2-9 of Table 4.5.2.2-2 in TS 38.508-1 [4] are performed on NGC Cell A. | - | - | - | - |
| 10 | The SS transmits a REGISTRATION REJECT with cause #15 (No suitable cells in tracking area). | <-- | REGISTRATION REJECT | - | - |
| 11 | The SS releases the RRC connection | - | - | - | - |
| 12 | The SS configures - NGC Cell A and E-UTRA Cell B as “Serving Cell” | - | - | - | - |
| 13 | Check: Is the registration procedure described in TS 36.508 [7] subclause 4.5.2.3 performed on E-UTRA Cell B? NOTE: The UE performs registration and the RRC connection is released. | - | - | 1 | - |

9.3.1.7.3.3	Specific message contents

Table 9.3.1.7.3.3-1: REGISTRATION REJECT (step 10 Table 9.3.1.7.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| 5GMM cause | ‘00001111’B | Cause #15(No suitable cells in tracking area) |  |

<End of modified section>