3GPP TSG-RAN5 Meeting #108	R5-254281r2

# Bengaluru, India, 25th Aug 2025 – 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5113 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new NR NTN enh 6.7.1.10 Test case for skipping TN measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM JAPAN LLC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test case 6.7.1.10 needs to be added as per NTN Enh WP |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case 6.7.1.10 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of WP will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.1.10 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.523-2 CR0617, TS/TR 38.508-1 CR3550, CR3530 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Editor to resolve for System information combination NR-XX |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: NTN added to the cell label, typo corrected. Meeting information, Meeting# corrected in the coverpage, added reference to CR3530 Corrected TP with right SIB, Corrected SIB4 Content. Main behaviour is added with additional step for graceful shutdown. Some internal table reference numbers are being corrected. Table 6.7.1.10.3.3-3,4,5 are deleted and Table 6.7.1.10.3.3-6 renamed to Table 6.7.1.10.3.3-3. Table 6.7.1.10.3.3-2 and Table 6.7.1.10.3.3-3 are updated to reflected changes as per R5-254024r3 r2: System information combination NR-XX resolved to NR-35, NR Cell 10 to be used instead of NR Cell 3 |  |  |  |  |  |  |  |  |

<Unchanged Sections Omitted>

#### 6.7.1.10	NR NTN Enhancements / TN Measurement Skipping

6.7.1.10.1	Test Purpose (TP)

(1)

with { UE supporting skipping TN measurement in NR RRC_IDLE state on a NR NTN Enh Cell and SIB4 contains list of tn-AreaIdList where TN measurement can be skipped}

ensure that {

when { a TN cell in tn-AreaIdList becomes higher ranked than the serving cell and the UE is in the coverage of that frequency provided via tn-AreaIdlist }

then { the UE selects to the TN cell }

}

6.7.1.10.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.304 clauses 5.2.4.2 and TS 38.331 clauses 5.2.2.4.27. Unless otherwise stated these are Rel-18 requirements.

[TS 38.304, clause 5.2.4.2]

-	If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ:

-	If distanceThresh and referenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation for NTN (quasi-)Earth-fixed cell and has obtained its location information:

-	If the distance between UE and the serving cell reference location referenceLocation is shorter than distanceThresh, the UE may not perform intra-frequency measurements;

-	Else, the UE shall perform intra-frequency measurements;

-	else if distanceThresh and movingReferenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation for NTN Earth-moving cell and has obtained its location information:

-	if the distance between UE's location and the serving cell reference location determined based on movingReferenceLocation is shorter than distanceThresh, the UE may not perform intra-frequency measurements;

-	else, the UE shall perform intra-frequency measurements;

-	Else, the UE may not perform intra-frequency measurements;

-	Else, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for NR inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For a NR inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current NR frequency, the UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133 [8].

-	For a NR inter-frequency with an equal or lower reselection priority than the reselection priority of the current NR frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current NR frequency:

-	If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ:

-	If distanceThresh and referenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation for NTN (quasi-)Earth-fixed cell and has obtained its UE location information:

-	If the distance between UE and the serving cell reference location referenceLocation is shorter than distanceThresh, the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority;

-	Else, the UE shall perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority according to TS 38.133 [8];

-	else if distanceThresh and movingReferenceLocation are broadcasted in SIB19, and if UE supports location-based measurement initiation for NTN Earth-moving cell and has obtained its location information:

-	if the distance between UE's location and the serving cell reference location determined based on movingReferenceLocation is shorter than distanceThresh, the UE may not perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority;

-	else, the UE shall perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority according to TS 38.133 [8];

-	Else, the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority;

-	Else, the UE shall perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority according to TS 38.133 [8].

-	If the UE supports relaxed measurement and relaxedMeasurement is present in SIB2, the UE may further relax the needed measurements, as specified in clause 5.2.4.9.

-	For UE camping on NTN cell, if the UE supports skipping TN measurement, and the UE has obtained its location information, and if coverageAreaInfoList and tn-AreaIdList are broadcast in system information, the UE may not perform measurements of a TN frequency when UE is not in the coverage of that frequency provided via tn-AreaIdList, regardless of the frequency priority.

If the t-Service of the serving cell is present in SIB19, and if UE supports time-based measurement initiation, the UE shall perform intra-frequency, inter-frequency or inter-RAT measurements before the t-Service, regardless of the distance between UE and the serving cell reference location or whether the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, or Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, The exact time to start measurement before t-Service is up to UE implementation. UE shall perform measurements of higher priority NR inter-frequency or inter-RAT frequencies according to TS 38.133 [8] regardless of the remaining service time of the serving cell (i.e. time remaining until t-Service).

NOTE 1:	When evaluating the distance between UE and the serving cell reference location, it is up to UE implementation to obtain UE location information.

NOTE 2: In the Earth-moving cell, it is up to UE implementation to maintain a valid serving cell reference location, which is derived based on the serving satellite ephemeris, epochTime and movingReferenceLocation.

[TS 38.331, clause 5.2.2.4.27]

No UE requirements related to the contents of SIB25 apply other than those specified elsewhere e.g. within procedures using the concerned system information, and/or within the corresponding field descriptions.

6.7.1.10.3	Test description

6.7.1.10.3.1	Pre-test conditions

System Simulator:

-	NTN NR Cell 1 and TN NR Cell 10 according to TS 38.508-1 [4] Table 4.4.2-2.

-	System information combination NR-35 is used in NTN NR Cell 1 and NR-4 is used in TN NR cell 10 as defined in TS 38.508-1 [4] clause 4.4.3.1.2.

UE:

-	The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C.

Preamble:

-	The UE is in state Registered, Idle Mode (state 1N-A) on NTN NR Cell 1 according to TS 38.508-1 [4].

6.7.1.10.3.2	Test procedure sequence

Table 6.7.1.10.3.2-1 illustrates the downlink power levels to be applied for NR Cell 1 and NR Cell 10 at various time instants of the test execution. Row marked "T0" denotes the conditions after the preamble, while rows marked "T1" are to be applied subsequently at the point indicated in the Main behaviour description in Table 6.7.1.10.3.2-2.

Table 6.7.1.10.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NTN NR Cell 1 | TN NR Cell 10 | Remarks |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -99 | -109 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -99 | -70 |  |

Table 6.7.1.10.3.2-2: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes the power level setting according to the row “T1” in table 6.7.1.10.3.2-1 | - | - | - | - |
| 2 | The SS changes NTN NR Cell1 SIB25 and notifies UE of the system information change on NR Cell 1 by sending Short Message on PDCCH using P-RNTI | <-- | NR RRC: Paging | - | - |
| 3 | Wait for 2.1* modification period second for the SS to transmit modified system information | - | - | - | - |
| 4 | Check:Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.9.2.2-1 indicate that the UE is camped on TN NR Cell 10? | --> | NR RRC: RRCSetupRequest | 1 | P |
| Note 1: The modification period, expressed in number of radio frames = modificationPeriodCoeff * defaultPagingCycle. |  |  |  |  |  |

6.7.1.10.3.3	Specific message contents

6.7.1.10.3.3-1: SIB4 for NTN NR Cell 1 (Preamble and all steps)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| InterFreqCarrierFreqList-v1800 ::= SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo-v1800 { |  |  |  |
| InterFreqCarrierFreqInfo-v1800[1] SEQUENCE { |  | entry 1 |  |
| tn-AreaIdList-r18 SEQUENCE (SIZE (1..maxTN-AreaInfo-r18)) OF TN-AreaId-r18 { |  |  |  |
| TN-AreaId-r18[1] | 1 | entry 1 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.1.10.3.3-2: SIB25 for NTN NR Cell 1(preamble and step 2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB25-r18 ::= SEQUENCE { |  |  |  |
| coverageAreaInfoList-r18 SEQUENCE (SIZE (1..maxTN-AreaInfo-r18)) OF CoverageAreaInfo-r18 { |  |  |  |
| CoverageAreaInfo-r18 [1] SEQUENCE { |  | entry 1 |  |
| tn-ReferenceLocation-r18 | ReferenceLocation with condition RefLocation_2 |  | preamble |
|  | ReferenceLocation with condition RefLocation_1 |  | Step 2 |
| tn-DistanceRadius-r18 | 10000 | 10 km |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.7.1.10.3.3-3: SIB19 of NR Cell 1 (preamble and all steps, 6.7.1.10.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-18C |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| referenceLocation-r17 | ReferenceLocation with condition RefLocation_2 |  |  |
| } |  |  |  |

<Unchanged Sections Omitted>