3GPP TSG WG RAN5 Meeting #108	R5-254298r1

# Bengaluru, India 25th - 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.579-7 | CR | 0007 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 6.2.16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, FirstNet, Element, Samsung R&D Institute UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc17_enh3MCPTT_eMCData3_MCOver5GS-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test case 6.2.16, On-network / File Distribution (FD) / FD Using HTTP / Group Standalone FD / Preconfigured Group Use Only, included in the Work Plan is not implemented. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new test case, 6.2.16 On-network / File Distribution (FD) / FD Using HTTP / Group Standalone FD / Preconfigured Group Use Only |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Work Plan will not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.16 (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Updates to table Table 6.2.16.3.2-1: - Added a timer in step 2 - Added PICS in step 3a1 |  |  |  |  |  |  |  |  |

Table 6.2.15.3.3-11: FD NOTIFICATION (Table 6.2.15.3.3-10)

Derivation Path: TS 37.579-1 [2], Table 5.5.3.8.8-1, condition FD_COMPLETED

### 6.2.16	On-network / File Distribution (FD) / FD Using HTTP / Group Standalone FD / Preconfigured Group Use Only

6.2.16.1	Test Purpose (TP)

(1)

with { UE (MCDATA Client) registered and authorised for MCDATA Service }

ensure that {

when { the MCDATA User requests to send a Group Standalone FD message to a group configured for preconfigured group use only }

then { UE (MCDATA Client) indicate to the MCDATA user that group standalone FD is not allowed on the indicated group and does not send a HTTP POST request }

}

6.2.16.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.282, clause 10.2.4.2.1.  Unless otherwise stated, these are Rel-17 requirements.

[TS 24.282, clause 10.2.4.2.1]

If a group standalone FD message is to be sent, the MCData client shall determine whether the group document contains a <list-service> element that contains a <preconfigured-group-use-only> element. If a <preconfigured-group-use-only> element exists and is set to the value "true", then the MCData client:

1)	should indicate to the MCData user that group standalone FD is not allowed on the indicated group; and

2)	shall skip the remainder of this procedure.

The MCData client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in clause 6.2.4.1;

2)	if a one-to-one standalone FD message is to be sent shall insert in the SIP MESSAGE request:

a)	an application/resource-lists+xml MIME body with the MCData ID of the target MCData user or the functional alias to be called, according to rules and procedures of IETF RFC 4826 [9]; and

b)	an application/vnd.3gpp.mcdata-info+xml MIME body with:

i)	a <request-type> element set to a value of "one-to-one-fd";

ii)	a <call-to-functional-alias-ind> set to "true" if the functional alias is used in the step a) above;

iii)	if the MCData client is aware of active functional aliases and if an active functional alias is to be included in the SIP MESSAGE request, the <functional-alias-URI> element set to the URI of the used functional alias; and

iv)	if the MCData user has requested an application priority, the <anyExt> element with the <user-requested-priority> element set to the user provided value;

3)	if a group standalone FD message is to be sent:

a)	if the "/<x>/<x>/Common/MCData/AllowedFD" leaf node present in the group document of the requested MCData group, configured on the group management client as specified in 3GPP TS 24.483 [42] is set to "false", shall reject the request for FD and not continue with the rest of the steps in this clause; and

b)	shall insert in the SIP MESSAGE request an application/vnd.3gpp.mcdata-info+xml MIME body with:

i)	the <request-type> element set to a value of "group-fd";

ii)	the <mcdata-request-uri> element set to the MCData group identity;

iii)	the <mcdata-client-id> element set to the MCData client ID of the originating MCData client; and

iv)	if the MCData client is aware of active functional aliases and if an active functional alias is to be included in the SIP MESSAGE request, the <functional-alias-URI> element set to the URI of the used functional alias; and

v)	if the MCData user has requested an application priority, the <anyExt> element with the <user-requested-priority> element set to the user provided value;

4)	shall generate a standalone FD message as specified in clause 6.2.2.2; and

5)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

Upon receiving a SIP 300 (Multiple Choices) response to the SIP MESSAGE request the MCData client shall use the MCData ID contained in the <mcdata-request-uri> element of the received application/vnd.3gpp.mcdata-info MIME body as the MCData ID of the invited MCData user and shall generate a new SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6], with the clarifications given in this clause and with the following additional clarifications:

1)	shall insert in the newly generated SIP MESSAGE request a MIME resource-lists body with the MCData ID of the invited MCData user in the <mcdata-request-uri> element of the application/vnd.3gpp.mcdata-info MIME body in the received SIP 300 (Multiple Choices) response;

2)	shall not include a <call-to-functional-alias-ind> element into the <mcdata-Params> element of the <mcdatainfo> element of the application/vnd.3gpp.mcdata-info+xml MIME body; and

3)	shall include a <called-functional-alias-URI> element into the <mcdata-Params> element of the <mcdatainfo> element of the application/vnd.3gpp.mcdata-info+xml MIME body with the target functional alias used in the initial SIP MESSAGE request for for sending one-to-one standalone FD message.

6.2.16.3	Test description

6.2.16.3.1	Pre-test conditions

Test configuration:

-	Single cell configuration according to TS 37.579-1 [2] clause 5.2.2.2.1.

-	Test File 1 for CO FD as specified in annex A.2.1 as specified in Annex A.2.2 is available at the UE for upload.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 37.579-1 [2] clause 5.4.2.

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCData.

6.2.16.3.2	Test procedure sequence

Table 6.2.16.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the UE (MCData client) send test file 1 (Annex A.2.1) for CO group FD over HTTP. (NOTE 1, NOTE 2) | - | - | - | - |
| 2 | Check: Does the UE (MCData client) send an HTTP POST request to upload a file to the media storage function within the next 5s? | --> | HTTP POST | 1 | F |
| - | EXCEPTION: Step 3a1 describes behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if the UE displays an indication to the MCData user that group standalone FD is not allowed on the indicated group1. | - | - | - | - |
| 3a1 | IF pc_MCX_DisplayInfoFDNotAllowedOnGroup THEN Check: Does the UE (MCData client) indicate to the MCData user that group standalone FD is not allowed on the indicated group? (NOTE 1) | - | - | 1 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. NOTE 2: Test file 1 for CO FD as specified in annex A.2.1. |  |  |  |  |  |

6.2.16.3.3	Specific message contents

Table 6.2.16.3.3-1: Group Configuration document (Preamble)

| Derivation Path: TS 37.579-1 [2], Table 5.5.7.4-2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| list-service[1] |  |  |  |  |
| mcpttgi:preconfigured-group-use-only | true |  | TS 24.481 [31] clause 7.2.4.2 |  |