3GPP TSG-RAN5 Meeting #108	R5-254391r2

# Bengaluru, India, 25th - 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4067 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on SUL test scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Current SUL definition on Rel-15 points out to a Secondary Cell (SCell) that will become SUL. However, Rel-15 specification does not include any Carrier Aggregation test scenario which were added in Rel-16 and onwards. Hence, our understanding is this SUL test scenario is a PCell + SUL band without Carrier Aggregation confiugration for a Secondary Cell (SCell) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed all references to Secondary Cell (SCell) and replacing to SUL instead. Removed all test configurations defining Carrier Aggregation configurations Updated Table 6.5.4.1.5-1 to accommodate test configuration for PCell and SUL carriers Removed whole Table 6.5.4.1.5-2 as SUL configurations have been added in Table 6.5.4.1.5-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 6.5.4.1 will remain wrongly defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependency on CR R4-2509849. Core Alignment. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: Addition of editor’s note R2: Adding test incomplete in the editor’s note and more specific reason of inclomplete aspects. |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES >>>

#### 6.5.4.1	NR SA FR1 UE UL carrier RRC reconfiguration delay

Editor's Note: This test case is incomplete in following aspects:

-	It is FFS in the test requirements when the UE shall be ready to start the transmission on the supplementary uplink carrier on SUL.

6.5.4.1.1	Test purpose

To verify that when the UE receives a RRC message implying NR UL or Supplementary UL carrier configuration, the UE shall be ready to start transmission on the newly configured carrier within the time limits specified in TS 38.133 [6] section 8.4.2 and 8.4.3 for configuring and deconfiguring, respectively.

6.5.4.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward supporting SUL.

6.5.4.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.5.4.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.6.5.4.1.

6.5.4.1.4	Test description

6.5.4.1.4.1	Initial conditions

The Test shall be tested using any of the test configuration in Table 6.5.4.1.4.1-1.

Table 6.5.4.1.4.1-1: Supported test configurations

| Configuration | PCell | SUL |
| --- | --- | --- |
| 6.5.4.1-1 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode | SUL: 15kHz SCS, 10MHz bandwidth, SUL duplex mode |
| 6.5.4.1-2 | 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode | SUL: 30kHz SCS, 40MHz bandwidth, SUL duplex mode |
| 6.5.4.1-3 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode | SUL: 15kHz SCS, 10MHz bandwidth, SUL duplex mode |
| 6.5.4.1-4 | 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode | SUL: 30kHz SCS, 40MHz bandwidth, SUL duplex mode |
| 6.5.4.1-5 | 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode | SUL: 15kHz SCS, 10MHz bandwidth, SUL duplex mode |
| 6.5.4.1-6 | 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode | SUL: 30kHz SCS, 40MHz bandwidth, SUL duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: Void |  |  |

Configure the test requirement and the DUT according to the parameters in Table 6.5.4.1.4.1-2.

Table 6.5.4.1.4.1-2: Initial conditions for NR SA FR1 UE UL carrier RRC reconfiguration delay

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.4-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.5.4.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.1. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 6.5.4.1.4.1-3.

2.	Message contents are defined in clause 6.5.4.1.4.3.

3.	 There is one cell with two UL carriers: FR1 PCell and FR1 SUL. The test parameters for PCell and SUL are given in Table 6.5.4.1.4.1-1, Table 6.5.4.1.4.1-3 and Table 6.5.4.1.5-1 below.

Table 6.5.4.1.4.1-3: General test parameters for NR standalone UE UL carrier RRC reconfiguration Delay on Pcell

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| RF Channel Number |  | Config 1,2,3, 4, 5, 6 | 1, 2 | Two UL radio channels are used for this test. |
| Active cell |  | Config 1,2,3, 4, 5, 6 | FR1 PCell FR1 SUL | FR1 PCell on RF channel number 1 FR1 SUL on RF channel number 2 |
| CP length |  | Config 1,2,3, 4, 5, 6 | Normal |  |
| DRX |  | Config 1,2,3, 4, 5, 6 | OFF |  |
| Measurement gap pattern Id |  | Config 1,2,3, 4, 5, 6 | OFF |  |
| Filter coefficient |  | Config 1,2,3, 4, 5, 6 | 0 | L3 filtering is not used |
| T1 | s | Config 1,2,3, 4, 5, 6 | 5 |  |
| T2 | s | Config 1,2,3, 4, 5, 6 | 5 |  |
| T3 | s | Config 1,2,3, 4, 5, 6 | 5 |  |

6.5.4.1.4.2	Test procedure

There is one cell with two UL carriers: FR1 PCell and FR1 SUL. Both NR uplink and supplementary uplink are broadcast by ServingCellConfigCommonSIB.

The test consists of three time periods, with duration of T1, T2 and T3 respectively. During time duration T1, only NR uplink of PCell is configured to UE. At the start of T2, a supplementary uplink  is configured to UE through RRCReconfiguration, then UE shall start transmission on the supplementary uplink. At the start of T3, the supplementary uplink is released through RRCReconfiguration.

At the start of the test the UE is in RRC_CONNECTED state with Cell 1 (NR uplink cell). The SS shall provide continuous UL grants to the UE, so it can transmit in the corresponding uplink carrier as soon as it is ready for transmission.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Setup PCell according to parameters given in Table 6.5.4.1.5-1. Propagation conditions are set according to Annex C clause C.2.2.

3.	Both PCell NR uplink and SUL supplementary uplink are broadcast by ServingCellConfigCommonSIB of Cell 1.

4.	During time duration T1, PCell NR uplink  is configured on the UE. Setup Cell 1 according to parameters given in Table 6.5.4.1.5-1.

5.	At the start of T2, the supplementary uplink is configured to UE through RRCReconfiguration, then UE shall start transmission on the supplementary uplink within 20ms. If UE transmits data on the supplementary uplink within 20ms from the start of T2, then count a success for the event "reconfiguration" otherwise count a failure for event "reconfiguration" and proceed with step 7.

6.	At the start of T3, the supplementary uplink is released through RRCReconfiguration, then UE shall transmit data only on the NR uplink carrier of SUL within 20ms. If UE stops transmitting data on supplementary uplink carrier on SUL within 20ms from the start of T3, then count a success for the event "deconfiguration" otherwise count a failure for event "deconfiguration".

7.	Repeat steps 1-6 until the confidence level according to Table G.2.3-1 in Annex G.2 is achieved.

6.5.4.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 6.5.4.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.8-1 Table H.3.8-2 Table 4.6.3-128 in 38.508-1 [14] with condition SUL Table 4.6.3-167 in 38.508-1 [14] with condition PUSCH_PUCCH_ON_SUL Table 4.6.3-169 in 38.508-1 [14] with condition SUL Table 4.6.3-202 in 38.508-1 [14] with condition SUL_SUL |

6.5.4.1.5	Test requirement

Table 6.5.4.1.5.1-1 define the primary level settings including test tolerances for UE UL carrier RRC reconfiguration delay test.

Table 6.5.4.1.5-1: NR Cell specific test parameters for NR standalone UE UL carrier RRC reconfiguration Delay on PCell

| Noc ÊsIot |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | Unit |  | Test Configuration |  | Test 1 |  |  |  |  |  |  |
|  |  |  |  |  | T1 |  | T2 |  | T3 |  |  |
| Channel number for NR UL carrier |  |  | Conf 1, 2, 3, 4, 5, 6 |  | 1 |  |  |  |  |  |  |
| Channel number for supplementary UL |  |  | Conf 1, 2, 3, 4, 5, 6 |  | 2 |  |  |  |  |  |  |
| TDD configuration |  |  | Conf 1, 2 |  | N/A |  |  |  |  |  |  |
|  |  |  | Conf 3, 4 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  |  | Conf 5, 6 |  | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel | MHz |  | Conf 1, 2, 3, 4 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  |  | Conf 5, 6 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWchannel for supplementary UL | MHz |  | Conf 1, 3, 5 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  |  | Conf 2, 4, 6 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| PUSCH parameters for NR UL carrier |  |  | Conf 1, 2, 3, 4 |  | G-FR1-A3-9 in [13] |  | G-FR1-A3-9 in [13] |  | G-FR1-A3-9 in [13] |  |  |
|  |  |  | Conf 5, 6 |  | G-FR1-A3-13 in [13] |  | G-FR1-A3-13 in [13] |  | G-FR1-A3-13 in [13] |  |  |
| PUCCH parameters For NR UL carrier |  |  | Conf 1, 2, 3, 4 |  | Table 8.3.3.1.2-1 in [13] |  | Table 8.3.3.1.2-1 in [13] |  | Table 8.3.3.1.2-1 in [13] |  |  |
|  |  |  | Conf 5, 6 |  | Table 8.3.3.1.2-2 in [13] |  | Table 8.3.3.1.2-2 in [13] |  | Table 8.3.3.1.2-2 in [13] |  |  |
| PUSCH parameters for supplementary UL |  |  | Conf 1, 3, 5 |  | N/A |  | G-FR1-A3-9 in [13] |  | N/A |  |  |
|  |  |  | Conf 2, 4, 6 |  | N/A |  | G-FR1-A3-13 in [13] |  | N/A |  |  |
| PUCCH parameters for supplementary UL |  |  | Conf 1, 3, 5 |  | N/A |  | N/A |  | N/A |  |  |
|  |  |  | Conf 2, 4, 6 |  | N/A |  | N/A |  | N/A |  |  |
| PDSCH reference measurement channel as defined in A.3.1.1 |  |  | Conf 1, 2 |  | SR.1.1 FDD |  |  |  |  |  |  |
|  |  |  | Conf 3, 4 |  | SR.1.1 TDD |  |  |  |  |  |  |
|  |  |  | Conf 5, 6 |  | SR 2.1 TDD |  |  |  |  |  |  |
| RMSI CORESET reference measurement channel as defined in A.3.1.2 |  |  | Conf 1, 2 |  | CR.1.1 FDD |  |  |  |  |  |  |
|  |  |  | Conf 3, 4 |  | CR.1.1 TDD |  |  |  |  |  |  |
|  |  |  | Conf 5, 6 |  | CR.2.1 TDD |  |  |  |  |  |  |
| RMC CORESET reference measurement channel as defined in A.3.1.3 |  |  | Conf 1, 2 |  | CCR.1.1 FDD |  |  |  |  |  |  |
|  |  |  | Conf 3, 4 |  | CCR.1.1 TDD |  |  |  |  |  |  |
|  |  |  | Conf 5, 6 |  | CCR.2.1 TDD |  |  |  |  |  |  |
| OCNG Pattern Note 1 |  |  | Conf 1, 2, 3, 4, 5, 6 |  | OP.1 |  |  |  |  |  |  |
| SSB configuration |  |  | Conf 1, 2, 3, 4 |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  |  | Conf 5, 6 |  | SSB.2 FR1 |  |  |  |  |  |  |
| SMTC configuration |  |  | Conf 1, 2, 3, 4, 5, 6 |  | SMTC.1 |  |  |  |  |  |  |
| CSI-RS for tracking |  |  | Conf 1, 2 |  | TRS.1.1 FDD |  |  |  |  |  |  |
|  |  |  | Conf 3, 4 |  | TRS.1.1 TDD |  |  |  |  |  |  |
|  |  |  | Conf 5, 6 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| DL initial BWP configuration |  |  | Conf 1, 2, 3, 4, 5, 6 |  | DLBWP.0.1 |  |  |  |  |  |  |
| DL dedicated BWP configuration |  |  | Conf 1, 2, 3, 4, 5, 6 |  | DLBWP.1.1 |  |  |  |  |  |  |
| UL dedicated BWP configuration |  |  | Conf 1, 2, 3, 4, 5, 6 |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS | dB |  | Conf 1, 2, 3, 4, 5, 6 |  | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS |  |  |  |  |  |  |  |  |  |  |  |
| Note 2 | dBm / 15kHz |  | Conf 1, 2, 3, 4, |  | -102 |  |  |  |  |  |  |
|  | dBm/ SCS |  | Conf 1, 2, 3, 4, |  | -102 |  |  |  |  |  |  |
|  |  |  | Conf 5, 6 |  | -99 |  |  |  |  |  |  |
|  | dB |  | Conf 1, 2, 3, 4, 5, 6 |  | 16 |  | 16 |  | 16 |  |  |
| Note 3 | dB |  | Conf 1, 2, 3, 4, 5, 6 |  | 16 |  | 16 |  | 16 |  |  |
| SS-RSRP Note 3 | dBm/ SCS |  | Conf 1, 2, 3, 4 |  | -86 |  | -86 |  | -86 |  |  |
|  |  |  | Conf 5, 6 |  | -83 |  | -83 |  | -83 |  |  |
| Io Note 3 | dBm/ 9.36 MHz |  | Conf 1, 2, 3, 4 |  | -57.9 |  | -57.9 |  | -57.9 |  |  |
|  | dBm/ 38.16MHz |  | Conf 5, 6 |  | -51.8 |  | -51.8 |  | -51.8 |  |  |
| Propagation Condition |  |  | Conf 1, 2, 3, 4, 5, 6 |  | AWGN |  |  |  |  |  |  |
| Antenna configuration |  |  | Conf 1, 2, 3, 4, 5, 6 |  | 1 x 2 |  |  |  |  |  |  |
| SUL RSRP threshold (rsrp-ThresholdSSB-SUL) | dBm |  | Conf 1,2,3,4 |  | -75 |  |  |  |  |  |
|  |  |  | Conf 5, 6 |  | -72 |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated, and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. NOTE 3: , Io, and SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Void. NOTE 5: Void. NOTE 6: NRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. |  |  |  |  |  |  |  |  |  |  |  |

The UE shall be ready to start transmission on the supplementary uplink carrier on SUL within 20 ms from the start of T2.

The UE shall stop the transmission on the supplementary uplink carrier on SUL within 20 ms from the start of T3.

All of the above test requirements shall be fulfilled in order for the observed UE UL carrier configuration delay and UE UL carrier release delay to be counted as correct. The rate of correct observed UE UL carrier configuration delay and UE UL carrier release delay during repeated tests shall be at least 90%.

## <<< END OF CHANGES >>>