3GPP TSG RAN WG5 #108	R5-254428r1

# Bangalore, India 25th - 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 1183 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to clause 6.2A.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | T(MPR) should be changed to T(MAX(MPR, A-MPR)) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change T(MPR) to T(MAX(MPR, A-MPR)) in clauses 6.2A.3.1, 6.2A.3.2, and 6.2A.3.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect parameters are in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.3.1, 6.2A.3.2, 6.2A.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Editorial |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: changed the WID. |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES 1 >>>

6.2A.3.1.5	Test requirement

The EIRP derived in step 9 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2A.3.1.5-1 to Table 6.2A.3.1.5-6.

Table 6.2A.3.1.5-0: Test Tolerance (A-MPR for CA) (Aggregated BW ≤ 400MHz)

| Power Class | Test Metric | FR2a | FR2b |
| --- | --- | --- | --- |
| PC1 | Max device size ≤ 30 cm | 3.38 dB, NTC | 3.38 dB, NTC |
| PC2 | Max device size ≤ 30 cm | FFS | FFS |
| PC3 | Max device size ≤ 30 cm | 3.24 dB, NTC | 3.24 dB, NTC |
| PC4 | Max device size ≤ 30 cm | FFS | FFS |
| PC5 | Max device size ≤ 30 cm | 3.38 dB, NTC | - |

Table 6.2A.3.1.5-1: A-MPR requirements for CA_NS_202 (Power Class 1)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258 | 40.0 | 6.5~9.7 | 11.0 | 7.0 | 22-TT | 55 |
| 2 | n257, n258 (NOTE 2) | 40.0 | 9~10.7 | 11.0 | 7.0 | 22-TT | 55 |
| 2 | n257, n258 (NOTE 3) | 40.0 | 11.2 | 11.0 | 7.0 | 21.8-TT | 55 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. NOTE 2: Cumulative aggregated BW < 800MHz. NOTE 3: Cumulative aggregated BW = 800MHz. |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-1b: A-MPR requirements for CA_NS_202 (Power Class 2)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258 | 29.0 | 0 | 2 | 1.5 | 25.5-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-1c: A-MPR requirements for CA_NS_202 (Power Class 3)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258 | 22.4 | 0 | 2 | 1.5 | 18.9-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-1d: A-MPR requirements for CA_NS_202 (Power Class 4)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258 | 34.0 | 0 | 2 | 1.5 | 30.5-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-1e: A-MPR requirements for CA_NS_202 (Power Class 5)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257 | 30.0 | 0 | 2 | 1.5 | 26.5-TT | 43 |
|  | n258 | 30.4 | 0 | 2 | 1.5 | 26.9-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-2: A-MPR requirements for CA_NS_203 (Power Class 1)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 40.0 | 0 | 6.5 | 5.0 | 28.5-TT | 55 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-3: A-MPR requirements for CA_NS_203 (Power Class 2)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 29.0 | 0 | 2.5 | 2.0 | 24.5-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-4: A-MPR requirements for CA_NS_203 (Power Class 3)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 22.4 | 0 | 2.5 | 2.0 | 17.9-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-5: A-MPR requirements for CA_NS_203 (Power Class 4)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 34.0 | 0 | 2.5 | 2.0 | 29.5-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.1.5-6: A-MPR requirements for CA_NS_203 (Power Class 5)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 30.4 | 0 | 2.5 | 2.0 | 25.9-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

## <<< END OF CHANGES 1 >>>

## <<< START OF CHANGES 2 >>>

6.2A.3.2.5	Test requirement

The EIRP derived in step 9 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2A.3.2.5-1 to Table 6.2A.3.2.5-5.

Table 6.2A.3.2.5-0: Test Tolerance (A-MPR for CA) (Aggregated BW ≤ 400MHz)

| Power Class | Test Metric | FR2a | FR2b |
| --- | --- | --- | --- |
| PC1 | Max device size ≤ 30 cm | 3.38 dB, NTC | 3.38 dB, NTC |
| PC2 | Max device size ≤ 30 cm | FFS | FFS |
| PC3 | Max device size ≤ 30 cm | 3.24 dB, NTC | 3.24 dB, NTC |
| PC4 | Max device size ≤ 30 cm | FFS | FFS |
| PC5 | Max device size ≤ 30 cm | 3.38 dB, NTC | - |

Table 6.2A.3.2.5-1: A-MPR requirements for CA_NS_202 (Power Class 1)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258 | 40.0 | 6.5~9.7 | 11.0 | 7.0 | 22-TT | 55 |
| 2 | n257, n258 (NOTE 2) | 40.0 | 9~10.7 | 11.0 | 7.0 | 22-TT | 55 |
| 2 | n257, n258 (NOTE 3) | 40.0 | 11.2 | 11.0 | 7.0 | 21.8-TT | 55 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.2.5-0. NOTE 2: Cumulative aggregated BW < 800MHz. NOTE 3: 800MHz <= Cumulative aggregated BW < 1400MHz. |  |  |  |  |  |  |  |

Table 6.2A.3.2.5-1b: A-MPR requirements for CA_NS_202 (Power Class 2)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258 | 29.0 | 0 | 2 | 1.5 | 25.5-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.2.5-1c: A-MPR requirements for CA_NS_202 (Power Class 3)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258 | 22.4 | 0 | 2 | 1.5 | 18.9-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.2.5-1d: A-MPR requirements for CA_NS_202 (Power Class 4)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258 | 34.0 | 0 | 2 | 1.5 | 30.5-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.2.5-1e: A-MPR requirements for CA_NS_202 (Power Class 5)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257 | 30.0 | 0 | 2 | 1.5 | 26.5-TT | 43 |
|  | n258 | 30.4 | 0 | 2 | 1.5 | 26.9-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.2.5-2: A-MPR requirements for CA_NS_203 (Power Class 1)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 40.0 | 0 | 6.5 | 5.0 | 28.5-TT | 55 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.2.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.2.5-3: A-MPR requirements for CA_NS_203 (Power Class 2)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 29.0 | 0 | 2.5 | 2.0 | 24.5-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.2.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.2.5-4: A-MPR requirements for CA_NS_203 (Power Class 3)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 22.4 | 0 | 2.5 | 2.0 | 17.9-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.2.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.2.5-5: A-MPR requirements for CA_NS_203 (Power Class 4)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 34.0 | 0 | 2.5 | 2.0 | 29.5-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.2.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.2.5-6: A-MPR requirements for CA_NS_203 (Power Class 5)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 30.4 | 0 | 2.5 | 2 | 25.9-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

## <<< END OF CHANGES 2 >>>

## <<< START OF CHANGES 3 >>>

6.2A.3.3.5	Test requirement

The EIRP derived in step 9 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2A.3.3.5-1 to Table 6.2A.3.3.5-6.

Table 6.2A.3.3.5-0: Test Tolerance (A-MPR for CA) (Aggregated BW ≤ 400MHz)

| Power Class | Test Metric | FR2a | FR2b |
| --- | --- | --- | --- |
| PC1 | Max device size ≤ 30 cm | 3.38 dB, NTC | 3.38 dB, NTC |
| PC2 | Max device size ≤ 30 cm | FFS | FFS |
| PC3 | Max device size ≤ 30 cm | 3.24 dB, NTC | 3.24 dB, NTC |
| PC4 | Max device size ≤ 30 cm | FFS | FFS |
| PC5 | Max device size ≤ 30 cm | 3.38 dB, NTC | - |

Table 6.2A.3.3.5-1: A-MPR requirements for CA_NS_202 (Power Class 1)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258 | 40.0 | 6.5~9.7 | 11.0 | 7.0 | 22-TT | 55 |
| 2 | n257, n258 (NOTE 2) | 40.0 | 9~10.7 | 11.0 | 7.0 | 22-TT | 55 |
| 2 | n257, n258 (NOTE 3) | 40.0 | 11.2 | 11.0 | 7.0 | 21.8-TT | 55 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.3.5-0. NOTE 2: Cumulative aggregated BW < 800MHz. NOTE 3: 800MHz <= Cumulative aggregated BW < 1400MHz. |  |  |  |  |  |  |  |

Table 6.2A.3.3.5-1b: A-MPR requirements for CA_NS_202 (Power Class 2)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258 | 29.0 | 0 | 2 | 1.5 | 25.5-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.3.5-1c: A-MPR requirements for CA_NS_202 (Power Class 3)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258 | 22.4 | 0 | 2 | 1.5 | 18.9-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.3.5-1d: A-MPR requirements for CA_NS_202 (Power Class 4)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257, n258 | 34.0 | 0 | 2 | 1.5 | 30.5-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.3.5-1e: A-MPR requirements for CA_NS_202 (Power Class 5)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n257 | 30.0 | 0 | 2 | 1.5 | 26.5-TT | 43 |
|  | n258 | 30.4 | 0 | 2 | 1.5 | 26.9-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.3.5-2: A-MPR requirements for CA_NS_203 (Power Class 1)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 40.0 | 0 | 6.5 | 5.0 | 28.5-TT | 55 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.3.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.3.5-3: A-MPR requirements for CA_NS_203 (Power Class 2)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 29.0 | 0 | 2.5 | 2.0 | 24.5-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.3.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.3.5-4: A-MPR requirements for CA_NS_203 (Power Class 3)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 22.4 | 0 | 2.5 | 2.0 | 17.9-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.3.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.3.5-5: A-MPR requirements for CA_NS_203 (Power Class 4)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 34.0 | 0 | 2.5 | 2.0 | 29.5-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.3.5-0. |  |  |  |  |  |  |  |

Table 6.2A.3.3.5-6: A-MPR requirements for CA_NS_203 (Power Class 5)

| Test ID | Band | Min peak EIRP (dBm) | MPR (dB) | A-MPR (dB) | T(MAX(MPR, A-MPR)) (dB) | Lower limit (dBm) | Upper limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | n258 | 30.4 | 0 | 2.5 | 2 | 25.9-TT | 43 |
| NOTE 1: TT for each band and accumulative aggregated bandwidth is specified in Table 6.2A.3.1.5-0. |  |  |  |  |  |  |  |

## <<< END OF CHANGES 3 >>>