3GPP TSG-RAN5 Meeting #108	R5-254613r1

Bengaluru, India, Aug 25th – 29th 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 3475 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case AMPR, ASEM, ASE for 4Tx Tx Diversity |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_FR1_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Additional maximum output power reduction for Tx Diversity for UE supporting 4Tx test case is absent in 6.2G.3_1 and need to be added. - Additional spectrum emission mask for Tx Diversity for UE supporting 4Tx test case is absent in 6.5G.2.2_1 and need to be added. - Additional spurious emissions for Tx Diversity for UE supporting 4Tx test case is absent in 6.5G.2.2_1 and need to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Additional maximum output power reduction for Tx Diversity for UE supporting 4Tx test case is added. - Additional spectrum emission mask for Tx Diversity for UE supporting 4Tx test case is added. - Additional spurious emissions for Tx Diversity for UE supporting 4Tx test case is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AMPR, ASEM and ASE for Tx Diversity for UE supporting 4Tx test case can not be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2G.3_1(new), 6.5G.2.2_1(new), 6.5G.3.3_1 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: - Update the Editor’s Notes in 6.5G.3.3_1. - Update the test tolerance values in Table 6.5G.2.2_1.5-1 to FFS to align with Editor’s Notes. |  |  |  |  |  |  |  |  |

<Start of Change1>

6.2G.3_1	UE additional maximum output power reduction for Tx Diversity for UE supporting 4Tx

6.2G.3_1.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2G.2. Outer and inner allocation notation used in clause 6.2G.3 is defined in clause 6.2.2. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

6.2G.3_1.2	Test applicability

The requirements of this test apply in test case 6.5G.2.2_1 Additional Spectrum Emission mask for Tx Diversity for UE supporting 4Tx for network signalling value NS_04 to all types of NR Power Class 1.5 reporting 4Tx Tx diversity UE release 18 and forward.

The requirements of this test apply in test case 6.5G.3_1.3 Additional Spurious Emissions Tx Diversity for UE supporting 4Tx for network signalling values NS_04, NS_47 and NS_50 to all types of NR Power Class 1.5 reporting 4Tx Tx diversity UE release 18 and forward.

6.2G.3_1.3	Minimum conformance requirements

For UE supporting Tx diversity, the A-MPR values specified in clause 6.2.3 shall apply to the maximum output power specified in Table 6.2.1-1, and the maximum output power is defined as the sum of the maximum output power at each UE antenna connector. Unless stated otherwise, an A-MPR of 0 dB shall be used.

Table 6.2G.3_1.3-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2G.3_1.3-2.

Table 6.2G.3_1.3-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | Table 5.3.2-1 | N/A |
| NS_04 | 6.5.2.3.3.2, 6.5.3.3.3.1 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | Clause 6.2.3.3.2 |
| NS_47 | 6.5.3.3.3.15 | n41 (Note 3) | 30 | Table 6.2.3.3.18-1 | Table 6.2.3.3.18-2 |
| NS_50 | 6.5.3.3.3.16 | n39 | 10, 15, 20, 25, 30, 35, 40 |  | Clause 6.2.3.3.19 |
| NS_55 | NOTE 4 | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | N/A |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: No A-MPR is applied for 5 MHz BWchannel where the lower channel edge is ≥1930 MHz, 10 MHz BWchannel where the lower channel edge is ≥1950 MHz and 15 MHz BWchannel where the lower channel edge is ≥1955 MHz. NOTE 3: Applicable when the NR carrier is within 2545 – 2575 MHz. NOTE 4: This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements. |  |  |  |  |  |

Table 6.2G.3_1.3-2: Mapping of Network Signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n34 | NS_01 |  |  |  |  |  |  |  |
| n39 | NS_01 | NS_50 |  |  |  |  |  |  |
| n40 | NS_01 |  |  |  |  |  |  |  |
| n41 | NS_01 | NS_04 | NS_47 |  |  |  |  |  |
| n77 | NS_01 | NS_55 |  |  |  |  |  |  |
| n78 | NS_01 |  |  |  |  |  |  |  |
| n79 | NS_01 |  |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [6]. |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.1 and 6.2G.3.

6.2G.3_1.4	Test description

6.2G.3_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2.3.4.1-2 for NS_04 PC1.5 and Table 6.2G.3.4.1-1 for NS_47 PC1.5, and in Table 6.2G.3.4.1-2 for NS_50 PC1.5. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2a for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2 and uplink signals according Annex G.0, G.1, G.2 and G.3.0.

4.	The UL Reference Measurement channels are set according to the applicable Table 6.2.3.4.1-2, Table 6.2G.3.4.1-1 and Table 6.2G.3.4.1-2.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2G.3_1.4.3.

6.2G.3_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to the applicable table from Table 6.2G.3.4.1-1 and Table 6.2G.3.4.1-2. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE Allow at least 200ms starting from the first TPC command in this step for the UE to reach PUMAX level.

3.	Measure the sum of the mean power of the UE at each antenna connector in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration one sub-frame (1ms). For TDD, only slots consisting of only UL symbols are under test.

NOTE 1:	When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2G.3.4.1-1, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2G.3_1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.1, with the following exceptions for each network signalling value.

6.2G.3_1.4.3.1	Message contents exceptions for network signalling value "NS_04"

1.	Information element additionalSpectrumEmission is set to NS_04. This can be set in the SIB1 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2G.3_1.4.3.1-1: AdditionalSpectrumEmission Additional spurious emissions test requirement for "NS_04"

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 1 (NS_04) |  |  |

6.2G.3_1.4.3.2	Message contents exceptions for network signalling value "NS_47"

Same message contents exceptions as 6.2G.3.4.3.2.

6.2G.3_1.4.3.3	Message contents exceptions for network signalled value "NS_50"

Same message contents exceptions as 6.2G.3.4.3.3.

6.2G.3_1.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2G.3_1.5-1. The allowed A-MPR values specified in table 6.2G.3.3-1 are in addition to the allowed MPR requirements specified in clause 6.2G.2_1. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2G.1_1.3-1 apply.

Table 6.2G.3_1.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2G.3_1.5-1: UE Power Class 1.5 test requirements (NS_04) for band n41

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR’ (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 29 | 8 | 7 | 1.5 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 8 | 29 | 8 | 5 | 0 | 21 | 5 | 3 | 31+TT | 16-TT |
| 9 | 29 | 8 | 0 | 0 | 21 | 5 | 3 | 31+TT | 16-TT |
| 10 | 29 | 3 | 5 | 0 | 24 | 5 | 3 | 31+TT | 19-TT |
| 11 | 29 | 8 | 0 | 1.5 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 12 | 29 | 3 | 0 | 0 | 26 | 3 | 3 | 31+TT | 23-TT |
| 13 | 29 | 8.5 | 7.5 | 1.5 | 19 | 5 | 3 | 31+TT | 14-TT |
| 14 | 29 | 8.5 | 6 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 15 | 29 | 8.5 | 0 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 16 | 29 | 3.5 | 6 | 0 | 23 | 5 | 3 | 31+TT | 18-TT |
| 17 | 29 | 8.5 | 0 | 1.5 | 19 | 5 | 3 | 31+TT | 14-TT |
| 18 | 29 | 3.5 | 0 | 0 | 25.5 | 3 | 3 | 31+TT | 22.5-TT |
| 19 | 29 | 8.5 | 7.5 | 1.5 | 19 | 5 | 3 | 31+TT | 14-TT |
| 20 | 29 | 8.5 | 6.5 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 21 | 29 | 8.5 | 0 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 22 | 29 | 4 | 6.5 | 0 | 22.5 | 5 | 3 | 31+TT | 17.5-TT |
| 23 | 29 | 8.5 | 0 | 1.5 | 19 | 5 | 3 | 31+TT | 14-TT |
| 24 | 29 | 4 | 0 | 0 | 25 | 3 | 3 | 31+TT | 22-TT |
| 25 | 29 | 8.5 | 8 | 1.5 | 19 | 5 | 3 | 31+TT | 14-TT |
| 26 | 29 | 8.5 | 6.5 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 27 | 29 | 8.5 | 0 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 28 | 29 | 4.7 | 6.5 | 0 | 22.5 | 5 | 3 | 31+TT | 17.5-TT |
| 29 | 29 | 8.5 | 0 | 1.5 | 19 | 5 | 3 | 31+TT | 14-TT |
| 30 | 29 | 4.7 | 0 | 0 | 24.3 | 5 | 3 | 31+TT | 19.3-TT |
| 31 | 29 | 9.5 | 9.5 | 1.5 | 18 | 5 | 3 | 31+TT | 13-TT |
| 32 | 29 | 9.5 | 8 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 33 | 29 | 9.5 | 0 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 34 | 29 | 7 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16-TT |
| 35 | 29 | 9.5 | 0 | 1.5 | 18 | 5 | 3 | 31+TT | 13-TT |
| 36 | 29 | 7 | 0 | 0 | 22 | 5 | 3 | 31+TT | 17-TT |
| 37 | 29 | 9.5 | 9 | 1.5 | 18 | 5 | 3 | 31+TT | 13-TT |
| 38 | 29 | 9.5 | 8 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 39 | 29 | 9.5 | 0 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 40 | 29 | 5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16-TT |
| 41 | 29 | 9.5 | 0 | 1.5 | 18 | 5 | 3 | 31+TT | 13-TT |
| 42 | 29 | 5 | 0 | 0 | 24 | 5 | 3 | 31+TT | 19-TT |
| 43 | 29 | 9.5 | 9 | 1.5 | 18 | 5 | 3 | 31+TT | 13-TT |
| 44 | 29 | 9.5 | 8 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 45 | 29 | 9.5 | 0 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 46 | 29 | 5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16-TT |
| 47 | 29 | 9.5 | 0 | 1.5 | 18 | 5 | 3 | 31+TT | 13-TT |
| 48 | 29 | 5 | 0 | 0 | 24 | 5 | 3 | 31+TT | 19-TT |
| 49 | 29 | 9.5 | 9 | 1.5 | 18 | 5 | 3 | 31+TT | 13-TT |
| 50 | 29 | 9.5 | 8 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 51 | 29 | 9.5 | 0 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 52 | 29 | 7 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16-TT |
| 53 | 29 | 9.5 | 0 | 1.5 | 18 | 5 | 3 | 31+TT | 13-TT |
| 54 | 29 | 7 | 0 | 0 | 22 | 5 | 3 | 31+TT | 17-TT |
| 55 | 29 | 9.5 | 11.5 | 1.5 | 16 | 5 | 3 | 31+TT | 11-TT |
| 56 | 29 | 9.5 | 9 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 57 | 29 | 9.5 | 0 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 58 | 29 | 9.5 | 9 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 59 | 29 | 9.5 | 0 | 1.5 | 18 | 5 | 3 | 31+TT | 13-TT |
| 60 | 29 | 9.5 | 0 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2G.3_1.5-3: UE Power Class 1.5 test requirements for NS_47 (contiguous allocation) for band n41

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 8 | 13 | 0 | 16 | 6 | 3 | 31+TT | 10-TT |
| 2 | 29 | 0 | 2 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 3 | 29 | 0 | 8 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16-TT |
| 4 | 29 | 0 | 3 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 5 | 29 | 0 | 3 | 8.5 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 6 | 29 | 0 | 3 | 8.5 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 7 | 29 | 0 | 3 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 8 | 29 | 0 | 3 | 3 | 0 | 26 | 3 | 3 | 31+TT | 23-TT |
| 9 | 29 | 0 | 8.5 | 13 | 0 | 16 | 6 | 3 | 31+TT | 10-TT |
| 10 | 29 | 0 | 2 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 11 | 29 | 0 | 8.5 | 8 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 12 | 29 | 0 | 3.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 13 | 29 | 0 | 3.5 | 8.5 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 14 | 29 | 0 | 3.5 | 8.5 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 15 | 29 | 0 | 3.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 16 | 29 | 0 | 3.5 | 3 | 0 | 25.5 | 5 | 3 | 31+TT | 20.5-TT |
| 17 | 29 | 0 | 8.5 | 13 | 0 | 16 | 6 | 3 | 31+TT | 10-TT |
| 18 | 29 | 0 | 2.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 19 | 29 | 0 | 8.5 | 8 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 20 | 29 | 0 | 4 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 21 | 29 | 0 | 4 | 8.5 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 22 | 29 | 0 | 4 | 8.5 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 23 | 29 | 0 | 4 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 24 | 29 | 0 | 4 | 3 | 0 | 25 | 3 | 3 | 31+TT | 22-TT |
| 25 | 29 | 0 | 8.5 | 13 | 0 | 16 | 6 | 3 | 31+TT | 10-TT |
| 26 | 29 | 0 | 4.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 27 | 29 | 0 | 8.5 | 8 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 28 | 29 | 0 | 4.7 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 29 | 29 | 0 | 4.7 | 8.5 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 30 | 29 | 0 | 4.7 | 8.5 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 31 | 29 | 0 | 4.7 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 32 | 29 | 0 | 4.7 | 0 | 0 | 24.3 | 5 | 3 | 31+TT | 19.3-TT |
| 33 | 29 | 0 | 9.5 | 13 | 0 | 16 | 6 | 3 | 31+TT | 10-TT |
| 34 | 29 | 0 | 7 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 35 | 29 | 0 | 9.5 | 8 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 36 | 29 | 0 | 7 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 37 | 29 | 0 | 7 | 8.5 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 38 | 29 | 0 | 7 | 8.5 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 39 | 29 | 0 | 7 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 40 | 29 | 0 | 7 | 0 | 0 | 22 | 5 | 3 | 31+TT | 17-TT |
| 41 | 29 | 0 | 9.5 | 13 | 0 | 16 | 6 | 3 | 31+TT | 10-TT |
| 42 | 29 | 0 | 3.5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 43 | 29 | 0 | 9.5 | 8 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 44 | 29 | 0 | 5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 45 | 29 | 0 | 5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 46 | 29 | 0 | 5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 47 | 29 | 0 | 5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 48 | 29 | 0 | 5 | 4 | 0 | 24 | 5 | 3 | 31+TT | 19-TT |
| 49 | 29 | 0 | 9.5 | 13 | 0 | 16 | 6 | 3 | 31+TT | 10-TT |
| 50 | 29 | 0 | 4 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 51 | 29 | 0 | 9.5 | 8 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 52 | 29 | 0 | 5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 53 | 29 | 0 | 5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 54 | 29 | 0 | 5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 55 | 29 | 0 | 5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 56 | 29 | 0 | 5 | 4 | 0 | 24 | 5 | 3 | 31+TT | 19-TT |
| 57 | 29 | 0 | 9.5 | 13 | 0 | 16 | 6 | 3 | 31+TT | 10-TT |
| 58 | 29 | 0 | 7 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 59 | 29 | 0 | 9.5 | 8 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 60 | 29 | 0 | 7 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 61 | 29 | 0 | 7 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 62 | 29 | 0 | 7 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 63 | 29 | 0 | 7 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 64 | 29 | 0 | 7 | 0 | 0 | 22 | 5 | 3 | 31+TT | 17-TT |
| 65 | 29 | 0 | 9.5 | 13 | 0 | 16 | 6 | 3 | 31+TT | 10-TT |
| 66 | 29 | 0 | 9.5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 67 | 29 | 0 | 9.5 | 8 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 68 | 29 | 0 | 9.5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 69 | 29 | 0 | 9.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 70 | 29 | 0 | 9.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 71 | 29 | 0 | 9.5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 72 | 29 | 0 | 9.5 | 0 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2G.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.3_4.5-4: UE Power Class 1.5 test requirements for NS_47 (contiguous allocation, large FWA form factor with 20 dB or above antenna isolation) for band n41

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 7.5 | 13 | 0 | 16 | 6 | 3 | 31+TT | 10-TT |
| 2 | 29 | 0 | 0.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 3 | 29 | 0 | 7.5 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16-TT |
| 4 | 29 | 0 | 2 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 5 | 29 | 0 | 2 | 8.5 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 6 | 29 | 0 | 2 | 8.5 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 7 | 29 | 0 | 2 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 8 | 29 | 0 | 2 | 3 | 0 | 26 | 3 | 3 | 31+TT | 23-TT |
| 9 | 29 | 0 | 8 | 13 | 0 | 16 | 6 | 3 | 31+TT | 10-TT |
| 10 | 29 | 0 | 0.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 11 | 29 | 0 | 8 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16-TT |
| 12 | 29 | 0 | 2.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 13 | 29 | 0 | 2.5 | 8.5 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 14 | 29 | 0 | 2.5 | 8.5 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 15 | 29 | 0 | 2.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 16 | 29 | 0 | 2.5 | 3 | 0 | 26 | 3 | 3 | 31+TT | 23-TT |
| 17 | 29 | 0 | 8 | 13 | 0 | 16 | 6 | 3 | 31+TT | 10-TT |
| 18 | 29 | 0 | 1.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 19 | 29 | 0 | 8 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16-TT |
| 20 | 29 | 0 | 3.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 21 | 29 | 0 | 3.5 | 8.5 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 22 | 29 | 0 | 3.5 | 8.5 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 23 | 29 | 0 | 3.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 24 | 29 | 0 | 3.5 | 3 | 0 | 25.5 | 3 | 3 | 31+TT | 22.5-TT |
| 25 | 29 | 0 | 8 | 13 | 0 | 16 | 6 | 3 | 31+TT | 10-TT |
| 26 | 29 | 0 | 3.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 27 | 29 | 0 | 8 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16-TT |
| 28 | 29 | 0 | 4 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 29 | 29 | 0 | 4 | 8.5 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 30 | 29 | 0 | 4 | 8.5 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 31 | 29 | 0 | 4 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 32 | 29 | 0 | 4 | 0 | 0 | 25 | 3 | 3 | 31+TT | 22-TT |
| 33 | 29 | 0 | 8 | 13 | 0 | 16 | 6 | 3 | 31+TT | 10-TT |
| 34 | 29 | 0 | 6.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 35 | 29 | 0 | 8 | 8 | 0 | 21 | 5 | 3 | 31+TT | 16-TT |
| 36 | 29 | 0 | 6.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 37 | 29 | 0 | 6.5 | 8.5 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 38 | 29 | 0 | 6.5 | 8.5 | 0 | 20.5 | 5 | 3 | 31+TT | 15.5-TT |
| 39 | 29 | 0 | 6.5 | 11 | 0 | 18 | 5 | 3 | 31+TT | 13-TT |
| 40 | 29 | 0 | 6.5 | 0 | 0 | 22.5 | 5 | 3 | 31+TT | 17.5-TT |
| 41 | 29 | 0 | 9.5 | 13 | 0 | 16 | 7 | 3 | 31+TT | 9-TT |
| 42 | 29 | 0 | 3.5 | 12.5 | 0 | 16.5 | 7 | 3 | 31+TT | 9.5-TT |
| 43 | 29 | 0 | 9.5 | 8 | 0 | 19.5 | 7 | 3 | 31+TT | 12.5-TT |
| 44 | 29 | 0 | 5 | 12.5 | 0 | 16.5 | 7 | 3 | 31+TT | 9.5-TT |
| 45 | 29 | 0 | 5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 46 | 29 | 0 | 5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 47 | 29 | 0 | 5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 48 | 29 | 0 | 5 | 4 | 0 | 24 | 5 | 3 | 31+TT | 19-TT |
| 49 | 29 | 0 | 9.5 | 13 | 0 | 16 | 6 | 3 | 31+TT | 10-TT |
| 50 | 29 | 0 | 4 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 51 | 29 | 0 | 9.5 | 8 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 52 | 29 | 0 | 5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 53 | 29 | 0 | 5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 54 | 29 | 0 | 5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 55 | 29 | 0 | 5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 56 | 29 | 0 | 5 | 4 | 0 | 24 | 5 | 3 | 31+TT | 19-TT |
| 57 | 29 | 0 | 9.5 | 13 | 0 | 16 | 6 | 3 | 31+TT | 10-TT |
| 58 | 29 | 0 | 7 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 59 | 29 | 0 | 9.5 | 8 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60 | 29 | 0 | 7 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 61 | 29 | 0 | 7 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 62 | 29 | 0 | 7 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 63 | 29 | 0 | 7 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 64 | 29 | 0 | 7 | 0 | 0 | 22 | 5 | 3 | 31+TT | 17-TT |
| 65 | 29 | 0 | 9.5 | 13 | 0 | 16 | 6 | 3 | 31+TT | 10-TT |
| 66 | 29 | 0 | 9.5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 67 | 29 | 0 | 9.5 | 8 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 68 | 29 | 0 | 9.5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 69 | 29 | 0 | 9.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 70 | 29 | 0 | 9.5 | 9.5 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| 71 | 29 | 0 | 9.5 | 12.5 | 0 | 16.5 | 6 | 3 | 31+TT | 10.5-TT |
| 72 | 29 | 0 | 9.5 | 0 | 0 | 19.5 | 5 | 3 | 31+TT | 14.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2G.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2G.3_1.5-5: UE Power Class 1.5 test requirements (NS_50) for band n39

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 8.5 | 6.0 | 0 | 20.5 |  | 5 |  | 3 | 31.0 + TT | 15.5-TT |
| 2 | 29 | 0 | 3.5 | 5.0 | 0 | 24 |  | 5 |  | 3 | 31.0 + TT | 19-TT |
| 3 | 29 | 0 | 2 | 5.0 | 0 | 24 |  | 5 |  | 3 | 31.0 + TT | 19-TT |
| 4 | 29 | 0 | 8.5 | 6.0 | 0 | 20.5 |  | 5 |  | 3 | 31.0 + TT | 15.5-TT |
| 5 | 29 | 0 | 3.5 | 6.5 | 0 | 22.5 |  | 5 |  | 3 | 31.0 + TT | 17.5-TT |
| 6 | 29 | 0 | 2 | 5.0 | 0 | 24 |  | 5 |  | 3 | 31.0 + TT | 19-TT |
| 7 | 29 | 0 | 2 | 3.0 | 0 | 26 |  | 3 |  | 3 | 31.0 + TT | 23-TT |
| 8 | 29 | 0 | 8.5 | 6.0 | 0 | 20.5 |  | 5 |  | 3 | 31.0 + TT | 15.5-TT |
| 9 | 29 | 0 | 3.5 | 6.5 | 0 | 22.5 |  | 5 |  | 3 | 31.0 + TT | 17.5-TT |
| 10 | 29 | 0 | 2 | 5.0 | 0 | 24 |  | 5 |  | 3 | 31.0 + TT | 19-TT |
| 11 | 29 | 0 | 2 | 3.0 | 0 | 26 |  | 3 |  | 3 | 31.0 + TT | 23-TT |
| 12 | 29 | 0 | 3.5 | 8.5 | 0 | 20.5 |  | 5 |  | 3 | 31.0 + TT | 15.5-TT |
| 13 | 29 | 0 | 8.5 | 6.0 | 0 | 20.5 |  | 5 |  | 3 | 31.0 + TT | 15.5-TT |
| 14 | 29 | 0 | 2 | 5.0 | 0 | 24 |  | 5 |  | 3 | 31.0 + TT | 19-TT |
| 15 | 29 | 0 | 2 | 3.5 | 0 | 25.5 |  | 3 |  | 3 | 31.0 + TT | 22.5-TT |
| 16 | 29 | 0 | 2 | 5.0 | 0 | 24 |  | 5 |  | 3 | 31.0 + TT | 19-TT |
| 17 | 29 | 0 | 2 | 3.0 | 0 | 26 |  | 3 |  | 3 | 31.0 + TT | 23-TT |
| 18 | 29 | 0 | 3.5 | 8.5 | 0 | 20.5 |  | 5 |  | 3 | 31.0 + TT | 15.5-TT |
| 19 | 29 | 0 | 8.5 | 6.0 | 0 | 20.5 |  | 5 |  | 3 | 31.0 + TT | 15.5-TT |
| 20 | 29 | 0 | 2 | 3.5 | 0 | 25.5 |  | 3 |  | 3 | 31.0 + TT | 22.5-TT |
| 21 | 29 | 0 | 2 | 5.0 | 0 | 24 |  | 5 |  | 3 | 31.0 + TT | 19-TT |
| 22 | 29 | 0 | 2 | 5.0 | 0 | 24 |  | 5 |  | 3 | 31.0 + TT | 19-TT |
| 23 | 29 | 0 | 2 | 3.0 | 0 | 26 |  | 3 |  | 3 | 31.0 + TT | 23-TT |
| 24 | 29 | 0 | 3.5 | 14.5 | 0 | 14.5 |  | 6 |  | 3 | 31.0 + TT | 8.5-TT |
| 25 | 29 | 0 | 2 | 6.5 | 0 | 22.5 |  | 5 |  | 3 | 31.0 + TT | 17.5-TT |
| 26 | 29 | 0 | 2 | 12.5 | 0 | 16.5 |  | 6 |  | 3 | 31.0 + TT | 10.5-TT |
| 27 | 29 | 0 | 2 | 5.0 | 0 | 24 |  | 5 |  | 3 | 31.0 + TT | 19-TT |
| 28 | 29 | 0 | 2 | 10.5 | 0 | 18.5 |  | 5 |  | 3 | 31.0 + TT | 13.5-TT |
| 29 | 29 | 0 | 3.5 | 14.5 | 0 | 14.5 |  | 6 |  | 3 | 31.0 + TT | 8.5-TT |
| 30 | 29 | 0 | 2 | 12.5 | 0 | 16.5 |  | 6 |  | 3 | 31.0 + TT | 10.5-TT |
| 31 | 29 | 0 | 2 | 6.5 | 0 | 22.5 |  | 5 |  | 3 | 31.0 + TT | 17.5-TT |
| 32 | 29 | 0 | 2 | 5.5 | 0 | 23.5 |  | 5 |  | 3 | 31.0 + TT | 18.5-TT |
| 33 | 29 | 0 | 2 | 5.0 | 0 | 24 |  | 5 |  | 3 | 31.0 + TT | 19-TT |
| 34 | 29 | 0 | 2 | 10.5 | 0 | 18.5 |  | 5 |  | 3 | 31.0 + TT | 13.5-TT |
| 35 | 29 | 0 | 9.5 | 6.0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 36 | 29 | 0 | 7 | 5.0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 37 | 29 | 0 | 7 | 5.0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 38 | 29 | 0 | 9.5 | 6.0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 39 | 29 | 0 | 7 | 6.5 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 40 | 29 | 0 | 7 | 5.0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 41 | 29 | 0 | 7 | 0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 42 | 29 | 0 | 9.5 | 6.0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 43 | 29 | 0 | 7 | 6.5 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 44 | 29 | 0 | 7 | 5.0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 45 | 29 | 0 | 7 | 0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 46 | 29 | 0 | 7 | 8.5 | 0 | 20.5 |  | 5 |  | 3 | 31.0 + TT | 15.5-TT |
| 47 | 29 | 0 | 9.5 | 6.0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 48 | 29 | 0 | 7 | 5.0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 49 | 29 | 0 | 7 | 0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 50 | 29 | 0 | 7 | 5.0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 51 | 29 | 0 | 7 | 0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 52 | 29 | 0 | 7 | 8.5 | 0 | 20.5 |  | 5 |  | 3 | 31.0 + TT | 15.5-TT |
| 53 | 29 | 0 | 9.5 | 6.0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 54 | 29 | 0 | 7 | 0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 55 | 29 | 0 | 7 | 5.0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 56 | 29 | 0 | 7 | 5.0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 57 | 29 | 0 | 7 | 0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 58 | 29 | 0 | 7 | 14.5 | 0 | 14.5 |  | 6 |  | 3 | 31.0 + TT | 8.5-TT |
| 59 | 29 | 0 | 7 | 6.5 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 60 | 29 | 0 | 7 | 12.5 | 0 | 16.5 |  | 6 |  | 3 | 31.0 + TT | 10.5-TT |
| 61 | 29 | 0 | 7 | 5.0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 62 | 29 | 0 | 7 | 10.5 | 0 | 18.5 |  | 5 |  | 3 | 31.0 + TT | 13.5-TT |
| 63 | 29 | 0 | 7 | 14.5 | 0 | 14.5 |  | 6 |  | 3 | 31.0 + TT | 8.5-TT |
| 64 | 29 | 0 | 7 | 12.5 | 0 | 16.5 |  | 6 |  | 3 | 31.0 + TT | 10.5-TT |
| 65 | 29 | 0 | 7 | 6.5 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 66 | 29 | 0 | 7 | 5.5 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 67 | 29 | 0 | 7 | 5.0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 68 | 29 | 0 | 7 | 10.5 | 0 | 18.5 |  | 5 |  | 3 | 31.0 + TT | 13.5-TT |
| 69 | 29 | 0 | 9.5 | 6.5 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 70 | 29 | 0 | 5 | 7.0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 71 | 29 | 0 | 3.5 | 7.0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 72 | 29 | 0 | 9.5 | 6.5 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 73 | 29 | 0 | 5 | 8.5 | 0 | 20.5 |  | 5 |  | 3 | 31.0 + TT | 15.5-TT |
| 74 | 29 | 0 | 3.5 | 7.0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 75 | 29 | 0 | 3.5 | 4.5 | 0 | 24.5 |  | 3 |  | 3 | 31.0 + TT | 21.5-TT |
| 76 | 29 | 0 | 9.5 | 6.5 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 77 | 29 | 0 | 5 | 8.5 | 0 | 20.5 |  | 5 |  | 3 | 31.0 + TT | 15.5-TT |
| 78 | 29 | 0 | 3.5 | 7.0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 79 | 29 | 0 | 3.5 | 4.5 | 0 | 24.5 |  | 3 |  | 3 | 31.0 + TT | 21.5-TT |
| 80 | 29 | 0 | 5 | 10.0 | 0 | 19 |  | 5 |  | 3 | 31.0 + TT | 14-TT |
| 81 | 29 | 0 | 9.5 | 6.5 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 82 | 29 | 0 | 3.5 | 7.0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 83 | 29 | 0 | 3.5 | 5.0 | 0 | 24 |  | 5 |  | 3 | 31.0 + TT | 19-TT |
| 84 | 29 | 0 | 3.5 | 7.0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 85 | 29 | 0 | 3.5 | 4.5 | 0 | 24.5 |  | 3 |  | 3 | 31.0 + TT | 21.5-TT |
| 86 | 29 | 0 | 5 | 10.0 | 0 | 19 |  | 5 |  | 3 | 31.0 + TT | 14-TT |
| 87 | 29 | 0 | 9.5 | 6.5 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 88 | 29 | 0 | 3.5 | 5.0 | 0 | 24 |  | 5 |  | 3 | 31.0 + TT | 19-TT |
| 89 | 29 | 0 | 3.5 | 7.0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 90 | 29 | 0 | 3.5 | 7.0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 91 | 29 | 0 | 3.5 | 4.5 | 0 | 24.5 |  | 3 |  | 3 | 31.0 + TT | 21.5-TT |
| 92 | 29 | 0 | 5 | 15.5 | 0 | 13.5 |  | 6 |  | 3 | 31.0 + TT | 7.5-TT |
| 93 | 29 | 0 | 3.5 | 8.5 | 0 | 20.5 |  | 5 |  | 3 | 31.0 + TT | 15.5-TT |
| 94 | 29 | 0 | 3.5 | 13.5 | 0 | 15.5 |  | 6 |  | 3 | 31.0 + TT | 9.5-TT |
| 95 | 29 | 0 | 3.5 | 7.0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 96 | 29 | 0 | 3.5 | 10.5 | 0 | 18.5 |  | 5 |  | 3 | 31.0 + TT | 13.5-TT |
| 97 | 29 | 0 | 5 | 15.5 | 0 | 13.5 |  | 6 |  | 3 | 31.0 + TT | 7.5-TT |
| 98 | 29 | 0 | 3.5 | 13.5 | 0 | 15.5 |  | 6 |  | 3 | 31.0 + TT | 9.5-TT |
| 99 | 29 | 0 | 3.5 | 8.5 | 0 | 20.5 |  | 5 |  | 3 | 31.0 + TT | 15.5-TT |
| 100 | 29 | 0 | 3.5 | 7.0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 101 | 29 | 0 | 3.5 | 7.0 | 0 | 22 |  | 5 |  | 3 | 31.0 + TT | 17-TT |
| 102 | 29 | 0 | 3.5 | 10.5 | 0 | 18.5 |  | 5 |  | 3 | 31.0 + TT | 13.5-TT |
| 103 | 29 | 0 | 9.5 | 0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 104 | 29 | 0 | 9.5 | 7.0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 105 | 29 | 0 | 9.5 | 7.0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 106 | 29 | 0 | 9.5 | 0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 107 | 29 | 0 | 9.5 | 8.5 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 108 | 29 | 0 | 9.5 | 7.0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 109 | 29 | 0 | 9.5 | 0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 110 | 29 | 0 | 9.5 | 0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 111 | 29 | 0 | 9.5 | 8.5 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 112 | 29 | 0 | 9.5 | 7.0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 113 | 29 | 0 | 9.5 | 0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 114 | 29 | 0 | 9.5 | 10.0 | 0 | 19 |  | 5 |  | 3 | 31.0 + TT | 14-TT |
| 115 | 29 | 0 | 9.5 | 0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 116 | 29 | 0 | 9.5 | 7.0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 117 | 29 | 0 | 9.5 | 0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 118 | 29 | 0 | 9.5 | 7.0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 119 | 29 | 0 | 9.5 | 0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 120 | 29 | 0 | 9.5 | 10.0 | 0 | 19 |  | 5 |  | 3 | 31.0 + TT | 14-TT |
| 121 | 29 | 0 | 9.5 | 0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 122 | 29 | 0 | 9.5 | 0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 123 | 29 | 0 | 9.5 | 7.0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 124 | 29 | 0 | 9.5 | 7.0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 125 | 29 | 0 | 9.5 | 0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 126 | 29 | 0 | 9.5 | 15.5 | 0 | 13.5 |  | 6 |  | 3 | 31.0 + TT | 7.5-TT |
| 127 | 29 | 0 | 9.5 | 8.5 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 128 | 29 | 0 | 9.5 | 13.5 | 0 | 15.5 |  | 6 |  | 3 | 31.0 + TT | 9.5-TT |
| 129 | 29 | 0 | 9.5 | 7.0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 130 | 29 | 0 | 9.5 | 10.5 | 0 | 18.5 |  | 5 |  | 3 | 31.0 + TT | 13.5-TT |
| 131 | 29 | 0 | 9.5 | 15.5 | 0 | 13.5 |  | 6 |  | 3 | 31.0 + TT | 7.5-TT |
| 132 | 29 | 0 | 9.5 | 13.5 | 0 | 15.5 |  | 6 |  | 3 | 31.0 + TT | 9.5-TT |
| 133 | 29 | 0 | 9.5 | 8.5 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 134 | 29 | 0 | 9.5 | 7.0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 135 | 29 | 0 | 9.5 | 7.0 | 0 | 19.5 |  | 5 |  | 3 | 31.0 + TT | 14.5-TT |
| 136 | 29 | 0 | 9.5 | 10.5 | 0 | 18.5 |  | 5 |  | 3 | 31.0 + TT | 13.5-TT |

The test requirements is adapted by ΔTIB,c as specified in clause 6.0.

< End of Change1 >

<Start of Change2 >

Table 6.5G.2.2.5.4-1: Additional requirements for "NS_06"

| Spectrum emission limit (dBm) / Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | Measurement bandwidth |
| ± 0 – 0.1 | -15 + TT | -18 + TT | -20 + TT | 30 kHz |
| ± 0.1 – 1 | -13 + TT | -13 + TT | -13 + TT | 100 kHz |
| ± 1 – 6 | -13 + TT | -13 + TT | -13 + TT | 1 MHz |
| ± 6 – 10 | -25 + TT | -13 + TT | -13 + TT | 1 MHz |
| ± 10 – 15 |  | -25 + TT | -13 + TT | 1 MHz |
| ± 15 – 20 |  |  | -25 + TT | 1 MHz |
| NOTE 1: TT for each frequency and channel bandwidth is specified in Table 6.5G.2.2.5-1. |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

6.5G.2.2_1	Additional spectrum emission mask for Tx Diversity for UE supporting 4Tx

Editor’s Note: This clause is incomplete. The following aspects are either missing or not yet determined:

- MU and TT are FFS.

6.5G.2.2_1.1	Test purpose

The test purpose same as in 6.5.2.3.1.

6.5G.2.2_1.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE release 18 and forward that support 4Tx Tx diversity.

6.5G.2.2_1.3	Minimum conformance requirements

The minimum conformance requirements same as in 6.5G.2.2.3.

6.5G.2.2_1.4	Test description

6.5G.2.2_1.4.1	Initial conditions

Same Initial conditions as in 6.5G.2.2.4.1 with the following exceptions:

- Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.2a for TE diagram and section A.3.2 for UE diagram.

6.5G.2.2_1.4.2	Test procedure

Same test procedure as specified in clause 6.5.2.3.4 with following exceptions:

Step 3 and 4 of Test procedure as in 6.5.2.3.4.2 is replaced by:

3.	Measure the mean power of the UE as the sum of the power from all UE transmit antenna connectors in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in applicable table of clause 6.2G.3_1.5. The period of measurement shall be at least the continuous duration one sub-frame (1ms). For TDD slots with transient periods are not under test.

4.	Measure the power of the transmitted signal as the sum of the power from all UE transmit antenna connectors with a measurement filter of bandwidths according to applicable test configuration tables in subclause 6.5G.2.2_1.5 and using a rms detector. If the sweep count is higher than one, the trace mode shall be average. The centre frequency of the filter shall be stepped in continuous steps according to the applicable test requirement table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

6.5G.2.2_1.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6, with the following exceptions for each network signalling value.

6.5G.2.2_1.4.3.2	Message contents exceptions (network signalling value "NS_04")

For "NS_04" see A-MPR test case in table 6.2G.3_1.4.3.1-1.

6.5G.2.2_1.5	Test requirement

Table 6.5G.2.2_1.5-1: Test Tolerance (Additional spectrum emission mask)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 100MHz | FFS | FFS | FFS |

6.5G.2.2_1.5.1	Test requirements (network signalling value "NS_04")

When "NS_04" is indicated in the cell:

-	the measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements inTable 6.2G.3_1.5-1 for UE power class 1.5.

and

-	the power of any UE emission shall fulfil requirements in table 6.5G.2.2_1.5.1-1.

Table 6.5G.2.2_1.5.1-1: Additional test requirements for "NS_04"

| ΔfOOB MHz |  | Spectrum emission limit (dBm) / measurement bandwidth for each channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 15 | 20 | 30 | 40 | 50 |  | 60 | 70 | 80 | 90 | 100 |  |
| ± 0 - 1 | -10+TT | -10+TT | -10+TT | -10+TT | -10+TT |  |  |  |  |  |  |  | 2 % channel bandwidth |
|  |  |  |  |  |  |  | -10 |  |  |  |  |  | 1 MHz |
| ± 1 - 5 |  | -10 + TT |  |  |  |  |  |  |  |  |  |  | 1 MHz |
| ± 5 - X |  | -13 + TT |  |  |  |  |  |  |  |  |  |  |  |
| ± X - (BWChannel + 5 MHz) |  | -25 + TT |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: X is defined in Table 6.5.2.3.3.2-1 for CP-OFDM and 6.5.2.3.3.2-2 for DFT-S-OFDM. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.5G.2.2.5-1. |  |  |  |  |  |  |  |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement.

< End of Change2 >

<Start of Change3 >

6.5G.3.3_1	Additional spurious emissions for Tx Diversity for UE supporting 4Tx

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- MU and TT are FFS.

6.5G.3.3_1.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

6.5G.3.3_1.2	Test applicability

This test case applies to all types of NR Power Class 1.5 UE release 18 and forward that support 4Tx Tx diversity.

6.5G.3.3_1.3	Minimum conformance requirements

The minimum conformance requirements same as in 6.5G.3.3.3.

6.5G.3.3_1.4	Test description

6.5G.3.3_1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All these configurations shall be tested with applicable test parameters for each channel bandwidth and sub-carrier spacing, are shown in Tables 6.5G.3.3_1.4.1-1 through Table 6.5G.3.3_1.4.1-3 for different NS values. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.5G.3.3_1.4.1-1: Test Configuration Table (network signalling value "NS_04")

Same test configuration as listed in Table 6.2.3.4.1-2 shall be used.

Table 6.5G.3.3_1.4.1-2: Test Configuration Table (network signalling value "NS_47")

Same test configuration as listed in Table 6.2G.3.4.1-1 shall be used.

Table 6.5G.3.3_1.4.1-3: Test Configuration Table (network signalling value "NS_50")

Same test configuration as listed in Table 6.2G.3.4.1-2 shall be used.

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.2.2a for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement channels are set according to the table in clause 6.5G.3.3_1.4.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5.  Message contents are defined in clause 6.5.3.3.4.3.

6.5G.3.3_1.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.5G.3.3_1.4.1-1 through Table 6.5G.3.3_1.4.1-3. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the sum of the mean power of the UE at each antenna connector in channel bandwidth of radio access mode according to the test configuration in Table 6.5G.3.3_1.4.1-1 through Table 6.5G.3.3_1.4.1-3 as appropriate, which shall meet the requirements in clause 6.5G.3.3_1.5 with allowed A-MPR values specified in 6.2G.3_1.5. The period of measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots and uplink symbols. For TDD, only slots consisting of only UL symbols are under test.

4.	Measure the sum power of the transmitted signal at all antenna connectors with a measurement filter of bandwidths according to clauses 6.5G.3.3_1.3.1 to 6.5G.3.3_1.3.3 as appropriate. The centre frequency of the filter shall be stepped in contiguous steps according to the same table the measured power shall be verified for each step. The measurement period shall capture the active time slots.During measurement the spectrum analyser shall be set to 'Detector' = RMS.

6.5G.3.3_1.4.3	Message contents

6.5G.3.3_1.4.3.1	Message contents exceptions (network signalling value "NS_04")

Same message contents exceptions as 6.5.3.3.4.3.1.

6.5G.3.3_1.4.3.2	Message contents exceptions (network signalling value "NS_47")

Same message contents exceptions as 6.5.3.3.4.3.15.

6.5G.3.3_1.4.3.3	Message contents exceptions (network signalling value "NS_50")

Same message contents exceptions as 6.5.3.3.4.3.16.

6.5G.3.3_1.5	Test requirement

This clause specifies the requirements for the specified NR band for an additional spectrum emission requirement with protected bands as indicated from Table 6.5G.3.3_1.5.1-1 to Table 6.5G.3.3_1.5.1-3 for different NS_values.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

6.5G.3.3_1.5.1	Test requirement (network signalling value “NS_04”)

When "NS_04" is indicated in the cell,

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Table 6.2G.3_1.5-1 for power class 1.5 UE.

The power of any UE emission shall not exceed the levels specified in Table 6.5G.3.3_1.5.1-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1.3-1 from the edge of the channel bandwidth.

Table 6.5G.3.3_1.5.1-1: Additional requirements for "NS_04"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 (MHz) |  |
| 2495 ≤ f < 2496 | -13 | 1% of Channel BW |
| 2490.5 ≤ f < 2495 | -13 | 1 MHz |
| 0.009 < f < 2490.5 | -25 | 1 MHz |

6.5G.3.3_1.5.2	Test requirement (network signalling value “NS_47”)

Same test requirement as 6.5.3.3.5.15.

6.5G.3.3_1.5.3	Test requirement (network signalling value “NS_50”)

Same test requirement as 6.5.3.3.5.16.

< End of Change3 >