3GPP TSG-RAN5 Meeting #108 	R5-255414r1

# Bengaluru, India, 25th August 2025 – 29th August  2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4105 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Core alignment and update for NR-NTN Cell reselection test cases. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Korea |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Core alignment and Message contents update is needed for NR-NTN Cell reselection test cases. Some of them are now completed, and editor’s notes can be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Aligned NR-NTN Cell reselection test cases with the latest version of 38.133. Added missing Message contents and updated few other aspects. - Removed editor’s notes from completed test cases - Reverted RAN4 dependent changes in General test parameter table of TC 14.1.5 - Reverted RAN4 dependent test procedure updates for TC 14.1.5 and updated Editor’s note. - Clarified the reason for marking 14.1.5 test case incomplete. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases would remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.1.1,14.1.2,14.1.3,14.1.4,14.1.5,14.1.6,14.1.7,14.1.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | RAN4 dependent: R4-2511501 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-254685r5 R1- Reverted RAN4 CR related contents due to RAN4 CR postponement in post meeting period |  |  |  |  |  |  |  |  |

<< Start of Changes >>

## 14.1	RRC_IDLE state mobility

### 14.1.0	Minimum conformance requirements

<< Skipped sections >>

### 14.1.1	NR SA FR1 Cell Reselection for Satellite Access

14.1.1.1	Test purpose

This test is to verify the requirement for the intra-frequency NR cell reselection requirements for satellite access specified in clause 14.1.0.1.

14.1.1.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access.

14.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.14.1.1.

14.1.1.4	Test description

The test scenario comprises of 1 NR carrier and 2 cells as given in tables 14.1.1.4.1-1, 14.1.1.4.1-3 and 14.1.1.5-1. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Only Cell 1 is already identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing Cell 2.

14.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.1.1.4.1-1.

Table 14.1.1.4.1-1: Supported test configurations for NR SA FR1 Cell reselection for Satellite Access

| Configuration | Description |
| --- | --- |
| 14.1.1-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.1.1-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. |  |

Configure the test equipment and the DUT according to the parameters in Table 14.1.1.4.1-2

Table 14.1.1.4.1-2: Initial conditions for NR SA FR1 Cell reselection for Satellite Access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.12-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.1.1.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 14.1.1.4.1-3.

2.	Message contents are defined in clause 14.1.1.4.3.

3.	Cell 1 and Cell 2 are NR cells with the power level set according to clauses C.1.2 and C.1.3 for this test. Both Cell 1 and Cell 2 are satellite access cells.

4.	The initial test environment conditions are setup according to section 14.0.5.

Table 14.1.1.4.1-3: General test parameters for NR SA FR1 Cell reselection for Satellite Access

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell 1 |  |
| T2 end condition | Active cell |  | Cell 2 |  |
|  | Neighbour cells |  | Cell 1 |  |
| Final condition | Active cell |  | Cell 1 |  |
|  | Neighbour cells |  | Cell 2 |  |
| RF Channel Number |  |  | 1 |  |
| Time offset between cells |  | ms | 3 | Asynchronous cells |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 102 | The detailed configuration is specified in TS 38.211 [7] clause 6.3.3.2 |
| rangeToBestCell |  |  | Not configured |  |
| Ephemeris information |  |  | Note 1 |  |
| T1 |  | s | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 40 (NOTE 1) | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 15 (NOTE 1) | T3 needs to be defined so that cell re-selection reaction time is taken into account. |
| Note 1: If the test is performed in a NGSO configuration, and the scaling factor Kmulti_SMTC defined in clause 14.1.0.1 is greater than 1, according to UE capabilities, the duration of times T2 and T3 shall be scaled for the same factor to allow the UE to complete the cell reselection within the duration of the test case. Note 2: Detailed ephemeris information is provided in TS 38.508-1 [14] |  |  |  |  |

14.1.1.4.2	Test procedure

The test scenario comprises of two satellite access NR FDD intra-frequency cells as given in table 14.1.1.4.1-3. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing Cell 2.

In the following test procedure "UE responds" means "UE starts transmitting preamble on PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility”.

For NGSO configurations, the number of LEO satellites that a UE can measure in parallel within an SMTC is a capability the UE reports via maxNumber-NGSO-SatellitesWithinOneSMTC-r17. This value will determine the requirement to be used in the test, as well as the duration of T2 and T3.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR and Test Mode On according to TS 38.508-1 [14] clause 4.5. Cell 1 is an active cell and Cell 2 is powered off.

2.	Set the parameters according to T1 in Table 14.1.1.5-1. Propagation conditions are set according to Annex C clause C.2.2. T1 starts.

3.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for this iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.1.1.5-1. T2 starts.

5.	The SS waits for random access requests information from the UE to perform cell re-selection to a newly detectable cell, Cell 2.

6.	If the UE responds on the newly detectable cell, Cell 2 during time duration T2 within 34 seconds for GSO configurations or for NGSO configurations for UEs supporting measurement of only one LEO satellite in parallel within an SMTC, or within 66 seconds for NGSO configurations for UEs supporting measurement of two or more LEO satellites in parallel within an SMTC, from the beginning of time period T2, then count a success for the event “Re-select newly detected Cell 2”. Otherwise count a fail for the event “Re-select newly detected Cell 2”.

7.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, continue with step 7a.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and skip to step 1.

7a	The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 2.

8.	The SS shall switch the power setting from T2 to T3 as specified in Table 14.1.1.5-1. T3 starts.

9.	The SS waits for random access requests information from the UE to perform cell re-selection to an already detected cell, Cell 1.

10.	If the UE responds on the already detected cell, Cell 1 during time duration T3 within 8 seconds for GSO configurations or for NGSO configurations for UEs supporting measurement of only one LEO satellite in parallel within an SMTC, or within 14.5 seconds for NGSO configurations for UEs supporting measurement of two or more LEO satellites in parallel within an SMTC, from the beginning of time period T3, then count a success for the event “Re-select already detected Cell 1”. Otherwise count a fail for the event “Re-select already detected Cell 1”.

11.	If the UE has re-selected to Cell 1 within T3, after the re-selection or when T3 expires, the SS shall send an RRCRelease message to the UE and then proceed with step 1 for the next iteration. Otherwise proceed with step 11a.

11a.The SS shall switch off and on the UE, and skip to step 1 for the next iteration.

12.	Repeat step 2-11 until a test verdict has been achieved. Each of the events “Re-select newly detected Cell 2” and “Re-select already detected Cell 1” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict. If both events pass, the test passes. If one event fails, the test fails.

14.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 14.1.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.1-2 |
| Default RRC messages and information elements contents exceptions | Table H.2.4-1 Table H.2.4-2 with Condition GSO for Config 1 or Condition NGSO for Config 2 Table H.2.4-3 |

Table 14.1.1.4.3-2: SIB1 (Cell 1 and Cell 2)

| Derivation Path: Table H.2.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |  |
| q-RxLevMin |  | -65 | -130 is actual value in dBm (-65 * 2 dBm) | 14.1.1-1, 14.1.1-2 |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.1.1.4.3-3: SIB2 (Cell 1 and Cell 2)

| Derivation Path: Table H.2.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -65 | Actual value is -65*2 = -130dBm | 14.1.1-1, 14.1.1-2 |
| smtc | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.2, SMTC.6 |  | 14.1.1-1, 14.1.1-2 AND Cell 1 |
|  | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.5, SMTC.2 |  | 14.1.1-1, 14.1.1-2 AND Cell 2 |
| deriveSSB-IndexFromCell | false |  | 14.1.1-1, 14.1.1-2 |
| } |  |  |  |
| } |  |  |  |

14.1.1.5	Test requirements

Table 14.1.1.5-1 defines the primary level settings including test tolerances for NR SA FR1 Cell reselection for Satellite Access.

Table 14.1.1.5-1: Cell specific parameters for NR SA FR1 Cell reselection for Satellite Access

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Satellite information |  | SSC.1 for Config 1 SSC.2 for Config 2 |  |  | NSC.1 for Config 1 NSC.2 for Config 2 |  |  |
| PDSCH RMC configuration |  | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| RMSI CORESET configuration |  | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| Dedicated CORESET configuration |  | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| OCNG Pattern |  | OP.1 defined in A.2.1 |  |  | OP.1 defined in A.2.1 |  |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| SSB configuration |  | SSB.1 FR1 |  |  | SSB.5 FR1 |  |  |
| SMTC configuration |  | #1: SMTC.2 for Cell 1 #2: SMTC.5 for Cell 2 |  |  | #1: SMTC.6 for Cell 1 #2: SMTC.2 for Cell 2 |  |  |
| RLM-RS |  | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | -130 |  |  | -130 |  |  |
| Pcompensation | dB | 0 |  |  | 0 |  |  |
| Qhysts | dB | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 16 | -3.55 | 3.24 | -infinity | 3.24 | -3.55 |
| Note2 | dBm/SCS | -98 |  |  |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |  |  |  |
|  | dB | 16 | 13 | 16.45 | -infinity | 16.45 | 13 |
| SS-RSRP Note3 | dBm/SCS | -82 | -85 | -81.55 | -infinity | -81.55 | -85 |
| Io | dBm/9.36 MHz | -53.94 | -51.91 | -51.91 | Same as parameters specified in Cell 1 columns- |  |  |
| Treselection | s | 0 | 0 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 60 |  |  | 60 |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 2.

The cell re-selection delay to a newly detectable cell shall be less than:

34 s if Kmulti_SMTC  is  equal to 1 (see note on Table 14.1.1.4.1-3); or

66 s if Kmulti_SMTC is equal to 2.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on Cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 1.

The cell re-selection delay to an already detected cell shall be less than:

8 s if Kmulti_SMTC  is  equal to 1 (see note on Table 14.1.1.4.1-3); or

14.5 s if Kmulti_SMTC is equal to 2.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Kmulti_SMTC *Tdetect, NR_Intra + TSI-NR, and to an already detected cell can be expressed as: Kmulti_SMTC *Tevaluate, NR_intra + TSI-NR,

Where:

Tdetect, NR_Intra	See Table 14.1.0.1-1 in clause 14.1.0.1

Tevaluate, NR_intra	See Table 14.1.0.1-1 in clause 14.1.0.1

Kmulti_SMTC  is described in clause 14.1.0.1

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280ms is assumed in this test case provided that SIB1 and SIB19 are scheduled with 20ms period and 80 ms period, respectively.

If Kmulti_SMTC = 1, Kmulti_SMTC *Tevaluate, NR_Intra + TSI-NR = 7.68 s; allow 8 s. And Kmulti_SMTC *Tdetect, NR_intra + TSI-NR = 33.28 s, allow 34s.

If K_multi_SMTC = 2,  Kmulti_SMTC *Tevaluate, NR_Intra + TSI-NR = 14.08 s; allow 14.5 s. And Kmulti_SMTC *Tdetect, NR_intra + TSI-NR = 65.28 s, allow 66s.

### 14.1.2	NR SA FR1 Cell Reselection for UE configured with the feature for enhanced requirements for Satellite Access

Editor's Note: This test case is incomplete due to following aspects.

- Test parameter and configuration  may need to be updated

14.1.2.1	Test purpose

This test is to verify the requirement for the intra-frequency NR cell reselection requirements for satellite access specified in clause 14.1.0.1.

14.1.2.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access and enhanced RRM idle mode measurement .

14.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.14.1.2.

14.1.2.4	Test description

The test scenario comprises of 1 NR carrier and 2 cells as given in tables 14.1.2.4.1-1, 14.1.2.4.1-3 and 14.1.2.5-1. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2. The flag enhancedMeasurementNGSO-r17 should be set.

14.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.1.2.4.1-1.

Table 14.1.2.4.1-1: Supported test configurations for NR SA FR1 Cell Reselection for UE configured with the feature for enhanced requirements for Satellite Access

| Configuration | Description |
| --- | --- |
| 14.1.2-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.1.2-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 14.1.2.4.1-2

Table 14.1.2.4.1-2: Initial conditions for NR SA FR1 Cell Reselection for UE configured with the feature for enhanced requirements for Satellite Access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.12-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.1.2.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 14.1.2.4.1-3.

2.	Message contents are defined in clause 14.1.2.4.3.

3.	Cell 1 and Cell 2 are NR cells with the power level set according to clauses C.1.2 and C.1.3 for this test. Both Cell 1 and Cell 2 are satellite access cells.

4.	The initial test environment conditions are setup according to section 14.0.5.

Table 14.1.2.4.1-3: General test parameters for NR SA FR1 Cell Reselection for UE configured with the feature for enhanced requirements for Satellite Access

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell1 |  |
| T2 end condition | Active cell |  | Cell2 |  |
|  | Neighbour cells |  | Cell1 |  |
| Final condition | Active cell |  | Cell1 |  |
|  | Neighbour cells |  | Cell2 |  |
| RF Channel Number |  |  | 1 |  |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 102 | The detailed configuration is specified in TS 38.211 [7] clause 6.3.3.2 |
| rangeToBestCell |  |  | Not configured |  |
| T1 |  | s | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 40 (NOTE 1) | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 15 (NOTE 1) | T3 needs to be defined so that cell re-selection reaction time is taken into account. |
| NOTE 1: If the test is performed in a NGSO configuration, and the scaling factor Kmulti_SMTC defined in clause 14.1.0.1 is greater than 1, according to UE capabilities, the duration of times T2 and T3 shall be scaled for the same factor to allow the UE to complete the cell reselection within the duration of the test case. |  |  |  |  |

14.1.2.4.2	Test procedure

The test scenario comprises of two satellite access NR FDD intra-frequency cells as given in table 14.1.2.4.1-3. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas.

In the following test procedure "UE responds" means "UE starts transmitting preamble on PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility”.

For NGSO configurations, the number of LEO satellites that a UE can measure in parallel within an SMTC is a capability the UE reports via maxNumber-NGSO-SatellitesWithinOneSMTC-r17. This value will determine the requirement to be used in the test, as well as the duration of T2 and T3.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR and Test Mode On according to TS 38.508-1 [14] clause 4.5. Cell 1 is an active cell and Cell 2 is powered off.

2.	Set the parameters according to T1 in Table 14.1.2.5-1. Propagation conditions are set according to Annex C clause C.2.2. T1 starts.

3.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for this iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.1.2.5-1. T2 starts.

5.	The SS waits for random access requests information from the UE to perform cell re-selection to a newly detectable cell, Cell 2.

6.	If the UE responds on the newly detectable cell, Cell 2 during time duration T2 within 11 seconds for GSO configurations or for NGSO configurations for UEs supporting measurement of only one LEO satellite in parallel within an SMTC, or within 20 seconds for NGSO configurations for UEs supporting measurement of two or more LEO satellites in parallel within an SMTC, from the beginning of time period T2, then count a success for the event “Re-select newly detected Cell 2”. Otherwise count a fail for the event “Re-select newly detected Cell 2”.

7.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, continue with step 7a.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and skip to step 11a.

7a	The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 2.

8.	The SS shall switch the power setting from T2 to T3 as specified in Table 14.1.2.5-1. T3 starts.

9.	The SS waits for random access requests information from the UE to perform cell re-selection to an already detected cell, Cell 1.

10.	If the UE responds on the already detected cell, Cell 1 during time duration T3 within 6 seconds for GSO configurations or for NGSO configurations for UEs supporting measurement of only one LEO satellite in parallel within an SMTC, or within 9 seconds for NGSO configurations for UEs supporting measurement of two or more LEO satellites in parallel within an SMTC, from the beginning of time period T3, then count a success for the event “Re-select already detected Cell 1”. Otherwise count a fail for the event “Re-select already detected Cell 1”.

11.	If the UE has re-selected to Cell 1 within T3, after the re-selection or when T3 expires, the SS shall send an RRCRelease message to the UE and then proceed with step 1 for the next iteration. Otherwise proceed with step 11a.

11a.The SS shall switch off and on the UE, and skip to step 1 for the next iteration.

12.	Repeat step 2-11 until a test verdict has been achieved. Each of the events “Re-select newly detected Cell 2” and “Re-select already detected Cell 1” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict. If both events pass, the test passes. If one event fails, the test fails.

14.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 14.1.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.1-2 |
| Default RRC messages and information elements contents exceptions | Table H.2.4-1 Table H.2.4-2 with Condition GSO for Config 1 or Condition NGSO for Config 2 Table H.2.4-3 |

Table 14.1.2.4.3-2: SIB1 (Cell 1 and Cell 2)

| Derivation Path: Table H.2.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |  |
| q-RxLevMin |  | -65 | -130 is actual value in dBm (-65 * 2 dBm) | 14.1.2-1, 14.1.2-2 |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.1.2.4.3-3: SIB2 (Cell 1 and Cell 2)

| Derivation Path: Table H.2.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -65 | Actual value is -65*2 = -130dBm | 14.1.2-1, 14.1.2-2 |
| smtc | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.2, SMTC.6 |  | 14.1.2-1, 14.1.2-2 AND Cell 1 |
|  | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.5, SMTC.2 |  | 14.1.2-1, 14.1.2-2 AND Cell 2 |
| deriveSSB-IndexFromCell | false |  | 14.1.2-1, 14.1.2-2 |
| } |  |  |  |
| } |  |  |  |

Table 14.1.2.4.3-4: ServingCellConfigCommonSIB

| Derivation Path: Table H.3.8-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| enhancedMeasurementNGSO-r17 | TRUE |  |  |
| } |  |  |  |

14.1.2.5	Test requirements

Table 14.1.2.5-1 defines the primary level settings including test tolerances for NR SA FR1 Cell Reselection for UE configured with the feature for enhanced requirements for Satellite Access.

Table 14.1.2.5-1: Cell specific parameters for NR SA FR1 Cell Reselection for UE configured with the feature for enhanced requirements for Satellite Access

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Satellite information |  | SSC.1 for Config 1 SSC.2 for Config 2 |  |  | NSC.1 for Config 1 NSC.2 for Config 2 |  |  |
| PDSCH RMC configuration |  | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| RMSI CORESET configuration |  | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| Dedicated CORESET configuration |  | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| OCNG Pattern |  | OP.1 defined in A.2.1 |  |  | OP.1 defined in A.2.1 |  |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| SSB configuration |  | SSB.1 FR1 |  |  | SSB.5 FR1 |  |  |
| SMTC configuration |  | #1: SMTC.2 for Cell 1 #2: SMTC.5 for Cell 2 |  |  | #1: SMTC.6 for Cell 1 #2: SMTC.2 for Cell 2 |  |  |
| RLM-RS |  | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | -130 |  |  | -130 |  |  |
| Pcompensation | dB | 0 |  |  | 0 |  |  |
| Qhysts | dB | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 16 | -3.55 | 3.24 | -infinity | 3.24 | -3.55 |
| Note2 | dBm/SCS | -98 |  |  |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |  |  |  |
|  | dB | 16 | 13 | 16.45 | -infinity | 16.45 | 13 |
| SS-RSRP Note3 | dBm/SCS | -82 | -85 | -81.55 | -infinity | -81.55 | -85 |
| Io | dBm/9.36 MHz | -53.94 | -51.91 | -51.91 | Same as parameters specified in Cell 1 columns- |  |  |
| Treselection | s | 0 | 0 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 60 |  |  | 60 |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2 and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility and periodic registration update on Cell 2.

The cell re-selection delay to a newly detectable cell shall be less than:

11 s if Kmulti_SMTC is equal to 1 (see note on Table 14.1.2.4.1-3); or

20 s if Kmulti_SMTC is equal to 2.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T3, to the moment when the UE camps on Cell 1 and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility and periodic registration update on cell 1.

The cell re-selection delay to an already detected cell shall be less than:

6 s if Kmulti_SMTC is equal to 1 (see note on Table 14.1.2.4.1-3); or

9 s if Kmulti_SMTC is equal to 2.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Kmulti_SMTC *Tdetect, NR_Intra_enh + TSI-NR, and to an already detected cell can be expressed as: Kmulti_SMTC *Tevaluate, NR_intra_enh + TSI-NR,

Where:

Tdetect, NR_Intra_enh	See Table 14.1.0.1-2 in clause 14.1.0.1

Tevaluate, NR_Intra_enh	See Table 14.1.0.1-2 in clause 14.1.0.1

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280ms is assumed in this test case provided that SIB1 and SIB19 are scheduled with 20ms period and 80 ms period, respectively.

If Kmulti_SMTC = 1, Kmulti_SMTC *Tevaluate, NR_Intra_enh + TSI-NR = 5.12 s; allow 6 s. And Kmulti_SMTC *Tdetect, NR_Intra_enh + TSI-NR = 10.24 s, allow 11 s.

If K_multi_SMTC = 2, Kmulti_SMTC *Tevaluate, NR_Intra_enh + TSI-NR = 8.96 s; allow 9 s. And Kmulti_SMTC *Tdetect, NR_Intra_enh + TSI-NR = 19.2 s, allow 20s.

In this test, SMTC.2 (period 20ms, duration 5ms, offset 0ms) and SMTC.7 (period 20ms, duration 5ms, offset 5ms) are configured. These are non-overlapping SMTCs therefore, according to clause 14.1.0.1, Kmulti_SMTC = 1 for GEO, Kmulti_SMTC = 1 for LEO for UEs supporting measurement of only one LEO satellite in parallel within an SMTC (NLEO,simul=1), and Kmulti_SMTC = 2 for LEO for Ues supporting measurement of two or more LEO satellites in parallel within an SMTC (NLEO,simul=2,3,4).

### 14.1.3	NR SA FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access

14.1.3.1	Test purpose

This test is to verify the requirement for the intra-frequency NR cell reselection requirements for satellite access specified in clause 14.1.0.1.

14.1.3.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access and time-based measurement initiation in idle mode.

14.1.3.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.14.1.3.

14.1.3.4	Test description

The test scenario comprises of 1 NR carrier and 2 cells as given in tables 14.1.3.4.1-1, 14.1.3.4.1-3 and 14.1.3.5-1. The test consists of two successive time periods, with time duration of T1 and T2, respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2. T-Service broadcasted in SIB19 of Cell 1 is set to the time point that is 36s after start of T2.

14.1.3.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.1.3.4.1-1.

Table 14.1.3.4.1-1: Supported test configurations for NR SA FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access

| Configuration | Description |
| --- | --- |
| 14.1.3-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.1.3-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. |  |

Configure the test equipment and the DUT according to the parameters in Table 14.1.3.4.1-2

Table 14.1.3.4.1-2: Initial conditions for NR SA FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.12-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.1.3.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 14.1.3.4.1-3.

2.	Message contents are defined in clause 14.1.3.4.3.

3.	Cell 1 and Cell 2 are NR cells with the power level set according to clauses C.1.2 and C.1.3 for this test. Both Cell 1 and Cell 2 are satellite access cells.

4.	The initial test environment conditions are setup according to section 14.0.5.

Table 14.1.3.4.1-3: General test parameters for NR SA FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell1 |  |
| T2 end condition | Active cell |  | Cell2 |  |
|  | Neighbour cells |  | Cell1 |  |
| RF Channel Number |  |  | 1 |  |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 102 | The detailed configuration is specified in TS 38.211 [7] clause 6.3.3.2 |
| rangeToBestCell |  |  | Not configured |  |
| T1 |  | s | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 40 (NOTE 1) | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| NOTE 1: If the test is performed in a NGSO configuration, and the scaling factor Kmulti_SMTC defined in clause 14.1.0.1 is greater than 1, according to UE capabilities, the duration of time T2 shall be scaled for the same factor to allow the UE to complete the cell reselection within the duration of the test case. |  |  |  |  |

14.1.3.4.2	Test procedure

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility”.

For NGSO configurations, the number of LEO satellites that a UE can measure in parallel within an SMTC is a capability the UE reports via maxNumber-NGSO-SatellitesWithinOneSMTC-r17. This value will determine the requirement to be used in the test, as well as the duration of T2.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR and Test Mode On according to TS 38.508-1 [14] clause 4.5. Cell 1 is an active cell and Cell 2 is powered off.

2.	Set the parameters according to T1 in Table 14.1.3.5-1. Propagation conditions are set according to Annex C clause C.2.2. T-Service broadcasted in SIB19 of Cell 1 is set to the time point that is 36s after start of T2, according to Table 14.1.3.4.3-4 (this timestamp indicates the UE when Cell 1 will stop providing service). T1 starts.

3.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for this iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.1.3.5-1. T2 starts.

5.	The SS waits for random access requests information from the UE to perform cell re-selection to a newly detectable cell, Cell 2.

6.	If the UE responds on the newly detectable cell, Cell 2 during time duration T2 within 34 seconds for GSO configurations or for NGSO configurations for Ues supporting measurement of only one LEO satellite in parallel within an SMTC, or within 66 seconds for NGSO configurations for UEs supporting measurement of two or more LEO satellites in parallel within an SMTC, from the beginning of time period T2, then count a success for the test.

7.	If the UE has re-selected to Cell 2 within T2, after the re-selection or when T2 expires, the SS shall send an RRCRelease message to the UE and then proceed with step 1 for the next iteration. Otherwise proceed with step 7a.

7a.	The SS shall switch off and on the UE, and skip to step 1 for the next iteration.

8.	Repeat step 2-7 until a test verdict has been achieved.

14.1.3.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 14.1.3.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.1-2 |
| Default RRC messages and information elements contents exceptions | Table H.2.4-1 Table H.2.4-2 with Condition GSO for Config 1 or Condition NGSO for Config 2 Table H.2.4-3 |

Table 14.1.3.4.3-2: SIB1 (Cell 1 and Cell 2)

| Derivation Path: Table H.2.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |  |
| q-RxLevMin |  | -65 | -130 is actual value in dBm (-65 * 2 dBm) | 14.1.3-1, 14.1.3-2 |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.1.3.4.3-3: SIB2 (Cell 1 and Cell 2)

| Derivation Path: Table H.2.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -65 | Actual value is -65*2 = -130dBm | 14.1.3-1, 14.1.3-2 |
| smtc | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.2, SMTC.6 |  | 14.1.3-1, 14.1.3-2 AND Cell 1 |
|  | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.2, SMTC.6 |  | 14.1.3-1, 14.1.3-2 AND Cell 2 |
| deriveSSB-IndexFromCell | false |  | 14.1.3-1, 14.1.3-2 |
| } |  |  |  |
| } |  |  |  |

Table 14.1.3.4.3-4: SIB19 (Cell 1)

| Derivation Path: Table H.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| t-Service-r17 | T+3600 | t-Service is set to expire 36 sec after start of T2. T is the time difference in seconds between start of T2 and the 00:00:00 UTC on Gregorian calendar date 1 January, 1900. Units in 10ms |  |
| } |  |  |  |

14.1.3.5	Test requirements

Table 14.1.3.5-1 defines the primary level settings including test tolerances for NR SA FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access.

Table 14.1.3.5-1: Cell specific parameters for NR SA FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | SSC.1 for Config 1 SSC.2 for Config 2 |  | NSC.1 for Config 1t NSC.2 for Config 2 |  |
| PDSCH RMC configuration |  | SR.1.1 FDD |  | SR.1.1 FDD |  |
| RMSI CORESET configuration |  | CR.1.1 FDD |  | CR.1.1 FDD |  |
| Dedicated CORESET configuration |  | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
| OCNG Pattern |  | OP.1 defined in A.2.1 |  | OP.1 defined in A.2.1 |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  | ULBWP.0.1 |  |
| SSB configuration |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration |  | #1: SMTC.2 for Cell 1 and Cell 2 |  | #1: SMTC.6 for Cell 1 and Cell 2 |  |
| RLM-RS |  | SSB |  | SSB |  |
| Qrxlevmin | dBm/SCS | -130 |  | -130 |  |
| Pcompensation | dB | 0 |  | 0 |  |
| Qhysts | dB | 0 |  | 0 |  |
| Qoffsets, n | dB | 0 |  | 0 |  |
| Cell_selection_and_ reselection_quality_measurement |  | SS-RSRP |  | SS-RSRP |  |
|  | dB | 16 | -3.55 | -infinity | 3.24 |
| Note2 | dBm/SCS | -98 |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |  |
|  | dB | 16 | 13 | -infinity | 16.45 |
| SS-RSRP Note3 | dBm/SCS | -82 | -85 | -infinity | -81.55 |
| Io | dBm/9.36 MHz | -53.94 | -51.91 | Same as parameters specified in Cell 1 columns- |  |
| Treselection | s | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 40 |  | 40 |  |
| Propagation Condition |  | AWGN |  |  |  |

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2 and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 2.

The cell re-selection delay to a newly detectable cell shall be less than:

34 s if Kmulti_SMTC is equal to 1 (see note on Table 14.1.3.4.1-3); or

66 s if Kmulti_SMTC is equal to 2.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Kmulti_SMTC *Tdetect, NR_Intra + TSI-NR,

Where:

Tdetect, NR_Intra	See Table 14.1.0.1-1 in clause 14.1.0.1

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280ms is assumed in this test case provided that SIB1 and SIB19 are scheduled with 20ms period and 80 ms period, respectively.

If Kmulti_SMTC = 1, Kmulti_SMTC *Tdetect, NR_intra + TSI-NR = 33.28 s, allow 34s.

If Kmulti_SMTC = 2, Kmulti_SMTC *Tdetect, NR_intra + TSI-NR = 65.28 s, allow 66s.

In this test, SMTC.1 (period 20ms, duration 1ms, offset 0ms) is configured on both cells. Since only one SMTC is configured there is no chance for overlap, therefore according to clause 14.1.0.1, Kmulti_SMTC = 1 for GEO, Kmulti_SMTC = 1 for LEO for UEs supporting measurement of only one LEO satellite in parallel within an SMTC (NLEO,simul=1), and Kmulti_SMTC = 2 for LEO for UEs supporting measurement of two or more LEO satellites in parallel within an SMTC (NLEO,simul=2,3,4).

### 14.1.4	NR SA FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access

Editor’s Note:

- Test requirements need to be updated from T2 start + 34/66 to T2 start + 38/70s

- Means to emulate UE movement speed at UE side is still FFS

- referenceLocation coordinates need to be updated

14.1.4.1	Test purpose

This test is to verify the requirement for the intra-frequency NR cell reselection requirements for satellite access specified in clause 14.1.0.1.

14.1.4.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access and location-based measurement initiation in idle mode.

14.1.4.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.14.1.4.

14.1.4.4	Test description

The test scenario comprises of 1 NR carrier and 2 cells as given in tables 14.1.4.4.1-1, 14.1.4.4.1-3 and 14.1.4.5-1. The test consists of two successive time periods, with time duration of T1 and T2, respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

At 4s after the start of T2, the UE location is changed such that the distance to the reference location broadcasted in SIB19 of Cell 1 is exceeded by the configured value in distanceThresh plus 50m.

14.1.4.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.1.4.4.1-1.

Table 14.1.4.4.1-1: Supported test configurations for NR SA FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access

| Configuration | Description |
| --- | --- |
| 14.1.4-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.1.4-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. |  |

Configure the test equipment and the DUT according to the parameters in Table 14.1.4.4.1-2

Table 14.1.4.4.1-2: Initial conditions for NR SA FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.12-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.1.4.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 14.1.4.4.1-3.

2.	Message contents are defined in clause 14.1.4.4.3.

3.	Cell 1 and Cell 2 are NR cells with the power level set according to clauses C.1.2 and C.1.3 for this test. Both Cell 1 and Cell 2 are satellite access cells.

4.	The initial test environment conditions are setup according to section 14.0.5.

Table 14.1.4.4.1-3: General test parameters for NR SA FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell1 |  |
| T2 end condition | Active cell |  | Cell2 |  |
|  | Neighbour cells |  | Cell1 |  |
| RF Channel Number |  |  | 1 |  |
| Time offset between cells |  |  | 3 ms | Asynchronous cells |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 102 | The detailed configuration is specified in TS 38.211 [7] clause 6.3.3.2 |
| rangeToBestCell |  |  | Not configured |  |
| T1 |  | s | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 40 (NOTE 1) | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| NOTE 1: If the test is performed in a NGSO configuration, and the scaling factor Kmulti_SMTC defined in clause 14.1.0.1 is greater than 1, according to UE capabilities, the duration of time T2 shall be scaled for the same factor to allow the UE to complete the cell reselection within the duration of the test case. |  |  |  |  |

14.1.4.4.2	Test procedure

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility”.

For NGSO configurations, the number of LEO satellites that a UE can measure in parallel within an SMTC is a capability the UE reports via maxNumber-NGSO-SatellitesWithinOneSMTC-r17. This value will determine the requirement to be used in the test, as well as the duration of T2.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR and Test Mode On according to TS 38.508-1 [14] clause 4.5. Cell 1 is an active cell and Cell 2 is powered off.

2.	Set the parameters according to T1 in Table 14.1.4.5-1. Propagation conditions are set according to Annex C clause C.2.2. T1 starts.

3.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for this iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.1.4.5-1. T2 starts.

5.	After 4 seconds from the start of T2, the UE location is changed such that the distance to the reference location broadcasted in SIB19 of Cell 1 is exceeded by the configured value in distanceThresh plus 50m, according to Table 14.1.4.4.3-4.

6.	The SS waits for random access requests information from the UE to perform cell re-selection to a newly detectable cell, Cell 2.

7.	If the UE responds on the newly detectable cell, Cell 2 during time duration T2 within 34 seconds for GSO configurations or for NGSO configurations for Ues supporting measurement of only one LEO satellite in parallel within an SMTC, or within 66 seconds for NGSO configurations for UEs supporting measurement of two or more LEO satellites in parallel within an SMTC, from the beginning of time period T2, then count a success for the test.

8.	If the UE has re-selected to Cell 2 within T2, after the re-selection or when T2 expires, the SS shall send an RRCRelease message to the UE and then proceed with step 1 for the next iteration. Otherwise proceed with step 8a.

8a.	The SS shall switch off and on the UE, and skip to step 1 for the next iteration.

9.	Repeat step 2-8 until a test verdict has been achieved.

14.1.4.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 14.1.4.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.1-2 |
| Default RRC messages and information elements contents exceptions | Table H.2.4-1 Table H.2.4-2 with Condition GSO for Config 1 or Condition NGSO for Config 2 Table H.2.4-3 |

Table 14.1.4.4.3-2: SIB1 (Cell 1 and Cell 2)

| Derivation Path: Table H.2.1-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |  |
| q-RxLevMin |  | -65 | -130 is actual value in dBm (-65 * 2 dBm) | 14.1.4-1, 14.1.4-2 |
| } |  |  |  |  |
| } |  |  |  |  |

Table 14.1.4.4.3-3: SIB2 (Cell 1 and Cell 2)

| Derivation Path: Table H.2.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -65 | Actual value is -65*2 = -130dBm | 14.1.4-1, 14.1.4-2 |
| smtc | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.2, SMTC.6 |  | 14.1.4-1, 14.1.4-2 AND Cell 1 |
|  | SSB-MTC specified in TS 38.508-1 [14] Table 7.3.1-3 with condition SMTC.2, SMTC.6 |  | 14.1.4-1, 14.1.4-2 AND Cell 2 |
| deriveSSB-IndexFromCell | false |  | 14.1.4-1, 14.1.4-2 |
| } |  |  |  |
| } |  |  |  |

Table 14.1.4.4.3-4: SIB19 (Cell 1)

| Derivation Path: Table H.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ReferenceLocation-r17 | FFS |  |  |
| distanceThresh-r17 | FFS |  |  |
| } |  |  |  |

14.1.4.5	Test requirements

Table 14.1.4.5-1 defines the primary level settings including test tolerances for NR SA FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access.

Table 14.1.4.5-1: Cell specific parameters for NR SA FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | SSC.1 for Config 1 SSC.2 for Config 2 |  | NSC.1 for Config 1 NSC.2 for Config 2 |  |
| PDSCH RMC configuration |  | SR.1.1 FDD |  | SR.1.1 FDD |  |
| RMSI CORESET configuration |  | CR.1.1 FDD |  | CR.1.1 FDD |  |
| Dedicated CORESET configuration |  | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
| OCNG Pattern |  | OP.1 defined in A.2.1 |  | OP.1 defined in A.2.1 |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  | ULBWP.0.1 |  |
| SSB configuration |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration |  | #1: SMTC.2 for Cell 1 and Cell 2 |  | #1: SMTC.6 for Cell 1 and Cell 2 |  |
| RLM-RS |  | SSB |  | SSB |  |
| Qrxlevmin | dBm/SCS | -130 |  | -130 |  |
| Pcompensation | dB | 0 |  | 0 |  |
| Qhysts | dB | 0 |  | 0 |  |
| Qoffsets, n | dB | 0 |  | 0 |  |
| Cell_selection_and_ reselection_quality_measurement |  | SS-RSRP |  | SS-RSRP |  |
|  | dB | 16 | -3.55 | -infinity | 3.24 |
| Note2 | dBm/SCS | -98 |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |  |
|  | dB | 16 | 13 | -infinity | 16.45 |
| SS-RSRP Note3 | dBm/SCS | -82 | -85 | -infinity | -81.55 |
| Io | dBm/9.36 MHz | -53.94 | -51.91 | Same as parameters specified in Cell 1 columns- |  |
| Treselection | s | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 40 |  | 40 |  |
| Propagation Condition |  | AWGN |  |  |  |

The cell reselection delay to a newly detectable cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2 and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 2.

The cell re-selection delay to a newly detectable cell shall be less than:

34 s if Kmulti_SMTC  is  equal to 1 (see note on Table 14.1.4.4.1-3); or

66 s if Kmulti_SMTC is equal to 2.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a newly detectable cell can be expressed as: Kmulti_SMTC *Tdetect, NR_Intra + TSI-NR, and to an already detected cell can be expressed as: Kmulti_SMTC *Tevaluate, NR_intra + TSI-NR,

Where:

Tdetect, NR_Intra	See Table 14.1.0.1-1 in clause 14.1.0.1

Tevaluate, NR_intra	See Table 14.1.0.1-1 in clause 14.1.0.1

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280ms is assumed in this test case provided that SIB1 and SIB19 are scheduled with 20ms period and 80 ms period, respectively.

If Kmulti_SMTC = 1, Kmulti_SMTC *Tdetect, NR_intra + TSI-NR = 33.28 s, allow 34s.

If Kmulti_SMTC = 2, Kmulti_SMTC *Tdetect, NR_intra + TSI-NR = 65.28 s, allow 66s.

In this test, SMTC.1 (period 20ms, duration 1ms, offset 0ms) is configured on both cells. Since only one SMTC is configured there is no chance for overlap, therefore according to clause 14.1.0.1, Kmulti_SMTC = 1 for GEO, Kmulti_SMTC = 1 for LEO for UEs supporting measurement of only one LEO satellite in parallel within an SMTC (NLEO,simul=1), and Kmulti_SMTC = 2 for LEO for UEs supporting measurement of two or more LEO satellites in parallel within an SMTC (NLEO,simul=2,3,4).

### 14.1.5	NR SA FR1-FR1 Cell Reselection for Satellite Access

Editor's Note: This test case is incomplete due to following aspects.

- Test case implementation is not feasible due to incorrect cell power level setting during T2 in TS 38.133 test parameter table.

14.1.5.1	Test purpose

This test is to verify the requirement for the inter-frequency NR cell reselection requirements for satellite access specified in clause 14.1.0.2.

14.1.5.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access.

14.1.5.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.14.1.5.

14.1.5.4	Test description

The test scenario comprises of 2 NR carriers and 2 cells as given in tables 14.1.5.4.1-1, 14.1.5.4.1-3 and 14.1.5.5-1. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Both Cell 1 and Cell 2 are already identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas and Cell 2 is of higher priority than Cell 1. Furthermore, UE has not registered with network for the tracking area containing Cell 2.

14.1.5.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.1.5.4.1-1.

Table 14.1.5.4.1-1: Supported test configurations for NR SA FR1-FR1 Cell reselection for Satellite Access

| Configuration | Description |
| --- | --- |
| 14.1.5-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.1.5-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. |  |

Configure the test equipment and the DUT according to the parameters in Table 14.1.5.4.1-2

Table 14.1.5.4.1-2: Initial conditions for NR SA FR1-FR1 Cell reselection for Satellite Access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.12-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.1.5.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 14.1.5.4.1-3.

2.	Message contents are defined in clause 14.1.5.4.3.

3.	Cell 1 and Cell 2 are NR cells on two different carriers with the power level set according to clauses C.1.2 and C.1.3 for this test. Both Cell 1 and Cell 2 are satellite access cells.

4.	The initial test environment conditions are setup according to section 14.0.5.

Table 14.1.5.4.1-3: General test parameters for NR SA FR1-FR1 Cell reselection for Satellite Access

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell 1 |  |
| T2 end condition | Active cell |  | 1, 2 | Cell 2 |  |
|  | Neighbour cells |  | 1, 2 | Cell 1 |  |
| Final condition | Active cell |  | 1, 2 | Cell 1 |  |
|  | Neighbour cells |  | 1, 2 | Cell 2 |  |
| RF Channel Number |  |  | 1, 2 | 1,2 |  |
| Time offset between cells |  | ms | 1, 2 | 3 | Asynchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1, 2 | SSB.1 FR1 |  |
| SMTC configuration#1 |  |  | 1, 2 | SMTC.2 | Configured in SIB2 of Cell 1 |
|  |  |  |  | SMTC.6 | Configured in SIB2 of Cell 2 |
| SMTC configuration#2 |  |  | 1, 2 | SMTC.2 | Configured in SIB2 of Cell 1 |
|  |  |  |  | SMTC.6 | Configured in SIB2 of Cell 2 |
| DRX cycle length |  | s | 1, 2 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 102 | The detailed configuration is specified in TS 38.211 [7] clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| Ephemeris information |  |  | 1, 2 | Note 1 |  |
| T1 |  | s | 1, 2 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2 | >7 | During T2, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 1, 2 | 75 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |
| Note 1: Detailed ephemeris information is provided in TS 38.508-1 [14] |  |  |  |  |  |

14.1.5.4.2	Test procedure

The test scenario comprises of two satellite access NR FDD inter-frequency cells as given in table 14.1.5.4.1-3. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing Cell 2.

In the following test procedure "UE responds" means "UE starts transmitting preamble on PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility”.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR and Test Mode On according to TS 38.508-1 [14] clause 4.5. Cell 1 is an active cell and Cell 2 is powered off.

2.	Set the parameters according to T1 in Table 14.1.5.5-1. Propagation conditions are set according to Annex C clause C.2.2. T1 starts.

3.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for this iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.1.5.5-1. T2 starts.

5.	The SS waits for random access requests information from the UE to perform cell re-selection to a newly detectable cell, Cell 2.

6.	If the UE responds on the newly detectable cell, Cell 2 during time duration T2 within 68 seconds for GSO configurations or for NGSO configurations for UEs supporting measurement of only one LEO satellite in parallel within an SMTC, [or within FFS seconds for NGSO configurations for UEs supporting measurement of two or more LEO satellites in parallel within an SMTC] from the beginning of time period T2, then count a success for the event “Re-select newly detected Cell 2”. Otherwise count a fail for the event “Re-select newly detected Cell 2”.

7.	If the UE has re-selected Cell 2 within T2, after the re-selection or when T2 expires, continue with step 7a.
Otherwise, if T2 expires and the UE has not yet re-selected Cell 2, the TE shall switch off and on the UE and skip to step 1.

7a	The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 2.

8.	The SS shall switch the power setting from T2 to T3 as specified in Table 14.1.5.5-1. T3 starts.

9.	The SS waits for random access requests information from the UE to perform cell re-selection to an already detected cell, Cell 1.

10.	If the UE responds on the already detected cell, Cell 1 during time duration T3 within 8 seconds for GSO configurations or for NGSO configurations for UEs supporting measurement of only one LEO satellite in parallel within an SMTC, [or within FFS seconds for NGSO configurations for UEs supporting measurement of two or more LEO satellites in parallel within an SMTC], from the beginning of time period T3, then count a success for the event “Re-select already detected Cell 1”. Otherwise count a fail for the event “Re-select already detected Cell 1”.

11.	If the UE has re-selected to Cell 1 within T3, after the re-selection or when T3 expires, the SS shall send an RRCRelease message to the UE and then proceed with step 1 for the next iteration. Otherwise proceed with step 11a.

11a.The SS shall switch off and on the UE, and skip to step 1 for the next iteration.

12.	Repeat step 2-11 until a test verdict has been achieved. Each of the events “Re-select newly detected Cell 2” and “Re-select already detected Cell 1” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict. If both events pass, the test passes. If one event fails, the test fails.

14.1.5.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 14.1.5.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.2-1 Table H.2.2-2 with Condition SSB.1 FR1, SMTC.2 and Asynchronous cells for configuration 14.1.5-1 and Cell 1 Table H.2.2-2 with Condition SSB.1 FR1, SMTC.6 and Asynchronous cells for configuration 14.1.5-1 and Cell 2 Table H.2.2-3 |
| Default RRC messages and information elements contents exceptions | Table H.2.4-1 Table H.2.4-2 with Condition GSO for Config 1 or Condition NGSO for Config 2 Table H.2.4-3 |

14.1.5.5	Test requirements

Table 14.1.5.5-1 defines the primary level settings including test tolerances for NR SA FR1-FR1 Cell reselection for Satellite Access.

Table 14.1.5.5-1: Cell specific parameters for NR SA FR1-FR1 Cell reselection for Satellite Access

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Satellite information |  | 1 | SSC.1 |  |  | SSC.1 |  |  |
|  |  | 2 | SSC.2 |  |  | SSC.2 |  |  |
| PDSCH RMC configuration |  | 1, 2 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| Dedicated CORESET RMC configuration |  | 1, 2 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| OCNG Pattern |  | 1, 2 | OP.1 defined in A.2.1 |  |  | OP.1 defined in A.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  |  | -140 |  |  |
| Pcompensation | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qhysts | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 1, 2 | 15.6 | 15.6 | 15.6 | -3.6 | -infinity | 13.6 |
| Note2 | dBm/SCS | 1, 2 | -98 |  |  | -100 | -98 | -98 |
| Note2 | dBm/15 kHz | 1, 2 | -98 |  |  | -100 | -98 | -98 |
|  | dB | 1, 2 | 15.6 | 15.6 | 15.6 | -3.6 | -infinity | 13.6 |
| SS-RSRP Note3 | dBm/SCS | 1, 2 | -82.4 | -82.4 | -82.4 | -103.6 | -infinity | -84.4 |
| Io | dBm/9.36 MHz | 1, 2 | -54.33 | -54.33 | -54.33 | -70.48 | -70.05 | -56.26 |
| Treselection | s | 1, 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| SnonIntraSearchP | dB | 1, 2 | 50 |  |  | 50 |  |  |
| Threshx, highP | dB | 1, 2 | 48 |  |  | 48 |  |  |
| Threshserving, lowP | dB | 1, 2 | 44 |  |  | 44 |  |  |
| Threshx, lowP | dB | 1, 2 | 50 |  |  | 50 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

The cell reselection delay to a higher priority cell is defined as the time from the beginning of time period T3, to the moment when the UE camps again on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 2.

The cell re-selection delay to a higher priority cell shall be less than 68 s.

The cell reselection delay to a lower priority cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on Cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 1.

The cell re-selection delay to a lower priority cell shall be less than 8 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, NR_inter + TSI-NR, and to a lower priority cell can be expressed as: Tevaluate, NR_inter + TSI-NR,

Where:

Thigher_priority_search	See clause 4.2.2.7 in TS 38.133 [6].

Tevaluate, NR_inter	See Table 14.1.0.2-1 in clause 14.1.0.2

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 67.68 s, allow 68 s for the cell re-selection delay to a higher priority cell.

### 14.1.6	NR SA FR1-FR1 Cell Reselection for UE configured with the feature for enhanced requirements for Satellite Access

Editor's Note: This test case is incomplete due to following aspects.

-	Test parameter and configuration may need to be updated

14.1.6.1	Test purpose

This test is to verify the requirement for the intra-frequency NR cell reselection requirements for satellite access specified in clause 14.1.0.2.

14.1.6.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access and enhanced RRM idle mode measurements in NTN bands.

14.1.6.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.14.1.6.

14.1.6.4	Test description

The test scenario comprises of 2 NR carriers and 2 cells as given in tables 14.1.6.4.1-1, 14.1.6.4.1-3 and 14.1.6.5-1. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Both Cell 1 and Cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and Cell 2 is of higher priority than Cell 1. Furthermore, UE has not registered with network for the tracking area containing cell 2. The flag enhancedMeasurementNGSO-r17 should be set.

14.1.6.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.1.6.4.1-1.

Table 14.1.6.4.1-1: Supported test configurations for NR SA FR1-FR1 Cell Reselection for UE configured with the feature for enhanced requirements for Satellite Access

| Configuration | Description |
| --- | --- |
| 14.1.6-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.1.6-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 14.1.6.4.1-2

Table 14.1.6.4.1-2: Initial conditions for NR SA FR1-FR1 Cell Reselection for UE configured with the feature for enhanced requirements for Satellite Access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.12-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.1.6.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 14.1.6.4.1-3.

2.	Message contents are defined in clause 14.1.6.4.3.

3.	Cell 1 and Cell 2 are NR cells on two different carriers with the power level set according to clauses C.1.2 and C.1.3 for this test. Both Cell 1 and Cell 2 are satellite access cells.

4.	The initial test environment conditions are setup according to section 14.0.5.

Table 14.1.6.4.1-3: General test parameters for NR SA FR1-FR1 Cell Reselection for UE configured with the feature for enhanced requirements for Satellite Access

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | Cell 1 |  |
| T2 end condition | Active cell |  | Cell 2 |  |
|  | Neighbour cells |  | Cell 1 |  |
| Final condition | Active cell |  | Cell 1 |  |
|  | Neighbour cells |  | Cell 2 |  |
| RF Channel Number |  |  | 1,2 |  |
| Time offset between cells |  | ms | 3 | Asynchronous cells |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | SSB.1 FR1 |  |
| SMTC configuration#1 |  |  | SMTC.2 | Configured in SIB2 of Cell 1 |
|  |  |  | SMTC.6 | Configured in SIB2 of Cell 2 |
| SMTC configuration#2 |  |  | SMTC.2 | Configured in SIB2 of Cell 1 |
|  |  |  | SMTC.6 | Configured in SIB2 of Cell 2 |
| DRX cycle length |  | s | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 102 | The detailed configuration is specified in TS 38.211 [7] clause 6.3.3.2 |
| rangeToBestCell |  |  | Not configured |  |
| Ephemeris information |  |  | Note 1 |  |
| T1 |  | s | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | >7 | During T2, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 75 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |
| Note 1: Detailed ephemeris information is provided in TS 38.508-1 [14] |  |  |  |  |

14.1.6.4.2	Test procedure

The test scenario comprises of two satellite access NR FDD inter-frequency cells as given in table 14.1.2.4.1-3. Both Cell 1 and Cell 2 are already identified by the UE prior to the start of the test. Cell 1 and Cell 2 belong to different tracking areas. Cell 2 is of higher priority than Cell 1. The flag enhancedMeasurementNGSO-r17 should be set.

In the following test procedure "UE responds" means "UE starts transmitting preamble on PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility”.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR and Test Mode On according to TS 38.508-1 [14] clause 4.5 on Cell 2.

2.	Set the parameters according to T1 in Table 14.1.6.5-1. Propagation conditions are set according to Annex C clause C.2.2. T1 starts.

3.	The SS waits for random access requests information from the UE to perform cell re-selection to Cell 1.

4.	If the UE responds on the already detected cell, Cell 1 during time duration T1 within 6 seconds for GSO configurations or for NGSO configurations for UEs supporting measurement of only one LEO satellite in parallel within an SMTC,  from the beginning of time period T1, then count a success for the event “Re-select already detected Cell 1”. Otherwise count a fail for the event “Re-select already detected Cell 1”.

5.	If the UE has re-selected Cell 1 within T1, after the re-selection or when T1 expires, continue with step 6.
Otherwise, if T1 expires and the UE has not yet re-selected Cell 1, the TE shall switch off and on the UE and skip to step 1.

6.	The SS shall send an RRCRelease message to ensure that the UE is in state RRC_IDLE on Cell 1.

7.	The SS shall switch the power setting from T1 to T2 as specified in Table 14.1.6.5-1. T2 starts. During time duration T2, Cell 2 shall be powered OFF and the physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) shall be changed to ensure Cell 2 is not detected by the UE.

8. When T2 expires, the SS shall switch the power setting from T2 to T3 as specified in Table 14.1.6.5-1. T3 starts.

9. 	The SS waits for random access requests information from the UE to perform cell re-selection on Cell 2.

10.	If the UE responds on the newly detectable cell, Cell 2 during time duration T3 within 66 seconds for GSO configurations or for NGSO configurations for UEs supporting measurement of only one LEO satellite in parallel within an SMTC, from the beginning of time period T3, then count a success for the event “Re-select newly detected Cell 2”. Otherwise count a fail for the event “Re-select newly detected Cell 2”.

11.	If the UE has re-selected to Cell 2 within T3, after the re-selection or when T3 expires, the SS shall send an RRCRelease message to the UE and then proceed with step 1 for the next iteration. Otherwise proceed with step 11a.

11a.The SS shall switch off and on the UE, and skip to step 1 for the next iteration.

12.	Repeat step 2-11 until a test verdict has been achieved. Each of the events “Re-select newly detected Cell 2” and “Re-select already detected Cell 1” is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G clause G.2 is achieved. Different events may require different times for a verdict. If both events pass, the test passes. If one event fails, the test fails.

14.1.6.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 14.1.6.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.2-1 Table H.2.2-2 with Condition SSB.1 FR1, SMTC.2 and Asynchronous cells for configuration 14.1.6-1 and Cell 1 Table H.2.2-2 with Condition SSB.1 FR1, SMTC.6 and Asynchronous cells for configuration 14.1.6-1 and Cell 2 Table H.2.2-3 |
| Default RRC messages and information elements contents exceptions | Table H.2.4-1 Table H.2.4-2 with Condition GSO for Config 1 or Condition NGSO for Config 2 Table H.2.4-3 |

Table 14.1.6.4.3-2: ServingCellConfigCommonSIB

| Derivation Path: Table H.3.8-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| enhancedMeasurementNGSO-r17 | TRUE |  |  |
| } |  |  |  |

14.1.6.5	Test requirements

Table 14.1.6.5-1 defines the primary level settings including test tolerances for NR SA FR1-FR1 Cell Reselection for UE configured with the feature for enhanced requirements for Satellite Access.

Table 14.1.6.5-1: Cell specific parameters for NR SA FR1-FR1 Cell Reselection for UE configured with the feature for enhanced requirements for Satellite Access

| Parameter | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Satellite information |  | SSC.1 for Config 1 SSC.2 for Config 2 |  |  | SSC.1 for Config 1 SSC.2 for Config 2 |  |  |
| PDSCH RMC configuration |  | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
| RMSI CORESET RMC configuration |  | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
| Dedicated CORESET RMC configuration |  | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
| OCNG Pattern |  | OP.1 defined in A.2.1 |  |  | OP.1 defined in A.2.1 |  |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | -130 |  |  | -130 |  |  |
| Pcompensation | dB | 0 |  |  | 0 |  |  |
| Qhysts | dB | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 15.6 | 15.6 | 15.6 | -3.6 | -infinity | 13.6 |
| Note2 | dBm/SCS | -98 |  |  | -100 | -98 | -98 |
| Note2 | dBm/15 kHz | -98 |  |  | -100 | -98 | -98 |
|  | dB | 15.6 | 15.6 | 15.6 | -3.6 | -infinity | 13.6 |
| SS-RSRP Note3 | dBm/SCS | -82.4 | -82.4 | -82.4 | -103.6 | -infinity | -84.4 |
| Io | dBm/9.36 MHz | -54.33 | -54.33 | -54.33 | -70.48 | -70.05 | -56.26 |
| Treselection | s | 0 | 0 | 0 | 0 | 0 | 0 |
| SnonIntraSearchP | dB | 40 Note1 |  |  | 50 |  |  |
| Threshx, highP | dB | 48 |  |  | 38 Note1 |  |  |
| Threshserving, lowP | dB | 44 |  |  | 44 |  |  |
| Threshx, lowP | dB | 40 Note1 |  |  | 50 |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |
| Note 1: Modified by TT analysis |  |  |  |  |  |  |  |

The cell re-selection delay to a higher priority cell is defined as the time from the beginning of time period T3, to the moment when the UE camps again on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility and periodic registration update on Cell 2.

The cell re-selection delay to a higher priority cell shall be less than 66 s.

The cell re-selection delay to a lower priority cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on Cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility and periodic registration update on Cell 1.

The cell re-selection delay to a lower priority cell shall be less than 6 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90 %.

NOTE:	The cell re-selection delay to a higher priority cell can be expressed as: Thigher_priority_search + Tevaluate, NR_inter_enh + TSI-NR, and to a lower priority cell can be expressed as: Tevaluate, NR_inter_enh + TSI-NR,

Where:

Thigher_priority_search	See clause 14.1.0.2

Tevaluate, NR_inter_enh	See Table 14.1.0.2-2 in clause 14.1.0.2

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 65.12 s, allow 66 s for the cell re-selection delay to a higher priority

### 14.1.7	NR SA FR1-FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access

-

14.1.7.1	Test purpose

This test is to verify the requirement for the inter-frequency NR cell reselection requirements for satellite access specified in clause 14.1.0.2.

14.1.7.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access and time-based measurement initiation.

14.1.7.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.14.1.7.

14.1.7.4	Test description

The test scenario comprises of 1 NR carrier and 2 cells as given in tables 14.1.7.4.1-1, 14.1.7.4.1-3 and 14.1.7.5-1. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2. t-Service broadcasted in SIB19 of Cell 1 is set to the time point that is 36s after start of T2.

14.1.7.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.1.7.4.1-1.

Table 14.1.7.4.1-1: Supported test configurations for NR SA FR1-FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access

| Configuration | Description |
| --- | --- |
| 14.1.7-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.1.7-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. |  |

Configure the test equipment and the DUT according to the parameters in Table 14.1.7.4.1-2

Table 14.1.7.4.1-2: Initial conditions for NR SA FR1-FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.12-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.1.7.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |

1.	The general test parameter settings are set up according to Table 14.1.7.4.1-3.

2.	Message contents are defined in clause 14.1.7.4.3.

3.	Cell 1 and Cell 2 are NR cells with the power level set according to clauses C.1.2 and C.1.3 for this test. Both Cell 1 and Cell 2 are satellite access cells.

4.	The initial test environment conditions are setup according to section 14.0.5.

Table 14.1.7.4.1-3: General test parameters for NR SA FR1-FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2 | Cell2 |  |
|  | Neighbour cells |  | 1, 2 | Cell1 |  |
| RF Channel Number |  |  | 1, 2 | 1 |  |
| Time offset between cells |  |  | 1, 2 | 3 ms | Asynchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 102 | The detailed configuration is specified in TS 38.211 [7] clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 1, 2 | 40 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

14.1.7.4.2	Test procedure

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility”.

For NGSO configurations, the number of LEO satellites that a UE can measure in parallel within an SMTC is a capability the UE reports via maxNumber-NGSO-SatellitesWithinOneSMTC-r17. This value will determine the requirement to be used in the test, as well as the duration of T2.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR and Test Mode On according to TS 38.508-1 [14] clause 4.5. Cell 1 is an active cell and Cell 2 is powered off.

2.	Set the parameters according to T1 in Table 14.1.7.5-1. Propagation conditions are set according to Annex C clause C.2.2. T-Service broadcasted in SIB19 of Cell 1 is set to the time point that is 36s after start of T2, according to Table 14.1.7.4.3-2 (this timestamp indicates the UE when Cell 1 will stop providing service). T1 starts.

3.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for this iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.1.7.5-1. T2 starts.

5.	The SS waits for random access requests information from the UE to perform cell re-selection to a newly detectable cell, Cell 2.

6.	If the UE responds on the newly detectable cell, Cell 2 during time duration T2 within 36 seconds for GSO configurations or for NGSO configurations for UEs supporting measurement of only one LEO satellite in parallel within an SMTC, , from the beginning of time period T2, then count a success for the test.

7.	If the UE has re-selected to Cell 2 within T2, after the re-selection or when T2 expires, the SS shall send an RRCRelease message to the UE and then proceed with step 1 for the next iteration. Otherwise proceed with step 7a.

7a.	The SS shall switch off and on the UE, and skip to step 1 for the next iteration.

8.	Repeat step 2-7 until a test verdict has been achieved.

14.1.7.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 14.1.7.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.2-1 Table H.2.2-2 with Condition SSB.1 FR1, SMTC.2 and Asynchronous cells for configuration 14.1.7-1 and Cell 1 Table H.2.2-2 with Condition SSB.1 FR1, SMTC.6 and Asynchronous cells for configuration 14.1.7-1 and Cell 2 Table H.2.2-3 |
| Default RRC messages and information elements contents exceptions | Table H.2.4-1 Table H.2.4-2 with Condition GSO for Config 1 or Condition NGSO for Config 2 Table H.2.4-3 |

Table 14.1.3.4.3-2: SIB19 (Cell 1)

| Derivation Path: Table H.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  |  | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |  |  |
| t-Service-r17 |  |  | T+3600 | t-Service is set to expire 36 sec after start of T2. T is the time difference in seconds between start of T2 and the 00:00:00 UTC on Gregorian calendar date 1 January, 1900. Units in 10ms |  |
| } |  |  |  |  |  |

14.1.7.5	Test requirements

Table 14.1.7.5-1 defines the primary level settings including test tolerances for NR SA FR1-FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access.

Table 14.1.7.5-1: Cell specific parameters for NR SA FR1-FR1 Time-based Measurement Initiation Cell Reselection for Satellite Access

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | SSC.1 for Config 1 SSC.2 for Config 2 |  | NSC.1 for Config 1 NSC.2 for Config 2 |  |
| PDSCH RMC configuration |  | SR.1.1 FDD |  | SR.1.1 FDD |  |
| RMSI CORESET configuration |  | CR.1.1 FDD |  | CR.1.1 FDD |  |
| Dedicated CORESET configuration |  | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
| OCNG Pattern |  | OP.1 defined in A.2.1 |  | OP.1 defined in A.2.1 |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  | ULBWP.0.1 |  |
| SSB configuration |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration |  | #1: SMTC.2 for Cell 1 #2: SMTC.6 for Cell 2 |  | #1: SMTC.2 for Cell 1 #2: SMTC.6 for Cell 2 |  |
| RLM-RS |  | SSB |  | SSB |  |
| Qrxlevmin | dBm/SCS | -130 |  | -130 |  |
| Pcompensation | dB | 0 |  | 0 |  |
| Qhysts | dB | 0 |  | 0 |  |
| Qoffsets, n | dB | 0 |  | 0 |  |
| Cell_selection_and_ reselection_quality_measurement |  | SS-RSRP |  | SS-RSRP |  |
|  | dB | 16 | 13 | -infinity | 16 |
| Note2 | dBm/SCS | -98 |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |  |
|  | dB | 16 | 13 | -infinity | 16 |
| SS-RSRP Note3 | dBm/SCS | -82 | -85 | -infinity | -82 |
| Io | dBm/9.36 MHz | -53.94 | -56.84 | -70.05 | -53.94 |
| Treselection | s | 0 | 0 | 0 | 0 |
| SnonIntrasearchP | dB | 50 |  | 50 |  |
| Threshx, highP | dB | 48 |  | 48 |  |
| Threshserving, lowP | dB | 54 |  | 44 |  |
| Threshx, lowP | dB | 50 |  | 40 |  |
| Propagation Condition |  | AWGN |  |  |  |

The cell reselection delay to a lower priority cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 2.

The cell re-selection delay to a lower priority cell shall be less than 36 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a lower priority cell can be expressed as: Tevaluate, NR_inter + TSI-NR,

Where:

Tevaluate, NR_inter	See Table 4.2C.2.4-1 in TS 38.133 [6] clause 4.2C.2.4

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280ms is assumed in this test case provided that SIB1 and SIB19 are scheduled with 80ms period.

(NLEO,simul=1)(NLEO,simul=2,3,4)

### 14.1.8	NR SA FR1-FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access

Editor’s Note:

-	Means to emulate UE movement speed at UE side is still FFS

-	referenceLocation coordinates need to be updated

14.1.8.1	Test purpose

This test is to verify the requirement for the inter-frequency NR cell reselection requirements for satellite access specified in clause 14.1.0.2.

14.1.8.2	Test applicability

This test applies to all types of NR UE release 17 and forward supporting satellite access and location-based measurement initiation.

14.1.8.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 14.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.14.1.8.

14.1.8.4	Test description

The test scenario comprises of 1 NR carrier and 2 cells as given in tables 14.1.8.4.1-1, 14.1.8.4.1-3 and 14.1.8.5-1. The test consists of two successive time periods, with time duration of T1and T2, respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

At 4s after the start of T2, the UE location is changed such that the distance to the reference location broadcasted in SIB19 of Cell 1 is exceeded by the configured value in distanceThresh plus 50m.

14.1.8.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 14.1.8.4.1-1.

Table 14.1.8.4.1-1: Supported test configurations for NR SA FR1-FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access

| Configuration | Description |
| --- | --- |
| 14.1.8-1 | GSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| 14.1.8-2 | NGSO, NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz |
| Note: If UE supports both NGSO and GSO, the GSO-based test cases can be skipped if the UE passes NGSO-based test cases. |  |

Configure the test equipment and the DUT according to the parameters in Table 14.1.8.4.1-2

Table 14.1.8.4.1-2: Initial conditions for NR SA FR1-FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.12-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 14.1.8.4.1-1 |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |

1.	The general test parameter settings are set up according to Table 14.1.8.4.1-3.

2.	Message contents are defined in clause 14.1.8.4.3.

3.	Cell 1 and Cell 2 are NR cells with the power level set according to clauses C.1.2 and C.1.3 for this test. Both Cell 1 and Cell 2 are satellite access cells.

4.	The initial test environment conditions are setup according to section 14.0.5.

Table 14.1.8.4.1-3: General test parameters for NR SA FR1-FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell1 |  |
| T2 end condition | Active cell |  | 1, 2 | Cell2 |  |
|  | Neighbour cells |  | 1, 2 | Cell1 |  |
| RF Channel Number |  |  | 1, 2 | 1 |  |
| Time offset between cells |  |  | 1, 2 | 3 ms | Asynchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| DRX cycle length |  | s | 1, 2 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 102 | The detailed configuration is specified in TS 38.211 [7] clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 1, 2 | 40 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

14.1.8.4.2	Test procedure

In the following test procedure “UE responds” means “UE starts transmitting preamble on PRACH for sending the RRCSetupRequest message to perform a registration procedure for mobility”.

For NGSO configurations, the number of LEO satellites that a UE can measure in parallel within an SMTC is a capability the UE reports via maxNumber-NGSO-SatellitesWithinOneSMTC-r17. This value will determine the requirement to be used in the test, as well as the duration of T2.

1.	Ensure the UE is in state RRC_IDLE with generic procedure parameters Connectivity NR and Test Mode On according to TS 38.508-1 [14] clause 4.5. Cell 1 is an active cell and Cell 2 is powered off.

2.	Set the parameters according to T1 in Table 14.1.8.5-1. Propagation conditions are set according to Annex C clause C.2.2. T1 starts.

3.	Set Cell 2 physical cell identity = ((current Cell 2 physical cell identity + 1) mod 1008) for this iteration of the test procedure loop.

4.	When T1 expires, the SS shall switch the power setting from T1 to T2 as specified in Table 14.1.8.5-1. T2 starts.

5.	After 4 seconds from the start of T2, the UE location is changed such that the distance to the reference location broadcasted in SIB19 of Cell 1 is exceeded by the configured value in distanceThresh plus 50m, according to Table 14.1.8.4.3-2.

6.	The SS waits for random access requests information from the UE to perform cell re-selection to a newly detectable cell, Cell 2.

7.	If the UE responds on the newly detectable cell, Cell 2 during time duration T2 within 36 seconds for GSO configurations or for NGSO configurations for UEs supporting measurement of only one LEO satellite in parallel within an SMTC, from the beginning of time period T2, then count a success for the test.

8.	If the UE has re-selected to Cell 2 within T2, after the re-selection or when T2 expires, the SS shall send an RRCRelease message to the UE and then proceed with step 1 for the next iteration. Otherwise proceed with step 8a.

8a.	The SS shall switch off and on the UE, and skip to step 1 for the next iteration.

9.	Repeat step 2-8 until a test verdict has been achieved.

14.1.8.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 4.6 with the following exceptions:

Table 14.1.8.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions | Table H.2.2-1 Table H.2.2-2 with Condition SSB.1 FR1, SMTC.2 and Asynchronous cells for configuration 14.1.8-1 and Cell 1 Table H.2.2-2 with Condition SSB.1 FR1, SMTC.6 and Asynchronous cells for configuration 14.1.8-1 and Cell 2 Table H.2.2-3 |
| Default RRC messages and information elements contents exceptions | Table H.2.4-1 Table H.2.4-2 with Condition GSO for Config 1 or Condition NGSO for Config 2 Table H.2.4-3 |

Table 14.1..4.3-2: SIB19 (Cell 1)

| Derivation Path: Table H.2.4-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ReferenceLocation-r17 | FFS |  |  |
| distanceThresh-r17 | FFS |  |  |
| } |  |  |  |

14.1.8.5	Test requirements

Table 14.1.8.5-1 defines the primary level settings including test tolerances for NR SA FR1-FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access.

Table 14.1.8.5-1: Cell specific parameters for NR SA FR1-FR1 Location-based Measurement Initiation Cell Reselection for Satellite Access

| Parameter | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T1 | T2 |
| Satellite information |  | SSC.1 for Config 1 SSC.2 for Config 2 |  | NSC.1 for Config 1 NSC.2 for Config 2 |  |
| PDSCH RMC configuration |  | SR.1.1 FDD |  | SR.1.1 FDD |  |
| RMSI CORESET configuration |  | CR.1.1 FDD |  | CR.1.1 FDD |  |
| Dedicated CORESET configuration |  | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
| OCNG Pattern |  | OP.1 defined in A.2.1 |  | OP.1 defined in A.2.1 |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  | ULBWP.0.1 |  |
| SSB configuration |  | SSB.1 FR1 |  | SSB.1 FR1 |  |
| SMTC configuration |  | #1: SMTC.2 for Cell 1 #2: SMTC.6 for Cell 2 |  | #1: SMTC.2 for Cell 1 #2: SMTC.6 for Cell 2 |  |
| RLM-RS |  | SSB |  | SSB |  |
| Qrxlevmin | dBm/SCS | -130 |  | -130 |  |
| Pcompensation | dB | 0 |  | 0 |  |
| Qhysts | dB | 0 |  | 0 |  |
| Qoffsets, n | dB | 0 |  | 0 |  |
| Cell_selection_and_ reselection_quality_measurement |  | SS-RSRP |  | SS-RSRP |  |
|  | dB | 16 | 13 | -infinity | 16 |
| Note2 | dBm/SCS | -98 |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |  |
|  | dB | 16 | 13 | -infinity | 16 |
| SS-RSRP Note3 | dBm/SCS | -82 | -85 | -infinity | -82 |
| Io | dBm/9.36 MHz | -53.94 | -56.84 | -70.05 | -53.94 |
| Treselection | s | 0 | 0 | 0 | 0 |
| SnonIntrasearchP | dB | 50 |  | 50 |  |
| Threshx, highP | dB | 48 |  | 48 |  |
| Threshserving, lowP | dB | 54 |  | 44 |  |
| Threshx, lowP | dB | 50 |  | 40 |  |
| Propagation Condition |  | AWGN |  |  |  |

The cell reselection delay to a lower priority cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Registration procedure for mobility and periodic registration update on Cell 2.

The cell re-selection delay to a lower priority cell shall be less than 36 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to a lower priority cell can be expressed as: Tevaluate, NR_inter + TSI-NR,

Where:

Tevaluate, NR_inter	See Table 4.2C.2.4-1 in TS 38.133 [6] clause 4.2C.2.4

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280ms is assumed in this test case provided that SIB1 and SIB19 are scheduled with 80ms period.

(NLEO,simul=1)(NLEO,simul=2,3,4)

<< End of Changes >>