3GPP TSG-RAN5 Meeting #108	R5-255416r1

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 1017 | rev | 1 | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to PDSCH RMC for PDCCH test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | QUALCOMM Europe Inc. - Italy |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Table A.3.3.1.1-3 appears twice - The “Number of DMRS REs” value is not aligned to the default dmrs-AdditionalPosition IE value specified in 38.508-1 for DCI Format 1_0 in PDSCH reference channel tables for PDCCH performance requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Re-number Table A.3.3.1.1-3 to A.3.3.1.1-4 - Correction to PDSCH reference channel tables under PDCCH performance requirements based on default value of dmrs-AdditionalPosition (pos1) from 38.508-1 Table 5.4.2.0-24 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test specification will be inconsistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-254885r1 |  |  |  |  |  |  |  |  |

<< Start of Changes >>

## A.3.3	Reference measurement channels for PDCCH performance requirements

### A.3.3.1	FDD

#### A.3.3.1.1	Reference measurement channels for SCS 15 kHz FR1

Table A.3.3.1.1-1: PDCCH Reference Channels (Time domain allocation 1 symbol)

| Parameter | Unit | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.1-1.1 FDD | R.PDCCH.1-1.2 FDD | R.PDCCH.1-1.3 FDD |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 |  |  |  |
| CORESET frequency domain allocation |  | 48 | 48 | 48 |  |  |  |
| CORESET time domain allocation |  | 1 | 1 | 1 |  |  |  |
| Aggregation level |  | 4 | 4 | 8 |  |  |  |
| DCI Format |  | 1_0 | 1_1 | 1_1 |  |  |  |
| Payload (without CRC) | Bits | 39 | 52 | 52 |  |  |  |

Table A.3.3.1.1-2: PDCCH Reference Channel (Time domain allocation 2 symbols)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.1-2.1 FDD | R.PDCCH.1-2.2 FDD | R.PDCCH.1-2.3 FDD | R.PDCCH.1-2.4 FDD | R.PDCCH.1-2.5 FDD | R.PDCCH.1-2.6 FDD | R.PDCCH.1-2.7 FDD |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| CORESET frequency domain allocation |  | 24 | 24 | 24 | 48 | 48 | 48 | 48 |
| CORESET time domain allocation |  | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| Aggregation level |  | 2 | 4 | 2 | 4 | 8 | 16 | 8 |
| DCI Format |  | 1_0 | 1_0 | 1_1 | 1_1 | 1_1 | 1_0 | 2_6 |
| Payload (without CRC) | Bits | 39 | 39 | 52 | 52 | 52 | 39 | 12 |

Table A.3.3.1.1-3: PDCCH Reference Channel (Time domain allocation 3 symbols)

| Parameter | Unit | Value |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDCCH.1-3.1 FDD |  |  |  |  |  |  |
| Subcarrier spacing | kHz | 15 |  |  |  |  |  |  |
| CORESET frequency domain allocation |  | 24 |  |  |  |  |  |  |
| CORESET time domain allocation |  | 3 |  |  |  |  |  |  |
| Aggregation level |  | 4 |  |  |  |  |  |  |
| DCI Format |  | 1_0 |  |  |  |  |  |  |
| Payload (without CRC) | Bits | 35Note1 |  |  |  |  |  |  |
| Note1: The payload is corresponding to BWP size of 15RB |  |  |  |  |  |  |  |  |

Table A.3.3.1.1-4: Additional PDSCH Reference Channel FDD

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| DCI Format |  | 1_0 | 1_1 |
| Channel bandwidth | MHz | 10 | 10 |
| Subcarrier spacing | kHz | 15 | 15 |
| Number of allocated resource blocks | PRBs | 52 | 52 |
| Number of consecutive PDSCH symbols |  | 12 | 12 |
| Allocated slots per 2 frames | Slots | 19 | 19 |
| MCS table |  | 64QAM | 64QAM |
| MCS index |  | 4 | 4 |
| Modulation |  | QPSK | QPSK |
| Target Coding Rate |  | 0.30 | 0.30 |
| Number of MIMO layers |  | 1 | 1 |
| Number of DMRS REs (Note 5) |  | 24 | 12 |
| Overhead for TBS determination |  | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 3752 | 4096 |
| Transport block CRC per Slot |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |
| For Slots i = 1,…, 19 | Bits | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |
| For Slot i = 0 | CBs | N/A | N/A |
| For Slots i = 1,…, 19 | CBs | 1 | 1 |
| Binary Channel Bits Per Slot |  |  |  |
| For Slot i = 0 | Bits | N/A | N/A |
| For Slots i = 10, 11 | Bits | 11856 | 13104 |
| For Slots i = 1,…, 9, 12, …, 19 | Bits | 12480 | 13728 |
| Max. Throughput averaged over 2 frames | Mbps | 3.5644 | 3.8912 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms. Note 2: Slot i is slot index per 2 frames. Note 3: For PDSCH & PDSCH DMRS precoding the following configuration shall be used: Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity. Note 4: For tests with DRX configured, PDCCH and PDSCH shall only be scheduled during the DRX on duration of the test. Note 5: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data. |  |  |  |

<< Skipped sections >>

Table A.3.3.2.2-3: Additional PDSCH Reference Channel TDD

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| DCI Format |  | 1-0 | 1-1 | 1-1 |
| TDD UL/DL pattern |  | FR1.30-1 | FR1.30-1 | FR1.30-1 |
| Channel bandwidth | MHz | 40 | 40 | 20 |
| Subcarrier spacing | kHz | 30 | 30 | 30 |
| Allocated resource blocks | PRBs | 106 | 106 | 51 |
| Number of consecutive PDSCH symbols |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 4 | 4 | 4 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6) for i from {1,…,39} |  | 12 | 12 | 12 |
| Allocated slots per 2 frames |  | 31 | 31 | 31 |
| MCS table |  | 64QAM | 64QAM | 64QAM |
| MCS index |  | 4 | 4 | 4 |
| Modulation |  | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 0.30 | 0.3 | 0.3 |
| Number of MIMO layers |  | 1 | 1 | 1 |
| Number of DMRS rEs (Note 3) |  |  |  |  |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} |  | 12 | 6 | 6 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6) for i from {1,…,39} |  | 24 | 12 | 12 |
| Overhead for TBS determination |  | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 2280 | 2664 | 1288 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6) for i from {1,…,39} | Bits | 7552 | 8456 | 4032 |
| Transport block CRC per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 16 | 16 | 16 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6) for i from {1,…,39} | Bits | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | CBs | 1 | 1 | 1 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6) for i from {1,…,39} | CBs | 1 | 2 | 1 |
| Binary Channel Bits Per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 10) = {8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = 7 for i from {0,…,39} | Bits | 7632 | 8904 | 4284 |
| For Slot i, if mod(i, 10) = {0,1,2,3,4,5,6) for i from {1,…,39} | Bits | 25440 | 27984 | 13464 |
| Max. Throughput averaged over 2 frames | Mbps | 10.65 | 11.94 | 5.70 |
| Note 1: For PDSCH & PDSCH DMRS precoding the following configuration shall be used: Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity. Note 2: For tests with DRX configured, PDCCH and PDSCH shall only be scheduled during the DRX on duration of the test. Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data. |  |  |  |  |

<< Skipped sections >>

Table A.3.3.2.5-3: Additional PDSCH Reference Channel TDD

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| DCI format |  | DCI 1_0 | DCI 1_1 | DCI 1_0 Test 2-2 |
| TDD UL/DL pattern |  |  |  |  |
| Channel bandwidth | MHz | 100 | 100 | 100 |
| Subcarrier spacing | kHz | 120 | 120 | 120 |
| Allocated resource blocks | PRBs | 66 | 66 | 66 |
| Number of consecutive PDSCH symbols |  |  |  |  |
| For Slots 0, 5 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 9 | 9 | 8 |
| For Slot i, if mod(i, 5) = {1,2} for i from {1,…,159} |  | 13 | 13 | 12 |
| For Slot i, if mod(i, 5) = {0} for i from {6,…,159} |  | 13 | 13 | 12 |
| Allocated slots per 2 frames |  | 126 | 126 | 126 |
| MCS table |  | 64QAM | 64QAM | 64QAM |
| MCS index |  | 4 | 4 | 4 |
| Modulation |  | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 0.30 | 0.30 | 0.30 |
| Number of MIMO layers |  | 1 | 1 | 1 |
| Number of DMRS REs (Note 4) |  |  |  |  |
| For Slots 0, 5 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} |  | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} |  | 24 | 12 | 24 |
| For Slot i, if mod(i, 5) = {1,2} for i from {1,…,159} |  | 24 | 12 | 24 |
| For Slot i, if mod(i, 5) = {0} for i from {6,…,159} |  | 24 | 12 | 24 |
| Overhead for TBS determination |  | 6 | 6 | 6 |
| Information Bit Payload per Slot |  |  |  |  |
| For Slots 0, 5 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 3104 | 3624 | 2600 |
| For Slot i, if mod(i, 5) = {1,2} for i from {1,…,159} | Bits | 4992 | 5504 | 4480 |
| For Slot i, if mod(i, 5) = {0} for i from {6,…,159} |  | 4992 | 5504 | 4480 |
| Transport block CRC per Slot |  |  |  |  |
| For Slots 0, 5 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 16 | 16 | 16 |
| For Slot i, if mod(i, 5) = {1,2} for i from {1,…,159} | Bits | 24 | 24 | 24 |
| For Slot i, if mod(i, 5) = {0} for i from {6,…,159} |  | 24 | 24 | 24 |
| Number of Code Blocks per Slot |  |  |  |  |
| For Slots 0, 5 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | CBs | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | CBs | 1 | 1 | 1 |
| For Slot i, if mod(i, 5) = {1,2} for i from {1,…,159} | CBs | 1 | 1 | 1 |
| For Slot i, if mod(i, 5) = {0} for i from {6,…,159} |  | 1 | 1 | 1 |
| Binary Channel Bits Per Slot |  |  |  |  |
| For Slots 0, 5 and Slot i, if mod(i, 5) = 4 for i from {0,…,159} | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 5) = 3 for i from {0,…, 159} | Bits | 10296 | 11880 | 8712 |
| For Slot i, if mod(i, 5) = {1,2} for i from {1,…,159} | Bits | 16632 | 18216 | 15048 |
| For Slot i, if mod(i, 5) = {0} for i from {6,…,159} |  | 16632 | 18216 | 15048 |
| Max. Throughput averaged over 2 frames | Mbps | 28.429 | 31.667 | 25.216 |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms. Note 2: Slot i is slot index per 2 frames Note 3: For PDSCH & PDSCH DMRS precoding the following configuration shall be used: Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity. Note 4: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data. |  |  |  |  |

<< End of changes >>