3GPP TSG-RAN5 Meeting #109	R R5-255921

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 3636 | rev | - | Current version: | 19.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of IE SIB24 and MBS-SessionInfoListMulticast for MBS Multicast reception in RRC_INACTIVE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS_enh_5MBS_Ph2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The contents of IE SIB24 and MBS-SessionInfoListMulticast haven’t been added |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the contents of IE SIB24 and MBS-SessionInfoListMulticast for MBS Multicast reception in RRC_INACTIVE according to TS 38.331 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases for MBS Multicast reception in RRC_INACTIVE will be not implementable |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.2, 4.6.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

4.6.2	System information blocks

<Unchanged Section skips>

–	SIB24

Table 4.6.2-23: SIB24

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB24-r18 ::= SEQUENCE { |  |  |  |
| multicastMCCH-Config-r18 SEQUENCE { |  |  |  |
| mcch-RepetitionPeriodAndOffset-r17 CHOICE { |  |  |  |
| rf2-r17 | 0 |  |  |
| } |  |  |  |
| mcch-WindowStartSlot-r17 | 2 |  |  |
| mcch-WindowDuration-r17 | sl8 |  |  |
| mcch-ModificationPeriod-r17 | rf8 |  |  |
| } |  |  |  |
| cfr-ConfigMCCH-MTCH-r18 | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| } |  |  |  |

<Unchanged Section skips>

4.6.7	MBS information elements

<Unchanged Section skips>

–	MBS-SessionInfoListMulticast

Table 4.6.7-6A: MBS-SessionInfoListMulticast

| Derivation Path: TS 38.331 [6], clause 6.3.6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MBS-SessionInfoListMulticast-r18 ::= SEQUENCE (SIZE (1..maxNrofMBS-Session-r17)) OF MBS-SessionInfoMulticast-r18 { | 1 entry |  |  |
| MBS-SessionInfoMulticast-r18[1] SEQUENCE { |  | entry 1 |  |
| mbs-SessionId-r18 | TMGI |  |  |
| g-RNTI-r18 | RNTI-Value |  |  |
| mrb-ListMulticast-r18 SEQUENCE (SIZE (1..maxMRB-r17)) OF MRB-InfoMulticast-r18 { | 1 entry |  |  |
| MRB-InfoMulticast-r18[1] { |  | entry 1 |  |
| pdcp-Config-r18 SEQUENCE { |  |  |  |
| pdcp-SN-SizeDL-r18 | len12bits |  |  |
| headerCompression-r18 CHOICE { |  |  |  |
| notUsed | NULL |  |  |
| } |  |  |  |
| t-Reordering-r17 | Not present |  |  |
| } |  |  |  |
| rlc-Config-r18 SEQUENCE { |  |  |  |
| logicalChannelIdentity-r18 CHOICE { |  |  |  |
| logicalChannelIdentitymulticast-r18 | 1 |  |  |
| } |  |  |  |
| sn-FieldLength-r18 | size6 |  |  |
| t-Reassembly-r18 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| mtch-SchedulingInfo-r18 | Not present |  |  |
| mtch-NeighbourCell-r18 | Not present |  |  |
| pdsch-ConfigIndex-r18 | Not present |  |  |
| mtch-SSB-MappingWindowIndex-r18 | Not present |  |  |
| thresholdIndex-r18 | 0 |  |  |
| pdcp-SyncIndicator-r18 | Not present |  |  |
| stopMonitoringRNTI-r18 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

<End of Changed Section>