3GPP TSG-RAN5 Meeting #109	R5-255985

Dallas, US, Nov 17th - 21st, 2025

Agenda item:	6.3.8.7

Source: 	China Unicom

Title: 	Discussion on R15 legacy signalling TCs applicability analysis for eRedCap UE

Document for:	Discussion

- Introduction

The eRedCap UE is defined as a further reduced NR capability UE type in R18，which can also be combined with R16 and forward enhanced NR features such as URLLC and UE power saving for vertical interests. The WID on UE Conformance – Enhanced support of reduced capability NR devices plus CT1 aspects in [1] has been introduced to RAN5, and the legacy signalling test cases adjustment for eRedCap UE has been discussed in [2] with the conclusion “Meeting concluded analysis and findings are technically sound and thorough. Proposed way forward seems technically feasible. Commercial need for legacy test cases to be executed on eRedCap UE must be established first and if needed, timeline input from interested Operators must be sought before starting the work. Rapporteurs and supporting companies of eRedCap to discuss about industry need for legacy test cases”.

This discussion paper shares China Unicom’s view on legacy test case modification for eRedCap UE.

- Discussion

The test coverage and status for this eRedCap UE WI is summarized as follows:

- RF test coverage: includes single carrier RF TCs and redcap specific RF TCs (MOP, Refsens). The RF test scope for eRedCap UE has been fully covered.

- RRM test coverage: No new RRM eRedCap specific TC was introduced in core WI, the RRM TCs for RedCap UE are also applicable to  eRedCap UE. The RRM test scope for eRedCap UE is expected to be completed by the R5#109 meeting.

- Demodulation test coverage: includes eRedCap specific Demod TCs and several RedCap specific demod TCs. The Demod test scope for eRedCap UE is expected to be completed by the R5#109 meeting.

- Signalling test coverage: includes eRedCap specific signalling TCs, legacy R15 signalling TCs for basic function and R16 and forward NR features TCs for industrial interests.  The eRedCap specific test cases have already been completed, however the legacy R15 test cases modifications for basic function and R16 and forward NR features test coverage modifications have not been initiated yet.

Observation 1: The legacy R15 signalling test coverage can be achieved with appropriate modification in the test environment and configurations.

The corresponding CRs for R15 legacy signalling test modification for eRedCap UE are provided from [3] to [9] in RAN5#109.

Observation 2: The earliest need of RAN5 conformance test cases implementation for eRedCap UE (to support lab/filed test, trials or commercial deployments) is 2026 H2.

- Proposal

The following proposals are made in this discussion paper.

Proposal 1: Initiate the modification of R15 legacy signalling test coverage at RAN5#109 meeting.

Proposal 2: Revisit the modification of legacy signalling test coverage for R16 and forward release NR features once there is industry interest.

- References

- RP-242487, “Revised WID on UE Conformance – Enhanced support of reduced capability NR devices plus CT1 aspects”, RAN#106, Dec 2024

- R5-247577,“Discussion paper on legacy test cases adjustment for eRedcap”, Huawei, RAN5#105, Nov 2024

- R5-256176, “Update IDLE mode  test case 6.1.2.3a to test eRedCap UE”, Huawei,  HiSilicon, China Unicom, RAN5#109, Nov 2025

- R5-256177，Update MAC test case 7.1.1.3.4 and 7.1.1.9.1 to test eRedCap UE，Huawei,  HiSilicon, China Unicom，RAN5#109, Nov 2025

- R5-256178	Update PDCP test case 7.1.3.4.1 and 7.1.3.5.4 to test eRedCap UE，Huawei,  HiSilicon, China Unicom, RAN5#109, Nov 2025

- R5-256179	Update RLC test case 7.1.2.3.x to test eRedCap UE，Huawei,  HiSilicon, China Unicom, RAN5#109, Nov 2025

- R5-256180	Update RRC test case 8.1.5.8.1and 8.1.5.2.2 to test eRedCap UE，Huawei,  HiSilicon, China Unicom, RAN5#109, Nov 2025

- R5-256181	Update TBsize test case 7.1.1.4.x.x to test eRedCap UE，Huawei,  HiSilicon, China Unicom, RAN5#109, Nov 2025

- R5-256182	Update eRedCap UE Test applicability for Legacy test cases，Huawei,  HiSilicon, China Unicom, RAN5#109, Nov 2025