3GPP TSG-RAN5 Meeting #109	R5-256154

# Dallas, US, Nov.17th – 21st 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 5209 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Cov_Enh2 SIG test case 7.1.1.3.24 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_cov_enh2-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To update cov enh2 SIG test case 7.1.1.3.24 in following aspects: - Conformant requirements needs to be updated to align with latest version of 38.214. - Typo in test procedure needs to be corrected. - Table 7.1.1.3.24.3.3-1 only applies to NR SA, needs to be updated to also cover MR-DC cases - Transform precoder indicator field shall also be included in the UL grant provided in step 2, 6, 10 and 14. - Condition TRANSFORM_PRECODER_ENABLED is needed when Transform precoder indicator = 0 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Conformant requirements are updated. - Typos are corrected; - message contents are updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

7.1.1.3.24	Correct Handling of UL grant / Dynamic waveform switch / DCI format 0_1

7.1.1.3.24.1	Test Purpose (TP)

(1)

with { UE is in RRC_connected state and is configured with transformPrecoder = disabled and dynamicTransformPrecoderFieldPresenceDCI-0-1-r18 in active BWP }

ensure that {

when { UE receives an DCI format 0_1 with Transform precoder indicator = 0 }

then { UE transmits PUSCH with DFT-s-OFDM waveform }

}

(2)

with { UE is in RRC_connected state and is configured with msg3-transformPrecoder not configured, transformPrecoder not configured and dynamicTransformPrecoderFieldPresenceDCI-0-1-r18 in active BWP }

ensure that {

when { UE receives an DCI format 0_1 with Transform precoder indicator = 0 }

then { UE transmits PUSCH with DFT-s-OFDM waveform }

}

(3)

with { UE is in RRC_connected state and is configured with transformPrecoder = enabled and dynamicTransformPrecoderFieldPresenceDCI-0-1-r18 in active BWP }

ensure that {

when { UE receives an DCI format 0_1 with Transform precoder indicator = 1 }

then { UE transmits PUSCH with CP-OFDM waveform }

}

(4)

with { UE is in RRC_connected state and is configured with msg3-transformPrecoder = enabled, transformPrecoder not configured and dynamicTransformPrecoderFieldPresenceDCI-0-1-r18 in active BWP }

ensure that {

when { UE receives an DCI format 0_1 with Transform precoder indicator = 1 }

then { UE transmits PUSCH with CP-OFDM waveform }

}

7.1.1.3.24.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.212, clause 7.3.1.1.2 and TS 38.214 clause 6.1.3. Unless otherwise stated these are Rel-18 requirements.

[TS 38.212, clause 7.3.1.1.2]

DCI format 0_1 is used for the scheduling of one or multiple PUSCH in one cell, or indicating CG downlink feedback information (CG-DFI) to a UE.

The following information is transmitted by means of the DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or SP-CSI-RNTI or MCS-C-RNTI:

…

Otherwise, all the remaining fields are set as follows:

-	UL/SUL indicator - 0 bit for UEs not configured with supplementaryUplink in ServingCellConfig in the cell or UEs configured with supplementaryUplink in ServingCellConfig in the cell but only one carrier in the cell is configured for PUSCH transmission; otherwise, 1 bit as defined in Table 7.3.1.1.1-1.

-	Bandwidth part indicator - 0, 1 or 2 bits as determined by the number of UL BWPs  configured by higher layers, excluding the initial UL bandwidth part. The bitwidth for this field is determined as bits, where

-	 if , in which case the bandwidth part indicator is equivalent to the ascending order of the higher layer parameter BWP-Id;

-	otherwise , in which case the bandwidth part indicator is defined in Table 7.3.1.1.2-1;

If a UE does not support active BWP change via DCI, the UE ignores this bit field.

…

-	Transform precoder indicator - 0 or 1 bit

-	1 bit if the higher layer parameter dynamicTransformPrecoderIndicationDCI-0-1 is configured to 'enabled ' and if the UE is configured to monitor DCI format 0_1 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI, where the bit value of 0 indicates that transform precoder is enabled and the bit value of 1 indicates that transform precoder is disabled. For a DCI format 0_1 with CRC scrambled by CS-RNTI and the value indicated by new data indicator field is 0, or for a DCI format 0_1 with CRC scrambled by SP-CSI-RNTI, the bit is reserved.

-	0 bit otherwise.

…

[TS 38.214, clause 6.1.3]

...

For PUSCH transmission scheduled by a PDCCH with CRC scrambled by CS-RNTI with NDI=1, C-RNTI, or MCS-C-RNTI or SP-CSI-RNTI:

…

-	If the DCI with the scheduling grant was not received with DCI format 0_0

-	If the DCI with the scheduling grant was received with DCI format 0_1 or 0_2 with CRC scrambled by C-RNTI, MCS-RNTI, or CS-RNTI with NDI=1 and if the UE is configured with a higher layer parameter dynamicTransformPrecoderIndicationDCI-0-1 in pusch-Config for DCI format 0_1 or dynamicTransformPrecoderIndicationDCI-0-2 in pusch-Config for DCI format 0_2 and the higher layer parameter is set to 'enabled',

-	the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the Transform precoder indicator field in the DCI with the scheduling grant.

…

-	If resourceAllocation in pusch-Config for DCI format 0_1 or resourceAllocationDCI-0-2 in pusch-Config for DCI format 0_2 is set to resourceAllocationType0, or if the resource allocation is set to resource allocation type 0 according to the DCI configuration as described in clauses 7.3.1.1.2 and 7.3.1.1.3 of [6, TS 38.212], or if dmrs-Type in DMRS-UplinkConfig is set to ‘type 2’ for this PUSCH transmission, the UE does not expect that the Transform precoder indicator field in the DCI with the scheduling grant indicates that transform precoding is enabled.

-	If the UE is configured with the higher layer parameter dmrs-TypeEnh in DMRS-UplinkConfig, and if the scheduling grant indicates that transform precoding is enabled for the scheduled PUSCH transmission, the UE ignores the higher layer parameters dmrs-TypeEnh in DMRS-UplinkConfig, if configured, for the DM-RS transmission of the scheduled PUSCH transmission.

-	Otherwise,

-	If the UE is configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter.

-	If the UE is not configured with the higher layer parameter transformPrecoder in pusch-Config, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder.

For PUSCH transmission with a configured grant

-	If the UE is configured with the higher layer parameter transformPrecoder in configuredGrantConfig, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to this parameter.

-	If the UE is not configured with the higher layer parameter transformPrecoder in configuredGrantConfig, the UE shall, for this PUSCH transmission, consider the transform precoding either enabled or disabled according to the higher layer configured parameter msg3-transformPrecoder.

…

7.1.1.3.24.3	Test description

7.1.1.3.24.3.1	Pre-test conditions

Same Pre-test conditions as in clause 7.1.1.0.

7.1.1.3.24.3.2	Test procedure sequence

Table 7.1.1.3.24.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits an RRCReconfiguration message to reconfigure dedicated BWP #1 with dynamicTransformPrecoderFieldPresenceDCI-0-1-r18 = enabled and transformPrecoder = disabled. (Note 1) | <-- | NR RRC: RRCReconfiguration | - | - |
| 2 | The UE transmits an RRCReconfigurationComplete message. (Note 2) | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 3 | The SS sends a DCI format 0_1 with Transform precoder indicator field set to 0. | <-- | DCI format 0_1 (Transform precoder indicator = 0) | - | - |
| 4 | Check: Does the UE transmit PUSCH with DFT-s-OFDM waveform? | --> | PUSCH (DFT-s-OFDM waveform) | 1 | P |
| 5 | The SS transmits an RRCReconfiguration message to reconfigure dedicated BWP #1 with transformPrecoder = enabled. (Note 1) | <-- | NR RRC: RRCReconfiguration | - | - |
| 6 | The UE transmits an RRCReconfigurationComplete message. (Note 2) | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 7 | The SS sends a DCI format 0_1 with Transform precoder indicator field set to 1. | <-- | DCI format 0_1 (Transform precoder indicator = 1) | - | - |
| 8 | Check: Does the UE transmit PUSCH with CP-OFDM waveform? | --> | PUSCH (CP-OFDM waveform) | 3 | P |
| 9 | The SS transmits an RRCReconfiguration message to set transformPrecoder in dedicated BWP #1 as not present. (Note 1) | <-- | NR RRC: RRCReconfiguration | - | - |
| 10 | The UE transmits an RRCReconfigurationComplete message. (Note 2) | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 11 | The SS sends a DCI format 0_1 with Transform precoder indicator field set to 0. | <-- | DCI format 0_1 (Transform precoder indicator = 0) | - | - |
| 12 | Check: Does the UE transmit PUSCH with DFT-s-OFDM waveform? | --> | PUSCH (DFT-s-OFDM waveform) | 2 | P |
| 13 | The SS transmits an RRCReconfiguration message to reconfigure msg3-transformPrecoder = enabled. (Note 1) | <-- | NR RRC: RRCReconfiguration | - | - |
| 14 | The UE transmits an RRCReconfigurationComplete message. (Note 2) | --> | NR RRC: RRCReconfigurationComplete | - | - |
| 15 | The SS sends a DCI format 0_1 with Transform precoder indicator field set to 1. | <-- | DCI format 0_1 (Transform precoder indicator = 1) | - | - |
| 16 | Check: Does the UE transmit PUSCH with CP-OFDM waveform? | --> | PUSCH (CP-OFDM waveform) | 4 | P |
| Note 1: For EN-DC the NR RRCReconfiguration message is contained in RRCConnectionReconfiguration. Note 2: For EN-DC the NR RRCReconfigurationComplete message is contained in RRCConnectionReconfigurationComplete. |  |  |  |  |  |

7.1.1.3.24.3.3	Specific message contents

Table 7.1.1.3.24.3.3-1: RRCReconfiguration (Step 1, 5, 9 and 13, Table 7.1.1.3.24.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-13 with condition NR |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |
| secondaryCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) Table 7.1.1.3.24.3.3-2 | EN-DC |
| nonCriticalExtension SEQUENCE { |  |  |  |
| masterCellGroup | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) Table 7.1.1.3.24.3.3-2 | NR |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.24.3.3-2: CellGroupConfig (Table 7.1.1.3.24.3.3-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| mac-CellGroupConfig | Not present |  |  |
| physicalCellGroupConfig | Not present |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | Not present |  |  |
| reconfigurationWithSync | Not present |  |  |
| rlf-TimersAndConstants | Not present |  |  |
| spCellConfigDedicated | ServingCellConfig | Table 7.1.1.3.24.3.3-3 |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.24.3.3-3: ServingCellConfig (Table 7.1.1.3.24.3.3-2)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-167 with condition DLBWP_add and ULBWP_add |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| uplinkBWP-ToAddModList SEQUENCE (SIZE (1..maxNrofBWPs)) OF SEQUENCE { | 1 entry |  |  |
| BWP-Uplink[1] SEQUENCE { |  | entry 1 |  |
| bwp-Common SEQUENCE { |  |  |  |
| rach-ConfigCommon CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| msg3-transformPrecoder | Not present |  | Step 1, Step 5, Step 9 |
|  | enabled |  | Step 13 |
| } |  |  |  |
| } |  |  |  |
| bwp-Dedicated SEQUENCE { |  |  |  |
| pusch-Config CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| transformPrecoder | disabled |  | Step 1 |
|  | enabled |  | Step 5 |
|  | Not present |  | Step 9, Step 13 |
| dynamicTransformPrecoderFieldPresenceDCI-0-1-r18 | enabled |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| firstActiveUplinkBWP-Id | 1 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 7.1.1.3.24.3.3-4: Physical layer parameters for DCI format 0_1 (Step 3, 6, 11 and 14, Table 7.1.1.3.24.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.3.6.1.1.2-1 with condition DWS and TRANSFORM_PRECODER_ENABLED |  |  |  |
| --- | --- | --- | --- |
| Parameter |  | Value | Value in binary | Condition |
| Transform precoder indicator |  | A value of "0" indicates that transform precoder is enabled, i.e. DFT-s-OFDM waveform is used. | “0” |  |

Table 7.1.1.3.24.3.3-5: Physical layer parameters for DCI format 0_1 (Step 2, 7, 10 and 15, Table 7.1.1.3.24.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.3.6.1.1.2-1 with condition DWS |  |  |  |
| --- | --- | --- | --- |
| Parameter |  | Value | Value in binary | Condition |
| Transform precoder indicator |  | A value of "1" indicates that transform precoder is disabled, i.e. CP-OFDM waveform is used. | “1” |  |

<End of Change 1>