3GPP TSG RAN WG5 #109	R5-256467

# Dallas, Texas, USA 17th - 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 4256 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Annex F modification for two TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Update Annex F for two test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add TC7.6.2.9 and TC7.6.5.1 to Table F.1.1.2-6 and Table F.1.3.2-4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test cases not finished. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | F.1.1.2, F.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<< START OF CHANGES 1 >>>

Table F.1.1.2-6: Maximum test system uncertainty for RRM requirements for SA FR2 test cases

| Subclause |  | Maximum Test System Uncertainty1 |  | Derivation of Test System Uncertainty |  |
| --- | --- | --- | --- | --- | --- |
| 7.1.1.1 NR SA FR2 cell re-selection |  | Noc, averaged over BWConfig, ±5.65 dB Noc, averaged over Measurement PRB, ±5.65 dB Es1/Noc averaged over BWConfig ±0.3 dB Es1/Noc averaged over Measurement PRB ±0.3 dB Es2/Noc averaged over BWConfig ±0.3 dB Es2/Noc averaged over Measurement PRB ±0.3 dB |  | Noc averaged over BWConfig is the AWGN absolute power of RF channel 1 averaged over BWConfig Noc averaged over Measurement PRB is the AWGN absolute power of RF channel 1 averaged over Measurement PRB Es1/Noc averaged over BWConfig is Ratio of cell 1 signal / AWGN averaged over BWConfig Es1/Noc averaged over Measurement PRB is Ratio of cell 1 signal / AWGN averaged over Measurement PRB Es2/Noc averaged over BWConfig is Ratio of cell 2 signal / AWGN averaged over BWConfig Es2/Noc averaged over Measurement PRB is Ratio of cell 2 signal / AWGN averaged over Measurement PRB |  |
| 7.1.1.2 NR SA FR2-FR2 cell re-selection |  | Noc1, averaged over BWConfig, ±5.65 dB Noc1, averaged over Measurement PRB, ±5.65 dB Noc2, averaged over BWConfig, ±5.65 dB Noc2, averaged over Measurement PRB, ±5.65 dB Es1/Noc1 averaged over BWConfig ±0.3 dB Es1/Noc1 averaged over Measurement PRB ±0.3 dB Es2/Noc2 averaged over BWConfig ±0.3 dB Es2/Noc2 averaged over Measurement PRB ±0.3 dB |  | Noc1 averaged over BWConfig is the AWGN absolute power of RF channel 1 averaged over BWConfig Noc1 averaged over Measurement PRB is the AWGN absolute power of RF channel 1 averaged over Measurement PRB Noc2 averaged over BWConfig is the AWGN absolute power of RF channel 2 averaged over BWConfig Noc2 averaged over Measurement PRB is the AWGN absolute power of RF channel 2 averaged over Measurement PRB Es1/Noc1 averaged over BWConfig is Ratio of cell 1 signal / AWGN averaged over BWConfig Es1/Noc1 averaged over Measurement PRB is Ratio of cell 1 signal / AWGN averaged over Measurement PRB Es2/Noc2 averaged over BWConfig is Ratio of cell 2 signal / AWGN averaged over BWConfig Es2/Noc2 averaged over Measurement PRB is Ratio of cell 2 signal / AWGN averaged over Measurement PRB |  |
| 7.1.1.3 NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  | Same as 7.1.1.1 |  | Same as 7.1.1.1 |  |
| 7.1.1.4 NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  | Same as 7.1.1.1 |  | Same as 7.1.1.1 |  |
| 7.1.1.5 NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion |  | Same as 7.1.1.2 |  | Same as 7.1.1.2 |  |
| 7.1.1.6 NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion |  | Same as 7.1.1.2 |  | Same as 7.1.1.2 |  |
| 7.1.1.7 NR SA FR2 cell re-selection for power class 6 UE configured with highSpeedMeasFlagFR2-r17 |  | Same as 7.1.1.2 |  | Same as 7.1.1.2 |  |
| 7.3.1.2 NR SA FR2 Intra-frequency handover; unknown target cell |  | Average Noc over BW, ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês1 / Noc over BW, ±0.3 dB, f <= 40.8 GHz Average Ês1 / Noc over meas PRB, ±0.3 dB, f <= 40.8 GHz Average Ês2 / Noc over BW, ±0.3 dB, f <= 40.8 GHz Average Ês2 / Noc over meas PRB, ±0.3 dB, f <= 40.8 GHz |  | Average Noc over BW is AWGN absolute power averaged over BW. Average Noc over meas PRB is AWGN absolute power averaged over measured PRB. Average Ês1 / Noc over BW is the ratio of cell 1 signal / AWGN averaged over BW Average Ês1 / Noc over measured PRB is the ratio of cell 1 signal / AWGN averaged over measured PRB Average Ês2 / Noc over BW is the ratio of cell 2 signal / AWGN averaged over BW. Average Ês2 / Noc over measured PRB is the ratio of cell 2 signal / AWGN averaged over measured PRB. Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.3.1.3 NR SA FR2-FR2 Inter-frequency handover; unknown target cell |  | Average Noc1 over BW ±5.65 dB, f <= 40.8 GHz Noc1 over meas PRBs ±5.65 dB f <= 40.8 GHz Average Noc2 over BW ±5.65 dB, f <= 40.8 GHz Noc2 over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês1 / Noc1 over BW, ±0.3 dB, f <= 40.8 GHz Average Ês1 / Noc1 over meas PRB, ±0.3 dB, f <= 40.8 GHz Average Ês2 / Noc2 over BW, ±0.3 dB, f <= 40.8 GHz Average Ês2 / Noc2 over meas PRB, ±0.3 dB, f <= 40.8 GHz |  | Average Noc1 over BW is AWGN absolute power averaged over BW on freq 1. Average Noc1 over meas PRB is AWGN absolute power averaged over measured PRB on freq 1. Average Noc2 over BW is AWGN absolute power averaged over BW on freq 2. Average Noc2 over meas PRB is AWGN absolute power averaged over measured PRB on freq 2. Average Ês1 / Noc1 over BW is the ratio of cell 1 signal / AWGN averaged over BW on freq 1. Average Ês1 / Noc1 over measured PRB is the ratio of cell 1 signal / AWGN averaged over measured PRB on freq 1. Average Ês2 / Noc2 over BW is the ratio of cell 2 signal / AWGN averaged over BW on freq 2. Average Ês2 / Noc2 over measured PRB is the ratio of cell 2 signal / AWGN averaged over measured PRB on freq 2. Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.3.1.4 NR SA FR1-FR2 synchronous DAPS handover |  | Noc, averaged over BWConfig, ±5.65 dB Noc, averaged over Measurement PRB, ±5.65 dB Es2/Noc averaged over BWConfig ±0.3 dB Es2/Noc averaged over Measurement PRB ±0.3 dB |  | Noc averaged over BWConfig is the AWGN absolute power of RF channel 2 averaged over BWConfig Noc averaged over Measurement PRB is the AWGN absolute power of RF channel 2 averaged over Measurement PRB Es2/Noc averaged over BWConfig is Ratio of cell 2 signal / AWGN averaged over BWConfig Es2/Noc averaged over Measurement PRB is Ratio of cell 2 signal / AWGN averaged over Measurement PRB |  |
| 7.3.1.5 NR SA FR1-FR2 asynchronous DAPS handover |  | Same as 7.3.1.4 |  | Same as 7.3.1.4 |  |
| 7.3.2.1.1 NR SA FR2 RRC re-establishment |  | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB Average Ês2 / Noc ±0.3 dB Ês2 / Noc over meas PRBs ±0.3 dB |  | Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.3.2.1.2 NR SA FR2 - FR2 RRC re-establishment |  | Average Noc1 ±5.65 dB, f <= 40.8 GHz Noc1 over meas PRBs ±5.65 dB f <= 40.8 GHz Average Noc2 ±5.65 dB, f <= 40.8 GHz Noc2 over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês1 / Noc1 ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Ês2 / Noc over meas PRBs ±0.3 dB |  | Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.3.2.1.3 NR SA FR2 RRC re-establishment without serving cell timing |  | Same as 7.3.2.1.1 |  | Same as 7.3.2.1.1 |  |
| 7.3.2.2.1 NR SA FR2 contention based random access |  | Average Ês ±6.0 dB Meas PRB Ês ±6.0 dB Uplink absolute power measurement ±6.0 dB Uplink relative power measurement ±FFS dB ±[48]Tc Uplink signal transmit timing relative to downlink |  | Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |
| 7.3.2.2.2 NR SA FR2 non-contention based random access |  | Same as 7.3.2.2.1 |  | Same as 7.3.2.2.1 |  |
| 7.3.2.3.1 NR SA FR2-FR2 RRC connection release with redirection |  | Same as 7.3.1.3 |  | Same as 7.3.1.3 |  |
| 7.3.3.1 NR SA FR2 conditional handover |  | Noc averaged over BWConfig, ±5.65 dB Noc, averaged over Measurement PRB, ±5.65 dB Es1/Noc averaged over BWConfig ±0.3 dB Es1/Noc averaged over Measurement PRB ±0.3 dB Es2/Noc ±0.3 dB Noc, averaged over BWConfig, ±5.65 dB Es2/Noc averaged over Measurement PRB ±0.3 dB |  | Noc averaged over BWConfig,is the AWGN absolute power of RF channel 1 averaged over BWConfig, Noc averaged over Measurement PRB,is the AWGN absolute power of RF channel 1 averaged over Measurement PRB, Es1/Noc averaged over BWConfig is Ratio of cell 1 signal / AWGN averaged over BWConfig Es1/Noc averaged over Measurement PRB is Ratio of cell 1 signal / AWGN averaged over Measurement PRB Es2/Noc averaged over BWConfig is Ratio of cell 2 signal / AWGN averaged over BWConfig Es2/Noc averaged over Measurement PRB is Ratio of cell 2 signal / AWGN averaged over Measurement PRB |  |
| 7.3.3.3 |  | Same as 7.3.3.1 |  | Same as 7.3.3.1 |  |
| 7.3.3.5 NR SA FR2 NR conditional handover including target MCG and target SCG from FR1-FR2 NR-DC to FR1-FR2 NR-DC |  | FR1-FR1 Cells: Noc1 ±1.5 dB Ês1 / Noc1 ±0.3 dB Noc3 ±1.5 dB Ês3 / Noc3 ±0.3 dB FR2-FR2 Cells: Noc averaged over BWConfig, ±5.65 dB Noc, averaged over Measurement PRB, ±5.65 dB Es2/Noc averaged over BWConfig ±0.3 dB Es2/Noc averaged over Measurement PRB ±0.3 dB Es4/Noc ±0.3 dB Noc, averaged over BWConfig, ±5.65 dB Es4/Noc averaged over Measurement PRB ±0.3 dB |  | FR1-FR1 Cells: Note: Noc1 is the AWGN on cell 1 frequency Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Noc3 is the AWGN on cell 2 frequency Ês3 / Noc3 is the ratio of cell 2 signal / AWGN FR2-FR2 Cells: Noc averaged over BWConfig,is the AWGN absolute power of RF channel 1 averaged over BWConfig, Noc averaged over Measurement PRB,is the AWGN absolute power of RF channel 1 averaged over Measurement PRB, Es2/Noc averaged over BWConfig is Ratio of cell 1 signal / AWGN averaged over BWConfig Es2/Noc averaged over Measurement PRB is Ratio of cell 2 signal / AWGN averaged over Measurement PRB Es4/Noc averaged over BWConfig is Ratio of cell 4 signal / AWGN averaged over BWConfig Es4/Noc averaged over Measurement PRB is Ratio of cell 2 signal / AWGN averaged over Measurement PRB |  |
| 7.3.4.1 NR SA FR2 RACH based Intra-frequency PCell switch from FR2 to FR2 |  | Same as 7.3.1.2 |  | Same as 7.3.1.2 |  |
| 7.3.4.2 NR SA FR2 RACH-less Intra-frequency PCell switch from FR2 to FR2 |  | Same as 7.3.1.2 |  | Same as 7.3.1.2 |  |
| 7.3.4.3 NR SA FR2 RACH-based Inter-frequency LTM PCell switch from FR2 to FR2 |  | Same as 7.3.1.3 |  | Same as 7.3.1.3 |  |
| 7.4.1.1 SA FR2 UE transmit timing accuracy |  | Noc ±5.65 dB Ês1 / Noc ±0.3 dB ±48Tc Uplink signal transmit timing relative to downlink ±40Tc Relative during UE timing adjustment |  | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |
| 7.4.1.3 SA FR2 UE transmit timing accuracy |  | Noc ±5.65 dB Ês1 / Noc ±0.3 dB ±48Tc Uplink signal transmit timing relative to downlink ±40Tc Relative during UE timing adjustment |  | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |
| 7.4.3.1 NR SA FR2 UE timing advance adjustment accuracy |  | Noc ±5.65 dB Ês1 / Noc ±0.3 dB ±40Tc timing advance adjustment |  | Ês1 / Noc is the ratio of cell 1 signal / AWGN Tc = 1/(480000 x 4096) seconds, the basic timing unit defined in TS 38.211 [7] |  |
| 7.5.1.1 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM-RS in non-DRX mode |  | Same as 5.5.1.1 |  | Same as 5.5.1.1 |  |
| 7.5.1.2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM-RS in non-DRX mode |  | Same as 5.5.1.1 |  | Same as 5.5.1.1 |  |
| 7.5.1.3 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM-RS in DRX mode |  | Same as 5.5.1.1 |  | Same as 5.5.1.1 |  |
| 7.5.1.4 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM-RS in DRX mode |  | Same as 5.5.1.1 |  | Same as 5.5.1.1 |  |
| 7.5.1.9 EN-DC FR2 radio link monitoring UE scheduling restrictions |  | Same as 5.5.1.1 |  | Same as 5.5.1.1 |  |
| 7.5.2.2 SA interruptions at NR SRS carrier-based switching |  | AWGN flatness and signal flatness ±3.6 dB Signal-to-noise ratio uncertainty ±0.3 dB |  | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness |  |
| 7.5.3.4 NR SA FR2 direct SCell activation at SCell addition of known SCell | Average Noc1 ±5.65 dB Average Noc2 ±5.65 dB Average Ês1 / Noc1 ±0.3 dB Average Ês2 / Noc2 ±0.3 dB |  | Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN |  |
| 7.5.3.5 NR SA FR2 direct SCell activation at handover with known SCell | Average Noc1 ±5.65 dB Average Noc2 ±5.65 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Average Ês3 / Noc ±0.3 dB |  | Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc1 is the ratio of cell 2 signal / AWGN Ês3 / Noc2 is the ratio of cell 3 signal / AWGN |  |
| 7.5.3.13 NR SA FR2 SCell Activation of SCell in FR2 intra-band in non-DRX | Average Noc1 ±5.65 dB Average Noc2 ±5.65 dB Average Ês1 / Noc ±0.3 dB Average Ês2 / Noc ±0.3 dB Meas PRB Average Noc1 ±5.65 dB Meas PRB Average Noc2 ±5.65 dB Meas PRB Average Ês1 / Noc ±0.3 dB Meas PRB Average Ês2 / Noc ±0.3 dB |  | Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB are the measurement PRB for SS-RSRP #RBJ to RBJ+19 for CSI-RSRP Meas PRB are # RB0 to # RB274. |  |
| 7.5.3.14 NR SA FR2 SCell Activation for known SCell in FR2 inter-band | Noc1 averaged over BWConfig, ±5.65 dB Noc1, averaged over Measurement PRB, ±5.65 dB Noc2 averaged over BWConfig, ±5.65 dB Noc2, averaged over Measurement PRB, ±5.65 dB Es1/Noc1 averaged over BWConfig ±0.3 dB Es1/Noc1 averaged over Measurement PRB ±0.3 dB Es2/Noc2 ±0.3 dB Noc, averaged over BWConfig, ±5.65 dB Es2/Noc2 averaged over Measurement PRB ±0.3 dB |  | Noc1 averaged over BWConfig,is the AWGN absolute power of RF channel 1 averaged over BWConfig Noc1 averaged over Measurement PRB,is the AWGN absolute power of RF channel 1 averaged over Measurement PRB Noc2 averaged over BWConfig,is the AWGN absolute power of RF channel 2 averaged over BWConfig, Noc2 averaged over Measurement PRB,is the AWGN absolute power of RF channel 2 averaged over Measurement PRB, Es1/Noc averaged over BWConfig is Ratio of cell 1 signal / AWGN averaged over BWConfig Es1/Noc averaged over Measurement PRB is Ratio of cell 1 signal / AWGN averaged over Measurement PRB Es2/Noc averaged over BWConfig is Ratio of cell 2 signal / AWGN averaged over BWConfig Es2/Noc averaged over Measurement PRB is Ratio of cell 2 signal / AWGN averaged over Measurement PRB |  |
| 7.5.5.1 NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX |  | Same as 5.5.5.1 |  | Same as 5.5.5.1 |  |
| 7.5.5.2 NR SA FR2 SSB-based beam failure detection and link recovery in DRX |  | Same as 5.5.5.1 |  | Same as 5.5.5.1 |  |
| 7.5.5.3 NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX |  | Noc ±5.65 dB, f<=40.8 GHz Ês1 / Noc ±0.3 dB Fading profile ±0.7 dB for 2Tx |  | Note: Noc is the AWGN on cell 1 frequency Ês1 / Noc is the SNR for the CSI-RS |  |
| 7.5.5.4 NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX |  | Same as 7.5.5.3 |  | Same as 7.5.5.3 |  |
| 7.5.5.5 NR SA FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX |  | Same as 5.5.5.1 |  | Same as 5.5.5.1 |  |
| 7.5.5.6 NR SA FR2 SCell CSI-RS-based beam failure detection and link recovery in non-DRX |  | Noc ±5.65 dB, f<=40.8 GHz Ês2 / Noc ±0.3 dB Fading profile ±0.7 dB for 2Tx |  | Note: Noc is the AWGN on cell 2 frequency Ês2 / Noc is the SNR for the CSI-RS |  |
| 7.5.5.7 NR SA FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX |  | Same as 7.5.5.6 |  | Same as 7.5.5.6 |  |
| 7.5.5.9 NR SA FR2 SCell TRP specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX |  | Same as 7.5.5.6 |  | Same as 7.5.5.6 |  |
| 7.5.5.10 NR SA FR2 PCell TRP specific SSB-based Beam Failure Detection and Link Recovery in non-DRX |  | Same as 5.5.5.1 |  | Same as 5.5.5.1 |  |
| 7.5.6.2.1 NR SA FR2 RRC-based DL active BWP switch in non-DRX |  | ±1.0 dB |  | Overall system uncertainty for fading conditions comprises two quantities: 1. gNB emulator Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness Items 1 and 2 are assumed to be uncorrelated so can be root sum squared: AWGN flatness and signal flatness has x 0.25 effect on the required SNR, so use sensitivity factor of x 0.25 for the uncertainty contribution. Test System uncertainty = SQRT (gNB emulator Signal-to-noise ratio uncertainty 2 + (0.25 x AWGN flatness and signal flatness) 2) gNB emulator Signal-to-noise ratio uncertainty ±0.3 dB AWGN flatness and signal flatness ±3.6 dB |  |
| 7.5.6.3.1 Rel16 test requirements for BWP switching on multiple CCs |  | Same as 7.5.6.2.1 |  | Same as 7.5.6.2.1 |  |
| 7.5.6.5.1 Rel16 test requirements for active BWP switch |  | Same as 7.5.6.2.1 |  | Same as 7.5.6.2.1 |  |
| 7.5.8.1.1 NR SA PCell FR2 MAC-CE based active TCI state switch for a known TCI state |  | Noc ±5.65 dB, f<=40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB No fading |  | Noc is the AWGN on cell 1 and 2 frequency Ês1 / Noc is Ratio of cell 1 signal / AWGN averaged over BW Ês2 / Noc is Ratio of cell 2 signal / AWGN averaged over BW |  |
| 7.5.8.2.1 NR SA PCell FR2 RRC based active TCI state switch for a known TCI state |  | Same as 7.5.8.1.1 |  | Same as 7.5.8.1.1 |  |
| 7.5.9.1.1 EN-DC PSCell FR2 uplink spatial relation switch for a known spatial relation |  | ±1.0 dB |  | The following parameters are proposed to be controlled: - AWGN flatness and signal flatness ±3.6 dB - Signal-to-noise ratio uncertainty ±0.3 dB Test System uncertainty on PSCell = SQRT (0.32 + (0.25 * 3.6)2) = 0.95 dB, rounded to 1.0 dB. |  |
| 7.5.9.2.1 EN-DC PSCell FR2 uplink spatial relation switch associated with a known DL-RS |  | Same as 7.5.9.1.1 |  | Same as 7.5.9.1.1 |  |
| 7.5.10.1 NR FR2 UE specific CBW change of PCell with non-DRX in SA |  | AWGN flatness and signal flatness ±3.6 dB Signal-to-noise ratio uncertainty ±0.3 dB |  | Overall system uncertainty for AWGN condition comprises two quantities: 1. Signal-to-noise ratio uncertainty 2. Effect of AWGN flatness and signal flatness |  |
| 7.5.11.1 UE UL carrier RRC reconfiguration delay |  | Noc ±5.65 dB, f<=40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB No fading |  | Note: Noc is the AWGN on cell 1 and 2 frequencies Ês1 / Noc is Ratio of cell 1 signal / AWGN averaged over BW Ês2 / Noc is Ratio of cell 2 signal / AWGN averaged over BW |  |
| 7.5.12.1 Addition and Release Delay of PSCell |  | Ês averaged over BW ±5.65 dB, f <= 40.8 GHz Ês averaged over measured PRB ±5.65 dB, f <= 40.8 GHz |  | Ês averaged over BW is the absolute power of cell 2 signal averaged over BW Ês averaged over measured PRB is the absolute power of cell 2 signal averaged over measured PRB |  |
| 7.5.13.6.1 NR SA FR2 MAC-CE based active uplink TCI state switch for 2 known TCI states |  | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB Average Ês2 / Noc ±0.3 dB Ês2 / Noc over meas PRBs ±0.3 dB Average Ês3 / Noc ±0.3 dB Ês3 / Noc over meas PRBs ±0.3 dB |  | Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Ês2 / Noc is the SNR for the SSB#2 Ês3 / Noc is the SNR for the SSB#3 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 for 120kHz SCSSSB |  |
| 7.5.14 NR SA FR2 PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2 |  | Noc averaged over BWConfig, ±5.65 dB Noc, averaged over Measurement PRB, ±5.65 dB Es/Noc averaged over BWConfig ±0.3 dB Es/Noc averaged over Measurement PRB ±0.3 dB |  | Noc averaged over BWConfig,is the AWGN absolute power of RF channel 2 averaged over BWConfig Noc averaged over Measurement PRB,is the AWGN absolute power of RF channel 2 averaged over Measurement PRB Es/Noc averaged over BWConfig is Ratio of cell 2 signal / AWGN averaged over BWConfig Es/Noc averaged over Measurement PRB is Ratio of cell 2 signal / AWGN averaged over Measurement PRB |  |
| 7.6.1.1 NR SA FR2 event-triggered reporting without gap in non-DRX |  | Ês1 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês1 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz Ês2 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês2 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz |  | Ês1 averaged over BW is the absolute power of cell 1 signal averaged over BW Ês1 averaged over measured PRB is the absolute power of cell 1 signal averaged over measured PRB Ês2 averaged over BW is the absolute power of cell 2 signal averaged over BW Ês2 averaged over measured PRB is the absolute power of cell 2 signal averaged over measured PRB |  |
| 7.6.1.2 NR SA FR2 event-triggered reporting without gap in DRX |  | Noc averaged over BW, ±5.65 dB, f <= 40.8 GHz Noc averaged over measured PRB, ±5.65 dB, f <= 40.8 GHz Ês1 / Noc averaged over BW, ±0.3 dB, f <= 40.8 GHz Ês1 / Noc averaged over measured PRB, ±0.3 dB, f <= 40.8 GHz Ês2 / Noc averaged over BW, ±0.3 dB, f <= 40.8 GHz Ês2 / Noc averaged over measured PRB, ±0.3 dB, f <= 40.8 GHz |  | Noc averaged over BW is AWGN absolute power averaged over BW. Noc averaged over measured PRB is AWGN absolute power averaged over measured PRB. Ês1 / Noc averaged over BW is the ratio of cell 1 signal / AWGN averaged over BW Ês1 / Noc averaged over measured PRB is the ratio of cell 1 signal / AWGN averaged over measured PRB Ês2 / Noc averaged over BW is the ratio of cell 2 signal / AWGN averaged over BW. Ês2 / Noc averaged over measured PRB is the ratio of cell 2 signal / AWGN averaged over measured PRB. |  |
| 7.6.1.3 NR SA FR2 event-triggered reporting without gap in non-DRX |  | Same as 7.6.1.1 |  | Same as 7.6.1.1 |  |
| 7.6.1.4 NR SA FR2 event-triggered reporting with gap in DRX |  | Same as 7.6.1.2 |  | Same as 7.6.1.2 |  |
| 7.6.2.1 NR SA FR2-FR2 event-triggered reporting in non-DRX |  | Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB Meas PRB Ês2 ±5.65 dB |  | Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.6.2.2 NR SA FR2-FR2 event-triggered reporting in DRX |  | Average Noc1 ±5.65 dB, f <= 40.8 GHz Noc1 over meas PRBs ±5.65 dB f <= 40.8 GHz Average Noc2 ±5.65 dB, f <= 40.8 GHz Noc2 over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês1 / Noc1 ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Ês2 / Noc2 over meas PRBs ±0.3 dB |  | Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.6.2.3 NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection |  | Same as 7.6.2.1 |  | Same as 7.6.2.1 |  |
| 7.6.2.4 NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection |  | Same as 7.6.2.2 |  | Same as 7.6.2.2 |  |
| 7.6.2.5 NR SA FR1-FR2 event-triggered reporting in non-DRX |  | Average Ês2 ±5.65 dB, f <= 40.8 GHz |  | Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.6.2.6 NR SA FR1-FR2 event-triggered reporting in DRX |  | Noc ±5.65 dB, f <= 40.8 GHz Ês2 / Noc ±0.3 dB |  | Ês2 / Noc is the ratio of cell 2 signal / AWGN |  |
| 7.6.2.7 NR SA FR1-FR2 event-triggered reporting in non-DRX with SSB time index detection |  | Same as 7.6.2.5 |  | Same as 7.6.2.5 |  |
| 7.6.2.8 NR SA FR1-FR2 event-triggered reporting in DRX with SSB time index detection |  | Same as 7.6.2.6 |  | Same as 7.6.2.6 |  |
| 7.6.2.9 SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17) |  | f <= 40.8 GHz Cell 1 Ês1 averaged over BWConfig uncertainty: ±5.65 dB Cell 1 Ês1 for PRBs #RBJ-RBJ+19 uncertainty: ±5.65 dB Cell 2 Ês2 averaged over BWConfig uncertainty: ±5.65 dB Cell 2 Ês2 for PRBs #RBJ-RBJ+19 uncertainty: ±5.65 dB |  | Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal |  |
| 7.6.2.10 |  | Same as 7.6.2.2 |  | Same as 7.6.2.2 |  |
| 7.6.2.11 |  | Same as 7.6.2.2 |  | Same as 7.6.2.2 |  |
| 7.6.19.1 NR SA FR2 event-triggered reporting without interruption outside gap |  | Same as 7.6.1.1 |  | Same as 7.6.1.1 |  |
| 7.6.19.2 NR SA FR2 event triggered reporting test without gap without interruption |  | Same as 7.6.1.1 |  | Same as 7.6.1.1 |  |
| 7.6.19.3 NR SA FR2-FR2 event triggered reporting test without gap without interruption under non-DRX |  | Same as 7.6.2.2 |  | Same as 7.6.2.2 |  |
| 7.6.2.20 NR SA FR2-FR2 event triggered reporting using MUSIM gap with higher priority |  | Same as 7.6.2.1 |  | Same as 7.6.2.1 |  |
| 7.6.2.21 NR SA FR2-FR2 event triggered reporting using MUSIM gap with shorter MGRP |  | Same as 7.6.2.1 |  | Same as 7.6.2.1 |  |
| 7.6.3.1 NR SA FR2 SSB-based L1-RSRP measurement in non-DRX |  | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB |  | Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.6.3.2 NR SA FR2 SSB-based L1-RSRP measurement in DRX |  | Same as 7.6.3.1 |  | Same as 7.6.3.1 |  |
| 7.6.3.3 NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX |  | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB |  | Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RS-RSRP |  |
| 7.6.3.4 NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX |  | Same as 7.6.3.3 |  | Same as 7.6.3.3 |  |
| 7.6.3.6 NR SA FR2 Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used |  | Same as 7.6.6.2 |  | Same as 7.6.6.2 |  |
| 7.6.4.1 NR SA FR2 SRS-RSRP measurement in non-DRX |  | Same as 5.6.4.1 |  | Same as 5.6.4.1 |  |
| 7.6.5.1 SA inter frequency CGI reporting in autonomous gaps test (PCell in FR2) |  | Average Noc1 ±5.65 dB, f <= 40.8 GHz Noc1 over meas PRBs ±5.65 dB f <= 40.8 GHz Average Noc2 ±5.65 dB, f <= 40.8 GHz Noc2 over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês1 / Noc1 ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB Average Ês2 / Noc2 ±0.3 dB Ês2 / Noc2 over meas PRBs ±0.3 dB |  | Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.6.6.1 NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy |  | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB |  | Ês0 / Noc is the SNR for the CSI-RS#0 Ês1 / Noc is the SNR for the CSI-RS#1 Meas PRB is the measurement PRB for CSI-RS-SINR #RBJ to RBJ+275 |  |
| 7.6.6.2 NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy |  | Average Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Average Ês0 / Noc ±0.3 dB Ês0 / Noc over meas PRBs ±0.3 dB Average Ês1 / Noc ±0.3 dB Ês1 / Noc over meas PRBs ±0.3 dB |  | Ês0 / Noc is the SNR for the SSB#0 Ês1 / Noc is the SNR for the SSB#1 Meas PRB is the measurement PRB for SS-SINR #RBJ to RBJ+19 |  |
| 7.6.6.3 NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy |  | Same as 7.6.6.1 |  | Same as 7.6.6.1 |  |
| 7.6.13.1 NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using PRS |  | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB |  | Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 1 signal / AWGN Meas PRB is the measurement PRB for PRS-RSRP #RBJ to RBJ+19 |  |
| 7.6.13.2 NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using TRS |  | Same as 7.6.13.1 |  | Same as 7.6.13.1 |  |
| 7.6.14.1 NR SA FR2 event triggered reporting tests with pre-configured measurement gaps and autonomous activation/deactivation |  | Ês1 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês1 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz Ês2 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês2 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz |  | Ês1 averaged over BW is the absolute power of cell 1 signal averaged over BW Ês1 averaged over measured PRB is the absolute power of cell 1 signal averaged over measured PRB Ês2 averaged over BW is the absolute power of cell 2 signal averaged over BW Ês2 averaged over measured PRB is the absolute power of cell 2 signal averaged over measured PRB |  |
| 7.6.14.2 NR SA FR2 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation |  | Same as 7.6.14.1 |  | Same as 7.6.14.1 |  |
| 7.6.15.1 NR SA FR2 SA event triggered reporting tests with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements |  | Ês1 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês1 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz Ês2 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês2 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz Ês3 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês3 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz |  | Ês1 averaged over BW is the absolute power of cell 1 signal averaged over BW Ês1 averaged over measured PRB is the absolute power of cell 1 signal averaged over measured PRB Ês2 averaged over BW is the absolute power of cell 2 signal averaged over BW Ês2 averaged over measured PRB is the absolute power of cell 2 signal averaged over measured PRB Ês3 averaged over BW is the absolute power of cell 2 signal averaged over BW Ês3 averaged over measured PRB is the absolute power of cell 2 signal averaged over measured PRB |  |
| 7.6.15.2 NR SA FR2 event triggered reporting tests with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2) |  | Same as 7.6.15.1 |  | Same as 7.6.15.1 |  |
| 7.6.15.3 NR SA FR2 event triggered reporting tests with concurrent measurement gaps with partially partial overlapping scenario for SSB-based measurements and PRS-based measurement |  | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Ês3 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB Meas PRB Ês3 / Noc ±0.3 dB |  | Noc is the AWGN on the NR freq 1 and 2 Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Ês3 / Noc is the ratio of cell 3 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.6.16.1 NR SA FR2 event triggered reporting tests with per-UE NCSG under non-DRX |  | Same as 7.6.1.1 |  | Same as 7.6.1.1 |  |
| 7.6.16.2 NR SA FR2 event triggered reporting tests on inter-frequency measurement with NCSG when DRX is not used(PCell in FR2) |  | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB |  | Noc is the AWGN on the NR freq 1 and 2 Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.6.16.3 NR SA FR2 event triggered reporting tests on deactivated SCell measurement via NCSG in non-DRX |  | Ês2 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês2 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz Ês3 averaged over BW ±5.65 dB, f <= 40.8 GHz Ês3 averaged over measured PRB ±5.65 dB, f <= 40.8 GHz |  | Ês2 averaged over BW is the absolute power of cell 2 signal averaged over BW Ês2 averaged over measured PRB is the absolute power of cell 2 signal averaged over measured PRB Ês3 averaged over BW is the absolute power of cell 3 signal averaged over BW Ês3 averaged over measured PRB is the absolute power of cell 3 signal averaged over measured PRB |  |
| 7.6.20.1 NR SA FR2 Intra-frequency SSB based L1-RSRP measurement |  | Average Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz |  | Ês1 is NR Cell 1 signal Ês2 is NR Cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.6.21.1 NR SA FR2 Inter-frequency SSB-based L1-RSRP measurement with measurement gap for LTM |  | Average Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz |  | Ês1 is NR Cell 1 signal Ês2 is NR Cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.7.1.1 NR SA FR2 SS-RSRP measurement accuracy |  | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz |  | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 1 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês1 is NR cell 1 signal Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.7.1.2 NR SA FR2-FR2 SS-RSRP measurement accuracy |  | Noc1 ±5.65 dB, f <= 40.8 GHz Noc1 over measPRBs ±5.65 dB, f <= 40.8 GHz Noc2 ±5.65 dB, f <= 40.8 GHz Noc2 over measPRBs ±5.65 dB, f <= 40.8 GHz Ês1 / Noc1 ±0.3 dB Ês2 / Noc2 ±0.3 dB Meas PRB Ês1 / Noc1 ±0.3 dB Meas PRB Ês2 / Noc2 ±0.3 dB |  | Noc1 is the AWGN on the NR freq 1 Noc2 is the AWGN on the NR freq 2 Ês1 / Noc1 is the ratio of cell 1 signal / AWGN Ês2 / Noc2 is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.7.1.3 NR SA FR1-FR2 SS-RSRP measurement accuracy |  | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês2 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB Test 2 Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz |  | Test 1 Noc is the AWGN on the NR freq Ês2 / Noc is the ratio of cell 1 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês2 is NR cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.7.2.1 NR SA FR2 SS-RSRQ measurement accuracy |  | Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB, f <= 40.8 GHz Ês1 / Noc ±0.3 dB Ês2 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Meas PRB Ês2 / Noc ±0.3 dB |  | Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 1 signal / AWGN Ês2 / Noc is the ratio of cell 2 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.7.2.2 NR SA FR2-FR2 SS-RSRQ measurement accuracy |  | Same as 7.7.1.2 |  | Same as 7.7.1.2 |  |
| 7.7.3.1 NR SA FR2 SS-SINR measurement accuracy |  | Same as 7.7.2.1 |  | Same as 7.7.2.1 |  |
| 7.7.3.3 NR SA FR2-FR2 SS-SINR measurement accuracy |  | Same as 7.7.1.2 |  | Same as 7.7.1.2 |  |
| 7.7.4.1 NR SA FR2 SSB based L1-RSRP measurement accuracy |  | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz |  | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 1 signal / AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 Ês1 is NR cell 1 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |
| 7.7.4.2 NR SA FR2 CSI-RS based L1-RSRP measurement accuracy |  | Test 1 Noc ±5.65 dB, f <= 40.8 GHz Noc over meas PRBs ±5.65 dB f <= 40.8 GHz Ês1 / Noc ±0.3 dB Meas PRB Ês1 / Noc ±0.3 dB Test 2 Average Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz |  | Test 1 Noc is the AWGN on the NR freq Ês1 / Noc is the ratio of cell 1 signal / AWGN Meas PRB is the measurement PRB for CSI-RS-RSRP Test 2 Ês1 is NR cell 1 signal Meas PRB is the measurement PRB for CSI-RS-RSRP |  |
| 7.7.5.1 NR SA FR2 SRS-RSRP measurement accuracy |  | Same as 5.7.5.1 |  | Same as 5.7.5.1 |  |
| 7.7.6.1 NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy |  | Same as 7.6.6.1 |  | Same as 7.6.6.1 |  |
| 7.7.6.2 NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy |  | Same as 7.6.6.2 |  | Same as 7.6.6.2 |  |
| 7.7.6.3 NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy |  | Same as 7.6.6.1 |  | Same as 7.6.6.1 |  |
| 7.7.8.1 NR SA intra-frequency FR2 CSI-RSRQ measurement accuracy |  | Same as 5.7.8.1 |  | Same as 5.7.8.1 |  |
| 7.7.8.2 NR SA inter-frequency FR2 CSI-RSRQ measurement accuracy |  | Same as 5.7.8.2 |  | Same as 5.7.8.2 |  |
| 7.7.9.1 NR SA intra-frequency FR2 CSI-SINR measurement accuracy |  | Same as 5.7.9.1 |  | Same as 5.7.9.1 |  |
| 7.7.9.2 NR SA inter-frequency FR2 CSI-SINR measurement accuracy |  | Same as 5.7.9.2 |  | Same as 5.7.9.2 |  |
| 7.7.15.1 NR SA FR2 SSB based inter-frequency L1-RSRP measurement |  | Test 1 : Average Noc1 ±5.65 dB, f <= 40.8 GHz Average Noc2 ±5.65 dB, f <= 40.8 GHz Average Ês1/Noc1 ±0.3 dB, f<=40.8 GHz Average Ês2/Noc2 ±0.3 dB, f<=40.8 GHz Meas PRB Noc1 ±5.65 dB, f <= 40.8 GHz Meas PRB Noc2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1/Noc1 ±0.3 dB, f<=40.8 GHz Meas PRB Ês2/Noc2 ±0.3 dB, f<=40.8 GHz Test 2 : Average Ês1 ±5.65 dB, f <= 40.8 GHz Average Ês2 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês1 ±5.65 dB, f <= 40.8 GHz Meas PRB Ês2 ±5.65 dB, f <= 40.8 GHz |  | Test 1 : Noc1 is the AWGN on the NR freq 1 Noc2 is the AWGN on the NR freq 2 Ês1/Noc1 is the ratio of Cell 1 signal/AWGN Ês2/Noc2 is the ratio of Cell 2 signal/AWGN Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 Test 2 : Ês1 is NR Cell 1 signal Ês2 is NR Cell 2 signal Meas PRB is the measurement PRB for SS-RSRP #RBJ to RBJ+19 |  |

## <<< END OF CHANGES 1 >>>

## <<< START OF CHANGES 2 >>>

Table F.1.3.2-4: Derivation of test requirements for NR SA FR2 RRM tests

| Test | Minimum requirement in TS 38.133 [6] |  | Test tolerance (TT) | Test requirement in TS 38.533 |
| --- | --- | --- | --- | --- |
| 7.1.1.1 NR SA FR2 cell re-selection | UE PC1 |  |  |  |
|  | During T1: Noc = -102 dBm/15kHz Es1/Noc = 8dB Es2/Noc = -infinity During T2: Noc = -102 dBm/15kHz Es1/Noc = -3dB Es2/Noc = 1.5dB During T3: Noc = -102 dBm/15kHz Es1/Noc = 1.5dB Es2/Noc = -3dB In signalling: SintraSearchP = 50 Qrxlevmin = -138dBm/SCS for config. 1 Qrxlevmin = -135dBm/SCS for config. 2 |  | During T1: 0dB 0dB N/A During T2: 0dB 0.5dB 1.0dB During T3: 0dB 1.0dB 0.5dB In signalling: 12 37 dB 37 dB | During T1: Noc = -102 dBm/15kHz Es1/Noc = 8dB Es2/Noc = -infinity During T2: Noc = -102 dBm/15kHz Es1/Noc = -2.50dB Es2/Noc = 2.5dB During T3: Noc = -102 dBm/15kHz Es1/Noc = 2.5dB Es2/Noc = -2.50dB In signalling: SintraSearchP = 62 Qrxlevmin = -101dBm/SCS for config. 1 Qrxlevmin = -98dBm/SCS for config. 2 |
|  | UE PC3 |  |  |  |
|  | During T1: Noc = -102 dBm/15kHz Es1/Noc = 8dB Es2/Noc = -infinity During T2: Noc = -102 dBm/15kHz Es1/Noc = -3dB Es2/Noc = 1.5dB During T3: Noc = -102 dBm/15kHz Es1/Noc = 1.5dB Es2/Noc = -3dB In signalling: Qrxlevmin = -138dBm/SCS for config. 1 Qrxlevmin = -135dBm/SCS for config. 2 |  | During T1: 0dB 0dB N/A During T2: 0dB 0.1dB 0.55dB During T3: 0dB 0.55dB 0.1dB In signalling: 18 dB 18 dB | During T1: Noc = -102 dBm/15kHz Es1/Noc = 8dB Es2/Noc = -infinity During T2: Noc = -102 dBm/15kHz Es1/Noc = -2.90dB Es2/Noc = 2.05dB During T3: Noc = -102 dBm/15kHz Es1/Noc = 2.05dB Es2/Noc = -2.90dB In signalling: Qrxlevmin = -120dBm/SCS for config. 1 Qrxlevmin = -117dBm/SCS for config. 2 |
| 7.1.1.2 NR SA FR2-FR2 cell re-selection | During T1: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 10.5dB Es2/Noc2 = -10.5dB During T2: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 10.5dB Es2/Noc2 = -infinity During T3: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 8dB Es2/Noc2 = 8.5dB In signalling: Cell 2 Qrxlevmin = -138dBm/SCS for config. 1 Cell 2 Qrxlevmin = -135dBm/SCS for config. 2 Cell 2 SnonintraSearchP = 50dB Cell 1 Threshx, highP = 48dB Cell 2 Threshserving, lowP = 44dB Cell 2 Threshx, lowP = 50dB |  | During T1: 0dB 0dB 0dB 7.6dB During T2: 0dB 0dB 0dB N/A During T3: 0dB 0dB 0dB 0dB In signalling: 16 dB 16 dB 12dB -16dB 18dB -32dB | During T1: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 10.5dB Es2/Noc2 = -2.9dB During T2: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 10.5dB Es2/Noc2 = -infinity During T3: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 8dB Es2/Noc2 = 8.5dB In signalling: Qrxlevmin = -124dBm/SCS for config. 1 Qrxlevmin = -121dBm/SCS for config. 2 Cell 2 SnonintraSearchP = 62dB Cell 1 Threshx, highP = 32dB Cell 2 Threshserving, lowP = 62dB Cell 2 Threshx, lowP = 18dB |
| 7.1.1.3 NR SA FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | During T1: Noc = -102 dBm/15kHz Es1/Noc = -3dB Es2/Noc = 1.5dB During T2: Noc = -102 dBm/15kHz Es1/Noc = 1.5dB Es2/Noc = -3dB In signalling: Qrxlevmin = -140dBm/SCS for config. 1 Qrxlevmin = -137dBm/SCS for config. 2 |  | During T1: 0dB 0.1dB 0.55dB During T2: 0dB 0.55dB 0.1dB In signalling: 20dB 20dB | During T1: Noc = -102 dBm/15kHz Es1/Noc = -2.90dB Es2/Noc = 2.05dB During T2: Noc = -102 dBm/15kHz Es1/Noc = 2.05dB Es2/Noc = -2.90dB In signalling: Qrxlevmin = -120dBm/SCS for config. 1 Qrxlevmin = -117dBm/SCS for config. 2 |
| 7.1.1.4 NR SA FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | During T1: Noc = -102 dBm/15kHz Es1/Noc = -3dB Es2/Noc = 1.5dB During T2: Noc = -102 dBm/15kHz Es1/Noc = 1.5dB Es2/Noc = -3dB In signalling: Qrxlevmin = -140dBm/SCS for config. 1 Qrxlevmin = -137dBm/SCS for config. 2 SsearchThresholdP = 35dB |  | During T1: 0dB 0.1dB 0.55dB During T2: 0dB 0.55dB 0.1dB In signalling: 20dB 20dB -27dB | During T1: Noc = -102 dBm/15kHz Es1/Noc = -2.90dB Es2/Noc = 2.05dB During T2: Noc = -102 dBm/15kHz Es1/Noc = 2.05dB Es2/Noc = -2.90dB In signalling: Qrxlevmin = -120dBm/SCS for config. 1 Qrxlevmin = -117dBm/SCS for config. 2 SsearchThresholdP = 8dB |
| 7.1.1.5 NR SA FR2-FR2 cell re-selection for UE fulfilling low mobility relaxed measurement criterion | During T1: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 10.5dB Es2/Noc2 = -10.5dB During T2: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 8dB Es2/Noc2 = 8.5dB In signalling: Cell 2 Qrxlevmin = -138dBm/SCS for config. 1 Cell 2 Qrxlevmin = -135dBm/SCS for config. 2 Cell 2 SnonintraSearchP = 50dB Cell 1 Threshx, highP = 48dB Cell 2 Threshserving, lowP = 44dB Cell 2 Threshx, lowP = 50dB SSearchDeltaP = 6dB |  | During T1: 0dB 0dB 0dB 7.6dB During T2: 0dB 0dB 0dB 0dB In signalling: 16 dB 16 dB 12dB -16dB 18dB -32dB 6dB | During T1: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 10.5dB Es2/Noc2 = -2.9dB During T2: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 8dB Es2/Noc2 = 8.5dB In signalling: Qrxlevmin = -124dBm/SCS for config. 1 Qrxlevmin = -121dBm/SCS for config. 2 Cell 2 SnonintraSearchP = 62dB Cell 1 Threshx, highP = 32dB Cell 2 Threshserving, lowP = 62dB Cell 2 Threshx, lowP = 18dB SSearchDeltaP = 12dB |
| 7.1.1.6 NR SA FR2-FR2 cell re-selection for UE fulfilling not-at-cell edge relaxed measurement criterion | During T1: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 10.5dB Es2/Noc2 = -10.5dB During T2: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 8dB Es2/Noc2 = 8.5dB In signalling: Cell 2 Qrxlevmin = -138dBm/SCS for config. 1 Cell 2 Qrxlevmin = -135dBm/SCS for config. 2 Cell 2 SnonintraSearchP = 50dB Cell 1 Threshx, highP = 48dB Cell 2 Threshserving, lowP = 44dB Cell 2 Threshx, lowP = 50dB Cell 2 SsearchThresholdP = 29dB |  | During T1: 0dB 0dB 0dB 7.6dB During T2: 0dB 0dB 0dB 0dB In signalling: 16 dB 16 dB 12dB -16dB 18dB -32dB -17dB | During T1: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 10.5dB Es2/Noc2 = -2.9dB During T2: Noc1 = -102 dBm/15kHz Noc2 = -102 dBm/15kHz Es1/Noc1 = 8dB Es2/Noc2 = 8.5dB In signalling: Qrxlevmin = -124dBm/SCS for config. 1 Qrxlevmin = -121dBm/SCS for config. 2 Cell 2 SnonintraSearchP = 62dB Cell 1 Threshx, highP = 32dB Cell 2 Threshserving, lowP = 62dB Cell 2 Threshx, lowP = 18dB Cell 2 SsearchThresholdP = 12dB |
| 7.1.1.7 NR SA FR2 cell re-selection for power class 6 UE configured with highSpeedMeasFlagFR2-r17 | During T1: Noc = -102 dBm/15kHz Es1/Noc = 8dB Es2/Noc = -infinity During T2: Noc = -102 dBm/15kHz Es1/Noc = -3dB Es2/Noc = 1.5dB During T3: Noc = -102 dBm/15kHz Es1/Noc = 1.5dB Es2/Noc = -3dB In signalling: Qrxlevmin = -138dBm/SCS for config. 1 Qrxlevmin = -135dBm/SCS for config. 2 |  | During T1: 0dB 0dB N/A During T2: 0dB 0.5dB 1dB During T3: 0dB 1dB 0.5dB In signalling: 43 dB 43 dB | During T1: Noc = -102 dBm/15kHz Es1/Noc = 8dB Es2/Noc = -infinity During T2: Noc = -102 dBm/15kHz Es1/Noc = -2.5dB Es2/Noc = 2.5dB During T3: Noc = -102 dBm/15kHz Es1/Noc = 2.5dB Es2/Noc = -2.5dB In signalling: Qrxlevmin = -95dBm/SCS for config. 1 Qrxlevmin = -92dBm/SCS for config. 2 |
| 7.3.1.2 NR SA FR2 Intra-frequency handover; unknown target cell | During T1: Noc: -104.7dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: -infinity During T2: Noc: -104.7dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +7.00dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB | During T1: Noc: -104.7dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: -infinity During T2: Noc: -104.7dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +7.00dB |
| 7.3.1.3 NR SA FR2-FR2 Inter-frequency handover; unknown target cell | During T1: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +5.00dB Ês2 / Noc2: -infinity During T2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +5.00dB Ês2 / Noc2: +5.00dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB | During T1: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +5.00dB Ês2 / Noc2: -infinity During T2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +5.00dB Ês2 / Noc2: +5.00dB |
| 7.3.1.4 NR SA FR1-FR2 synchronous DAPS handover | During T1: Noc -104.7dBm/15kHz Es2/Noc -infinity During T2-T5: Noc -104.7 dBm/15kHz Es2/Noc 10 dB |  | During T1: 0 dB 0 dB During T2-T5: 0 dB 0 dB | During T1: Noc -104.7dBm/15kHz Es2/Noc -infinity During T2-T5: Noc -104.7 dBm/15kHz Es2/Noc 10 dB |
| 7.3.1.5 NR SA FR1-FR2 asynchronous DAPS handover | Same as 7.3.1.4 |  | Same as 7.3.1.4 | Same as 7.3.1.4 |
| 7.3.2.1.1 NR SA FR2 RRC re-establishment | During T1: Noc = -104.7 dBm/15 kHz Ês1 / Noc = 4 dB Ês2 / Noc = 2 dB During T2: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 2 dB During T3: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 2 dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB During T3: 0dB 0dB 0dB | During T1: Noc = -104.7 dBm/15 kHz Ês1 / Noc = 4 dB Ês2 / Noc = 2 dB During T2: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 2 dB During T3: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 2 dB |
| 7.3.2.1.2 NR SA FR2 - FR2 RRC re-establishment | During T1: Noc1 = -92.1 dBm/15 kHz Noc2 = -92.1 dBm/15 kHz Ês1 / Noc1 = 0 dB Ês2 / Noc 2= - Infinity During T2: Noc1 = -92.1 dBm/15 kHz Noc2 = -92.1 dBm/15 kHz Ês1 / Noc1 = - Infinity Ês2 / Noc2 = - Infinity During T3: Noc1 = -92.1 dBm/15 kHz Noc2 = -92.1 dBm/15 kHz Ês1 / Noc1 = - Infinity Ês2 / Noc2 = 0 dB |  | During T1: -0.2dB -0.2dB 0dB 0dB During T2: -0.2dB -0.2dB 0dB 0dB During T3: -0.2dB -0.2dB 0dB 0dB | During T1: Noc1 = -92.3 dBm/15 kHz Noc2 = -92.3 dBm/15 kHz Ês1 / Noc1 = 0 dB Ês2 / Noc 2= - Infinity During T2: Noc1 = -92.3 dBm/15 kHz Noc2 = -92.3 dBm/15 kHz Ês1 / Noc1 = - Infinity Ês2 / Noc2 = - Infinity During T3: Noc1 = -92.3 dBm/15 kHz Noc2 = -92.3 dBm/15 kHz Ês1 / Noc1 = - Infinity Ês2 / Noc2 = 0 dB |
| 7.3.2.1.3 NR SA FR2 RRC re-establishment without serving cell timing | During T1: Noc = -104.7 dBm/15 kHz Ês1 / Noc = 5 dB Ês2 / Noc = - infinity During T2: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = - infinity During T3: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 5 dB |  | During T1: 0dB 0dB N/A During T2: 0dB N/A N/A During T3: 0dB N/A 0dB | During T1: Noc = -104.7 dBm/15 kHz Ês1 / Noc = 5 dB Ês2 / Noc = - infinity During T2: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = - infinity During T3: Noc = -104.7 dBm/15 kHz Ês1 / Noc = - Infinity Ês2 / Noc = 5 dB |
| 7.3.2.2.1 NR SA FR2 contention based random access | Absolute uplink power: Pint ≥ P ≥ Pmin ±14.0dB PUMAX ≥ P > Pint ±12.0dB Relative uplink power step: Pint ≥ P ≥ Pmin ±6.0dB PUMAX ≥ P > Pint ±4.0dB Uplink timing: 120kHz SCS Te ±3.5*64*Tc |  | Not applicable due to UL calibration FFS dB FFS dB [48]Tc | Absolute uplink power: ±FFSdB Relative uplink power step: ±(6+FFS)dB, either PRACH being compared ≤ (Pmax – 6dB) ±(4+FFS)dB, both PRACHs being compared > (Pmax – 6dB) Uplink timing: 120kHz SCS Te ±224±[48]*Tc |
| 7.3.2.2.2 NR SA FR2 non-contention based random access | Same as 7.3.2.2.1 |  | Same as 7.3.2.2.1 | Same as 7.3.2.2.1 |
| 7.3.2.3.1 NR SA FR2-FR2 RRC connection release with redirection | Same as 7.3.1.3 |  | Same as 7.3.1.3 | Same as 7.3.1.3 |
| 7.3.3.1 NR SA FR2 conditional handover | During T1: Noc -104.7dBm/15kHz Es1/Noc 6 dB Es2/Noc -infinity During T2: Noc -104.7dBm/15kHz Es1/Noc 6 dB Es2/Noc 11dB In signalling: A3-offset = -1dB |  | During T1 -1.5dB 0dB 0dB During T2 -1.5dB 0dB 0dB In signalling: -1dB | During T1: Noc -106.2 dBm/15kHz Es1/Noc 6 dB Es2/Noc -infinity During T2: Noc -106.2 dBm/15kHz Es1/Noc 6 dB Es2/Noc 11dB In signalling: A3-offset = -2dB |
| 7.3.3.3 | Same as 7.3.3.1 |  | Same as 7.3.3.1 | Same as 7.3.3.1 |
| 7.3.3.5 NR SA FR2 NR conditional handover including target MCG and target SCG from FR1-FR2 NR-DC to FR1-FR2 NR-DC | Test Config 1, 2: During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Noc3: -104.7dBm/15KHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity Ês3 / Noc3: +6dB Ês4 / Noc3: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Noc3: -104.7dBm/15KHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB Ês3 / Noc3: +6dB Ês4 / Noc2: +7dB Test Config 3: During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Noc3: -104.7dBm/15KHz Ês1 / Noc1: +4dB Ês2 / Noc2: infinity Ês3 / Noc3: +6dB Ês4 / Noc3: infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Noc3: -104.7dBm/15KHz Ês1 / Noc1: +4dB Ês2 / Noc2: +5dB Ês3 / Noc3: +6dB Ês4 / Noc2: +7dB In signalling: A3-offset: -4dB |  | Test Config 1, 2: During T1: 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 1.7dB 0dB 0dB Test Config 3: During T1: 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 1.7dB 0dB 0dB In signalling: 0dB | Test Config 1,2: During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Noc3: -104.7dBm/15KHz Ês1 / Noc1: +4dB Ês2 / Noc2: -infinity Ês3 / Noc3: +6dB Ês4 / Noc3: -infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Noc3: -104.7dBm/15KHz Ês1 / Noc1: +4dB Ês2 / Noc2: +6.7dB Ês3 / Noc3: +6dB Ês4 / Noc3: +7dB Test Config 3: During T1: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Noc3: -104.7dBm/15KHz Ês1 / Noc1: +4dB Ês2 / Noc2: infinity Ês3 / Noc3: +6dB Ês4 / Noc3: infinity During T2: Noc1: -98dBm/15kHz Noc2: -98dBm/15kHz Noc3: -104.7dBm/15KHz Ês1 / Noc1: +4dB Ês2 / Noc2: +6.7dB Ês3 / Noc3: +6dB Ês4 / Noc3: +7dB In signalling: A3-offset: -4dB |
| 7.3.4.1 NR SA FR2 RACH based Intra-frequency PCell switch from FR2 to FR2 | During T1~T4: Noc: -104.7dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +7.00dB |  | During T1~T4: 0dB 0dB 0dB | During T1~T4: Noc: -104.7dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +7.00dB |
| 7.3.4.2 NR SA FR2 RACH-less Intra-frequency PCell switch from FR2 to FR2 | During T1~T5: Noc: -104.7dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +7.00dB |  | During T1~T5: 0dB 0dB 0dB | During T1~T5: Noc: -104.7dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +7.00dB |
| 7.3.4.3 NR SA FR2 RACH-based Inter-frequency LTM PCell switch from FR2 to FR2 | During T1~T4: Noc: -104.7dBm/15kHz Ês1 / Noc: +5.00dB Ês2 / Noc: +5.00dB |  | During T1~T4: 0dB 0dB 0dB | During T1~T4: Noc: -104.7dBm/15kHz Ês1 / Noc: +5.00dB Ês2 / Noc: +5.00dB |
| 7.4.1.1 SA FR2 UE transmit timing accuracy | Test 1 (no DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Max step size Tq: 2.5*64*Tc Min adjust rate Tp: 2.5*64*Tc Max adjust rate: 2.5*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB |  | ±0.75*64*Tc +0.625*64Tc -3.725*64*Tc +1.225*64*Tc 0dB 0dB ±0.75*64*Tc 0dB 0dB | Test 1 (no DRX): Uplink timing: ±3.75*64*Tc Max step size Tq: 3.125*64*Tc Min adjust rate: -1.225*64*Tc Max adjust rate: 3.725*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 2 (with DRX): Uplink timing: ±3.75*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB |
| 7.4.1.3 NR SA FR2 UE Transmit Timing Test with 2-TA for FR2 UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18 | Test 1 (no DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Max step size Tq: 2.5*64*Tc Min adjust rate Tp: 2.5*64*Tc Max adjust rate: 2.5*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 2 (no DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Max step size Tq: 2.5*64*Tc Min adjust rate Tp: 2.5*64*Tc Max adjust rate: 2.5*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 3 (with DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 4 (with DRX): Uplink timing: ±3*64*Tc for 240 kHz SSB SCS Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB |  | ±0.75*64*Tc +0.625*64Tc -3.725*64*Tc +1.225*64*Tc 0dB 0dB ±0.75*64*Tc +0.625*64Tc -3.725*64*Tc +1.225*64*Tc 0dB 0dB ±0.75*64*Tc 0dB 0dB ±0.75*64*Tc 0dB 0dB | Test 1 (no DRX): Uplink timing: ±3.75*64*Tc Max step size Tq: 3.125*64*Tc Min adjust rate: -1.225*64*Tc Max adjust rate: 3.725*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 2 (no DRX): Uplink timing: ±3.75*64*Tc Max step size Tq: 3.125*64*Tc Min adjust rate: -1.225*64*Tc Max adjust rate: 3.725*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 3 (with DRX): Uplink timing: ±3.75*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB Test 4 (with DRX): Uplink timing: ±3.75*64*Tc Noc = -112 dBm/15 kHz Ês1 / Noc: +4.0dB |
| 7.4.3.1 NR SA FR2 UE timing advance adjustment accuracy | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE Timing Advance Adjustment Accuracy for 120kHz SCS = ±32 Tc |  | 0dB 0dB +/- 40 Tc | Noc = -112 dBm/15 kHz Ês1 / Noc = 4 dB UE TAAA for 120kHz SCS = ±72 Tc |
| 7.5.1.1 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM-RS in non-DRX mode | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -92.1dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB |  | During T1: -2.7 dB 2.1 dB 2.1 dB During T2: -2.7 dB 2.1 dB 0 dB During T3: -2.7 dB 0 dB 0 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB During T2: Noc: -94.8 dBm/15kHz Ês1 / Noc: -3.9 dB Ês2 / Noc: -15 dB During T3: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB |
| 7.5.1.2 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM-RS in non-DRX mode | During T1: Noc: -92.1 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -92.1dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -4.5 dB During T5: Noc: -92.1dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: +2 dB |  | During T1: -2.7 dB 2.1 dB 2.1 dB During T2: -2.7 dB 2.1 dB 0 dB During T3: -2.7 dB 0 dB 0 dB During T4: -2.7 dB 0 dB 0 dB During T5: -2.7 dB 0 dB 2.1 dB | During T1: Noc: -94.8 dBm/15kHz Ês1 / Noc: +4.1 dB Ês2 / Noc: +4.1 dB During T2: Noc: -94.8 dBm/15kHz Ês1 / Noc: -3.9 dB Ês2 / Noc: -15 dB During T3: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -4.5 dB During T5: Noc: -94.8 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: +4.1 dB |
| 7.5.1.3 Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM-RS in DRX mode | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB |  | During T1: 0 dB 1.3 dB 1.3 dB During T2: 0 dB 1.3 dB -0.4 dB During T3: 0 dB -0.4 dB -0.4 dB | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.3 dB Ês2 / Noc: +3.3 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.7 dB Ês2 / Noc: -15.4 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15.4 dB Ês2 / Noc: -15.4 dB |
| 7.5.1.4 Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM-RS in DRX mode | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: +2 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -6 dB Ês2 / Noc: -15 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15 dB Ês2 / Noc: -15 dB During T4: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.5 dB Ês2 / Noc: -15 dB During T5: Noc: -104.7 dBm/15kHz Ês1 / Noc: +2 dB Ês2 / Noc: -14 dB |  | During T1: 0 dB 1.3 dB 1.3 dB During T2: 0 dB 1.3 dB -0.4 dB During T3: 0 dB -0.4 dB -0.4 dB During T4: 0 dB -0.4 dB -0.4 dB During T5: 0 dB 1.3 dB -0.4 dB | During T1: Noc: -104.7 dBm/15kHz Ês1 / Noc: +3.3 dB Ês2 / Noc: +3.3 dB During T2: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.7 dB Ês2 / Noc: -15.4 dB During T3: Noc: -104.7 dBm/15kHz Ês1 / Noc: -15.4 dB Ês2 / Noc: -15.4 dB During T4: Noc: -104.7 dBm/15kHz Ês1 / Noc: -4.9 dB Ês2 / Noc: -15.4 dB During T5: Noc: -104.7 dBm/15kHz Ês1 / Noc: 3.3 dB Ês2 / Noc: -15.4 dB |
| 7.5.1.9 NR SA FR2 radio link monitoring UE scheduling restrictions | Same as 5.5.1.9 |  | Same as 5.5.1.9 | Same as 5.5.1.9 |
| 7.5.2.2 SA interruptions at NR SRS carrier-based switching | During T1: Noc: -112 dBm/15kHz Ês / Noc: +4 dB During T2: Noc: -112 dBm/15kHz Ês / Noc: +4 dB |  | During T1: 0 dB 0 dB During T2: 0 dB 0 dB | During T1: Noc: -112 dBm/15kHz Ês / Noc: +4 dB During T2: Noc: -112 dBm/15kHz Ês / Noc: +4 dB |
| 7.5.3.4 NR SA FR2 direct SCell activation at SCell addition of known SCell |  | During T1: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc2: +14 dB During T2: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc2: +14 dB During T3: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc2: +14 dB In signalling: A3-offset = -15dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB In signalling: 0dB | During T1: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc2: +14 dB During T2: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc2: +14 dB During T3: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc2: +14 dB In signalling: A3-offset = -15dB |
| 7.5.3.5 NR SA FR2 direct SCell activation at SCell addition of known SCell |  | During T1: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc1: +14 dB Ês3 / Noc2: +14 dB During T2: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc1: +14 dB Ês3 / Noc2: +14 dB During T3: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc1: +14 dB Ês3 / Noc2: +14 dB In signalling: A3-offset = -15dB | During T1: 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB 0dB In signalling: 0dB | During T1: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc1: +14 dB Ês3 / Noc2: +14 dB During T2: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc1: +14 dB Ês3 / Noc2: +14 dB During T3: Noc1: -112 dBm/15kHz Noc2: -112 dBm/15kHz Ês1 / Noc1: +14 dB Ês2 / Noc1: +14 dB Ês3 / Noc2: +14 dB In signalling: A3-offset = -15dB |
| 7.5.3.13 NR SA FR2 SCell Activation for SCell in FR2 intra-band in non-DRX |  | During T1: Noc1: -104.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc1: +7 dB Ês2 / Noc2: +7 dB During T2: Noc1: -104.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc1: +7 dB Ês2 / Noc2: +7 dB In signalling: A3-offset = -15dB | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB In signalling: 0dB | During T1: Noc1: -104.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc1: +7 dB Ês2 / Noc2: +7 dB During T2: Noc1: -104.7 dBm/15kHz Noc2: -104.7 dBm/15kHz Ês1 / Noc1: +7 dB Ês2 / Noc2: +7 dB In signalling: A3-offset = -15dB |
| 7.5.3.14 NR SA FR2 SCell Activation for known SCell in FR2 inter-band | During T1: Noc1: -92.1 dBm/15kHz Noc2: -92.1 dBm/15kHz Ês1 / Noc1: 0 dB Ês2 / Noc2: 0 B During T2: Noc1: -92.1 dBm/15kHz Noc2: -92.1 dBm/15kHz Ês1 / Noc1: 0 dB Ês2 / Noc2: 0 B In signalling: A3-offset = -15dB |  | During T1: -4.6dB -4.6dB 0dB 0dB During T2: -4.6dB -4.6dB 0dB 0dB In signalling: 0dB | During T1: Noc1: -96.7 dBm/15kHz Noc2: -96.7 dBm/15kHz Ês1 / Noc1: 0 dB Ês2 / Noc2: 0 B During T2: Noc1: -96.7 dBm/15kHz Noc2: -96.7 dBm/15kHz Ês1 / Noc1: 0 dB Ês2 / Noc2: 0 B In signalling: A3-offset = -15dB |
| 7.5.5.1 NR SA FR2 SSB-based beam failure detection and link recovery in non-DRX | During T1: SNR_SSB q0: 5 dB SNR_SSB q1: 0.2 dB During T2: SNR_SSB q0: -3 dB SNR_SSB q1: 0.2 dB During T3-T5: SNR_SSB q0: -12 dB SNR_SSB q1: 20.2 dB In signalling: rsrp-ThresholdSSB: -95 dBm/120kHz for Config 1 rsrp-ThresholdSSB: -92 dBm/240kHz for Config 2 |  | +8.70 dB 0 dB +8.70 dB 0 dB 0 dB -0.2 dB -14 dB -14 dB | During T1: SNR_SSB q0: 13.7 dB SNR_SSB q1: 0.2 dB During T2: SNR_SSB q0: 5.7 dB SNR_SSB q1: 0.2 dB During T3-T5: SNR_SSB q0: -12 dB SNR_SSB q1: 20 dB In signalling: rsrp-ThresholdSSB: -109 dBm/120kHz for Config 1 srp-ThresholdSSB: -106 dBm/240kHz for Config 2 |
| 7.5.5.2 NR SA FR2 SSB-based beam failure detection and link recovery in DRX | Same as 7.5.5.1 |  | Same as 7.5.5.1 | Same as 7.5.5.1 |
| 7.5.5.3 NR SA FR2 CSI-RS-based beam failure detection and link recovery in non-DRX | Noc = -104.7 dBm/120 kHz Ês1 / Noc for CSI-RS set q0 T1: 5 dB T2: -3 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês1 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20.2 dB T4: 20.2 dB T5: 20.2 dB rsrp-ThresholdSSB: -95 dBm/SCS kHz |  | 0 dB 8.7 dB 8.7 dB 0 dB 0 dB 0 dB 0 dB 0 dB -0.2 dB -0.2 dB -0.2 dB -14 dB | Noc = -104.7 dBm/120 kHz Ês1 / Noc for CSI-RS set q0 T1: 13.7 dB T2: 5.7 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês1 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20 dB T4: 20 dB T5: 20 dB rsrp-ThresholdSSB: -109.0 dBm/SCS kHz |
| 7.5.5.4 NR SA FR2 CSI-RS-based beam failure detection and link recovery in DRX | Same as 7.5.5.3 |  | Same as 7.5.5.3 | Same as 7.5.5.3 |
| 7.5.5.5 NR SA FR2 scheduling available restriction during SSB-based beam failure detection and link recovery in non-DRX | Same as 7.5.5.1 |  | Same as 7.5.5.1 | Same as 7.5.5.1 |
| 7.5.5.6 NR SA FR2 SCell CSI-RS-based beam failure detection and link recovery in non-DRX | Noc = -104.7 dBm/120 kHz Ês2 / Noc for CSI-RS set q0 T1: 5 dB T2: -3 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês2 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20.2 dB T4: 20.2 dB T5: 20.2 dB rsrp-ThresholdBFR: -95 dBm/SCS kHz |  | 0 dB 8.7 dB 8.7 dB 0 dB 0 dB 0 dB 0 dB 0 dB -0.2 dB -0.2 dB -0.2 dB -14 dB | Noc = -104.7 dBm/120 kHz Ês2 / Noc for CSI-RS set q0 T1: 13.7 dB T2: 5.7 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês2 / Noc for CSI-RS set q1 T1: 0.2 dB T2: 0.2 dB T3: 20 dB T4: 20 dB T5: 20 dB rsrp-ThresholdBFR: -109.0 dBm/SCS kHz |
| 7.5.5.7 NR SA FR2 SCell CSI-RS-based beam failure detection and link recovery in DRX | Same as 7.5.5.6 |  | Same as 7.5.5.6 | Same as 7.5.5.6 |
| 7.5.5.9 NR SA FR2 SCell TRP specific CSI-RS-based Beam Failure Detection and Link Recovery in DRX | Noc = -104.7 dBm/120 kHz Ês1 / Noc for CSI-RS set q0,0 T1: 5 dB T2: -3 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês1 / Noc for CSI-RS set q0,1 T1-T5: 5 dB Ês1 / Noc for CSI-RS set q1,0 T1: 0.2 dB T2: 0.2 dB T3: 20.2 dB T4: 20.2 dB T5: 20.2 dB rsrp-ThresholdSSB: -95 dBm/SCS kHz |  | 0 dB 8.7 dB 8.7 dB 0 dB 0 dB 0 dB 8.7dB 0 dB 0 dB -0.2 dB -0.2 dB -0.2 dB -14 dB | Noc = -104.7 dBm/120 kHz Ês1 / Noc for CSI-RS set q0,0 T1: 13.7 dB T2: 5.7 dB T3: -12 dB T4: -12 dB T5: -12 dB Ês1 / Noc for CSI-RS set q0,1 T1-T5: 13.7 dB Ês1 / Noc for CSI-RS set q1,0 T1: 0.2 dB T2: 0.2 dB T3: 20 dB T4: 20 dB T5: 20 dB rsrp-ThresholdSSB: -109.0 dBm/SCS kHz |
| 7.5.5.10 NR SA FR2 PCell TRP specific SSB-based Beam Failure Detection and Link Recovery in non-DRX | During T1: SNR_SSB q0,0: 5 dB SNR_SSB q0,1: 5 dB SNR_SSB q1,0: 0.2 dB During T2: SNR_SSB q0,0: -3 dB SNR_SSB q0,1: 5 dB SNR_SSB q1,0: 0.2 dB During T3-T5: SNR_SSB q0,0: -12 dB SNR_SSB q0,1: 5 dB SNR_SSB q1,0: 20.2 dB In signalling: rsrp-ThresholdSSB: -95 dBm/120kHz for Config 1 rsrp-ThresholdSSB: -92 dBm/240kHz for Config 2 |  | +8.70 dB +8.70 dB 0 dB +8.50 dB +8.70 dB 0 dB 0 dB +8.70 dB -0.2 dB -14 dB -14 dB | During T1: SNR_SSB q0,0: 13.7 dB SNR_SSB q0,1: 13.7 dB SNR_SSB q1,0: 0.2 dB During T2: SNR_SSB q0,0: 5.5 dB SNR_SSB q0,1: 13.7 dB SNR_SSB q1,0: 0.2 dB During T3-T5: SNR_SSB q0,0: -12 dB SNR_SSB q0,1: 13.7 dB SNR_SSB q1,0: 20 dB In signalling: rsrp-ThresholdSSB: -109 dBm/120kHz for Config 1 srp-ThresholdSSB: -106 dBm/240kHz for Config 2 |
| 7.5.6.2.1 NR SA FR2 RRC-based DL active BWP switch in non-DRX | During T1: Noc2: -112dBm/15kHz Ês2 / Iot2: +18dB |  | During T1: 0dB 0dB | During T1: Noc2: -112dBm/15kHz Ês2 / Iot2: +18dB |
| 7.5.6.3.1 Active BWP switch on multiple SCells with non-DRX in SA | During T1/T2/T3: Noc1: -111.7dBm/15kHz Noc2: -111.7dBm/15kHz Noc3: -111.7dBm/15kHz Ês1 / Iot1: +7dB Ês2 / Iot2: +7dB Ês3 / Iot3: +7dB |  | During T1/T2/T3: 0dB 0dB 0dB 0dB 0dB 0dB | During T1/T2/T3: Noc1: -111.7dBm/15kHz Noc2: -111.7dBm/15kHz Noc3: -111.7dBm/15kHz Ês1 / Iot1: +7dB Ês2 / Iot2: +7dB Ês3 / Iot3: +7dB |
| 7.5.6.5.1 Active BWP switch on multiple SCells with non-DRX in SA | During T1: Noc1: -111.7dBm/15kHz Noc2: -111.7dBm/15kHz Ês1 / Iot1: +18dB Ês2 / Iot2: +18dB |  | During T1: 0dB 0dB 0dB 0dB | During T1: Noc1: -111.7dBm/15kHz Noc2: -111.7dBm/15kHz Ês1 / Iot1: +18dB Ês2 / Iot2: +18dB |
| 7.5.8.1.1 NR SA PCell FR2 MAC-CE based active TCI state switch for a known TCI state | During T1: SSB#0: Ês / Iot, BB: +8.3dB SSB#0: Ês: -80.6dB SSB#1: Ês / Iot, BB: - infinity SSB#1: Ês: - infinity During T2: SSB#0: Ês / Iot, BB: +8.3dB SSB#0: Ês: -80.6dB SSB#1: Ês / Iot, BB: +8.3dB SSB#1: Ês: -80.6dB |  | During T1: 0dB -0.3dB 0dB 0dB During T2: 0dB -0.3FFSdB 0dB -0.3FFSdB | During T1: SSB#0: Ês / Iot, BB: +8.3dB SSB#0: Ês: -80.9dB SSB#1: Ês / Iot, BB: - infinity SSB#1: Ês: - infinity During T2: SSB#0: Ês / Iot, BB: +8.3dB SSB#0: Ês: -80.9dB SSB#1: Ês / Iot, BB: +8.3dB SSB#1: Ês: -80.9dB |
| 7.5.8.2.1 NR SA PCell FR2 RRC based active TCI state switch for a known TCI state | Same as 7.5.8.1.1 |  | Same as 7.5.8.1.1 | Same as 7.5.8.1.1 |
| 7.5.9.1.1 EN-DC PSCell FR2 uplink spatial relation switch for a known spatial relation | During T1: Ês / Noc: 1 dB During T2: Ês / Noc: 1 dB During T3: Ês / Noc: -infinity dB During T4: Ês / Noc: 1 dB |  | During T1: 0 dB During T2: 0 dB During T3: 0 dB During T4: 0 dB | During T1: Ês / Noc: 1 dB During T2: Ês / Noc: 1 dB During T3: Ês / Noc: -infinity dB During T4: Ês / Noc: 1 dB |
| 7.5.9.2.1 EN-DC PSCell FR2 uplink spatial relation switch associated with a known DL-RS | During T1: Ês / Noc: 1 dB During T2: Ês / Noc: 1 dB During T3: Ês / Noc: -infinity dB During T4: Ês / Noc: 1 dB |  | During T1: 0 dB During T2: 0 dB During T3: 0 dB During T4: 0 dB | During T1: Ês / Noc: 1 dB During T2: Ês / Noc: 1 dB During T3: Ês / Noc: -infinity dB During T4: Ês / Noc: 1 dB |
| 7.5.10.1 NR FR2 UE specific CBW change of PCell with non-DRX in SA | During T1: Noc2: -112 dBm/15kHz Ês2 / Iot2: +18dB |  | During T1: 0 dB 0 dB | During T1: Noc2: -112 dBm/15kHz Ês2 / Iot2: +18dB |
| 7.5.11.1 NR SA FR2 UE UL carrier RRC reconfiguration delay | During T1: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: +7dB During T2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: +7dB During T3: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: +7dB |  | During T1: 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB During T3: 0dB 0dB 0dB 0dB | During T1: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: +7dB During T2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: +7dB During T3: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +7dB Ês2 / Noc2: +7dB |
| 7.5.12.1 Addition and Release Delay of PSCell | During T1: Ês: -infinity During T2~T4: Ês: -81dBm/SCS |  | During T1: 0dB During T2~T4: 0dB | During T1: Ês: -infinity During T2~T4: Ês: -81dBm/SCS |
| 7.5.13.6.1 NR SA FR2 MAC-CE based active uplink TCI state switch for 2 known TCI states | During T1: Noc: -83.10dBm/120kHz Ês0 / Noc: 1 dB Ês1/ Noc: -infinity Ês2 / Noc: -infinity Ês3 / Noc: -infinity During T2: Noc: -83.10dBm/120kHz Ês0 / Noc: 1 dB Ês1/ Noc: 1 dB Ês2 / Noc: 1 dB Ês3 / Noc: 1 dB |  | During T1: -3dB 2.4dB 0dB 0dB 0dB During T2: -3dB 2.4dB 2.4dB 2.4dB 2.4dB | During T1: Noc: -86.10dBm/120kHz Ês0 / Noc: 3.4 dB Ês1/ Noc: -infinity Ês2 / Noc: -infinity Ês3 / Noc: -infinity During T2: Noc: -86.10dBm/120kHz Ês0 / Noc: 3.4 dB Ês1/ Noc: 3.4 dB Ês2 / Noc: 3.4 dB Ês3 / Noc: 3.4 dB |
| 7.5.14 NR SA FR2 PSCell RACH-less based Activation and deactivation for FR1+FR2 inter-band with target PSCell in FR2 | During T1~T4: Noc: -104.7 dBm/15kHz Ês / Noc: 7 dB |  | During T1~T4: 0.9dB 0dB | During T1~T4: Noc: -103.8 dBm/15kHz Ês / Noc: 7 dB |
| 7.6.1.1 NR SA FR2 event-triggered reporting without gap in non-DRX | During T1: Ês1: -86dBm/120kHz Ês2: -infinity During T2: Ês1: -86dBm/120kHz Ês2: -86dBm/120kHz |  | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Ês1: -86dBm/120kHz Ês2: -infinity During T2: Ês1: -86dBm/120kHz Ês2: -86dBm/120kHz |
| 7.6.1.2 NR SA FR2 event-triggered reporting without gap in DRX | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Signalled A3-offset: -6.0 |  | During T1: -3.5dB 0dB 0dB During T2: -3.5dB 0dB 0dB Signalled A3-offset: -1.0 | During T1: Noc: -101.5dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -infinity During T2: Noc: -101.5dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB Signalled A3-offset: -7.0 |
| 7.6.1.3 NR SA FR2 event-triggered reporting with gap in non-DRX | Same as 7.6.1.1 |  | Same as 7.6.1.1 | Same as 7.6.1.1 |
| 7.6.1.4 NR SA FR2 event-triggered reporting with gap in DRX | Same as 7.6.1.2 |  | Same as 7.6.1.2 | Same as 7.6.1.2 |
| 7.6.2.1 NR SA FR2-FR2 event-triggered reporting in non-DRX | During T1: Ês1: -87 dBm/120kHz Ês2: - infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz |  | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Ês1: -87 dBm/120kHz Ês2: - infinity During T2: Ês1: -87 dBm/120kHz Ês2: -87 dBm/120kHz |
| 7.6.2.2 NR SA FR2-FR2 event-triggered reporting in DRX | During T1: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: -infinity During T2: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +9.00dB Signalled A3-offset: -6.0 |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB Signalled A3-offset: -6.0 | During T1: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: -infinity During T2: Noc: -104.70dBm/15kHz Ês1 / Noc: +6.00dB Ês2 / Noc: +9.00dB Signalled A3-offset: -12.0 |
| 7.6.2.3 NR SA FR2-FR2 event-triggered reporting in non-DRX with SSB time index detection | Same as 7.6.2.1 |  | Same as 7.6.2.1 | Same as 7.6.2.1 |
| 7.6.2.4 NR SA FR2-FR2 event-triggered reporting in DRX with SSB time index detection | Same as 7.6.2.2 |  | Same as 7.6.2.1 | Same as 7.6.2.1 |
| 7.6.2.5 SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1) | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz |  | During T1: 0dB During T2: 0dB | During T1: Ês2: - Infinity During T2: Ês2: -87 dBm/120kHz |
| 7.6.2.6 SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1) | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB |  | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: Noc = -104.7 dBm/15 kHz Ês2 / Noc = - Infinity During T2: Noc = -104.7 dBm/15 kHz Ês2 / Noc = 9 dB |
| 7.6.2.7 SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1) | Same as 5.6.2.5 |  | Same as 5.6.2.5 | Same as 5.6.2.5 |
| 7.6.2.8 SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1) | Same as 5.6.2.6 |  | Same as 5.6.2.6 | Same as 5.6.2.6 |
| 7.6.2.9 SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17) | During T1: SS-RSRP1: -87 dBm/SCS SS-RSRP2: -Infinity During T2: SS-RSRP1: -87 dBm/SCS SS-RSRP2: -87 dBm/SCS |  | During T1: 0dB 0dB During T2: 0dB 0dB | During T1: SS-RSRP1: -87 dBm/SCS SS-RSRP2: -Infinity During T2: SS-RSRP1: -87 dBm/SCS SS-RSRP2: -87 dBm/SCS |
| 7.6.2.10 | During T1: Noc: -102dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -Infinity During T2: Noc: -102dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +8.00dB Signalled A3-Offset: - 6dB |  | During T1: -0.7dB 0dB 0dB During T2: -0.7dB 0dB 0dB Signalled A3-offset: -4 dB | During T1: Noc: -102.7dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -Infinity During T2: Noc: -102.7dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +8.00dB Signalled A3-Offset: - 10dB |
| 7.6.2.11 | During T1: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -Infinity During T2: Noc: -98dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |  | During T1: -1.2dB 0dB 0dB During T2: -1.2dB 0dB 0dB | During T1: Noc: -99.2dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: -Infinity During T2: Noc: -99.2dBm/15kHz Ês1 / Noc: +4.00dB Ês2 / Noc: +4.00dB |
| 7.6.2.20 NR SA FR2-FR2 event triggered reporting using MUSIM gap with higher priority | Same as 7.6.2.1 |  | Same as 7.6.2.1 | Same as 7.6.2.1 |
| 7.6.2.21 NR SA FR2-FR2 event triggered reporting using MUSIM gap with shorter MGRP | Same as 7.6.2.1 |  | Same as 7.6.2.1 | Same as 7.6.2.1 |
| 7.6.3.1 NR SA FR2 SSB-based L1-RSRP measurement in non-DRX | Same as 5.6.3.1 |  | Same as 5.6.3.1 | Same as 5.6.3.1 |
| 7.6.3.2 NR SA FR2 SSB-based L1-RSRP measurement in DRX | Same as 5.6.3.1 |  | Same as 5.6.3.1 | Same as 5.6.3.1 |
| 7.6.3.3 NR SA FR2 CSI-RS-based L1-RSRP measurement in non-DRX | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-RSRP values ± 29dB Reported Differential CSI-RSRP values ± 7dB |  | 0dB 0dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB CSI-RS#1: RSRP_40 to RSRP_99 CSI-RS#0: DIFFRSRP_1 to DIFFRSRP_7 |
| 7.6.3.4 NR SA FR2 CSI-RS-based L1-RSRP measurement in DRX | Same as 7.6.3.3 |  | Same as 7.6.3.3 | Same as 7.6.3.3 |
| 7.6.3.6 NR SA FR2 Inter-cell SSB based L1-RSRP measurements on FR2 SCell when DRX is not used | Same as 7.6.6.2 |  | Same as 7.6.6.2 | Same as 7.6.62 |
| 7.6.4.1 NR SA FR2 SRS-RSRP measurement in non-DRX | Same as 5.6.4.1 |  | Same as 5.6.4.1 | Same as 5.6.4.1 |
| 7.6.5.1 SA inter frequency CGI reporting in autonomous gaps test (PCell in FR2) | During T1: Noc: -99.03 dBm/15kHz Ês1 / Noc: +3 dB Ês2 / Noc: -3 dB During T2: Noc: -99.03 dBm/15kHz Ês1 / Noc: +3 dB Ês2 / Noc: -3 dB |  | During T1: 0 dB 0 dB 6 dB During T2: 0 dB 0 dB 6 dB | During T1: Noc: -99.03 dBm/15kHz Ês1 / Noc: +3 dB Ês2 / Noc: +3 dB During T2: Noc: -99.03 dBm/15kHz Ês1 / Noc: +3 dB Ês2 / Noc: +3 dB |
| 7.6.6.1 NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | Same as 5.6.6.1 |  | Same as 5.6.6.1 | Same as 5.6.6.1 |
| 7.6.6.2 NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported SS-SINR values ± 4.5dB Reported Differential SS-SINR values ± 3.5dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB Via mapping | During T1: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: -infinity During T2: Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB SSB#1: SINR_53 to SINR_72 SSB#0: DIFFSINR_5 to DIFFSINR_12 |
| 7.6.6.3 NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | Noc: -105dBm/15kHz Ês0 / Noc: 0.00dB Ês1 / Noc: +9.00dB Reported CSI-SINR values ± 4.0dB Reported Differential CSI-SINR values ± 3.0dB |  | 0dB 1.5dB 0dB Via mapping | Noc: -105dBm/15kHz Ês0 / Noc: +1.50dB Ês1 / Noc: +9.00dB CSI-RS#1: SINR_54 to SINR_71 CSI-RS#0: DIFFSINR_4 to DIFFSINR_10 |
| 7.6.13.1 NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using PRS | During T1: Noc : -98 dBm/15kHz Ês / Noc: infinity During T2: Noc: -98 dBm/15kHz Ês / Noc: -2 dB |  | During T1: 0 dB 0 dB During T2: 0 dB 0 dB | During T1: Noc : -98 dBm/15kHz Ês / Noc: infinity During T2: Noc: -98 dBm/15kHz Ês / Noc: -2 dB |
| 7.6.13.2 NR SA FR2 UE Rx-Tx time difference measurement for propagation delay compensation using TRS | Same as 7.6.13.1 |  | Same as 7.6.13.1 | Same as 7.6.13.1 |
| 7.6.14.1 NR SA FR2 event triggered reporting tests with pre-configured measurement gaps and autonomous activation/deactivation | During T1 – T2: Ês1: -89dBm/120kHz Ês2: -infinity During T3: Ês1: -89dBm/120kHz Ês2: -89dBm/120kHz Signalled A4-Thresholds: -120 dBm/SCS |  | During T1 - T2: 0dB 0dB During T3: 0dB 0dB 0 dB | During T1 – T2: Ês1: -89dBm/120kHz Ês2: -infinity During T3: Ês1: -89dBm/120kHz Ês2: -89dBm/120kHz Signalled A4-Thresholds: -120 dBm/SCS |
| 7.6.14.2 NR SA FR2 event triggered reporting tests with pre-configured measurement gaps and network-controlled activation/deactivation | During T1 – T2: Ês1: -89dBm/120kHz Ês2: -infinity During T3: Ês1: -89dBm/120kHz Ês2: -89dBm/120kHz Signalled A3-Offset: -11 dB |  | During T1 – T2: 0dB 0dB During T3: 0dB 0dB 0 dB | During T1 – T2: Ês1: -89dBm/120kHz Ês2: -infinity During T3: Ês1: -89dBm/120kHz Ês2: -89dBm/120kHz Signalled A3-Offset: -11 dB |
| 7.6.15.1 NR SA FR2 SA event triggered reporting tests with fully non-overlapping concurrent MGs for SSB-based inter-frequency measurements | During T1: Ês1: -87dBm/120kHz Ês2: -infinity Ês3: -infinity During T2: Ês1: -87dBm/120kHz Ês2: -87dBm/120kHz Ês3: -87dBm/120kHz Signalled offsetMO: 16 dB Signalled A3-Offset: -11 dB |  | During T1: 0dB 0dB 0dB During T2: 0dB 0dB 0dB Signalled offsetMO: 0dB Signalled A3-offset: 0 dB | During T1: Ês1: -87dBm/120kHz Ês2: -infinity Ês3: -infinity During T2: Ês1: -87dBm/120kHz Ês2: -87dBm/120kHz Ês3: -87dBm/120kHz Signalled offsetMO: 16dB Signalled A3-offset: -11dB |
| 7.6.15.2 NR SA FR2 event triggered reporting tests with concurrent measurement gaps without SSB time index detection when DRX is not used (PCell in FR2) | Same as 7.6.15.1 |  | Same as 7.6.15.1 | Same as 7.6.15.1 |
| 7.6.15.3 NR SA FR2 event triggered reporting tests with concurrent measurement gaps with partially partial overlapping scenario for SSB-based measurements and PRS-based measurement | During T1: Noc1: -102.0dBm/15kHz Noc2: -104.7dBm/15kHz Noc3: -102.0dBm/15kHz SSB Ês1: -89.7dBm/120kHz SSB Ês2: -infinity SSB Ês3: -infinity PRS Ês1: -infinity PRS Ês3: -infinity During T2: Noc1: -102.0dBm/15kHz Noc2: -104.7dBm/15kHz Noc3: -102.0dBm/15kHz SSB Ês1: -89.7dBm/120kHz SSB Ês2: -86.7dBm/120kHz SSB Ês3: -86.7dBm/120kHz PRS Ês1: -96dBm/120kHz PRS Ês3: -103dBm/120kHz Signalled A3-Offset: -6 dB |  | During T1: 0dB 0dB 0dB 0dB 0dB 0dB 0dB 0dB During T2: 0dB 0dB 0dB 0dB 0dB -0.6dB 0dB 0.2dB Signalled A3-offset: -6 dB | During T1: Noc1: -102.0dBm/15kHz Noc2: -104.7dBm/15kHz Noc3: -102.0dBm/15kHz SSB Ês1: -89.7dBm/120kHz SSB Ês2: -infinity SSB Ês3: -infinity PRS Ês1: -infinity PRS Ês3: -infinity During T2: Noc1: -102.0dBm/15kHz Noc2: -104.7dBm/15kHz Noc3: -102.0dBm/15kHz SSB Ês1: -89.7dBm/120kHz SSB Ês2: -86.7dBm/120kHz SSB Ês3: -87.3dBm/120kHz PRS Ês1: -96dBm/120kHz PRS Ês3: -102.8dBm/120kHz Signalled A3-offset: -12dB |
| 7.6.16.1 NR SA FR2 event triggered reporting tests with per-UE NCSG under non-DRX | Same as 7.6.1.1 |  | Same as 7.6.1.1 | Same as 7.6.1.1 |
| 7.6.16.2 NR SA FR2 event triggered reporting tests on inter-frequency measurement with NCSG when DRX is not used(PCell in FR2) | During T1: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +6 dB Ês2 / Noc2: -infinity dB During T2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +6 dB Ês2 / Noc2: +9 dB Signalled offsetMO: 16 dB Signalled A3-Offset: -11 dB |  | During T1: 0 dB 0 dB 2.7 dB 0 dB During T2: 0 dB 0 dB 2.7 dB 0 dB 0 dB 0 dB | During T1: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +8.7 dB Ês2 / Noc2: -infinity dB During T2: Noc1: -104.7dBm/15kHz Noc2: -104.7dBm/15kHz Ês1 / Noc1: +8.7 dB Ês2 / Noc2: +9 dB Signalled offsetMO: 16 dB Signalled A3-Offset: -11 dB |
| 7.6.16.3 NR SA FR2 event triggered reporting tests on deactivated SCell measurement via NCSG in non-DRX | During T1: Ês2: -89dBm/120kHz Ês3: -infinity During T2: Ês2: -89dBm/120kHz Ês3: -89dBm/120kHz Signalled A3-Offset: -11 dB |  | During T1: 0 dB 0 dB During T2: 0 dB 0 dB 0 dB | During T1: Ês2: -89dBm/120kHz Ês3: -infinity During T2: Ês2: -89dBm/120kHz Ês3: -89dBm/120kHz Signalled A3-Offset: -11 dB |
| 7.6.19.1 NR SA FR2 event-triggered reporting without interruption outside gap | Same as 7.6.1.1 |  | Same as 7.6.1.1 | Same as 7.6.1.1 |
| 7.6.19.2 NR SA FR2 event triggered reporting test without gap without interruption | Same as 7.6.1.1 |  | Same as 7.6.1.1 | Same as 7.6.1.1 |
| 7.6.19.3 NR SA FR2-FR2 event triggered reporting test without gap without interruption under non-DRX | Same as 7.6.2.2 |  | Same as 7.6.2.10 | Same as 7.6.2.10 |
| 7.6.20.1 NR SA FR2 Intra-frequency SSB based L1-RSRP measurement | Signalled A3-Offset: -15dB During T1: Ês/IotBB1: -1.5dB Ês/IotBB2: -1.5dB During T2: Ês/IotBB1: +16.5dB Ês/IotBB2: +16.5dB |  | 0dB During T1: 1.35dB 1.35dB During T2: 0dB 0dB | Signalled A3-Offset: -15dB During T1: Ês/IotBB1: -0.15dB Ês/IotBB2: -0.15dB During T2: Ês/IotBB1: +16.5dB Ês/IotBB2: +16.5dB |
| 7.6.21.1 NR SA FR2 Inter-frequency SSB-based L1-RSRP measurement with measurement gap for LTM | Signalled A3-Offset: -15dB Signalled Cell 2 offsetMO: 16dB During T1: Ês/IotBB1: 16.5dB Ês/IotBB2: -1.5dB During T2: Ês/IotBB1: +16.5dB Ês/IotBB2: +16.5dB |  | 0dB 0dB During T1: 0dB 6dB During T2: 0dB 0dB | Signalled A3-Offset: -15dB Signalled Cell 2 offsetMO: 16dB During T1: Ês/IotBB1: 16.5dB Ês/IotBB2: 4.5dB During T2: Ês/IotBB1: +16.5dB Ês/IotBB2: +16.5dB |
| 7.7.1.1 NR SA FR2 SS-RSRP measurement accuracy | UE PC1 |  |  |  |
|  | Test 1: Noc: -91.60dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.0dB Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 Ês2: -108.2dBm/120kHz n257 Ês3: -108.2dBm/120kHz n260 Ês2: -105.2dBm/120kHz n260 Ês3: -105.2dBm/120kHz Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB+ FFS MU additionally |  | Test 1: -5.80dB 0dB 0.4dB Via Mapping Test 2: 6dB 6dB 6dB 6dB Via Mapping | Test 1: Noc: -97.40dBm/15kHz Ês2 / Noc: +6.0dB Ês3 / Noc: +1.4dB Cell 2: RSRP_61 to RSRP_126 Cell 3: RSRP_56 to RSRP_126 Cell 3: RSRP_x-12 to RSRP_X+2 Test 2: n257 Ês2: -102.2dBm/120kHz n257 Ês3: -102.2dBm/120kHz n260 Ês2: -99.2dBm/120kHz n260 Ês3: -99.2dBm/120kHz Band dependent. See test case tables in clause 5.7.1.1.5 |
|  | UE PC3 |  |  |  |
|  | Test 1: Noc: -91.60dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.0dB Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 Ês1: -110.0dBm/120kHz n257 Ês2: -110.0dBm/120kHz n260 Ês1: -107.4dBm/120kHz n260 Ês2: -107.4dBm/120kHz Reported absolute RSRP values: ±8dB Reported relative RSRP values: ±6dB For extreme conditions allow ±3dB+ FFS MU additionally |  | Test 1: -5.80dB 0dB 0.4dB Via Mapping Test 2: 7.70dB 7.70dB 7.70dB 7.70dB Via Mapping | Test 1: Noc: -97.40dBm/15kHz Ês1 / Noc: +6.0dB Ês2 / Noc: +1.4dB Cell 1: RSRP_50 to RSRP_108 Cell 2: RSRP_46 to RSRP_103 Cell 2: RSRP_x-12 to RSRP_X+2 Test 2: n257 Ês1: -102.3dBm/120kHz n257 Ês2: -102.3dBm/120kHz n260 Ês1: -99.7dBm/120kHz n260 Ês2: -99.7dBm/120kHz Band dependent. See test case tables in clause 5.7.1.1.5 |
| 7.7.1.2 NR SA FR2-FR2 SS-RSRP measurement accuracy | Test 1 Config 1: Noc1: -90.60dBm/15kHz Noc2: -90.60dBm/15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 2 Config 1: n257 Noc1: -108.13dBm/15kHz n257 Noc2: -113.13dBm/15kHz n260 Noc1: -105.53dBm/15kHz n260 Noc2: -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: -1.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 1 Config 2: Noc1: -93.70dBm/15kHz Noc2: -93.70dBm/15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally Test 2 Config 2: n257 Noc1: -108.13dBm/15kHz n257 Noc2: -113.13dBm/15kHz n260 Noc1: -105.53dBm/15kHz n260 Noc2: -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: -1.0dB Reported absolute RSRP values: ±8.0dB For extreme conditions allow ±3dB+ FFS MU additionally Reported relative RSRP values: ±6.0dB For extreme conditions allow ±3dB+ FFS MU additionally |  | Test 1 Config 1: -5.70dB -5.70dB 0dB 0dB Via Mapping Via Mapping Test 2 Config 1: -6.60dB 0dB -6.60dB 0dB 0dB 2dB Via Mapping Via Mapping Test 1 Config 2: -5.60dB -5.60dB 0dB 0dB Via Mapping Via Mapping Test 2 Config 2: -6.60dB 0dB -6.60dB 0dB 0dB 2dB Via Mapping Via Mapping | Test 1 Config 1: Noc1: -96.30dBm/15kHz Noc2: -96.30dBm /15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Cell 1 n257: RSRP_41 to RSRP_109 n260: RSRP_39 to RSRP_109 Cell 2: RSRP_52 to RSRP_109 Cell 2: n257: RSRP_x-23 to RSRP_x+26 n260: RSRP_x-23 to RSRP_x+27 Test 2 Config 1: -114.73dBm/15kHz -113.13dBm/15kHz -112.13dBm/15kHz -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: 1.0dB Cell 1 n257: RSRP_33 to RSRP_101 n260: RSRP_34 to RSRP_104 Cell 2: n257: RSRP_32 to RSRP_87 n260: RSRP_34 to RSRP_90 Cell 2: n257: RSRP_x-37 to RSRP_x+17 n260: RSRP_x-37 to RSRP_x+19 Test 1 Config 2: Noc1: -99.30dBm/15kHz Noc2: -99.30dBm /15kHz Ês1 / Noc1: +6.0dB Ês2 / Noc2: +6.0dB Cell 1 n257: RSRP_41 to RSRP_109 n260: RSRP_39 to RSRP_109 Cell 2: RSRP_52 to RSRP_109 Cell 2: n257: RSRP_x-23 to RSRP_x+26 n260: RSRP_x-23 to RSRP_x+27 Test 2 Config 2: -114.73dBm/15kHz -113.13dBm/15kHz -112.13dBm/15kHz -110.53dBm/15kHz Ês1 / Noc1: 17.0dB Ês2 / Noc2: 1.0dB Cell 1 n257: RSRP_36 to RSRP_104 n260: RSRP_37 to RSRP_107 Cell 2: n257: RSRP_35 to RSRP_90 n260: RSRP_37 to RSRP_93 Cell 2: n257: RSRP_x-37 to RSRP_x+17 n260: RSRP_x-37 to RSRP_x+19 |
| 7.7.1.3 NR SA FR1-FR2 SS-RSRP measurement accuracy | Test 1: Noc: -90dBm/15kHz Ês2 / Noc: +5.0dB Reported absolute RSRP values: ±8dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 Ês2: -98.2dBm/120kHz n260 Ês2: -93.9dBm/120kHz Reported absolute RSRP values: ±8dBFor extreme conditions allow ±3dB+ FFS MU additionally |  | Test 1: -5.50dB 0dB Via Mapping Test 2: 5.50dB 5.50dB Via Mapping | Test 1: Noc: -95.50dBm/15kHz Ês2 / Noc: +5dB Band dependent. See test case tables in clause 7.7.1.3.5 Test 2: n257 Ês2: -92.7dBm/120kHz n260 Ês2: -88.4dBm/120kHz Band dependent. See test case tables in clause 7.7.1.3.5 |
| 7.7.2.1 NR SA FR2 SS-RSRQ measurement accuracy | Test 1: Noc: -95.0dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Reported absolute RSRQ values: ±2.5dB For extreme conditions allow ±1.5dB+ FFS MU additionally Test 2: Noc: -95.0dBm/15kHz Ês1 / Noc: -3.0dB Ês2 / Noc: -3.0dB Reported absolute RSRQ values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally |  | Test 1: -5.70dB 0dB 0dB Via Mapping Test 2: -1.70dB 0dB 0dB Via Mapping | Test 1: Noc: -100.70dBm/15kHz Ês1 / Noc: +3.0dB Ês2 / Noc: +3.0dB Cell 2: All bands: RSRQ_41 to RSRQ_73 Test 2: Noc: -96.70dBm/15kHz Ês1 / Noc: -3.0dB Ês2 / Noc: -3.0dB Cell 2: n257, n258, n261: RSRQ_35 to RSRQ_71 n260: RSRQ_34 to RSRQ_71 |
| 7.7.2.2 NR SA FR2-FR2 SS-RSRQ measurement accuracy | Test 1: Noc1: -94.03dBm/15kHz Noc2: -94.03dBm/15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Reported absolute RSRQ values: ±2.5dB For extreme conditions allow ±1.5dB+ FFS MU additionally Reported relative RSRQ values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Test 2: Noc1: -94.03dBm/15kHz Noc2: -94.03dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute RSRQ values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Reported relative RSRQ values: ±4.0dB For extreme conditions allow ±0.0dB+ FFS MU additionally |  | Test 1: -1.90dB -1.90dB 0dB 0dB Via Mapping Via Mapping Test 2: -1.41dB -1.41dB 0dB 0dB Via Mapping Via Mapping | Test 1: Noc1: -95.93dBm/15kHz Noc2: -95.93dBm /15kHz Ês1 / Noc1: -1.75dB Ês2 / Noc2: -1.75dB Cell 1 and Cell 2: RSRQ_41 to RSRQ_73 Cell 2: RSRQ_x-7 to RSRQ_x+7 Test 2: Noc1: -95.44dBm/15kHz Noc2: -95.44dBm /15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Cell 1 and Cell 2: RSRQ_37 to RSRQ_74 Cell 2: RSRQ_x-9 to RSRQ_x+9 |
| 7.7.3.1 NR SA FR2 SS-SINR measurement accuracy | Test 1: Noc: -105.0dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +2.66dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 2: Noc: -105.0dBm/15kHz Ês1 / Noc: -3.0dB Ês2 / Noc: -3.0dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally |  | Test 1: 0dB 0dB 0dB Via Mapping Test 2: 0dB 0.2dB 0.2dB Via Mapping | Test 1: Noc: -105.0dBm/15kHz Ês1 / Noc: +4.54dB Ês2 / Noc: +2.66dB Cell 2: n257, n258, n261: SINR_22 to RSRQ_58 n260: SINR_21 to RSRQ_58 Test 2: Noc: -105.0dBm/15kHz Ês1 / Noc: -2.8dB Ês2 / Noc: -2.8dB Cell 2: n257, n258, n261: SINR_18 to RSRQ_55 n260: SINR_18 to RSRQ_54 |
| 7.7.3.2 NR SA FR2-FR2 SS-SINR measurement accuracy | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Reported absolute SINR values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 2: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: +11.0dB Ês2 / Noc2: +11.0dB Reported absolute SINR values: ±3.0dB For extreme conditions allow ±1.0dB+ FFS MU additionally Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -3.0dB Ês2 / Noc2: -3.0dB Reported absolute SINR values: ±3.5dB For extreme conditions allow ±0.5dB+ FFS MU additionally Reported absolute SINR values: ±4.0dB For extreme conditions allow ±0.0dB+ FFS MU additionally |  | Test 1: 0dB 0dB 0dB 0dB Via Mapping Via Mapping Test 2: -0.1dB -0.1dB 0dB 0dB Via Mapping Via Mapping Test 3: 0dB 0dB 0.9dB 0.9dB Via Mapping Via Mapping | Test 1: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -0.5dB Ês2 / Noc2: -0.5dB Cell 1 and Cell 2: n257, n258, n261: SINR_27 to SINR_62 n260: SINR_27 to SINR_61 Cell 2: SINR_x-8 to SINR_x+8 Test 2: Noc1: -105.1dBm/15kHz Noc2: -105.1dBm/15kHz Ês1 / Noc1: +11.0dB Ês2 / Noc2: +11.0dB Cell 1 and Cell 2: n257, n258, n261: SINR_48 to SINR_87 n260: SINR_48 to SINR_86 Cell 2: SINR_x-17 to SINR_x+17 Test 3: Noc1: -105.0dBm/15kHz Noc2: -105.0dBm/15kHz Ês1 / Noc1: -2.1dB Ês2 / Noc2: -2.1dB Cell 1 and Cell 2: n257, n258, n261: SINR_23 to SINR_60 n260: SINR_22 to SINR_59 Cell 2: SINR_x-9 to SINR_x+9 |
| 7.7.4.1 NR SA FR2 SSB based L1-RSRP measurement accuracy | Test 1: Noc: -100dBm/15kHz SSB#0 Ês / Noc: +10.0dB SSB#1 Ês / Noc: -2.0dB Reported absolute RSRP values: ±8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 SSB#0 Ês: -108.1dBm/120kHz n257 SSB#1 Ês: -105.5dBm/120kHz n260 SSB#0 Ês: -108.1dBm/120kHz n260 SSB#1 Ês: -105.5dBm/120kHz Reported absolute RSRP values: -6.5 +8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB+ FFS MU additionally |  | Test 1: -4.1dB 0dB 0.4dB Test 2: 5.7dB | Test 1: Noc: -104.1dBm/15kHz SSB#0 Ês / Noc: +10.0dB SSB#1 Ês / Noc: -1.6dB SSB#0: RSRP_42 to RSRP_101 SSB#1: RSRP_31 to RSRP_89 SSB#1-SSB#0: -9 to -2 Test 2: n257 SSB#0 Ês: -102.4dBm/120kHz n257 SSB#1 Ês: -99.8dBm/120kHz n260 SSB#0 Ês: -102.4dBm/120kHz n260 SSB#1 Ês: -99.8dBm/120kHz SSB#0 and SSB#1 n257: RSRP_27 to RSRP_83 n260: RSRP_30 to RSRP_86 SSB#1-SSB#0: -4 to +4 |
| 7.7.4.2 NR SA FR2 CSI-RS based L1-RSRP measurement accuracy | Test 1: Noc: -100dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -2.0dB Reported absolute RSRP values: ±8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB + FFS MU additionally Test 2: n257 CSI-RS#0 Ês: -108.1dBm/120kHz n257 CSI-RS#1 Ês: -105.5dBm/120kHz n260 CSI-RS#0 Ês: -108.1dBm/120kHz n260 CSI-RS#1 Ês: -105.5dBm/120kHz Reported absolute RSRP values: -6.5 +8.5dB Reported relative RSRP values: ±6.5dB For extreme conditions allow ±3dB+ FFS MU additionally |  | Test 1: -4.1dB 0dB 0.2dB Test 2: 5.7dB | Test 1: Noc: -104.1dBm/15kHz CSI-RS#0 Ês / Noc: +10.0dB CSI-RS#1 Ês / Noc: -1.8dB CSI-RS#0: RSRP_42 to RSRP_101 CSI-RS#1: RSRP_30 to RSRP_89 CSI-RS#1-CSI-RS#0: -9 to -2 Test 2: n257 CSI-RS#0 Ês: -102.4dBm/120kHz n257 CSI-RS#1 Ês: -99.8dBm/120kHz n260 CSI-RS#0 Ês: -102.4dBm/120kHz n260 CSI-RS#1 Ês: -99.8dBm/120kHz CSI-RS#0 and CSI-RS#1 n257: RSRP_27 to RSRP_83 n260: RSRP_30 to RSRP_86 CSI-RS#1-CSI-RS#0: -4 to +4 |
| 7.7.5.1 NR SA FR2 SRS-RSRP measurement accuracy | Same as 5.7.5.1 |  | Same as 5.7.5.1 | Same as 5.7.5.1 |
| 7.7.6.1 NR SA FR2 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy | Noc: -100dBm/15kHz Ês0 / Noc: 10.00dB Ês1 / Noc: -2.00dB Reported CSI-SINR values ± 5.5dB Reported Differential CSI-SINR values ± 4.5dB |  | -4.1dB 0dB 0.2dB Via mapping | Noc: -104.1dBm/15kHz Ês0 / Noc: 10.00dB Ês1 / Noc: -1.8dB CSI-RS#0: SINR_53 to SINR_76 CSI-RS#0: DIFFSINR_6 to DIFFSINR_15 |
| 7.7.6.2 NR SA FR2 SSB based CMR and dedicated IMR L1-SINR measurement accuracy | Noc: -100dBm/15kHz Ês0 / Noc: 10.00dB Ês1 / Noc: -2.00dB Reported SS-SINR values ± 4.5dB Reported Differential SS-SINR values ± 3.5dB |  | -4.1dB 0dB 0.2dB Via mapping | Noc: -104.1dBm/15kHz Ês0 / Noc: 10.00dB Ês1 / Noc: -1.8dB SSB#0: SINR_55 to SINR_74 SSB#0: DIFFSINR_7 to DIFFSINR_15 |
| 7.7.6.3 NR SA FR2 CSI-RS based CMR and dedicated IMR L1-SINR measurement accuracy | Noc: -100dBm/15kHz Ês0 / Noc: 10.00dB Ês1 / Noc: 0.00dB Reported CSI-SINR values ± 4.5dB Reported Differential CSI-SINR values ± 3.5dB |  | -4.1dB 0dB 0.2dB Via mapping | Noc: -104.1dBm/15kHz Ês0 / Noc: 10.00dB Ês1 / Noc: 0.2dB CSI-RS#0: SINR_55 to SINR_74 CSI-RS#0: DIFFSINR_5 to DIFFSINR_13 |
| 7.7.8.1 NR SA intra-frequency FR2 CSI-RSRQ measurement accuracy | Same as 5.7.8.1 |  | Same as 5.7.8.1 | Same as 5.7.8.1 |
| 7.7.8.2 NR SA inter-frequency FR2 CSI-RSRQ measurement accuracy | Same as 5.7.8.2 |  | Same as 5.7.8.2 | Same as 5.7.8.2 |
| 7.7.9.1 NR SA intra-frequency FR2 CSI-SINR measurement accuracy | Same as 5.7.9.1 |  | Same as 5.7.9.1 | Same as 5.7.9.1 |
| 7.7.9.2 NR SA inter-frequency FR2 CSI-SINR measurement accuracy | Same as 5.7.9.2 |  | Same as 5.7.9.2 | Same as 5.7.9.2 |
| 7.7.15.1 NR SA FR2 SSB based inter-frequency L1-RSRP measurement | Test configuration 1 Signalled a4-Threshold: -120dBm Signalled offsetMO: 16dB Test 1: Noc1: -100dBm/15kHz Noc2: -100dBm/15kHz Ês1/Noc1 : +10.0dB Ês2/Noc2: -2.0dB Reported absolute L1-RSRP values: Normal condition: ±8.5dB Extreme condition: ±11.5dB+FFS MU Test 2: n257 Ês1: -109.1dBm/120kHz n259 Ês1: -105.5dBm/120kHz n260 Ês1: -106.5dBm/120kHz n257 Ês2: -109.1dBm/120kHz n259 Ês2: -105.5dBm/120kHz n260 Ês2: -106.5dBm/120kHz Reported absolute L1-RSRP values: Normal condition: ±6.5dB/±8.5dB Extreme condition: (±9.5dB/±11.5dB) + FFS MU |  | Test configuraiton 1 0dB 0dB Test 1: -4.1dB 0dB 0dB 0dB Via mapping Test 2: 3.65dB 3.65dB 3.65dB 6.65dB 6.65dB 6.65dB Via mapping | Test configuaiton 1 Signalled a4-Threshold: -120dBm Signalled offsetMO: 16dB Test 1: Noc1: -104.1dBm/15kHz Noc2: -100dBm/15kHz Ês1/Noc1: +10.0dB Ês2/Noc2: -2dB Normal condition: RSRP_39 to RSRP_98 Test 2: n257 Ês1: -105.45dBm/120kHz n259 Ês1: -101.85dBm/120kHz n260 Ês1: -102.85dBm/120kHz n257 Ês2: -102.45dBm/120kHz n259 Ês2: -98.85dBm/120kHz n260 Ês2: -99.85dBm/120kHz Normal condition: n257: RSRP_32 to RSRP_88 n259: RSRP_36 to RSRP_92 n260: RSRP_35 to RSRP_91 |
|  | Test configuration 2 Signalled a4-Threshold: -120dBm Signalled offsetMO: 16dB Test 1: Noc1: -100dBm/15kHz Noc2: -100dBm/15kHz Ês1/Noc1: +10.0dB Ês2/Noc2: -2.0dB Reported absolute L1-RSRP values: Normal condition: ±8.5dB Extreme condition: ±11.5dB+FFS MU Test 2: n257 Ês1: -106.1dBm/240kHz n259 Ês1: -102.5dBm/240kHz n260 Ês1: -103.5dBm/240kHz n257 Ês2: -106.1dBm/240kHz n259 Ês2: -102.5dBm/240kHz n260 Ês2: -103.5dBm/240kHz Reported absolute L1-RSRP values: Normal condition: ±6.5dB/±8.5dB Extreme condition: (±9.5dB/±11.5dB) + FFS MU |  | Test configuraiton 2 0dB 0dB Test 1: -4.1dB 0dB 0dB 0dB Via mapping Test 2: 3.65dB 3.65dB 3.65dB 6.65dB 6.65dB 6.65dB Via mapping | Test configuaiton 2 Signalled a4-Threshold: -120dBm Signalled offsetMO: 16dB Test 1: Noc1: -104.1dBm/15kHz Noc2: -100dBm/15kHz Ês1/Noc1: +10.0dB Ês2/Noc2: -2dB Normal condition: RSRP_42 to RSRP_101 Test 2: n257 Ês1: -102.45dBm/240kHz n259 Ês1: -98.85dBm/240kHz n260 Ês1: -99.85dBm/240kHz n257 Ês2: -99.45dBm/240kHz n259 Ês2: -95.85dBm/240kHz n260 Ês2: -96.85dBm/240kHz Normal condition: n257: RSRP_35 to RSRP_91 n259: RSRP_39 to RSRP_95 n260: RSRP_38 to RSRP_94 |

## <<< END OF CHANGES 2 >>>