3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210793

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4631 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE testcase 9.2.1.1.7a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-07-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to 36.523-1 Table 9.2.1.1.7a.3.3-1 at Step 8 in the Attach ACCEPT message the SS should include the PLMN2, PLMN3 and PLMN4 in the Equivalent PLMN list. The current TTCN sets the IE Equivalent PLMN list to OMIT. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added Equivalent PLMNs in ATTACH ACCEPT msg in step 8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN not in line with Prose. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.1.1.7a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Corrections required

## 1.1 Change 1

| Function name | function fl_TC_9_2_1_1_7a_Body() |
| --- | --- |
| Reason for change | According to 36.523-1 Table 9.2.1.1.7a.3.3-1 at Step 8 in the Attach ACCEPT message the SS should include the PLMN2, PLMN3 and PLMN4 in the Equivalent PLMN list. The current TTCN sets the IE Equivalent PLMN list to OMIT. This needs to be addressed. |
| Summary of change | Added Equivalent PLMNs in ATTACH ACCEPT msg in step 8. |
| TTCN module | ATS_LTE\ 9_2\ NAS_Attach_EOnly.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_9_2_1_1_7a_Body() runs on EUTRA_PTC

{

…

//@siclog "Step 3-9Ib1" siclog@

//+  Steps 2-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed.

f_EUTRA_InitialRegistration(eutra_CellA, PREAMBLE, STATE2_IDLEUPDATE, EPS_Only); // @sic R5-160741, R5s160273, R5s160752 sic@

//@siclog "Steps 10 - 10a" siclog@

//+  The UE is switched off.

f_EUTRA_UE_Detach_SwitchOffUe(eutra_CellA, RRC_IDLE, EPS_Only, - , true); // @sic R5s120597r1 sic@ @sic R5s160273 R5s160376 sic@

…

}

After change

function fl_TC_9_2_1_1_7a_Body() runs on EUTRA_PTC

{

…

//@siclog "Step 3-9Ib1" siclog@

//+  Steps 2-18b1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed.

f_EUTRA_InitialRegistration(eutra_CellA, PREAMBLE, STATE2_IDLEUPDATE, EPS_Only,0,

cs_PLMN_List_2PLMNs(f_Asn2Nas_PlmnId(f_EUTRA_CellInfo_GetGutiPLMN(eutra_CellB)),

f_Asn2Nas_PlmnId(f_EUTRA_CellInfo_GetGutiPLMN(eutra_CellC)))); // @sic R5-160741, R5s160273, R5s160752 sic@

//@siclog "Steps 10 - 10a" siclog@

//+  The UE is switched off.

f_EUTRA_UE_Detach_SwitchOffUe(eutra_CellA, RRC_IDLE, EPS_Only, - , true); // @sic R5s120597r1 sic@ @sic R5s160273 R5s160376 sic@

…

}