3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250464

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4942 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE testcase 7.1.3.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | DRX has been configured on cell1. Once the UE switches to cell2, the DRX configuration remains valid. In step 8, for a UE supporting pc_FeatrGrp_5_F or pc_FeatrGrp_5_T, the MAC PDU should be sent within the "On Duration" of the DRX cycle, according to the DRX configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In step 8, based on the configuration of CDRX, the transmission time of the MAC PDU is calculated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1 Change 1	2

2.1 Change 2	13

# Overview

This document lists all the essential changes needed to correct problems in the iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact: 	Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

# Corrections required

## 2.1 Change 1

| Function name | f_TC_7_1_3_9_EUTRA |
| --- | --- |
| Reason for change | DRX has been configured on cell1. Once the UE switches to cell2, the DRX configuration remains valid. In step 8, for a UE supporting pc_FeatrGrp_5_F or pc_FeatrGrp_5_T, the MAC PDU should be sent within the "On Duration" of the DRX cycle, according to the DRX configuration |
| Summary of change | In step 8, based on the configuration of DRX, the transmission time of the MAC PDU is calculated. |
| TTCN module | LTE\7_1\MAC_713.ttcn |
| MCC160 Comments |  |

Before Change:

| /* * @desc REFERENCE TS 36.523-1 clause 7.1.3.9 * @status APPROVED (LTE) */ function f_TC_7_1_3_9_EUTRA ( ) runs on EUTRA_PTC { /* L2/MAC * 7.1.3.9 : MAC reset-DL */ const HarqProcessId_Type tsc_HARQ_Process := 3; // selected the highest allowed HARQ process in TDD config 0 var SubFrameTiming_Type v_Timing; var SubFrameTiming_Type v_Timing2; var SRB_COMMON_IND v_SRB_COMMON_IND; var SYSTEM_IND v_ReceivedSYSIND; var float v_HarqIndWindowSize; var EUTRA_SecurityParams_Type v_Auth_Params; //@sic R5s110576 sic@ var template (value) RadioBearerList_Type v_DRB :={cs_OneDRB_ConfigAM(tsc_DRB1, cds_PDCP_Config_DRB_AM_NoPDCP_StatusReport, //@sic R5s110687 sic@ cs_RLC_Configuration_AM(cds_TX_AM_RLC_t_PollRetransmit(ms250)))}; var template (value) DRB_ToAddModList v_DRB_ToAddModList := {cds_508_DRB_ToAddMod_Common(tsc_DRB1, cds_PDCP_Config_DRB_AM_NoPDCP_StatusReport,//@sic R5s110687 sic@ cs_508_RLC_Config_DRB_AM(cds_TX_AM_RLC_t_PollRetransmit(ms250)), cs_508_LogicalChannelConfig_DRB_AM)}; var octetstring v_EncodedPdcpPdu; var octetstring v_EncodedRlcPdu; //var SubFrameNumber_Type v_SubFrameRRCReconfigAck := 8; //@sic R5s170939 R5s210064 sic@ var boolean v_Step3NackRcvd := false; var template (omit) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs := omit; //@sic R5s190055 R5-192717 sic@ var Dl_Bandwidth_Type v_DL_Bandwidth; //@sic R5-253974 sic@ timer t_Watchdog:=5.0; // General purpose wait timer /////////////////////////////////////////////////////////////////////////////////// // PREAMBLE ////////////////////////////////////////////////////////////////////////////////// f_EUTRA_Init (c1); // Config Cell 1 for 1 AM DRB f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_SetCellPower(eutra_Cell1, -85); //@sic R5s100456, cell config moved above set attenuation sic@ // bring up cell 2 with T0 setting f_EUTRA_CellInfo_InitMaxReferencePower(eutra_Cell2, -79); f_EUTRA_CellConfig_Def (eutra_Cell2); f_SS_ConfigDLHARQProcess_Subset(eutra_Cell1, -, cs_HarqProcessList_012);// To allow only first 3 HARQ processes for autonomous SS selection f_SS_ConfigDLHARQProcess_Subset(eutra_Cell2, -, cs_HarqProcessList_012);// To allow only first 3 HARQ processes for autonomous SS selection // Bring UE to state 4 f_EUTRA_Preamble ( eutra_Cell1,STATE2A_TESTLOOP_ModeA ); f_EUTRA_AS_CipheringAlgorithm_Set(eea0); // set null AS ciphering algorithm //@sic R5s110576 sic@ f_EUTRA_SS_CommonRadioBearerConfig(eutra_Cell1, v_DRB); f_EUTRA_LoopBackActivation_State4_TA_Infinity_LB_NoScaling(false, v_DRB_ToAddModList); // p_PDCP_StatusReportRequired is false f_Delay(0.660); // SS is not stopped for TA transmission to prevent UE from entering DRX f_SS_ConfigUL_HARQ_IndMode (eutra_Cell2, tsc_L1Mac_IndicationMode_Enable); // preconfig for RACH procedure f_EUTRA_SS_ConfigRachProcedure_Def(eutra_Cell2, tsc_C_RNTI_Def2); /* @sic acc. to R5s100029 change 3 R5s120629 sic@ */ f_SS_Config1TM_DRB_NoMACHeader(eutra_Cell1, tsc_RbId_DRB1, cs_MAC_TestMode_NoHeaderManipulationDLOnly); //@sic R5s150413 sic@ f_EUTRA_SS_CommonRadioBearerConfig_SingleDRB(eutra_Cell2, cs_OneDRB_ConfigAM); v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA_CellInfo_GetReconfig_BR(eutra_Cell2); //@sic R5s190055 R5-192717 sic@ v_DL_Bandwidth := f_EUTRA_CellInfo_GetDL_ChBandwidth(eutra_Cell1); //@sic R5-253974 sic@ /////////////////////////////////////////////////////////////////////////////////// // Test Body ////////////////////////////////////////////////////////////////////////////////// f_EUTRA_TestBody_Set(true, layer2); // @sic R5-101050 sic@ //@siclog "Step 1" siclog@ // To stop UL Grant Transmision //@sic R5s110201 sic@ f_EUTRA_StartULGrantTransmissionOnSR_Reception_No_PUCCH_Sync(eutra_Cell1,cs_TimingInfo_Now, cs_DciInfo_CcchDcchDtchUL_Def); //@sic R5s100852r2 MCC change sic@ // to configure SS to report reception of HARQ ACK or NACK f_SS_ConfigUL_HARQ_IndMode (eutra_Cell1, tsc_L1Mac_IndicationMode_Enable); // 38 bytes PDCP SDU +2 byte PDCP header // 2 bytes RLC Header +2 byte MAC Header =352 bits, nearest TB size =376 => padding =24 bits // 1 byte padding header and 2 bits padding //@siclog "Step 1" siclog@ // Sequence is changed to facilitate SS implementations. f_EUTRA_SetCellPower (eutra_Cell2, -79); v_Timing := f_EUTRA_GetNextSendOccasion(eutra_Cell1, 400);// schedule 400 ms in future // set CRC error as error mode f_SS_ConfigCRC_ErrorMode(eutra_Cell1, cs_DL_SCH_CRC_C_RNTI_Error(Erroneous), cs_TimingInfo_EUTRA_NB(v_Timing)); v_Timing.Subframe.Number := 0; // suitable for both FDD and TDD v_Timing := f_EUTRA_TimingInfoAdd(v_Timing, 30, v_DL_Bandwidth);// schedule 20-30 ms in future //@siclog "Step 2" siclog@ //@sic R5s120629 sic@ v_EncodedPdcpPdu := f_PDCP_PDU_Encvalue (cs_PDCP_PDU_LongSN(cs_PDCP_DataPdu_LongSN(tsc_B12_SQN0, crs_PDCP_SDU_L36))); //@sic R5s180176 sic@ v_EncodedRlcPdu := f_RLC_AMD_1PDU_Encvalue(0, tsc_NoPoll, v_EncodedPdcpPdu); DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU (eutra_Cell1, tsc_RbId_DRB1, cs_TimingInfo_EUTRA_NB(v_Timing), //cs_TimingInfo_Now, cs_MAC_PDUList_1 (cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1, v_EncodedRlcPdu)), cs_HarqProcessAssignment_Specific(tsc_HARQ_Process))); // set CRC error as normal mode 60 ms after the scheduled transmission // so that 5 transmissions are performed. v_Timing := f_EUTRA_TimingInfoAdd(v_Timing, 60, v_DL_Bandwidth); f_SS_ConfigCRC_ErrorMode(eutra_Cell1, cs_DL_SCH_CRC_C_RNTI_Error(Normal), cs_TimingInfo_EUTRA_NB( v_Timing )); //@sic R5s120629 sic@ f_SS_Config1TM_DRB_NoMACHeader(eutra_Cell1, tsc_RbId_DRB1,cs_MAC_TestMode_NormalMode,-,cs_TimingInfo_EUTRA_NB( v_Timing )); //@siclog "Step 5" siclog@ // Tell the SS about the HO f_EUTRA_SS_PdcpHandoverCtrl(eutra_Cell2, cas_PdcpHandoverInit_REQ(eutra_Cell1, eutra_Cell2)); // @sic R5s100556 sic@ v_Auth_Params := f_EUTRA_Security_Get(); v_Auth_Params := f_EUTRA_SS_AS_ActivateSecurity_HO_Reest(eutra_Cell2, v_Auth_Params, 0); f_EUTRA_Security_Set ( v_Auth_Params ); f_EUTRA_SS_ConfigRachProcedure_HO(eutra_Cell2, cs_TimingInfo_Now, tsc_C_RNTI_Def2); f_EUTRA_CellInfo_SetC_RNTI (eutra_Cell2, tsc_C_RNTI_Def2); // stop periodic TA on source cell f_EUTRA_SS_CommonCellConfig(eutra_Cell1, cas_PUCCH_Synch_Config_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_UplinkTimeAlignment_Synch_None)); // Note: since the UE is configured for TA infinitive it does not matter that SS reconfiguration is not scheduled //@sic R5s150413 sic@ f_SS_Config1TM_DRB_NoMACHeader(eutra_Cell2, tsc_RbId_DRB1, cs_MAC_TestMode_NoHeaderManipulationDLOnly); // configure UL grant configuration on target cell @sic R5s100515 sic@ f_EUTRA_ULGrantConfiguration_Common(eutra_Cell2, cs_TimingInfo_Now, cs_DciInfo_CcchDcchDtchUL_Def, cs_UplinkTimeAlignment_Synch_None, cs_UL_GrantConfig_OnSR); v_Timing2 := f_EUTRA_TimingInfoAdd(v_Timing, 10, v_DL_Bandwidth); //@sic R5s100240 sic@ //@sic R5s190055 R5-192717 sic@ f_EUTRA_508RRC_ConnReconfigHO_IntraLTE (eutra_Cell1, eutra_Cell2, cs_TimingInfo_SubFrameOffset( v_Timing2.SFN.Number, 4, v_Timing2.HSFN.Number ), v_RRCConnectionReconfiguration_v890_IEs); // next sub frame suitable for both FDD and TDD @sic R5w160204r1 sic@ /* if (pc_ue_CategoryDL_M1) //@sic R5s170939 R5s180176 r5s180381 R5s210064 sic@ { v_SubFrameRRCReconfigAck := 0; v_Timing2.HSFN.Number:= (v_Timing2.HSFN.Number + ((v_Timing2.SFN.Number) + 1)/ 1024) mod 1024; v_Timing2.SFN.Number := ((v_Timing2.SFN.Number) + 1) mod 1024; } */ //@siclog "Step 3" siclog@ t_Watchdog.start; SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell1, ?, nack )); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 3"); t_Watchdog.stop; //@siclog "Step 6" siclog@ // Check: does the UE transmit a RRCConnectionReconfigurationComplete message to confirm the reconfiguration of the radio resources? t_Watchdog.start; alt { [] SRB.receive ( car_SRB1_RrcPdu_IND ( eutra_Cell2, cr_508_RRCConnectionReconfigurationComplete ( tsc_RRC_TI_Def ) ) ) -> value v_SRB_COMMON_IND { v_Timing := v_SRB_COMMON_IND.Common.TimingInfo.SubFrame; //store TimeStamp when RRCConnectionReconfiguration has been received f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6"); t_Watchdog.stop; } [] SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell1, ?, nack )) {//@sic R5s210064 sic@ v_Step3NackRcvd := true; repeat; // any NACKS received } //@sic R5s170939 R5s210064 sic@ [v_Step3NackRcvd] SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell1, ?, ack )) // //note: retransmissions not allowed in this test case { repeat; // ACK received for RRC connection reconfiguration in step 5 } }; //@siclog "Step 7" siclog@ t_Watchdog.start (0.5); // watch dog timer to check 100ms as per prose which is less than 500ms, actual 100ms is checked based on time stamps alt { [] SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell1, ?, nack )) -> value v_ReceivedSYSIND { //Check if the received HARQ Ind is within a window of 100 ms v_Timing2 := v_ReceivedSYSIND.Common.TimingInfo.SubFrame; v_HarqIndWindowSize := f_EUTRA_SetTimerToleranceMin(eutra_Cell1, l2Timer, 0.1); //100 ms to wait safely no SR Rx if (f_SubFrameTimingDuration_Float(v_Timing, v_Timing2) < v_HarqIndWindowSize) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 7: UE is sending HARQ NACK in cell 1"); } } [] SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell2, ?, nack )) -> value v_ReceivedSYSIND { //Check if the received HARQ Ind is within a window of 100 ms v_Timing2 := v_ReceivedSYSIND.Common.TimingInfo.SubFrame; v_HarqIndWindowSize := f_EUTRA_SetTimerToleranceMin(eutra_Cell1, l2Timer, 0.1); //100 ms to wait safely no SR Rx if (f_SubFrameTimingDuration_Float(v_Timing, v_Timing2) < v_HarqIndWindowSize) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 7: UE is sending HARQ NACK in cell 2"); } } [] SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell2, ?, ack )) // HARQ ACK for Status PDU sent by SS RLC for RLC reconfiguration complete { repeat; } [] t_Watchdog.timeout { } } // disable harq reporting in cell 2 f_SS_ConfigUL_HARQ_IndMode (eutra_Cell2, tsc_L1Mac_IndicationMode_Disable); // default grant config f_EUTRA_StartDefULGrantTransmission(eutra_Cell2); f_EUTRA_SS_PdcpHandoverCtrl(eutra_Cell2, cas_PdcpHandoverComplete_REQ(eutra_Cell2)); // @sic R5s100556, R5s100560 sic@ // configure RACH procedure for target cell back to default @sic R5s100520 change 11 sic@ f_EUTRA_SS_ConfigRachProcedure_Def(eutra_Cell2); /* @sic acc. to R5s100029 change 3 sic@ */ //@sic R5s150992 sic@ f_EUTRA_SS_SRBs_DRBs_Reset(eutra_Cell1); // to enable reception of SR in cell 2 //@sic R5s110456 sic@ f_SS_ConfigSchedulingRequestIndMode(eutra_Cell2, tsc_L1Mac_IndicationMode_Enable); //@siclog "Step 8" siclog@ // SS guarantees NDI to be same as step2, as //"The initial value of the NDI shall be the same for all HARQ processes and cells". //@sic R5s120629 sic@ v_EncodedPdcpPdu := f_PDCP_PDU_Encvalue (cs_PDCP_PDU_LongSN(cs_PDCP_DataPdu_LongSN(tsc_B12_SQN0, crs_PDCP_SDU_L36_1))); v_EncodedRlcPdu := f_RLC_AMD_1PDU_Encvalue(0, tsc_NoPoll, v_EncodedPdcpPdu); DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU (eutra_Cell2, tsc_RbId_DRB1, cs_TimingInfo_Now, cs_MAC_PDUList_1 (cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1, v_EncodedRlcPdu)), cs_HarqProcessAssignment_Specific(tsc_HARQ_Process))); //@siclog "Step 9" siclog@ t_Watchdog.start; SYSIND.receive ( car_SchedulingReq_IND (eutra_Cell2, ?)); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 9: UE sent SR in cell 2"); t_Watchdog.stop; //@siclog "Step 10" siclog@ // Automatically taken care of //@siclog "Step 11" siclog@ t_Watchdog.start; //@sic R5s120629 sic@ DRB.receive (car_DRB_COMMON_IND_RLC_PDUList (eutra_Cell2, tsc_RbId_DRB1, cr_TimingInfo_Any, {cr_RLC_AMD_1PDU(?, int2bit(0, 10), v_EncodedPdcpPdu)})); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 11"); t_Watchdog.stop; f_RLC_Status_PDU_Tx (tsc_LchId_DRB1, 1, eutra_Cell2);//@sic R5s120629 sic@ // @sic R5s240367 sic@ f_DelayForDLPDU_Tx() ; //Delay of 100 ms for RLC status PDU transmission f_EUTRA_TestBody_Set(false); //test body ends /////////////////////////////////////////////////////////////////////////////////// // Post Amble ////////////////////////////////////////////////////////////////////////////////// // to configure SS to stop report reception of HARQ ACK or NACK f_SS_ConfigUL_HARQ_IndMode (eutra_Cell1,tsc_L1Mac_IndicationMode_Disable ); // to disable reception of SR in cell 2 f_SS_ConfigSchedulingRequestIndMode(eutra_Cell2, tsc_L1Mac_IndicationMode_Disable); f_SS_Config1TM_DRB_NoMACHeader(eutra_Cell2, tsc_RbId_DRB1,cs_MAC_TestMode_NormalMode); // To deactivate test loop mode before RRC release //@sic R5-103847 Deactivate test loop and open test loop is removed sic@ // Release RRC Connection and Switch off f_EUTRA_Postamble (eutra_Cell2, E3_TESTMODE); //@sic R5-103875 sic@ //Remove the cell1, non camping //@sic R5-103847 taken care in postamble now sic@ }; // end of 7.1.3.9 |  |
| --- | --- |

After Change:

| /* * @desc REFERENCE TS 36.523-1 clause 7.1.3.9 * @status APPROVED (LTE) */ function f_TC_7_1_3_9_EUTRA ( ) runs on EUTRA_PTC { /* L2/MAC * 7.1.3.9 : MAC reset-DL */ const HarqProcessId_Type tsc_HARQ_Process := 3; // selected the highest allowed HARQ process in TDD config 0 const integer tsc_LongDRXCycle := 1280; const integer tsc_DrxStartOffset := 4; const integer tsc_OnDurationTimer := 6; var SubFrameTiming_Type v_Timing; var SubFrameTiming_Type v_Timing2; var SRB_COMMON_IND v_SRB_COMMON_IND; var SYSTEM_IND v_ReceivedSYSIND; var float v_HarqIndWindowSize; var EUTRA_SecurityParams_Type v_Auth_Params; var EUTRA_FDD_TDD_Mode_Type v_EUTRA_FDD_TDD_Mode; //@sic R5s110576 sic@ var template (value) RadioBearerList_Type v_DRB :={cs_OneDRB_ConfigAM(tsc_DRB1, cds_PDCP_Config_DRB_AM_NoPDCP_StatusReport, //@sic R5s110687 sic@ cs_RLC_Configuration_AM(cds_TX_AM_RLC_t_PollRetransmit(ms250)))}; var template (value) DRB_ToAddModList v_DRB_ToAddModList := {cds_508_DRB_ToAddMod_Common(tsc_DRB1, cds_PDCP_Config_DRB_AM_NoPDCP_StatusReport,//@sic R5s110687 sic@ cs_508_RLC_Config_DRB_AM(cds_TX_AM_RLC_t_PollRetransmit(ms250)), cs_508_LogicalChannelConfig_DRB_AM)}; var octetstring v_EncodedPdcpPdu; var octetstring v_EncodedRlcPdu; //var SubFrameNumber_Type v_SubFrameRRCReconfigAck := 8; //@sic R5s170939 R5s210064 sic@ var boolean v_Step3NackRcvd := false; var template (omit) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs := omit; //@sic R5s190055 R5-192717 sic@ var Dl_Bandwidth_Type v_DL_Bandwidth; //@sic R5-253974 sic@ timer t_Watchdog:=5.0; // General purpose wait timer /////////////////////////////////////////////////////////////////////////////////// // PREAMBLE ////////////////////////////////////////////////////////////////////////////////// f_EUTRA_Init (c1); // Config Cell 1 for 1 AM DRB f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_SetCellPower(eutra_Cell1, -85); //@sic R5s100456, cell config moved above set attenuation sic@ // bring up cell 2 with T0 setting f_EUTRA_CellInfo_InitMaxReferencePower(eutra_Cell2, -79); f_EUTRA_CellConfig_Def (eutra_Cell2); f_SS_ConfigDLHARQProcess_Subset(eutra_Cell1, -, cs_HarqProcessList_012);// To allow only first 3 HARQ processes for autonomous SS selection f_SS_ConfigDLHARQProcess_Subset(eutra_Cell2, -, cs_HarqProcessList_012);// To allow only first 3 HARQ processes for autonomous SS selection // Bring UE to state 4 f_EUTRA_Preamble ( eutra_Cell1,STATE2A_TESTLOOP_ModeA ); f_EUTRA_AS_CipheringAlgorithm_Set(eea0); // set null AS ciphering algorithm //@sic R5s110576 sic@ f_EUTRA_SS_CommonRadioBearerConfig(eutra_Cell1, v_DRB); f_EUTRA_LoopBackActivation_State4_TA_Infinity_LB_NoScaling(false, v_DRB_ToAddModList); // p_PDCP_StatusReportRequired is false f_Delay(0.660); // SS is not stopped for TA transmission to prevent UE from entering DRX f_SS_ConfigUL_HARQ_IndMode (eutra_Cell2, tsc_L1Mac_IndicationMode_Enable); // preconfig for RACH procedure f_EUTRA_SS_ConfigRachProcedure_Def(eutra_Cell2, tsc_C_RNTI_Def2); /* @sic acc. to R5s100029 change 3 R5s120629 sic@ */ f_SS_Config1TM_DRB_NoMACHeader(eutra_Cell1, tsc_RbId_DRB1, cs_MAC_TestMode_NoHeaderManipulationDLOnly); //@sic R5s150413 sic@ f_EUTRA_SS_CommonRadioBearerConfig_SingleDRB(eutra_Cell2, cs_OneDRB_ConfigAM); v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA_CellInfo_GetReconfig_BR(eutra_Cell2); //@sic R5s190055 R5-192717 sic@ v_DL_Bandwidth := f_EUTRA_CellInfo_GetDL_ChBandwidth(eutra_Cell1); //@sic R5-253974 sic@ /////////////////////////////////////////////////////////////////////////////////// // Test Body ////////////////////////////////////////////////////////////////////////////////// f_EUTRA_TestBody_Set(true, layer2); // @sic R5-101050 sic@ //@siclog "Step 1" siclog@ // To stop UL Grant Transmision //@sic R5s110201 sic@ f_EUTRA_StartULGrantTransmissionOnSR_Reception_No_PUCCH_Sync(eutra_Cell1,cs_TimingInfo_Now, cs_DciInfo_CcchDcchDtchUL_Def); //@sic R5s100852r2 MCC change sic@ // to configure SS to report reception of HARQ ACK or NACK f_SS_ConfigUL_HARQ_IndMode (eutra_Cell1, tsc_L1Mac_IndicationMode_Enable); // 38 bytes PDCP SDU +2 byte PDCP header // 2 bytes RLC Header +2 byte MAC Header =352 bits, nearest TB size =376 => padding =24 bits // 1 byte padding header and 2 bits padding //@siclog "Step 1" siclog@ // Sequence is changed to facilitate SS implementations. f_EUTRA_SetCellPower (eutra_Cell2, -79); v_Timing := f_EUTRA_GetNextSendOccasion(eutra_Cell1, 400);// schedule 400 ms in future // set CRC error as error mode f_SS_ConfigCRC_ErrorMode(eutra_Cell1, cs_DL_SCH_CRC_C_RNTI_Error(Erroneous), cs_TimingInfo_EUTRA_NB(v_Timing)); v_Timing.Subframe.Number := 0; // suitable for both FDD and TDD v_Timing := f_EUTRA_TimingInfoAdd(v_Timing, 30, v_DL_Bandwidth);// schedule 20-30 ms in future //@siclog "Step 2" siclog@ //@sic R5s120629 sic@ v_EncodedPdcpPdu := f_PDCP_PDU_Encvalue (cs_PDCP_PDU_LongSN(cs_PDCP_DataPdu_LongSN(tsc_B12_SQN0, crs_PDCP_SDU_L36))); //@sic R5s180176 sic@ v_EncodedRlcPdu := f_RLC_AMD_1PDU_Encvalue(0, tsc_NoPoll, v_EncodedPdcpPdu); DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU (eutra_Cell1, tsc_RbId_DRB1, cs_TimingInfo_EUTRA_NB(v_Timing), //cs_TimingInfo_Now, cs_MAC_PDUList_1 (cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1, v_EncodedRlcPdu)), cs_HarqProcessAssignment_Specific(tsc_HARQ_Process))); // set CRC error as normal mode 60 ms after the scheduled transmission // so that 5 transmissions are performed. v_Timing := f_EUTRA_TimingInfoAdd(v_Timing, 60, v_DL_Bandwidth); f_SS_ConfigCRC_ErrorMode(eutra_Cell1, cs_DL_SCH_CRC_C_RNTI_Error(Normal), cs_TimingInfo_EUTRA_NB( v_Timing )); //@sic R5s120629 sic@ f_SS_Config1TM_DRB_NoMACHeader(eutra_Cell1, tsc_RbId_DRB1,cs_MAC_TestMode_NormalMode,-,cs_TimingInfo_EUTRA_NB( v_Timing )); //@siclog "Step 5" siclog@ // Tell the SS about the HO f_EUTRA_SS_PdcpHandoverCtrl(eutra_Cell2, cas_PdcpHandoverInit_REQ(eutra_Cell1, eutra_Cell2)); // @sic R5s100556 sic@ v_Auth_Params := f_EUTRA_Security_Get(); v_Auth_Params := f_EUTRA_SS_AS_ActivateSecurity_HO_Reest(eutra_Cell2, v_Auth_Params, 0); f_EUTRA_Security_Set ( v_Auth_Params ); f_EUTRA_SS_ConfigRachProcedure_HO(eutra_Cell2, cs_TimingInfo_Now, tsc_C_RNTI_Def2); f_EUTRA_CellInfo_SetC_RNTI (eutra_Cell2, tsc_C_RNTI_Def2); // stop periodic TA on source cell f_EUTRA_SS_CommonCellConfig(eutra_Cell1, cas_PUCCH_Synch_Config_REQ(eutra_Cell1, cs_TimingInfo_Now, cs_UplinkTimeAlignment_Synch_None)); // Note: since the UE is configured for TA infinitive it does not matter that SS reconfiguration is not scheduled //@sic R5s150413 sic@ f_SS_Config1TM_DRB_NoMACHeader(eutra_Cell2, tsc_RbId_DRB1, cs_MAC_TestMode_NoHeaderManipulationDLOnly); // configure UL grant configuration on target cell @sic R5s100515 sic@ f_EUTRA_ULGrantConfiguration_Common(eutra_Cell2, cs_TimingInfo_Now, cs_DciInfo_CcchDcchDtchUL_Def, cs_UplinkTimeAlignment_Synch_None, cs_UL_GrantConfig_OnSR); v_Timing2 := f_EUTRA_TimingInfoAdd(v_Timing, 10, v_DL_Bandwidth); //@sic R5s100240 sic@ //@sic R5s190055 R5-192717 sic@ f_EUTRA_508RRC_ConnReconfigHO_IntraLTE (eutra_Cell1, eutra_Cell2, cs_TimingInfo_SubFrameOffset( v_Timing2.SFN.Number, 4, v_Timing2.HSFN.Number ), v_RRCConnectionReconfiguration_v890_IEs); // next sub frame suitable for both FDD and TDD @sic R5w160204r1 sic@ /* if (pc_ue_CategoryDL_M1) //@sic R5s170939 R5s180176 r5s180381 R5s210064 sic@ { v_SubFrameRRCReconfigAck := 0; v_Timing2.HSFN.Number:= (v_Timing2.HSFN.Number + ((v_Timing2.SFN.Number) + 1)/ 1024) mod 1024; v_Timing2.SFN.Number := ((v_Timing2.SFN.Number) + 1) mod 1024; } */ //@siclog "Step 3" siclog@ t_Watchdog.start; SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell1, ?, nack )); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 3"); t_Watchdog.stop; //@siclog "Step 6" siclog@ // Check: does the UE transmit a RRCConnectionReconfigurationComplete message to confirm the reconfiguration of the radio resources? t_Watchdog.start; alt { [] SRB.receive ( car_SRB1_RrcPdu_IND ( eutra_Cell2, cr_508_RRCConnectionReconfigurationComplete ( tsc_RRC_TI_Def ) ) ) -> value v_SRB_COMMON_IND { v_Timing := v_SRB_COMMON_IND.Common.TimingInfo.SubFrame; //store TimeStamp when RRCConnectionReconfiguration has been received f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6"); t_Watchdog.stop; } [] SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell1, ?, nack )) {//@sic R5s210064 sic@ v_Step3NackRcvd := true; repeat; // any NACKS received } //@sic R5s170939 R5s210064 sic@ [v_Step3NackRcvd] SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell1, ?, ack )) // //note: retransmissions not allowed in this test case { repeat; // ACK received for RRC connection reconfiguration in step 5 } }; //@siclog "Step 7" siclog@ t_Watchdog.start (0.5); // watch dog timer to check 100ms as per prose which is less than 500ms, actual 100ms is checked based on time stamps alt { [] SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell1, ?, nack )) -> value v_ReceivedSYSIND { //Check if the received HARQ Ind is within a window of 100 ms v_Timing2 := v_ReceivedSYSIND.Common.TimingInfo.SubFrame; v_HarqIndWindowSize := f_EUTRA_SetTimerToleranceMin(eutra_Cell1, l2Timer, 0.1); //100 ms to wait safely no SR Rx if (f_SubFrameTimingDuration_Float(v_Timing, v_Timing2) < v_HarqIndWindowSize) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 7: UE is sending HARQ NACK in cell 1"); } } [] SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell2, ?, nack )) -> value v_ReceivedSYSIND { //Check if the received HARQ Ind is within a window of 100 ms v_Timing2 := v_ReceivedSYSIND.Common.TimingInfo.SubFrame; v_HarqIndWindowSize := f_EUTRA_SetTimerToleranceMin(eutra_Cell1, l2Timer, 0.1); //100 ms to wait safely no SR Rx if (f_SubFrameTimingDuration_Float(v_Timing, v_Timing2) < v_HarqIndWindowSize) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 7: UE is sending HARQ NACK in cell 2"); } } [] SYSIND.receive ( car_UL_HARQ_IND (eutra_Cell2, ?, ack )) // HARQ ACK for Status PDU sent by SS RLC for RLC reconfiguration complete { repeat; } [] t_Watchdog.timeout { } } // disable harq reporting in cell 2 f_SS_ConfigUL_HARQ_IndMode (eutra_Cell2, tsc_L1Mac_IndicationMode_Disable); // default grant config f_EUTRA_StartDefULGrantTransmission(eutra_Cell2); f_EUTRA_SS_PdcpHandoverCtrl(eutra_Cell2, cas_PdcpHandoverComplete_REQ(eutra_Cell2)); // @sic R5s100556, R5s100560 sic@ // configure RACH procedure for target cell back to default @sic R5s100520 change 11 sic@ f_EUTRA_SS_ConfigRachProcedure_Def(eutra_Cell2); /* @sic acc. to R5s100029 change 3 sic@ */ //@sic R5s150992 sic@ f_EUTRA_SS_SRBs_DRBs_Reset(eutra_Cell1); // to enable reception of SR in cell 2 //@sic R5s110456 sic@ f_SS_ConfigSchedulingRequestIndMode(eutra_Cell2, tsc_L1Mac_IndicationMode_Enable); //@siclog "Step 8" siclog@ // SS guarantees NDI to be same as step2, as //"The initial value of the NDI shall be the same for all HARQ processes and cells". //@sic R5s120629 sic@ v_EncodedPdcpPdu := f_PDCP_PDU_Encvalue (cs_PDCP_PDU_LongSN(cs_PDCP_DataPdu_LongSN(tsc_B12_SQN0, crs_PDCP_SDU_L36_1))); v_EncodedRlcPdu := f_RLC_AMD_1PDU_Encvalue(0, tsc_NoPoll, v_EncodedPdcpPdu); if (pc_FeatrGrp_5_F or pc_FeatrGrp_5_T) { v_EUTRA_FDD_TDD_Mode := f_EUTRA_CellInfo_GetFDD_TDD(eutra_Cell2); v_Timing2 := f_GetNextSendOccasion_7_1_3_X_DRX(eutra_Cell2, v_EUTRA_FDD_TDD_Mode, tsc_LongDRXCycle, tsc_DrxStartOffset, tsc_OnDurationTimer - 1); DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU (eutra_Cell2, tsc_RbId_DRB1, cs_TimingInfo_EUTRA_NB(v_Timing2), cs_MAC_PDUList_1 (cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1, v_EncodedRlcPdu)), cs_HarqProcessAssignment_Specific(tsc_HARQ_Process))); } else { DRB.send (cas_DRB_COMMOM_REQ_MAC_PDU (eutra_Cell2, tsc_RbId_DRB1, cs_TimingInfo_Now, cs_MAC_PDUList_1 (cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1, v_EncodedRlcPdu)), cs_HarqProcessAssignment_Specific(tsc_HARQ_Process))); } //@siclog "Step 9" siclog@ t_Watchdog.start; SYSIND.receive ( car_SchedulingReq_IND (eutra_Cell2, ?)); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 9: UE sent SR in cell 2"); t_Watchdog.stop; //@siclog "Step 10" siclog@ // Automatically taken care of //@siclog "Step 11" siclog@ t_Watchdog.start; //@sic R5s120629 sic@ DRB.receive (car_DRB_COMMON_IND_RLC_PDUList (eutra_Cell2, tsc_RbId_DRB1, cr_TimingInfo_Any, {cr_RLC_AMD_1PDU(?, int2bit(0, 10), v_EncodedPdcpPdu)})); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 11"); t_Watchdog.stop; f_RLC_Status_PDU_Tx (tsc_LchId_DRB1, 1, eutra_Cell2);//@sic R5s120629 sic@ // @sic R5s240367 sic@ f_DelayForDLPDU_Tx() ; //Delay of 100 ms for RLC status PDU transmission f_EUTRA_TestBody_Set(false); //test body ends /////////////////////////////////////////////////////////////////////////////////// // Post Amble ////////////////////////////////////////////////////////////////////////////////// // to configure SS to stop report reception of HARQ ACK or NACK f_SS_ConfigUL_HARQ_IndMode (eutra_Cell1,tsc_L1Mac_IndicationMode_Disable ); // to disable reception of SR in cell 2 f_SS_ConfigSchedulingRequestIndMode(eutra_Cell2, tsc_L1Mac_IndicationMode_Disable); f_SS_Config1TM_DRB_NoMACHeader(eutra_Cell2, tsc_RbId_DRB1,cs_MAC_TestMode_NormalMode); // To deactivate test loop mode before RRC release //@sic R5-103847 Deactivate test loop and open test loop is removed sic@ // Release RRC Connection and Switch off f_EUTRA_Postamble (eutra_Cell2, E3_TESTMODE); //@sic R5-103875 sic@ //Remove the cell1, non camping //@sic R5-103847 taken care in postamble now sic@ }; // end of 7.1.3.9 |  |
| --- | --- |

## 2.1 Change 2

| Function name | Add import and function f_GetNextSendOccasion_7_1_3_X_DRX |
| --- | --- |
| Reason for change | Add “import from Parameters all” is for the use of pc_FeatrGrp_5_F and pc_FeatrGrp_5_T Add function f_GetNextSendOccasion_7_1_3_X_DRX calculate On Duration in the DRX cycle |
| Summary of change | Using the required variables, calculate the DRX time |
| TTCN module | LTE\7_1\MAC_713.ttcn |
| MCC160 Comments |  |

Before Change:

| import from EUTRA_CellInfo all; import from EUTRA_CellInfoInit all; import from EUTRA_ConfigurationSteps all; import from EUTRA_RRCSteps all; import from EUTRA_CommonProcedures all; import from EUTRA_LoopBack all; import from EPS_NAS_Templates all; import from NAS_CommonTemplates all; import from EUTRA_ASP_TypeDefs all; import from EUTRA_AspCommon_Templates all; import from EUTRA_CommonDefs all; import from EUTRA_NB_CommonDefs all; import from EUTRA_NB_CommonL2 all; import from EUTRA_NB_CommonMAC all; import from EUTRA_NB_ASP_L2DataDefs all; import from EUTRA_NB_ASP_Templates all; import from EUTRA_CellCfg_Templates all; import from EUTRA_RRC_ASN1_Definitions language "ASN.1:2002" all with {encode "UNALIGNED_PER_OctetAligned"}; import from EUTRA_RRC_Templates all; import from EUTRA_ASP_SrbDefs all; import from EUTRA_SRB_Templates all; import from EUTRA_Component all; import from EUTRA_Paging all; import from EUTRA_L2_CommonTemplates all; import from EUTRA_L2_CommonFunctions all; import from EUTRA_MAC_Common all; import from MAC_Templates all; import from MAC_Functions all; import from EUTRA_SecuritySteps all; import from EUTRA_Timing all; import from CommonDefs all; import from CommonAspDefs all; import from Common4G5G_Templates all; import from EPS_NAS_MsgContainers all; import from EUTRA_SecurityFunctions all; import from EUTRA_DRB_Templates all; import from Common4G5G_Timing all; //========================================================================= // Test case main functions //========================================================================= |  |
| --- | --- |

After Change:

| import from EUTRA_CellInfo all; import from EUTRA_CellInfoInit all; import from EUTRA_ConfigurationSteps all; import from EUTRA_RRCSteps all; import from EUTRA_CommonProcedures all; import from EUTRA_LoopBack all; import from EPS_NAS_Templates all; import from NAS_CommonTemplates all; import from EUTRA_ASP_TypeDefs all; import from EUTRA_AspCommon_Templates all; import from EUTRA_CommonDefs all; import from EUTRA_NB_CommonDefs all; import from EUTRA_NB_CommonL2 all; import from EUTRA_NB_CommonMAC all; import from EUTRA_NB_ASP_L2DataDefs all; import from EUTRA_NB_ASP_Templates all; import from EUTRA_CellCfg_Templates all; import from EUTRA_RRC_ASN1_Definitions language "ASN.1:2002" all with {encode "UNALIGNED_PER_OctetAligned"}; import from EUTRA_RRC_Templates all; import from EUTRA_ASP_SrbDefs all; import from EUTRA_SRB_Templates all; import from EUTRA_Component all; import from EUTRA_Paging all; import from EUTRA_L2_CommonTemplates all; import from EUTRA_L2_CommonFunctions all; import from EUTRA_MAC_Common all; import from MAC_Templates all; import from MAC_Functions all; import from EUTRA_SecuritySteps all; import from EUTRA_Timing all; import from CommonDefs all; import from CommonAspDefs all; import from Common4G5G_Templates all; import from EPS_NAS_MsgContainers all; import from EUTRA_SecurityFunctions all; import from EUTRA_DRB_Templates all; import from Common4G5G_Timing all; import from Parameters all; //========================================================================= // Test case main functions //========================================================================= /* * @desc Evaluate a suitable timing info in future and lying in On-Duration of the cell * @param p_CellId * @param p_EUTRA_FDD_TDD_Mode * @param p_DrxCycle * @param p_DrxStartOffset * @param p_CountY * @return SubFrameTiming_Type * @status APPROVED (LTE) */ function f_GetNextSendOccasion_7_1_3_X_DRX(EUTRA_CellId_Type p_CellId, EUTRA_FDD_TDD_Mode_Type p_EUTRA_FDD_TDD_Mode, integer p_DrxCycle, integer p_DrxStartOffset, integer p_CountY) runs on EUTRA_PTC return SubFrameTiming_Type { var SubFrameTiming_Type v_SubFrameInfo := f_EUTRA_GetCurrentTiming(p_CellId); var integer v_DRXFrame := p_DrxCycle; var integer v_SubframeNumber; var integer v_FrameNumber; var integer v_HSFN; var integer v_NormalSF; var SubFrameTiming_Type v_Timing; var Dl_Bandwidth_Type v_DL_Bandwidth := f_EUTRA_CellInfo_GetDL_ChBandwidth(p_CellId); //@sic R5-253974 sic@ // 2 cycles ahead is selected to provide enough time for SS v_FrameNumber := (((v_SubFrameInfo.SFN.Number * 10 + v_SubFrameInfo.Subframe.Number) / v_DRXFrame) + 2) * v_DRXFrame; v_SubframeNumber := v_FrameNumber mod 10; v_FrameNumber := (v_FrameNumber/10) mod 1024; v_HSFN := (v_SubFrameInfo.HSFN.Number + ((v_FrameNumber/10)/1024)) mod 1024; // @sic R5w160204r1 sic@ //@sic R5s170922 sic@ v_Timing := valueof(cs_SubFrameTiming(v_FrameNumber, v_SubframeNumber, v_HSFN)); v_Timing := f_EUTRA_TimingInfoAdd(v_Timing, p_DrxStartOffset, v_DL_Bandwidth); v_NormalSF := f_DRX_NormalSF(v_Timing.Subframe.Number, p_CountY, p_EUTRA_FDD_TDD_Mode); //@sic R5s110297 R5s180010 sic@ return f_EUTRA_TimingInfoAdd(v_Timing, v_NormalSF, v_DL_Bandwidth); } |  |
| --- | --- |