3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250483

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3776 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Rel-17 NR NTN test case 6.7.2.2 in GSO mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-21 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add NR5GC test case 6.7.2.2 to the IWD_25wk38 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 6.7.2.2 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.508-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required to testcase 6.7.2.2	4

2.1	Change 1	5

3	Branches executed	7

4	Execution Log Files	8

4.1	Exynos 2500	8

5	References	9

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 6.7.2.2 which is part of the NR5GC test suite in iwd-TTCN3-B2024-06_D25wk38.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Narendra Kalahasti

Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	6.7.2.2.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D25wk38

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Samsung Exynos 2500

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_6_7_2_2_NR5GC () |
| --- | --- |
| Reason for change | - As per Table 6.7.2.2.3.3-3, need to add interFrequency cell information on SIB4 for neighbor cell. - T-service time calculation should be done at the same time when Configuration update command is about to be scheduled, so moved f_NR5GC_CellInfo_SetSIB19_t_Service_r17 to the test body for cell 1 and cell 11. For that f_UTC_TimestampSeconds also need to be moved from this function to testbody. |
| Summary of change | - Added f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_DL_CarrierFreq for cell 12 and cell11. - Deleted f_NR5GC_CellInfo_SetSIB19_t_Service_r17 and f_UTC_TimestampSeconds. |
| TTCN module | NTN_Inactive_NR5GC.ttcn |

Before Change:

function f_TC_6_7_2_2_NR5GC() runs on NR5GC_PTC {// NR NTN / Cell reselection, Sintrasearch, Snonintrasearch in RRC_Inactive state / time based measurements var integer v_TimestampSeconds; //NR Cell 11 and NR Cell 12 use System information combination NR-32 as defined in TS 38.508-1 [4] Table 4.4.3.1.2-1. f_NR5GC_Init(NR_32,-,{nr_Cell11,nr_Cell12}); //NR Cell 1 uses System information combination NR-29 as defined in TS 38.508-1 [4] Table 4.4.3.1.2-1. f_NR_ModifySysinfoCombinationToNR_29(nr_Cell1); //Update SIB2 as per Table 6.7.2.2.3.3-2: SIB2 of NR Cell 1, NR Cell 11 and NR Cell 12 (preamble and all steps, Table 6.7.2.2.3.2-2) f_NR_CellInfo_SetSIB2_S_NonIntraSearchP(nr_Cell1, 5); f_NR_CellInfo_SetSIB2_S_NonIntraSearchP(nr_Cell11, 5); f_NR_CellInfo_SetSIB2_S_NonIntraSearchP(nr_Cell12, 5); f_NR_CellInfo_SetSIB2_ThreshServingLowP(nr_Cell11,10); f_NR_CellInfo_SetSIB2_CellReselectionPriority(nr_Cell1, 5); f_NR_CellInfo_SetSIB2_CellReselectionPriority(nr_Cell11, 5); f_NR_CellInfo_SetSIB2_CellReselectionPriority(nr_Cell12, 1); f_NR_CellInfo_SetSIB2_S_IntraSearchP(nr_Cell1, 5); f_NR_CellInfo_SetSIB2_S_IntraSearchP(nr_Cell11, 5); f_NR_CellInfo_SetSIB2_S_IntraSearchP(nr_Cell12, 5); //Update SIB4 as per Table 6.7.2.2.3.3-3: SIB4 of NR Cell 11 and NR Cell 12 (preamble and all steps, Table 6.7.2.2.3.2-2) f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell11, 0, 1); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell12, 0, 5); //Update SIB19 as per Table 6.7.2.2.3.3-4: SIB19 of NR Cell 1 and NR Cell 11 (preamble and all steps, Table 6.7.2.2.3.2-2 v_TimestampSeconds := 100*f_UTC_TimestampSeconds(); // Note 1: Value is calculated in multiples of 10 ms after 00:00:00 on Gregorian calendar date 1 January, 1900 f_NR5GC_CellInfo_SetSIB19_t_Service_r17(nr_Cell1, v_TimestampSeconds + 360); //+6min f_NR5GC_CellInfo_SetSIB19_t_Service_r17(nr_Cell11,v_TimestampSeconds + 540); //+9min f_NR_CellInfo_SetSIB19_NTN_RefLocation_AndDistanceThresh(nr_Cell1,hex2oct('2273BA566355'H),20000); f_NR_CellInfo_SetSIB19_NTN_RefLocation_AndDistanceThresh(nr_Cell11,hex2oct('2273BA566355'H),20000); // Create and configure cells f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell11); f_NR_CellConfig_Def(nr_Cell12); // Preamble: The UE is in state 2N-A on NR Cell 1(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-2. f_NR5GC_Preamble(nr_Cell1, STATE_CONNECTED_3A); f_NR_TestBody_Set(true); f_TC_6_7_2_2_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell12, STATE_IDLE_1A); } //end of f_TC_6_7_2_2_NR5GC

After Change:

function f_TC_6_7_2_2_NR5GC() runs on NR5GC_PTC {// NR NTN / Cell reselection, Sintrasearch, Snonintrasearch in RRC_Inactive state / time based measurements var ARFCN_ValueNR v_SSB_Freq_Cell12; var ARFCN_ValueNR v_SSB_Freq_Cell1; //NR Cell 11 and NR Cell 12 use System information combination NR-32 as defined in TS 38.508-1 [4] Table 4.4.3.1.2-1. f_NR5GC_Init(NR_32,-,{nr_Cell11,nr_Cell12}); //NR Cell 1 uses System information combination NR-29 as defined in TS 38.508-1 [4] Table 4.4.3.1.2-1. f_NR_ModifySysinfoCombinationToNR_29(nr_Cell1); //Update SIB2 as per Table 6.7.2.2.3.3-2: SIB2 of NR Cell 1, NR Cell 11 and NR Cell 12 (preamble and all steps, Table 6.7.2.2.3.2-2) f_NR_CellInfo_SetSIB2_S_NonIntraSearchP(nr_Cell1, 5); f_NR_CellInfo_SetSIB2_S_NonIntraSearchP(nr_Cell11, 5); f_NR_CellInfo_SetSIB2_S_NonIntraSearchP(nr_Cell12, 5); f_NR_CellInfo_SetSIB2_ThreshServingLowP(nr_Cell11,10); f_NR_CellInfo_SetSIB2_CellReselectionPriority(nr_Cell1, 5); f_NR_CellInfo_SetSIB2_CellReselectionPriority(nr_Cell11, 5); f_NR_CellInfo_SetSIB2_CellReselectionPriority(nr_Cell12, 1); f_NR_CellInfo_SetSIB2_S_IntraSearchP(nr_Cell1, 5); f_NR_CellInfo_SetSIB2_S_IntraSearchP(nr_Cell11, 5); f_NR_CellInfo_SetSIB2_S_IntraSearchP(nr_Cell12, 5); v_SSB_Freq_Cell12 := f_NR_CellInfo_GetFrequencySSB(nr_Cell12); v_SSB_Freq_Cell1 := f_NR_CellInfo_GetFrequencySSB(nr_Cell1); //Update SIB4 as per Table 6.7.2.2.3.3-3: SIB4 of NR Cell 11 and NR Cell 12 (preamble and all steps, Table 6.7.2.2.3.2-2) f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell11, 0, 1); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell12, 0, 5); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_DL_CarrierFreq(nr_Cell11, 0, v_SSB_Freq_Cell12); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_DL_CarrierFreq(nr_Cell12, 0, v_SSB_Freq_Cell1); //Update SIB19 as per Table 6.7.2.2.3.3-4: SIB19 of NR Cell 1 and NR Cell 11 (preamble and all steps, Table 6.7.2.2.3.2-2 f_NR_CellInfo_SetSIB19_NTN_RefLocation_AndDistanceThresh(nr_Cell1,hex2oct('2273BA566355'H),20000); f_NR_CellInfo_SetSIB19_NTN_RefLocation_AndDistanceThresh(nr_Cell11,hex2oct('2273BA566355'H),20000); // Create and configure cells f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell11); f_NR_CellConfig_Def(nr_Cell12); // Preamble: The UE is in state 2N-A on NR Cell 1(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-2. f_NR5GC_Preamble(nr_Cell1, STATE_CONNECTED_3A); f_NR_TestBody_Set(true); f_TC_6_7_2_2_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell12, STATE_IDLE_1A); } //end of f_TC_6_7_2_2_NR5GC

## Change 2

| Function name | f_TC_6_7_2_2_NR5GC_TestBody () |
| --- | --- |
| Reason for change | - For this case Cell 11 and Cell 12 gets initialized as per 6.7.2.2.3.1 pre-condition - Timer t_WaitForRRCResume_Step4 and t_WaitForRRCResume_Step11 should be set in seconds instead minutes. - Current TTCN function f_BCD_TimestampWithTimezone is miscalculating timezone, so need to correct that. - As per chnage1.2 Need to update t-service-r17 just before configuration update command, so added f_UTC_TimestampSeconds and f_NR5GC_CellInfo_SetSIB19_t_Service_r17 for cell 1 and cell 11 on test body. Also corrected and added in 10 mili seconds for t-service-r17 - After Sib19 modification need to update SIBs. - At step7-8 and 14-16, after RRC resume, SRB2 will be resumed. |
| Summary of change | - Updated cell 11 and cell 12 from cell 2 and cell3. - Multiplied same with 60 to convert them in seconds. - Modified f_BCD_TimestampWithTimezone calculations on change3. - Added function f_UTC_TimestampSeconds and f_NR5GC_CellInfo_SetSIB19_t_Service_r17. Also updated added value in 10 ms, like 36000 and 54000. - Added f_NR_ModifySysinfo for cell1 and cell11. - Added tsc_NR_RbId_SRB2 on f_NR5GC_MobilityRegistration_Steps4_6 |
| TTCN module | NTN_Inactive_NR5GC.ttcn |

Before Change:

function f_TC_6_7_2_2_NR5GC_TestBody() runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; var H14_Type v_TimestampWithTimezone; var O1_Type v_Timezone; var O1_Type v_Year; var O1_Type v_Month; var O1_Type v_Day; var O1_Type v_Hour; var O1_Type v_Minute; var O1_Type v_Second; var B2_Type v_DaylightSavingTime := '00'B; var template (omit) NG_NAS_MSG_Indication_Type v_DedicatedNAS_Message; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; timer t_WaitForRRCResume_Step4 := 6.0; timer t_WaitForRRCResume_Step11 := 9.0; //Table 6.7.2.2.3.2-1: Time instances of cell power level and parameter changes //T0 -> Done in the preamble v_CellPowerList_AtT1:={// NR_Cell1-> -94, NR_Cell2-> -88 cs_NR_CellPower(nr_Cell1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE) }; v_CellPowerList_AtT2:={// NR_Cell1-> Off, NR_Cell2-> -94, NR_Cell3-> -88 cs_NR_CellPower(nr_Cell1,tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell2, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell3, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE) }; // Calculation of Time Zone parameters - it is done in a consecutive call to fx_GetSystemTime after calculation of v_TimestampSeconds. // As a tolerance is applied to steps 4 and 6 it is taken for granted v_TimestampSeconds and v_TimestampWithTimezone can be used v_TimestampWithTimezone := f_BCD_TimestampWithTimezone(); v_Year := hex2oct(substr(v_TimestampWithTimezone, 0, 2)); v_Month := hex2oct(substr(v_TimestampWithTimezone, 2, 2)); v_Day := hex2oct(substr(v_TimestampWithTimezone, 4, 2)); v_Hour := hex2oct(substr(v_TimestampWithTimezone, 6, 2)); v_Minute := hex2oct(substr(v_TimestampWithTimezone, 8, 2)); v_Second := hex2oct(substr(v_TimestampWithTimezone, 10, 2)); v_Timezone := hex2oct(substr(v_TimestampWithTimezone, 12, 2)); if (fl_IsDaylightSavingTime()){ v_DaylightSavingTime := '01'B; } //@siclog "Step 1" siclog@ //The SS transmits a CONFIGURATION UPDATE COMMAND message including Local time zone, Universal time and local time zone, Network daylight saving time information. (Note 1) SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB2, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_CONFIGURATION_UPDATE_COMMAND(-, -, -, -, -, -, -, cs_TimeZone(v_Timezone), cs_TimeZoneAndTime(v_Year, v_Month, v_Day, v_Hour, v_Minute, v_Second,v_Timezone), cs_DaylightSavingTime_tlv(v_DaylightSavingTime))))); t_WaitForRRCResume_Step4.start; t_WaitForRRCResume_Step11.start; //@siclog "Step 2" siclog@ //The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. f_NR_RRC_InactiveDef(nr_Cell1, ResumeNoSecurity, -, -, -, -, -, -, nr_Cell11); // @sic R5s250008 sic@ //@siclog "Step 3" siclog@ //The SS re-adjusts the SS/PBCH EPRE levels according to row "T1" in table 6.1.2.18a.3.2-1. f_NR_SetCellPowerList(v_CellPowerList_AtT1); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell11); //@siclog "Step 4" siclog@ //Check: Does the UE transmit an RRCResumeRequest message on NR Cell 11 within 6mins after step 1? alt { []SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, mo_Signalling))){ t_WaitForRRCResume_Step4.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } [] t_WaitForRRCResume_Step4.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } } //@siclog "Step 5" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell11); //@siclog "Step 6" siclog@ //The UE transmits an RRCResumeComplete message and a REGISTRATION REQUEST message indicating "mobility registration updating" is sent to update the registration of NR Cell11. v_DedicatedNAS_Message := f_NR_RRCResumeComplete_Def(nr_Cell11); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, valueof(v_DedicatedNAS_Message.Pdu), Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 7-8" siclog@ //Steps 4 to 5 of the generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 with condition MOBILITY are performed. //NOTE: PLMN1 and PLMN2 are listed as an Equivalent PLMN in the REGISTRATION ACCEPT message f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell11, v_GMM_MobilityInfo, omit, omit, noRrcConnectionRelease,-,-, cs_PLMN_List_1PLMN(f_NR_Asn2Nas_PlmnId(tsc_NR_HPLMN_002_11))); //Table 6.7.2.2.3.3-5: REGISTRATION ACCEPT // @siclog "Step 9" siclog@ // The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. f_NR_RRC_InactiveDef(nr_Cell11, ResumeNoSecurity, -, -, -, -, -, -, nr_Cell12); // @sic R5s250008 sic@ //@siclog "Step 10" siclog@ //TThe SS re-adjusts the SS/PBCH EPRE levels according to row "T2" in table 6.1.2.18a.3.2-1. f_NR_SetCellPowerList(v_CellPowerList_AtT2); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell12); //@siclog "Step 11" siclog@ //Check: Does the UE transmit an RRCResumeRequest message on NR Cell 12 within 9mins after step 1? alt { []SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, mo_Signalling))){ t_WaitForRRCResume_Step11.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); } [] t_WaitForRRCResume_Step11.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 11"); } } //@siclog "Step 12" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell12); //@siclog "Step 13" siclog@ //The UE transmits an RRCResumeComplete message and a REGISTRATION REQUEST message indicating "mobility registration updating" is sent to update the registration of NR Cell12. v_DedicatedNAS_Message := f_NR_RRCResumeComplete_Def(nr_Cell12); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, valueof(v_DedicatedNAS_Message.Pdu), Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } // @siclog "Step 14-16" siclog@ //Steps 4 to 5 of the generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 with condition MOBILITY are performed. //NOTE: PLMN1 and PLMN2 are listed as an Equivalent PLMN in the REGISTRATION ACCEPT message // The SS transmits an RRCRelease message f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell12, v_GMM_MobilityInfo, omit, omit, -,-,-, cs_PLMN_List_1PLMN(f_NR_Asn2Nas_PlmnId(tsc_NR_HPLMN_Def))); //Table 6.7.2.2.3.3-5: REGISTRATION ACCEPT } // end f_TC_6_7_2_2_NR5GC_TestBody

After Change:

function f_TC_6_7_2_2_NR5GC_TestBody() runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; var integer v_TimestampSeconds; var H14_Type v_TimestampWithTimezone; var O1_Type v_Timezone; var O1_Type v_Year; var O1_Type v_Month; var O1_Type v_Day; var O1_Type v_Hour; var O1_Type v_Minute; var O1_Type v_Second; var B2_Type v_DaylightSavingTime := '00'B; var template (omit) NG_NAS_MSG_Indication_Type v_DedicatedNAS_Message; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; timer t_WaitForRRCResume_Step4 := 6.0 * 60.0; timer t_WaitForRRCResume_Step11 := 9.0 * 60.0; //Table 6.7.2.2.3.2-1: Time instances of cell power level and parameter changes //T0 -> Done in the preamble v_CellPowerList_AtT1:={// NR_Cell1-> -94, NR_Cell2-> -88 cs_NR_CellPower(nr_Cell1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell11, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE) }; v_CellPowerList_AtT2:={// NR_Cell1-> Off, NR_Cell2-> -94, NR_Cell3-> -88 cs_NR_CellPower(nr_Cell1,tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell11, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE) }; // Calculation of Time Zone parameters - it is done in a consecutive call to fx_GetSystemTime after calculation of v_TimestampSeconds. // As a tolerance is applied to steps 4 and 6 it is taken for granted v_TimestampSeconds and v_TimestampWithTimezone can be used v_TimestampWithTimezone := f_BCD_TimestampWithTimezone(); v_Year := hex2oct(substr(v_TimestampWithTimezone, 0, 2)); v_Month := hex2oct(substr(v_TimestampWithTimezone, 2, 2)); v_Day := hex2oct(substr(v_TimestampWithTimezone, 4, 2)); v_Hour := hex2oct(substr(v_TimestampWithTimezone, 6, 2)); v_Minute := hex2oct(substr(v_TimestampWithTimezone, 8, 2)); v_Second := hex2oct(substr(v_TimestampWithTimezone, 10, 2)); v_Timezone := hex2oct(substr(v_TimestampWithTimezone, 12, 2)); if (fl_IsDaylightSavingTime()){ v_DaylightSavingTime := '01'B; } v_TimestampSeconds := 100*f_UTC_TimestampSeconds(); // Note 1: Value is calculated in multiples of 10 ms after 00:00:00 on Gregorian calendar date 1 January, 1900 f_NR5GC_CellInfo_SetSIB19_t_Service_r17(nr_Cell1, v_TimestampSeconds + 36000); f_NR5GC_CellInfo_SetSIB19_t_Service_r17(nr_Cell11,v_TimestampSeconds + 54000); f_NR_ModifySysinfo(nr_Cell1); f_NR_ModifySysinfo(nr_Cell11); //@siclog "Step 1" siclog@ //The SS transmits a CONFIGURATION UPDATE COMMAND message including Local time zone, Universal time and local time zone, Network daylight saving time information. (Note 1) SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB2, cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_CONFIGURATION_UPDATE_COMMAND(-, -, -, -, -, -, -, cs_TimeZone(v_Timezone), cs_TimeZoneAndTime(v_Year, v_Month, v_Day, v_Hour, v_Minute, v_Second,v_Timezone), cs_DaylightSavingTime_tlv(v_DaylightSavingTime))))); t_WaitForRRCResume_Step4.start; t_WaitForRRCResume_Step11.start; //@siclog "Step 2" siclog@ //The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. f_NR_RRC_InactiveDef(nr_Cell1, ResumeNoSecurity, -, -, -, -, -, -, nr_Cell11); // @sic R5s250008 sic@ //@siclog "Step 3" siclog@ //The SS re-adjusts the SS/PBCH EPRE levels according to row "T1" in table 6.1.2.18a.3.2-1. f_NR_SetCellPowerList(v_CellPowerList_AtT1); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell11); //@siclog "Step 4" siclog@ //Check: Does the UE transmit an RRCResumeRequest message on NR Cell 11 within 6mins after step 1? alt { []SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, mo_Signalling))){ t_WaitForRRCResume_Step4.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } [] t_WaitForRRCResume_Step4.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } } //@siclog "Step 5" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell11); //@siclog "Step 6" siclog@ //The UE transmits an RRCResumeComplete message and a REGISTRATION REQUEST message indicating "mobility registration updating" is sent to update the registration of NR Cell11. v_DedicatedNAS_Message := f_NR_RRCResumeComplete_Def(nr_Cell11); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, valueof(v_DedicatedNAS_Message.Pdu), Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } //@siclog "Step 7-8" siclog@ //Steps 4 to 5 of the generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 with condition MOBILITY are performed. //NOTE: PLMN1 and PLMN2 are listed as an Equivalent PLMN in the REGISTRATION ACCEPT message f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell11, v_GMM_MobilityInfo, omit, omit, noRrcConnectionRelease,-,tsc_NR_RbId_SRB2, cs_PLMN_List_1PLMN(f_NR_Asn2Nas_PlmnId(tsc_NR_HPLMN_002_11))); //Table 6.7.2.2.3.3-5: REGISTRATION ACCEPT // @siclog "Step 9" siclog@ // The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. f_NR_RRC_InactiveDef(nr_Cell11, ResumeNoSecurity, -, -, -, -, -, -, nr_Cell12); // @sic R5s250008 sic@ //@siclog "Step 10" siclog@ //TThe SS re-adjusts the SS/PBCH EPRE levels according to row "T2" in table 6.1.2.18a.3.2-1. f_NR_SetCellPowerList(v_CellPowerList_AtT2); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell12); //@siclog "Step 11" siclog@ //Check: Does the UE transmit an RRCResumeRequest message on NR Cell 12 within 9mins after step 1? alt { []SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, mo_Signalling))){ t_WaitForRRCResume_Step11.stop; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); } [] t_WaitForRRCResume_Step11.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 11"); } } //@siclog "Step 12" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell12); //@siclog "Step 13" siclog@ //The UE transmits an RRCResumeComplete message and a REGISTRATION REQUEST message indicating "mobility registration updating" is sent to update the registration of NR Cell12. v_DedicatedNAS_Message := f_NR_RRCResumeComplete_Def(nr_Cell12); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, valueof(v_DedicatedNAS_Message.Pdu), Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } // @siclog "Step 14-16" siclog@ //Steps 4 to 5 of the generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 with condition MOBILITY are performed. //NOTE: PLMN1 and PLMN2 are listed as an Equivalent PLMN in the REGISTRATION ACCEPT message // The SS transmits an RRCRelease message f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell12, v_GMM_MobilityInfo, omit, omit, -,-,tsc_NR_RbId_SRB2, cs_PLMN_List_1PLMN(f_NR_Asn2Nas_PlmnId(tsc_NR_HPLMN_Def))); //Table 6.7.2.2.3.3-5: REGISTRATION ACCEPT } // end f_TC_6_7_2_2_NR5GC_TestBody

## Change 3

| Function name | f_BCD_TimestampWithTimezone () |
| --- | --- |
| Reason for change | - Current TTCN is adding wrongly 128 as integer to update sign value of timezone which later goes to f_SwappedNibbles which only handles 2 digit operations, but current TTCN is providing it 3 digit, So f_SwappedNibbles return unexpected value as timezone, which resulted different timezone at UE side (For KST, its was returning ‘46’O from SS, which resulted timezone at UE side as GMT + 16 which is unexpected timezone) |
| Summary of change | - Modified v_SignVal to H2 type, and after performing f_SwappedNibbles operation on interger value, did bitwise OR operation to update sign value of the Timezone. Which resulted to currect timezone (For KST, its returning ‘63’O from SS, which resulted timezone at UE side as GMT + 9) |
| TTCN module | CommonDefs.ttcn |

Before Change:

function f_BCD_TimestampWithTimezone() return H14_Type { /* @sic R5-1307270: common function to provide timestamp for MDT test cases too sic@ */ var Struct_tm_Type v_LocalTime; var integer v_TimezoneSeconds; var integer v_TimezoneVal; var integer v_SignVal := 0; // positive value per default var H2_Type v_Year; var H2_Type v_Month; var H2_Type v_Day; var H2_Type v_Hour; var H2_Type v_Minute; var H2_Type v_Second; var H2_Type v_Timezone; fx_GetSystemTime(v_LocalTime, v_TimezoneSeconds); v_Year := f_SwappedNibbles(v_LocalTime.tm_year + 1900); /* acc. to definition of struct tm */ v_Month := f_SwappedNibbles(v_LocalTime.tm_mon + 1); /* acc. to definition of struct tm */ v_Day := f_SwappedNibbles(v_LocalTime.tm_mday); v_Hour := f_SwappedNibbles(v_LocalTime.tm_hour); v_Minute := f_SwappedNibbles(v_LocalTime.tm_min mod 60); /* v_LocalTime.tm_min can be 0..61 on some systems */ v_Second := f_SwappedNibbles(v_LocalTime.tm_sec); /* Calculation of the timezone (-> 23.040): */ if (v_LocalTime.tm_isdst > 0) { /* The value contained in the Time Zone field must take into account daylight saving time, such that when the sending entity changes from regular (winter) time to daylight saving (summer) time, there is a change to the value in the Time Zone field, for example in the UK the winter setting is 00000000 and the summer setting is 01000000. */ v_TimezoneSeconds := v_TimezoneSeconds + 3600; } if (v_TimezoneSeconds < 0) { v_TimezoneSeconds := -v_TimezoneSeconds; v_SignVal := 128; /* In the first of the two semi octets, the first bit (bit 3 of the seventh octet of the TP Service Centre Time Stamp field) represents the algebraic sign of this difference (0: positive, 1: negative) */ } v_TimezoneVal := v_TimezoneSeconds / (60 * 15); /* The Time Zone indicates the difference, expressed in quarters of an hour, between the local time and GMT. */ v_TimezoneVal := v_TimezoneVal mod 128; /* ... just to be sure */ v_TimezoneVal := v_TimezoneVal + v_SignVal; /* set the upper bit */ v_Timezone := f_SwappedNibbles(v_TimezoneVal); return v_Year & v_Month & v_Day & v_Hour & v_Minute & v_Second & v_Timezone; }

After Change:

function f_BCD_TimestampWithTimezone_6716() return H14_Type { /* @sic R5-1307270: common function to provide timestamp for MDT test cases too sic@ */ var Struct_tm_Type v_LocalTime; var integer v_TimezoneSeconds; var integer v_TimezoneVal; var H2_Type v_SignVal := '00'H; // positive value per default var H2_Type v_Year; var H2_Type v_Month; var H2_Type v_Day; var H2_Type v_Hour; var H2_Type v_Minute; var H2_Type v_Second; var H2_Type v_Timezone; var integer v_TimezoneMinutes; var integer v_Timezone_i; fx_GetSystemTime(v_LocalTime, v_TimezoneSeconds); v_Year := f_SwappedNibbles(v_LocalTime.tm_year + 1900); /* acc. to definition of struct tm */ v_Month := f_SwappedNibbles(v_LocalTime.tm_mon + 1); /* acc. to definition of struct tm */ v_Day := f_SwappedNibbles(v_LocalTime.tm_mday); v_Hour := f_SwappedNibbles(v_LocalTime.tm_hour); v_Minute := f_SwappedNibbles(v_LocalTime.tm_min mod 60); /* v_LocalTime.tm_min can be 0..61 on some systems */ v_Second := f_SwappedNibbles(v_LocalTime.tm_sec); /* Calculation of the timezone (-> 23.040): */ if (v_LocalTime.tm_isdst > 0) { /* The value contained in the Time Zone field must take into account daylight saving time, such that when the sending entity changes from regular (winter) time to daylight saving (summer) time, there is a change to the value in the Time Zone field, for example in the UK the winter setting is 00000000 and the summer setting is 01000000. */ v_TimezoneSeconds := v_TimezoneSeconds + 3600; } if (v_TimezoneSeconds < 0) { v_TimezoneSeconds := -v_TimezoneSeconds; v_SignVal := '08'H; /* In the first of the two semi octets, the first bit (bit 3 of the seventh octet of the TP Service Centre Time Stamp field) // represents the algebraic sign of this difference (0: positive, 1: negative) */ } v_TimezoneVal := v_TimezoneSeconds / (60 * 15); /* The Time Zone indicates the difference, expressed in quarters of an hour, between the local time and GMT. */ v_Timezone := f_SwappedNibbles(v_TimezoneVal); v_Timezone := v_Timezone or4b v_SignVal; /* set the upper bit */ return v_Year & v_Month & v_Day & v_Hour & v_Minute & v_Second & v_Timezone; }

## Change 4

| Function name | cds_38508_UE_TimersAndConstants_NTN () |
| --- | --- |
| Reason for change | - For GSO, the configured CellSpecificKoffset is 550 ms, whereas for NGSO it is only 5 ms. Since CellSpecificKoffset directly adds to the RACH access delay, the overall RACH procedure in the GSO case becomes significantly longer. As a result, the UE takes more time before it can actually transmit the RRCResumeRequest. This additional delay can cause T319 (the supervision timer for the resume procedure) to expire before the network responds. When T319 expires, the UE abandons the resume attempt and instead initiates a new connection establishment by sending an RRCSetupRequest #draft ProseCR would be raised for RAN5#110 meeting |
| Summary of change | - Increased T319 timer to ms2000 |
| TTCN module | NR_SysInfo_Templates.ttcn |

Before Change:

template (value) UE_TimersAndConstants cds_38508_UE_TimersAndConstants_NTN modifies cs_38508_UE_TimersAndConstants := { /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ // UE-TimersAndConstants for TC 7.1.2.3.9a //@sic R5s250100 R5-253086 sic@ t300 := ms2000 };

After Change:

template (value) UE_TimersAndConstants cds_38508_UE_TimersAndConstants_NTN modifies cs_38508_UE_TimersAndConstants := { /* @status APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */ // UE-TimersAndConstants for TC 7.1.2.3.9a //@sic R5s250100 R5-253086 sic@ t300 := ms2000, t319 := ms2000 };

## Change 5

| Function name | cs_NTN_Config_NGSO_Cell3 () |
| --- | --- |
| Reason for change | - As per 38.508-1, Table 6.3.4.2-1 Fd Column for 3rd cell has 19.95 as Frequency doppler has been mentioned, Soc accordingly NtnFrequencyDopplerShift has to be updated on TTCN. |
| Summary of change | - Updated template cs_38508_NTN_Config with 199500 |
| TTCN module | NR_NTN_Configuration.ttcn |

Before Change:

template (value) NR_NTN_Config_Type cs_NTN_Config_NGSO_Cell3 := cs_38508_NTN_Config(cs_NTN_TimingOffset_NGSO_NTNCell2_NTNCell3,-,-195000);

After Change:

template (value) NR_NTN_Config_Type cs_NTN_Config_NGSO_Cell3 := cs_38508_NTN_Config(cs_NTN_TimingOffset_NGSO_NTNCell2_NTNCell3,-,-199500);

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n255” using NIA2 Integrity and NEA0 ciphering algorithms on Anritsu 5G Conformance Test platform.

# Execution Log Files

## Samsung Exynos 2500

The Samsung Exynos 2500 UE  passed this test case on Anritsu 5G Protocol Conformance Test platform ME7834NR on GSO mode. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_6_7_2_2_LOG.html

- PICS/PIXIT parameter file:

TC_6_7_2_2_PIXIT.xml

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s250482: Supporting information for correction of NR-NTN testcase 6.7.2.2 |
| --- | --- |