3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250489

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3779 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to measurement templates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2025-11-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The contents of measResultSFTD_NR and measResultCellListSFTD_NR IEs in cr_38508_MeasResults do not correspond the contents defined in Table 4.6.3-79 in TS 38.508-1. - The contents cs_38508_QuantityConfig_EUTRA do not specify quantityConfigNR-List as defined in TS 38.508-1, Table 4.6.3-127 - The templates cs_MeasGapRelease_Base and cs_38508_MeasGapConfig_Base are identical. It is proposed to use cs_38508_MeasGapConfig_Base further at all ocasions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated assignements of measResultSFTD_NR and measResultCellListSFTD_NR IEs to ‘omit’ in cr_38508_MeasResults - Updated cs_38508_QuantityConfig_EUTRA with quantityConfigNR-List contents - Changed cds_MeasGapRelease_FR1 to be derived from the cs_38508_MeasGapConfig_Base template |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The template contents remain to be set not as defined in TS 38.508-1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.2.1.NR5GC, 8.1.3.2.2.NR5GC, 8.1.3.2.3.NR5GC, 8.1.3.2.4.NR5GC, 8.1.3.3.2.NR5GC, 8.1.6.2.1.NR5GC, 8.1.6.2.3.NR5GC, 8.2.3.11.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	7

2.3	Change 3	7

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact:	Olivier Genoud
	olivier.genoud@etsi.org

# Corrections required

## Change 1

| Template name | cr_38508_MeasResults |
| --- | --- |
| Reason for change | The measResultSFTD_NR and measResultCellListSFTD_NR IEs are not set as per default contents in 38.508-1 as defined in R5-216154. |
| Summary of change | Updated measResultSFTD_NR and measResultCellListSFTD_NR IEs to “Not present” |
| TTCN module | NR_Measurement_Templates.ttcn |

Before Change

template (present) MeasResults cr_38508_MeasResults(MeasId p_MeasId := tsc_NR_MeasId1, PhysCellId p_PhysCellId, template MeasResults.measResultNeighCells p_MeasResultNeighCells := omit, template (present) MeasQuantityResults p_MeasQuantityResults := f_NR_Get_MeasQuantityResults(), template UL_PDCP_DelayValueResultList_r16 p_UL_PDCP_DelayValueResultList_r16 := omit) := { // @sic R5-186068, R5-194847, R5s190200, R5s190144, R5-194849, R5-201097 sic@ /* @status APPROVED (ENDC, NR5GC, NR5GC_IRAT) */ measId := p_MeasId, measResultServingMOList := { { //1 entry servCellId := f_NR_ServCellIndex(), //@sic R5-192274 sic@ measResultServingCell := cr_NR_MeasResultNR_CellResults(p_PhysCellId, p_MeasQuantityResults), measResultBestNeighCell := omit } }, measResultNeighCells := p_MeasResultNeighCells, measResultServFreqListEUTRA_SCG := omit, /* @sic R5-216154, R5s190543 BASELINE MOVING 2019 sic@ */ measResultServFreqListNR_SCG := *, /* @sic R5s190543 BASELINE MOVING 2019 sic@ */ measResultSFTD_EUTRA := omit, /* @sic R5-216154, R5s190543 BASELINE MOVING 2019 sic@ */ measResultSFTD_NR := *, /* @sic R5-216154, R5s190543 BASELINE MOVING 2019 sic@ */ measResultCellListSFTD_NR := *, /* @sic R5-216154, R5s201387 BASELINE MOVING 2020 sic@ */ measResultForRSSI_r16 := omit, /* @sic R5-216154, R5s201387 BASELINE MOVING 2020 sic@ */ locationInfo_r16 := omit, /* @sic R5-216154, R5s201387 BASELINE MOVING 2020 sic@ */ ul_PDCP_DelayValueResultList_r16 := p_UL_PDCP_DelayValueResultList_r16, /* @sic R5-216154, R5s201387 BASELINE MOVING 2020 sic@ */ measResultsSL_r16 := omit, /* @sic R5-216154, R5s201387 BASELINE MOVING 2020 sic@ */ measResultCLI_r16 := omit, /* @sic R5-216154, R5s201387 BASELINE MOVING 2020 sic@ */ measResultRxTxTimeDiff_r17 := omit, /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */ sl_MeasResultServingRelay_r17 := *, /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */ ul_PDCP_ExcessDelayResultList_r17:= omit, /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */ coarseLocationInfo_r17 := omit, /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */ altitudeUE_r18 := omit, /* @sic R5s240531 BASELINE MOVING 2024 rel-18 sic@ */ cellsMetReportOnLeaveList_r18 := omit /* @sic R5s240531 BASELINE MOVING 2024 rel-18 sic@ */ };

After Change

template (present) MeasResults cr_38508_MeasResults(MeasId p_MeasId := tsc_NR_MeasId1, PhysCellId p_PhysCellId, template MeasResults.measResultNeighCells p_MeasResultNeighCells := omit, template (present) MeasQuantityResults p_MeasQuantityResults := f_NR_Get_MeasQuantityResults(), template UL_PDCP_DelayValueResultList_r16 p_UL_PDCP_DelayValueResultList_r16 := omit) := { // @sic R5-186068, R5-194847, R5s190200, R5s190144, R5-194849, R5-201097 sic@ /* @status APPROVED (ENDC, NR5GC, NR5GC_IRAT) */ measId := p_MeasId, measResultServingMOList := { { //1 entry servCellId := f_NR_ServCellIndex(), //@sic R5-192274 sic@ measResultServingCell := cr_NR_MeasResultNR_CellResults(p_PhysCellId, p_MeasQuantityResults), measResultBestNeighCell := omit } }, measResultNeighCells := p_MeasResultNeighCells, measResultServFreqListEUTRA_SCG := omit, /* @sic R5-216154, R5s190543 BASELINE MOVING 2019 sic@ */ measResultServFreqListNR_SCG := *, /* @sic R5s190543 BASELINE MOVING 2019 sic@ */ measResultSFTD_EUTRA := omit, /* @sic R5-216154, R5s190543 BASELINE MOVING 2019 sic@ */ measResultSFTD_NR := omit, /* @sic R5-216154, R5s190543 BASELINE MOVING 2019 sic@ */ measResultCellListSFTD_NR := omit, /* @sic R5-216154, R5s201387 BASELINE MOVING 2020 sic@ */ measResultForRSSI_r16 := omit, /* @sic R5-216154, R5s201387 BASELINE MOVING 2020 sic@ */ locationInfo_r16 := omit, /* @sic R5-216154, R5s201387 BASELINE MOVING 2020 sic@ */ ul_PDCP_DelayValueResultList_r16 := p_UL_PDCP_DelayValueResultList_r16, /* @sic R5-216154, R5s201387 BASELINE MOVING 2020 sic@ */ measResultsSL_r16 := omit, /* @sic R5-216154, R5s201387 BASELINE MOVING 2020 sic@ */ measResultCLI_r16 := omit, /* @sic R5-216154, R5s201387 BASELINE MOVING 2020 sic@ */ measResultRxTxTimeDiff_r17 := omit, /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */ sl_MeasResultServingRelay_r17 := *, /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */ ul_PDCP_ExcessDelayResultList_r17:= omit, /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */ coarseLocationInfo_r17 := omit, /* @sic R5s221179 BASELINE MOVING 2022 rel-17 sic@ */ altitudeUE_r18 := omit, /* @sic R5s240531 BASELINE MOVING 2024 rel-18 sic@ */ cellsMetReportOnLeaveList_r18 := omit /* @sic R5s240531 BASELINE MOVING 2024 rel-18 sic@ */ };

## Change 2

| Template name | cs_38508_QuantityConfig_EUTRA |
| --- | --- |
| Reason for change | The current contents of the template do not match definitions in TS 38.508-1, Table 4.6.3-127 |
| Summary of change | Updated cs_38508_QuantityConfig_EUTRA to include quantityConfigNR-List entries |
| TTCN module | NR_Measurement_Templates.ttcn |

Before Change

template (value) QuantityConfig cs_38508_QuantityConfig_EUTRA := { /* @status APPROVED (NR5GC_IRAT) */ //@sic R5-195345 sic@ quantityConfigNR_List := omit, quantityConfigEUTRA := cs_38508_FilterConfig, quantityConfigUTRA_FDD_r16 := omit, /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ quantityConfigCLI_r16 := omit /* @sic R5s201387 BASELINE MOVING 2020 sic@ */ };

After Change

template (value) QuantityConfig cs_38508_QuantityConfig_EUTRA := cs_38508_QuantityConfig(cs_38508_FilterConfig)

## Change 3

| Template name | cds_MeasGapRelease_FR1 |
| --- | --- |
| Reason for change | The template cs_MeasGapRelease_Base is turned to be redundant since it is identical to cs_38508_MeasGapConfig_Base template. The removal of the duplicate affects cds_MeasGapRelease_FR1 |
| Summary of change | Updated cds_MeasGapRelease_FR1 to be derived from cs_38508_MeasGapConfig_Base. |
| TTCN module | RRCMeasurement_ENDC_NR |

Before Change

template (value) MeasGapConfig cds_MeasGapRelease_FR1 modifies cs_MeasGapRelease_Base := { /* @status APPROVED (ENDC) */ gapFR1 := { release := NULL } };

After Change

template (value) MeasGapConfig cds_MeasGapRelease_FR1 modifies cs_38508_MeasGapConfig_Base := { /* @status APPROVED (ENDC) */ gapFR1 := { release := NULL } };