3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250491

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 1006 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR eCall Testcase 11.14 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ, Anritsu, Keysight |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCallCEN-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add test case 11.14 to the IMS_NR5GC test suite. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 11.14 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.14.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

1.1 Verification Test Summary	2

2	Corrections required	3

2.1	Change 1	3

3	Branches executed	4

4	Execution Log Files	4

4.1	Qualcomm Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies	4

5	References	5

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.14 which is part of the IMS_NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:		Parikshit Bhise

Parikshit.bhise@rohde-schwarz.com

Kalahasti Narendra

Narendra.kalahasti@anritsu.com

Mohit Kanchan

Mohit.kanchan@keysight.com

## 1.1 Verification Test Summary

Test Case: 	11.14.NR5GC

Test Group:                           	IMS_NR5GC_eCallPSAPCallbackTestcases.ttcn

ATS Version:	iwd-TTCN3-B2024-06_D25wk38

System Simulator used:	R&S® 5G Protocol Conformance Test platform,

Anritsu Protocol Conformance Test System ME7834NR,

Keysight 5G Protocol Conformance Toolset

UE used:		Qualcomm Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_NR5GC_IPCAN_MainLoop |
| --- | --- |
| Reason for change | In test case 11.14 the call to function f_IMS_ECallOnly_TestCall_IMS1 -> f_IMS_IPCAN_StartProcedure triggers the call f_NR5GC_IPCAN_MainLoop -> f_NR5GC_IPCAN_StartProcedure with argument set as IPCAN_ECallOnly_TestCall. But this Argument is not handled as part of setting up the v_IPCAN_Registered. So v_IPCAN_Registered gets set as false due to which the f_NR5GC_IPCAN_Release function passes the STATE_DEREGISTERED value to the f_NR_Postamble and the TE is not handling the IMS Deregister request from the UE. This needs to be addressed. |
| Summary of change | Updated the procedure to set the v_IPCAN_Registered := true by adding IPCAN_ECallOnly_TestCall argument to the selection criteria. |
| TTCN module | IPCAN_NR5GC.ttcn |

Before Change:

<<SKIPPED CODE>> case (IPCAN_STARTPROCEDURE) { v_StartProcedureResult := f_NR5GC_IPCAN_StartProcedure(IMS[v_PortIndex], v_CellId, v_Command.TestProcedure, v_PdnIndex, // @sic R5-255081 sic@ v_Command.TestConfiguration, v_ConnectionEstablished); v_ConnectionEstablished := true; select ( v_Command.TestProcedure ) { case (IPCAN_InitialRegistration, IPCAN_ECallOnly, IPCAN_ECallOnly_Aborted) { // @sic R5s211495, R5-254979 sic@ v_IPCAN_Registered := true; } } } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> case (IPCAN_STARTPROCEDURE) { v_StartProcedureResult := f_NR5GC_IPCAN_StartProcedure(IMS[v_PortIndex], v_CellId, v_Command.TestProcedure, v_PdnIndex, // @sic R5-255081 sic@ v_Command.TestConfiguration, v_ConnectionEstablished); v_ConnectionEstablished := true; select ( v_Command.TestProcedure ) { case (IPCAN_InitialRegistration, IPCAN_ECallOnly, IPCAN_ECallOnly_Aborted, IPCAN_ECallOnly_TestCall) { // @sic R5s211495, R5-254979 sic@ //WA#WI1464497,WA#11_14 v_IPCAN_Registered := true; } } } <<SKIPPED CODE>>

# Branches executed

The test case was executed on on NR band 41 using ciphering algorithm NEA2 and integrity algorithm NIA2 on R&S® 5G Protocol Conformance Test platform

This TC was executed in 5GS Stand Alone mode (Option 2) on NR Primary Band “n41” using NIA1 Integrity and NEA1 ciphering algorithms on Anritsu Protocol Conformance Test System ME7834NR.

This testcase was executed in 5GS stand alone mode (option 2) on NR primary band n41 using ciphering algorithm NEA1 and integrity algorithm NIA1 on Keysight 5G Protocol Conformance Toolset

# Execution Log Files

## Qualcomm Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies

R&S:

The Qualcomm Snapdragon Auto 5G Modem-RF Gen 2, Qualcomm Technologies UE passed this test case on Rohde & Schwarz CMW500. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_11_14_Qualcomm.log

(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file

Anritsu:

The Qualcomm Snapdragon Auto 5G Modem-RF Gen 2 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_11_14_LOG.html

- PICS/PIXIT parameter file:

TC_11_14_PIXIT.xml

Keysight:

The Qualcomm Snapdragon Auto 5G Modem-RF Gen 2 UE passed this test case on Keysight 5G Protocol Conformance Toolset and R&S® 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight\TC_11_14_LOG

- PICS/PIXIT parameter file:

Keysight\TC_11_14_PIXIT.xml

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s250492: Supporting information for addition of NR eCall Testcase 11.14 |
| --- | --- |