3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250494

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4937 | rev | 1 | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of LTE RRC test case 8.1.2.5a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test |  |  |  |  |  | Date: |  |  | 2025-09-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add test case 8.1.2.5a to the LTE_A_R12 test suite. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 8.1.2.5a required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.5a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 36.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1 Overview	3

1.1 Verification Test Summary	3

2 Corrections required	3

2.1 Change 1	3

2.2 Change 2	9

3	Branches executed	12

4	Execution Log Files	13

4.1	Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE	13

5	References	13

# 1 Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 8.1.2.5a which is part of the LTE_A_R12 test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:		Bhise Parikshit

parikshit.bhise@rohde-schwarz.com

## 1.1 Verification Test Summary

Test Case: 	8.1.2.5a

Test Group:                        /LTE_A_R12/13/AccessControl_R12.ttcn

ATS Version:	iwd-TTCN3-B2024-06_D25wk24

System Simulator used:	Rohde & Schwarz CMW500,

UE used:	Qualcomm Snapdragon® X80 5G Modem

Verification Status:	PASS

# 2 Corrections required

## 2.1 Change 1

| Function name | function f_TC_8_1_2_5a_EUTRA () runs on EUTRA_PTC |
| --- | --- |
| Reason for change | - As per Prose, in preabmle the UE is in Idle mode (state 2) according to TS 36.508[18] after the UE is in state Loopback Activated (state 4) with condition UE TEST LOOP MODE B on Cell 1 (serving cell) according to [18]. This is not implemented correctly in TTCN, so needs to be corrected. - In the existing TTCN implementation the step 1 is not align with Table 8.1.2.5a.3.2-2 of the prose. - The current TTCN implementation does not correctly handle step 13 of the test case, where the AC barring for MO data in SIB2 needs to be removed. This oversight prevents the UE from sending the RRC connection request in step 20, hence this needs to be resolved. - In the preamble, an IP PDU delay of 5s is applied, which is applied by the UE to the IP PDU received at Step 9.After this, this IP PDU delay is not applied for further IP PDUs (this is inline with 36.509).As a result, when the SS transmits the IP PDU at Step 17 the UE immediately loops it back and this is received during the 1s timer at Step 18 causing TC failure (Proposed Prose-CR) - In the existing TTCN implementation, the function f_EUTRA_ServiceRequestAndActivate_SRB2_DRB is used at steps 21-28, which handles steps 3-10 of the procedure outlined in 36.508 clause 4.5.3.3. However, step 3 (the RRC connection request) of this procedure has already been implemented in step 20 of the test case. This redundancy needs to be resolved to ensure proper execution of the test case. - Additionally, step 29, where the UE loops back the IP packet, is not implemented accurately in the existing TTCN. The IP packet received by the SS may be confused with the RRC Connection Reconfiguration Complete message sent by the UE in the previous steps (21-28), which could lead to a failure of the test case. |
| Summary of change | - In Preamble added the function f_EUTRA_RRC_ConnectionRelease to transition into EUTRA Idle state - Aligned the TTCN with Prose. - Modifed the TTCN code by introducing the function f_EUTRA_CellInfo_SetSIB2_AC_BarringPerPlmnList_r12 and ACB for MO data in Sib2 is assigned an omit value. - Added steps 16A-D to implement the open TEST LOOP and Close TEST LOOP UE procedures. - Introudced a new function f_EUTRA_ServiceRequestAndActivate_SRB2_DRB_TC8_1_2_5a and step 3 of procedure in 36.508 cl. 4.5.3.3 is removed for this function. - Removed the reception of the DRB in step 29 and added SRB and DRB reception using interleave statement. |
| TTCN module | LTE_A_R12\8_1\ RRC_ConnEst_R12.ttcn |
| MCC160 comment |  |

Before Change:

function f_TC_8_1_2_5a_EUTRA() runs on EUTRA_PTC { /* RRC connection establishment / 0% access probability for MO data, no restriction for MO signalling / AC-Barring per PLMN */ var template (value) CellPowerList_Type v_CellPowerList_AtT1; var float v_TimerValue; var PDCP_SDUList_Type v_PDCP_SDUList_DRB1 := { f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()) }; f_EUTRA_Init(c1); //Set cell power level for cell 1 and 11 to be used at creation f_EUTRA_CellInfo_InitMaxReferencePower(eutra_Cell11, -79); //Create and configure cells f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_CellConfig_Def(eutra_Cell11); f_EUTRA_CellInfo_SetPLMN_2Entries (eutra_Cell1, cs_HPLMN_Def, //PLMNid 1 (index 0) cs_HPLMN_002_11 //PLMNid 2 (index 1) ); f_EUTRA_CellInfo_SetPLMN_2Entries (eutra_Cell11, cs_HPLMN_Def, //PLMNid 1 (index 0) cs_HPLMN_002_11 //PLMNid 2 (index 1) ); f_UT_USIM_Insert(UT, "Insert special USIM"); //USIM with PLMN2 is set in it as 'forbidden PLMN' // Preamble f_EUTRA_Preamble(eutra_Cell1, STATE2A_TESTLOOP_ModeB); f_EUTRA_RbEst_Def(eutra_Cell1); f_EUTRA_CloseUE_TestLoopModeB(eutra_Cell1, '05'O); f_EUTRA_TestBody_Set(true); //@siclog "Step 1" siclog@ //The SS changes SystemInformationBlockType2 parameters to 0% access probability for MO Data and transmits a Paging message including systemInfoModification. (Note 1) //The systemInfoValueTag in the SystemInformationBlockType1 is increased. f_EUTRA_CellInfo_SetSIB2_AC_BarringPerPlmnList_r12 (eutra_Cell11, 1, false,omit, omit, omit, cs_AC_BarringConfig(p00, s128, '11111'B)); f_EUTRA_ModifySysinfo(eutra_Cell11); //@siclog "Step 2" siclog@ //Wait for 15 s (Note 2) to allow the new system information to take effect. f_Delay( 15.0); //@siclog "Step 3" siclog@ //The SS changes Cell 1 and Cell 11 level according to the row "T1" in table 8.1.2.5a.3.2-1. v_CellPowerList_AtT1 := { cs_CellPower(eutra_Cell11, -79) }; f_EUTRA_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 4" siclog@ //Check: Does the UE transmit an RRCConnectionRequest on Cell 11? f_EUTRA_RRC_ConnectionRequest_Def(eutra_Cell11, cr_EstablishmentCause_MoSignalling); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.1.2.5a Step 4"); //@siclog "Steps 5-8" siclog@ //Steps 2 to 5 of the generic test procedure in TS 36.508 [18] subclause 6.4.2.7 are performed on Cell 11. //Note: The UE performs a TAU procedure. f_EUTRA_TrackingAreaUpdate_WithoutRrcConnReq (eutra_Cell11); //@siclog "Step 9" siclog@ //The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context on Cell 11. DRB.send(cas_DRB_COMMON_REQ_PDCP_SDU(eutra_Cell11, tsc_RbId_DRB1, cs_TimingInfo_Now, v_PDCP_SDUList_DRB1)); //@siclog "Step 10" siclog@ //Wait for 1 s after the IP packet has been transmitted in step 9. (Note 3) f_Delay(1.0); //@siclog "Step 11" siclog@ //The SS transmits an RRCConnectionRelease message on Cell 11. f_EUTRA_RRC_ConnectionRelease(eutra_Cell11); //@siclog "Step 12" siclog@ //Check: Does the UE transmit an RRCConnectionRequest message on Cell 11 within 30s? v_TimerValue := 30.0; if (f_EUTRA_RRC_RRCConnectionRequest_Check(eutra_Cell11, v_TimerValue)) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test Case 8.1.2.5a Step 12"); } //@siclog "Step 13" siclog@ //SS changes SIB2 according to TS 36.508, table 4.4.3.3-1 and transmits a Paging message including systemInfoModification. The systemInfoValueTag in the SystemInformationBlockType1 is increased f_EUTRA_CellInfo_SetSIB2_SSAC_BarringForMMTEL_Video (eutra_Cell11, omit); f_EUTRA_ModifySysinfo(eutra_Cell11); //@siclog "Step 14" siclog@ //Wait for 15 s (Note 2) to allow the new system information to take effect. f_Delay(15.0); //@siclog "Step 15" siclog@ //Wait for 167 s (Note 4) to make the Tbarring expire f_Delay(167.0); //@siclog "Step 16" siclog@ //Generic test procedure in TS 36.508 subclause 4.5.3.3 is performed on Cell 11. //NOTE: The UE performs the establishment of the new data radio bearer associated with the default EPS bearer context. f_EUTRA_RbEst_Def(eutra_Cell11); //@siclog "Step 17" siclog@ //The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context on Cell 11. DRB.send(cas_DRB_COMMON_REQ_PDCP_SDU(eutra_Cell11, tsc_RbId_DRB1, cs_TimingInfo_Now, v_PDCP_SDUList_DRB1)); //@siclog "Step 18" siclog@ //Wait for 1 s for the UE to enter E-UTRA RRC_IDLE state. f_Delay(1.0); //@siclog "Step 19" siclog@ //The SS transmits an RRCConnectionRelease message on Cell 11. f_EUTRA_RRC_ConnectionRelease(eutra_Cell11); //@siclog "Step 20" siclog@ //Check: Does the UE transmit an RRCConnectionRequest message on Cell 11 within 30s? f_EUTRA_RRC_ConnectionRequest_Def(eutra_Cell11, cr_EstablishmentCause_MoData); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.1.2.5a Step 20"); //@siclog "Step 21-28" siclog@ //Steps 3 to 10 generic test procedure in TS 36.508 subclause 6.4.2.2 are performed. f_EUTRA_ServiceRequestAndActivate_SRB2_DRB(eutra_Cell11); //@siclog "Step 29" siclog@ //The UE loop backs the IP packet received in step 18 on the DRB associated with the default EPS bearer context on Cell 11. DRB.receive(car_DRB_COMMON_IND_PDCP_SDUList(eutra_Cell11, tsc_RbId_DRB1, ?, v_PDCP_SDUList_DRB1)); f_EUTRA_TestBody_Set(false); //Switch off UE and expect Detach Request f_EUTRA_Postamble(eutra_Cell11, E3_TESTMODE); }

After Change:

function f_TC_8_1_2_5a_EUTRA() runs on EUTRA_PTC { /* RRC connection establishment / 0% access probability for MO data, no restriction for MO signalling / AC-Barring per PLMN */ var template (value) CellPowerList_Type v_CellPowerList_AtT1; var float v_TimerValue; var PDCP_SDUList_Type v_PDCP_SDUList_DRB1 := { f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()) }; f_EUTRA_Init(c1); //Set cell power level for cell 1 and 11 to be used at creation f_EUTRA_CellInfo_InitMaxReferencePower(eutra_Cell11, -79); //Create and configure cells f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_CellConfig_Def(eutra_Cell11); f_EUTRA_CellInfo_SetPLMN_2Entries (eutra_Cell1, cs_HPLMN_Def, //PLMNid 1 (index 0) cs_HPLMN_002_11, //PLMNid 2 (index 1) -, true, 1); //@sic R5-253053 sic@ f_EUTRA_CellInfo_SetPLMN_2Entries (eutra_Cell11, cs_HPLMN_Def, //PLMNid 1 (index 0) cs_HPLMN_002_11, //PLMNid 2 (index 1) -, true, 1); //@sic R5-253053 sic@ // Preamble f_EUTRA_Preamble(eutra_Cell1, STATE2A_TESTLOOP_ModeB); f_EUTRA_RbEst_Def(eutra_Cell1); f_EUTRA_CloseUE_TestLoopModeB(eutra_Cell1, '05'O); f_EUTRA_RRC_ConnectionRelease( eutra_Cell1 f_EUTRA_TestBody_Set(true); //@siclog "Step 1" siclog@ //The SS changes configures SystemInformationBlockType2 parameters to 0% access probability for MO Data f_EUTRA_CellInfo_SetSIB2_AC_BarringPerPlmnList_r12 (eutra_Cell11, 1, omit, omit, omit, cs_AC_BarringConfig(p00, s128, '11111'B)); f_EUTRA_ModifySysinfo(eutra_Cell11); //@siclog "Step 2 - Void" siclog@ //@sic R5-253053 sic@ //@siclog "Step 3" siclog@ //The SS changes Cell 1 and Cell 11 level according to the row "T1" in table 8.1.2.5a.3.2-1. v_CellPowerList_AtT1 := { cs_CellPower(eutra_Cell11, -79) }; f_EUTRA_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 4" siclog@ //Check: Does the UE transmit an RRCConnectionRequest on Cell 11? f_EUTRA_RRC_ConnectionRequest_Def(eutra_Cell11, cr_EstablishmentCause_MoSignalling); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.1.2.5a Step 4"); //@siclog "Steps 5-8A" siclog@ //Steps 2 to 6 of the generic test procedure in TS 36.508 [18] subclause 6.4.2.7 are performed on Cell 11. //Note: The UE performs a TAU procedure and the RRC connection is released. f_EUTRA_TrackingAreaUpdate_WithoutRrcConnReq (eutra_Cell11); //@siclog "Steps 8B" siclog@ //Wait for 5 s for the UE to enter E-UTRA RRC_IDLE state. //@sic R5-253053 sic@ f_Delay(5.0); //@siclog "Steps 8C" siclog@ //Generic test procedure in TS 36.508 subclause 4.5.3.3 is performed on Cell 11. //NOTE: The UE performs the establishment of the new data radio bearer associated with the default EPS bearer context. //@sic R5-253053 sic@ f_EUTRA_RbEst_Def(eutra_Cell11); //@siclog "Step 9" siclog@ //The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context on Cell 11. DRB.send(cas_DRB_COMMON_REQ_PDCP_SDU(eutra_Cell11, tsc_RbId_DRB1, cs_TimingInfo_Now, v_PDCP_SDUList_DRB1)); //@siclog "Step 10" siclog@ //Wait for 1 s after the IP packet has been transmitted in step 9. (Note 3) f_Delay(1.0); //@siclog "Step 11" siclog@ //The SS transmits an RRCConnectionRelease message on Cell 11. f_EUTRA_RRC_ConnectionRelease(eutra_Cell11); //@siclog "Step 12" siclog@ //Check: Does the UE transmit an RRCConnectionRequest message on Cell 11 within 30s? v_TimerValue := 30.0; if (f_EUTRA_RRC_RRCConnectionRequest_Check(eutra_Cell11, v_TimerValue)) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "Test Case 8.1.2.5a Step 12"); } //@siclog "Step 13" siclog@ //SS changes SIB2 according to TS 36.508, table 4.4.3.3-1 and transmits a Paging message including systemInfoModification. The systemInfoValueTag in the SystemInformationBlockType1 is increased f_EUTRA_CellInfo_SetSIB2_AC_BarringPerPlmnList_r12 (eutra_Cell11,1,-, -, -, omit); f_EUTRA_ModifySysinfo(eutra_Cell11); //@siclog "Step 14" siclog@ //Wait for 15 s (Note 2) to allow the new system information to take effect. f_Delay(15.0); //@siclog "Step 15" siclog@ //Wait for 167 s (Note 4) to make the Tbarring expire f_Delay(167.0); //@siclog "Step 16" siclog@ //Generic test procedure in TS 36.508 subclause 4.5.3.3 is performed on Cell 11. //NOTE: The UE performs the establishment of the new data radio bearer associated with the default EPS bearer context. f_EUTRA_RbEst_Def(eutra_Cell11); f_EUTRA_OpenUE_TestLoopMode(eutra_Cell11); f_EUTRA_CloseUE_TestLoopModeB(eutra_Cell11, '05'O); //@siclog "Step 17" siclog@ //The SS transmits one IP packet to the UE on the DRB associated with the default EPS bearer context on Cell 11. DRB.send(cas_DRB_COMMON_REQ_PDCP_SDU(eutra_Cell11, tsc_RbId_DRB1, cs_TimingInfo_Now, v_PDCP_SDUList_DRB1)); //@siclog "Step 18" siclog@ //Wait for 1 s for the UE to enter E-UTRA RRC_IDLE state. f_Delay(1.0); //@siclog "Step 19" siclog@ //The SS transmits an RRCConnectionRelease message on Cell 11. f_EUTRA_RRC_ConnectionRelease(eutra_Cell11); //@siclog "Step 20" siclog@ //Check: Does the UE transmit an RRCConnectionRequest message on Cell 11 within 30s? f_EUTRA_RRC_ConnectionRequest_Def(eutra_Cell11, cr_EstablishmentCause_MoData); f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 8.1.2.5a Step 20"); //@siclog "Step 21-28" siclog@ //Steps 3 to 10 generic test procedure in TS 36.508 subclause 6.4.2.2 are performed. f_EUTRA_ServiceRequestAndActivate_SRB2_DRB_TC_8_1_2_5a(eutra_Cell11); //@siclog "Step 29" siclog@ //The UE loop backs the IP packet received in step 18 on the DRB associated with the default EPS bearer context on Cell 11. interleave { [] SRB.receive(car_SRB1_RrcPdu_IND(eutra_Cell11, cr_508_RRCConnectionReconfigurationComplete(tsc_RRC_TI_Def))) {} [] DRB.receive (car_DRB_COMMON_IND_PDCP_SDUList(eutra_Cell11, tsc_RbId_DRB1, ?, v_PDCP_SDUList_DRB1)) {} } f_EUTRA_TestBody_Set(false); //Switch off UE and expect Detach Request f_EUTRA_Postamble(eutra_Cell11, E3_TESTMODE); }

## 2.2 Change 2

| Function name | function f_EUTRA_ServiceRequestAndActivate_SRB2_DRB_TC8_1_2_5a |
| --- | --- |
| Reason for change | Please see change 1 point 5. |
| Summary of change | New function |
| TTCN module | LTE_A_R12\Common\EUTRA\ EUTRA_NASSteps.ttcn |
| MCC160 comment |  |

function f_EUTRA_ServiceRequestAndActivate_SRB2_DRB_TC_8_1_2_5a(EUTRA_CellId_Type p_CellId, UInt_Type p_NoOfAddDRBs_AM := 0, UInt_Type p_NoOfAddDRBs_UM := 0, template (value) DRB_ConfigurationAtSS_Type p_DrbConfigAtSS := tsc_DRB_ConfigurationAtSS_Default, template (omit) DRB_ToAddModList p_DRB_ToAddModList := omit, template (omit) MAC_MainConfig_Type p_MAC_MainConfig := omit, template (omit) PhysicalConfigDedicated p_PhysicalConfigDedicated := omit, template (omit) EPS_QoSList_Type p_QoS := omit, template (omit) Tft_List_Type p_Tft := omit, boolean p_PDCP_StatusReportRequired := true, template (present) EstablishmentCause p_EstablishmentCause := ?, template (omit) RLF_TimersAndConstants_r9 p_RLF_TimersAndConstants_r9 := omit, template (omit) DownlinkAntennaGroupConfig_Type p_DownlinkAntennaGroupConfig := omit, template (omit) DRB_Identity p_DrbFor2ndPDN := omit, template (omit) RRCConnectionReconfiguration_v890_IEs p_RRCConnectionReconfiguration_v890_IEs := omit, template (omit) UplinkDataCompression_r15_Type p_UDC_r15 := omit) runs on EUTRA_PTC { /* @sic R5-144797: new parameter p_DrbFor2ndPDN sic@ */ /* @sic R5s150338: new parameter p_DrbConfigAtSS sic@ */ /* @sic R5s180648: got NetworkFeatureSupport from global variable, rather than passing it in sic@ */ var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var NAS_MSG_Indication_Type v_NasInd; var template (present) NAS_UL_Message_Type v_ServiceRequest; var NAS_KsiValue v_KsiValue; var GutiParameters_Type v_GutiParams; var EPS_M_Tmsi v_Mtmsi; var boolean v_ESRPS; // ===================================================== // step 3-10 of procedure in 36.508 cl. 4.5.3.3 // Step 3 - 5 of Generic RB establishment procedure //Check the Service Request message v_KsiValue := f_EUTRA_SecurityKSIasme_Get(); v_ESRPS := f_EPS_NetworkFeatureSupport_CheckFeature(f_EUTRA_MobileInfo_GetNetworkFeatureSupport(), esrps); // @sic R5s180648 sic@ if (v_ESRPS) { /* @sic R5-177058, R5s170915 sic@ */ v_GutiParams := f_EUTRA_CellInfo_GetGuti(p_CellId); v_Mtmsi := bit2oct(v_GutiParams.M_TMSI); v_ServiceRequest := cr_EXT_SERVICE_REQUEST_Common(v_KsiValue, tsc_EST_PacketService_S1, v_Mtmsi); //@sic R5s180270 sic@ } else { v_ServiceRequest := cr_508_SERVICE_REQUEST(v_KsiValue); } f_EUTRA_RRC_ConnectionSetup_Def (p_CellId, -, v_RRC_TI); v_NasInd := f_EUTRA_RRCConnectionSetupComplete_Def(p_CellId, // @sic R5s120121 MCC160 implementation sic@ cr_508_RRCConnectionSetupComplete(v_RRC_TI, ?, -, -), // @sic R5s110468 sic@ cr_NAS_Indication(tsc_SHT_IntegrityProtected, v_ServiceRequest)); /* Step 6 - 8 */ f_EUTRA_Activate_SRB2_DRB_SendRrcMsg(p_CellId, v_RRC_TI, v_NasInd.SecurityProtection.NasCount, p_NoOfAddDRBs_AM, p_NoOfAddDRBs_UM, p_DrbConfigAtSS, /* @sic R5s150338 sic@ */ p_DRB_ToAddModList, p_MAC_MainConfig, p_PhysicalConfigDedicated, p_QoS, p_Tft, p_PDCP_StatusReportRequired, p_RLF_TimersAndConstants_r9, // @sic R5s120808 sic@ p_DownlinkAntennaGroupConfig, p_DrbFor2ndPDN, p_RRCConnectionReconfiguration_v890_IEs, p_UDC_r15); // @sic R5-175050 sic@ }

# 3	Branches executed

The test case was executed on on EUTRA band 4 using ciphering algorithm eea1 and integrity algorithm eia1

# 4	Execution Log Files

## 4.1	Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE

The Qualcomm Snapdragon® X80 5G Modem-RF System, Qualcomm Technologies’ 6th generation modem-to-antenna 5G solution UE passed this test case on Rohde & Schwarz CMW500. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

tc_8_1_2_5a_Qualcomm.log

(Note: PICS/PIXIT settings are captured at the beginning of the TLI log)

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# 5	References

| [1] | R5s250380: Supporting information for Addition of LTE RRC test case 8.1.2.5a |
| --- | --- |