3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250503

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4951 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add new verified and e-mail agreed TTCN test cases in the TC lists in 36.523-3 (prose), Annex A |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI12_Test |  |  |  |  |  | Date: |  |  | 2025-12-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - New RAN5 e-mail agreed TTCN test cases have not been documented in the test case lists (cut-off date on 1st December 2025). Updating the approved TC list is required. - Annex F for the TTCN type definitions should be updated to document the changes in the TTCN delivery iwd-25wk50 (Friday 12th December 2025). - Test case titles need to be aligned with latest updates. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added the new e-mail agreed TTCN test cases in the test suite lists. - Updated Annex F. - Updated titles of test cases 11.3.1 and 11.3.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The new RAN approved test cases at this meeting will not be included in the ATSs in the upcoming version. The list will be incomplete. - The ASP changes will not be correctly documented. - Test cases titles will not be aligned with other specs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A, Annex F |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Annex A (normative):
Test Suites

This annex contains the approved TTCN Test Suites. The test suites have been produced using the Testing and Test Control Notation version 3 (TTCN-3) according to ES 201 873-1 [13].

# A.1	Baseline of specifications

Table A.1 shows the baseline of the relevant core specifications and the test specifications which the delivered TTCN test suites are referred to.

Table A.1: References of the test and Core specifications

| Core specifications baseline | TS 36.331 [19] |
| --- | --- |
|  | TS 24.301 [21] |
| Test specifications | TS 36.508 [3] |
|  | TS 36.509 [4] |
|  | TS 36.523-1 [1] |
|  | TS 36.523-2 [2] |

# A.2	E-UTRA Test Suites

Table A.2 lists all approved test cases.

For a given test case, the following variants are distinguished (if applicable):

- FDD: E-UTRA FDD mode; and UTRA FDD mode in case the test case is Inter-RAT with UTRA cell(s).

- TDD: E-UTRA TDD mode; and UTRA TDD mode in case the test case is Inter-RAT with UTRA cell(s).

- T/F: E-UTRA TDD mode and UTRA FDD mode; only applicable to Inter-RAT test cases with UTRA cell(s).

An "X" in columns FDD, TDD or T/F indicates the test case is approved for the respective variant.

An "-" in columns FDD, TDD or T/F indicates the test case is not applicable to the respective variant.

Table A.2: E-UTRA/EPS TTCN test cases

| Test case |  | Description |  | FDD |  | TDD |  | T/F |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6.1.1.1 |  | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN/Automatic mode |  | X |  | X |  | - |  |
| 6.1.1.1a |  | PLMN selection / Automatic mode / between FDD and TDD |  | X |  | X |  | - |  |
| 6.1.1.1b |  | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / Single Frequency operation |  | X |  | X |  | - |  |
| 6.1.1.2 |  | PLMN selection of “Other PLMN/access technology combinations” / Automatic mode |  | X |  | X |  | - |  |
| 6.1.1.2a |  | PLMN selection of “Other PLMN/access technology combinations” / Automatic mode / Single Frequency operation |  | X |  | X |  | - |  |
| 6.1.1.3 |  | Cell reselection of ePLMN in manual mode |  | X |  | X |  | - |  |
| 6.1.1.3a |  | Cell reselection of ePLMN in manual mode / between FDD and TDD |  | X |  | X |  | - |  |
| 6.1.1.3b |  | Cell reselection of ePLMN in manual mode / Single Frequency operation |  | X |  | X |  | - |  |
| 6.1.1.4 |  | PLMN selection in shared network environment / Automatic mode |  | X |  | X |  | - |  |
| 6.1.1.4a |  | PLMN selection in shared network environment / Automatic mode / between FDD and TDD |  | X |  | X |  | - |  |
| 6.1.1.6 |  | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / User reselection |  | X |  | X |  | - |  |
| 6.1.1.6a |  | PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / User reselection / Single Frequency operation |  | X |  | X |  | - |  |
| 6.1.1.7 |  | PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer |  | X |  | X |  | - |  |
| 6.1.1.7a |  | PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer / Single Frequency operation |  | X |  | X |  | - |  |
| 6.1.1.8 |  | PLMN selection of RPLMN or (E)HPLMN / Automatic mode |  | X |  | X |  | - |  |
| 6.1.1.9 |  | PLMN selection of RPLMN or (E)HPLMN; Manual mode |  | X |  | X |  | - |  |
| 6.1.2.2 |  | Cell selection, Qrxlevmin |  | X |  | X |  | - |  |
| 6.1.2.2a |  | Cell selection / Qqualmin |  | X |  | X |  | - |  |
| 6.1.2.2b |  | Cell selection / UE Cat 0 not allowed |  | X |  |  |  | - |  |
| 6.1.2.2.c |  | Cell selection / Qrxlevmin / Enhanced coverage |  | X |  | X |  | - |  |
| 6.1.2.2.d |  | Cell selection / Qqualmin / Enhanced coverage |  | X |  | X |  | - |  |
| 6.1.2.3 |  | Cell selection/Intra E-UTRAN/Serving cell becomes non-suitable (S<0 or barred) |  | X |  | X |  | - |  |
| 6.1.2.3a |  | Cell selection / Intra E-UTRAN / Serving cell becomes non-suitable (Srxlev > 0 and Squal <0) |  | X |  | X |  | - |  |
| 6.1.2.4 |  | Cell reselection |  | X |  | X |  | - |  |
| 6.1.2.5 |  | Cell reselection for inter-band operation |  | X |  | X |  | - |  |
| 6.1.2.5a |  | Cell reselection for interband operation/ Power Class 2 UE operation/ Between FDD and TDD |  | X |  | - |  | - |  |
| 6.1.2.5b |  | Cell reselection for interband operation using Pcompensation / Between FDD and TDD |  | X |  | - |  | - |  |
| 6.1.2.5c |  | Inter-band cell reselection / Extended frequency list |  | X |  | X |  | - |  |
| 6.1.2.6 |  | Cell reselection using Qhyst, Qoffset and Treselection |  | X |  | X |  | - |  |
| 6.1.2.6a |  | Cell reselection using Treselection / Enhanced coverage |  | X |  | X |  | - |  |
| 6.1.2.7 |  | Cell reselection/Equivalent PLMN |  | X |  | X |  | - |  |
| 6.1.2.7a |  | Cell reselection / Equivalent PLMN / Single Frequency operation |  | X |  | X |  | - |  |
| 6.1.2.8 |  | Cell reselection using cell status and cell reservations/Access control class 0 to 9 |  | X |  | X |  | - |  |
| 6.1.2.8a |  | Cell reselection using cell status and cell reservations / Access control class 0 to 9 / Single Frequency operation |  | X |  | X |  | - |  |
| 6.1.2.9 |  | Cell reselection using cell status and cell reservations/Access control class 11 to15 |  | X |  | X |  | - |  |
| 6.1.2.9a |  | Cell reselection using cell status and cell reservations / Access control class 11 to 15 / Single Frequency operation |  | X |  | X |  | - |  |
| 6.1.2.10 |  | Cell reselection in shared network environment |  | X |  | X |  | - |  |
| 6.1.2.11 |  | Inter-frequency cell reselection |  | X |  | X |  | - |  |
| 6.1.2.11a |  | Inter-frequency cell reselection / Extended frequency list |  | X |  | X |  | - |  |
| 6.1.2.12 |  | Cell reselection / Cell-specific reselection parameters provided by the network in a neighbouring cell list |  | X |  | X |  | - |  |
| 6.1.2.13 |  | Cell re-selection, Sintrasearch, Snonintrasearch |  | X |  | X |  | - |  |
| 6.1.2.14 |  | Speed-dependent cell reselection |  | X |  | X |  | - |  |
| 6.1.2.15 |  | Inter-frequency cell reselection according to cell reselection priority provided by SIBs |  | X |  | X |  | - |  |
| 6.1.2.15a |  | Inter-frequency cell reselection according to cell reselection priority provided by SIBs / Between FDD and TDD |  | X |  | X |  | - |  |
| 6.1.2.15b |  | Inter-band cell reselection according to cell reselection priority provided by SIBs |  | X |  | X |  | - |  |
| 6.1.2.16 |  | Cell reselection / interband operation / Between FDD and TDD |  | X |  | X |  | - |  |
| 6.1.2.17 |  | Cell reselection for Squal to check against SIntraSearchQ and SnonIntraSearchQ |  | X |  | X |  | - |  |
| 6.1.2.18 |  | Inter-frequency cell reselection based on common priority information with parameters ThreshX, HighQ, ThreshX, LowQ and ThreshServing, LowQ |  | X |  | X |  | - |  |
| 6.1.2.19 |  | Intra-frequency cell reselection / MFBI |  | X |  | X |  | - |  |
| 6.1.2.20 |  | Inter-frequency cell reselection / MFBI |  | X |  | X |  | - |  |
| 6.1.2.21 |  | Inter-band cell reselection / MFBI |  | X |  | X |  | - |  |
| 6.1.2.22 |  | Cell reselection / MFBI / UE does not support multiBandInfoList |  | X |  |  |  | - |  |
| 6.1.2.23 |  | Inter-Band cell reselection / MFBI frequency band priority adjustment/inter-band CA |  | X |  |  |  |  |  |
| 6.2.1.1 |  | Inter-RAT PLMN selection / Selection of correct RAT for OPLMN / Automatic mode |  | X |  | X |  | X |  |
| 6.2.1.2 |  | Inter-RAT PLMN selection / Selection of correct RAT for UPLMN / Automatic mode |  | X |  | X |  | X |  |
| 6.2.1.3 |  | Inter-RAT PLMN selection / Selection of correct PLMN and RAT in shared network environment / Automatic mode |  | X |  | X |  | X |  |
| 6.2.1.4 |  | Inter-RAT PLMN selection / Selection of correct RAT from the OPLMN list / Manual mode |  | X |  | X |  | - |  |
| 6.2.1.6 |  | Inter-RAT background HPLMN search / Search for correct RAT for HPLMN / Automatic mode |  | X |  | X |  | - |  |
| 6.2.2.1 |  | Inter-RAT cell selection/From E-UTRA RRC_IDLE to UTRA_Idle/Serving cell becomes non-suitable |  | X |  | X |  | X |  |
| 6.2.2.2 |  | Inter-RAT cell selection / From E-UTRA RRC_IDLE to GSM_Idle/GPRS Packet_idle / Serving cell becomes non-suitable |  | X |  | X |  | - |  |
| 6.2.2.3 |  | Inter-RAT cell selection / From E-UTRA RRC_IDLE to HRPD Idle / Serving cell becomes non-suitable |  | X |  | X |  | - |  |
| 6.2.2.4 |  | Inter-RAT cell selection / From E-UTRA RRC_IDLE to 1xRTT Dormant / Serving cell becomes non-suitable |  | X |  | X |  | - |  |
| 6.2.2.5 |  | Cell selection / No USIM |  | X |  | X |  | X |  |
| 6.2.2.6 |  | Inter-RAT Cell selection / From GSM_Idle/GPRS Packet_idle to E-UTRA RRC_IDLE / Serving cell becomes non-suitable |  | X |  | X |  | - |  |
| 6.2.2.7 |  | Inter-RAT Cell selection / From GSM_Idle/GPRS Packet_idle to E-UTRA RRC_IDLE / Serving cell is barred |  | X |  | X |  | - |  |
| 6.2.2.8 |  | Inter-RAT cell selection / From UTRA_Idle to E-UTRA RRC_IDLE / Serving cell becomes non-suitable |  | X |  | X |  | X |  |
| 6.2.3.1 |  | Inter-RAT cell reselection / From E-UTRA RRC_IDLE to GSM_Idle/GPRS Packet_Idle |  | X |  | X |  | - |  |
| 6.2.3.1a |  | Inter-RAT cell reselection / From E-UTRA RRC_IDLE to GSM_Idle/GPRS Packet_Idle (Squal < ThreshServing, LowQ, Srxlev > ThreshX, LowP and Srxlev > ThreshX, HighP) |  | X |  | X |  | - |  |
| 6.2.3.3 |  | Inter-RAT cell reselection/From UTRA_Idle to E-UTRA RRC_IDLE |  | X |  | X |  | X |  |
| 6.2.3.3a |  | Inter-RAT cell reselection / From UTRA_Idle to E-UTRA RRC_IDLE (QqualminEUTRA, SqualServingCell < Threshserving,low2, SqualnonServingCell,x > Threshx, low2 and SqualnonServingCell,x > Threshx, high2) |  | X |  | - |  | - |  |
| 6.2.3.4 |  | Inter-RAT cell reselection / From UTRA CELL_PCH state to E-UTRA RRC_IDLE |  | X |  | X |  | X |  |
| 6.2.3.4a |  | Inter-RAT cell reselection / From UTRA_CELL_PCH state to E-UTRA RRC_IDLE based on RSRQ+RSRP evaluation |  | X |  | X |  | X |  |
| 6.2.3.5 |  | Inter-RAT cell reselection/From E-UTRA RRC_IDLE to UTRA_Idle |  | X |  | X |  | X |  |
| 6.2.3.5a |  | Inter-RAT cell reselection / From E-UTRA RRC_IDLE to UTRA_Idle (Squal > ThreshX, HighQ, Squal < ThreshServing, LowQ, Squal > ThreshX, LowQ and SnonIntraSearchQ) |  | X |  | - |  | - |  |
| 6.2.3.6 |  | Inter-RAT cell reselection / From E-UTRA RRC_IDLE to UTRA_Idle according to RAT priority provided by dedicated signalling |  | X |  | X |  | X |  |
| 6.2.3.7 |  | Inter-RAT cell reselection / From E-UTRA RRC_IDLE to HRPD Idle / HRPD cell is higher reselection priority than E-UTRA |  | X |  | X |  | - |  |
| 6.2.3.7a |  | Inter-RAT cell reselection / From E-UTRA RRC_IDLE to HRPD Idle / HRPD cell is higher reselection priority than E-UTRA (Srxlev > ThreshX, HighP) |  | X |  | X |  | - |  |
| 6.2.3.8 |  | Inter-RAT cell reselection / From E-UTRA RRC_IDLE to HRPD Idle / HRPD cell is lower reselection priority than E-UTRA |  | X |  | X |  | - |  |
| 6.2.3.8a |  | Inter-RAT cell reselection / From E-UTRA RRC_IDLE to HRPD Idle / HRPD cell is lower reselection priority than E-UTRA (Squal < ThreshServing, LowQ and Srxlev > ThreshX, LowP) |  | X |  | X |  | - |  |
| 6.2.3.9 |  | Inter-RAT cell reselection / From E-UTRA RRC_IDLE to 1xRTT Dormant / 1xRTT cell is higher reselection priority than E-UTRA |  | X |  | X |  | - |  |
| 6.2.3.9a |  | Inter-RAT cell reselection / From E-UTRA RRC_IDLE to 1xRTT Dormant / 1xRTT cell is higher reselection priority than E-UTRA (Srxlev > ThreshX, HighP) |  | X |  | X |  | - |  |
| 6.2.3.10 |  | Inter-RAT cell reselection / From E-UTRA RRC_IDLE to 1xRTT Dormant / 1xRTT cell is lower reselection priority than E-UTRA |  | X |  | X |  | - |  |
| 6.2.3.10a |  | Inter-RAT cell reselection / From E-UTRA RRC_IDLE to 1xRTT Dormant / 1xRTT cell is lower reselection priority than E-UTRA (Squal < ThreshServing, LowQ and Srxlev > ThreshX, LowP) |  | X |  | X |  | - |  |
| 6.2.3.13 |  | Inter-RAT cell reselection / From UTRA_Idle to E-UTRA RRC_IDLE according to RAT priority provided by dedicated signalling |  | X |  | X |  | X |  |
| 6.2.3.14 |  | Inter-RAT cell reselection / From GSM_Idle/GPRS Packet_Idle to E-UTRA / Priority of E-UTRA cells are higher than the serving cell |  | X |  | X |  | - |  |
| 6.2.3.15 |  | Inter-RAT cell reselection / From GSM_Idle/GPRS Packet_Idle to E-UTRA / Priority of E-UTRA cells are lower than the serving cell |  | X |  | X |  | - |  |
| 6.2.3.16 |  | Inter-RAT cell reselection / From GSM_Idle to E-UTRAN /based on H_PRIO criteria |  | X |  | X |  | - |  |
| 6.2.3.17 |  | Inter-RAT cell reselection / From GSM_Idle/GPRS Packet_Idle to E-UTRA / Priority E-UTRA cells |  | X |  | X |  | - |  |
| 6.2.3.18 |  | Inter-RAT cell reselection / From GSM_Idle/GPRS Packet_Idle to E-UTRA / Not allowed E-UTRA cells |  | X |  | X |  | - |  |
| 6.2.3.19 |  | Inter-RAT cell redirection to E-UTRA cell from GSM TCH mode |  | X |  | X |  | - |  |
| 6.2.3.21 |  | Inter-RAT autonomous cell reselection GPRS Packet_transfer NC0 mode to E-UTRA |  | X |  | X |  | - |  |
| 6.2.3.23 |  | Inter-RAT cell reselection from GPRS Packet_transfer to E-UTRA in CCN mode / PACKET CELL CHANGE CONTINUE |  | X |  | X |  | - |  |
| 6.2.3.24 |  | Inter-RAT cell reselection from GPRS Packet_transfer to E-UTRA in CCN mode / PACKET CELL CHANGE ORDER |  | X |  | X |  | - |  |
| 6.2.3.26 |  | Inter-RAT autonomous cell reselection GPRS Packet_transfer NC1 mode to E-UTRA Cell |  | X |  | X |  | - |  |
| 6.2.3.27 |  | Inter-RAT cell selection from GPRS Packet_transfer to E-UTRA (NC2 mode) |  | X |  | X |  | - |  |
| 6.2.3.28 |  | Inter-RAT cell reselection from GPRS Packet_transfer to E-UTRA |  | X |  | X |  | - |  |
| 6.2.3.29 |  | Inter-RAT Cell Reselection from GPRS Packet_transfer NC1 mode to E-UTRA in CCN mode (PACKET MEASUREMENT ORDER) |  | X |  | X |  | - |  |
| 6.2.3.30 |  | Inter-RAT cell reselection failure from GPRS Packet_transfer to E-UTRA |  | X |  | X |  | - |  |
| 6.2.3.31 |  | Inter-RAT cell reselection / From UTRA_Idle (low priority) to E-UTRA RRC_IDLE (high priority) according to RAT priority provided by dedicated signalling |  | X |  | X |  | X |  |
| 6.2.3.32 |  | Inter-RAT cell re-selection / From E-UTRA RRC_IDLE to UTRA_Idle, Snonintrasearch |  | X |  | X |  | X |  |
| 6.2.3.33 |  | Inter-RAT cell reselection / From E-UTRA RRC_IDLE to UTRA_Idle / Squal based cell reselection parameters are broadcasted in E-UTRAN / UE does not support Squal based cell reselection in UTRAN |  | X |  | X |  | X |  |
| 6.2.3.34 |  | Inter-RAT cell reselection from E-UTRA to UTRA / MFBI |  | X |  |  |  |  |  |
| 6.2.3.35 |  | Inter-RAT cell reselection from UTRA to E-UTRA / MFBI |  | X |  |  |  |  |  |
| 6.2.4.1 |  | Inter-RAT absolute priority based reselection in UTRA CELL_FACH to E-UTRA RRC_IDLE (Higher Priority Layers, Srxlev,x > Threshx,high and Srxlev,serv > Sprioritysearch1 and SqualServ > Sprioritysearch2) |  | X |  | - |  | X |  |
| 6.2.4.2 |  | Inter-RAT absolute priority based reselection in UTRA CELL_FACH (Higher Priority Layers, no cell reselection to E-UTRA RRC_IDLE when Srxlev,serv < Sprioritysearch1) |  | X |  | - |  | X |  |
| 6.2.4.3 |  | Inter-RAT absolute priority based reselection in UTRA CELL_FACH to E-UTRA RRC_IDLE (Higher Priority Layers, Squal,x > Threshx,high2 and Srxlev,serv > Sprioritysearch1 and SqualServ > Sprioritysearch2) |  | X |  | - |  | X |  |
| 6.2.4.4 |  | Inter-RAT absolute priority based reselection in UTRA CELL_FACH (lower priority) to E-UTRA RRC_IDLE (higher priority) (All Layers, Srxlev,x > Threshx,high) |  | X |  | - |  | X |  |
| 6.2.4.5 |  | Inter-RAT absolute priority based reselection in UTRA CELL_FACH (lower priority) to E-UTRA RRC_IDLE (higher priority) (All Layers, Squal,x >ThreshX,high2) |  | X |  | - |  | X |  |
| 6.2.4.6 |  | Inter-RAT absolute priority based reselection in UTRA CELL_FACH (higher priority) to E-UTRA RRC_IDLE (lower priority) (All Layers, Srxlev,serv < Sprioritysearch1 , Srxlev,serv <Thresh serv,low and Srxlev,x > ThreshX,low) |  | X |  | - |  | X |  |
| 6.2.4.7 |  | Inter-RAT absolute priority based reselection in UTRA CELL_FACH (higher priority) to E-UTRA RRC_IDLE (lower priority) (All Layers, Srxlev,serv < Sprioritysearch1 , Squal,serv <Thresh serv,low2 && Squal,x > ThreshX,low2) |  | X |  | - |  | X |  |
| 6.3.1 |  | Inter-frequency cell reselection / From E-UTRA RRC_IDLE non-CSG cell to E-UTRA RRC_IDLE CSG cell |  | X |  | X |  |  |  |
| 6.3.2 |  | Inter-RAT cell reselection / From GSM_Idle/GPRS Packet_Idle to E-UTRA idle CSG cell |  | X |  |  |  | - |  |
| 6.3.3 |  | Inter-RAT cell reselection / From UTRA_Idle to E-UTRA RRC_IDLE CSG cell |  | X |  | X |  | X |  |
| 6.3.4 |  | Inter-RAT cell reselection / From UTRA CELL_PCH state to E-UTRA RRC_IDLE CSG cell |  | X |  |  |  |  |  |
| 6.3.5 |  | Manual support for CSG ID selection |  | X |  | X |  | - |  |
| 6.3.6 |  | Ignoring CSG cells in cell selection/reselection when allowed CSG list is empty or not supported |  | X |  | X |  | - |  |
| 6.3.7 |  | Inter-RAT Cell reselection from E-UTRA idle non-CSG cell to a UTRA CSG cell |  | X |  | X |  | X |  |
| 6.3.9 |  | Manual CSG ID selection across PLMNs |  | X |  | X |  | - |  |
| 6.4.1 |  | Manual CSG ID selection / Hybrid cell whose CSG ID is not in the Allowed CSG list nor Operator’s list |  | X |  | X |  | - |  |
| 6.4.2 |  | Inter-frequency cell reselection / From E-UTRA RRC_IDLE non-CSG cell to E-UTRA RRC_IDLE member hybrid cell |  | X |  | X |  | - |  |
| 6.4.3 |  | Inter-RAT cell reselection / From E-UTRA RRC_IDLE non-CSG cell to UTRA_Idle member hybrid cell |  |  | X |  | X |  | X |
| 6.4.4 |  | Inter-RAT cell reselection / From E-UTRA RRC_IDLE non-member hybrid cell to UTRA_Idle member hybrid cell |  | X |  |  |  | X |  |
| 6.4.5 |  | Inter-RAT cell reselection / From UTRA_Idle to E-UTRA RRC_IDLE member hybrid cell |  | X |  | X |  | X |  |
| 6.4.6 |  | Inter-RAT cell reselection / From UTRA CELL_PCH to E-UTRA RRC_IDLE member hybrid cell |  | X |  |  |  |  |  |
| 6.4.7 |  | Inter-RAT cell reselection / From GSM_Idle/GPRS Packet_Idle to E-UTRA RRC_IDLE member hybrid cell |  | X |  |  |  | - |  |
| 7.1.1.1 |  | CCCH mapped to UL SCH/ DL-SCH/Reserved LCID (Logical Channel ID) |  | X |  | X |  | - |  |
| 7.1.1.1a |  | CCCH mapped to UL SCH/ DL-SCH / UE Cat 0 |  | X |  |  |  | - |  |
| 7.1.1.2 |  | DTCH or DCCH mapped to UL SCH/ DL-SCH/Reserved Logical Channel ID |  | X |  | X |  | - |  |
| 7.1.2.1 |  | Correct selection of RACH parameters/Random access preamble and PRACH resource explicitly signalled to the UE by RRC/Non-contention based random access procedure |  | X |  | X |  | - |  |
| 7.1.2.2 |  | Correct selection of RACH parameters/Random access preamble and PRACH resource explicitly signalled to the UE in PDCCH Order/Non-contention based random access procedure |  | X |  | X |  | - |  |
| 7.1.2.3 |  | Correct selection of RACH parameters/Preamble selected by MAC itself/Contention based random access procedure |  | X |  | X |  | - |  |
| 7.1.2.3a |  | Correct selection of RACH parameters / Preamble selected by MAC itself / Contention based random access procedure / Enhanced Coverage |  | X |  | X |  | - |  |
| 7.1.2.4 |  | Random access procedure/Successful |  | X |  | X |  | - |  |
| 7.1.2.5 |  | Random access procedure/MAC PDU containing multiple RARs |  | X |  | X |  | - |  |
| 7.1.2.6 |  | Maintenance of uplink time alignment |  | X |  | X |  | - |  |
| 7.1.2.7 |  | MAC contention resolution/Temporary C-RNTI |  | X |  | X |  | - |  |
| 7.1.2.8 |  | MAC contention resolution/C-RNTI |  | X |  | X |  | - |  |
| 7.1.2.9 |  | MAC backoff indicator |  | X |  | X |  | - |  |
| 7.1.2.10.1 |  | CA / Random access procedure / SCell / Intra-band Contiguous CA |  | X |  | X |  | - |  |
| 7.1.2.10.2 |  | CA / Random access procedure / SCell / Inter-band CA |  | X |  | X |  | - |  |
| 7.1.2.10.3 |  | CA / Random access procedure / SCell / Intra-band non-contiguous CA |  | X |  |  |  | - |  |
| 7.1.2.11.2 |  | CA / Maintenance of uplink time alignment / Multiple TA / Inter-band CA |  | X |  | X |  | - |  |
| 7.1.3.1 |  | Correct handling of DL assignment/Dynamic case |  | X |  | X |  | - |  |
| 7.1.3.2 |  | Correct handling of DL assignment / Semi-persistent case |  | X |  | X |  | - |  |
| 7.1.3.3 |  | MAC PDU header handling |  | X |  | X |  | - |  |
| 7.1.3.3a |  | MAC PDU header handling / UE with limited TB size |  | X |  | X |  | - |  |
| 7.1.3.4 |  | Correct HARQ process handling/DCCH and DTCH |  | X |  | X |  | - |  |
| 7.1.3.4a |  | Correct HARQ process handling / DCCH and DTCH/ Enhanced Coverage / CE Mode A |  | X |  |  |  | - |  |
| 7.1.3.5 |  | Correct HARQ process handling/CCCH |  | X |  | X |  | - |  |
| 7.1.3.5a |  | Correct HARQ process handling / CCCH/ Enhanced Coverage / CE Mode A |  | X |  | X |  | - |  |
| 7.1.3.6 |  | Correct HARQ process handling/BCCH |  | X |  | X |  | - |  |
| 7.1.3.7 |  | MAC padding |  | X |  | X |  | - |  |
| 7.1.3.9 |  | MAC reset DL |  | X |  | X |  | - |  |
| 7.1.3.11.1 |  | CA / Correct HARQ process handling / DCCH and DTCH / P cell and Scell / Intra-band Contiguous CA |  | X |  | X |  | - |  |
| 7.1.3.11.2 |  | CA / Correct HARQ process handling / DCCH and DTCH / P cell and Scell / Inter-band CA |  | X |  | X |  | - |  |
| 7.1.3.11.3 |  | CA / Correct HARQ process handling / DCCH and DTCH / P cell and Scell / Intra-band non-Contiguous CA |  | X |  |  |  | - |  |
| 7.1.3.11.4 |  | FDD-TDD CA / Correct HARQ process handling / DCCH and DTCH / FDD PCell and TDD SCell |  | X |  |  |  | - |  |
| 7.1.3.11.5 |  | FDD-TDD CA / Correct HARQ process handling / DCCH and DTCH / TDD PCell and FDD SCell |  | X |  |  |  | - |  |
| 7.1.3.12 |  | TDD additional special subframe configuration / Special subframe pattern 9 with Normal Cyclic Prefix / CRS based transmission scheme |  | - |  | X |  | - |  |
| 7.1.3.12a |  | TDD additional special subframe configuration / Special subframe pattern 7 with Extended Cyclic Prefix / CRS based transmission scheme |  | - |  | X |  | - |  |
| 7.1.3.13 |  | TDD additional special subframe configuration / Special subframe pattern 9 with Normal Cyclic Prefix / UE-specific reference signals based transmission scheme |  | - |  | X |  | - |  |
| 7.1.3.13a |  | TDD additional special subframe configuration / Special subframe pattern 7 with Extended Cyclic Prefix / UE-specific reference signals based transmission scheme |  | - |  | X |  | - |  |
| 7.1.4.1 |  | Correct handling of UL assignment/Dynamic case |  | X |  | X |  | - |  |
| 7.1.4.1a |  | Correct handling of UL assignment / Dynamic case / Skip padding transmissions |  | X |  |  |  | - |  |
| 7.1.4.2 |  | Correct handling of UL assignment / Semi-persistent case |  | X |  | X |  | - |  |
| 7.1.4.3 |  | Logical channel prioritization handling |  | X |  | X |  | - |  |
| 7.1.4.4 |  | Correct handling of MAC control information/Scheduling requests and PUCCH |  | X |  | X |  | - |  |
| 7.1.4.5 |  | Correct handling of MAC control information/Scheduling requests and random access procedure |  | X |  | X |  | - |  |
| 7.1.4.6 |  | Correct handling of MAC control information/Buffer status/UL data arrive in the UE Tx buffer and retransmission of BSR/Regular BSR |  | X |  | X |  | - |  |
| 7.1.4.7 |  | Correct handling of MAC control information/Buffer Status/UL resources are allocated/Padding BSR |  | X |  | X |  | - |  |
| 7.1.4.7a |  | Correct handling of MAC control information / Buffer Status / UL resources are allocated / Cancellation of Padding BSR |  | X |  | X |  | - |  |
| 7.1.4.8 |  | Correct handling of MAC control information/Buffer status/Periodic BSR timer expires |  | X |  | X |  | - |  |
| 7.1.4.10 |  | MAC padding |  | X |  | X |  | - |  |
| 7.1.4.11 |  | Correct HARQ process handling |  | X |  | X |  | - |  |
| 7.1.4.12 |  | MAC reset UL |  | X |  | X |  | - |  |
| 7.1.4.13 |  | MAC PDU header handling |  | X |  | X |  | - |  |
| 7.1.4.14 |  | Correct HARQ process handling / TTI bundling |  | X |  | X |  | - |  |
| 7.1.4.15 |  | UE power headroom reporting/Periodic reporting |  | X |  | X |  | - |  |
| 7.1.4.16 |  | UE power headroom Reporting/DL pathloss change reporting |  | X |  | X |  | - |  |
| 7.1.4.18 |  | CA / Correct handling of MAC control information / Buffer Status / UL data arrive in the UE Tx buffer / Extended buffer size |  | X |  | X |  | - |  |
| 7.1.4.19.1 |  | CA / UE power headroom reporting / SCell activation and DL pathloss change reporting / Extended PHR / Intra-band Contiguous |  | X |  | X |  | - |  |
| 7.1.4.19.2 |  | CA / UE power headroom reporting / SCell activation and DL pathloss change reporting / Extended PHR / Inter-band CA |  | X |  | X |  | - |  |
| 7.1.4.19.3 |  | CA / UE power headroom reporting / SCell activation and DL pathloss change reporting / Extended PHR / Intra-band non-Contiguous CA |  | X |  | X |  | - |  |
| 7.1.4.20.1 |  | CA / Correct handling of MAC control information / Buffer status / Intra-band Contiguous CA |  | X |  | X |  | - |  |
| 7.1.4.20.2 |  | CA / Correct handling of MAC control information / Buffer status / Inter-band CA |  | X |  | X |  | - |  |
| 7.1.4.20.3 |  | CA / Correct handling of MAC control information / Buffer status / Intra-band non-Contiguous CA |  | X |  |  |  | - |  |
| 7.1.4.21 |  | CA / UE power headroom reporting / Extended PHR |  | X |  | X |  | - |  |
| 7.1.4.23 |  | Correct HARQ process handling / TTI bundling with enhanced HARQ pattern |  | X |  |  |  | - |  |
| 7.1.4.24 |  | Correct HARQ process handling / TTI bundling without resource allocation restriction |  | X |  | X |  | - |  |
| 7.1.4.24b |  | Correct HARQ process handling / Enhanced Coverage / CE Mode A |  | X |  |  |  | - |  |
| 7.1.4a.1 |  | Correct downlink reception and uplink transmission when specific valid subframes is signalled for BL UE |  | X |  | X |  | - |  |
| 7.1.5.1 |  | Inter-TTI PUSCH hopping by uplink grant |  | X |  | X |  | - |  |
| 7.1.5.2 |  | Predefined intra-TTI PUSCH hopping (N_sb=1) |  | X |  | X |  | - |  |
| 7.1.5.3 |  | Predefined intra-TTI PUSCH hopping (N_sb=2/3/4) |  | X |  | X |  | - |  |
| 7.1.5.4 |  | Predefined inter-TTI PUSCH hopping (N_sb=1) |  | X |  | X |  | - |  |
| 7.1.5.5 |  | Predefined inter-TTI PUSCH hopping (N_sb=2/3/4) |  | X |  | X |  | - |  |
| 7.1.6.1 |  | DRX operation/Short cycle not configured/Parameters configured by RRC |  | X |  | X |  | - |  |
| 7.1.6.1a |  | DRX operation / Short cycle not configured / Parameters configured by RRC / Enhanced Coverage / CE Mode A |  | X |  | X |  | - |  |
| 7.1.6.2 |  | DRX operation/Short cycle not configured/DRX command MAC control element reception |  | X |  | X |  | - |  |
| 7.1.6.3 |  | DRX operation / Short cycle configured / Parameters configured by RRC |  | X |  | X |  | - |  |
| 7.1.6.4 |  | DRX Operation / Short cycle configured / DRX command MAC control element reception |  | X |  | X |  | - |  |
| 7.1.6.5 |  | eDRX operation / Long cycle configured / Parameters configured by RRC |  | X |  | X |  | - |  |
| 7.1.7.1.1 |  | DL-SCH transport block size selection/DCI format 1/RA type 0 |  | X |  | X |  | - |  |
| 7.1.7.1.2 |  | DL-SCH transport block size selection/DCI format 1/RA type 1 |  | X |  | X |  | - |  |
| 7.1.7.1.3 |  | DL-SCH transport block size selection/DCI format 1A/RA type 2/Localised VRB |  | X |  | X |  | - |  |
| 7.1.7.1.4 |  | DL-SCH transport block size selection/DCI format 1A/RA type 2/Distributed VRB |  | X |  | X |  | - |  |
| 7.1.7.1.5 |  | DL-SCH transport block size selection / DCI format 2A / RA type 0 / Two transport blocks enabled / Transport block to codeword swap flag value set to 0 |  | X |  | X |  | - |  |
| 7.1.7.1.6 |  | DL-SCH transport block size selection / DCI format 2A / RA type 1/ Two transport blocks enabled / Transport block to codeword swap flag value set to 1 |  | X |  | X |  | - |  |
| 7.1.7.1.6a |  | DL-SCH transport block size selection / DCI format 2A / RA type 0 and RA type 1 / Two transport blocks enabled / 3 and 4 Layer Spatial Multiplexing |  | X |  | X |  | - |  |
| 7.1.7.1.7 |  | DL-SCH transport block size selection / DCI format 1 / RA type 0 / 256QAM |  | X |  | X |  | - |  |
| 7.1.7.1.8 |  | DL-SCH transport block size selection / DCI format 1 / RA type 1 / 256QAM |  | X |  | X |  | - |  |
| 7.1.7.1.9 |  | DL-SCH transport block size selection / DCI format 1B / RA type 2 / Localised VRB / 256QAM |  | X |  | X |  | - |  |
| 7.1.7.1.10 |  | DL-SCH transport block size selection / DCI format 1B / RA type 2 / Distributed VRB / 256QAM |  | X |  | X |  | - |  |
| 7.1.7.1.11 |  | DL-SCH transport block size selection / DCI format 2A / RA type 0 / Two transport blocks enabled / Transport block to codeword swap flag value set to ’0’ / 256QAM |  | X |  |  |  | - |  |
| 7.1.7.1.12 |  | DL-SCH Transport Block Size selection / DCI format 2A / RA type 1 / Two transport blocks enabled / Transport block to codeword swap flag value set to ’1’ / 256QAM |  | X |  | X |  | - |  |
| 7.1.7.1.12a |  | DL-SCH transport block size selection / DCI format 2A / RA type 0 and RA type 1 / Two transport blocks enabled / 3 and 4 Layer Spatial Multiplexing / 256QAM |  | X |  | X |  | - |  |
| 7.1.7.1.13 |  | DL-SCH transport block size selection / DCI format 6-1A / RA type 2 / Localised VRB |  | X |  | X |  | - |  |
| 7.1.7.2.1 |  | UL-SCH transport block size selection/DCI format 0 |  | X |  | X |  | - |  |
| 7.1.7.2.2 |  | UL-SCH transport block size selection / DCI format 6-0A |  | X |  | X |  | - |  |
| 7.1.8.1 |  | Periodic RI reporting using PUCCH / Category 1 UE / Transmission mode 3/4 |  | X |  | X |  | - |  |
| 7.1.9.1.1 |  | CA / Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer/ Intra-band Contiguous CA |  | X |  | X |  | - |  |
| 7.1.9.1.2 |  | CA / Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer/ Inter-band CA |  | X |  | X |  | - |  |
| 7.1.9.1.3 |  | CA / Activation/Deactivation of SCells / Activation/Deactivation MAC control element reception / sCellDeactivationTimer / Intra-band non-Contiguous CA |  | X |  | X |  | - |  |
| 7.1.10.1 |  | Sending SR on PUCCH with DMRS generated by using virtual cell identity / nPUCCH-Identity |  | X |  | X |  | - |  |
| 7.1.10.2 |  | 7.1.10.2 Transmitting data on PUSCH with DMRS generated by using virtual cell identity / nPUSCH-Identity |  | X |  | X |  | - |  |
| 7.1.11.1 |  | LAA transmits common control information in PDCCH scrambled with CC-RNTI |  | X |  |  |  | - |  |
| 7.2.2.1 |  | UM RLC/Segmentation and reassembly/5-bit SN/Framing info field |  | X |  | X |  | - |  |
| 7.2.2.2 |  | UM RLC/Segmentation and reassembly/10-bit SN/Framing info field |  | X |  | X |  | - |  |
| 7.2.2.3 |  | UM RLC/Reassembly/5-bit SN/LI value > PDU size |  | X |  | X |  | - |  |
| 7.2.2.4 |  | UM RLC/Reassembly/10-bit SN/LI value > PDU size |  | X |  | X |  | - |  |
| 7.2.2.5.1 |  | UM RLC/5-bit SN/Correct use of sequence numbering |  | X |  | X |  | - |  |
| 7.2.2.5.2 |  | UM RLC/5-bit SN/Correct use of sequence numbering |  | X |  | X |  | - |  |
| 7.2.2.6 |  | UM RLC/Concatenation, segmentation and reassembly |  | X |  | X |  | - |  |
| 7.2.2.7 |  | UM RLC/In sequence delivery of upper layer PDUs without residual loss of RLC PDUs/Maximum re-ordering delay below t-Reordering |  | X |  | X |  | - |  |
| 7.2.2.8 |  | UM RLC/In sequence delivery of upper layer PDUs without residual loss of RLC PDUs/Maximum re-ordering delay exceeds t-Reordering |  | X |  | X |  | - |  |
| 7.2.2.9 |  | UM RLC/In sequence delivery of upper layer PDUs with residual loss of RLC PDUs/Maximum re-ordering delay exceeds t-Reordering |  | X |  | X |  | - |  |
| 7.2.2.10 |  | UM RLC/Duplicate detection of RLC PDUs |  | X |  | X |  | - |  |
| 7.2.2.11 |  | UM RLC/RLC re-establishment procedure |  | X |  | X |  | - |  |
| 7.2.3.1 |  | AM RLC/Concatenation and reassembly |  | X |  | X |  | - |  |
| 7.2.3.2 |  | AM RLC/Segmentation and reassembly/No PDU segmentation |  | X |  | X |  | - |  |
| 7.2.3.3 |  | AM RLC/Segmentation and reassembly/Framing info field |  | X |  | X |  | - |  |
| 7.2.3.4 |  | AM RLC/Segmentation and reassembly/Different numbers of length indicators |  | X |  | X |  | - |  |
| 7.2.3.5 |  | AM RLC/Reassembly/LI value > PDU size |  | X |  | X |  | - |  |
| 7.2.3.6 |  | AM RLC/Correct use of sequence numbering |  | X |  | X |  | - |  |
| 7.2.3.7 |  | AM RLC/Control of transmit window |  | X |  | X |  | - |  |
| 7.2.3.8 |  | AM RLC/Control of receive window |  | X |  | X |  | - |  |
| 7.2.3.9 |  | AM RLC/Polling for status |  | X |  | X |  | - |  |
| 7.2.3.10 |  | AM RLC/Receiver status triggers |  | X |  | X |  | - |  |
| 7.2.3.13 |  | AM RLC/Reconfiguration of RLC parameters by upper layers |  | X |  | X |  | - |  |
| 7.2.3.14 |  | AM RLC/In sequence delivery of upper layers PDUs |  | X |  | X |  | - |  |
| 7.2.3.15 |  | AM RLC/Re-ordering of RLC PDU segments |  | X |  | X |  | - |  |
| 7.2.3.16 |  | AM RLC/Re-transmission of RLC PDU without re-segmentation |  | X |  | X |  | - |  |
| 7.2.3.17 |  | AM RLC/Re-segmentation RLC PDU/SO, FI, LSF |  | X |  | X |  | - |  |
| 7.2.3.18 |  | AM RLC/Reassembly/AMD PDU reassembly from AMD PDU segments, Segmentation Offset and Last Segment Flag fields |  | X |  | X |  | - |  |
| 7.2.3.20 |  | AM RLC/Duplicate detection of RLC PDUs |  | X |  | X |  | - |  |
| 7.2.3.21 |  | AM RLC/RLC re-establishment at RRC connection reconfiguration including mobilityControlInfo IE |  | X |  | X |  | - |  |
| 7.3.1.1 |  | Maintenance of PDCP sequence numbers/User plane/RLC AM |  | X |  | X |  | - |  |
| 7.3.1.2 |  | Maintenance of PDCP sequence numbers/User plane/RLC UM/Short PDCP SN (7 bits) |  | X |  | X |  | - |  |
| 7.3.1.3 |  | Maintenance of PDCP sequence numbers/User plane/RLC UM/Long PDCP SN (12 bits) |  | X |  | X |  | - |  |
| 7.3.3.1 |  | Ciphering and deciphering/Correct functionality of EPS AS encryption algorithms/SNOW 3G |  | X |  | X |  | - |  |
| 7.3.3.2 |  | Ciphering and deciphering/Correct functionality of EPS UP encryption algorithms/SNOW 3G |  | X |  | X |  | - |  |
| 7.3.3.3 |  | Ciphering and deciphering/Correct functionality of EPS AS encryption algorithms/AES |  | X |  | X |  | - |  |
| 7.3.3.4 |  | Ciphering and deciphering/Correct functionality of EPS UP encryption algorithms/AES |  | X |  | X |  | - |  |
| 7.3.3.5 |  | Ciphering and deciphering / Correct functionality of EPS AS encryption algorithms / ZUC |  | X |  | X |  | - |  |
| 7.3.3.6 |  | Ciphering and deciphering / Correct functionality of EPS AS encryption algorithms / ZUC |  | X |  | X |  | - |  |
| 7.3.4.1 |  | Integrity protection/Correct functionality of EPS AS integrity algorithms/SNOW 3G |  | X |  | X |  | - |  |
| 7.3.4.2 |  | Integrity protection/Correct functionality of EPS AS integrity algorithms/AES |  | X |  | X |  | - |  |
| 7.3.4.3 |  | Integrity protection / Correct functionality of EPS AS integrity algorithms / ZUC |  | X |  | X |  | - |  |
| 7.3.5.2 |  | PDCP handover/Lossless handover/PDCP sequence number maintenance |  | X |  | X |  | - |  |
| 7.3.5.3 |  | PDCP handover/Non-lossless handover/PDCP sequence number maintenance |  | X |  | X |  | - |  |
| 7.3.5.4 |  | PDCP handover/Lossless handover/PDCP status report to convey the information on missing or acknowledged PDCP SDUs at handover |  | X |  | X |  | - |  |
| 7.3.5.5 |  | PDCP handover/In-order delivery and duplicate elimination in the downlink |  | X |  | X |  | - |  |
| 7.3.6.1 |  | PDCP discard |  | X |  | X |  | - |  |
| 8.1.1.1 |  | RRC/Paging for connection in idle mode |  | X |  | X |  | - |  |
| 8.1.1.1a |  | RRC / Direct Indication Information / Notification of BCCH modification in idle mode |  | X |  |  |  | - |  |
| 8.1.1.2 |  | RRC/Paging for notification of BCCH modification in idle mode |  | X |  | X |  | - |  |
| 8.1.1.2a |  | RRC / Paging for notification of BCCH modification in idle mode / DRX cycle longer than the modification period / eDRX cycle with eDRX Allowed / Not Allowed |  | X |  |  |  | - |  |
| 8.1.1.3 |  | RRC / Paging for connection in idle mode / Multiple paging records |  | X |  | X |  | - |  |
| 8.1.1.4 |  | RRC / Paging for connection in idle mode / Shared network environment |  | X |  | X |  | - |  |
| 8.1.1.6 |  | RRC/BCCH modification in connected mode |  | X |  | X |  | - |  |
| 8.1.1.7 |  | RRC / Paging / EAB active |  | X |  | X |  | - |  |
| 8.1.2.2 |  | RRC connection establishment/Reject with wait time |  | X |  | X |  | - |  |
| 8.1.2.3 |  | RRC connection establishment/Return to idle state after T300 timeout |  | X |  | X |  | - |  |
| 8.1.2.5 |  | RRC connection establishment/0% access probability for MO calls, no restriction for MO signalling |  | X |  | X |  | - |  |
| 8.1.2.5a |  | RRC connection establishment / 0% access probability for MO data, no restriction for MO signalling / AC-Barring per PLMN |  | X |  |  |  | - |  |
| 8.1.2.6 |  | RRC connection establishment / Non-zero percent access probability for MO calls, no restriction for MO signalling |  | X |  | X |  | - |  |
| 8.1.2.7 |  | RRC connection establishment/0% access probability for AC 0 to 9, AC 10 is barred, AC 11 to 15 are not barred, access for UE with access class in the range 11 to 15 is allowed |  | X |  | X |  | - |  |
| 8.1.2.7a |  | RRC connection establishment / 0% access probability for AC 0 to 9, AC 10 is barred, AC 11 to 15 are not barred, access for UE with access class in the range 11 to 15 is allowed / AC-Barring per PLMN |  | X |  |  |  | - |  |
| 8.1.2.8 |  | RRC connection establishment / Range of access baring time |  | X |  | X |  | - |  |
| 8.1.2.9 |  | RRC Connection Establishment / 0% access probability for MO calls, non-zero percent access probability for MO signalling |  | X |  | X |  | - |  |
| 8.1.2.9a |  | RRC Connection Establishment / 0% access probability for MO data, non-zero percent access probability for MO signalling / AC-Barring per PLMN |  | X |  |  |  | - |  |
| 8.1.2.11 |  | RRC connection establishment of emergency call |  | X |  | X |  | - |  |
| 8.1.2.12 |  | RRC connection establishment of emergency call / Limited Service |  | X |  | X |  | - |  |
| 8.1.2.13 |  | RRC connection establishment / 0% access probability for MO calls, 0% access probability for MO signalling |  | X |  | X |  | - |  |
| 8.1.2.13a |  | RRC connection establishment / 0% access probability for MO calls, 0% access probability for MO signalling / AC-Barring per PLMN |  | X |  |  |  | - |  |
| 8.1.2.14 |  | RRC connection establishment / High speed flag |  | X |  | X |  | - |  |
| 8.1.2.15 |  | RRC connection establishment / Extended and spare fields in SI |  | X |  |  |  | - |  |
| 8.1.3.1 |  | RRC connection release/Success |  | X |  | X |  | - |  |
| 8.1.3.4 |  | RRC connection release/Redirection to another E-UTRAN frequency |  | X |  | X |  | - |  |
| 8.1.3.5 |  | RRC connection release/Success/With priority information |  | X |  | X |  | - |  |
| 8.1.3.5a |  | RRC connection release / Success / With extended priority information |  | X |  | X |  | - |  |
| 8.1.3.6 |  | RRC connection release/Redirection from E-UTRAN to UTRAN |  | X |  | X |  | X |  |
| 8.1.3.6a |  | RRC connection release / Redirection from E-UTRAN to UTRAN / Pre-redirection info |  | X |  | X |  | X |  |
| 8.1.3.7 |  | RRC connection release / Redirection from UTRAN to E-UTRAN |  | X |  | X |  | X |  |
| 8.1.3.8 |  | RRC connection release / Redirection from E-UTRAN to GERAN |  | X |  | X |  | - |  |
| 8.1.3.9 |  | RRC connection release / Redirection from E-UTRAN to HRPD |  | X |  | X |  | - |  |
| 8.1.3.10 |  | RRC connection release / Redirection from E-UTRAN to 1xRTT |  | X |  | X |  | - |  |
| 8.1.3.11 |  | RRC connection release / Redirection to another E-UTRAN band |  | X |  | X |  | - |  |
| 8.1.3.11a |  | RRC connection release / Redirection to another E-UTRAN band / Between FDD and TDD |  | X |  | X |  | - |  |
| 8.1.3.12 |  | RRC connection release / Success / With priority information / Inter-band |  | X |  | X |  | - |  |
| 8.1.3.12a |  | RRC connection release / Success / With priority information / Inter-band / Between FDD and TDD |  | X |  | X |  | - |  |
| 8.1.3.12b |  | RRC connection release / Success / With priority information / Inter-band(Single frequency operation in source band) |  | X |  | X |  | - |  |
| 8.2.1.1 |  | RRC connection reconfiguration/Radio bearer establishment for transition from RRC_IDLE to RRC_CONNECTED/Success/Default bearer/Early bearer establishment |  | X |  | X |  | - |  |
| 8.2.1.3 |  | RRC connection reconfiguration/Radio bearer establishment/Success/Dedicated bearer |  | X |  | X |  | - |  |
| 8.2.1.5 |  | RRC connection reconfiguration / Radio bearer establishment for transition from RRC_IDLE to RRC CONNECTED / Success / Latency check |  | X |  | X |  | - |  |
| 8.2.1.6 |  | RRC connection reconfiguration / Radio bearer establishment for transition from RRC_IDLE to RRC CONNECTED / Success / Latency check / SecurityModeCommand and RRCConnectionReconfiguration transmitted in the same TTI |  | X |  | X |  | - |  |
| 8.2.1.7 |  | RRC connection reconfiguration/Radio bearer establishment/Success/SRB2 |  | X |  | X |  | - |  |
| 8.2.1.8 |  | RRC connection reconfiguration / Radio bearer establishment / Success / Dedicated bearer / ROHC configured |  | X |  | X |  | - |  |
| 8.2.2.1 |  | RRC connection reconfiguration/Radio resource reconfiguration/Success |  | X |  | X |  | - |  |
| 8.2.2.2 |  | RRC connection reconfiguration/SRB/DRB reconfiguration/Success |  | X |  | X |  | - |  |
| 8.2.2.3.1 |  | CA / RRC connection reconfiguration / SCell addition/ modification/release / Success / Intra-band contiguous CA |  | X |  | X |  | - |  |
| 8.2.2.3.2 |  | CA / RRC connection reconfiguration / SCell addition/ modification/release / Success / Inter-band CA |  | X |  | X |  | - |  |
| 8.2.2.3.3 |  | CA / RRC connection reconfiguration / SCell addition/ modification/release / Success / Intra-band non-contiguous CA |  | X |  | X |  | - |  |
| 8.2.2.4.1 |  | CA / RRC connection reconfiguration / SCell SI change / Success / Intra-band Contiguous CA |  | X |  | X |  | - |  |
| 8.2.2.4.2 |  | CA / RRC connection reconfiguration / SCell SI change / Success / Inter-band CA |  | X |  | X |  | - |  |
| 8.2.2.4.3 |  | CA / RRC connection reconfiguration / SCell SI change / Success / Intra-band non-contiguous CA |  | X |  | X |  | - |  |
| 8.2.2.5.1 |  | CA / RRC connection reconfiguration / SCell addition without UL / Success / Intra-band contiguous CA |  | X |  | X |  | - |  |
| 8.2.2.5.2 |  | CA / RRC connection reconfiguration / SCell addition without UL / Success / Inter-band CA |  | X |  | X |  | - |  |
| 8.2.2.5.3 |  | CA / RRC connection reconfiguration / SCell addition without UL / Success / Intra-band non-contiguous CA |  | X |  | X |  | - |  |
| 8.2.2.6.1 |  | RRC connection reconfiguration/ UE Assistance Information/power preference indication setup and release |  | X |  | X |  | - |  |
| 8.2.2.6.2 |  | RRC connection reconfiguration/ UE Assistance Information/power preference indication release on connection re-establishment |  | X |  | X |  | - |  |
| 8.2.2.6.3 |  | RRC connection reconfiguration/ UE Assistance Information/T340 running |  | X |  | X |  | - |  |
| 8.2.2.7.1 |  | CA / RRC connection reconfiguration / sTAG addition/ modification/release / Success / Intra-band Contiguous CA |  | X |  | X |  | X |  |
| 8.2.2.7.2 |  | CA / RRC connection reconfiguration / sTAG addition/ modification/release / Success / Inter-band CA |  | X |  | X |  | - |  |
| 8.2.2.7.3 |  | CA / RRC connection reconfiguration / sTAG addition/ modification/release / Success / Intra-band non-contiguous CA |  |  |  | X |  | - |  |
| 8.2.2.8 |  | RRC connection reconfiguration / SIB1 information / Success |  | X |  | X |  | - |  |
| 8.2.3.1 |  | RRC connection reconfiguration/Radio bearer release/Success |  | X |  | X |  | - |  |
| 8.2.4.1 |  | RRC connection reconfiguration/Handover/Success/Dedicated preamble |  | X |  | X |  | - |  |
| 8.2.4.2 |  | RRC connection reconfiguration/Handover/Success/Common preamble |  | X |  | X |  | - |  |
| 8.2.4.3 |  | RRC connection reconfiguration/Handover/Success/Intra-cell/Security reconfiguration |  | X |  | X |  | - |  |
| 8.2.4.4 |  | RRC connection reconfiguration/Handover/Failure/Intra-cell/Security reconfiguration |  | X |  | X |  | - |  |
| 8.2.4.5 |  | RRC connection reconfiguration/Handover/All parameters included |  | X |  | X |  | - |  |
| 8.2.4.6 |  | RRC connection reconfiguration/Handover/Success/Inter-frequency |  | X |  | X |  | - |  |
| 8.2.4.7 |  | RRC connection reconfiguration/Handover/Failure/Re-establishment successful |  | X |  | X |  | - |  |
| 8.2.4.8 |  | RRC connection reconfiguration / Handover / Failure / Re-establishment failure |  | X |  | X |  | - |  |
| 8.2.4.9 |  | RRC connection reconfiguration/Handover/Inter-band blind handover/Success |  | X |  | X |  | - |  |
| 8.2.4.10 |  | RRC connection reconfiguration / Handover / Between FDD and TDD |  | X |  | X |  | - |  |
| 8.2.4.12 |  | RRC connection reconfiguration / Handover / Setup and release of MIMO |  | X |  | X |  | - |  |
| 8.2.4.13 |  | RRC connection reconfiguration / Handover / Success (with measurement) / Inter-band |  | X |  | X |  | - |  |
| 8.2.4.13a |  | RRC connection reconfiguration / Handover / Success (with measurement) / Inter-band / between FDD and TDD |  | X |  | X |  | - |  |
| 8.2.4.14 |  | RRC connection reconfiguration / Handover / Failure / Re-establishment successful / Inter-band |  | X |  | X |  | - |  |
| 8.2.4.14a |  | RRC connection reconfiguration / Handover / Failure / Re-establishment successful / Inter-band / between FDD and TDD |  | X |  | X |  | - |  |
| 8.2.4.15 |  | RRC connection reconfiguration / Handover / Failure / Re-establishment failure / Inter-band |  | X |  | X |  | - |  |
| 8.2.4.15a |  | RRC connection reconfiguration / Handover / Failure / Re-establishment failure / Inter-band / Between FDD and TDD |  | X |  | X |  | - |  |
| 8.2.4.16.1 |  | CA / RRC connection reconfiguration / Setup and Change of MIMO / Intra-band Contiguous CA |  | X |  | X |  | - |  |
| 8.2.4.16.2 |  | CA / RRC connection reconfiguration / Setup and Change of MIMO / Inter-band CA |  | X |  | X |  | - |  |
| 8.2.4.16.3 |  | CA / RRC connection reconfiguration / Setup and Change of MIMO / Intra-band non-contiguous CA |  | X |  | X |  | - |  |
| 8.2.4.17.1 |  | CA / RRC connection reconfiguration / Handover / Success / PCell Change and SCell addition / Intra-band contiguous CA |  | X |  | X |  | - |  |
| 8.2.4.17.2 |  | CA / RRC connection reconfiguration / Handover / Success / PCell Change and SCell addition / Inter-band CA |  | X |  | X |  | - |  |
| 8.2.4.17.3 |  | CA / RRC connection reconfiguration / Handover / Success / PCell Change and SCell addition / Intra-band non-contiguous CA |  | X |  | X |  | - |  |
| 8.2.4.18.1 |  | CA / RRC connection reconfiguration / Handover / Success / SCell release / Intra-band contiguous CA |  | X |  | X |  | - |  |
| 8.2.4.18.2 |  | CA / RRC connection reconfiguration / Handover / Success / SCell release / Inter-band CA |  | X |  | X |  | - |  |
| 8.2.4.18.3 |  | CA / RRC connection reconfiguration / Handover / Success / SCell release / Intra-band non-contiguous CA |  | X |  | X |  | - |  |
| 8.2.4.19.1 |  | CA / RRC connection reconfiguration / Handover / Success / PCell Change/ Scell no Change / Intra-band contiguous CA |  | X |  | X |  | - |  |
| 8.2.4.19.2 |  | CA / RRC connection reconfiguration / Handover / Success / PCell Change/ Scell no Change / Inter-band CA |  | X |  | X |  | - |  |
| 8.2.4.19.3 |  | CA / RRC connection reconfiguration / Handover / Success / PCell Change/ Scell no Change / Intra-band non-contiguous CA |  | X |  | X |  | - |  |
| 8.2.4.20.1 |  | CA / RRC connection reconfiguration / Handover / Success / Scell Change / Intra-band Contiguous CA |  | X |  | X |  | - |  |
| 8.2.4.20.2 |  | CA / RRC connection reconfiguration / Handover / Success / Scell Change / Inter-band CA |  | X |  | X |  | - |  |
| 8.2.4.20.3 |  | CA / RRC connection reconfiguration / Handover / Success / SCell Change / Intra-band non-Contiguous CAChange / Intra-band non-contiguous CA |  | X |  | X |  | - |  |
| 8.2.4.21.1 |  | CA / RRC connection reconfiguration / Handover / Success / SCell release / Intra-band Contiguous CA |  | X |  | X |  | - |  |
| 8.2.4.21.2 |  | CA / RRC connection reconfiguration / Handover / Success / SCell release / Inter-band CA |  | X |  | X |  | - |  |
| 8.2.4.21.3 |  | CA / RRC connection reconfiguration / Handover / Success / SCell release / Intra-band non-contiguous CA |  | X |  | X |  | - |  |
| 8.2.4.22 |  | RRC connection reconfiguration / Handover / MFBI / Target cell broadcasting information disregarded by the UE |  | X |  | X |  | - |  |
| 8.2.4.23.1 |  | CA / RRC connection reconfiguration / Handover / Failure / Re-establishment successful / Intra-band Contiguous CA |  | X |  | X |  | - |  |
| 8.2.4.23.2 |  | CA / RRC connection reconfiguration / Handover / Failure / Re-establishment successful / Inter-band CA |  | X |  | X |  | - |  |
| 8.2.4.23.3 |  | CA / RRC connection reconfiguration / Handover / Failure / Re-establishment successful / Intra-band non-Contiguous CA |  | X |  | X |  | - |  |
| 8.2.4.27 |  | RRC connection reconfiguration / Handover / Success / Intra-frequency in Enhanced Coverage |  | X |  | X |  | - |  |
| 8.2.4.28 |  | eCall Only mode / RRC connection reconfiguration / Inter-frequency Handover / Success |  | X |  |  |  | - |  |
| 8.3.1.1 |  | Measurement configuration control and reporting/Intra E-UTRAN measurements/Event A1 |  | X |  | X |  | - |  |
| 8.3.1.2 |  | Measurement configuration control and reporting/Intra E-UTRAN measurements/Event A2 |  | X |  | X |  | - |  |
| 8.3.1.3 |  | Measurement configuration control and reporting/Intra E-UTRAN measurements/Two simultaneous events A3 (intra and inter-frequency measurements) |  | X |  | X |  | - |  |
| 8.3.1.3a |  | Measurement configuration control and reporting / Intra E-UTRAN measurements / Two simultaneous events A3 (intra and inter-frequency measurements) / RSRQ based measurements |  | X |  | X |  | - |  |
| 8.3.1.4 |  | Measurement configuration control and reporting/Intra E-UTRAN measurements/Periodic reporting (intra and inter-frequency measurements) |  | X |  | X |  | - |  |
| 8.3.1.5 |  | Measurement configuration control and reporting/Intra E-UTRAN measurements/Two simultaneous event A3 (intra-frequency measurements) |  | X |  | X |  | - |  |
| 8.3.1.6 |  | Measurement configuration control and reporting / Intra E-UTRAN measurements / Two simultaneous events A2 and A3 (inter-frequency measurements) |  | X |  | X |  | - |  |
| 8.3.1.7 |  | Measurement configuration control and reporting / Intra E-UTRAN measurements / Exclude-listed cells |  | X |  | X |  | - |  |
| 8.3.1.8 |  | Measurement configuration control and reporting/Intra E-UTRAN measurements/Handover/IE measurement configuration present |  | X |  | X |  | - |  |
| 8.3.1.9 |  | Measurement configuration control and reporting/Intra E-UTRAN measurements/Intra-frequency handover/IE measurement configuration not present |  | X |  | X |  | - |  |
| 8.3.1.9a |  | Measurement configuration control and reporting / Intra E-UTRAN measurements / Intra-frequency handover / IE measurement configuration not present / Single Frequency operation |  | X |  | X |  | - |  |
| 8.3.1.10 |  | Measurement configuration control and reporting/Intra E-UTRAN measurements/Inter-frequency handover/IE measurement configuration not present |  | X |  | X |  | - |  |
| 8.3.1.11 |  | Measurement configuration control and reporting/Intra E-UTRAN measurements/Continuation of the measurements after RRC connection re-establishment |  | X |  | X |  | - |  |
| 8.3.1.11a |  | Measurement configuration control and reporting / Intra Frequency measurements / Continuation of the measurements after RRC connection re-establishment / Single Frequency operation |  | X |  | X |  | - |  |
| 8.3.1.12 |  | Measurement configuration control and reporting / Intra E-UTRAN measurements / Two simultaneous events A3 (Inter-band measurements) |  | X |  | X |  | - |  |
| 8.3.1.12a |  | Measurement configuration control and reporting / Intra E-UTRAN measurements / Two simultaneous events A3 (inter-band measurements) / Between FDD and TDD |  | X |  | X |  | - |  |
| 8.3.1.13 |  | Measurement configuration control and reporting / Intra E-UTRAN measurements / Periodic reporting (intra-frequency and inter-band measurements) |  | X |  | X |  | - |  |
| 8.3.1.13a |  | Measurement configuration control and reporting / Intra E-UTRAN measurements / Periodic reporting (intra-frequency and inter-band measurements) / Between FDD and TDD |  | X |  | X |  | - |  |
| 8.3.1.14 |  | Measurement configuration control and reporting / Intra E-UTRAN measurements / Two simultaneous events A2 and A3 (Inter-band measurements) |  | X |  | X |  | - |  |
| 8.3.1.14a |  | Measurement configuration control and reporting / Intra E-UTRAN measurements / Two simultaneous events A2 and A3 (inter-band measurements) / Between FDD and TDD |  | X |  | X |  | - |  |
| 8.3.1.15 |  | Measurement configuration control and reporting / Intra E-UTRAN measurements / Inter-band handover / IE measurement configuration not present |  | X |  | X |  | - |  |
| 8.3.1.15a |  | Measurement configuration control and reporting / Intra E-UTRAN measurements / Inter-band handover / IE measurement configuration not present / Between FDD and TDD |  | X |  | X |  | - |  |
| 8.3.1.16 |  | Measurement configuration control and reporting / Intra E-UTRAN measurements / Continuation of the measurements after RRC connection re-establishment / Inter-band |  | X |  | X |  | - |  |
| 8.3.1.16a |  | Measurement configuration control and reporting / Intra E-UTRAN measurements / Continuation of the measurements after RRC connection re-establishment / Inter-band / Between FDD and TDD |  | X |  | X |  | - |  |
| 8.3.1.17.1 |  | CA / Measurement configuration control and reporting / Intra E-UTRAN measurements / Event A6 / Intra-band contiguous CA |  | X |  | X |  | - |  |
| 8.3.1.17.2 |  | CA / Measurement configuration control and reporting / Intra E-UTRAN measurements / Event A6 / Inter-band CA |  | X |  | X |  | - |  |
| 8.3.1.17.3 |  | CA / Measurement configuration control and reporting / Intra E-UTRAN measurements / Event A6 / Intra-band non-contiguous CA |  | X |  | X |  | - |  |
| 8.3.1.18.1 |  | CA / Measurement configuration control and reporting / Intra E-UTRAN measurements / Additional measurement reporting / Intra-band contiguous CA |  | X |  | X |  | - |  |
| 8.3.1.18.2 |  | CA / Measurement configuration control and reporting / Intra E-UTRAN measurements / Additional measurement reporting / Inter-band CA |  | X |  | X |  | - |  |
| 8.3.1.18.3 |  | CA / Measurement configuration control and reporting / Intra E-UTRAN measurements / Additional measurement reporting / Intra-band non-contiguous CA |  | X |  | X |  | - |  |
| 8.3.1.19 |  | eICIC / Measurement configuration control and reporting / CSI change |  | X |  | X |  | - |  |
| 8.3.1.21 |  | eICIC / Measurement configuration control and reporting/Event A3 Handover / Neighbour RSRP measurement configuration change |  | X |  | X |  | - |  |
| 8.3.1.22.1 |  | CA / Measurement configuration control and reporting / Intra E-UTRAN measurements / Event A1 / Event A2 / Intra-band Contiguous CA |  | X |  | X |  | - |  |
| 8.3.1.22.2 |  | CA / Measurement configuration control and reporting / Intra E-UTRAN measurements / Event A1 / Event A2 / Inter-band CA |  | X |  | X |  | - |  |
| 8.3.1.22.3 |  | CA / Measurement configuration control and reporting / Intra E-UTRAN measurements / Event A1/Event A2 / Intra-band non-contiguous CA |  | X |  | X |  | - |  |
| 8.3.1.23 |  | Measurement configuration control and reporting / Intra E-UTRAN measurements / Event A4 |  | X |  | X |  | - |  |
| 8.3.1.24 |  | Measurement configuration control and reporting / Intra E-UTRAN measurements / Event A5 |  | X |  | X |  | - |  |
| 8.3.1.25 |  | Measurement configuration control and reporting / Intra E-UTRAN measurements / Event A5 / RSRQ based measurements |  | X |  | X |  | - |  |
| 8.3.1.26 |  | Measurement configuration control and reporting / Intra E-UTRAN measurements / Event A5 (Inter-frequency measurements) |  | X |  | X |  | - |  |
| 8.3.1.27 |  | Measurement configuration control and reporting / Intra E-UTRAN measurements / Event A5 (Inter-frequency measurements) / RSRQ based measurements |  | X |  | X |  | - |  |
| 8.3.1.28 |  | eICIC / Measurement configuration control and reporting/Event A1 / RSRP and RSRQ measurement / Serving ABS |  | X |  | X |  | - |  |
| 8.3.2.1 |  | Measurement configuration control and reporting / Inter-RAT measurements / Event B2 / Measurement of GERAN cells |  | X |  | X |  | - |  |
| 8.3.2.2 |  | Measurement configuration control and reporting / Inter-RAT measurements / Periodic reporting / Measurement of GERAN cells |  | X |  | X |  | - |  |
| 8.3.2.3 |  | Measurement configuration control and reporting/Inter-RAT measurements/Event B2/Measurement of UTRAN cells |  | X |  | X |  | X |  |
| 8.3.2.3a |  | Measurement configuration control and reporting / Inter-RAT measurements / Event B2 / Measurement of UTRAN cells / RSRQ based measurements |  | X |  | X |  | X |  |
| 8.3.2.4 |  | Measurement configuration control and reporting / Inter-RAT measurements / Periodic reporting / Measurement of UTRAN cells |  | X |  | X |  | X |  |
| 8.3.2.5 |  | Measurement configuration control and reporting / Inter-RAT measurements / Periodic reporting / Measurements of E-UTRAN, UTRAN and GERAN cells |  | X |  | X |  | X |  |
| 8.3.2.6 |  | Measurement configuration control and reporting / Inter-RAT measurements / Simultaneous A2 and two B2 / Measurements of E-UTRAN, UTRAN and GERAN cells |  | X |  | X |  | X |  |
| 8.3.2.7 |  | Measurement configuration control and reporting/Inter-RAT measurements/Event B2/Measurement of HRPD cells |  | X |  | X |  | - |  |
| 8.3.2.8 |  | Measurement configuration control and reporting / Inter-RAT measurements / Periodic reporting / Measurement of HRPD cells |  | X |  | X |  | - |  |
| 8.3.2.9 |  | Measurement configuration control and reporting / Inter-RAT measurements / Event B2 / Measurement of 1xRTT cells |  | X |  | X |  | - |  |
| 8.3.2.10 |  | Measurement configuration control and reporting / Inter-RAT measurements / Periodic reporting / Measurement of 1xRTT cells |  | X |  | X |  | - |  |
| 8.3.2.11 |  | Measurement configuration control and reporting / Inter-RAT Measurements / Event B1 / Measurement of UTRAN cells |  | X |  | X |  | X |  |
| 8.3.3.1 |  | Measurement configuration control and reporting/SON/ANR/CGI reporting of E-UTRAN cell |  | X |  | X |  | - |  |
| 8.3.3.2 |  | Measurement configuration control and reporting / SON / ANR / CGI reporting of UTRAN cell |  | X |  |  |  | X |  |
| 8.3.3.3 |  | Measurement configuration control and reporting / SON / ANR / CGI reporting of GERAN cell |  | X |  | X |  | - |  |
| 8.3.3.4 |  | Measurement configuration control and reporting / SON / ANR / CGI reporting of HRPD cell |  | X |  | X |  | - |  |
| 8.3.4.1 |  | Intra-frequencySI acquisition / CSG cell and non-CSG cell |  | X |  | X |  | - |  |
| 8.3.4.2 |  | Inter-freq SI acquisition / Non-member hybrid cell |  | X |  | X |  | - |  |
| 8.3.4.3 |  | Inter-freq SI acquisition / Member hybrid cell |  | X |  | X |  | - |  |
| 8.3.4.4 |  | Inter-RAT SI acquisition / RRC_CONNECTED / UMTS member CSG cell |  | X |  | X |  | X |  |
| 8.3.4.5 |  | Inter-frequency E-UTRAN FDD – FDD / CSG Proximity Indication |  | X |  | - |  | - |  |
| 8.4.1.2 |  | Inter-RAT handover / From E-UTRA to UTRA PS / Data |  | X |  | X |  | X |  |
| 8.4.1.4 |  | Inter-RAT handover / From E-UTRA to UTRA HSPA / Data |  | X |  | X |  | X |  |
| 8.4.1.5 |  | Inter-RAT handover / from E-UTRA to UTRA HSUPA/HSDPA / Data |  | X |  |  |  | X |  |
| 8.4.2.2 |  | Inter-RAT handover / From UTRA PS to E-UTRA / Data |  | X |  | X |  | X |  |
| 8.4.2.4 |  | Inter-RAT handover / From UTRA HSPA to E-UTRA / Data |  | X |  | X |  | X |  |
| 8.4.2.7.1 |  | CA / RRC connection reconfiguration / Handover UTRAN to E-UTRAN/ Success / SCell addition / Intra-band Contiguous CA |  | X |  | X |  | X |  |
| 8.4.2.7.2 |  | CA / RRC connection reconfiguration / Handover UTRAN to E-UTRAN/ Success / SCell addition / Inter-band CA |  | X |  | X |  | X |  |
| 8.4.2.7.3 |  | CA / RRC connection reconfiguration / Handover UTRAN to E-UTRAN/ Success / SCell addition / Intra-band non-contiguous CA |  | X |  | X |  | X |  |
| 8.4.3.2 |  | Inter-RAT cell change order / From E-UTRA data RRC_CONNECTED to GPRS / Without NACC |  | X |  | X |  | - |  |
| 8.4.3.3 |  | Inter-RAT cell change order / From E-UTRA data to GPRS / With NACC |  | X |  | X |  | - |  |
| 8.4.7.3 |  | Pre-registration at 1xRTT and inter-RAT handover / CS fallback from E-UTRA RRC_IDLE to 1xRTT |  | X |  | X |  | - |  |
| 8.4.7.4 |  | Pre-Registration at 1xRTT and inter-RAT handover / CS fallback caused by addition of CS service / From E-UTRA Data to 1xRTT |  | X |  | X |  | - |  |
| 8.4.7.5 |  | Pre-registration at 1xRTT and inter-RAT handover / Enhanced CS fallback from E-UTRA RRC_IDLE to 1xRTT / MT call |  | X |  |  |  |  |  |
| 8.4.7.6 |  | Pre-registration at 1xRTT and inter-RAT handover / Enhanced CS fallback from E-UTRA RRC_CONNECTED to 1xRTT / MO call |  | X |  | X |  | - |  |
| 8.4.7.7 |  | Pre-registration at 1xRTT and inter-RAT handover / Enhanced CS fallback from E-UTRA RRC_CONNECTED to e1XCSFB ECAM-based 1xRTT / MO call |  | X |  | X |  | - |  |
| 8.4.7.8 |  | Pre-registration at 1xRTT and inter-RAT Handover / Enhanced CS fallback from E-UTRA RRC_CONNECTED to 1xRTT / ECAM-based MT call |  | X |  | X |  | - |  |
| 8.4.7.9 |  | Pre-registration at 1xRTT and inter-RAT Handover / Enhanced CS fallback from E-UTRA RRC_CONNECTED to 1xRTT / Extended Service Reject / MO call |  | X |  | X |  | - |  |
| 8.4.7.10 |  | Pre-registration at 1xRTT and inter-RAT Handover / Enhanced CS fallback from E-UTRA call failure – GCSNA with Reorder Order |  | X |  | X |  | - |  |
| 8.5.1.1 |  | Radio link failure/RRC connection re-establishment Success |  | X |  | X |  | - |  |
| 8.5.1.2 |  | Radio link failure/T301 expiry |  | X |  | X |  | - |  |
| 8.5.1.3 |  | Radio link failure/T311 expiry |  | X |  | X |  | - |  |
| 8.5.1.4 |  | Radio link failure / RRC connection re-establishment reject |  | X |  | X |  | - |  |
| 8.5.1.5 |  | Radio link failure/Radio link recovery while T310 is running |  | X |  | X |  | - |  |
| 8.5.1.6 |  | Radio link failure / T311 expiry / Dedicated RLF timer |  | X |  | X |  | - |  |
| 8.5.1.7.1 |  | CA / No Radio link failure on Scell/ RRC Connection Continues on PCell / Intra-band Contiguous CA |  | X |  | X |  | - |  |
| 8.5.1.7.2 |  | CA / No Radio link failure on Scell/ RRC Connection Continues on PCell / Inter-band CA |  | X |  | X |  | - |  |
| 8.5.1.7.3 |  | CA / No Radio link failure on Scell/ RRC Connection Continues on PCell / Intra-band non-contiguous CA |  | X |  | X |  | - |  |
| 8.5.2.1 |  | Redirection to E-UTRAN / From UTRAN upon reception of RRC CONNECTION REJECT |  | X |  | X |  | X |  |
| 8.5.4.1 |  | UE capability transfer/Success |  | X |  | X |  | - |  |
| 8.5.4.2 |  | Network-requested CA Band Combination Capability Signalling / Number of UE supported CA band combinations less than or equal to 128 |  | X |  | X |  | - |  |
| 8.5.4.3 |  | Network-requested CA Band Combination Capability Signalling / Number of UE supported CA band combinations exceeds 128 |  | X |  | X |  | - |  |
| 8.5.4.4 |  | UE Capability Transfer/ Success/ UE Cat 0/ UE Paging Info |  | X |  |  |  | - |  |
| 8.5.5.1 |  | RACS / UL Message Segment transfer / UECapabilityInformation / Success |  | X |  | X |  | - |  |
| 8.6.1.1 |  | Immediate MDT / Reporting / Location information |  | X |  | X |  | - |  |
| 8.6.1.2 |  | Immediate MDT / Reporting / Location information / Request from eNB / Event A2 |  | X |  | X |  | - |  |
| 8.6.1.3 |  | Immediate MDT / Measurement / Latency metrics for UL PDCP Packet Delay per QCI |  | X |  | X |  | - |  |
| 8.6.2.1 |  | Logged MDT / Intra-frequency measurement, logging and reporting |  | X |  | X |  | - |  |
| 8.6.2.2 |  | Logged MDT / Inter-frequency measurement, logging and reporting |  | X |  | X |  | - |  |
| 8.6.2.3 |  | Logged MDT / Logging and reporting / Limiting area scope |  | X |  | X |  | - |  |
| 8.6.2.3a |  | Logged MDT / Logging and reporting / Limiting area scope / TAC list with PLMN identity |  | X |  | X |  |  |  |
| 8.6.2.4 |  | Logged MDT / Logging and reporting / Indication of logged measurements at E-UTRA handover |  | X |  | X |  | - |  |
| 8.6.2.5 |  | Logged MDT / Logging and reporting / Indication of logged measurements at E-UTRA re-establishment |  | X |  | X |  | - |  |
| 8.6.2.6 |  | Logged MDT / Release of logged MDT measurement configuration / Expire of duration timer |  | X |  | X |  | - |  |
| 8.6.2.7 |  | Logged MDT / Release of logged MDT measurement configuration / Reception of new logged measurement configuration, Detach or UE power off |  | X |  | X |  | - |  |
| 8.6.2.8 |  | Logged MDT / Maintaining logged measurement configuration / UE state transitions and mobility |  | X |  | X |  | - |  |
| 8.6.2.9 |  | Logged MDT / Location information |  | X |  | X |  | - |  |
| 8.6.2.10 |  | Logged MDT / Logging and reporting / Reporting at RRC connection establishment / PLMN list |  | X |  | X |  | - |  |
| 8.6.2.11 |  | Logged MDT / Logging and reporting / Reporting at intra LTE handover / PLMN list |  | X |  | X |  | - |  |
| 8.6.2.12 |  | Logged MDT / Logging and reporting / Reporting at RRC connection re-establishment / PLMN list |  | X |  | X |  | - |  |
| 8.6.3.1 |  | Logged MDT / UTRAN Inter-RAT measurement, logging and reporting |  | X |  |  |  | X |  |
| 8.6.3.4 |  | Logged MDT / Logging and reporting / Reporting at UTRAN Inter-RAT handover / PLMN list |  | X |  |  |  | X |  |
| 8.6.4.1 |  | Radio Link Failure logging / Reporting of Intra-frequency measurements |  | X |  | X |  | - |  |
| 8.6.4.2 |  | Radio Link Failure logging / Reporting of Inter-frequency measurements |  | X |  | X |  | - |  |
| 8.6.4.3 |  | Radio Link Failure logging / Reporting at RRC connection establishment and reestablishment |  | X |  | X |  | - |  |
| 8.6.4.4 |  | Radio Link Failure logging / Reporting at E-UTRA handover |  | X |  | X |  | - |  |
| 8.6.4.5 |  | Radio Link Failure logging / Reporting of ECGI of the PCell |  | X |  | X |  | - |  |
| 8.6.4.7 |  | Radio Link Failure logging / Location information |  | X |  | X |  | - |  |
| 8.6.4.8 |  | Radio Link Failure logging / Logging and reporting / Reporting at RRC connection establishment / PLMN list |  | X |  | X |  | - |  |
| 8.6.4.9 |  | Radio Link Failure logging / Logging and reporting / Reporting at intra LTE handover / PLMN list |  | X |  | X |  | - |  |
| 8.6.4.10 |  | Radio Link Failure logging / Logging and reporting / Reporting at RRC connection re-establishment / PLMN list |  | X |  | X |  | - |  |
| 8.6.4.11 |  | Radio Link Failure logging / Logging and reporting / Dropped QCI |  | X |  | X |  | - |  |
| 8.6.5.1 |  | Radio Link Failure logging / Reporting at UTRAN Inter-RAT handover |  | X |  | X |  | X |  |
| 8.6.5.1a |  | Radio Link Failure logging / Reporting at UTRAN Inter-RAT handover / PLMN list |  | X |  |  |  | X |  |
| 8.6.5.4 |  | Radio Link Failure logging / Reporting of selected UTRA cell |  | X |  | X |  | X |  |
| 8.6.6.1 |  | Handover Failure logging / Reporting of Intra-frequency measurements |  | X |  | X |  | - |  |
| 8.6.6.2 |  | Handover Failure logging / Reporting of Inter-frequency measurements |  | X |  | X |  | - |  |
| 8.6.6.4 |  | Handover Failure logging / Location information |  | X |  | X |  | - |  |
| 8.6.6.5 |  | Handover Failure logging / Logging and reporting / Reporting at RRC connection establishment / PLMN list |  | X |  | X |  | - |  |
| 8.6.6.6 |  | Handover Failure logging / Logging and reporting / Reporting at intra LTE handover / PLMN list |  | X |  | X |  | - |  |
| 8.6.6.7 |  | Handover Failure logging / Logging and reporting / Reporting at RRC connection re-establishment / PLMN list |  | X |  | X |  | - |  |
| 8.6.7.1 |  | Handover Failure logging / Reporting of UTRAN Inter-RAT measurements |  | X |  | X |  | X |  |
| 8.6.7.2 |  | Handover Failure logging / Reporting of GERAN Inter-RAT measurements |  | X |  | X |  | - |  |
| 8.6.7.3 |  | Handover Failure logging / Reporting of CDMA2000 Inter-RAT measurements |  | X |  |  |  | - |  |
| 8.6.7.4 |  | Handover Failure logging / Reporting at UTRAN Inter-RAT handover / PLMN list |  | X |  | X |  |  |  |
| 8.6.8.1 |  | Connection Establishment Failure logging / Logging and reporting / T300 expiry |  | X |  | X |  | - |  |
| 8.6.8.2 |  | Connection Establishment Failure logging / Logging and reporting / Reporting at intra-LTE handover |  | X |  | X |  | - |  |
| 8.6.8.3 |  | Connection Establishment Failure logging / Logging and reporting / Reporting at RRC connection re-establishment |  | X |  | X |  | - |  |
| 8.6.8.4 |  | Connection Establishment Failure logging / Logging and reporting / Location Information |  | X |  | X |  | - |  |
| 8.6.8.5 |  | Connection Establishment Failure logging / Logging and reporting / Reporting of Intra-frequency measurements |  | X |  | X |  | - |  |
| 8.6.8.6 |  | Connection Establishment Failure logging / Logging and reporting / Reporting of Inter-frequency measurements |  | X |  | X |  | - |  |
| 8.6.9.1 |  | Connection Establishment Failure logging / Logging and reporting / Reporting at UTRAN Inter-RAT handover |  | X |  | X |  | X |  |
| 8.6.9.2 |  | Connection Establishment Failure logging / Logging and reporting / Reporting of UTRAN Inter-RAT measurements |  | X |  | X |  | X |  |
| 8.6.9.3 |  | Connection Establishment Failure logging / Logging and reporting / Reporting of GERAN Inter-RAT measurements |  | X |  | X |  | - |  |
| 8.6.10.1 |  | Inter-RAT Immediate MDT / Reporting / Location information / Event B2 |  | X |  | X |  | X |  |
| 8.6.11.1 |  | RACH Optimisation |  | X |  | X |  | - |  |
| 8.7.1 |  | Inter-RAT / ANR measurement, logging and reporting / E-UTRAN cell |  | X |  |  |  |  |  |
| 9.1.2.1 |  | Authentication accepted |  | X |  | X |  | - |  |
| 9.1.2.3 |  | Authentication not accepted by the network, GUTI used, authentication reject and re-authentication |  | X |  | X |  | - |  |
| 9.1.2.4 |  | Authentication not accepted by the UE/MAC code failure |  | X |  | X |  | - |  |
| 9.1.2.5 |  | Authentication not accepted by the UE/SQN failure |  | X |  | X |  | - |  |
| 9.1.2.6 |  | Abnormal cases/Network failing the authentication check |  | X |  | X |  | - |  |
| 9.1.2.7 |  | Authentication not accepted by the UE/ non-EPS authentication unacceptable |  | X |  | X |  | - |  |
| 9.1.3.1 |  | NAS security mode command accepted by the UE |  | X |  | X |  | - |  |
| 9.1.3.2 |  | NAS security mode command not accepted by the UE |  | X |  | X |  | - |  |
| 9.1.3.3 |  | No emergency bearer service / NAS security mode command with EIA0 not accepted by the UE |  | X |  | X |  | - |  |
| 9.1.4.2 |  | Identification procedure/IMEI requested |  | X |  | X |  | - |  |
| 9.1.5.1 |  | EMM information procedure |  | X |  | X |  | - |  |
| 9.2.1.1.1 |  | Attach Procedure/Success/Valid GUTI |  | X |  | X |  | - |  |
| 9.2.1.1.1a |  | Attach/Success/Last visited TAI, TAI list and equivalent PLMN list handling |  | X |  | X |  | - |  |
| 9.2.1.1.1b |  | Attach Procedure / Success / Last visited TAI, TAI list and equivalent PLMN list handling / Single Frequency operation |  | X |  | X |  | - |  |
| 9.2.1.1.2 |  | Attach Procedure/Success/With IMSI/GUTI reallocation |  | X |  | X |  | - |  |
| 9.2.1.1.2a |  | AttachWithIMSI configured / Selected PLMN is neither the registered PLMN nor in the list of equivalent PLMNs / Success |  | X |  | X |  | - |  |
| 9.2.1.1.7 |  | Attach/Success/List of equivalent PLMNs in the ATTACH ACCEPT message |  | X |  | X |  | - |  |
| 9.2.1.1.7a |  | Attach Procedure / Success / List of equivalent PLMNs in the ATTACH ACCEPT message / Single Frequency operation |  | X |  | X |  | - |  |
| 9.2.1.1.7b |  | Attach / Success / native GUMMEI |  | X |  | X |  | - |  |
| 9.2.1.1.7c |  | Attach / Success / PSM |  | X |  | X |  | - |  |
| 9.2.1.1.9 |  | Attach/Rejected/IMSI invalid |  | X |  | X |  | - |  |
| 9.2.1.1.10 |  | Attach/Rejected/Illegal ME |  | X |  | X |  | - |  |
| 9.2.1.1.11 |  | Attach / Rejected / EPS services and non-EPS services not allowed |  | X |  | X |  | X |  |
| 9.2.1.1.12 |  | Attach / Rejected / EPS services not allowed |  | X |  | X |  | X |  |
| 9.2.1.1.13 |  | Attach/Rejected/PLMN not allowed |  | X |  | X |  | - |  |
| 9.2.1.1.13a |  | Attach / Rejected / PLMN not allowed / Single Frequency operation |  | X |  | X |  | - |  |
| 9.2.1.1.14 |  | Attach/Rejected/Tracking area not allowed |  | X |  | X |  | - |  |
| 9.2.1.1.15 |  | Attach/Rejected/Roaming not allowed in this tracking area |  | X |  | X |  | - |  |
| 9.2.1.1.15a |  | Attach / Rejected / Roaming not allowed in this tracking area / Single Frequency operation |  | X |  | X |  | - |  |
| 9.2.1.1.16 |  | Attach/Rejected/EPS services not allowed in this PLMN |  | X |  | X |  | - |  |
| 9.2.1.1.16a |  | Attach / Rejected / EPS services not allowed in this PLMN / Single Frequency operation |  | X |  | X |  | -- |  |
| 9.2.1.1.17 |  | Attach/Rejected/No suitable cells in tracking area |  | X |  | X |  | - |  |
| 9.2.1.1.18 |  | Attach / Rejected / Not authorized for this CSG |  | X |  | X |  | - |  |
| 9.2.1.1.19 |  | Attach/Abnormal case/Failure due to non integrity protection |  | X |  | X |  | - |  |
| 9.2.1.1.20 |  | Attach/Abnormal case/Access barred because of access class barring or NAS signalling connection establishment rejected by the network |  | X |  | X |  | - |  |
| 9.2.1.1.21 |  | Attach/Abnormal case/Success after several attempts due to no network response |  | X |  | X |  | - |  |
| 9.2.1.1.22 |  | Attach/Abnormal case/Unsuccessful attach after 5 attempts |  | X |  | X |  | - |  |
| 9.2.1.1.23 |  | Attach/Abnormal case/Repeated rejects for network failures |  | X |  | X |  | - |  |
| 9.2.1.1.24 |  | Attach/Abnormal case/Change of cell into a new tracking area |  | X |  | X |  | - |  |
| 9.2.1.1.25 |  | Attach/Abnormal case/Mobile originated detach required |  | X |  | X |  | - |  |
| 9.2.1.1.26 |  | Attach/Abnormal case/Detach procedure collision |  | X |  | X |  | - |  |
| 9.2.1.1.27 |  | Attach / Abnormal case / Network reject with Extended Wait Timer |  | X |  | X |  | - |  |
| 9.2.1.1.27a |  | Attach Procedure / EAB broadcast handling / ExtendedAccessBarring configured in the UE |  | X |  | X |  | - |  |
| 9.2.1.1.28 |  | Attach / Success / IMS |  | X |  | X |  | - |  |
| 9.2.1.1.28a |  | Attach / Success / IMS / Second PDN |  | X |  | X |  | - |  |
| 9.2.1.1.29 |  | Attach / Rejected / IMEI not accepted |  | X |  | X |  | - |  |
| 9.2.1.1.30 |  | Attach / Abnormal case / ESM failure |  | X |  | X |  | - |  |
| 9.2.1.2.1 |  | Combined attach/Success/EPS and non-EPS services |  | X |  | X |  | - |  |
| 9.2.1.2.1b |  | Combined attach / Success / SMS only |  | X |  | X |  | X |  |
| 9.2.1.2.1c |  | Combined attach procedure / Success / CS Fallback not preferred |  | X |  | X |  | X |  |
| 9.2.1.2.1d |  | Combined attach procedure / Success / EPS and CS Fallback not preferred / data centric UE |  | X |  | X |  | X |  |
| 9.2.1.2.2 |  | Combined attach / Success / EPS services only / IMSI unknown in HSS |  | X |  | X |  | - |  |
| 9.2.1.2.3 |  | Combined attach / Success / EPS services only / MSC temporarily not reachable |  | X |  | X |  | - |  |
| 9.2.1.2.4 |  | Combined attach/Success/EPS services only/CS domain not available |  | X |  | X |  | - |  |
| 9.2.1.2.4a |  | Successful combined attach procedure / EPS service only / congestion |  | X |  | X |  | - |  |
| 9.2.1.2.5 |  | Combined attach / Rejected / IMSI invalid |  | X |  | X |  | X |  |
| 9.2.1.2.6 |  | Combined attach / Rejected / Illegal ME |  | X |  | X |  | X |  |
| 9.2.1.2.7 |  | Combined attach / Rejected / EPS services and non-EPS services not allowed |  | X |  | X |  | X |  |
| 9.2.1.2.8 |  | Combined attach / Rejected / EPS services not allowed |  | X |  | X |  | X |  |
| 9.2.1.2.9 |  | Combined attach / Rejected / PLMN not allowed |  | X |  | X |  | X |  |
| 9.2.1.2.10 |  | Combined attach / Rejected / Tracking area not allowed |  | X |  | X |  | - |  |
| 9.2.1.2.11 |  | Combined attach / Rejected / Roaming not allowed in this tracking area |  | X |  | X |  | X |  |
| 9.2.1.2.12 |  | Combined attach / Rejected / EPS services not allowed in this PLMN |  | X |  | X |  | - |  |
| 9.2.1.2.13 |  | Combined attach / Rejected / No suitable cells in tracking area |  | X |  | X |  | X |  |
| 9.2.1.2.14 |  | Combined attach / Rejected / Not authorized for this CSG |  | X |  | X |  | - |  |
| 9.2.1.2.15 |  | Combined attach / Abnormal case / Handling of the EPS attach attempt counter |  | X |  | X |  | X |  |
| 9.2.2.1.1 |  | UE initiated detach/UE switched off |  | X |  | X |  | - |  |
| 9.2.2.1.2 |  | UE initiated detach/USIM removed from the UE |  | X |  | X |  | - |  |
| 9.2.2.1.3 |  | UE initiated detach/EPS capability of the UE is disabled |  | X |  | X |  | X |  |
| 9.2.2.1.4 |  | UE initiated detach / detach for non-EPS services |  | X |  | X |  | - |  |
| 9.2.2.1.6 |  | UE initiated detach/Abnormal case/Local detach after 5 attempts due to no network response |  | X |  | X |  | - |  |
| 9.2.2.1.7 |  | UE initiated detach/Abnormal case/Detach procedure collision |  | X |  | X |  | - |  |
| 9.2.2.1.8 |  | UE initiated detach/Abnormal case/Detach and EMM common procedure collision |  | X |  | X |  | - |  |
| 9.2.2.1.9 |  | UE initiated detach/Abnormal case/Change of cell into a new tracking area |  | X |  | X |  | - |  |
| 9.2.2.1.10 |  | UE initiated detach / Mapped security context |  | X |  | X |  | X |  |
| 9.2.2.2.1 |  | NW initiated detach/Re-attach required |  | X |  | X |  | - |  |
| 9.2.2.2.2 |  | NW initiated detach/IMSI detach |  | X |  | X |  | - |  |
| 9.2.2.2.14 |  | NW initiated detach/Abnormal case/EMM cause not included |  | X |  | X |  | - |  |
| 9.2.3.1.1 |  | Normal tracking area update/Accepted |  | X |  | X |  | - |  |
| 9.2.3.1.1a |  | Normal tracking area update / Accepted / PSM |  | X |  | X |  | - |  |
| 9.2.3.1.4 |  | Normal tracking area update/List of equivalent PLMNs in the TRACKING AREA UPDATE ACCEPT message |  | X |  | X |  | - |  |
| 9.2.3.1.5 |  | Periodic tracking area update/Accepted |  | X |  | X |  | - |  |
| 9.2.3.1.5a |  | Periodic tracking area update / Accepted / Per-device timer |  | X |  | X |  | - |  |
| 9.2.3.1.5b |  | Periodic tracking area update / Accepted / PSM / T3312 Extended Value |  | X |  | X |  | - |  |
| 9.2.3.1.6 |  | Normal tracking area update / UE with ISR active moves to E-UTRAN |  | X |  | X |  | X |  |
| 9.2.3.1.8 |  | UE receives an indication that the RRC connection was released with cause “load balancing TAU required” |  | X |  | X |  | - |  |
| 9.2.3.1.8a |  | Normal tracking area update / low priority override |  | X |  | X |  | - |  |
| 9.2.3.1.8b |  | Normal tracking area update / EAB active |  | X |  | X |  | - |  |
| 9.2.3.1.9 |  | Normal tracking area update / Correct handling of CSG list |  | X |  | X |  | - |  |
| 9.2.3.1.9a |  | Normal tracking area update/NAS signalling connection recovery |  | X |  | X |  | - |  |
| 9.2.3.1.10 |  | Normal tracking area update / Rejected / IMSI invalid |  | X |  | X |  | X |  |
| 9.2.3.1.11 |  | Normal tracking area update / Rejected / Illegal ME |  | X |  | X |  | X |  |
| 9.2.3.1.12 |  | Normal tracking area update / Rejected / EPS service not allowed |  | X |  | X |  | X |  |
| 9.2.3.1.13 |  | Normal tracking area update/Rejected/UE identity cannot be derived by the network |  | X |  | X |  | - |  |
| 9.2.3.1.14 |  | Normal tracking area update/Rejected/UE implicitly detached |  | X |  | X |  | - |  |
| 9.2.3.1.15 |  | Normal tracking area update / Rejected / PLMN not allowed |  | X |  | X |  | X |  |
| 9.2.3.1.15a |  | Normal tracking area update / Rejected / PLMN not allowed / Single Frequency operation |  | X |  | X |  | X |  |
| 9.2.3.1.16 |  | Normal tracking area update/Rejected/Tracking area not allowed |  | X |  | X |  | - |  |
| 9.2.3.1.17 |  | Normal tracking area update / Rejected / Roaming not allowed in this tracking area |  | X |  | X |  | X |  |
| 9.2.3.1.18 |  | Normal tracking area update / Rejected / EPS services not allowed in this PLMN |  | X |  | X |  | X |  |
| 9.2.3.1.18a |  | Normal tracking area update / Rejected / EPS services not allowed in this PLMN / Single Frequency operation |  | X |  | X |  | X |  |
| 9.2.3.1.19 |  | Normal tracking area update/Rejected/No suitable cells in tracking area |  | X |  | X |  | - |  |
| 9.2.3.1.20 |  | Normal tracking area update / Rejected / Not authorized for this CSG |  | X |  | X |  | - |  |
| 9.2.3.1.20a |  | Normal tracking area update / Rejected / Congestion |  | X |  | X |  | - |  |
| 9.2.3.1.22 |  | Normal tracking area update / Abnormal case / access barred due to access class control or NAS signalling connection establishment rejected by the network |  | X |  | X |  | - |  |
| 9.2.3.1.23 |  | Normal tracking area update/Abnormal case/Success after several attempts due to no network response/TA belongs to TAI list and status is UPDATED |  | X |  | X |  | - |  |
| 9.2.3.1.25 |  | Normal tracking area update/Abnormal case/Failure after 5 attempts due to no network response |  | X |  | X |  | - |  |
| 9.2.3.1.26 |  | Normal tracking area update/Abnormal case/TRACKING AREA UPDATE REJECT |  | X |  | X |  | - |  |
| 9.2.3.1.27 |  | Normal tracking area update/Abnormal case/Change of cell into a new tracking area |  | X |  | X |  | - |  |
| 9.2.3.1.28 |  | Normal tracking area update/Abnormal case/Tracking area updating and detach procedure collision |  | X |  | X |  | - |  |
| 9.2.3.2.1 |  | Combined tracking area update/Successful |  | X |  | X |  | - |  |
| 9.2.3.2.1a |  | Combined tracking area update / Successful / Check of last visited TAI and handling of TAI list, LAI and TMSI |  | X |  | X |  | X |  |
| 9.2.3.2.1b |  | Combined tracking area update / Success / SMS only |  | X |  | X |  | X |  |
| 9.2.3.2.1c |  | Combined tracking area update / Success / CS Fallback not preferred |  | X |  | X |  | X |  |
| 9.2.3.2.2 |  | Combined tracking area update / Successful for EPS services only / IMSI unknown in HSS |  | X |  | X |  | - |  |
| 9.2.3.2.3 |  | Combined tracking area update / Successful for EPS services only / MSC temporarily not reachable |  | X |  | X |  | X |  |
| 9.2.3.2.4 |  | Combined tracking area update / Successful for EPS services only / CS domain not available |  | X |  | X |  | - |  |
| 9.2.3.2.4a |  | Combined tracking area update / Successful for EPS services only / congestion |  | X |  | X |  | - |  |
| 9.2.3.2.5 |  | Combined tracking area update / Rejected / IMSI invalid |  | X |  | X |  | X |  |
| 9.2.3.2.6 |  | Combined tracking area update / Rejected / Illegal ME |  | X |  | X |  | X |  |
| 9.2.3.2.7 |  | Combined tracking area update / Rejected / EPS services and non-EPS services not allowed |  | X |  | X |  | X |  |
| 9.2.3.2.8 |  | Combined tracking area update / Rejected / EPS services not allowed |  | X |  | X |  | X |  |
| 9.2.3.2.9 |  | Combined tracking area update / Rejected / UE identity cannot be derived by the network |  | X |  | X |  | X |  |
| 9.2.3.2.10 |  | Combined tracking area update / Rejected / UE implicitly detached |  | X |  | X |  | - |  |
| 9.2.3.2.11 |  | Combined tracking area update / Rejected / PLMN not allowed |  | X |  | X |  | X |  |
| 9.2.3.2.12 |  | Combined tracking area update / Rejected / Tracking area not allowed |  | X |  | X |  | - |  |
| 9.2.3.2.13 |  | Combined tracking area update / Rejected / Roaming not allowed in this tracking area |  | X |  | X |  | X |  |
| 9.2.3.2.14 |  | Combined tracking area update / Rejected / EPS services not allowed in this PLMN |  | X |  | X |  | X |  |
| 9.2.3.2.15 |  | Combined tracking area update / Rejected / No suitable cells in tracking area |  | X |  | X |  | - |  |
| 9.2.3.2.16 |  | Combined tracking area update / Rejected / Not authorized for this CSG |  | X |  | X |  | - |  |
| 9.2.3.2.17 |  | Combined tracking area update / Abnormal case / handling of the EPS tracking area updating attempt counter |  | X |  | X |  | - |  |
| 9.2.3.3.1 |  | First Iu mode to S1 mode inter-system change after attach |  | X |  | X |  | X |  |
| 9.2.3.3.2 |  | Iu mode to S1 mode intersystem change / ISR is active / Expiry of T3312 in E-UTRAN or T3412 in UTRAN and further intersystem change |  | X |  | X |  | X |  |
| 9.2.3.3.3 |  | Iu mode to S1 mode intersystem change / Periodic TAU and RAU / ISR activated, T34xx expired |  | X |  | X |  | X |  |
| 9.2.3.3.4 |  | First S1 mode to Iu mode inter-system change after attach |  | X |  | X |  | X |  |
| 9.2.3.3.5 |  | Periodic routing area update |  | X |  | X |  | X |  |
| 9.2.3.3.5a |  | Periodic location update |  | X |  | X |  | X |  |
| 9.2.3.4.1 |  | TAU/RAU procedure for inter-system cell reselection between A/Gb and S1 modes |  | X |  | X |  | - |  |
| 9.2.4.1.1 |  | Attach and Normal tracking area update Procedure / Success / With and without Idle eDRX parameters |  | X |  | X |  | - |  |
| 9.2.4.1.2 |  | Attach & Normal tracking area update Procedure / Success / With and without Idle eDRX and PSM parameters |  | X |  | X |  | - |  |
| 9.2.5.1 |  | RACS / Network assigned UE radio capability ID |  | X |  | X |  | - |  |
| 9.2.5.2 |  | RACS / USIM change / Handling of URCID |  | X |  | X |  | - |  |
| 9.2.5.3 |  | RACS /Handling of delete indication for NW assigned UE radio capability ID |  | X |  | X |  | - |  |
| 9.3.1.1 |  | Service request initiated by UE for user data |  | X |  | X |  | - |  |
| 9.3.1.3 |  | Service request / Mobile originating CS fallback |  | X |  | X |  | - |  |
| 9.3.1.4 |  | Service request / Rejected / IMSI invalid |  | X |  | X |  | X |  |
| 9.3.1.5 |  | Service request / Rejected / Illegal ME |  | X |  | X |  | X |  |
| 9.3.1.6 |  | Service request / Rejected / EPS services not allowed |  | X |  | X |  | X |  |
| 9.3.1.7 |  | Service request/Rejected/UE identity cannot be derived by the network |  | X |  | X |  | - |  |
| 9.3.1.7a |  | Service request/Rejected/UE implicitly detached |  | X |  | X |  | - |  |
| 9.3.1.12a |  | Extended service request / Rejected / CS domain temporarily not available |  | X |  | X |  | - |  |
| 9.3.1.16 |  | Service request/Abnormal case/Switch off |  | X |  | X |  | - |  |
| 9.3.1.17 |  | Service request/Abnormal case/Procedure collision |  | X |  | X |  | - |  |
| 9.3.1.18 |  | Service request / Rejected / Not authorized for this CSG |  | X |  | X |  | - |  |
| 9.3.2.1 |  | Paging procedure |  | X |  | X |  | - |  |
| 9.3.2.2 |  | Paging for CS fallback/Idle mode |  | X |  | X |  | - |  |
| 9.3.2.2a |  | Paging for CS fallback/Connected mode |  | X |  | X |  | - |  |
| 9.4.1 |  | Integrity protection/Correct functionality of EPS NAS integrity algorithm/SNOW3G |  | X |  | X |  | - |  |
| 9.4.2 |  | Integrity protection/Correct functionality of EPS NAS integrity algorithm/AES |  | X |  | X |  | - |  |
| 9.4.3 |  | Ciphering and deciphering/Correct functionality of EPS NAS encryption algorithm/SNOW3G |  | X |  | X |  | - |  |
| 9.4.4 |  | Ciphering and deciphering/Correct functionality of EPS NAS encryption algorithm/AES |  | X |  | X |  | - |  |
| 9.4.5 |  | Integrity protection / Correct functionality of EPS NAS integrity algorithm / ZUC |  | X |  | X |  | - |  |
| 9.4.6 |  | Ciphering and deciphering / Correct functionality of EPS NAS encryption algorithm / ZUC |  | X |  | X |  | - |  |
| 10.2.1 |  | Dedicated EPS bearer context activation/Success |  | X |  | X |  | - |  |
| 10.3.1 |  | EPS bearer context modification/Success |  | X |  | X |  | - |  |
| 10.4.1 |  | EPS bearer context deactivation/Success |  | X |  | X |  | - |  |
| 10.4.2 |  | EPS bearer context deactivation / Re-establishment |  | X |  | X |  | - |  |
| 10.5.1 |  | UE requested PDN connectivity procedure accepted by the network |  | X |  | X |  | - |  |
| 10.5.1a |  | UE requested PDN connectivity accepted / Dual priority / T3396 override |  | X |  | X |  | - |  |
| 10.5.1b |  | UE requested PDN connectivity accepted / Dual priority / T3346 override |  | X |  | X |  | - |  |
| 10.5.3 |  | UE requested PDN connectivity procedure not accepted |  | X |  | X |  | - |  |
| 10.5.4 |  | UE requested PDN connectivity not accepted / Network reject with Extended Wait Timer |  | X |  | X |  | - |  |
| 10.6.1 |  | UE requested PDN disconnect procedure accepted by the network |  | X |  | X |  | - |  |
| 10.7.1 |  | UE requested bearer resource allocation, accepted by the network/New EPS bearer context |  | X |  | X |  | - |  |
| 10.7.2 |  | UE requested bearer resource allocation accepted by the network/Existing EPS bearer context |  | X |  | X |  | - |  |
| 10.7.3 |  | UE requested bearer resource allocation not accepted by the network |  | X |  | X |  | - |  |
| 10.7.4 |  | UE requested bearer resource allocation/Expiry of timer T3480 |  | X |  | X |  | - |  |
| 10.7.5 |  | UE requested bearer resource allocation / BEARER RESOURCE ALLOCATION REJECT message including cause #43 “unknown EPS bearer context” |  | X |  | X |  | - |  |
| 10.8.1 |  | UE requested bearer resource modification accepted by the network/New EPS bearer context |  | X |  | X |  | - |  |
| 10.8.2 |  | UE requested bearer resource modification accepted by the network/Existing EPS bearer context |  | X |  | X |  | - |  |
| 10.8.3 |  | UE requested bearer resource modification not accepted by the network |  | X |  | X |  | - |  |
| 10.8.4 |  | UE requested bearer resource modification / Cause #36 “regular deactivation” |  | X |  | X |  | - |  |
| 10.8.5 |  | UE requested bearer resource modification / BEARER RESOURCE MODIFICATION REJECT message including cause #43 “unknown EPS bearer context” |  | X |  | X |  | - |  |
| 10.8.6 |  | UE requested bearer resource modification / Collision of a UE requested bearer resource modification procedure and EPS bearer context deactivation procedure |  | X |  | X |  | - |  |
| 10.8.7 |  | UE requested bearer resource modification / Expiry of timer T3481 |  | X |  | X |  | - |  |
| 10.8.8 |  | UE requested bearer resource modification / Dual priority / low priority override |  | X |  | X |  | - |  |
| 10.9.1 |  | UE routing of uplinks packets |  | X |  | X |  | - |  |
| 11.1.1 |  | MT-SMS over SGs/Idle mode |  | X |  | X |  | - |  |
| 11.1.2 |  | MT-SMS over SGs/Active mode |  | X |  | X |  | - |  |
| 11.1.3 |  | MO-SMS over SGs/Idle mode |  | X |  | X |  | - |  |
| 11.1.4 |  | MO-SMS over SGs/Active mode |  | X |  | X |  | - |  |
| 11.1.5 |  | Multiple MO-SMS over SGs / Idle mode |  | X |  | X |  | - |  |
| 11.1.6 |  | Multiple MO-SMS over SGs / Active mode |  | X |  | X |  | - |  |
| 11.2.1 |  | Emergency bearer services / Normal cell / NORMAL-SERVICE / Local Emergency Numbers List sent in the Attach / PDN connect new emergency EPS bearer context / Service request / Emergency PDN disconnect |  | X |  | X |  | - |  |
| 11.2.2 |  | Emergency bearer services / Normal cell / LIMITED-SERVICE / Attach / PDN connect / Service request / PDN disconnect / Detach / Temporary storage of EMM information |  | X |  | X |  | - |  |
| 11.2.3 |  | Emergency bearer services / CSG cell / LIMITED-SERVICE / Attach / Security mode control procedure without prior authentication / PDN connect / Service request / PDN disconnect / Detach upon UE switched off / Temporary storage of EMM information |  | X |  | X |  | - |  |
| 11.2.4 |  | Emergency bearer services / Normal cell / NO-IMSI / Attach / No EPS security context / PDN connect / Service request / Timer T3412 expires |  | X |  | X |  | - |  |
| 11.2.5 |  | Emergency bearer services / Normal cell / NORMAL-SERVICE / Local Emergency Numbers List NOT sent in the Attach / PDN connect new emergency EPS bearer context / Authentication SQN code failure – MME aborts authentication continues using current security context / Service request / Emergency PDN disconnect |  | X |  | X |  | - |  |
| 11.2.6 |  | Handling of Local Emergency Numbers List provided during Attach and Normal tracking area update procedures |  | X |  | X |  | - |  |
| 11.2.7 |  | UE has PDN connection only for emergency bearer services / Normal tracking area update / Accepted / Local Emergency Numbers List is not sent by the network / Handling of the lists of forbidden tracking areas |  | X |  | X |  | - |  |
| 11.2.8 |  | Attach for emergency bearer services / Rejected / No suitable cells in tracking area / Emergency call using the CS domain / UTRA or GERAN |  | X |  | X |  | X |  |
| 11.2.10 |  | LIMITED-SERVICE / EPS does not support IMS Emergency / Emergency call using the CS domain |  | X |  | X |  | X |  |
| 11.2.11 |  | LIMITED-SERVICE / Inter-system mobility / E-UTRA to UTRA CS / SRVCC Emergency Call Handover to UTRAN |  | X |  | X |  | X |  |
| 11.2.12 |  | LIMITED-SERVICE / Inter-system mobility / E-UTRA to GSM CS / SRVCC Emergency Call Handover to GERAN |  | X |  | X |  | - |  |
| 11.3.1 |  | eCall Only mode/T3444/eCall inactivity procedure after an eCall over IMS |  | X |  |  |  | - |  |
| 11.3.2 |  | eCall Only mode / T3445 / eCall inactivity procedure after a call to URI for test service |  | X |  |  |  | - |  |
| 11.3.6 |  | eCall Only mode/Limited service state/call to URI for test service should not be attempted/eCall over IMS should be attempted |  | X |  |  |  | - |  |
| 12.2.1 |  | Data transfer of E-UTRA radio bearer combinations 1, 3, 6 and 9 |  | X |  | X |  | - |  |
| 12.2.2 |  | Data transfer of E-UTRA radio bearer combinations 2, 4, 7 and 10 |  | X |  | X |  | - |  |
| 12.2.3 |  | Data transfer of E-UTRA radio bearer combinations 5, 6, 8, 11 and 12 |  | X |  | X |  | - |  |
| 12.2.4 |  | Data transfer of E-UTRA radio bearer combination 13 |  | X |  | X |  | - |  |
| 12.3.1 |  | Data transfer of E-UTRA radio bearer combinations 1, 3, 6 and 9 / MIMO |  | X |  | X |  | - |  |
| 12.3.2 |  | Data transfer of E-UTRA radio bearer combinations 2, 4, 7 and 10 / MIMO |  | X |  | X |  | - |  |
| 12.3.3 |  | Data transfer of E-UTRA radio bearer combinations 5, 6, 8, 11 and 12 / MIMO |  | X |  | X |  | - |  |
| 12.3.4 |  | Data transfer of E-UTRA radio bearer combination 13 / MIMO |  | X |  | X |  | - |  |
| 13.1.1 |  | Activation and deactivation of additional data radio bearer in E-UTRA |  | X |  | X |  | - |  |
| 13.1.2 |  | Call setup from E-UTRAN RRC_IDLE / CS fallback to UTRAN with redirection / MO call |  | X |  | X |  | X |  |
| 13.1.2a |  | Call setup from E-UTRAN RRC_IDLE / CS fallback to UTRAN with redirection including System Information / MO call |  | X |  | X |  | X |  |
| 13.1.3 |  | Call setup from E-UTRAN RRC_CONNECTED / CS fallback to UTRAN with redirection / MT call |  | X |  | X |  | X |  |
| 13.1.4 |  | Call setup from E-UTRAN RRC_IDLE / CS fallback to UTRAN with handover / MT call |  | X |  | X |  | X |  |
| 13.1.5 |  | Call setup from E-UTRAN RRC_CONNECTED / CS fallback to UTRAN with handover / MO call |  | X |  | X |  | X |  |
| 13.1.7 |  | Call setup from E-UTRA RRC_IDLE / CS fallback to GSM with redirection / MT call |  | X |  | X |  | - |  |
| 13.1.8 |  | Call setup from E-UTRA RRC_CONNECTED / CS fallback to GSM with redirection / MO call |  | X |  | X |  | - |  |
| 13.1.9 |  | Call setup from E-UTRA RRC_IDLE / CS fallback to GSM with CCO without NACC / MO call |  | X |  | X |  | - |  |
| 13.1.10 |  | Call setup from E-UTRA RRC_CONNECTED / CS fallback to GSM with CCO without NACC / MT call |  | X |  | X |  | - |  |
| 13.1.15 |  | Call setup from E-UTRAN RRC_IDLE / CS fallback to UTRAN with redirection / MT call / UTRAN cell is barred |  | X |  | X |  | X |  |
| 13.1.16 |  | Emergency call setup from E-UTRAN RRC_IDLE / CS fallback to UTRAN with handover |  | X |  | X |  | X |  |
| 13.1.17 |  | Call setup from E-UTRAN RRC_IDLE / mobile originating 1xCS fallback emergency call to 1xRTT |  | X |  | X |  | - |  |
| 13.1.18 |  | Call setup from E-UTRAN RRC_IDLE / mobile originating enhanced 1xCS fallback emergency call to 1xRTT |  | X |  | X |  | - |  |
| 13.1.19 |  | Emergency call setup from E-UTRAN RRC_IDLE / IMS VoPS supported / EMC BS not supported / CS fallback to UTRAN or GERAN with redirection |  | X |  | X |  | X |  |
| 13.1.20 |  | Emergency call setup from E-UTRAN RRC_IDLE / IMS VoPS not supported / EMC BS supported / CS fallback to UTRAN or GERAN with redirection |  | X |  | X |  | X |  |
| 13.2.1 |  | RRC connection reconfiguration/E-UTRA to E-UTRA |  | X |  | X |  | - |  |
| 13.3.1.1 |  | Intra-system connection re-establishment/Radio link recovery while T310 is running |  | X |  | X |  | - |  |
| 13.3.1.2 |  | Intra-system connection re-establishment/Re-establishment of a new connection when further data is to be transferred |  | X |  | X |  | - |  |
| 13.3.1.3 |  | RRC connection reconfiguration / Full configuration / DRB establishment |  | X |  | X |  | - |  |
| 13.3.2.1 |  | Inter-system connection re-establishment / E-UTRAN to UTRAN / Further data are to be transferred |  | X |  | X |  | X |  |
| 13.3.2.2 |  | Inter-system connection re-establishment / E-UTRAN to GPRS / Further data are to be transferred |  | X |  | X |  | - |  |
| 13.4.1.2 |  | Inter-frequency mobility/E-UTRA to E-UTRA packet |  | X |  | X |  | - |  |
| 13.4.1.3 |  | Intra-system mobility / E-UTRA FDD to E-UTRA TDD to E-UTRA FDD packet |  | X |  | X |  | - |  |
| 13.4.1.4 |  | Inter-band mobility / E-UTRA to E-UTRA packet |  | X |  | X |  | - |  |
| 13.4.1.5 |  | RRC connection reconfiguration / Handover / Full configuration / DRB establishment |  | X |  | X |  | - |  |
| 13.4.2.1 |  | Inter-system mobility / E-UTRA to UTRA packet |  | X |  | X |  | X |  |
| 13.4.2.4 |  | Inter-system mobility / Service based redirection from UTRA to E-UTRA |  | X |  | X |  | X |  |
| 13.4.2.5 |  | Inter-system mobility / Service based redirection from GSM/GPRS to E-UTRA |  | X |  | X |  | - |  |
| 13.4.3.1 |  | Inter-system mobility / E-UTRA voice to UTRA CS voice / SRVCC |  | X |  |  |  |  |  |
| 13.4.3.2 |  | Inter-system mobility / E-UTRA PS voice + PS data to UTRA CS voice + PS data / SRVCC |  | X |  |  |  |  |  |
| 13.4.3.3 |  | Inter-system mobility / E-UTRA voice to GSM CS voice / SRVCC |  | X |  | X |  | - |  |
| 13.4.3.4 |  | Inter-system mobility / E-UTRA voice to UTRA CS voice / Unsuccessful case / Retry on old cell / SRVCC |  | X |  |  |  |  |  |
| 13.4.3.5 |  | Inter-system mobility / E-UTRA voice to GSM CS voice / Unsuccessful case / Retry on old cell / SRVCC |  | X |  | X |  | - |  |
| 13.4.3.6 |  | Inter-system mobility / E-UTRA PS voice + PS Data / HO cancelled / Notification procedure / SRVCC |  | X |  |  |  | X |  |
| 13.4.3.7 |  | Inter-system mobility / E-UTRA voice to UTRA CS voice / aSRVCC / MO call |  | X |  |  |  | X |  |
| 13.4.3.8 |  | Inter-system mobility / E-UTRA voice to UTRA CS voice / aSRVCC / MO call / Forked responses |  | X |  |  |  | X |  |
| 13.4.3.9 |  | Inter-system mobility / E-UTRA voice to UTRA CS voice / aSRVCC / MO call / SRVCC HO failure |  | X |  | X |  | X |  |
| 13.4.3.10 |  | Inter-system mobility / E-UTRA voice to UTRA CS voice / aSRVCC / MT call |  | X |  |  |  | X |  |
| 13.4.3.11 |  | Inter-system mobility / E-UTRA voice to UTRA CS voice / aSRVCC / MT call / SRVCC HO failure |  | X |  | X |  | X |  |
| 13.4.3.13 |  | Inter-system mobility / E-UTRA voice to UTRA CS voice / aSRVCC / MT call / User answers in PS domain / SRVCC HO cancelled |  | X |  | X |  | X |  |
| 13.4.3.14 |  | Inter-system mobility / E-UTRA PS voice + PS data to UTRA CS voice + PS data / aSRVCC / MO call |  | X |  |  |  | X |  |
| 13.4.3.15 |  | Inter-system mobility / E-UTRA PS voice + PS data to UTRA CS voice + PS data / aSRVCC / MO call / SRVCC HO cancelled |  | X |  |  |  | X |  |
| 13.4.3.16 |  | Inter-system mobility / E-UTRA PS voice + PS data to UTRA CS voice + PS data / aSRVCC / MT call |  | X |  |  |  | X |  |
| 13.4.3.18 |  | Inter-system mobility / E-UTRA PS voice + PS data to UTRA CS voice + PS data / bSRVCC / MO call |  | X |  |  |  |  |  |
| 13.4.3.19 |  | Inter-system mobility / E-UTRA PS voice + PS data to UTRA CS voice + PS data / bSRVCC / MO call / SRVCC HO cancelled |  | X |  |  |  | X |  |
| 13.4.3.20 |  | Inter-system mobility / E-UTRA voice to UTRA CS voice / bSRVCC / MO call / SRVCC HO failure |  | X |  |  |  | X |  |
| 13.4.3.21 |  | Inter-system mobility / E-UTRA PS voice to GSM CS voice / bSRVCC / MO call |  | X |  | X |  | - |  |
| 13.4.3.22 |  | Inter-system mobility / E-UTRA PS voice to GSM CS voice / bSRVCC / MO call / SRVCC HO cancelled |  | X |  | X |  | - |  |
| 13.4.3.23 |  | Inter-system mobility / E-UTRA voice to GSM CS voice / bSRVCC / MO call / SRVCC HO failure |  | X |  | X |  | - |  |
| 13.4.3.24 |  | Inter-system mobility / E-UTRA voice to GSM CS voice / aSRVCC / MO call |  | X |  | X |  | - |  |
| 13.4.3.25 |  | Inter-system mobility / E-UTRA voice to GSM CS voice / aSRVCC / MO call / Forked responses |  | X |  | X |  | - |  |
| 13.4.3.26 |  | Inter-system mobility / E-UTRA voice to GSM CS voice / aSRVCC / MO call / SRVCC HO failure |  | X |  | X |  | - |  |
| 13.4.3.27 |  | Inter-system mobility / E-UTRA voice to GSM CS voice / aSRVCC / MT call |  | X |  | X |  | - |  |
| 13.4.3.28 |  | Inter-system mobility / E-UTRA voice to GSM CS voice / aSRVCC / MT call / SRVCC HO failure |  | X |  | X |  | - |  |
| 13.4.3.30 |  | Inter-system mobility / E-UTRA voice to GSM CS voice / aSRVCC / MT call / User answers in PS domain/ SRVCC HO cancelled |  | X |  | X |  | - |  |
| 13.4.3.32 |  | Inter-system mobility / UTRA CS voice to E-UTRA voice / rSRVCC |  | X |  |  |  |  |  |
| 13.4.3.34 |  | Inter-system mobility / UTRA CS voice to E-UTRA voice / alerting / rSRVCC / MO call |  | X |  |  |  |  |  |
| 13.4.3.41 |  | Inter-system mobility / E-UTRA PS voice to GSM CS voice / HO cancelled / Notification procedure / SRVCC |  | X |  | X |  | - |  |
| 13.4.4.1 |  | Pre-registration at 1xRTT and Cell reselection / 1x Zone Registration |  | X |  | X |  | - |  |
| 13.4.4.2 |  | Pre-registration at 1xRTT and Cell reselection / 1x Ordered Registration |  | X |  |  |  |  |  |
| 13.4.4.5 |  | Pre-Registration at 1xRTT / Power Down Registration |  | X |  | X |  | - |  |
| 13.5.1 |  | MTSI MO speech call / SSAC / 0% access probability for MTSI MO speech call |  | X |  | X |  | - |  |
| 13.5.1a |  | MTSI MO speech call / SSAC in Connected mode / 0% access probability for MTSI MO speech call |  | X |  | X |  | - |  |
| 13.5.1c |  | MTSI MO speech call / SSAC / 0% access probability for MTSI MO speech call / AC-Barring per PLMN |  | X |  |  |  | - |  |
| 13.5.1d |  | MTSI MO speech call / SSAC in Connected mode / 0% access probability for MTSI MO speech call / AC-Barring per PLMN |  | X |  |  |  | - |  |
| 13.5.2 |  | MTSI MO video call / SSAC / 0% access probability for MTSI MO video call |  | X |  | X |  | - |  |
| 13.5.2a |  | MTSI MO video call / SSAC in connected mode / 0% access probability for MTSI MO video call |  | X |  | X |  | - |  |
| 13.5.2c |  | MTSI MO video call / SSAC / 0% access probability for MTSI MO video call / AC-Barring per PLMN |  | X |  |  |  | - |  |
| 13.5.2d |  | MTSI MO video call / SSAC in Connected mode / 0% access probability for MTSI MO video call / AC-Barring per PLMN |  | X |  |  |  | - |  |
| 13.5.3 |  | Emergency call / Success / SSAC / 0% access probability for MTSI MO speech call |  | X |  | X |  | - |  |
| 13.5.3a |  | Emergency call / Success / SSAC in connected mode / 0% access probability for MTSI MO speech call |  | X |  | X |  | - |  |
| 13.5.3b |  | Emergency call / Success / SSAC / 0% access probability for MTSI MO speech call / AC-Barring per PLMN |  | X |  |  |  | - |  |
| 13.5.3c |  | Emergency call / Success / SSAC in Connected mode / 0% access probability for MTSI MO speech call / AC-Barring per PLMN |  | X |  |  |  | - |  |
| 13.5.4 |  | MTSI MO speech call / SCM / 0% access probability skip for MTSI MO speech call |  | X |  | X |  | - |  |
| 13.5.4a |  | MTSI MO speech call / SCM / 0% access probability skip for MTSI MO speech call / AC-Barring per PLMN |  | X |  |  |  | - |  |
| 13.5.5 |  | MTSI MO video call / SCM / 0% access probability skip for MTSI MO video call |  | X |  | X |  | - |  |
| 13.5.5a |  | MTSI MO video call / SCM / 0% access probability skip for MTSI MO video call / AC-Barring per PLMN |  | X |  |  |  | - |  |
| 13.5.6 |  | MTSI MO SMS / SCM / 0% access probability skip for MTSI MO SMS over IP |  | X |  | X |  | - |  |
| 13.5.6a |  | MTSI MO SMS / SCM / 0% access probability skip for MTSI MO SMS over IP / AC-Barring per PLMN |  | X |  |  |  | - |  |
| 14.1 |  | ETWS reception in RRC_IDLE state / Duplicate detection |  | X |  |  |  | - |  |
| 14.2 |  | ETWS reception in RRC_CONNECTED state / Duplicate detection |  | X |  |  |  | - |  |
| 17.1.1 |  | MCCH information acquisition/ UE is switched on |  | X |  | X |  | - |  |
| 17.1.2 |  | MCCH information acquisition/UE cell reselection to a cell in a new MBSFN area |  | X |  | X |  | - |  |
| 17.1.3 |  | MCCH information acquisition/UE handover to a cell in a new MBSFN area |  | X |  | X |  | - |  |
| 17.1.4 |  | MCCH information acquisition/ UE is receiving an MBMS service |  | X |  | X |  | - |  |
| 17.1.5 |  | MCCH information acquisition/UE is not receiving MBMS data |  | X |  | X |  | - |  |
| 17.2.1 |  | UE acquire the MBMS data based on the SIB13 and MCCH message / MCCH and MTCH are on the same MCH |  | X |  | X |  | - |  |
| 17.2.2 |  | UE acquire the MBMS data based on the SIB13 and MCCH message / MCCH and MTCH are on different MCHs |  | X |  | X |  | - |  |
| 17.2.3 |  | UE receives the MBMS data when this data is in the beginning of the MSAP |  | X |  | X |  | - |  |
| 17.2.4 |  | Reception of PDCCH DCI format 0 and PHICH in MBSFN subframes. |  | X |  | X |  |  |  |
| 17.3.1 |  | MBMS Counting / UE not receiving MBMS service |  | X |  | X |  | - |  |
| 17.3.2 |  | MBMS Counting / UE receiving MBMS service |  | X |  | X |  | - |  |
| 17.4.1 |  | Cell reselection to intra-frequency cell to start MBMS service |  | X |  | X |  | - |  |
| 17.4.1a |  | Cell reselection to intra-frequency cell to continue MBMS service reception / Single Frequency operation (inter-band neighbouring cell) |  | X |  | X |  | - |  |
| 17.4.2 |  | Cell reselection to inter- frequency cell to start MBMS service |  | X |  | X |  | - |  |
| 17.4.2a |  | Cell reselection to inter- band cell to start MBMS service reception |  | X |  | X |  | - |  |
| 17.4.3 |  | Handover to inter-frequency cell to start MBMS service reception |  | X |  | X |  | - |  |
| 17.4.3a |  | Handover to inter-band cell to start MBMS service reception |  | X |  | X |  | - |  |
| 17.4.4 |  | MBMS service continuity / Handover to inter-frequency cell / MBMS Interest Indication on cell not broadcasting SIB15 |  | X |  | X |  | - |  |
| 17.4.5 |  | Conditional retransmission of MBMS Interest Indication after handover |  | X |  | X |  | - |  |
| 17.4.6 |  | MBMS service continuity / E-UTRAN release of unicast bearer |  | X |  | X |  | - |  |
| 17.4.7 |  | MBMS Interest Indication after Radio Link Failure |  | X |  | X |  | - |  |
| 17.4.8 |  | Continue MBMS service reception after E-UTRAN release of unicast bearer |  | X |  | X |  | - |  |
| 17.4.9.1 |  | CA / Start MBMS reception on Non-Serving Cell / Continue MBMS reception on SCell after SCell addition / Intra-band Contiguous CA |  | X |  |  |  | - |  |
| 17.4.9.2 |  | CA / Start MBMS reception on Non-Serving Cell / Continue MBMS reception on SCell after SCell addition / Inter-band CA |  | X |  |  |  | - |  |
| 17.4.10.1 |  | CA / Start MBMS reception on SCell / Continue MBMS reception on Non-Serving after SCell release / Intra-band Contiguous CA |  | X |  | X |  | - |  |
| 17.4.10.2 |  | CA / Start MBMS reception on SCell / Continue MBMS reception on Non-Serving after SCell release / Inter-band CA |  | X |  | X |  | - |  |
| 17.4.11.1 |  | CA / Start MBMS reception on PCell / Continue MBMS reception after swap of SCell and PCell / Intra-band Contiguous CA |  | X |  | X |  | - |  |
| 17.4.11.2 |  | CA / Start MBMS reception on PCell / Continue MBMS reception after swap of SCell and PCell / Inter-band CA |  | X |  | X |  | - |  |
| 18.1.1 |  | PWS reception in RRC_IDLE state / Duplicate detection |  | X |  | - |  | - |  |
| 18.1.2 |  | PWS reception in RRC_CONNECTED state / Duplicate detection |  | X |  | - |  | - |  |
| 18.1.3 |  | PWS reception in RRC_CONNECTED State/Power On |  | X |  | - |  | - |  |
| 22.1.1.M3 |  | NB-IoT / Control Plane CIoT EPS optimisation for EPS services - Module 3 |  | X |  | - |  | - |  |
| 22.1.2 |  | NB-IoT / NTN / GNSS position |  | X |  | - |  | - |  |
| 22.2.1 |  | NB-IoT / PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode |  | X |  | - |  | - |  |
| 22.2.2 |  | NB-IoT / PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Manual mode |  | X |  | - |  | - |  |
| 22.2.3 |  | NB-IoT / PLMN selection / Periodic reselection / MinimumPeriodicSearchTimer |  | X |  | - |  | - |  |
| 22.2.4 |  | NB-IoT / Cell selection / Qrxlevmin and Qqualmin / Serving cell becomes non-suitable (S<0 or barred or Srxlev > 0 and Squal < 0) |  | X |  | - |  | - |  |
| 22.2.5 |  | NB-IoT / Cell reselection / Qhyst, Qoffset, Treselection and Cell-specific reselection parameters |  | X |  | - |  | - |  |
| 22.2.6 |  | NB-IoT / Cell reselection using cell status and cell reservations / Access control class 0 to 9 |  | X |  | - |  | - |  |
| 22.2.7 |  | NB-IoT / Cell reselection using cell status and cell reservations / Access control class 11 to 15 |  | X |  | - |  | - |  |
| 22.2.8 |  | NB-IoT / Cell reselection in shared network environment |  | X |  | - |  | - |  |
| 22.2.9 |  | NB-IoT / Inter-frequency cell reselection |  | X |  | - |  | - |  |
| 22.2.10 |  | NB-IoT / Cell reselection / MFBI |  | X |  | - |  | - |  |
| 22.2.13 |  | NB-IoT / NTN / Multi-TAC |  | X |  | - |  | - |  |
| 22.3.1.1 |  | NB-IoT / RACH Procedure/Preamble Selected by MAC/Temporary C-RNTI |  | X |  | - |  | - |  |
| 22.3.1.2 |  | NB-IoT / Correct Handling of DL MAC PDU/Assignment/HARQ process / TimeAlignmentTimer expiry |  | X |  | - |  | - |  |
| 22.3.1.3 |  | NB-IoT / Correct Handling of UL MAC PDU/Assignment/HARQ process/Padding |  | X |  | - |  | - |  |
| 22.3.1.4 |  | NB-IoT / Correct handling of MAC control information / Buffer status |  | X |  | - |  | - |  |
| 22.3.1.5 |  | NB-IoT / DRX operation / DRX cycle configured / Parameters configured by RRC/ DRX command MAC control element reception |  | X |  | - |  | - |  |
| 22.3.1.6 |  | NB-IoT / DL-SCH /UL-SCH transport block size selection / DCI format N1/ N0 |  | X |  | - |  | - |  |
| 22.3.1.6a |  | NB-IoT / DL-SCH /UL-SCH transport block size selection / DCI format N1/ N0 / Category NB2 |  | X |  | - |  | - |  |
| 22.3.1.7 |  | NB-IoT / RACH Procedure / Contention free random access (CFRA) |  | X |  | - |  | - |  |
| 22.3.1.8 |  | NB-IoT / RACH Procedure / Non-anchor carriers |  | X |  | - |  | - |  |
| 22.3.1.9 |  | NB-IoT / Correct HARQ process / 2 HARQ processes |  | X |  | - |  | - |  |
| 22.3.1.10 |  | NB-IoT / RACH Procedure / Early contention resolution |  | X |  | - |  | - |  |
| 22.3.1.13 |  | NB-IoT / NTN / UE specific TA report / UE specific Koffset |  | X |  | - |  | - |  |
| 22.3.2.1 |  | NB-IoT / AM RLC / Correct use of sequence numbering / Concatenation and reassembly /Polling for status |  | X |  | - |  | - |  |
| 22.3.2.2 |  | NB-IoT / AM RLC / Receiver status triggers |  | X |  | - |  | - |  |
| 22.3.2.3 |  | NB-IoT / AM RLC / In sequence delivery of upper layers PDUs/ Different numbers of length indicators |  | X |  | - |  | - |  |
| 22.3.2.4 |  | NB-IoT / AM RLC / Re-segmentation RLC PDU / SO, FI, LSF/ Re-transmission of RLC PDU |  | X |  | - |  | - |  |
| 22.3.2.5 |  | NB-IoT / AM RLC / Segmentation and Reassembly / AMD PDU reassembly Re-ordering, from AMD PDU segments / FI, SO and LSF |  | X |  | - |  | - |  |
| 22.3.2.7 |  | NB-IoT / AM RLC / Receiver status triggers / Non-zero t-Reordering configured |  | X |  | - |  | - |  |
| 22.3.2.7a |  | NB-IoT / NTN / AM RLC / Receiver status triggers / extended t-Reordering configured |  | X |  | - |  | - |  |
| 22.3.3.1 |  | NB-IoT / Maintenance of PDCP sequence numbers / User plane / RLC AM |  | X |  | - |  | - |  |
| 22.3.3.2 |  | NB-IoT / Integrity protection / Ciphering and deciphering / Correct functionality of EPS AS and UP encryption algorithms / SNOW3G |  | X |  | - |  | - |  |
| 22.3.3.3 |  | NB-IoT / Integrity protection / Ciphering and deciphering / Correct functionality of EPS AS and UP encryption algorithms / AES |  | X |  | - |  | - |  |
| 22.3.3.4 |  | NB-IoT / Integrity protection / Ciphering and deciphering / Correct functionality of EPS AS and UP encryption algorithms / ZUC |  | X |  | - |  | - |  |
| 22.3.3.5 |  | NB-IoT / PDCP re-establishment / stored UE AS context is used and drb-ContinueROHC is configured |  | X |  | - |  | - |  |
| 22.3.3.6 |  | NB-IoT / PDCP discard |  | X |  | - |  | - |  |
| 22.4.1 |  | NB-IoT / Notification of BCCH modification in idle mode / eDRX cycle longer than the modification period |  | X |  | - |  | - |  |
| 22.4.2 |  | NB-IoT / Paging for connection in idle mode / Multiple paging records |  | X |  | - |  | - |  |
| 22.4.4 |  | NB-IoT / RRC connection establishment / Paging / Access Barring for UE with AC 0 to 9 / ab-Category a, b and c |  | X |  | - |  | - |  |
| 22.4.5 |  | NB-IoT / RRC connection establishment / Paging / Access Barring for UE with AC 11 to 15 / ab-Category a, b and c |  | X |  | - |  | - |  |
| 22.4.6 |  | NB-IoT / Paging for notification of BCCH modification in idle mode / Direct indication for SI update |  | X |  | - |  | - |  |
| 22.4.7 |  | NB-IoT / RRC connection release with extendedWait/extendedWait ignored/ RRC connection establishment / Reject with extendedWait |  | X |  | - |  | - |  |
| 22.4.8 |  | NB-IoT / RRC connection establishment / Access Barring for UE with AC 0 to 9 / MO exception data |  | X |  | - |  | - |  |
| 22.4.9 |  | NB-IoT / RRC connection establishment / Access Barring for UE with AC 11 to 15 / MO exception data / ab-Category a, b and c |  | X |  | - |  | - |  |
| 22.4.11 |  | NB-IoT / RRC connection release / Redirection to another NB-IoT frequency |  | X |  | - |  | - |  |
| 22.4.12 |  | NB-IoT / RRC connection release / Redirection to another NB-IoT band |  | X |  | - |  | - |  |
| 22.4.13 |  | NB-IoT / UE capability transfer / Success |  | X |  | - |  | - |  |
| 22.4.13a |  | NB-IoT / NTN / UE capability transfer / Success |  | X |  | - |  | - |  |
| 22.4.14 |  | NB-IoT / RRC Connection Establishment / Multi-Carrier |  | X |  | - |  | - |  |
| 22.4.15 |  | NB-IoT / RRC connection suspend-resume / Success / different cell |  | X |  | - |  | - |  |
| 22.4.16 |  | NB-IoT / RRC connection suspend-resume / Failure / Network reject |  | X |  | - |  | - |  |
| 22.4.18 |  | NB-IoT / RRC connection reconfiguration / SRB reconfiguration / Success |  | X |  | - |  | - |  |
| 22.4.19 |  | NB-IoT / Radio link failure / T301 expiry/ T311 expiry |  | X |  | - |  | - |  |
| 22.4.19a |  | NB-IoT / Radio link failure / T301 expiry / T311 expiry / RRC connection re-establishment |  | X |  | - |  | - |  |
| 22.4.20 |  | NB-IoT / Radio link failure / RRC connection re-establishment reject |  | X |  | - |  | - |  |
| 22.4.20a |  | NB-IoT / Radio link failure / RRC connection re-establishment reject / RRC connection re-establishment |  | X |  | - |  | - |  |
| 22.4.21 |  | NB-IoT / Radio link failure / Radio link recovery while T310 is running |  | X |  | - |  | - |  |
| 22.4.22 |  | NB-IoT / Radio link failure / T311 expiry / Dedicated RLF timer (UP CIoT) |  | X |  | - |  | - |  |
| 22.4.23 |  | NB-IoT / Radio link failure / T310 expiry / Dedicated RLF timer (CP CIoT) |  | X |  | - |  | - |  |
| 22.4.24 |  | NB-IoT / RRC / Paging for connection in idle mode / Non-anchor carriers |  | X |  | - |  | - |  |
| 22.4.26 |  | NB-IoT / RRC connection establishment / Extended and spare fields in SI |  | X |  | - |  | - |  |
| 22.4.27 |  | NB-IoT / RRC connection establishment / Access barring enhancement |  | X |  |  |  | - |  |
| 22.4.30 |  | NB-IoT / NTN / Ephemeris information update / T317 Expiry / T318 Expiry |  | X |  | - |  | - |  |
| 22.5.1 |  | NB-IoT / Authentication not accepted by the network, GUTI used / Authentication not accepted by the UE, SQN failure / Authentication not accepted by the UE, non-EPS authentication unacceptable / Network failing the authentication check |  | X |  | - |  | - |  |
| 22.5.2 |  | NB-IoT / NAS Security / Handling of null integrity protection and null ciphering algorithms / NAS count reset to zero / Security mode command with not matching replayed security capabilities / Provision of IMEISV and IMEI |  | X |  | - |  | - |  |
| 22.5.3 |  | NB-IoT / NW initiated detach Re-attach required / UE initiated detach Abnormal case EMM common procedure collision / UE initiated detach Abnormal case Local detach after 5 attempts due to no network response |  | X |  | - |  | - |  |
| 22.5.4 |  | NB-IoT / Attach to new PLMN GUTI reallocation / Network reject with Extended Wait Timer / Paging with IMSI / Attach Rejected Illegal ME/UE / Detach upon switch-off |  | X |  | - |  | - |  |
| 22.5.5 |  | NB-IoT / Attach Procedure / Success / List of equivalent PLMNs in the ATTACH ACCEPT message / Attach / Rejected / PLMN not allowed |  | X |  | - |  | - |  |
| 22.5.6 |  | NB-IoT / UE in NB-S1 mode supporting CIoT Optimizations / Attach Abnormal cases / EPS services not allowed / Failure due to non integrity protection / Unsuccessful attach after 5 attempts / Repeated rejects for network failures / Change of cell into a new tracking area / Detach procedure collision / UE initiated detach USIM removed from the UE |  | X |  | - |  | - |  |
| 22.5.7a |  | NB-IoT / Periodic tracking area update Accepted / Normal tracking area update List of equivalent PLMNs in the TRACKING AREA UPDATE ACCEPT message / Normal tracking area update Rejected (IMSI invalid / Illegal ME / UE identity cannot be derived by the network / UE implicitly detached / PLMN not allowed |  | X |  | - |  | - |  |
| 22.5.7b |  | NB-IoT / Normal tracking area update Rejected (Tracking area not allowed / No suitable cells in tracking area / Roaming not allowed in this tracking area / Congestion) / UE initiated detach Abnormal case Change of cell into a new tracking area |  | X |  | - |  | - |  |
| 22.5.8 |  | NB-IoT / Normal tracking area update Abnormal case / Success or fail after several attempts due to no network response / TA belongs to TAI list and status is UPDATED / TRACKING AREA UPDATE REJECT / Change of cell into a new tracking area / Tracking area updating and detach procedure collision |  | X |  | - |  | - |  |
| 22.5.9 |  | NB-IoT / UE in NB-S1 mode supporting CIoT Optimizations / Paging with not matching identity / Control Plane Service request Rejected (IMSI invalid / Illegal ME / EPS services not allowed / UE identity cannot be derived by the network / UE implicitly detached) |  | X |  | - |  | - |  |
| 22.5.10 |  | NB-IoT / EPS NAS integrity and encryption / SNOW 3G |  | X |  | - |  | - |  |
| 22.5.11 |  | NB-IoT / EPS NAS integrity and encryption / AES |  | X |  | - |  | - |  |
| 22.5.12 |  | NB-IoT / EPS NAS integrity and encryption / ZUC |  | X |  | - |  | - |  |
| 22.5.13 |  | NB-IoT / Attach Procedure / Success / Last visited TAI, TAI list and equivalent PLMN list handling |  | X |  | - |  | - |  |
| 22.5.14 |  | NB-IoT / Attach / Rejected / Tracking Area not allowed / Roaming not allowed in this tracking area / No suitable cells in tracking area |  | X |  | - |  | - |  |
| 22.5.15 |  | NB-IoT / Normal tracking area update / low priority override |  | X |  | - |  | - |  |
| 22.5.16 |  | NB-IoT / Normal tracking area update / Rejected / EPS service not allowed /EPS services not allowed in this PLMN |  | X |  | - |  | - |  |
| 22.5.17 |  | NB-IoT / Attach Success /Normal tracking area update accepted / Periodic tracking area update T3412 Extended Value / PSM |  | X |  | - |  | - |  |
| 22.5.18 |  | NB-IoT / Attach & Normal tracking area update Procedure / Success / without Idle eDRX parameters / With Idle eDRX parameters/ With and without Idle eDRX and PSM parameters |  | X |  | - |  | - |  |
| 22.5.20 |  | NB-IoT/ UE in NB-S1 mode supporting control plane data back-off timer/ Service reject with extended wait time CP data/ Release with extended wait time CP data/ Attach accept with extended wait time CP data |  | X |  | - |  | - |  |
| 22.5.21 |  | NB-IoT / APN rate control for MO exception data |  | X |  | - |  | - |  |
| 22.5.23 |  | NB-IoT / NTN / GNSS position reporting / reject cause #78 "PLMN not allowed to operate at the present UE location" |  | X |  | - |  | - |  |
| 22.6.1 |  | NB-IoT / UE routing of uplinks packets / User Plane / UE requested PDN disconnect procedure accepted by the network |  | X |  | - |  | - |  |
| 22.6.1a |  | NB-IoT / UE routing of uplinks packets / Control Plane |  | X |  | - |  | - |  |
| 22.6.2 |  | NB-IoT / UE requested bearer resource modification accepted by the network / Default EPS bearer context |  | X |  | - |  | - |  |
| 22.6.3 |  | NB-IoT / UE requested bearer resource modification error handling (Resource modification not accepted by the network) / Expiry of timer T3481/ Default EPS bearer context |  | X |  | - |  | - |  |
| 22.6.5 |  | NB-IoT / UE requested PDN connectivity procedure not accepted / UE requested PDN connectivity accepted Dual priority T3396 override UE requested PDN connectivity accepted / Dual priority / T3346 override |  | X |  | - |  | - |  |
| 23.1.1 |  | CIoT / Control Plane MO and MT IP and non-IP Data Transfer / Serving PLMN Rate Control / APN Rate Control |  | X |  |  |  | - |  |
| 23.1.2 |  | CIoT Optimization / Control Plane / MT and MO SMS Data Transfer |  | X |  |  |  | - |  |
| 24.1.2 |  | V2X Sidelink Communication / Pre-configured authorisation / Utilisation of the pre-configured resources / Transmission |  | - |  | X |  | - |  |
| 24.1.4 |  | V2X Sidelink Communication/ Pre-configured authorisation / Utilisation of the pre-configured resources / Reception |  | - |  | X |  | - |  |
| 24.1.16 |  | V2X Sidelink Communication/ Pre-configured authorisation / Utilisation of the pre-configured resources / CBR measurement |  | - |  | X |  | - |  |
| 24.1.19 |  | V2X Sidelink Communication/ Pre-configured authorisation / Utilisation of the pre-configured resources / CBR measurement/Transmission based on CR limit |  | - |  | X |  | - |  |

The Test Suite in TTCN-3 is contained in multiple ASCII files which accompany the present document.

Annex F (informative)
TTCN-3 Definitions

# F.1	NBIOT_ASP_TypeDefs

Type definitions for configuration of the system simulator;
Common design principles:
Semantics of OMIT: for all TTCN-3 type definitions used in ASPs omit means "keep as it is" =>
- on initial configuration in general all fields shall be provided
- no default values for fields are foreseen
- if necessary non-existence of information shall be explicitly configured
  (e.g. with a union of "no configuration" and "configuration parameters"
- fields within structures imported from the core spec are excepted from this rule

## F.1.1	ASN1_Container

Definitions containing ASN.1 types for backward compatibility;
NOTE 1: PCCH_Message and BCCH_DL_SCH_Message already have a critical extension mechanism by RRC type definition
NOTE 2: BCCH_BCH_Message contains the MIB and therefore is considered to be not extendable
NOTE 3: "simple types" are not considered: C_RNTI, PhysCellId, CellIdentity, ARFCN_ValueEUTRA

NPUSCH_ConfigDedicated_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NPUSCH_ConfigDedicated_Type |  |  |
| Comment |  |  |  |
| R13 | NPUSCH_ConfigDedicated_NB_r13 |  |  |
| Npusch_TxDuration_r17 | NPUSCH_TxDuration_NB_r17 | opt | used in NTN cell |

NPRACH_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NPRACH_Config_Type |  |  |
| Comment |  |  |  |
| R13 | NPRACH_ConfigSIB_NB_r13 |  |  |
| R14 | NPRACH_ParametersList_NB_r14 | opt |  |
| R17 | NPRACH_TxDuration_r17_Type | opt | used in NTN cell |

NPRACH_TxDuration_r17_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NPRACH_TxDuration_r17_Type |  |  |
| Comment |  |  |  |
| Nprach_TxDurationFmt01_r17 | NPRACH_TxDurationFmt01_NB_r17 | opt |  |
| Nprach_TxDurationFmt2_r17 | NPRACH_TxDurationFmt2_NB_r17 | opt |  |

NB_RACH_ConfigCommon_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_RACH_ConfigCommon_Type |  |  |
| Comment |  |  |  |
| R13 | RACH_ConfigCommon_NB_r13 |  |  |

NB_PDCP_Config_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_PDCP_Config_Type |  |
| Comment |  |  |
| R13 | PDCP_Config_NB_r13 |  |

NB_UL_AM_RLC_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_UL_AM_RLC_Type |  |  |
| Comment |  |  |  |
| R13 | UL_AM_RLC_NB_r13 |  |  |

NB_DL_AM_RLC_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_DL_AM_RLC_Type |  |  |
| Comment |  |  |  |
| R13 | DL_AM_RLC_NB_r13 |  |  |

NPUSCH_ConfigCommon_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NPUSCH_ConfigCommon_Type |  |  |
| Comment |  |  |  |
| R13 | NPUSCH_ConfigCommon_NB_r13 |  |  |
| Npusch_TxDuration_r17 | NPUSCH_TxDuration_NB_r17 | opt | used in NTN cell |

NB_DRX_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_DRX_Config_Type |  |  |
| Comment |  |  |  |
| R13 | DRX_Config_NB_r13 |  |  |

CarrierConfigDedicated_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CarrierConfigDedicated_Type |  |  |
| Comment |  |  |  |
| R13 | CarrierConfigDedicated_NB_r13 |  |  |

DL_GapConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DL_GapConfig_Type |  |  |
| Comment |  |  |  |
| R13 | DL_GapConfig_NB_r13 |  |  |

DL_Bitmap_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DL_Bitmap_Type |  |  |
| Comment |  |  |  |
| R13 | DL_Bitmap_NB_r13 |  |  |

NB_DL_ConfigCommon_NB_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_DL_ConfigCommon_NB_Type |  |  |
| Comment |  |  |  |
| R14 | DL_ConfigCommon_NB_r14 |  |  |

NB_UL_ConfigCommon_NB_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_UL_ConfigCommon_NB_Type |  |  |
| Comment |  |  |  |
| R14 | UL_ConfigCommon_NB_r14 |  |  |
| R15 | UL_ConfigCommon_NB_v1530 | opt | if present, configures NPRACH parameters Fmt2 for the UL non-anchor carrier configured by R14 |

## F.1.2	System_Configuration

Formal ASP Definitions for system configuration

System_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NB_DPR_DataVolumnIndex_Type | integer (0..15) |  |
| NB_DPR_PowerHeadroomLevel_Type | integer (0..3) |  |

NB_SystemRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_SystemRequest_Type |  |
| Comment |  |  |
| Cell | NB_CellConfigRequest_Type | configure/release a cell |
| CellAttenuationList | NB_CellAttenuationList_Type | power attenuation for one or several cells; all cells included in the list shall be changed at the same time; all cells in the list shall reach the new cell power within a maximum of 100ms (10 frames) acc. to the tolerances given in TS 36.508 NOTE: In the common ASP part the CellId shall be set - to the cell the timing information refers to if activation time shall be applied - to nbiot_Cell_NonSpecific when there is no activation time |
| RadioBearerList | NB_RadioBearerList_Type | configure/release one or several SRBs and/or DRBs |
| EnquireTiming | Null_Type | get HSFN, SFN and sub-frame number for this cell |
| AS_Security | NB_AS_Security_Type | StartRestart/Release of AS security |
| Paging | NB_PagingTrigger_Type | to trigger SS to send paging at the given paging occasion (as calculated in TTCN) or a Direct Indication |
| L1MacIndCtrl | NB_L1Mac_IndicationControl_Type | to configure SS to generate indications for L1/MAC events |
| PdcpCount | NB_PDCP_CountReq_Type | to set or enquire PDCP COUNT for one or more RBs |
| RlcCounts | NB_RLC_CountsReq_Type | to set or enquire RLC Counts VTS and VRR |
| NpdcchOrder | NB_RA_NPDCCH_Order_Type | to configure SS to transmit a NPDCCH order with configured C-RNTI to the UE to trigger RA procedure; result in DCI Format N1 transmission as in TS 36.212, clause 6.4.3.2; the SS shall schedule the NPDCCH order in the same way as DCI for DL transmissions |
| PtmConfig | NB_PTM_Config_Type | Configuration of Point To Multipoint (PTM) |
| WakeUpSignal | Null_Type | To trigger SS to send Wake Up Signal; CellId : identifier of the cell where the UE is active RoutingInfo : None TimingInfo : Wake Up Signal transmission start time, calculated by TTCN ControlInfo : CnfFlag:=false; FollowOnFlag:=false |

NB_SystemConfirm_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_SystemConfirm_Type |  |
| Comment | confirmations for system configuration; in general to be sent after the configuration has been done |  |
| Cell | Null_Type | (no further parameters from SS) |
| CellAttenuationList | Null_Type | (no further parameters from SS) NOTE 1: the confirmation shall be sent when all cells have changed power levels NOTE 2: for the CellId in the common ASP part the same rules are applied as for the SYSTEM REQ |
| RadioBearerList | Null_Type | (no further parameters from SS) |
| EnquireTiming | Null_Type | SFN and sub-frame number are included in the TimingInfo |
| AS_Security | Null_Type | (no further parameters from SS) |
| Paging | Null_Type | normally not needed but defined for completeness |
| L1MacIndCtrl | Null_Type | (no further parameters from SS) |
| PdcpCount | NB_PDCP_CountCnf_Type | as response to 'Get' a list is returned containing COUNT information for the requested RBs |
| RlcCounts | NB_RLC_CountsCnf_Type | as response to 'Get' a list is returned containing RLC Counts VTS and VRR |
| NpdcchOrder | Null_Type | confirmation for NPDCCH Order |
| WakeUpSignal | Null_Type | confirmation for activation of wake up signal |

NB_HarqError_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_HarqError_Type |  |
| Comment |  |  |
| UL | Null_Type | there is just one HARQ process for NB-IoT and uplink HARQ operation is asynchronous (see TS 36.321 clause 5.4.2) |
| DL | Null_Type |  |

NB_RachPreamble_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_RachPreamble_Type |  |  |
| Comment |  |  |  |
| RAPID | integer |  | indicates the RAPID of the preamble used (integer (0..63)); according to TS 36.321 clause 6.2.2 "For NB-IoT, the Random Access Preamble IDentifier field corresponds to the start subcarrier index"; in case of NPDCCH order it is calculated according to TS 36.321 clause 5.2.1 for CBRA or CFRA |
| RepetitionsPerPreambleAttempt | integer |  | number of preambles the SS has detected for the preamble attempt |
| NonAnchorCarrierId | integer | opt | Id of the non-anchor carrier (-> NB_NonAnchorCarrierCommonUL_Type.Id) on which the UE as sent the RACH preamble; omit when the RACH preamble has been sent on the anchor carrier |

NB_DPR_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_DPR_Type |  |  |
| Comment |  |  |  |
| DataVolumnIndex | NB_DPR_DataVolumnIndex_Type |  |  |
| PowerHeadroomLevel | NB_DPR_PowerHeadroomLevel_Type |  |  |

NB_SystemIndication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_SystemIndication_Type |  |
| Comment |  |  |
| Error | charstring | indicates an error situation in SS; is not explicitly handled in TTCN but causes an INCONC due to default behaviour; an additional error code can be signalled in the common part of the ASP; |
| RachPreamble | NB_RachPreamble_Type | RACH preamble being sent by the UE NOTE 1: for NB-IoT there can be more than one repetition per preamble attempt (numRepetitionsPerPreambleAttempt-r13 in NPRACH-ConfigSIB-NB-DEFAULT in SystemInformationBlockType2-NB); nevertheless the SS shall raise only one indication per preamble attempt and indicate the number of attempts in the 'RepetitionsPerPreambleAttempt' field of the indication NOTE 2: The SS shall provide start of the attempt's first preamble as timing information of the primitive (in a lab environment in general this is the start time of the NPRACH preamble transmission according to clause 10.1.6.1 of TS 36.211 - depending on nprach-Periodicity and nprach-StartTime) |
| DPR | NB_DPR_Type | indicates MAC DPR control element (TS 36.321 clause 6.1.3.10) being contained in MAC PDU |
| HarqError | NB_HarqError_Type | indicates detection of HARQ error: 1. HARQ CRC error for UL data 2. HARQ NACK from the UE unless SS is configured to report HARQ ACK/NACK |
| UL_HARQ | HARQ_Type | to report the UL HARQ ACK/NACK; In case of HARQ repetitions (i.e. ack-NACK-NumRepetitions-Msg4-r13 OR ack-NACK-NumRepetitions-r13 > r1) the SS shall raise only one indication per logical HARQ, but not an indication per HARQ repetition (e.g. with ack-NACK-NumRepetitions-r13 = r8 in NPUSCH-ConfigDedicated-NB-r13 still only one HARQ indication shall be raised). The SS shall provide start of UL HARQ feedback as timing information of the primitive. |
| TimingAdvanceReport | MAC_CTRL_TimingAdvanceReport_Type | To get UE reported TA |
| PurReception | Null_Type | To report that PUR was used by the UE |

## F.1.3	Cell_Configuration

Specific Info for Cell Configuration Primitive

### F.1.3.1	Cell_Configuration_Common

Cell_Configuration_Common: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NB_ToRS_EPRE_Ratio_Type | integer (-35..0) | any-resource-element to NRS ratio in dB (e.g. NPDSCH-to-NRS ratio; see TS 36.213, clause 16.2.2) |

NB_CellConfigRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_CellConfigRequest_Type |  |
| Comment |  |  |
| AddOrReconfigure | NB_CellConfigInfo_Type | for cell configuration: CellId : identifier of the cell to be configured RoutingInfo : None TimingInfo : Now (for initial configuration and for reconfiguration in general) ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |
| Release | Null_Type | to remove a cell completely - CellId : identifier of the cell to be released; nbiot_Cell_NonSpecific, in case all cells shall be released RoutingInfo : None TimingInfo : Now ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |

NB_CellConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_CellConfigInfo_Type |  |  |
| Comment | common information for initial cell configuration or reconfiguration; in case of reconfiguration OMIT means 'keep configuration as it is' |  |  |
| Basic | NB_BasicCellConfig_Type | opt | basic information for a cell (e.g. broadcasting) |
| Active | NB_ActiveCellConfig_Type | opt | add. configuration for active cell (i.e. cell being capable to receive RACH preamble) |

NB_BasicCellConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_BasicCellConfig_Type |  |  |
| Comment |  |  |  |
| StaticCellInfo | NB_StaticCellInfo_Type | opt | Common information which does not change during a test |
| PhysicalLayerConfigDL | NB_PhysicalLayerConfigDL_Type | opt | default settings regarding DL physical control channels |
| InitialCellPower | InitialCellPower_Type | opt | reference cell power for the NRS of each antenna in DL NOTE 1: the power of the RS of an antenna may be reduced by antenna specific configuration NOTE 2: in general the power may be adjusted on a per resource element basis => all physical channel/signal power settings shall be adjusted relatively to the NRS; if there are more than one TX antennas each one may have its own attenuation; independently from those relative power settings the cell power can easily be adjusted by just changing the reference power |
| DownlinkBitmapConfig | NB_DownlinkBitmapConfig_Type | opt | Usage of bitmap or no bitmap to define valid NB-IoT DL subframes; omit means "keep as it is" |
| BcchConfig | NB_BcchConfig_Type | opt | configuration of BCCH/BCH; SS is triggered to configure RLC/MAC regardingly; BCCH data on the NPDSCH NPBCH: MIB; NPDSCH: scheduling and resource allocation; SIBs |
| PcchConfig | NB_PcchConfig_Type | opt | configuration of PCCH/PCH; SS is triggered to configure RLC/MAC regardingly; PCCH data on the NPDSCH is distinguished by the P-RNTI (needed even to modify SI => shall be configured for CELL_BROADCASTING) |

NB_ActiveCellConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_ActiveCellConfig_Type |  |  |
| Comment |  |  |  |
| C_RNTI | EUTRA_ASN1_C_RNTI_Type | opt | (pre-)configured C-RNTI; affects scrambling of NPDSCH/NPUSCH and CRC of NPDCCH(s); shall be used implicitly in RACH procedure (i.e. as CE in RAR) |
| PhysicalLayerConfigUL | NB_PhysicalLayerConfigUL_Type | opt | parameters for NPRACH, NPUSCH Omitted when no UL frequency is applied to the band |
| RachProcedureConfig | NB_RachProcedureConfig_Type | opt | to configure the SS's behaviour for the RACH procedure Omitted when no UL frequency is applied to the band |
| CcchDcchDtchConfig | NB_CcchDcchDtchConfig_Type | opt | Parameters related to CCCH/DCCH/DTCH in UL and DL |
| NonAnchorCarrier | NB_NonAnchorCarrier_Type | opt | configuration for non-Anchor Carrier ("None" if there is no non-Anchor Carrier) |
| CarrierConfigCommonDL | NB_CarrierConfigCommonDL_Type | opt | to configure multiple NonAnchorCarriers for Paging in accordance with TS 36.304 clause 7.1.1 (r14 and later) |
| CarrierConfigCommonUL | NB_CarrierConfigCommonUL_Type | opt | to configure multiple NonAnchorCarriers for RACH in accordance with TS 36.321 clause 5.1.1 (r14 and later) |
| NtnConfig | NB_NTN_Config_Type | opt | to configure parameters specifically needed for NTN operation |
| PurConfig | NB_Preconfigured_UL_ResourceConfig_Type | opt | to configure parameters relevant for PUR operation |

NB_StaticCellInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_StaticCellInfo_Type |  |  |
| Comment | Common information which (normally) does not change during a test; therefore all fields are mandatory |  |  |
| Common | NB_CommonStaticCellInfo_Type |  | information common for UL and DL |
| Earfcn | NB_CarrierFreq_Type |  | DL or UL EARFCN as defined in TS 36.101 |

NB_CommonStaticCellInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_CommonStaticCellInfo_Type |  |  |
| Comment | information common for UL and DL; all fields are mandatory |  |  |
| PhysicalCellId | EUTRA_ASN1_PhysCellId_Type |  | N(Ncell, ID): imported from core spec; -> narrowband reference signals -> scrambling of all DL physical channels: NPBCH, NPDCCH, and NPDSCH (together with nRNTI) |
| NB_Band | integer |  |  |
| CellTimingInfo | CellTimingInfo_Type |  |  |
| OperationModeInfo | NB_OperationModeInfo_Type |  |  |
| LteCellInfo | NB_CommonStaticLteCellInfo_Type | opt | Only used for non-standalone operation mode, omitted otherwise. Provides information on the associated E-UTRA cell within which the NBIOT cell is operating, to enable LTE CRS broadcast or skipping (see TS 36.523-3 cl. 7A.1). |

NB_CommonStaticLteCellInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_CommonStaticLteCellInfo_Type |  |  |
| Comment |  |  |  |
| Earfcn | integer |  | E-UTRA cell DL EARFCN. |
| Bandwidth | EUTRA_ASN1_Dl_Bandwidth_Type |  | E-UTRA cell DL bandwidth. |
| CyclicPrefix | EUTRA_CyclicPrefix_Type |  | E-UTRA cell cyclic prefix. |
| PhysicalCellId | EUTRA_ASN1_PhysCellId_Type | opt | E-UTRA cell physical cell identity. If omitted, it is the same identity as the NBIOT cell identity. |
| EutraControlRegionSize | NB_EutraControlRegionSize_Type | opt | Only used for the in-band operation modes, omitted otherwise. Indicates the control region size of the E-UTRA cell. |

### F.1.3.2	Downlink_Physical_Layer_Configuration

Downlink physical layer configuration:
- DL antenna configuration
- control region (NPDCCH)
- primary/secondary sync signals
- power control for physical channels and signals

AntennaConfig_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | AntennaConfig_Type |
| Comment | This type specifies how many Tx antennas shall be used by the SS |
| AN1 | single Tx antenna shall be used by the SS |
| AN2 | two Tx antennas shall be used by the SS |

NB_PhysicalLayerConfigDL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_PhysicalLayerConfigDL_Type |  |  |
| Comment | all fields are declared as optional to allow single reconfigurations; in this case omit means "keep as it is" |  |  |
| Antenna | AntennaConfig_Type | opt |  |
| NPbch | NPbchConfig_Type | opt |  |
| NPdcch | NPdcchConfig_Type | opt |  |
| NPdsch | NPdschConfig_Type | opt |  |
| NPss | NB_PrimarySyncSignal_Type | opt |  |
| NSss | NB_SecondarySyncSignal_Type | opt |  |
| LteCrs | NB_LTE_CellSpecificReferenceSignal_Type | opt | if omitted in initial configuration the CRS shall be considered as being not present, i.e. shall not be transmitted by the SS |
| NWus | NB_WUS_Config_Type | opt | if omitted in initial configuration, the narrow band Wake-Up Signal shall be considered as being not present; if is present WUS shall be transmitted by the SS at the activation time given by WakeUpSignal trigger |

NB_DownlinkBitmapConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_DownlinkBitmapConfig_Type |  |
| Comment | to specify valid NB-IoT DL subframes according to TS 36.213 clause 16.4 |  |
| Bitmap | DL_Bitmap_Type | valid NB-IoT DL subframe specified by bitmap and does not contain NPSS/NSSS/NPBCH/NB-SIB1 transmission |
| None | Null_Type | valid NB-IoT DL subframe does not contain NPSS/NSSS/NPBCH/NB-SIB1 transmission |

NB_NonAnchorCarrier_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_NonAnchorCarrier_Type |  |
| Comment |  |  |
| Config | CarrierConfigDedicated_Type | SS shall apply given configuration of non-anchor-carrier in accordance with TS 36.331 clause 5.3.10.6 and TS 36.321 clause 5.1.1 |
| None | Null_Type |  |

#### F.1.3.2.1	Physical_Channels

Physical_Channels: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NPdcchSearchSpaceCandidateIndex_Type | integer (0..7) | Index of the search space candidate to be used for UL grant or DL assignment (corresponds to "u" in the calculations of clause 16.6 in TS 36.213) |

NPbchConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NPbchConfig_Type |  |  |
| Comment |  |  |  |
| RelativeTxPower | NB_ToRS_EPRE_Ratio_Type | opt | power ratio for NPBCH's resource elements relative to the NRS |

NPdcchFormat_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NPdcchFormat_Type |
| Comment | NPDCCH format according to 36.211 claues 10.2.5.1 => aggregation level according to 36.213 clause 16.6 |
| npdcchFormat0 |  |
| npdcchFormat1 |  |

NPdcchUESpecificSearchSpace_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NPdcchUESpecificSearchSpace_Type |  |  |
| Comment | parameters for UE specific search space (UESS, TS 36.213 clause 16.6); In addition the SS needs to consider the "DCISubFrameRepetitionNumber" in the DCI UL/DL configuration to determine the value of R (actual number of NPDCCH repetitions for the DCI) |  |  |
| NPdcchFormat | NPdcchFormat_Type |  | Aggregation level: According to tables 16.6-1, 16.6-2, 16.6-3 in TS 36.213 for typical search space configuration as per TS 36.508 NPDCCH format 1 is used i.e. there is just one search space candidate per NPDCCH transmission. Use cases for NPDCCH format 0 are FFS |
| NumRepetitions | NPDCCH_NumRepetitions_UESS_Type |  | Rmax for UE specific search space; TS 36.213 clause 16.6 |
| StartSF | NPDCCH_StartSF_UESS_Type |  | G for UE specific search space; TS 36.213 clause 16.6 |
| Offset | NPDCCH_Offset_UESS_Type |  | offset for UE specific search space; TS 36.213 clause 16.6 |
| SearchSpaceCandidateForDL | NPdcchSearchSpaceCandidateIndex_Type |  | index of the search space candidate to be used for DL assignment |
| SearchSpaceCandidateForUL | NPdcchSearchSpaceCandidateIndex_Type |  | index of the search space candidate to be used for UL grant |

NPdcchType1CommonSearchSpace_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NPdcchType1CommonSearchSpace_Type |  |  |
| Comment | parameters for Type1 common search space for paging (Type1CSS, TS 36.213 clause 16.6); In addition the SS needs to consider the "DCISubFrameRepetitionNumber" in the DCI DL configuration to determine the value of R (actual number of NPDCCH repetitions for the DCI) |  |  |
| NumRepetitions | NPDCCH_NumRepetitions_Type1CSS_Type |  | Rmax for Type1 common search space; TS 36.213 clause 16.6 |

NPdcchType2CommonSearchSpace_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NPdcchType2CommonSearchSpace_Type |  |  |
| Comment | parameters for Type2 common search space for RACH procedure (Type2CSS, TS 36.213 clause 16.6); In addition the SS needs to consider the "DCISubFrameRepetitionNumber" in the DCI DL configuration to determine the value of R (actual number of NPDCCH repetitions for the DCI); NOTE: NumRepetitions, StartSF and Offset are also included in each entry of the nprach-ParametersList of NPRACH_Config_Type (NB_PhysicalLayerConfigUL_Type); nevertheless NPRACH_Config_Type contains a list of NPRACH resources whereas the SS needs to consider only one search space corresponding to the NPRACH resource the UE shall select |  |  |
| NumRepetitions | NPDCCH_NumRepetitions_Type2CSS_Type |  | Rmax for Type2 common search space; TS 36.213 clause 16.6 |
| StartSF | NPDCCH_StartSF_Type2CSS_Type |  | G for Type2 common search space; TS 36.213 clause 16.6 |
| Offset | NPDCCH_Offset_Type2CSS_Type |  | offset for Type2 common search space; TS 36.213 clause 16.6 |
| SearchSpaceCandidateForRAR | NPdcchSearchSpaceCandidateIndex_Type |  | index of the search space candidate to be used for random access response |
| SearchSpaceCandidateForMsg4 | NPdcchSearchSpaceCandidateIndex_Type |  | index of the search space candidate to be used for Msg4 scheduling; this can be an DL transmission (contention resolution id based) or an UL transmission (C-RNTI based); in case of early contention resolution the SS shall use this index also for the subsequent DL transmission |

NPdcchConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NPdcchConfig_Type |  |  |
| Comment |  |  |  |
| UESS | NPdcchUESpecificSearchSpace_Type | opt | parameters for UE specific search space |
| Type1CSS | NPdcchType1CommonSearchSpace_Type | opt | parameters for Type1 common search space |
| Type2CSS | NPdcchType2CommonSearchSpace_Type | opt | parameters for Type2 common search space |
| RelativeTxPower | NB_ToRS_EPRE_Ratio_Type | opt | power ratio for NPDCCH's resource elements relative to the NRS |

NPdschRelativeTxPower_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NPdschRelativeTxPower_Type |  |  |
| Comment | FFS |  |  |
| RachResponse | NB_ToRS_EPRE_Ratio_Type | opt |  |
| BcchOnNPdsch | NB_ToRS_EPRE_Ratio_Type | opt |  |
| PcchOnNPdsch | NB_ToRS_EPRE_Ratio_Type | opt |  |
| CcchOnNPdsch | NB_ToRS_EPRE_Ratio_Type | opt |  |
| DcchDtchOnNPdsch | NB_ToRS_EPRE_Ratio_Type | opt |  |

NPdschConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NPdschConfig_Type |  |  |
| Comment |  |  |  |
| RelativeTxPower | NPdschRelativeTxPower_Type | opt |  |

#### F.1.3.2.2	Physical_Signals

NB_PrimarySyncSignal_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_PrimarySyncSignal_Type |  |  |
| Comment |  |  |  |
| RelativeTxPower | NB_ToRS_EPRE_Ratio_Type | opt | power ratio for NPSS's resource elements relative to the NRS (NOTE: applicable even though NRS is not in the same subframe) |

NB_SecondarySyncSignal_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_SecondarySyncSignal_Type |  |  |
| Comment |  |  |  |
| RelativeTxPower | NB_ToRS_EPRE_Ratio_Type | opt | power ratio for NPSS's resource elements relative to the NRS (NOTE: applicable even though NRS is not in the same subframe) |

NB_LTE_CellSpecificReferenceSignal_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_LTE_CellSpecificReferenceSignal_Type |  |  |
| Comment |  |  |  |
| RelativeTxPower | NB_ToRS_EPRE_Ratio_Type | opt | power ratio for the LTE CRS in case of In-band transmission with same PCI |

NWUS_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NWUS_Config_Type |  |  |
| Comment |  |  |  |
| TimeOffset | TimeOffsetInFrames |  | Number of frames used to calculate start of the first PO to which the NWUS is associated as per 36.211 cl 10.2.6B.1 and 36.304 cl 7.4 |
| WUS_Duration | WUS_Durations_Type |  | Actual duration of NWUS in subframes, corresponds to M in 36.211 cl. 10.2.6B.1 |
| RelativeTxPower | WUS_PowerBoost_Type | opt | The ratio of NWUS EPRE to NRS EPRE. If omit, default value of 0 dB shall be used |

NB_WUS_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_WUS_Config_Type |  |  |
| Comment |  |  |  |
| NWUS_Config | NWUS_Config_Type | opt |  |

### F.1.3.3	Uplink_Physical_Layer_Configuration

Uplink physical channel configuration: NPRACH, NPUSCH and UL RS

NB_SubCarrierSpacingUL_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NB_SubCarrierSpacingUL_Type |
| Comment |  |
| subCarrierSpacing_15kHz |  |
| subCarrierSpacing_3_75kHz |  |

NPUSCH_Configuration_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NPUSCH_Configuration_Type |  |  |
| Comment |  |  |  |
| Common | NPUSCH_ConfigCommon_Type | opt |  |
| Dedicated | NPUSCH_ConfigDedicated_Type | opt |  |

NB_PhysicalLayerConfigUL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_PhysicalLayerConfigUL_Type |  |  |
| Comment | NOTE: In general SS is required to keep the UE's UL power constant |  |  |
| NPrach | NPRACH_Config_Type | opt | parameters acc. TS 36.331, clause 6.7.3.2 |
| NPusch | NPUSCH_Configuration_Type | opt | parameters acc. TS 36.331, clause 6.7.3.2 (including configuration of RS) |
| TimingAdvance | SS_TimingAdvanceConfig_Type | opt | to adjust timing advance; at cell initialization absolute timing advance is configured as 0; absolute timing advance may be modified as part of the Random Access procedure configuration; in some MAC test cases timing advance may be configured to a non-zero (11 bit value) at the beginning and modified by (6 bit) timing advance commands during the test |
| SubCarrierSpacingUL | NB_SubCarrierSpacingUL_Type | opt | 15kHz or 3.75kHz UL carrier spacing; cyclic prefix as per 36.211 Table 10.1.5-1 accordingly |

### F.1.3.4	Common_MAC_Configuration

Transport channel and MAC related procedures and configuration

Common_MAC_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NPDSCH_ImcsValue_Type | integer | Modulation and coding scheme index coding; TS 36.213 Table 16.4.1.5.1-1 |

NB_PdcchDciFormat_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NB_PdcchDciFormat_Type |
| Comment |  |
| dci_N0 | physical layer parameters acc. TS 36.508 Table 8.1.3.6.1.1-1 (for NPUSCH) |
| dci_N1 | physical layer parameters acc. TS 36.508 Table 8.1.3.6.1.1-3 (for NPDSCH) |
| dci_N2 | physical layer parameters acc. TS 36.508 Table 8.1.3.6.1.1-3 (for Paging) |

NB_CRC_ErrorMode_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NB_CRC_ErrorMode_Type |
| Comment |  |
| noError | SS shall not generate any CRC error |
| crcErrorWithRetransmission | SS shall generate CRC error for DL transmissions and schedule retransmissions for each HARQ NACK |
| crcErrorWithoutRetransmission | SS shall generate CRC error for DL transmissions but not do any retransmissions |

NB_DciDlInfoCommon_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_DciDlInfoCommon_Type |  |  |
| Comment | Downlink control information according to 36.212 clause 6.4.3 |  |  |
| Format | NB_PdcchDciFormat_Type |  | NPDSCH: N1; Paging: N2 |
| RepetitionNumber | integer |  | Defined in TS 36.213, clause 16.4.1.3 |
| DCISubFrameRepetitionNumber | integer |  | Defined in TS 36.213, clause 16.6 |
| SchedulingDelay | integer | opt | Defined in TS 36.213, clause 16.4.1 Present for N1; not present for N2 (Paging) |
| HARQ_ACKResource | integer | opt | Defined in TS 36.213, clause 16.4.2 present for N1; not present for N2 (Paging) |
| CRC_ErrorMode | NB_CRC_ErrorMode_Type |  | no CRC error, CRC error with or without subsequent retransmission by the SS |
| HARQ_ProcessNumber | integer | opt | 1 bit to be added by the SS to the DCI in case of DCI format N1 being mapped onto the UE specific search space given by the C-RNTI (e.g. in case of RA_RNTI the SS shall skip this field even when it is present); if present, HARQ_ProcessNumber is 0 or 1 addressing the HARQ process to be used; see TS 36.212 clause 6.4.3.2 Note: When two HARQ processes are configured at the UE, this field is provided to the SS otherwise it is omit (-> PhysicalConfigDedicated_NB_r13.twoHARQ_ProcessesConfig_r14:=true) |

NB_DciDlInfoExplicit_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_DciDlInfoExplicit_Type |  |  |
| Comment | Downlink control information according to 36.212 clause 6.4.3 |  |  |
| Format | NB_PdcchDciFormat_Type |  | NPDSCH: N1; Paging: N2 |
| Imcs | NPDSCH_ImcsValue_Type |  | MCS index of TS 36.213 Table 16.4.1.5.1-1 |
| ResourceAssignment | integer |  | Defined in TS 36.213, clause 16.4.1.3: to determine the number of subframes |
| RepetitionNumber | integer |  | Defined in TS 36.213, clause 16.4.1.3 |
| DCISubFrameRepetitionNumber | integer |  | Defined in TS 36.213, clause 16.6 |
| SchedulingDelay | integer | opt | Defined in TS 36.213, clause 16.4.1 Present for N1; not present for N2 (Paging) |
| HARQ_ACKResource | integer | opt | Defined in clause 16.4.2 present for N1; not present for N2 (Paging) |
| CRC_ErrorMode | NB_CRC_ErrorMode_Type |  | no CRC error, CRC error with or without subsequent retransmission by the SS |
| HARQ_ProcessNumber | integer | opt | (see NB_DciDlInfoCommon_Type) |

NB_DciDlInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_DciDlInfo_Type |  |
| Comment |  |  |
| Auto | NB_DciDlInfoCommon_Type | SS shall choose the appropriate TBS |
| Explicit | NB_DciDlInfoExplicit_Type | used in MAC or RAB tests where exact TBS needs to be specified |

NB_AdaptiveHarqNack_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_AdaptiveHarqNack_Type |  |  |
| Comment |  |  |  |
| RedundancyVersion | integer |  | to be used in DCI for HARQ NACK to request UL restransmission |

NB_PurL1Response_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_PurL1Response_Type |  |  |
| Comment | Downlink control information according to 36.212 clause 6.4.3.1 |  |  |
| ACKorFallbackInd | B1_Type |  | Defined in TS 36.212, clause 6.4.3.1. |
| NPUSCHRepetitionAdjustment | integer |  | Defined in TS 36.213, Table 16.5.1.1-3. |
| TimingAdvance | TimingAdvanceIndex_Type | opt | Defined in TS 36.212, clause 6.4.3.1, present if ACKorFallbackInd is set to 0. |

NB_UL_TransRetransmission_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_UL_TransRetransmission_Type |  |
| Comment |  |  |
| NewTransmission | Null_Type | new transmission of data with redundancy version RV=0 (acc. to TS 36.321 clause 5.4.2.2); NDI is toggled |
| ReTransmissionAdaptive | NB_AdaptiveHarqNack_Type | DCI requesting retransmission (NDI not toggled) |

NB_UL_TransRetransmissionList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NB_UL_TransRetransmissionList_Type |
| Comment | to allow multiple retransmissions |
| record length (1..infinity) of NB_UL_TransRetransmission_Type |  |

NB_DciUlInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_DciUlInfo_Type |  |  |
| Comment | Downlink control information according to 36.212 clause 6.4.3 |  |  |
| SubCarrierIndication | integer |  | Defined in TS 36.213, clause 16.5.1.1 |
| ResourceAssignment | integer |  | Defined in TS 36.213, clause 16.5.1.2 |
| SchedulingDelay | integer |  | Defined in TS 36.213, clause 16.5.1 |
| Imcs | NPUSCH_ImcsValue_Type |  | Defined in TS 36.213, clause 16.5.1.2 |
| RedundancyVersion | integer |  | Defined in TS 36.213, clause 16.5.1.2 |
| RepetitionNumber | integer |  | Defined in TS 36.213, clause 16.5.1.1 |
| DCISubFrameRepetitionNumber | integer |  | Defined in TS 36.213, clause 16.6 |
| TransRetransmissionList | NB_UL_TransRetransmissionList_Type |  | for possible adaptive restransmissions: when there is more than one entry in the list the SS shall sent DCIs for the subsequent entries as ACK/NACK response according to TS 36.213 clause 16.5.2 |
| HARQ_ProcessNumber | integer | opt | 1 bit to be added by the SS to the DCI if the corresponding DCI format is mapped onto the UE specific search space given by the C-RNTI if present, HARQ_ProcessNumber is 0 or 1 addressing the HARQ process to be used; see TS 36.212 clause 6.4.3.1 Note: When two HARQ processes are configured at the UE, this field is provided to the SS otherwise it is omit (-> PhysicalConfigDedicated_NB_r13.twoHARQ_ProcessesConfig_r14:=true) |

NPUSCH_ImcsValue_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NPUSCH_ImcsValue_Type |  |
| Comment |  |  |
| None | Null_Type | 0 bit - not present if format N0 CRC is scrambled by PUR-RNTI |
| Value | integer | 4 bits - Modulation and coding scheme index coding; TS 36.213 Table 16.5.1.2-2 |

NB_PurResponse_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_PurResponse_Type |  |
| Comment |  |  |
| ULGrant | NB_DciUlInfo_Type | An UL grant configuration for the retransmission over PUR; when configured it means the Imcs in the DCI N0 scrambled with a PUR-RNTI is not present according to TS 36.212, clause 6.4.3.1. |
| L1Response | NB_PurL1Response_Type | When L1Response is configured in the DCI N0 scrambled by PUR-RNTI, SS shall set Imcs value to '1110'B as defined in TS 36.212, clause 6.4.3.1. |
| MacPdu | octetstring | Octet string containing a MAC PDU. It may accommodate either a Timing Advance MAC CE or the upper layers response to the message transmitted on PUR. The SS shall use DCI N1 scrambled with PUR-RNTI to schedule this transmission |
| None | Null_Type | If configured, the SS shall not send a response to the PUR transmission. |

NB_PeriodicGrant_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_PeriodicGrant_Type |  |  |
| Comment | configuration of UL grants to be assigned periodically acc. to configuration of the UE specific search space; DL assignments have higher priority than UL grant schedules: when there is a DL transmission to be scheduled for some search space the UL grant shall be scheduled in the next search space which is not used for DL assignment |  |  |
| DciInfo | NB_DciUlInfo_Type |  | DCI format: N0 (TS 36.212, clause 6.4.3.1) |
| NoOfRepetitions | TransmissionRepetition_Type |  | number of UL Grants to be automatically transmitted or continuous repetition |
| Periodicity | integer | opt | 1 => every search space, 2 => every 2nd search space, 3 => every 3rd search space, etc. may be omitted if only one UL grant shall be assigned and shall be ignored by the SS in this case; the periodicity is related to beginning of the UL grant scheduling, i.e. independent from whether or single grant is postponed due to pending DL assignment |

NB_UESS_GrantScheduling_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_UESS_GrantScheduling_Type |  |
| Comment | Configuration of (periodic) UL grants for the UE specific search space; NOTE: UL grants for Type2 common search space during RACH procedure are configured as part of the RACH procedure configuration |  |
| None | Null_Type | no UL grant configured => UE needs to trigger RACH procedure to get UL resources; to used also to stop periodic UL grant scheduling when the UE goes back to IDLE |
| Periodic | NB_PeriodicGrant_Type | configuration of one or several grants |
| PeriodicAfterRachContResolution | NB_PeriodicGrant_Type | The SS shall schedule UL grants periodically after sending the DCI for contention resolution: a) contention resolution id based contention resolution: DCI is DL assignment of Msg4 sent to the UE b) C-RNTI based contention resolution: DCI is grant for UL transmission The periodicity starts from the search space of DCI used for contention resolution NOTE: for now it is not foreseen that the SS needs to evaluate BSRs for (automatic) UL grant assignments; FFS |

NB_PUR_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_PUR_Config_Type |  |  |
| Comment | This type may be further extended with new releases |  |  |
| R16 | PUR_Config_NB_r16 |  | Rel-16 PUR parameters acc. to TS 36.331 cl.6.7.3.2 |
| PurResponseList | NB_PurResponseConfigList_Type |  | List of consecutive SS responses to corresponding messages received over PUR. |

NB_PurResponseConfigList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NB_PurResponseConfigList_Type |
| Comment | For each PUR-Config the SS shall be set up with a sequence of preconfigured responses along with their delays to PUR transmissions that are expected to be received. There is one-to-one mapping between the received PUR messages and corresponding responses. For the nth message received on PUR, the SS shall use the nth entry of the list to generate the preconfigured response. |
| record of NB_PurResponseConfig_Type |  |

NB_PurResponseConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_PurResponseConfig_Type |  |  |
| Comment | Configuration of a specific PUR response and transmission timing within the PUR response window |  |  |
| PurResponse | NB_PurResponse_Type |  | The type of the response to an UL reception received over the preconfigured uplink resource. |
| PurResponseOffset | integer |  | The number of subframes with respect to the subframe in which the reception on PUR took place, the SS shall wait before scheduling the corresponding response |

NB_Preconfigured_UL_ResourceConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_Preconfigured_UL_ResourceConfig_Type |  |
| Comment |  |  |
| None | Null_Type | to release preconfigured UL resource at SS |
| Config | NB_PUR_Config_Type | to set up a preconfigured UL resource at SS |

### F.1.3.5	Random_Access_Procedure

NB_RAR_UplinkGrant_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_RAR_UplinkGrant_Type |  |  |
| Comment | TS 36.213, clause 16.3.3 First bit for subcarrier spacing shall be set according to SubCarrierSpacingUL in NB_PhysicalLayerConfigUL_Type |  |  |
| SubCarrierIndication | B6_Type |  | Defined in TS 36.213 clause 16.5.1.1 |
| SchedulingDelay | B2_Type |  | Defined in TS 36.213 clause 16.5.1 where NB-IoT DL subframe n is the last subframe in which the NPDSCH associated with the NB random access response grant is transmitted |
| Msg3RepetitionNumber | B3_Type |  | Defined in TS 36.213 clause 16.5.1.1 |
| MCSIndex | B3_Type |  | Indicating TBS, modulation and number of RUs for Msg3 according to TS 36.213 table 16.3.3-1 |

NB_CRNTI_ContentionResolutionCtrl_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_CRNTI_ContentionResolutionCtrl_Type |  |
| Comment | configuration for Random Access Procedure in RRC_CONNECTED (see TS 36.300, clause 10.1.5.1); when SS receives C-RNTI MAC element sent by the UE after Random Access Response, SS shall deal with the C-RNTI as specified in this structure |  |
| AutomaticGrant | NB_DciUlInfo_Type | before expiry of the contention resolution timer SS shall automatically send an UL grant to the UE by addressing NPDCCH using C-RNTI as sent by the UE in Msg3 but scheduled according to Type2 common search space; the UL grant is explicitly specified by NB_DciUlInfo_Type |
| None | Null_Type | Used in case of dedicated preamble transmission (contention free random access) or to simulate failure cases for contention based random access procedure; SS shall not address PDCCH using C-RNTI => in case of contention based random access procedure: expiry of contention resolution timer on UE side |

NB_ContentionResolutionCtrl_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_ContentionResolutionCtrl_Type |  |
| Comment | NOTE: SS only needs to consider one kind of contention resolution at one time; in the initial configuration of a cell TCRNTI_Based shall be configured; whether or not a UE triggers RACH procedures in RRC_CONNECTED depends on RRC signalling (logicalChannelSR-Prohibit, logicalChannelSR-ProhibitTimer); when the UE is supposed to trigger RACH procedures in RRC_CONNECTED the SS needs to be configured for C-RNTI based contention resolution after the temporary C-RNTI based contention resolution has been done at RRC connection establishment |  |
| TCRNTI_Based | TCRNTI_ContentionResolutionCtrl_Type | TCRNTI based contention resolution (e.g. initial access), hence involves inclusion contention resolution identity in DL message 4 of RACH procedure; in case of early contention resolution the SS shall schedule the subsequent DL transmission in the Type2 common search space; NOTE: The subsequent DL transmission can be a CCCH message (e.g. RRC Connection Setup) but also a DCCH message (RRC Connection Resume) in which case according to the NOTE in TS 36.331 clause 5.3.3.3a the UE uses the common search space until successful connection resumption |
| CRNTI_Based | NB_CRNTI_ContentionResolutionCtrl_Type | CRNTI based contention resolution (e.g. in case UE is being in RRC_CONNECTED) or contention free random access: in case of CRNTI based contention resolution the uplink message in step 3 (of RACH procedure) is followed by PDCCH transmission with UE C-RNTI to end procedure |

NB_RandomAccessResponseParameters_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_RandomAccessResponseParameters_Type |  |  |
| Comment | paramenters to control content of RAR sent to the UE NOTE: in case SS detects a PRACH using preamble format 2 the SS shall use the MAC RAR as defined in TS 36.321 Figure 6.1.5-3c |  |  |
| RapId | RAR_RapIdCtrl_Type |  | to control Random Access Preamble Id to be sent back to the UE; used in RAR MAC sub-header |
| InitialGrant | NB_RAR_UplinkGrant_Type |  | initial UL grant |
| TimingAdvance | RACH_TimingAdvance_Type |  | timing advance: granularity of 0.52 micro sec (16*Ts); see TS 36.300, clause 5.2.7.3, TS 36.321, clause 6.1.3.5; NOTE: timing advance has impact not only on the RA procedure; SS in general needs to adjust its timing accordingly |
| TempC_RNTI | TempC_RNTI_Type |  | NOTE: For initial Random Access Procedure at network (SS) side there is no temporary C-RNTI: network assigns the C-RNTI which is used by any UE as being temporary; the UE which 'wins' the contention resolution keeps the (temporary) C-RNTI; other UEs need to repeat the RACH procedure; => at the SS the TempC_RNTI shall be 'SameAsC_RNTI' For Random Access Procedure in RRC_CONNECTED state the NW assigns a temporary C-RNTI which is replaced by the one stored at the UE; => TempC_RNTI may be 'SameAsC_RNTI' (in this case temp. C-RNTI and C-RNTI are equal what is not likely in a real network), or there is an explicit temp. C-RNTI what is used during RA procedure only (as in a real network) |

NB_RarList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NB_RarList_Type |
| Comment | in general MAC PDU may contain one or several RARs; normally only one RAR is contained |
| record of NB_RandomAccessResponseParameters_Type |  |

NB_RandomAccessResponse_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_RandomAccessResponse_Type |  |
| Comment |  |  |
| None | Null_Type | used for unsuccessful RA procedure |
| List | NB_RarList_Type | normally one RAR to be sent to the UE; in general there can be more than one RAR |

NB_RandomAccessResponseCtrl_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_RandomAccessResponseCtrl_Type |  |  |
| Comment | configuration for Random Access Response mapped to DL-SCH mapped to NPDSCH TransmissionMode: single antenna mode when there is only one antenna configured, transmit diversity else; RNTI: RA-RNTI (TS 36.321, clause 7.1); if both RAR msg and backoff indicator are 'None' SS shall not respond on random access preamble |  |  |
| DciInfo | NB_DciDlInfoCommon_Type |  | DCI format N1 addressed by RA-RNTI |
| Rar | NB_RandomAccessResponse_Type |  | RAR to be sent to the UE |
| BackoffInd | RandomAccessBackoffIndicator_Type |  | possible backoff indicator; 'None' for normal cases |

NB_RandomAccessResponseConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_RandomAccessResponseConfig_Type |  |
| Comment |  |  |
| Ctrl | NB_RandomAccessResponseCtrl_Type | contains information to control sending of RAR |
| None | Null_Type | to be used when there is no RAR to be sent at all |

NB_RachProcedure_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_RachProcedure_Type |  |  |
| Comment |  |  |  |
| RAResponse | NB_RandomAccessResponseConfig_Type |  | control of how the SS shall react on RA preamble; this may be - the RAP id as expected by the UE - a RAP id not matching to the UE's RAP - a backoff indicator - nothing at all |
| ContentionResolutionCtrl | NB_ContentionResolutionCtrl_Type |  |  |

NB_RachProcedureList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NB_RachProcedureList_Type |
| Comment | to simulate RACH procedure with one or more than one attempt by the UE: 1. Normal cases: one single RandomAccessResponse is sent to the UE matching the UE's RACH preamble; contention resolution is successful immediately => list contains only one element which is used for any RA procedure (Even if a RACH procedure is repeated by the UE for any reason this element shall be used; e.g. it needs not to be handled as error when the UE sends another RACH preamble instead of the RRC connection request message) 2. Special cases: SS shall start with the first element in the list and use the RAR as specified in this element; if the RAR matches at the UE side the UE will send UL data and contention resolution is performed as configured for this element; if the RAR does not match the UE sends another RAP and SS continues with the next element in the list; in this case the contention resolution of the respective element is not used; if the end of the list is reached and further RACH preambles are sent by the UE SS shall repeatively apply the last element of the list (this is necessary because there might be not enough time to reconfigure SS after the end of the list has been reached and there shall be well-defined behaviour after the list has been processed); to change from a special mode to normal mode the RachProcedureList is reconfigured by TTCN to achieve transparency and readability of the code; NOTE: for NB-IoT there can be more than one repetition per preamble attempt (numRepetitionsPerPreambleAttempt-r13 in NPRACH-ConfigSIB-NB-DEFAULT in SystemInformationBlockType2-NB); the SS gets configured with the nprach-ParametersList (NPRACH_Config_Type in NB_PhysicalLayerConfigUL_Type) and it is up to SS implementation how to get out how many preamble repetitions an attempt has; => NB_PhysicalLayerConfigUL_Type contains one entry per attempt and the SS shall report the number of preambles per attempt in a RachPreamble indication (if configured) |
| record of NB_RachProcedure_Type |  |

NB_RachProcedureConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_RachProcedureConfig_Type |  |  |
| Comment | parameters to control the random access procedure; TS 36.321, clause 5.1 |  |  |
| RACH_ConfigCommon | NB_RACH_ConfigCommon_Type | opt | acc. TS 36.331, clause 6.7.3.2; may not be necessary for SS; omit: "keep as it is" |
| RachProcedureList | NB_RachProcedureList_Type | opt | in normal cases there is one element which is used for any RA procedure; special cases are used in MAC test cases; omit means "keep as it is" |

NB_RA_NPDCCH_Order_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_RA_NPDCCH_Order_Type |  |  |
| Comment | TS 36.212, clause 6.4.3.2 |  |  |
| NprachRepetitions | integer |  | TS 36.213, Table 16.3.2-1: 0, 1, 3 |
| SubcarrierIndex | integer |  | ra-PreambleIndex (acc. to 36.321 cl. 5.1.2) as indicated in subcarrier indication field of the DCI (36.213 cl. 16.3.2 and 36.213 cl. 6.4.3.2): NOTE: the corresponding RAPID gets calculated according to 36.321 cl. 5.1.2 |

### F.1.3.6	System_Information_Control

Primitive to configuration BCCH/BCH

NB_SiSchedul_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_SiSchedul_Type |  |  |
| Comment | for specific SI scheduling and repetitions |  |  |
| Periodicity | NB_SiPeriodicity_Type | opt |  |
| RepetitionPattern | NB_SiRepetitionPattern_Type | opt | Indicates the starting radio frames within the SI window used for SI message transmission |
| SchedulingInfoSI | NB_SiTransportBlockSize_Type | opt | This field indicates the transport block size in number of bits used to broadcast the SI message |

NB_SiSchedulList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NB_SiSchedulList_Type |
| Comment |  |
| record length(1..maxSI_Message_NB_r13) of NB_SiSchedul_Type |  |

NB_AllSiSchedul_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_AllSiSchedul_Type |  |  |
| Comment | SI-message scheduling according to TS 36.331, clause 5.1.2.1a and clause 5.2.3a |  |  |
| WindowLength | NB_SiWindowLength_Type | opt | Common window size for all Sis to calculate start of each SI window acc. TS 36.331, clause 5.2.3a |
| RadioFrameOffset | integer | opt | Integer (0..15); Frame offset for scheduling of SI-messages (see TS 36.331, clause 5.2.3a) |
| SiList | NB_SiSchedulList_Type | opt | List of scheduling info for the SIs containing one or more SIBs |

SIScheduleConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SIScheduleConfig_Type |  |  |
| Comment | configuration for BCCH mapped to DL-SCH mapped to NPDSCH SIB1-NB: fixed scheduling in time domain according to TS 36.331 clause 5.2.1.2a (subframe 4, periodicity 2560ms, repetitions in every other frame in 160ms) |  |  |
| SchedulingInfoSIB1_NB | NB_SchedulingInfoSIB1_Type | opt | Integer (0..15) TBSize and Repetition number refer to TS 36.213 table 16.4.1.5.2-1 |
| SiSchedul | NB_AllSiSchedul_Type | opt | scheduling of SIs and TBSize in time domain |

NB_SI_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NB_SI_List_Type |
| Comment | TS 36.331, clause 6.7.1 BCCH-DL-SCH-Message-NB and clause 6.7.2 SystemInformation-NB |
| record of BCCH_DL_SCH_Message_NB |  |

NB_BcchInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_BcchInfo_Type |  |  |
| Comment | all fields are declared as optional to allow modification of single field; acc. to TS 36.331, clause 9.1.1.1 "RRC will perform padding, if required due to the granularity of the TF signalling, as defined in 8.5."; therefore this needs to be done by the system simulator |  |  |
| MIB_NB | BCCH_BCH_Message_NB | opt | TS 36.331, clause 6.7.1 BCCH-BCH-Message_NB and clause 6.7.2 MasterInformationBlock-NB; NOTE: the sequence numbers included in MIB-NB need to be handled and maintained by the system simulator; that means that the sequence numbers being setup by TTCN will be overwritten by SS |
| SIB1_NB | BCCH_DL_SCH_Message_NB | opt | TS 36.331, clause 6.7.1 BCCH-DL-SCH-Message-NB and clause 6.7.2 SystemInformationBlockType1-NB |
| SIs | NB_SI_List_Type | opt | list of SIs corresponding to SiList of AllSiSchedul_Type (i.e. element i of AllSiSchedul_Type's SiList specifies the scheduling for SIs[i]) |

NB_BcchConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_BcchConfig_Type |  |  |
| Comment | all fields are optional to allow single modifications; activation time may be applied in the common part of the ASP; NOTE 1: mapping/scheduling and contents of the System Information in general is done in one go (i.e. there are no separate ports for SIB data and configuration) |  |  |
| SIConfig | SIScheduleConfig_Type | opt |  |
| BcchInfo | NB_BcchInfo_Type | opt |  |

### F.1.3.7	Paging_Control

Primitive to configuration PCCH/PCH

NB_PcchConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_PcchConfig_Type |  |  |
| Comment | configuration for PCCH mapped to NPCH TransmissionMode: single antenna mode when there is only one antenna configured, transmit diversity else; RNTI: P-RNTI (TS 36.321, clause 7.1) NOTE: acc. to TS 36.331, clause 9.1.1.3 there is no PDCP and RLC/MAC are in TM |  |  |
| DciInfoPaging | NB_DciDlInfoCommon_Type | opt | DCI format: N2, Paging (TS 36.212, clause 6.4.3.3) |
| NoOfDciRepetitionsForDirectIndication | integer | opt | Absolute number of NPDCCH repetitions (R) for direct indication |

### F.1.3.8	NonAnchorCarrier_Common

configuration of non-anchor carrier for paging and RACH procedure according to rel-14 enhancements

NB_NonAnchorCarrier_SearchSpaceType1CSS_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_NonAnchorCarrier_SearchSpaceType1CSS_Type |  |
| Comment |  |  |
| SameAsAnchorCarrier | Null_Type | SS shall use the same Type1 common search space configuration as for the anchor carrier (-> NPdcchConfig_Type) |
| Explicit | NPdcchType1CommonSearchSpace_Type | to be used when test case requires different search space configuration for anchor and non-anchor configuration |

NB_NonAnchorCarrier_SearchSpaceType2CSS_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_NonAnchorCarrier_SearchSpaceType2CSS_Type |  |
| Comment |  |  |
| SameAsAnchorCarrier | Null_Type | SS shall use the same Type2 common search space configuration as for the anchor carrier (-> NPdcchConfig_Type)) |
| Explicit | NPdcchType2CommonSearchSpace_Type | to be used when test case requires different search space configuration for anchor and non-anchor configuration |

NB_NonAnchorCarrierCommonDL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_NonAnchorCarrierCommonDL_Type |  |  |
| Comment |  |  |  |
| Id | integer |  | identifier to address the carrier to be used for paging (-> NB_PagingTrigger_Type) |
| ConfigCommon | NB_DL_ConfigCommon_NB_Type | opt | ASN.1 configuration as provided to the UE |
| Type1CSS | NB_NonAnchorCarrier_SearchSpaceType1CSS_Type | opt | Type1CSS search space configuration to be applied by the SS for the given non-anchor carrier |

NB_NonAnchorCarrierCommonDL_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NB_NonAnchorCarrierCommonDL_List_Type |
| Comment |  |
| record of NB_NonAnchorCarrierCommonDL_Type |  |

NB_CarrierConfigCommonDL_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_CarrierConfigCommonDL_Type |  |
| Comment |  |  |
| Config | NB_NonAnchorCarrierCommonDL_List_Type |  |
| None | Null_Type |  |

NB_NpdcchCarrier_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_NpdcchCarrier_Type |  |
| Comment | to specify the DL carrier corresponding to the UL carrier on which the RACH preamble has been received |  |
| AnchorCarrier | Null_Type | NPDCCCH on anchor carrier |
| Id | integer | identifier of a DL non-anchor carrier carrying the NPDCCH |

NB_NonAnchorCarrierCommonUL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_NonAnchorCarrierCommonUL_Type |  |  |
| Comment |  |  |  |
| Id | integer |  | identifier to indicate the carrier on which the UE has sent a RACH preamble (-> NB_RachPreamble_Type) |
| ConfigCommon | NB_UL_ConfigCommon_NB_Type | opt | ASN.1 configuration as provided to the UE |
| Type2CSS | NB_NonAnchorCarrier_SearchSpaceType2CSS_Type | opt | Type2CSS search space configuration to be applied by the SS for the given non-anchor carrier |
| NpdcchCarrierId | NB_NpdcchCarrier_Type | opt | identifier of the corresponding DL carrier; NOTE 1: the DL carrier shall be the same as being addressed by npdcch-CarrierIndex in SIB22 NOTE 2: in general the number and order of carriers being configured at the SS and being contained in SIB22 does not need to be the same |

NB_NonAnchorCarrierCommonUL_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NB_NonAnchorCarrierCommonUL_List_Type |
| Comment |  |
| record of NB_NonAnchorCarrierCommonUL_Type |  |

NB_CarrierConfigCommonUL_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_CarrierConfigCommonUL_Type |  |
| Comment |  |  |
| Config | NB_NonAnchorCarrierCommonUL_List_Type |  |
| None | Null_Type |  |

### F.1.3.9	UE_Specific_Channel_Configuration

NB_DrxCtrl_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_DrxCtrl_Type |  |
| Comment | DRX configuration for connected mode (TS 36.321, clause 5.7) |  |
| None | Null_Type | DRX not configured |
| Config | NB_DRX_Config_Type | DRX is configured as signalled to the UE; NOTE: the release branch of DRX-Config in general is not used for configuration of the SS |

NB_CcchDcchDtchConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_CcchDcchDtchConfig_Type |  |  |
| Comment |  |  |  |
| DL | NB_CcchDcchDtchConfigDL_Type | opt | Scheduling, parameters related to CCCH, DCCH and DTCH in DL |
| UL | NB_CcchDcchDtchConfigUL_Type | opt | Scheduling, parameters related to CCCH, DCCH and DTCH in UL |
| DrxCtrl | NB_DrxCtrl_Type | opt | DRX configuration as sent to the UE (or 'None' when the UE does not support connected mode DRX) |

#### F.1.3.9.1	UE_Specific_Channel_Configuration_DL

Scheduling and other information for CCCH/DCCH/DTCH mapped to DL-SCH mapped to NPDSCH

NB_DownlinkGapConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_DownlinkGapConfig_Type |  |
| Comment | to specify DL gaps for NPDSCH according to TS 36.211 clause 10.2.3.4 |  |
| DownlinkGapConfig | DL_GapConfig_Type | DL gaps shall be used according to the DL gap configuration (TS 36.331 clause 6.7.3.2) |
| None | Null_Type | there are no gaps in DL |

NB_CcchDcchDtchConfigDL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_CcchDcchDtchConfigDL_Type |  |  |
| Comment | configuration for CCCH/DCCH/DTCH mapped to DL-SCH mapped to NPDSCH RNTI: C-RNTI (TS 36.321, clause 7.1); all fields optional (omit = "keep as it is") since DCI format and modulation may be changed during a test; for initial configuration all fields are mandatory |  |  |
| DciInfo | NB_DciDlInfo_Type | opt |  |
| GapConfig | NB_DownlinkGapConfig_Type | opt | to specify the DL gap configuration for NPDSCH |

#### F.1.3.9.2	UE_Specific_Channel_Configuration_UL

Scheduling information for CCCH/DCCH/DTCH mapped to UL-SCH mapped to NPUSCH

NB_CcchDcchDtchConfigUL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_CcchDcchDtchConfigUL_Type |  |  |
| Comment | scheduling for CCCH/DCCH/DTCH mapped to UL-SCH mapped to NPUSCH NOTE 1: for definition of the possible UL grants the location of the NPUSCH (TS 36.211, clause 10.1.3) and the NPRACH (TS 36.211, clause 10.1.6) need to be taken into account; NOTE 2: In contrast to the DL where the scheduling can be done (with consideration of some restrictions) by SS on a per need basis in the UL the scheduling depends on information provided by the UE: e.g. BSR (buffer status report) see TS 36.523-3 clause 7.2 for further information. |  |  |
| UplinkTimeAlignment_Synch | UplinkTimeAlignment_Synch_Type | opt | parameters to control automatic control of timing advance |
| UESS_GrantScheduling | NB_UESS_GrantScheduling_Type | opt | UL grant allocation to be applied |

### F.1.3.10	NB_NTN_Configuration

NB_NTN_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NB_NTN_K_Offset_Type | integer (0..1023) |  |
| NB_NTN_K_Mac_Type | integer (0..512) |  |

NB_NTN_TimingOffset_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_NTN_TimingOffset_Type |  |  |
| Comment |  |  |  |
| Koffset | NB_NTN_K_Offset_Type | opt | Koffset in ms as defined in TS 36.213 clause 16. If omit, keep as it is. |
| Kmac | NB_NTN_K_Mac_Type | opt | Kmac in ms as defined in TS 36.213 clause 16.6. If omit, keep as it is. |

NB_NTN_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_NTN_Config_Type |  |  |
| Comment |  |  |  |
| NtnTimingOffset | NB_NTN_TimingOffset_Type | opt | to configure PHY/MAC layers with NTN parameters Koffset and Kmac needed for NTN operation. See clause 7A.14.2.3 for further information. |
| NtnFrequencyDopplerShift | integer | opt | to configure service link with a constant frequency Doppler shift. The unit is ppm (parts per million), actual value = field value*10^-5. See clause 7A.14.2.3 for further information. |

## F.1.4	Cell_Power_Attenuation

NB_CellAttenuationConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_CellAttenuationConfig_Type |  |  |
| Comment |  |  |  |
| CellId | NBIOT_CellId_Type |  |  |
| Attenuation | Attenuation_Type |  |  |
| TimingInfo | TimingInfo_Type | opt |  |

NB_CellAttenuationList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NB_CellAttenuationList_Type |
| Comment |  |
| record of NB_CellAttenuationConfig_Type |  |

## F.1.5	Radio_Bearer_Configuration

Radio Bearer Configuration: SRBs/DRBs

NB_PDCP_TestModeConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_PDCP_TestModeConfig_Type |  |
| Comment |  |  |
| None | Null_Type |  |
| Transparent | Null_Type | requires PDCP to be configured as transparent => - TTCN uses AS ciphering in both directions, employing the dummy ciphering algorithm - SS does not interpret, insert or remove PDCP headers - SS does not maintain PDCP sequence numbers and state variables |

NB_PDCP_RbConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_PDCP_RbConfig_Type |  |
| Comment |  |  |
| Srb | Null_Type |  |
| Drb | NB_PDCP_Config_Type | PDCP-Configuration acc. to TS 36.331, clause 6.7.3.2 |

NB_PDCP_ConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_PDCP_ConfigInfo_Type |  |  |
| Comment |  |  |  |
| Rb | NB_PDCP_RbConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is" |
| TestMode | NB_PDCP_TestModeConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is" |

NB_PDCP_Configuration_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_PDCP_Configuration_Type |  |
| Comment |  |  |
| None | Null_Type | for SRB0 & SRB1bis no PDCP is configured; |
| Config | NB_PDCP_ConfigInfo_Type |  |

NB_RadioBearerConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_RadioBearerConfigInfo_Type |  |  |
| Comment | semantics of omit: "keep as it is" |  |  |
| Pdcp | NB_PDCP_Configuration_Type | opt | for SRB0 & SRB1bis: "Pdcp.None:=true" mandatory for initial configuration; omit means "keep as it is" |
| Rlc | NB_RLC_Configuration_Type | opt | mandatory for initial configuration; omit means "keep as it is" |
| LogicalChannelId | LogicalChannelId_Type | opt | DRBs: DTCH-LogicalChannelIdentity as for rb-MappingInfo in DRB-ToAddModifyList (according to 36.331 a value of 3 shall not be used in DRB-ToAddModifyList); SRBs: for SRBs specified configurations acc. to TS 36.331, clause 9.1.2 shall be applied: SRB1: ul-LogicalChannel-Identity = dl-LogicalChannel-Identity = 1 SRB1bis: ul-LogicalChannel-Identity = dl-LogicalChannel-Identity = 3 for SRB0 being mapped to CCCH the LCID is '00000'B acc. to TS 36.321, clause 6.2.1; mandatory for initial configuration; omit means "keep as it is" |
| Mac | NB_MAC_Configuration_Type | opt |  |

NB_RadioBearerConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_RadioBearerConfig_Type |  |
| Comment |  |  |
| AddOrReconfigure | NB_RadioBearerConfigInfo_Type | add / re-configure RB - CellId : identifier of the cell being configured RoutingInfo : None TimingInfo : 'Now' in common cases ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |
| Release | Null_Type | release RB - CellId : identifier of the cell being configured RoutingInfo : None TimingInfo : 'Now' in common cases ControlInfo : CnfFlag:=true; FollowOnFlag:=false (in general) |

NB_RadioBearer_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_RadioBearer_Type |  |  |
| Comment |  |  |  |
| Id | NB_RadioBearerId_Type |  | either for SRB or DRB |
| L2TestMode | IndicationAndControlMode_Type | opt | to enable/disable L2 testmode: omit : "keep as it is"; "omit" in initial configuration means that L2 testmode is disabled enable : SS shall route uplink data to N_L2DATA port instead of N_SRB port disable : SS shall route uplink data to N_SRB port as usual |
| Config | NB_RadioBearerConfig_Type |  |  |

NB_RadioBearerList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NB_RadioBearerList_Type |
| Comment | array of SRBs and/or DRBs |
| record length (1..tsc_NB_MaxRB) of NB_RadioBearer_Type |  |

### F.1.5.1	RLC_Configuration

RLC configuration: radio bearer specific

RLC_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NB_SS_RLC_TM_Type | Null_Type | TM to configure SRB0; no parameters to be defined |

NB_RLC_TestModeInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_RLC_TestModeInfo_Type |  |
| Comment |  |  |
| TransparentMode_UMDwith5BitSN | Null_Type | shall be set when TTCN expects RLC PDUs as UMD in UL with an SN of 5 bits; valid only when the RLC is configured in TM |

NB_RLC_TestModeConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_RLC_TestModeConfig_Type |  |
| Comment |  |  |
| None | Null_Type |  |
| Info | NB_RLC_TestModeInfo_Type |  |

NB_SS_RLC_AM_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_SS_RLC_AM_Type |  |  |
| Comment |  |  |  |
| Tx | NB_UL_AM_RLC_Type | opt | the UE's UL setting to be used in SS's tx direction |
| Rx | NB_DL_AM_RLC_Type | opt | the UE's DL setting to be used in SS's rx direction |

NB_RLC_RbConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_RLC_RbConfig_Type |  |
| Comment |  |  |
| AM | NB_SS_RLC_AM_Type |  |
| TM | NB_SS_RLC_TM_Type | normally SRB0 only; may be used for test purposes also |

NB_RLC_Configuration_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_RLC_Configuration_Type |  |  |
| Comment |  |  |  |
| Rb | NB_RLC_RbConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is" |
| TestMode | NB_RLC_TestModeConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is" |

### F.1.5.2	MAC_Configuration

MAC configuration: radio bearer specific configuration

NB_MAC_TestModeConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_MAC_TestModeConfig_Type |  |
| Comment |  |  |
| None | Null_Type |  |
| Transparent | Null_Type | In UL and DL the SS' MAC layer is transparent i.e. SS does not add or remove any MAC header |

NB_MAC_LogicalChannelConfig_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_MAC_LogicalChannelConfig_Type |  |  |
| Comment |  |  |  |
| Priority | integer |  | logical channel priority for the DL as described in TS 36.321, clause 5.4.3.1 for the UL |

NB_MAC_Configuration_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_MAC_Configuration_Type |  |  |
| Comment |  |  |  |
| LogicalChannel | NB_MAC_LogicalChannelConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is" |
| TestMode | NB_MAC_TestModeConfig_Type | opt | mandatory for initial configuration; omit means "keep as it is"; for none MAC tests "TestMode.None:=true" |

## F.1.6	AS_Security

Primitive for control of AS security

NB_PdcpSQN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_PdcpSQN_Type |  |  |
| Comment |  |  |  |
| Format | NB_PdcpCountFormat_Type |  | 5 bit, 7 bit or 12 bit SQN |
| Value | integer |  | SQN value (5 bit, 7 bit or 12 bit SQN) NOTE: in TTCN the test case writer is responsible to deal with potential overflows (e.g. there shall be a "mod 32", "mod 128" or "mod 4096" according to the format) |

NB_PDCP_ActTime_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_PDCP_ActTime_Type |  |
| Comment | The sequence number in UL and DL for SRB1 should be one more than the present SQN, as Ciphering starts in UL and DL soon after SMC and SMComp; For DRBs it should be the present SQN |  |
| None | Null_Type | No Activation time; to be used if Ciphering is not applied |
| SQN | NB_PdcpSQN_Type | PDCP sequence number |

NB_SecurityActTime_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_SecurityActTime_Type |  |  |
| Comment |  |  |  |
| RadioBearerId | NB_RadioBearerId_Type |  |  |
| UL | NB_PDCP_ActTime_Type |  |  |
| DL | NB_PDCP_ActTime_Type |  |  |

NB_SecurityActTimeList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NB_SecurityActTimeList_Type |
| Comment |  |
| record length (1..tsc_NB_MaxRB) of NB_SecurityActTime_Type |  |

NB_AS_IntegrityInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_AS_IntegrityInfo_Type |  |  |
| Comment | for initial configuration activation time is not needed for integrity protection as all messages in DL after security activation are integrity protected; this means this ASP is invoked before transmission of Security mode command; if there is a integrity violation in UL SS shall set the IndicationStatus in the common ASP part to flag the integrity error (IndicationStatus.Error.Integrity.Pdcp := true); integrity to be provided for each SRB as per core spec |  |  |
| Algorithm | IntegrityProtAlgorithm_Type |  | IntegrityProtAlgorithm_Type being defined in RRC ASN.1 |
| KRRCint | B128_Key_Type |  |  |
| ActTimeList | NB_SecurityActTimeList_Type | opt | omit for initial configuration (i.e. all SRBs to be integrity protected immediately) |

NB_AS_CipheringInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_AS_CipheringInfo_Type |  |  |
| Comment |  |  |  |
| Algorithm | EUTRA_ASN1_CipheringAlgorithm_r12_Type |  | CipheringAlgorithm_Type being defined in RRC ASN.1 |
| KRRCenc | B128_Key_Type |  |  |
| KUPenc | B128_Key_Type |  | KUPenc is used by SS when DRBs are configured |
| ActTimeList | NB_SecurityActTimeList_Type |  |  |

NB_AS_SecStartRestart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_AS_SecStartRestart_Type |  |  |
| Comment |  |  |  |
| Integrity | NB_AS_IntegrityInfo_Type | opt | optional to allow separated activation of integrity and ciphering; omit: keep as it is |
| Ciphering | NB_AS_CipheringInfo_Type | opt | optional to allow separated activation of integrity and ciphering; omit: keep as it is |

NB_AS_Security_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_AS_Security_Type |  |
| Comment | Security mode command procedure (TS 36.331, clause 5.3.4): both SMC and SMComp are integrity protected (nevertheless SS shall be able to cope with unprotected SM reject); ciphering is started just after SMComp (acc. to TS 36.331, clause 5.3.4.3 and 5.3.1.1) |  |
| StartRestart | NB_AS_SecStartRestart_Type | information to start/restart AS security protection in the PDCP |
| Release | Null_Type | to release AS security protection in the PDCP |

## F.1.7	Paging_Trigger

NB_PagingMessage_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_PagingMessage_Type |  |
| Comment |  |  |
| Paging | PCCH_Message_NB |  |
| DirectIndication | B8_Type |  |

NB_PagingTrigger_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_PagingTrigger_Type |  |  |
| Comment | CellId : identifier of the cell where the UE is active RoutingInfo : None TimingInfo : Calculated paging occasion ControlInfo : CnfFlag:=false; FollowOnFlag:=false primitive to trigger transmission of a paging on the NPCCH at a calculated paging occasion (TS 36.304, clause 7); the paging occasion is calculated by TTCN and activation time is applied; as for BCCH Info acc. to TS 36.331, clause 9.1.1.3 "RRC will perform padding, if required due to the granularity of the TF signalling, as defined in 8.5."; therefore this needs to be done by the system simulator |  |  |
| Paging | NB_PagingMessage_Type |  | paging to be scheduled in the Type1 common search space beginning from the time given in the timing information of the common part of the ASP; the DCI is transmitted on the NPDCCH using P-RNTI |
| PagingCarrierId | integer | opt | When the UE supports paging on a non-anchor carrier and paging configuration for non-anchor carrier is provided in system information: Id of the DL non-anchor carrier (-> NB_NonAnchorCarrierCommonDL_Type.Id) on which the SS shall do the paging; the SS configuration of the carrier with the given Id corresponds to the entry in dl-ConfigList-r14 of SIB22 which has the same carrier frequency; NOTE: it is up to TTCN implementation to determine the carrier which fulfils the equation given in TS 36.304 clause 7.1 to determine the paging carrier; omit when paging happens on anchor-carrier |

## F.1.8	RLC_Counts

Primitives to enquire RLC Counts AM_VTS and AM_VRR

NB_RLC_CountsInfoList_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_RLC_CountsInfoList_Type |  |  |
| Comment |  |  |  |
| AM_VTS | integer |  | sequnce number for DL |
| AM_VRR | integer |  | sequnce number for UL |

NB_RLC_CountsReq_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_RLC_CountsReq_Type |  |
| Comment |  |  |
| Get | Null_Type | Request RLC Counts |
| Set | NB_RLC_CountsInfoList_Type | Set RLC Counts |

NB_RLC_CountsCnf_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_RLC_CountsCnf_Type |  |
| Comment |  |  |
| Get | NB_RLC_CountsInfoList_Type | VTS and VRR |
| Set | Null_Type |  |

## F.1.9	PDCP_Count

Primitives to enquire PDCP COUNT

NB_PdcpCountFormat_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NB_PdcpCountFormat_Type |
| Comment |  |
| PdcpCount_Srb | 27 bit HFN; 5 bit SQN |
| PdcpCount_DrbShortSQN | 25 bit HFN; 7 bit SQN |

NB_PdcpCount_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_PdcpCount_Type |  |  |
| Comment |  |  |  |
| Format | NB_PdcpCountFormat_Type |  |  |
| Value | PdcpCountValue_Type |  |  |

NB_PdcpCountInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_PdcpCountInfo_Type |  |  |
| Comment | - at the start of the PDCP entity Next_PDCP_RX_SN resp. Next_PDCP_TX_SN are set to 0 - when receiving or sending a PDCP PDU the COUNT associated is established from the current values of Next_PDCP_RX_SN resp. Next_PDCP_TX_SN - Next_PDCP_RX_SN resp. Next_PDCP_TX_SN are incremented afterwards |  |  |
| RadioBearerId | NB_RadioBearerId_Type |  |  |
| UL | NB_PdcpCount_Type | opt | omit: keep as it is |
| DL | NB_PdcpCount_Type | opt | omit: keep as it is |

NB_PdcpCountInfoList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NB_PdcpCountInfoList_Type |
| Comment |  |
| record length (1..tsc_NB_MaxRB) of NB_PdcpCountInfo_Type |  |

NB_PdcpCountGetReq_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_PdcpCountGetReq_Type |  |
| Comment |  |  |
| AllRBs | Null_Type | return COUNT values for all RBs being configured |
| SingleRB | NB_RadioBearerId_Type |  |

NB_PDCP_CountReq_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_PDCP_CountReq_Type |  |
| Comment |  |  |
| Get | NB_PdcpCountGetReq_Type | Request PDCP count for one or all RBs being configured at the PDCP |
| Set | NB_PdcpCountInfoList_Type | Set PDCP count for one or all RBs being configured at the PDCP; list for RBs which's COUNT shall be manipulated |

NB_PDCP_CountCnf_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_PDCP_CountCnf_Type |  |
| Comment |  |  |
| Get | NB_PdcpCountInfoList_Type | RBs in ascending order; SRBs first |
| Set | Null_Type |  |

## F.1.10	L1_MAC_Test_Mode

Primitive for control of L1/MAC Test Modes

NB_L1Mac_IndicationControl_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_L1Mac_IndicationControl_Type |  |  |
| Comment |  |  |  |
| RachPreamble | IndicationAndControlMode_Type | opt | To enable/disable reporting of NPRACH preamble received |
| DPR | IndicationAndControlMode_Type | opt | To enable/disable reporting of MAC DPR control elements |
| HarqError | IndicationAndControlMode_Type | opt | To enable/disable reporting of HARQ errors |
| UL_HARQ | IndicationAndControlMode_Type | opt | To enable/disable reporting of HARQ ACK/NACK |
| TimingAdvanceReport | IndicationAndControlMode_Type | opt | To enable/disable reporting of timing advance |
| PurReception | IndicationAndControlMode_Type | opt | To enable/disable reporting of a reception on a preconfigured uplink resource |

## F.1.11	PTM_Configuration

configuration of Point To Multipoint (PTM) according to rel-14 enhancements

NBIOT_ASP_TypeDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_MaxSCMTCH_NB | integer | 64 | Maximum number of SC-MTCHs in one cell for NB-IoT |

PTM_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NPdcchType1ACommonSearchSpace_Type | NPDCCH_SC_MCCH_Config_NB_r14 | parameters for Type1A common search space for SC-MCCH (Type1ACSS, TS 36.213 clause 16.6) |

NPdcchType2ACommonSearchSpace_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NPdcchType2ACommonSearchSpace_Type |  |  |
| Comment | parameters for Type2A common search space for SC-MTCH (Type2ACSS, TS 36.213 clause 16.6); |  |  |
| NumRepetitions | NPDCCH_NumRepetitions_SC_MTCH_Type |  | Rmax for Type2A common search space; TS 36.213 clause 16.6 |
| StartSF | NPDCCH_StartSF_SC_MTCH_Type |  | G for Type2A common search space; TS 36.213 clause 16.6 |
| Offset | NPDCCH_Offset_SC_MTCH_Type |  | offset for Type2A common search space; TS 36.213 clause 16.6 |
| Npdsch_MaxTBS | NPDCCH_NPDSCH_MaxTBS_SC_MTCH_Type |  | Maximum NPDSCH TBS for the SC-MTCH |

NB_SC_MTCH_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_SC_MTCH_Config_Type |  |  |
| Comment |  |  |  |
| GRnti | B16_Type |  |  |
| ScMtchCarrierConfig | NB_SC_MTCH_CarrierConfig_Type |  |  |
| ScMtchSchedulingInfo | SC_MTCH_SchedulingInfo_NB_r14 | opt |  |

NB_SC_MTCH_Info_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_SC_MTCH_Info_Type |  |  |
| Comment |  |  |  |
| Type2ACSS | NPdcchType2ACommonSearchSpace_Type |  | parameters for Type2A common search space |
| ScMrbId | SC_MRB_Identity_Type |  |  |
| Config | NB_SC_MTCH_Config_Type |  |  |

NB_SC_MTCH_InfoList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | NB_SC_MTCH_InfoList_Type |
| Comment |  |
| record length(1.. tsc_MaxSCMTCH_NB) of NB_SC_MTCH_Info_Type |  |

NB_SC_MCCH_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_SC_MCCH_Config_Type |  |  |
| Comment |  |  |  |
| Type1ACSS | NPdcchType1ACommonSearchSpace_Type |  | parameters for Type1A common search space |
| ScMcchCarrierConfig | NB_SC_MCCH_CarrierConfig_Type |  |  |
| ScMcchRepetitionPeriod | NB_SC_MCCH_RepetitionPeriod_Type |  |  |
| ScMcchOffset | NB_SC_MCCH_Offset_Type |  |  |
| ScMcchModificationPeriod | NB_SC_MCCH_ModificationPeriod_Type |  |  |
| ScMcchSchedulingInfo | NB_SC_MCCH_SchedulingInfo_Type | opt |  |

NB_SCPTM_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_SCPTM_Config_Type |  |  |
| Comment | all fields are optional to allow single modifications; in case of reconfiguration OMIT means 'keep configuration as it is' |  |  |
| ScMcchConfig | NB_SC_MCCH_Config_Type | opt | SC-MCCH scheduling configuration on NPDCCH as per SystemInformationBlockType20-NB acc. to TS 36.331 cl 9.1.1.7 there is no PDCP and SC-MCCH uses the RLC-UM mode configuration/scheduling and contents of the SC-MCCH Information is done in one go (i.e. there are no separate ports for SC-MCCH data and configuration) |
| ScptmConfiguration | SCPTMConfiguration_NB_r14 | opt | Message containing SC-MTCH configuration to be broadcasted on SC-MCCH |
| ScMtchInfoList | NB_SC_MTCH_InfoList_Type | opt | Configure/release SC-MTCH SC-MRB |

NB_PTM_Config_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_PTM_Config_Type |  |  |
| Comment |  |  |  |
| ScptmConfig | NB_SCPTM_Config_Type |  | PTM services are provided via SC-PTM |

## F.1.12	System_Interface

NB_SYSTEM_CTRL_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_SYSTEM_CTRL_REQ |  |  |
| Comment |  |  |  |
| Common | NB_ReqAspCommonPart_Type |  | TimingInfo depends on respective primitive: |
| Request | NB_SystemRequest_Type |  | - Cell TimingInfo: 'now' (in general) - CellAttenuationList TimingInfo: 'now' (in general, but activation time may be used also) - RadioBearerList TimingInfo: 'now' in general; activation time may be used in special case for release and/or reconfiguration of one or several RBs; |

NB_SYSTEM_CTRL_CNF

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_SYSTEM_CTRL_CNF |  |  |
| Comment |  |  |  |
| Common | NB_CnfAspCommonPart_Type |  | TimingInfo is ignored by TTCN (apart from EnquireTiming) => SS may set TimingInfo to "None" |
| Confirm | NB_SystemConfirm_Type |  |  |

NB_SYSTEM_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_SYSTEM_IND |  |  |
| Comment |  |  |  |
| Common | NB_IndAspCommonPart_Type |  | The SS shall provide TimingInfo (HSFN + SFN + subframe number) depending on the respective indication: |
| Indication | NB_SystemIndication_Type |  |  |

NBIOT_SYSTEM_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NBIOT_SYSTEM_PORT |  |
| Comment | NBIOT PTC: Port for system configuration |  |
| out | NB_SYSTEM_CTRL_REQ |  |
| in | NB_SYSTEM_CTRL_CNF |  |

NBIOT_SYSIND_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NBIOT_SYSIND_PORT |  |
| Comment | NBIOT PTC: Port for system indications |  |
| in | NB_SYSTEM_IND |  |

# F.2	NBIOT_ASP_SrbDefs

## F.2.1	SRB_DATA_ASPs

ASP Definitions to send/receive peer-to-peer messages on SRBs

NB_C_Plane_Request_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_C_Plane_Request_Type |  |  |
| Comment | RRC and/or NAS PDU to be send to the UE; Note: it may be necessary to allow more than one NAS PDU (-> "record of") |  |  |
| Rrc | NB_RRC_MSG_Request_Type | opt | omit: NAS message shall be present; NAS message shall be sent in DLInformationTransfer present: if NAS message is present also, (piggybacked) NAS PDU shall be security protected (if necessary) and inserted in RRC PDU's DedicatedInfoNAS |
| Nas | NAS_MSG_RequestList_Type | opt | omit: RRC message shall be present; RRC message does not contain (piggybacked) NAS PDU present: if RRC message is omitted => NAS message shall be sent embedded in DLInformationTransfer if RRC message is present => NAS message is piggybacked in RRC message in case of RRC message is sent on CCCH, NAS message shall be omitted |

NB_C_Plane_Indication_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_C_Plane_Indication_Type |  |  |
| Comment | RRC and/or NAS PDU to be received from the UE; Note: it may be necessary to allow more than one NAS PDU (-> "record of") |  |  |
| Rrc | NB_RRC_MSG_Indication_Type | opt | omit: NAS message shall be present; NAS message is received in ULInformationTransfer present: if NAS message is present also, DedicatedInfoNAS contains unstructured and ciphered NAS message and the NAS message is the deciphered message in structured format |
| Nas | NAS_MSG_IndicationList_Type | opt | omit: RRC message shall be present; RRC message does not contain (piggybacked) NAS PDU present: if RRC message is omitted => NAS message has been received in ULInformationTransfer if RRC message is present => NAS message has been piggybacked in RRC message |

NB_SRB_COMMON_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_SRB_COMMON_REQ |  |  |
| Comment | common ASP to send PDUs to SRB0, SRB1 or SRB1bis |  |  |
| Common | NB_ReqAspCommonPart_Type |  | CellId identifier of the cell RoutingInfo SRB0, SRB1, SRB1bis TimingInfo Now in normal cases; For latency tests TimingInfo can be set to the SFN/subframe in which the RRC messages shall be sent out (in this case and if the RRC PDU is too long to be sent in one TTI the TimingInfo corresponds to the first TTI) ControlInfo CnfFlag:=false; FollowOnFlag true: Indicates that the message(s) to be sent on the same TTI will follow NOTE 1: When FollowOnFlag is true, TimingInfo shall always be "Now". Otherwise SS shall produce an error NOTE 2: the follow on flag applies only for messages of the same SRB false: Indicates that no more message(s) will follow |
| Signalling | NB_C_Plane_Request_Type |  |  |

NB_SRB_COMMON_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_SRB_COMMON_IND |  |  |
| Comment | common ASP to receive PDUs from SRB0, SRB1 or SRB1bis |  |  |
| Common | NB_IndAspCommonPart_Type |  | CellId identifier of the cell RoutingInfo SRB0, SRB1, SRB1bis TimingInfo time when message has been received (as received from the SS by the NAS emulator) |
| Signalling | NB_C_Plane_Indication_Type |  |  |

## F.2.2	Port_Definitions

NBIOT_SRB_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NBIOT_SRB_PORT |  |
| Comment | NBIOT PTC: Port for Sending/Receiving data on SRBs |  |
| out | NB_SRB_COMMON_REQ |  |
| in | NB_SRB_COMMON_IND |  |

NASEMU_NBIOT_SRB_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NASEMU_NBIOT_SRB_PORT |  |
| Comment | NASEMU PTC: Port for Sending/Receiving data on SRBs (interface to NBIOT PTC) |  |
| out | NB_SRB_COMMON_IND |  |
| in | NB_SRB_COMMON_REQ |  |

# F.3	NBIOT_ASP_L2DataDefs

## F.3.1	System_Interface

NB_L2_DATA_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_L2_DATA_REQ |  |  |
| Comment | common ASP to send L2 data |  |  |
| Common | NB_ReqAspCommonPart_Type |  | CellId : identifier of the cell RoutingInfo : SRB/DRB id TimingInfo : starting point when to start sending sequence of data PDUs e.g. SFN = X, subframe number = x; L2Data.SubframeDataList[i].SubframeOffset := offset_i; => L2Data.SubframeDataList[i].PduSduList shall be scheduled in the next search space after SFN = X + ((x + offset_i) / 10); subframe number = (x + offset_i) % 10 ControlInfo : CnfFlag:=false; FollowOnFlag:=false |
| L2Data | L2Data_Request_Type |  |  |

NB_L2_DATA_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_L2_DATA_IND |  |  |
| Comment | common ASP to receive L2 data |  |  |
| Common | NB_IndAspCommonPart_Type |  | CellId : identifier of the cell RoutingInfo : SRB/DRB id TimingInfo : time when message has been received according to clause 7A.6 of TS 36.523-3 |
| L2Data | L2Data_Indication_Type |  |  |

NBIOT_L2DATA_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NBIOT_L2DATA_PORT |  |
| Comment |  |  |
| out | NB_L2_DATA_REQ |  |
| in | NB_L2_DATA_IND |  |

# F.4	EUTRA_NB_ASP_L2DataDefs

ASP interface for DRBs

## F.4.1	PDU_TypeDefs

### F.4.1.1	MAC_PDU

MAC_PDU: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| MAC_CTRL_C_RNTI_Type | C_RNTI | TS 36.321, clause 6.1.3.2 |
| MAC_CTRL_ContentionResolutionId_Type | ContentionResolutionId_Type | TS 36.321, clause 6.1.3.4 fix 48-bit size; consists of a single field defined UE Contention Resolution Identity (first 48 bits of the uplink CCCH SDU transmitted by MAC) |
| MAC_CTRL_TimingAdvance_Type | B8_Type | TS 36.321, clause 6.1.3.5 indicates the amount of timing adjustment in 0.5 ms that the UE has to apply; the length of the field is 8 bits |
| MAC_SDU_Type | octetstring |  |

MAC_PDU_Length_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_PDU_Length_Type |  |  |
| Comment | NOTE: since F and L field are either both present or both omitted they are put into this record; to allow homogeneous (direct) encoding the PDU length is not defined as union; TTCN-3 does allow length restrictions to one lenght or a range of length but not to two specific lengthes; further restriction may be achieved by appropriate templates (parameter either 7 or 15 bit) |  |  |
| Format | B1_Type |  | F: The Format field indicates the size of the Length field as indicated in table 6.2.1-3. There is one F field per MAC PDU subheader except for the last subheader and sub-headers corresponding to fixed-sized MAC control elements. The size of the F field is 1 bit. If the size of the MAC SDU or MAC control element is less than 128 bytes, the UE shall set the value of the F field to 0, otherwise the UE shall set it to 1 |
| Value | B7_15_Type |  | L: The Length field indicates the length of the corresponding MAC SDU or MAC control element in bytes. There is one L field per MAC PDU subheader except for the last subheader and sub-headers corresponding to fixed-sized MAC control elements. The size of the L field is indicated by the F field |

MAC_PDU_SubHeader_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_PDU_SubHeader_Type |  |  |
| Comment |  |  |  |
| Reserved | B2_Type |  | Reserved bits |
| Extension | B1_Type |  | E: The Extension field is a flag indicating if more fields are present in the MAC header or not. The E field is set to "1" to indicate another set of at least R/R/E/LCID fields. The E field is set to "0" to indicate that either a MAC SDU, a MAC control element or padding starts at the next byte |
| LCID | B5_Type |  | LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC control element or padding as described in tables 6.2.1-1 and 6.2.1-2 for the DL and UL-SCH respectively. There is one LCID field for each MAC SDU, MAC control element or padding included in the MAC PDU. The LCID field size is 5 bits; NOTE: In case of DRX command the sub-header corresponds to a control element of length zero (i.e. there is no control element) |
| Length | MAC_PDU_Length_Type | opt |  |

MAC_Header_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | MAC_Header_Type |
| Comment |  |
| record of MAC_PDU_SubHeader_Type |  |

MAC_CTRL_ShortBSR_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_CTRL_ShortBSR_Type |  |  |
| Comment | TS 36.321, clause 6.1.3.1 |  |  |
| LCG | B2_Type |  |  |
| Value | B6_Type |  |  |

MAC_CTRL_LongBSR_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_CTRL_LongBSR_Type |  |  |
| Comment | TS 36.321, clause 6.1.3.1 |  |  |
| Value_LCG1 | B6_Type |  |  |
| Value_LCG2 | B6_Type |  |  |
| Value_LCG3 | B6_Type |  |  |
| Value_LCG4 | B6_Type |  |  |

MAC_CTRL_PowerHeadRoom_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_CTRL_PowerHeadRoom_Type |  |  |
| Comment | TS 36.321, clause 6.1.3.6 |  |  |
| Reserved | B2_Type |  |  |
| Value | B6_Type |  |  |

MAC_CTRL_ElementList_Type

| TTCN-3 Set Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_CTRL_ElementList_Type |  |  |
| Comment | NOTE 1: for simplification UL and DL are not distinguished even though the control elements are either UL or DL NOTE 2: type is defined as set: the ordering is not significant; nevertheless the ordering is well-defined by the sub-headers; for codec implementations it is in any case necessary to evaluate the sub-header information in order to encode/decode the payload |  |  |
| ShortBSR | MAC_CTRL_ShortBSR_Type | opt | UL only |
| LongBSR | MAC_CTRL_LongBSR_Type | opt | UL only |
| C_RNTI | MAC_CTRL_C_RNTI_Type | opt | UL only |
| ContentionResolutionID | MAC_CTRL_ContentionResolutionId_Type | opt | DL only |
| TimingAdvance | MAC_CTRL_TimingAdvance_Type | opt | DL only |
| PowerHeadRoom | MAC_CTRL_PowerHeadRoom_Type | opt | UL only |
| ScellActDeact | MAC_CTRL_ScellActDeact_Type | opt | DL only |
| ExtPowerHeadRoom | MAC_CTRL_ExtPowerHeadRoom_Type | opt | UL only. Only one among PowerHeadroom, ExtPowerHeadroom and DC_PowerHeadRoom may be present |
| DC_PowerHeadRoom | MAC_CTRL_DC_PowerHeadRoom_Type | opt | UL only. Only one among PowerHeadroom, ExtPowerHeadroom and DC_PowerHeadRoom may be present |
| TimingAdvanceReport | MAC_CTRL_TimingAdvanceReport_Type | opt | UL only |
| DifferentialKoffset | MAC_CTRL_DifferentialKoffset_Type | opt | DL only |
| GNSS_Meas | MAC_CTRL_GNSS_Meas_Type | opt | DL only |
| GNSS_ValidityDuration | MAC_CTRL_GNSS_ValidityDuration_Type | opt | UL only |

MAC_SDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | MAC_SDUList_Type |
| Comment |  |
| record of MAC_SDU_Type |  |

MAC_PDU_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_PDU_Type |  |  |
| Comment |  |  |  |
| Header | MAC_Header_Type |  | list of MAC PDU SubHeaders corresponding to MAC control elements and MAC SDUs |
| CtrlElementList | MAC_CTRL_ElementList_Type | opt | Mac control elements; acc. to TS 36.321, clause 6.1.2 "MAC control elements, are always placed before any MAC SDU." |
| SduList | MAC_SDUList_Type | opt | MAC SDUs, which can typically be RLC PDUs |
| Padding | octetstring | opt | Octet aligned Padding if more than or equal to 2 bytes |

MAC_PDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | MAC_PDUList_Type |
| Comment |  |
| record of MAC_PDU_Type |  |

### F.4.1.2	RLC_PDU

RLC_PDU: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| RLC_SDU_Type | octetstring |  |

RLC_PDU_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RLC_PDU_Type |  |
| Comment |  |  |
| TMD | RLC_TMD_PDU_Type |  |
| UMD | RLC_UMD_PDU_Type |  |
| AMD | RLC_AMD_PDU_Type |  |
| AMD_Ext | RLC_AMD_PDU_Ext_Type |  |
| AMD_SegExt | RLC_AMD_PDU_SegExt_Type |  |
| Status | RLC_AM_StatusPDU_Type |  |
| Status_Ext | RLC_AM_StatusPDU_Ext_Type |  |

RLC_PDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RLC_PDUList_Type |
| Comment |  |
| record of RLC_PDU_Type |  |

RLC_SDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RLC_SDUList_Type |
| Comment |  |
| record of RLC_SDU_Type |  |

#### F.4.1.2.1	Common

RLC PDU definition: common AM/UM field definitions

Common: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| RLC_FramingInfo_Type | B2_Type | 00 - First byte of the Data field corresponds to the first byte of a RLC SDU. Last byte of the Data field corresponds to the last byte of a RLC SDU. 01 - First byte of the Data field corresponds to the first byte of a RLC SDU. Last byte of the Data field does not correspond to the last byte of a RLC SDU. 10 - First byte of the Data field does not correspond to the first byte of a RLC SDU. Last byte of the Data field corresponds to the last byte of a RLC SDU. 11 - First byte of the Data field does not correspond to the first byte of a RLC SDU. Last byte of the Data field does not correspond to the last byte of a RLC SDU. |

RLC_LengthIndicator_LI11_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_LengthIndicator_LI11_Type |  |  |
| Comment |  |  |  |
| Extension | B1_Type |  | 0 - Data field follows from the octet following the LI field following this E field 1 - A set of E field and LI field follows from the bit following the LI field following this E field |
| LengthIndicator | B11_Type |  | Length Indicator |

RLC_LengthIndicator_LI15_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_LengthIndicator_LI15_Type |  |  |
| Comment |  |  |  |
| Extension | B1_Type |  | 0 - Data field follows from the octet following the LI field following this E field 1 - A set of E field and LI field follows from the bit following the LI field following this E field |
| LengthIndicator | B15_Type |  | 15 bit Length Indicator |

RLC_LI11_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RLC_LI11_List_Type |
| Comment |  |
| record of RLC_LengthIndicator_LI11_Type |  |

RLC_LI15_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RLC_LI15_List_Type |
| Comment |  |
| record of RLC_LengthIndicator_LI15_Type |  |

RLC_LI_List_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RLC_LI_List_Type |  |
| Comment |  |  |
| LI11 | RLC_LI11_List_Type |  |
| LI15 | RLC_LI15_List_Type |  |

RLC_PDU_Header_FlexPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_PDU_Header_FlexPart_Type |  |  |
| Comment | Flexible part of the header with a number of K LIs |  |  |
| LengthIndicator | RLC_LI_List_Type |  | List of E, LI fields |
| Padding | B4_Type | opt | optional 4 bit padding present in case of odd number of LI's |

#### F.4.1.2.2	TM_Data

RLC PDU definition: UM  (TS 36.322, clause 6.2.1.2)

TM_Data: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| RLC_TMD_PDU_Type | octetstring | TS 36.322, clause 6.2.1.2 |

#### F.4.1.2.3	UM_Data

RLC PDU definition: UM  (TS 36.322, clause 6.2.1.3)
NOTE:
To allow direct encoding the definition for RLC UM Data PDU is split into data PDU with 5/10 bit sequence number

UM_Data: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| RLC_DataField_Type | octetstring | restrictions imposed from LI size of 11 bits is not applicable when the LI's are not present |

RLC_UMD_Header_FixPartShortSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_UMD_Header_FixPartShortSN_Type |  |  |
| Comment | TS 36.322, clause 6.2.1.3 Figure 6.2.1.3-1, 6.2.1.3-3 and 6.2.1.3-4); one octet |  |  |
| FramingInfo | RLC_FramingInfo_Type |  | 2 bits FI |
| Extension | B1_Type |  | 1 bit E |
| SequenceNumber | B5_Type |  | 5 bits SN |

RLC_UMD_Header_FixPartLongSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_UMD_Header_FixPartLongSN_Type |  |  |
| Comment | TS 36.322, clause 6.2.1.3 Figure 6.2.1.3-2, 6.2.1.3-5 and 6.2.1.3-6); two octets |  |  |
| Reserved | B3_Type |  | 3 bits reserved |
| FramingInfo | RLC_FramingInfo_Type |  | 2 bits FI |
| Extension | B1_Type |  | 1 bit E |
| SequenceNumber | B10_Type |  | 10 bits SN |

RLC_UMD_HeaderShortSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_UMD_HeaderShortSN_Type |  |  |
| Comment |  |  |  |
| FixPart | RLC_UMD_Header_FixPartShortSN_Type |  |  |
| FlexPart | RLC_PDU_Header_FlexPart_Type | opt |  |

RLC_UMD_HeaderLongSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_UMD_HeaderLongSN_Type |  |  |
| Comment |  |  |  |
| FixPart | RLC_UMD_Header_FixPartLongSN_Type |  |  |
| FlexPart | RLC_PDU_Header_FlexPart_Type | opt |  |

RLC_DataFieldList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RLC_DataFieldList_Type |
| Comment | One to one correspondence with sub headers (LengthIndicatorList_Type) |
| record of RLC_DataField_Type |  |

RLC_UMD_PDU_ShortSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_UMD_PDU_ShortSN_Type |  |  |
| Comment |  |  |  |
| Header | RLC_UMD_HeaderShortSN_Type |  |  |
| Data | RLC_DataFieldList_Type |  |  |

RLC_UMD_PDU_LongSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_UMD_PDU_LongSN_Type |  |  |
| Comment |  |  |  |
| Header | RLC_UMD_HeaderLongSN_Type |  |  |
| Data | RLC_DataFieldList_Type |  |  |

RLC_UMD_PDU_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RLC_UMD_PDU_Type |  |
| Comment |  |  |
| ShortSN | RLC_UMD_PDU_ShortSN_Type |  |
| LongSN | RLC_UMD_PDU_LongSN_Type |  |

#### F.4.1.2.4	AM_Data

RLC PDU definition: AM (TS 36.322, clause 6.2.1.4 and 6.2.1.5)

RLC_AMD_Header_FixPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_Header_FixPart_Type |  |  |
| Comment | TS 36.322, clause 6.2.1.4 Figure 6.2.1.4-1, 6.2.1.4-2 and 6.2.1.4-3); 2 or 4 octets (AMD PDU or AMD PDU segment) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| ReSeg | B1_Type |  | 0 - AMD PDU 1 - AMD PDU segment |
| Poll | B1_Type |  | 0 - Status report not requested 1 - Status report is requested |
| FramingInfo | RLC_FramingInfo_Type |  | 2 bit FI |
| Extension | B1_Type |  | 1 bit E |
| SN | B10_Type |  | Sequence number |

RLC_AMD_Header_FixPartExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_Header_FixPartExt_Type |  |  |
| Comment | TS 36.322, clause 6.2.1.4 Figure 6.2.1.4-1a, 6.2.1.4-2a and 6.2.1.4-3a); 3 octets |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| ReSeg | B1_Type |  | 0 - AMD PDU 1 - AMD PDU segment |
| Poll | B1_Type |  | 0 - Status report not requested 1 - Status report is requested |
| FramingInfo | RLC_FramingInfo_Type |  | 2 bit FI |
| Extension | B1_Type |  | 1 bit E |
| Reserved | B2_Type |  | 2 reserved bits |
| SN_Ext | B16_Type |  | Sequence number |

RLC_AMD_Header_FixPartSegExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_Header_FixPartSegExt_Type |  |  |
| Comment | TS 36.322, clause 6.2.1.5 Figure 6.2.1.5-1a); 5 octets |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| ReSeg | B1_Type |  | 0 - AMD PDU 1 - AMD PDU segment |
| Poll | B1_Type |  | 0 - Status report not requested 1 - Status report is requested |
| FramingInfo | RLC_FramingInfo_Type |  | 2 bit FI |
| Extension | B1_Type |  | 1 bit E |
| LastSegmentFlag | B1_Type |  | 0 - Last byte of the AMD PDU segment does not correspond to the last byte of an AMD PDU 1 - Last byte of the AMD PDU segment corresponds to the last byte of an AMD PDU |
| Reserved | B1_Type |  | 1 reserved bit |
| SN_Ext | B16_Type |  | Sequence number |

RLC_AMD_Header_SegmentPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_Header_SegmentPart_Type |  |  |
| Comment | AMD PDU segment related info in PDU header acc. TS 36.322, clause 6.2.1.5 |  |  |
| LastSegmentFlag | B1_Type |  | 0 - Last byte of the AMD PDU segment does not correspond to the last byte of an AMD PDU 1 - Last byte of the AMD PDU segment corresponds to the last byte of an AMD PDU |
| SegOffset | B15_Type |  | The SO field indicates the position of the AMD PDU segment in bytes within the original AMD PDU. Specifically, the SO field indicates the position within the Data field of the original AMD PDU to which the first byte of the Data field of the AMD PDU segment corresponds to. |

RLC_AMD_Header_SegmentPartExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_Header_SegmentPartExt_Type |  |  |
| Comment | AMD PDU segment related info in PDU header acc. TS 36.322, clause 6.2.1.5 |  |  |
| SegOffset | B16_Type |  | The SO field indicates the position of the AMD PDU segment in bytes within the original AMD PDU. Specifically, the SO field indicates the position within the Data field of the original AMD PDU to which the first byte of the Data field of the AMD PDU segment corresponds to. |

RLC_AMD_Header_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_Header_Type |  |  |
| Comment |  |  |  |
| FixPart | RLC_AMD_Header_FixPart_Type |  |  |
| SegmentPart | RLC_AMD_Header_SegmentPart_Type | opt | present in case of AMD Seg PDU only |
| FlexPart | RLC_PDU_Header_FlexPart_Type | opt |  |

RLC_AMD_HeaderExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_HeaderExt_Type |  |  |
| Comment |  |  |  |
| FixPartExt | RLC_AMD_Header_FixPartExt_Type |  |  |
| FlexPart | RLC_PDU_Header_FlexPart_Type | opt |  |

RLC_AMD_HeaderSegExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_HeaderSegExt_Type |  |  |
| Comment |  |  |  |
| FixPartSegExt | RLC_AMD_Header_FixPartSegExt_Type |  |  |
| SegmentPart | RLC_AMD_Header_SegmentPartExt_Type | opt | present in case of AMD Seg PDU only |
| FlexPart | RLC_PDU_Header_FlexPart_Type | opt |  |

RLC_AMD_PDU_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_PDU_Type |  |  |
| Comment |  |  |  |
| Header | RLC_AMD_Header_Type |  |  |
| Data | RLC_DataFieldList_Type |  |  |

RLC_AMD_PDU_Ext_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_PDU_Ext_Type |  |  |
| Comment |  |  |  |
| HeaderExt | RLC_AMD_HeaderExt_Type |  |  |
| Data | RLC_DataFieldList_Type |  |  |

RLC_AMD_PDU_SegExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AMD_PDU_SegExt_Type |  |  |
| Comment |  |  |  |
| HeaderSegExt | RLC_AMD_HeaderSegExt_Type |  |  |
| Data | RLC_DataFieldList_Type |  |  |

#### F.4.1.2.5	AM_Status

AM Status PDU (TS 36.322, clause 6.2.1.6)

AM_Status: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| RLC_Status_Padding_Type | bitstring length (1..7) | NOTE: in TTCN-3 length restriction cannot be done inline in record definition => explicit type definition necessary |

RLC_Status_ACK_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_Status_ACK_Type |  |  |
| Comment |  |  |  |
| ACK_SN | B10_Type |  | Acknowledgement SN (TS 36.322, clause 6.2.2.14) |
| Extn1 | B1_Type |  | 0 - a set of NACK_SN, E1 and E2 does not follow. 1 - a set of NACK_SN, E1 and E2 follows. |

RLC_Status_ACK_Ext_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_Status_ACK_Ext_Type |  |  |
| Comment |  |  |  |
| ACK_SN_Ext | B16_Type |  | Acknowledgement SN (TS 36.322, clause 6.2.2.14) |
| Extn1 | B1_Type |  | 0 - a set of NACK_SN, E1 and E2 does not follow. 1 - a set of NACK_SN, E1 and E2 follows. |

RLC_Status_SegOffset_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_Status_SegOffset_Type |  |  |
| Comment |  |  |  |
| Start | B15_Type |  | SOstart field indicates the position of the first byte of the portion of the AMD PDU in bytes within the Data field of the AMD PDU |
| End | B15_Type |  | SOend field indicates the position of the last byte of the portion of the AMD PDU in bytes within the Data field of the AMD PDU. The special SOend value '111111111111111'B is used to indicate that the missing portion of the AMD PDU includes all bytes to the last byte of the AMD PDU |

RLC_Status_SegOffset_Ext_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_Status_SegOffset_Ext_Type |  |  |
| Comment |  |  |  |
| Start | B16_Type |  | SOstart field indicates the position of the first byte of the portion of the AMD PDU in bytes within the Data field of the AMD PDU |
| End | B16_Type |  | SOend field indicates the position of the last byte of the portion of the AMD PDU in bytes within the Data field of the AMD PDU. The special SOend value '111111111111111'B is used to indicate that the missing portion of the AMD PDU includes all bytes to the last byte of the AMD PDU |

RLC_Status_NACK_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_Status_NACK_Type |  |  |
| Comment |  |  |  |
| NACK_SN | B10_Type |  |  |
| Extn1 | B1_Type |  | 0 - A set of NACK_SN, E1 and E2 does not follow. 1 - A set of NACK_SN, E1 and E2 follows. |
| Extn2 | B1_Type |  | 0 - A set of SOstart and SOend does not follow for this NACK_SN. 1 - A set of SOstart and SOend follows for this NACK_SN. |
| SO | RLC_Status_SegOffset_Type | opt |  |

RLC_Status_NACK_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RLC_Status_NACK_List_Type |
| Comment |  |
| record of RLC_Status_NACK_Type |  |

RLC_Status_NACK_Ext_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_Status_NACK_Ext_Type |  |  |
| Comment |  |  |  |
| NACK_SN_Ext | B16_Type |  |  |
| Extn1 | B1_Type |  | 0 - A set of NACK_SN, E1 and E2 does not follow. 1 - A set of NACK_SN, E1 and E2 follows. |
| Extn2 | B1_Type |  | 0 - A set of SOstart and SOend does not follow for this NACK_SN. 1 - A set of SOstart and SOend follows for this NACK_SN. |
| SO | RLC_Status_SegOffset_Ext_Type | opt |  |

RLC_Status_NACK_Ext_List_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | RLC_Status_NACK_Ext_List_Type |
| Comment |  |
| record of RLC_Status_NACK_Ext_Type |  |

RLC_AM_StatusPDU_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AM_StatusPDU_Type |  |  |
| Comment | RLC STATUS PDU (TS 36.322, clause 6.2.1.6) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| CP_Type | B3_Type |  | 000 - STATUS PDU 001..111 - Reserved (=> PDU to be discarded by the receiving entity for this release of the protocol) |
| Ack | RLC_Status_ACK_Type |  | ACK_SN and E1 bit |
| NackList | RLC_Status_NACK_List_Type | opt | presence depends on Extn1 bit of Ack filed (RLC_Status_ACK_Type) |
| Padding | RLC_Status_Padding_Type | opt | 1..7 bit padding if needed for octet alignment |

RLC_AM_StatusPDU_Ext_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | RLC_AM_StatusPDU_Ext_Type |  |  |
| Comment | RLC STATUS PDU (TS 36.322, clause 6.2.1.6) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| CP_Type | B3_Type |  | 000 - STATUS PDU 001..111 - Reserved (=> PDU to be discarded by the receiving entity for this release of the protocol) |
| Ack_Ext | RLC_Status_ACK_Ext_Type |  | ACK_SN and E1 bit |
| Nack_Ext_List | RLC_Status_NACK_Ext_List_Type | opt | presence depends on Extn1 bit of Ack field (RLC_Status_ACK_Type) |
| Padding | RLC_Status_Padding_Type | opt | 1..7 bit padding if needed for octet alignment |

### F.4.1.3	PDCP

PDCP user plane SDU and PDU definitions
NOTE:
To allow direct encoding the definition for PDCP Data PDU is split into data PDU with long/short sequence number

PDCP: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| PDCP_SDU_Type | octetstring |  |

PDCP_SDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | PDCP_SDUList_Type |
| Comment |  |
| record of PDCP_SDU_Type |  |

PDCP_DataPdu_LongSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_LongSN_Type |  |  |
| Comment | User plane PDCP Data PDU with long sequence number (TS 36.323, clause 6.2.3) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| Reserved | B3_Type |  | 3 reserved bits |
| SequenceNumber | B12_Type |  | 12 bit sequence number |
| SDU | PDCP_SDU_Type |  | content (octetstring) |

PDCP_DataPdu_ShortSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_ShortSN_Type |  |  |
| Comment | User plane PDCP Data PDU with short sequence number (TS 36.323, clause 6.2.4) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| SequenceNumber | B7_Type |  | 7 bit sequence number |
| SDU | PDCP_SDU_Type |  | content (octetstring) |

PDCP_DataPdu_ExtSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_ExtSN_Type |  |  |
| Comment | User plane PDCP Data PDU with extended sequence number (TS 36.323, clause 6.2.9) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| SequenceNumber | B15_Type |  | 15 bit sequence number |
| SDU | PDCP_SDU_Type |  | content (octetstring) |

PDCP_DataPdu_18bitSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_18bitSN_Type |  |  |
| Comment | User plane PDCP Data PDU with 18 bit sequence number (TS 36.323, clause 6.2.11) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PollingBit | B1_Type |  | 1 - indicates the UE is requested to send a PDCP status report or a LWA status report for LWA |
| Reserved | B4_Type |  | 4 reserved bits |
| SequenceNumber | B18_Type |  | 18 bit sequence number |
| SDU | PDCP_SDU_Type |  | content (octetstring) |

PDCP_DataPdu_LongSN_UDC_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_LongSN_UDC_Type |  |  |
| Comment | User plane PDCP Data PDU with long sequence number for UDC (TS 36.323, clause 6.2.14) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| Reserved | B3_Type |  | 3 reserved bits |
| SequenceNumber | B12_Type |  | 12 bit sequence number |
| FU | B1_Type |  | 0 - Packet is not compressed using UDC protocol 1 - Packet is compressed using UDC protocol |
| FR | B1_Type |  | 0 - Compression buffer is not reset 1 - Compression buffer has been reset |
| Reserved2 | B2_Type |  | 2 reserved bits |
| Checksum | B4_Type |  | 4 validation bits for the compression buffer content |
| SDU | PDCP_SDU_Type |  | content (octetstring) |

PDCP_DataPdu_ExtSN_UDC_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_ExtSN_UDC_Type |  |  |
| Comment | User plane PDCP Data PDU with long sequence number for UDC (TS 36.323, clause 6.2.15) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| SequenceNumber | B15_Type |  | 15 bit sequence number |
| FU | B1_Type |  | 0 - Packet is not compressed using UDC protocol 1 - Packet is compressed using UDC protocol |
| FR | B1_Type |  | 0 - Compression buffer is not reset 1 - Compression buffer has been reset |
| Reserved2 | B2_Type |  | 2 reserved bits |
| Checksum | B4_Type |  | 4 validation bits for the compression buffer content |
| SDU | PDCP_SDU_Type |  | content (octetstring) |

PDCP_DataPdu_18bitSN_UDC_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_18bitSN_UDC_Type |  |  |
| Comment | User plane PDCP Data PDU with long sequence number for UDC (TS 36.323, clause 6.2.16) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PollingBit | B1_Type |  | 1 - indicates the UE is requested to send a PDCP status report or a LWA status report for LWA |
| Reserved | B4_Type |  | 4 reserved bits |
| SequenceNumber | B18_Type |  | 18 bit sequence number |
| FU | B1_Type |  | 0 - Packet is not compressed using UDC protocol 1 - Packet is compressed using UDC protocol |
| FR | B1_Type |  | 0 - Compression buffer is not reset 1 - Compression buffer has been reset |
| Reserved2 | B2_Type |  | 2 reserved bits |
| Checksum | B4_Type |  | 4 validation bits for the compression buffer content |
| SDU | PDCP_SDU_Type |  | content (octetstring) |

PDCP_DataPdu_SLRB_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_SLRB_Type |  |  |
| Comment | User plane PDCP Data PDU for SLRB (TS 36.323, clause 6.2.10.1) |  |  |
| SDU_Type | B3_Type |  | 000 - IP 001 - ARP 010 - PC5 Signaling 011 - Non-IP 100 - 111 reserved |
| PGK_Index | B5_Type |  | 5 LSBs of PGK Identity (TS 33.303, clause 6.2.3) |
| PTK_Identity | B16_Type |  | PTK Identity (TS 33.303, clause 6.2.3) |
| SequenceNumber | B16_Type |  | 16 bit sequence number |
| SDU | PDCP_SDU_Type |  | content (octetstring) |

PDCP_DataPdu_SLRB_1to1_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_DataPdu_SLRB_1to1_Type |  |  |
| Comment | User plane PDCP Data PDU for SLRB (TS 36.323, clause 6.2.10.2) |  |  |
| SDU_Type | B3_Type |  | 000 - IP 001 - ARP 010 - PC5 Signaling 011 - Non-IP 100 - 111 reserved |
| Reserved | B5_Type |  | 5 reserved bits |
| KD_sess_ID | B16_Type |  | KD_sess Identity (TS 33.303, clause 6.5.3.1) |
| SequenceNumber | B16_Type |  | 16 bit sequence number |
| SDU | PDCP_SDU_Type |  | content (octetstring) |
| MAC_I | O4_Type |  | MAC-I |

PDCP_Ctrl_ROHC_FB_PDU_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_ROHC_FB_PDU_Type |  |  |
| Comment | PDCP Control PDU for interspersed ROHC feedback packet (TS 36.323, clause 6.2.5) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| Reserved | B4_Type |  |  |
| ROHC_FB | octetstring |  | Contains one ROHC packet with only feedback, i.e. a ROHC packet that is not associated with a PDCP |

PDCP_Ctrl_StatusReport_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_StatusReport_Type |  |  |
| Comment | PDCP Control PDU for PDCP status report (TS 36.323, clause 6.2.6) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| FMS | B12_Type |  | PDCP SN of the first missing PDCP SDU. |
| Bitmap | octetstring | opt | The MSB of the first octet of the type "Bitmap" indicates whether or not the PDCP SDU with the SN (FMS + 1) modulo 4096 has been received and, optionally decompressed correctly. 0 - PDCP SDU with PDCP SN = (FMS + bit position) modulo 4096 is missing in the receiver. The bit position of Nth bit in the Bitmap is N, i.e. the bit position of the first bit in the Bitmap is 1. 1 - PDCP SDU with PDCP SN = (FMS + bit position) modulo 4096 does not need to be retransmitted. The bit position of Nth bit in the Bitmap is N, i.e. the bit position of the first bit in the Bitmap is 1. |

PDCP_Ctrl_StatusReportExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_StatusReportExt_Type |  |  |
| Comment | PDCP Control PDU for PDCP status report using a 15 bit SN (TS 36.323, clause 6.2.6) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| Reserved | B5_Type |  | 5 reserved bits |
| FMS_Ext | B15_Type |  | PDCP SN of the first missing PDCP SDU. |
| Bitmap | octetstring | opt | The MSB of the first octet of the type "Bitmap" indicates whether or not the PDCP SDU with the SN (FMS + 1) modulo (Maximum_PDCP_SN + 1) has been received and, optionally decompressed correctly. 0 - PDCP SDU with PDCP SN = (FMS + bit position) modulo (Maximum_PDCP_SN + 1) is missing in the receiver. The bit position of Nth bit in the Bitmap is N, i.e. the bit position of the first bit in the Bitmap is 1. 1 - PDCP SDU with PDCP SN = (FMS + bit position) modulo (Maximum_PDCP_SN + 1) does not need to be retransmitted. The bit position of Nth bit in the Bitmap is N, i.e. the bit position of the first bit in the Bitmap is 1. |

PDCP_Ctrl_StatusReport_18bitSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_StatusReport_18bitSN_Type |  |  |
| Comment | PDCP Control PDU for PDCP status report using a 18 bit SN (TS 36.323, clause 6.2.6) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| Reserved | B2_Type |  | 2 reserved bits |
| FMS_18bitSN | B18_Type |  | PDCP SN of the first missing PDCP SDU. |
| Bitmap | octetstring | opt | The MSB of the first octet of the type "Bitmap" indicates whether or not the PDCP SDU with the SN (FMS + 1) modulo (Maximum_PDCP_SN + 1) has been received and, optionally decompressed correctly. 0 - PDCP SDU with PDCP SN = (FMS + bit position) modulo (Maximum_PDCP_SN + 1) is missing in the receiver. The bit position of Nth bit in the Bitmap is N, i.e. the bit position of the first bit in the Bitmap is 1. 1 - PDCP SDU with PDCP SN = (FMS + bit position) modulo (Maximum_PDCP_SN + 1) does not need to be retransmitted. The bit position of Nth bit in the Bitmap is N, i.e. the bit position of the first bit in the Bitmap is 1. |

PDCP_Ctrl_LWA_StatusReport_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_LWA_StatusReport_Type |  |  |
| Comment | PDCP Control PDU for LWA status report (TS 36.323, clause 6.2.12) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| FMS | B12_Type |  | PDCP SN of the first missing PDCP SDU. |
| HRW | B12_Type |  | PDCP SN of the PDCP SDU received on WLAN with highest associated PDCP COUNT value. |
| NMP | B12_Type |  | Number of Missing PDCP PDUs with PDCP SNs below HRW starting from and including FMS. |

PDCP_Ctrl_LWA_StatusReportExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_LWA_StatusReportExt_Type |  |  |
| Comment | PDCP Control PDU for LWA status report (TS 36.323, clause 6.2.12) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| Reserved | B5_Type |  | 5 reserved bits |
| FMS_Ext | B15_Type |  | PDCP SN of the first missing PDCP SDU. |
| Reserved2 | B1_Type |  | 1 reserved bit |
| HRW_Ext | B15_Type |  | PDCP SN of the PDCP SDU received on WLAN with highest associated PDCP COUNT value. |
| Reserved3 | B1_Type |  | 1 reserved bit |
| NMP_Ext | B15_Type |  | Number of Missing PDCP PDUs with PDCP SNs below HRW starting from and including FMS. |

PDCP_Ctrl_LWA_StatusReport_18bitSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_LWA_StatusReport_18bitSN_Type |  |  |
| Comment | PDCP Control PDU for LWA status report (TS 36.323, clause 6.2.12) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| Reserved | B2_Type |  | 2 reserved bits |
| FMS_18bitSN | B18_Type |  | PDCP SN of the first missing PDCP SDU. |
| HRW_18bitSN | B18_Type |  | PDCP SN of the PDCP SDU received on WLAN with highest associated PDCP COUNT value. |
| Reserved2 | B4_Type |  | 4 reserved bits |
| NMP_18bitSN | B18_Type |  | Number of Missing PDCP PDUs with PDCP SNs below HRW starting from and including FMS. |

PDCP_Ctrl_LWA_EndMarker_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_LWA_EndMarker_Type |  |  |
| Comment | PDCP Control PDU for LWA end marker (TS 36.323, clause 6.2.13) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| LSN | B12_Type |  | PDCP SN of the last PDCP PDU for which the data part is ciphered with the key used before PDCP re-establishment. |

PDCP_Ctrl_LWA_EndMarkerExt_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_LWA_EndMarkerExt_Type |  |  |
| Comment | PDCP Control PDU for LWA end marker (TS 36.323, clause 6.2.13) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| Reserved | B5_Type |  | 5 reserved bits |
| LSN_Ext | B15_Type |  | PDCP SN of the last PDCP PDU for which the data part is ciphered with the key used before PDCP re-establishment. |

PDCP_Ctrl_LWA_EndMarker_18bitSN_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_LWA_EndMarker_18bitSN_Type |  |  |
| Comment | PDCP Control PDU for LWA end marker (TS 36.323, clause 6.2.13) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 000 - PDCP status report 001 - Header Compression Feedback Information 010 - LWA status report 011 - LWA end-marker packet 100..111 - reserved |
| Reserved | B2_Type |  | 2 reserved bits |
| LSN_18bitSN | B18_Type |  | PDCP SN of the last PDCP PDU for which the data part is ciphered with the key used before PDCP re-establishment. |

PDCP_Ctrl_UDC_FB_PDU_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_UDC_FB_PDU_Type |  |  |
| Comment | PDCP Control PDU for UDC feedback packet (TS 36.323, clause 6.2.17) |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 100 - UDC feedback packet |
| FE | B1_Type |  | 0 - No Error 1 - Checksum Error Notification |
| Reserved | B3_Type |  |  |

PDCP_Ctrl_EHC_FB_PDU_ShortCID_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_EHC_FB_PDU_ShortCID_Type |  |  |
| Comment |  |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 101 - EHC feedback packet |
| Reserved | B4_Type |  | 4 bits reserved |
| Reserved2 | B1_Type |  | 1 bit reserved |
| CID | B7_Type |  | 7 bits CID |

PDCP_Ctrl_EHC_FB_PDU_LongCID_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PDCP_Ctrl_EHC_FB_PDU_LongCID_Type |  |  |
| Comment |  |  |  |
| D_C | B1_Type |  | 0 - Control PDU 1 - Data PDU |
| PDU_Type | B3_Type |  | 101 - EHC feedback packet |
| Reserved | B4_Type |  | 4 bits reserved |
| Reserved2 | B1_Type |  | 1 bit reserved |
| CID | B15_Type |  | 15 bits CID |

PDCP_PDU_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | PDCP_PDU_Type |  |
| Comment |  |  |
| DataLongSN | PDCP_DataPdu_LongSN_Type | user plane PDCP data PDU with 12 Bit Seq Number |
| DataShortSN | PDCP_DataPdu_ShortSN_Type | user plane PDCP data PDU with 7 Bit Seq Number |
| DataExtSN | PDCP_DataPdu_ExtSN_Type | user plane PDCP data PDU with 15 Bit Seq Number |
| Data_18bitSN | PDCP_DataPdu_18bitSN_Type | user plane PDCP data PDU with 18 Bit Seq Number |
| RohcFeedback | PDCP_Ctrl_ROHC_FB_PDU_Type | PDCP Control PDU for interspersed ROHC feedback packet |
| StatusReport | PDCP_Ctrl_StatusReport_Type | PDCP Control PDU for PDCP status report |
| StatusReportExt | PDCP_Ctrl_StatusReportExt_Type | PDCP Control PDU for PDCP status report using a 15 bit SN |
| StatusReport_18bitSN | PDCP_Ctrl_StatusReport_18bitSN_Type | PDCP Control PDU for PDCP status report using a 18 bit SN |
| LWA_StatusReport | PDCP_Ctrl_LWA_StatusReport_Type | PDCP Control PDU for LWA status report |
| LWA_StatusReportExt | PDCP_Ctrl_LWA_StatusReportExt_Type | PDCP Control PDU for LWA status report using a 15 bit SN |
| LWA_StatusReport_18bitSN | PDCP_Ctrl_LWA_StatusReport_18bitSN_Type | PDCP Control PDU for LWA status report using a 18 bit SN |
| DataSLRB | PDCP_DataPdu_SLRB_Type | user plane PDCP Data PDU for SLRB |
| DataSLRB_1to1 | PDCP_DataPdu_SLRB_1to1_Type | user plane PDCP Data PDU for SLRB one to one communication |
| LWA_EndMarker | PDCP_Ctrl_LWA_EndMarker_Type | PDCP Control PDU for LWA end marker |
| LWA_EndMarkerExt | PDCP_Ctrl_LWA_EndMarkerExt_Type | PDCP Control PDU for LWA end marker using a 15 bit SN |
| LWA_EndMarker_18bitSN | PDCP_Ctrl_LWA_EndMarker_18bitSN_Type | PDCP Control PDU for LWA end marker using a 18 bit SN |
| DataLongSN_UDC | PDCP_DataPdu_LongSN_UDC_Type | user plane PDCP data PDU with 12 Bit Seq Number for UDC |
| DataExtSN_UDC | PDCP_DataPdu_ExtSN_UDC_Type | user plane PDCP data PDU with 15 Bit Seq Number for UDC |
| Data18bitSN_UDC | PDCP_DataPdu_18bitSN_UDC_Type | user plane PDCP data PDU with 18 Bit Seq Number for UDC |
| UdcFeedback | PDCP_Ctrl_UDC_FB_PDU_Type | PDCP Control PDU for UDC feedback packet |
| EhcFeedback_ShortCID | PDCP_Ctrl_EHC_FB_PDU_ShortCID_Type | PDCP Control PDU for EHC feedback packet Short CID |
| EhcFeedback_LongCID | PDCP_Ctrl_EHC_FB_PDU_LongCID_Type | PDCP Control PDU for EHC feedback packet Long CID |

PDCP_PDUList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | PDCP_PDUList_Type |
| Comment |  |
| record of PDCP_PDU_Type |  |

## F.4.2	DRB_Primitive_Definitions

Primitive definitions to send/receive data PDUs over DRB's

### F.4.2.1	DRB_Common

L2DataList_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | L2DataList_Type |  |
| Comment | MAC: acc. to rel-8 protocols there is not more than one MAC PDU per TTI; any MAC PDU is completely included in one subframe RLC: one or more RLC PDUs per TTI (e.g. RLC Data + Status PDU on a logical channel; more than one RLC Data PDU in one MAC PDU is valid too) any RLC PDU is completely included in one subframe PDCP: one or more PDUs per TTI; one PDCP PDU may be included in more than one subframe |  |
| MacPdu | MAC_PDUList_Type | SS configuration: RLC TM mode, MAC no header removal (PDCP is not configured) |
| RlcPdu | RLC_PDUList_Type | SS configuration: RLC TM mode, MAC header removal (PDCP is not configured) |
| PdcpPdu | PDCP_PDUList_Type | SS configuration: RLC AM/UM mode, PDCP no header removal |
| PdcpSdu | PDCP_SDUList_Type | SS configuration: RLC AM/UM mode, PDCP header removal |
| NrPdcpSdu | PDCP_SDUList_Type | SS configuration: RLC AM/UM mode, PDCP header removal |
| RlcSdu | RLC_SDUList_Type | SS configuration: RLC UM mode with no PDCP, for example MRB |
| SdapPdu | SDAP_PDUList_Type | SS configuration: RLC AM/UM mode, PDCP normal mode (automatic handling of PDCP header), no handling of SDAP header |
| SdapSdu | SDAP_SDUList_Type | SS configuration: RLC AM/UM mode, PDCP normal mode (automatic handling of PDCP header), automatic handling of SDAP header |

HarqProcessAssignment_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | HarqProcessAssignment_Type |  |
| Comment | in DL the HARQ process id may be specified by the test case or automatically assigned by SS |  |
| Id | HarqProcessId_Type | HARQ process as specified by the test case NOTE1: the scope of this type is only for data being sent in one TTI; if data needs more than one TTI the HarqProcessId is undefined for the 2nd TTI onward what shall be handled as an error at the SS; SS may send a SYSTEM_IND indicating an error in this case; NOTE2: The initial value of the NDI shall be the same for all HARQ processes and cells |
| Automatic | Null_Type | HARQ process id automatically assigned by SS |

### F.4.2.2	Downlink

DRB_DataPerSubframe_DL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DRB_DataPerSubframe_DL_Type |  |  |
| Comment | common definition for one or several PDUs/SDUs; in EUTRA the DL data is sent in the subframe given by the subframe offset; in NBIOT the DL transmission is scheduled in the search space candidate given by "SubframeOffset" for NBIOT it specifies the NOTE 1: For MAC and RLC PDUs a single PDU is always sent in one subframe; SS shall raise an error indication (using SYSTEM_IND) when that is not possible NOTE 2: For PDCP the data may be spread over more than one subframe (segmented by the RLC); the TTCN implemetation is responsible to calculate appropriate offsets accordingly; the exact timing depends on (and is exactly specified by) configuration of the DL scheduling; SS shall raise an error when there is any conflict |  |  |
| SubframeOffset | integer |  | EUTRA: Subframe offset relative to the absolute timing information given in the common part of the ASP; NBIOT: Offset of the search space candidate relative to the (default) search space candidate as configured for the NPDCCH UE specific search space; the search space candidate is in the next search space starting at or after the point in time given by the timing information of the common part of the ASP NOTE 1: Notes: Acc. to TS 36.523-3, clause 7.3.3 in case of TDD or half-duplex configuration only subframes available for DL are taken into consideration NOTE 2: if a PDCP PDU or SDU takes more than one subframe, SubframeOffset specifies the first TTI |
| HarqProcess | HarqProcessAssignment_Type | opt | HARQ process to be used: specific value (0..7) or automatically assigned by SS; in automatic mode SS chooses HARQ process out of the set configured by CcchDcchDtchConfigDL_Type.HarqProcessConfig NOTE 1: for PDCP SDUs or PDUs automatic mode shall be used; otherwise SS shall raise an error NOTE 2: for NB-IoT the HarqProcess shall be omitted by TTCN and ignored by the SS |
| PduSduList | L2DataList_Type |  | list of PDUs/SDUs to be sent in one TTI |

DRB_DataPerSubframeList_DL_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | DRB_DataPerSubframeList_DL_Type |
| Comment | list of user plane data to be sent in sub-frames given by the SubframeOffset in the single elements of the list; Timing: the start time for the whole sequence is given by the timing info of the ASP (common information); the timing for the respective data pdus is given by the SubframeOffset relative to the common timing info; design consideration: repetitions of this sequence are not foreseen (in which case the subframe offset could not be related to the timing info of the ASP) |
| record of DRB_DataPerSubframe_DL_Type |  |

L2Data_Request_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | L2Data_Request_Type |  |  |
| Comment | NOTE: formal type definition to allow later enhancements; L2Data_Request_Type defines a sequence of subframes in which data shall be sent |  |  |
| SubframeDataList | DRB_DataPerSubframeList_DL_Type |  |  |

### F.4.2.3	Uplink

DRB_DataPerSubframe_UL_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | DRB_DataPerSubframe_UL_Type |  |  |
| Comment | common definition for one or several PDUs/SDUs being received in one subframe or to receive one PDCP PDU or SDU being spread over more than one TTI; NOTE: There is a fix relation between HARQ process id and subframe in UL => it is not necessary to include HARQ process id for UL data |  |  |
| PduSduList | L2DataList_Type |  | list of PDUs/SDUs being received in one TTI; elements of the list appear in the same order as the PDUs/SDUs in the MAC PDU; for PDCP when a PDU or SDU takes more than one TTI the list only contains this PDU or SDU |
| NoOfTTIs | integer |  | in case of PDCP: number of TTIs the SDU or PDU has taken NOTE 1: for the time being the NoOfTTIs is not checked by TTCN-3 and may be set to 1 by SS; NOTE 2: the timing info in common part of the ASP refers to the last TTI NOTE 3: when NoOfTTIs > 1 => PduSduList shall only contain one PDCP PDU or SDU in case of MAC or RLC PDUs: NoOfTTIs shall always be 1 (acc. to TS 36.321 MAC is not doing segmentation of RLC PDUs and acc. to TS 36.322, clause 6.2.2.2 the maximum RLC data is calculated to fit into a MAC PDU and RLC does segmentation accordingly) |

L2Data_Indication_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | L2Data_Indication_Type |  |  |
| Comment | NOTE: formal type definition to allow later enhancements; L2Data_Indication_Type defines data being received in a single subframe i.e. PDUs of subsequent TTIs are indicated in separated ASPs |  |  |
| SubframeData | DRB_DataPerSubframe_UL_Type |  |  |

# F.5	NasEmu_AspTypes_NBIOT

System interface between NAS emulation and system adaptor

## F.5.1	System_Interface

NBIOT_RRC_PDU_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NBIOT_RRC_PDU_REQ |  |  |
| Comment |  |  |  |
| Common | NB_ReqAspCommonPart_Type |  | CellId : identifier of the cell RoutingInfo : SRB0, SRB1, SRB1bis TimingInfo : Now in normal cases; For latency tests TimingInfo can be set to the SFN/subframe in which the RRC messages shall be sent out NOTE 1: if the RRC PDU is too long to be sent in one TTI the TimingInfo corresponds to the first TTI NOTE 2: the TimingInfo is not changed by the NAS Emu (i.e. the timing info as coming from the test case (SRB_COMMON_REQ) is handed through by the NAS Emu) ControlInfo CnfFlag:=false; FollowOnFlag true: Indicates that the message(s) to be sent on the same TTI will follow NOTE 1: If the TimingInfo is not the same for messages to be sent on the same TTI, the SS shall produce an error NOTE 2: the follow on flag applies only for messages of the same SRB false: Indicates that no more message(s) will follow |
| RrcPdu | NB_RRC_MSG_Request_Type |  |  |

NBIOT_RRC_PDU_IND

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NBIOT_RRC_PDU_IND |  |  |
| Comment | common ASP to receive PDUs from SRB0, SRB1, SRB1bis |  |  |
| Common | NB_IndAspCommonPart_Type |  | CellId : identifier of the cell RoutingInfo : SRB0, SRB1, SRB1bis TimingInfo : time when message has been received (frame and sub-frame number); this is handed through to the test case by the NAS emulation NOTE: normally an RRC PDU is expected in one TTI; nevertheless if it is spread over more than one TTIs TimingInfo shall refer to the end of the PDU i.e. to the last RLC PDU being received; Status : OK or RRC integrity error |
| RrcPdu | NB_RRC_MSG_Indication_Type |  |  |

NASEMU_NBIOT_SYSTEM_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NASEMU_NBIOT_SYSTEM_PORT |  |
| Comment | NASEMU PTC: Port for Sending/Receiving data to/from the SYSTEM Interface |  |
| out | NBIOT_RRC_PDU_REQ |  |
| in | NBIOT_RRC_PDU_IND |  |

# F.6	NBIOT_CommonDefs

## F.6.1	NBIOT_Common_Types

NBIOT_CellId_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | NBIOT_CellId_Type |
| Comment |  |
| nbiot_Cell_NonSpecific |  |
| nbiot_Cell1 |  |
| nbiot_Cell2 |  |
| nbiot_Cell3 |  |
| nbiot_Cell4 |  |
| nbiot_Cell5 |  |
| nbiot_Cell6 |  |
| nbiot_Cell10 |  |
| nbiot_Cell11 |  |
| nbiot_Cell12 |  |
| nbiot_Cell13 |  |
| nbiot_Cell14 |  |
| nbiot_Cell18 |  |
| nbiot_Cell23 |  |
| nbiot_Cell50 |  |
| nbiot_Cell51 |  |
| nbiot_Cell52 |  |
| nbiot_Cell53 |  |
| nbiot_Cell54 |  |
| nbiot_Cell55 |  |
| nbiot_Cell56 |  |
| nbiot_Cell57 |  |
| nbiot_Cell58 |  |
| nbiot_Cell59 |  |
| nbiot_Cell60 |  |
| nbiot_Cell61 |  |
| nbiot_Cell62 |  |
| nbiot_Cell63 |  |

NB_CarrierFreq_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_CarrierFreq_Type |  |  |
| Comment |  |  |  |
| DL | CarrierFreq_NB_r13 |  |  |
| UL | CarrierFreq_NB_r13 |  |  |

NB_RRC_MSG_Request_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_RRC_MSG_Request_Type |  |
| Comment | DL RRC PDU on CCCH or DCCH |  |
| Ccch | DL_CCCH_Message_NB |  |
| Dcch | DL_DCCH_Message_NB |  |

NB_RRC_MSG_Indication_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_RRC_MSG_Indication_Type |  |
| Comment | UL RRC PDU on CCCH or DCCH |  |
| Ccch | UL_CCCH_Message_NB |  |
| Dcch | UL_DCCH_Message_NB |  |

## F.6.2	NBIOT_RRC_Nested_Types

NBIOT_RRC_Nested_Types: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NB_OperationModeInfo_Type | MasterInformationBlock_NB.operationModeInfo_r13 |  |
| NB_SchedulingInfoSIB1_Type | MasterInformationBlock_NB.schedulingInfoSIB1_r13 |  |
| NB_SiWindowLength_Type | SystemInformationBlockType1_NB.si_WindowLength_r13 |  |
| NB_EutraControlRegionSize_Type | SystemInformationBlockType1_NB.eutraControlRegionSize_r13 |  |
| NB_SiPeriodicity_Type | SchedulingInfoList_NB_r13[0].si_Periodicity_r13 |  |
| NB_SiRepetitionPattern_Type | SchedulingInfoList_NB_r13[0].si_RepetitionPattern_r13 |  |
| NB_SiTransportBlockSize_Type | SchedulingInfoList_NB_r13[0].si_TB_r13 |  |
| NPDCCH_NumRepetitions_UESS_Type | NPDCCH_ConfigDedicated_NB_r13.npdcch_NumRepetitions_r13 |  |
| NPDCCH_StartSF_UESS_Type | NPDCCH_ConfigDedicated_NB_r13.npdcch_StartSF_USS_r13 |  |
| NPDCCH_Offset_UESS_Type | NPDCCH_ConfigDedicated_NB_r13.npdcch_Offset_USS_r13 |  |
| NPDCCH_NumRepetitions_Type1CSS_Type | PCCH_Config_NB_r13.npdcch_NumRepetitionPaging_r13 |  |
| NPDCCH_NumRepetitions_Type2CSS_Type | NPRACH_Parameters_NB_r13.npdcch_NumRepetitions_RA_r13 |  |
| NPDCCH_StartSF_Type2CSS_Type | NPRACH_Parameters_NB_r13.npdcch_StartSF_CSS_RA_r13 |  |
| NPDCCH_Offset_Type2CSS_Type | NPRACH_Parameters_NB_r13.npdcch_Offset_RA_r13 |  |
| NB_SC_MCCH_CarrierConfig_Type | SystemInformationBlockType20_NB_r14.sc_mcch_CarrierConfig_r14 |  |
| NB_SC_MCCH_RepetitionPeriod_Type | SystemInformationBlockType20_NB_r14.sc_mcch_RepetitionPeriod_r14 |  |
| NB_SC_MCCH_Offset_Type | SystemInformationBlockType20_NB_r14.sc_mcch_Offset_r14 |  |
| NB_SC_MCCH_ModificationPeriod_Type | SystemInformationBlockType20_NB_r14.sc_mcch_ModificationPeriod_r14 |  |
| NB_SC_MCCH_SchedulingInfo_Type | SystemInformationBlockType20_NB_r14.sc_mcch_SchedulingInfo_r14 |  |
| NPDCCH_NumRepetitions_SC_MTCH_Type | SC_MTCH_Info_NB_r14.npdcch_NumRepetitions_SC_MTCH_r14 |  |
| NPDCCH_StartSF_SC_MTCH_Type | SC_MTCH_Info_NB_r14.npdcch_StartSF_SC_MTCH_r14 |  |
| NPDCCH_Offset_SC_MTCH_Type | SC_MTCH_Info_NB_r14.npdcch_Offset_SC_MTCH_r14 |  |
| NPDCCH_NPDSCH_MaxTBS_SC_MTCH_Type | SC_MTCH_Info_NB_r14.npdcch_NPDSCH_MaxTBS_SC_MTCH_r14 |  |
| NB_SC_MTCH_CarrierConfig_Type | SC_MTCH_Info_NB_r14.sc_mtch_CarrierConfig_r14 |  |

## F.6.3	NBIOT_ASP_CommonPart

Definition of ASP common parts for REQ-, CNF- and IND-ASPs

### F.6.3.1	NBIOT_ASP_CommonPart_Definitions

#### F.6.3.1.1	NBIOT_Routing_Info

NBIOT_CommonDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_NB_MaxRB | integer | 2 + 3 | 2 DRBs + 3 SRBs |
| tsc_SRB1bis | integer | 3 |  |

NBIOT_Routing_Info: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NB_SRB_Identity_Type | integer (tsc_SRB0, tsc_SRB1, tsc_SRB1bis) |  |

NB_RadioBearerId_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_RadioBearerId_Type |  |
| Comment |  |  |
| Srb | NB_SRB_Identity_Type |  |
| Drb | EUTRA_ASN1_DRB_Identity_Type |  |
| ScMrb | SC_MRB_Identity_Type |  |

NB_RoutingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NB_RoutingInfo_Type |  |
| Comment |  |  |
| None | Null_Type |  |
| L2Test | Null_Type | to be used to let the NAS emulator route back encoded an RRC message back to the test case instead of forwarding it to the SS |
| RadioBearerId | NB_RadioBearerId_Type |  |

### F.6.3.2	REQ_ASP_CommonPart

NB_ReqAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_ReqAspCommonPart_Type |  |  |
| Comment |  |  |  |
| CellId | NBIOT_CellId_Type |  |  |
| RoutingInfo | NB_RoutingInfo_Type |  |  |
| TimingInfo | TimingInfo_Type |  |  |
| ControlInfo | ReqAspControlInfo_Type |  |  |

### F.6.3.3	NBIOT_CNF_ASP_CommonPart

NB_CnfAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_CnfAspCommonPart_Type |  |  |
| Comment |  |  |  |
| CellId | NBIOT_CellId_Type |  |  |
| RoutingInfo | NB_RoutingInfo_Type |  |  |
| TimingInfo | TimingInfo_Type |  |  |
| Result | ConfirmationResult_Type |  |  |

### F.6.3.4	NBIOT_IND_ASP_CommonPart

NB_IndAspCommonPart_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NB_IndAspCommonPart_Type |  |  |
| Comment |  |  |  |
| CellId | NBIOT_CellId_Type |  |  |
| RoutingInfo | NB_RoutingInfo_Type |  |  |
| TimingInfo | TimingInfo_Type |  |  |
| Status | IndicationStatus_Type |  |  |

# F.7	NBIOT_Imported_EUTRA_ASN1_Types

NBIOT_Imported_EUTRA_ASN1_Types: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| EUTRA_ASN1_DRB_Identity_Type | DRB_Identity |  |
| EUTRA_ASN1_PhysCellId_Type | PhysCellId |  |
| EUTRA_ASN1_C_RNTI_Type | C_RNTI |  |
| EUTRA_ASN1_CipheringAlgorithm_r12_Type | CipheringAlgorithm_r12 |  |
| EUTRA_ASN1_Dl_Bandwidth_Type | CarrierBandwidthEUTRA.dl_Bandwidth |  |

# F.8	NBIOT_ASP_VirtualNoiseDefs

ASP definitions for virtual noise generation in NBIOT cells.
The noise is configured for an already existing NBIOT cell.
For UEs with 2 antenna connectors the AWGN (Additive white Gaussian noise) signals applied to each receiver antenna connector shall be uncorrelated.

NBIOT_ASP_VirtualNoiseDefs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NBIOT_VngConfigConfirm_Type | Null_Type |  |

NBIOT_VngConfigInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NBIOT_VngConfigInfo_Type |  |  |
| Comment |  |  |  |
| NocLevel | integer |  | Noc level; calculation is according to 36.523-3 cl 7.22 |

NBIOT_VngConfigRequest_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NBIOT_VngConfigRequest_Type |  |
| Comment | configure/activate noise for a given cell; NOTE: it is assumed the the associated NBIOT cell has been created beforehand |  |
| Configure | NBIOT_VngConfigInfo_Type | configuration of the virtual noise generator; regardless of the power level the noise generator is off before it gets activated for this cell; in case the configuration needs to be changed during a test, the noise generator shall be deactivated for this cell |
| Activate | Null_Type | noise is activated (switched on) for the given cell acc. to the previous configuration; while being active the configuration shall not be modified |
| Deactivate | Null_Type | deactivate noise for given cell |

NBIOT_VNG_CTRL_REQ

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NBIOT_VNG_CTRL_REQ |  |  |
| Comment |  |  |  |
| Common | NB_ReqAspCommonPart_Type |  | CellId : as for the associated NBIOT cell RoutingInfo : None TimingInfo : Now ControlInfo : CnfFlag:=true; FollowOnFlag:=false |
| Request | NBIOT_VngConfigRequest_Type |  |  |

NBIOT_VNG_CTRL_CNF

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NBIOT_VNG_CTRL_CNF |  |  |
| Comment |  |  |  |
| Common | NB_CnfAspCommonPart_Type |  | TimingInfo is ignored by TTCN (apart from EnquireTiming) => SS may set TimingInfo to "None" |
| Confirm | NBIOT_VngConfigConfirm_Type |  |  |

NBIOT_VNG_PORT

| TTCN-3 Port Type |  |  |
| --- | --- | --- |
| Name | NBIOT_VNG_PORT |  |
| Comment | NBIOT PTC: Port for virtual noise generator |  |
| out | NBIOT_VNG_CTRL_REQ |  |
| in | NBIOT_VNG_CTRL_CNF |  |

# F.9	CommonDefs

CommonDefs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| B1_Type | bitstring length(1) |  |
| B2_Type | bitstring length(2) |  |
| B3_Type | bitstring length(3) |  |
| B4_Type | bitstring length(4) |  |
| B5_Type | bitstring length(5) |  |
| B6_Type | bitstring length(6) |  |
| B7_Type | bitstring length(7) |  |
| B7_15_Type | bitstring length(7..15) | NOTE: length restriction can only be a range but not two destinct lengths |
| B8_Type | bitstring length(8) |  |
| B10_Type | bitstring length(10) |  |
| B11_Type | bitstring length(11) |  |
| B12_Type | bitstring length(12) |  |
| B14_Type | bitstring length(14) |  |
| B15_Type | bitstring length(15) |  |
| B16_Type | bitstring length(16) |  |
| B18_Type | bitstring length(18) |  |
| B32_Type | bitstring length(32) |  |
| B128_Type | bitstring length(128) |  |
| B128_Key_Type | B128_Type | 128 bit security key |
| O4_Type | octetstring length(4) |  |
| Null_Type | boolean (true) | dummy type for 'typeless' fields in unions |
| PdcpCountValue_Type | B32_Type |  |

IndicationAndControlMode_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | IndicationAndControlMode_Type |
| Comment |  |
| enable |  |
| disable |  |

# F.10	CommonAspDefs

## F.10.1	Cell_Configuration_Common

CellTimingInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | CellTimingInfo_Type |  |  |
| Comment | Cell Timing |  |  |
| TcOffset | integer (0..63) | opt | For NR according to TS 38.211 clause 4.1 Ts/Tc = 64 with Tc = 1/(480000 * 4096) and Ts = 1/(15000 * 2048) as for EUTRA; => for NR to specify granularity per Tc; for EUTRA to be set to 0 (and/or to be ignored by the SS) |
| Tcell | integer (0..307199) |  | frame duration Tf = 307200 * Ts = 10ms; System Time Unit Ts = 1/(15000 * 2048) |
| SfnOffset | integer (0..1023) |  |  |
| HsfnOffset | integer (0..1023) |  |  |

## F.10.2	MAC_Layer

TransmissionRepetition_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TransmissionRepetition_Type |  |
| Comment |  |  |
| Continuous | Null_Type |  |
| NumOfCycles | integer (1..infinity) |  |

RAR_RapIdCtrl_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RAR_RapIdCtrl_Type |  |
| Comment |  |  |
| Automatic | Null_Type | SS shall automatically use same RAPID as received from the UE |
| Unmatched | Null_Type | SS shall use RAPID being different from preamble sent by the UE; SS shall calculate this RAPID acc. to RAPID := (RAPID + 3..63) mod 64 if single RAR is transmitted in a MAC PDU then only 3 is added if multiple RAR's are transmitted in MAC PDU, then for first unmatched RAR 3 is added, second unmatched 4 is added, third unmatched 5 is added and so on |

## F.10.3	System_Indications

HARQ_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | HARQ_Type |
| Comment | ack represents HARQ ACK; nack represents HARQ NACK |
| ack |  |
| nack |  |

## F.10.4	ASP_CommonPart

Definition of ASP common parts for REQ-, CNF- and IND-ASPs

### F.10.4.1	ASP_CommonPart_Definitions

#### F.10.4.1.1	Routing_Info

CommonAspDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_SRB0 | integer | 0 |  |
| tsc_SRB1 | integer | 1 |  |

#### F.10.4.1.2	Timing_Info

Timing_Info: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| SystemFrameNumber_Type | integer (0..1023) |  |
| SubFrameNumber_Type | integer (0..9) |  |
| SymbolNumber_Type | integer (0..13) |  |
| HyperSystemFrameNumberInfo_Type | SystemFrameNumberInfo_Type |  |

SubFrameInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SubFrameInfo_Type |  |
| Comment |  |  |
| Number | SubFrameNumber_Type |  |
| Any | Null_Type | no specific sub-frame, in which case the SS may choose the next available subframe (valid for REQ ASPs only) |

SystemFrameNumberInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SystemFrameNumberInfo_Type |  |
| Comment |  |  |
| Number | SystemFrameNumber_Type |  |
| Any | Null_Type | no specific frame number, in which case the SS may choose the next available SFN (valid for REQ ASPs only) |

SlotOffset_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SlotOffset_Type |  |
| Comment | slots per subframe according to TS 38.211 Table 4.3.2-1 |  |
| Numerology0 | Null_Type | mu=0; only one slot per subframe |
| Numerology1 | integer (0..1) | mu=1; 2 slots per subframe |
| Numerology2 | integer (0..3) | mu=2; 4 slots per subframe |
| Numerology3 | integer (0..7) | mu=3; 8 slots per subframe |
| Numerology4 | integer (0..15) | mu=4; 16 slots per subframe |

SlotTimingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SlotTimingInfo_Type |  |
| Comment | EUTRA, NBIOT: REQ ASPs: TTCN shall set the SlotTimingInfo to "FirstSlot" for EUTRA, NBIOT addressing the whole subframe IND ASPs: TTCN shall ignore the SlotTimingInfo sent by the SS for EUTRA, NBIOT NR: REQ ASPs: Any:=true only if the slot number is not relevant, in which case the SS may choose the next available slot of the given subframe IND ASPs: Any:=true only if there is no slot information available for the particular kind of indication |  |
| SlotOffset | SlotOffset_Type | to address a particular slot in a subframe |
| FirstSlot | Null_Type | to address the first slot independent from the numerology (REQ ASPs only) or for REQ ASPs in EUTRA and NBIOT |
| Any | Null_Type | for IND ASPs in EUTRA and NBIOT or if slot number is not relevant or not available |

SymbolTimingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SymbolTimingInfo_Type |  |
| Comment | EUTRA, NBIOT: REQ ASPs: TTCN shall set the SymbolTimingInfo to "FirstSymbol" for EUTRA, NBIOT addressing the whole subframe IND ASPs: TTCN shall ignore the SymbolTimingInfo sent by the SS for EUTRA, NBIOT NR: REQ ASPs: Any:=true only if the symbol number is not relevant, in which case the SS may choose the next available symbol of the given slot IND ASPs: Any:=true only if there is no symbol information available for the particular kind of indication |  |
| SymbolOffset | SymbolNumber_Type | to address a particular symbol in a slot |
| FirstSymbol | Null_Type | to address the first symbol (REQ ASPs only) or for REQ ASPs in EUTRA and NBIOT |
| Any | Null_Type | for IND ASPs in EUTRA and NBIOT or if symbol number is not relevant or not available |

SubFrameTiming_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SubFrameTiming_Type |  |  |
| Comment |  |  |  |
| SFN | SystemFrameNumberInfo_Type |  |  |
| Subframe | SubFrameInfo_Type |  |  |
| HSFN | HyperSystemFrameNumberInfo_Type |  |  |
| Slot | SlotTimingInfo_Type |  |  |
| Symbol | SymbolTimingInfo_Type |  |  |

TimingInfo_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TimingInfo_Type |  |
| Comment |  |  |
| SubFrame | SubFrameTiming_Type |  |
| Now | Null_Type | to be used in REQ ASPs when there is no 'activation time' |
| None | Null_Type | only to be used in SYSTEM_CTRL_CNF or NR_SYSTEM_CTRL_CNF but not for EnquireTiming |

### F.10.4.2	REQ_ASP_CommonPart

ReqAspControlInfo_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | ReqAspControlInfo_Type |  |  |
| Comment |  |  |  |
| CnfFlag | boolean |  | true => SS shall send CNF: when the REQ is with no timing information (no activation time), SS shall send the confirmation when the configuration is done, i.e. when the test case may continue. Example: when there is a configuration followed by a send event it shall not be necessary to have a wait timer in between but the CNF triggers the send event. If there are other triggers e.g. like the UE sending a message, CnfFlag shall be set to false by the test case to avoid racing conditions with the CNF and the signalling message. When there is an activation time SS shall send the CNF after the configuration has been scheduled; that means SS shall not wait until the activation time has been expired. When FollowOnFlag is true, CnfFlag shall be set to false. |
| FollowOnFlag | boolean |  | false => no further (related) information true: further related information will be sent to SS (semantics depending on respective ASP) |

### F.10.4.3	CNF_ASP_CommonPart

ConfirmationResult_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | ConfirmationResult_Type |  |
| Comment |  |  |
| Success | Null_Type |  |
| Error | integer | may contain SS specific error code; this will not be evaluated by TTCN |

### F.10.4.4	IND_ASP_CommonPart

IntegrityErrorIndication_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | IntegrityErrorIndication_Type |  |  |
| Comment |  |  |  |
| Nas | boolean |  | NAS Integrity: set to true when received MAC does not match calculated MAC |
| Pdcp | boolean |  | PDCP Integrity: set to true when received MAC does not match calculated MAC |

ErrorIndication_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | ErrorIndication_Type |  |  |
| Comment |  |  |  |
| Integrity | IntegrityErrorIndication_Type |  | Integrity error: received MAC does not match calculated MAC |
| System | integer |  | any other error: may be SS specific error code; this will not be evaluated by TTCN; e.g. an error shall be raised when the UE requests retransmission of an RLC PDU |

IndicationStatus_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | IndicationStatus_Type |  |
| Comment |  |  |
| Ok | Null_Type |  |
| Error | ErrorIndication_Type |  |

# F.11	EUTRA_NB_ASP_TypeDefs

Type definitions for configuration of the system simulator;
Common design principles:
Semantics of OMIT: for all TTCN-3 type definitions used in ASPs omit means "keep as it is" =>
- on initial configuration in general all fields shall be provided
- no default values for fields are foreseen
- if necessary non-existence of information shall be explicitly configured
  (e.g. with a union of "no configuration" and "configuration parameters"
- fields within structures imported from the core spec are excepted from this rule

## F.11.1	Cell_Configuration

Specific Info for Cell Configuration Primitive

### F.11.1.1	Cell_Configuration_Common

EUTRA_NB_ASP_TypeDefs: Constant Definitions

| TTCN-3 Basic Types |  |  |  |
| --- | --- | --- | --- |
| tsc_CellAttenuation_Off | Attenuation_Type | {Off:=true} |  |

Cell_Configuration_Common: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| InitialAttenuation_Type | Attenuation_Type (tsc_CellAttenuation_Off) | Attenuation restricted to 'Off' |

Attenuation_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | Attenuation_Type |  |
| Comment | attenuation of the reference power |  |
| Value | integer (0..149) | cell power reference power reduced by the given attenuation (value is in dB) |
| Off | Null_Type | even though in TS 36.508 -145dBm for LTE and -150dBm for NB-IoT is given for a non suitable cell we specify an explicit "Off" value here |

InitialCellPower_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | InitialCellPower_Type |  |  |
| Comment |  |  |  |
| MaxReferencePower | AbsoluteCellPower_Type |  | maximum value of cell reference power (RS EPRE in dBm/15kHz as per TS 36.508, clause 4.3.4.1); a cell is initialised with this reference power; its value is the upper bound of the cell power during the test case |
| Attenuation | InitialAttenuation_Type |  | initial attenuation |

EUTRA_CyclicPrefix_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | EUTRA_CyclicPrefix_Type |
| Comment | NOTE: in DL extended cyclic prefix depends on sub-carrier spacing |
| normal |  |
| extended |  |

### F.11.1.2	Downlink_Physical_Layer_Configuration

Downlink_Physical_Layer_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| WUS_Durations_Type | integer (1,2,4,8,16,32,64,128,256,512,1024) | In 36.213 Table 16.9-1 in cl 16.9 for NB-IoT and Table 17-1 in cl 17 for BL/CE UE, values are given in subframes |
| TimeOffsetInFrames | integer (4,8,16,24,100,200) | Corresponds to timeOffsetDRX/timeOffset-eDRX-Short-r15/ timeOffset-eDRX-Long-r15 as defined in WUS-Config-NB-r15 for NB-IoT or WUS-Config-r15 for BL/CE UE |

WUS_PowerBoost_Type

| TTCN-3 Enumerated Type |  |
| --- | --- |
| Name | WUS_PowerBoost_Type |
| Comment |  |
| db0 |  |
| dB1dot8 |  |
| dB3 |  |
| dB4dot8 |  |

### F.11.1.3	Uplink_Physical_Layer_Configuration

SS_TimingAdvanceConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | SS_TimingAdvanceConfig_Type |  |
| Comment |  |  |
| InitialValue | RACH_TimingAdvance_Type | initial value corresponding to what is sent to the UE in RACH response (range acc. 11 bit value; 0 in normal cases) |
| Relative | TimingAdvanceIndex_Type | timing advance command to adjust changes of timing advance acc. to TS 36.213, clause 4.2.3; (range acc. 6 bit value: -31..32) |
| NtnTA | NTN_TimingAdvanceConfig_Type | timing advance configuration for NTN operation |

### F.11.1.4	Common_MAC_Configuration

Transport channel and MAC related procedures and configuration

Common_MAC_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| TimingAdvanceIndex_Type | integer (0..63) | acc. to TS 36.321, clause 6.1.3.5 "Timing Advance Command MAC Control Element" and TS 36.213, clause 4.2.3 "Transmission timing adjustments" |
| TimingAdvance_Period_Type | integer (400, 600, 1020, 1530, 2040, 4090, 8190) | the values correspond to 80 % of TimeAlignmentTimer (acc. to TS 36.523-3, clause 7.2) (TS 36.331, clause 6.3.2: sf500, sf750, sf1280, sf1920, sf2560, sf5120, sf10240) rounded to nearest multiple of 10 |

UplinkTimeAlignment_AutoSynch_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | UplinkTimeAlignment_AutoSynch_Type |  |  |
| Comment |  |  |  |
| TimingAdvance | TimingAdvanceIndex_Type |  |  |
| TA_Period | TimingAdvance_Period_Type |  | time period after which TA MAC control elements need to be automatically transmitted |
| TA_Repetition | TransmissionRepetition_Type |  | number of TA MAC control element repetitions to be automatically transmitted or 'Continuous' |

UplinkTimeAlignment_Synch_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | UplinkTimeAlignment_Synch_Type |  |
| Comment |  |  |
| None | Null_Type | no PUCCH Synchronisation applied |
| Auto | UplinkTimeAlignment_AutoSynch_Type | SS automatically maintains PUCCH synchronization at UE If the cell is a - Rel 10 or earlier cell - or Rel 11 or later Pcell - or the Rel-11 or later scell with no STAG-ID configured, the TAG-ID is set to '00' i.e. P-TAG in Timing advance MCE If the cell is Rel 11 scell with STAG-ID configured, the configured stag-ID is used as TAG-ID in Timing advance MCE |

### F.11.1.5	Random_Access_Procedure

Random_Access_Procedure: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| RACH_TimingAdvance_Type | integer (0..2047) | 11 bit timing advance as used in RACH response (absolute value) |

ContentionResolution_ContainedDlschSdu_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | ContentionResolution_ContainedDlschSdu_Type |  |
| Comment |  |  |
| RlcPduCCCH | octetstring | octetstring of an RLC PDU containing e.g. the RRC Connection Setup; to be sent in the same MAC PDU as the MAC Contention Resolution Control Element (Msg4) |
| PdcpSduDCCH | octetstring | octetstring of a PDCP SDU containing e.g. the RRC Connection Resume; to be sent in the same MAC PDU as the MAC Contention Resolution Control Element (Msg4); the SS shall - apply integrity protection, - add a PDCP header accordingly, - add an AM RLC header, according to TS 36.523-3 clause 7A.8a and clause 7A.13 The logical channel id to be used for the MAC DL-SCH SDU shall be '00001'B corresponding to SRB1 |
| None | Null_Type | MAC PDU containing the MAC Contention Resolution Control Element does not contain an RLC PDU (i.e. RRC Connection Setup is sent in another PDU) |

ContentionResolution_ContainedId_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | ContentionResolution_ContainedId_Type |  |
| Comment |  |  |
| XorMask | ContentionResolutionId_Type | When SS receives Contention Resolution ID from the UE, SS shall XOR it with the given mask and use this as Contention Resolution ID; this allows to get an unmatching Contention Resolution ID; in normal cases mask shall be set to tsc_ContentionResolutionId_Unchanged (i.e. the Contention Resolution ID remains unchanged) |
| None | Null_Type | MAC Contention Resolution Control Element is not contained in the MAC PDU sent out as response on Msg3 |

TCRNTI_ContentionResolutionMacPdu_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | TCRNTI_ContentionResolutionMacPdu_Type |  |  |
| Comment | NOTE: Either ContainedId or ContainedRlcPdu (or both) shall not be 'none'; (if no Contention Resolution Mac Pdu shall be sent, TCRNTI_ContentionResolutionCtrl_Type.NoContResolID shall be used instead) |  |  |
| ContainedId | ContentionResolution_ContainedId_Type |  | Either the Contention Resolution ID as received from the UE or a modified Contention Resolution ID (XorMask != tsc_ContentionResolutionId_Unchanged) or no Contention Resolution ID at all |
| ContainedRlcPdu | ContentionResolution_ContainedDlschSdu_Type |  | the MAC PDU containing the MAC Contention Resolution Control Element may contain the RRC Connection Setup; in this case the RRC PDU shall be completely encoded been contained in an RLC PDU |

TCRNTI_ContentionResolutionCtrl_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TCRNTI_ContentionResolutionCtrl_Type |  |
| Comment | when the UE responds on a Random Access Response with a RRC Connection Request on CCCH and not with a C-RNTI SS shall assume initial Random Access Procedure (TS 36.300, clause 10.1.5.1), i.e. sends a ContentionResolutionId back to the UE |  |
| MacPdu | TCRNTI_ContentionResolutionMacPdu_Type | MAC PDU containing the Contention Resolution ID and optionally an RRC PDU (RRC Connection Setup) |
| MacPdu_CRC_Error | TCRNTI_ContentionResolutionMacPdu_Type | same as MacPdu (see above), but SS shall generate CRC error by toggling CRC bits; no retransmissions shall be made as UE shall not send a NACK |
| NoContResolID | Null_Type | SS shall not include contention resolution ID (i.e. no MAC PDU shall be sent); used for contention resolution fail case |

TempC_RNTI_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | TempC_RNTI_Type |  |
| Comment |  |  |
| SameAsC_RNTI | Null_Type | in the RA response SS shall use the same C-RNTI as configured in ActiveCellConfig_Type; this is useful for initial random access |
| Explicit | C_RNTI | in the RA response SS shall use different value as configured in ActiveCellConfig_Type; this can be used when the UE already is in RRC_CONNECTED to have a temporary C-RNTI different from the one used by the UE; NOTE: when the UE is not in RRC_CONNECTED there shall be no explicit temp. C-RNTI since then the UE would assume this value as C-RNTI |

RandomAccessBackoffIndicator_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | RandomAccessBackoffIndicator_Type |  |
| Comment |  |  |
| None | Null_Type | normal case, no back off indicator included |
| Index | integer (0..15) | Backoff Parameter values acc. TS 36.321, clause 7.2; values 0..12 are defined, 13..15 may be used in error case |

### F.11.1.6	NTN_Configuration

NTN_Configuration: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| NTN_T_TA_Type | ServingSatelliteInfo_r17.nta_CommonParameters_r17.nta_Common_r17 | acc. to TS 36.331 clause 6.3.1 |

NTN_SemistaticTA_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | NTN_SemistaticTA_Type |  |  |
| Comment | see Table 7A.14.2.2-1 for further information |  |  |
| InitialValue | RACH_TimingAdvance_Type |  | initial value corresponding to what is sent to the UE in RACH response; (range acc. 11 bit value; 0 in normal cases) |
| NtnCommonTA | NTN_T_TA_Type |  | round trip time (RTT) between the UL time synchronisation reference point and the serving satellite |
| NtnUeSpecificTA | NTN_T_TA_Type |  | round trip time (RTT) between the UE and the serving satellite |

NTN_TimingAdvanceConfig_Type

| TTCN-3 Union Type |  |  |
| --- | --- | --- |
| Name | NTN_TimingAdvanceConfig_Type |  |
| Comment |  |  |
| Semistatic | NTN_SemistaticTA_Type | semi-static configuration of the NTN cell's Timing Advance (TA); SS shall not autonomously change the value of TA over time; TTCN may re-configure the TA value |

# F.12	EUTRA_NB_CommonDefs

## F.12.1	Common_Types

Common_Types: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| LogicalChannelId_Type | integer (0..10) | acc. TS 36.331, clause 6.3.2 and clause 6.7.3.2 for DRBs DTCH-LogicalChannelIdentity is INTEGER (3..10); additionally we have 0..2 for the SRBs |
| HarqProcessId_Type | integer (0..14) | The values 0..7 represent the ID of HARQ process ID; value range 0..14 is for TDD |
| ContentionResolutionId_Type | bitstring length(48) | used in EUTRA_ASP_DrbDefs and EUTRA_ASP_Typedefs |
| AbsoluteCellPower_Type | integer (-150..0) | absolute cell power (dBm) |

## F.12.2	RRC_Nested_Types

RRC_Nested_Types: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| IntegrityProtAlgorithm_Type | SecurityAlgorithmConfig.integrityProtAlgorithm |  |

## F.12.3	L2Data_CommonDefs

Common definitions of L2 control elements used in L2 messages (EUTRA_NB_ASP_L2DataDefs) as well as for SS configuration (EUTRA_ASP_TypeDefs)

L2Data_CommonDefs: Basic Type Definitions

| TTCN-3 Basic Types |  |  |
| --- | --- | --- |
| MAC_CTRL_ScellActDeact_Type | ScellBitMap_Type | 36.321 clause 6.1.3.8 |

ScellBitMap_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | ScellBitMap_Type |  |  |
| Comment |  |  |  |
| Value | B7_Type |  | 7 MSB bits the C Fields C7 to C1. 1 => the corresponding Scell is Active 0 => Inactive |
| Reserved | B1_Type |  | LSBit Reserved. Shall be set to 0 |

PH_Record_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | PH_Record_Type |  |  |
| Comment | 36.321 clause 6.1.3.6a |  |  |
| P_Bit | B1_Type |  | P bit: 1 indicates the UE applies power backoff due to power management |
| V_Bit | B1_Type |  | V bit: Indicates if the PH value is based on a real transmission or a reference format. For Type 1 PH, V=0 indicates real transmission on PUSCH and V=1 indicates that a PUSCH reference format is used |
| Valve | B6_Type |  | The power headroom level. Ph Type 2 (if configured) for Pcell and Type 1 for Pcell and Scell |
| Reserved | B2_Type | opt | 2 reservid bits. Present if V=1 |
| PCMaxc | B6_Type | opt | Present if V=1 |

PH_RecordList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | PH_RecordList_Type |
| Comment | If simultaneousPUCCH-PUSCH is not configured at least oneType 1 PH record for Pcell is present. Additional one record per Scell for which respective 'C' bit is set as 1. If simultaneousPUCCH-PUSCH is configured then one PH Type 2 record for P cell followed by PH Type 1 record for P cell is present. Additional one record per Scell for which respective 'C' bit is set as 1 |
| record length(1..9) of PH_Record_Type |  |

MAC_CTRL_ExtPowerHeadRoom_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_CTRL_ExtPowerHeadRoom_Type |  |  |
| Comment |  |  |  |
| EPH_Octet1 | ScellBitMap_Type |  |  |
| PH_RecordList | PH_RecordList_Type |  | At least one record for Pcell is present. Additional one record per Scell for which respective 'C' bit is set as 1 |

DC_PH_RecordList_Type

| TTCN-3 Record of Type |  |
| --- | --- |
| Name | DC_PH_RecordList_Type |
| Comment | If simultaneousPUCCH-PUSCH is not configured at least oneType 1 PH record for Pcell is present. Additional one record per PScell/Scell for which respective 'C' bit is set as 1. If simultaneousPUCCH-PUSCH is configured then one PH Type 2 record for Pcell followed one PH Type 2 record for PScell followed by PH Type 1 record for Pcell is present. Additional one record per PSCell/Scell for which respective 'C' bit is set as 1 |
| record length(1..10) of PH_Record_Type |  |

MAC_CTRL_DC_PowerHeadRoom_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_CTRL_DC_PowerHeadRoom_Type |  |  |
| Comment |  |  |  |
| DC_PH_Octet1 | ScellBitMap_Type |  |  |
| DC_PH_RecordList | DC_PH_RecordList_Type |  | At least one record for Pcell is present. Additional one record per PScell/Scell for which respective 'C' bit is set as 1 |

MAC_CTRL_DifferentialKoffset_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_CTRL_DifferentialKoffset_Type |  |  |
| Comment | TS 36.321, clause 6.1.3.22 |  |  |
| Reserved | B2_Type |  | 2 bits are reserved |
| Value | B6_Type |  | the differential Koffset in subframes |

MAC_CTRL_TimingAdvanceReport_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_CTRL_TimingAdvanceReport_Type |  |  |
| Comment | TS 36.321, clause 6.1.3.20 |  |  |
| Reserved | B2_Type |  | 2 bits are reserved |
| Value | B14_Type |  | the least integer number of subframes greater than or equal to the Timing Advance value |

MAC_CTRL_GNSS_Meas_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_CTRL_GNSS_Meas_Type |  |  |
| Comment | TS 36.321, clause 6.1.3.22 |  |  |
| Type | B1_Type |  | a flag indicating UE is triggered to perform GNSS measurement or just a GNSS measurement gap length is provided for UE-autonomous measurement. "0" indicates UE needs to perform GNSS measurement with the GNSS measurement gap length configured in this MAC CE. "1" indicates the GNSS measurement gap length configured in this MAC CE needs to be stored and used for the subsequent UE-autonomous GNSS measurement |
| Reserved | B3_Type |  | 3 reserved bits, set to 0 |
| Value | B4_Type |  | the configuration of GNSS measurement gap length as shown in TS 36.321 Table 6.1.3.22-1 |

MAC_CTRL_GNSS_ValidityDuration_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | MAC_CTRL_GNSS_ValidityDuration_Type |  |  |
| Comment | TS 36.321, clause 6.1.3.23 |  |  |
| Reserved | B4_Type |  | 4 reserved bits, set to 0 |
| Value | B4_Type |  | corresponds to the remaining GNSS validity duration defined in TS 36.331. |

## F.12.4	SC_PTM_Types

SC_MRB_Identity_Type

| TTCN-3 Record Type |  |  |  |
| --- | --- | --- | --- |
| Name | SC_MRB_Identity_Type |  |  |
| Comment |  |  |  |
| MbmsSessionInfo | MBMSSessionInfo_r13 |  |  |

# F.13	References to TTCN-3

| References to TTCN-3 |  |  |
| --- | --- | --- |
| NBIOT_ASP_TypeDefs | Common/NBIOT_ASP_TypeDefs.ttcn | Rev 42246 |
| NBIOT_ASP_SrbDefs | Common/NBIOT_ASP_SrbDefs.ttcn | Rev 20533 |
| NBIOT_ASP_L2DataDefs | Common/NBIOT_ASP_L2DataDefs.ttcn | Rev 20533 |
| EUTRA_NB_ASP_L2DataDefs | Common4G/EUTRA_NB_ASP_L2DataDefs.ttcn | Rev 40476 |
| NasEmu_AspTypes_NBIOT | NasEmulation/NasEmu_AspTypes_NBIOT.ttcn | Rev 17143 |
| NBIOT_CommonDefs | Common/NBIOT_CommonDefs.ttcn | Rev 41723 |
| NBIOT_Imported_EUTRA_ASN1_Types | Common/NBIOT_Imported_EUTRA_ASN1_Types.ttcn | Rev 40020 |
| NBIOT_ASP_VirtualNoiseDefs | Common/NBIOT_ASP_VirtualNoiseDefs.ttcn | Rev 20533 |
| CommonDefs | Common/CommonDefs.ttcn | Rev 42165 |
| CommonAspDefs | Common/CommonAspDefs.ttcn | Rev 39143 |
| EUTRA_NB_ASP_TypeDefs | Common4G/EUTRA_NB_ASP_TypeDefs.ttcn | Rev 39067 |
| EUTRA_NB_CommonDefs | Common4G/EUTRA_NB_CommonDefs.ttcn | Rev 41990 |