3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250426

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3760 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR NTN testcase 7.1.2.3.9a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to current TTCN implement, at step 3, the function f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, p_RLC_Rec.t_ReassemblyExt_r17) adds extra 10% duration to t_ReassemblyExt_r17(2200ms), the actual value of delay is 2420ms, it is not consistent with the requirement of step3 in Table 7.1.2.3.9a.3.2-1 in 38.523-1; also, UE may sends STATUS PDU(Step 4) after t_ReassemblyExt_r17(2200ms) expired but before the 2420ms timeout, it results in failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | At step3, remove f_NR_SetTimerToleranceMax(), subtract 2.0 from t_ReassemblyExt_r17 directly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE shall fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.9a.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: the wait time of 2 seconds provides sufficient tolerance for t_ReassemblyExt_r17 of 2.2 seconds |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	1

1	Overview	3

2	Corrections required to test case 7.1.2.3.9a	4

2.1	Change 1	4

# Overview

This document lists all the essencial changes needed to correct problems in the ATS iwd-TTCN3-B2025-10_D25wk38 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

# Corrections required to test case 7.1.2.3.9a

## Change 1

| Function name | f_TC_7_1_2_3_9a_NR_TestBody () |
| --- | --- |
| Reason for change | According to current TTCN implement, at step 3, the function f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, p_RLC_Rec.t_ReassemblyExt_r17) adds extra 10% duration to t_ReassemblyExt_r17(2200ms), the actual value of delay is 2420ms, it is not consistent with the requirement of step3 in Table 7.1.2.3.9a.3.2-1 in 38.523-1; also, UE may sends STATUS PDU(Step 4) after t_ReassemblyExt_r17(2200ms) expired but before the 2420ms timeout, it results in failure. |
| Summary of change | At step3, remove f_NR_SetTimerToleranceMax(), subtract 2.0 from t_ReassemblyExt_r17 directly. |
| TTCN module | RLC_NTN_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_7_1_2_3_9a_NR_TestBody(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { // NR NTN / AM RLC / Reassembling of AMD PDUs var integer v_RLC_Len1stSeg := 7; var integer v_RLC_Len2ndSeg := 5; var integer v_RLC_SDUsize := v_RLC_Len1stSeg + v_RLC_Len2ndSeg; // = 12 var template (value) NR_RLC_Status_NackListSN18Bit_Type v_NR_NackSN18Bit_List; var template (present) NR_RLC_AM_StatusPDU_Type v_NR_RLC_StatusPDU; var template (present) NR_RLC_PDUList_Type v_NR_RLC_PDUList; var NR_RLC_DataFieldList_Type v_NR_RLC_DataList; timer t_WatchDog; // generate 1 RLC SDUs = PDCP PDUs for transmission v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 1); // @siclog "Step 1" siclog@ // The SS transmits AMD PDU#1 containing the last segment (5 bytes) of RLC SDU#1 (SI field=10, SO=7). f_NR_TxAMD_PDUpartofSDU (p_RLC_Rec, p_NR_DRB_Id, cs_TimingInfo_Now, v_NR_RLC_DataList[tsc_RLC_SDU1], -, v_RLC_Len1stSeg, v_RLC_Len2ndSeg, tsc_NR_SI_EndOfSDU, true); p_RLC_Rec.AM_RX_Next_Ack := 1; // @sic R5s250100 ch. 13.1 sic@ // @siclog "Step 2" siclog@ // Check: For 2 sec after step 1, does the UE transmit an RLC STATUS PDU with ACK_SN=1, NACK_SN=0 with SOStart=0 and SOEnd=6? v_NR_NackSN18Bit_List := { crs_NR_RLC_NACK_SN18_SOs(0, 0, v_RLC_Len1stSeg-1) }; v_NR_RLC_StatusPDU := f_NR_RLC_Get_STATUS_PDU_RX(p_RLC_Rec, v_NR_NackSN18Bit_List); v_NR_RLC_PDUList := { cr_NR_RLC_PDU_Status(v_NR_RLC_StatusPDU) }; t_WatchDog.start(2.0); // @sic R5-251943 sic@ alt { [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_NR_RLC_PDUList)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } [] t_WatchDog.timeout {} } // @siclog "Step 3" siclog@ // Wait for t-ReassemblyExt-r17 from step 1 to ensure that t-ReassemblyExt-r17 for the AMD PDU#1 expires at UE side. f_Delay(f_NR_SetTimerToleranceMax (nr_Cell1, l2Timer, p_RLC_Rec.t_ReassemblyExt_r17)-2.0); // 2 sec of step 2 have already elapsed ... @sic R5s250274 sic@ // @siclog "Step 4" siclog@ // Check: For 2 sec after step 3, does the UE transmit an RLC STATUS PDU with ACK_SN=1, NACK_SN=0 with SOStart=0 and SOEnd=6? (Note 2) // @sic R5-250100 ch. 13.2 sic@ t_WatchDog.start(2.0); // @sic R5-250100 ch. 13.3, R5-251943 sic@ alt { [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_NR_RLC_PDUList)) { t_WatchDog.stop; // @sic R5-250100 ch. 13.4 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } [] t_WatchDog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } } } }

After Change:

function f_TC_7_1_2_3_9a_NR_TestBody(NR_RLC_SS_State_Type p_RLC_Rec, DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC { // NR NTN / AM RLC / Reassembling of AMD PDUs var integer v_RLC_Len1stSeg := 7; var integer v_RLC_Len2ndSeg := 5; var integer v_RLC_SDUsize := v_RLC_Len1stSeg + v_RLC_Len2ndSeg; // = 12 var template (value) NR_RLC_Status_NackListSN18Bit_Type v_NR_NackSN18Bit_List; var template (present) NR_RLC_AM_StatusPDU_Type v_NR_RLC_StatusPDU; var template (present) NR_RLC_PDUList_Type v_NR_RLC_PDUList; var NR_RLC_DataFieldList_Type v_NR_RLC_DataList; timer t_WatchDog; // generate 1 RLC SDUs = PDCP PDUs for transmission v_NR_RLC_DataList := fl_NR_GenerateRLC_AM_SDUs(p_RLC_Rec, v_RLC_SDUsize, 1); // @siclog "Step 1" siclog@ // The SS transmits AMD PDU#1 containing the last segment (5 bytes) of RLC SDU#1 (SI field=10, SO=7). f_NR_TxAMD_PDUpartofSDU (p_RLC_Rec, p_NR_DRB_Id, cs_TimingInfo_Now, v_NR_RLC_DataList[tsc_RLC_SDU1], -, v_RLC_Len1stSeg, v_RLC_Len2ndSeg, tsc_NR_SI_EndOfSDU, true); p_RLC_Rec.AM_RX_Next_Ack := 1; // @sic R5s250100 ch. 13.1 sic@ // @siclog "Step 2" siclog@ // Check: For 2 sec after step 1, does the UE transmit an RLC STATUS PDU with ACK_SN=1, NACK_SN=0 with SOStart=0 and SOEnd=6? v_NR_NackSN18Bit_List := { crs_NR_RLC_NACK_SN18_SOs(0, 0, v_RLC_Len1stSeg-1) }; v_NR_RLC_StatusPDU := f_NR_RLC_Get_STATUS_PDU_RX(p_RLC_Rec, v_NR_NackSN18Bit_List); v_NR_RLC_PDUList := { cr_NR_RLC_PDU_Status(v_NR_RLC_StatusPDU) }; t_WatchDog.start(2.0); // @sic R5-251943 sic@ alt { [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_NR_RLC_PDUList)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 2"); } [] t_WatchDog.timeout {} } // @siclog "Step 3" siclog@ // Wait for t-ReassemblyExt-r17 from step 1 to ensure that t-ReassemblyExt-r17 for the AMD PDU#1 expires at UE side. f_Delay(p_RLC_Rec.t_ReassemblyExt_r17-2.0); // 2 sec of step 2 has already elapsed // @siclog "Step 4" siclog@ // Check: For 2 sec after step 3, does the UE transmit an RLC STATUS PDU with ACK_SN=1, NACK_SN=0 with SOStart=0 and SOEnd=6? (Note 2) // @sic R5-250100 ch. 13.2 sic@ t_WatchDog.start(2.0); // @sic R5-250100 ch. 13.3, R5-251943 sic@ alt { [] DRB.receive (car_NR_DRB_COMMON_IND_RLC_PDUList(nr_Cell1, p_NR_DRB_Id, -, v_NR_RLC_PDUList)) { t_WatchDog.stop; // @sic R5-250100 ch. 13.4 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } [] t_WatchDog.timeout { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } } } }