3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250451

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3761 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR NTN testcase 8.1.5.14.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In the current implementation of f_NR_SetSysinfoCombinationToNR_1, the list of SIs is set to omit, it means that the SI list is empty and stop broadcasting SIB19.so clear the SiSchedulList to meet the requirement. - According to step 23 and step 28 of Table 8.1.5.14.1.3.2-1 in 38.523-1, TC requires to stop and resume SIB19; but the current TTCN implementation, the function f_NR_ModifySysinfo() is for changing SIBs message and taking the BCCH modification period length into account. It takes more time to stop SIB19. To meet the requirement of TC procedure, SS need to stop broadcasting SIB19 immediately. - From step 22 to step 26 in Table 8.1.5.14.1.3.2-1 of 38.523-1, SS start T430(20s) and wait until expiry of T430, in the meantime at step 25, SS sends DL data to UE.That means UE triggers SR to loopback the DL data. With current settings, for NGSO mode,the duration of SR is 10ms * 64 = 640ms, it is too short to meet the requirement of testing. UE send Prach before T430 timeout, So enlarge the duration of SRs for testing.A prose CR will be submitted in coming RAN5 for discussion.. - The current TTCN implementation does not consider the scenario abut if the UE requires an external trigger to establish a PDU session(pc_noOf_PDUsSameConnection == 0). - According to step 23 and step 28 of Table 8.1.5.14.1.3.2-1 in 38.523-1, TC requires to stop and resume SIB19; The current TTCN implementation, function f_NR_ModifySysinfo() is for changing SIBs message and taking the BCCH modification period length into account. It takes more time to stop SIB19. To meet the requirement of TC procedure, SS need to stop broadcasting SIB19 immediately. - From step 22 to step 26 in Table 8.1.5.14.1.3.2-1 of 38.523-1, SS start T430(20s) and wait until expiry of T430, in the meantime at step 25, SS sends DL data to UE.UE will trigers SR to loopback DL data. With current TTCN, for NGSO,SS sends the DL data too early, UE reaches the Sr_TransMax(n64) before T430 timeout, then UE sends Prach and TC failed.To meet testing requirement, need to postpone sending DL data (step 25) about 18s to ensure as close as the T430 timeout to avoid UE trigers Prach procedure. - According to the Table 4.4.3.1.2-3 in 38.508-1, for SIB combination of NR-29, SIB1,SIB2 and SIB19 are included, but the implementation of the function f_NR_SetSysinfoCombinationToNR_29() does not include SIB2, replace it with f_NR_ModifySysinfoCombinationToNR_29() to meet the specification requirements. Resume broadcast SIB19 immediately, same as point 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clear schedulingInfoList in si_SchedulingInfo. - Add a new function f_NR_ModifySysinfo_SIB19() to stop SIB19 immediately. - Configure the sr_ProhibitTimer to ms64 to avoid UE send Prach early than T430 time out. - Add function f_NR5GC_RRC_Idle_Steps16_20 () to take care about the scenario on pc_noOf_PDUsSameConnection == 0. - Invoke new function f_NR_ModifySysinfo_SIB19() to stop broadcasting SIB19. - Add new timer t_BeforeStep24 of duration about 18s to postpond sending DL data at step 25. - Invoke f_NR_ModifySysinfoCombinationToNR_29() to include SIB1, SIB2 and SIB19 in schedulingInfoList. Invoke new function f_NR_ModifySysinfo_SIB19() to resume broadcasting SIB19. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE shall fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.14.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	1

1	Overview	1

2	Corrections required to test case 8.1.5.14.1	1

2.1	Change 1	1

2.2	Change 2	1

2.3	Change 3	1

2.4	Change 4	1

- Overview

This document lists all the essencial changes needed to correct problems in the ATS iwd-TTCN3-B2025-10_D25wk38 related to the title of this CR.

Contact:	Parikshit Bhise

parikshit.bhise@rohde-schwarz.com

- Corrections required to test case 8.1.5.14.1

  - Change 1

| Function name | fl_NR_InitialiseSiScheduling() |
| --- | --- |
| Reason for change | - In the current implementation of f_NR_SetSysinfoCombinationToNR_1, the list of SIs is set to omit, it means that the SI list is empty and stop broadcasting SIB19.so clear the SiSchedulList to meet the requirement. |
| Summary of change | - 1.Clear schedulingInfoList in si_SchedulingInfo. |
| TTCN module | NR_ConfigurationSteps.ttcn |
| MCC160 Comment | Not needed.The issue is handled in R5-256678. Please see MCC160 implementation below. |

Before Change:

function fl_NR_InitialiseSiScheduling(NR_CellId_Type p_NR_CellId, boolean p_BroadcastAllSI := false) runs on NR_BASE_PTC return template (omit) NR_AllOtherSiSchedul_Type { var template (omit) NR_AllOtherSiSchedul_Type v_AllOtherSiSchedul := omit; var NR_CellSysInfo_Type v_NR_CellSysInfo := f_NR_CellInfo_GetSysinfo(p_NR_CellId); var SI_SchedulingInfo.si_WindowLength v_SiWindowLength; var integer v_NoOfSIs; var integer v_BroadcastIndex; var integer i; var template (omit) NR_OtherSiSchedulList_Type v_SiSchedulList := omit; //@sic R5s210361 sic@ var template (omit) NR_OtherSiSchedulList_Type v_SegmentedSiSchedulList := omit; var template (omit) PosSiSchedulInfo_Type v_PosSiSchedulInfo := omit; var SI_SchedulingInfo v_SI_SchedulingInfo; var template (omit) SI_SchedulingInfo_v1700 v_SI_SchedulingInfo_v1700; //@sic R5-233379 sic@ var integer v_NoOfSIsSegmented; var NR_RedundancyVersion_Type v_RV := 0; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId); var integer v_SegmentedSiPosistionInSIB1; var integer v_LengthofSchedulingInfoList; //@sic R5-233379 sic@ var integer j :=0; //@sic R5-233379 sic@ var boolean v_SI_Brodacasted:= true; //@sic R5-233379 sic@ var integer v_SI_SlotOffset := f_NR_GetSI_SlotOffset(p_NR_CellId); select (v_NR_CellSysInfo.NR_SysinfoCombination) { case (MibOnly, NR_1) {} //do nothing - no SI case else { //Include SI and/or Segmented SI v_SI_SchedulingInfo := f_NR_GetSIB1_SiSchedulingInfo(p_NR_CellId); v_SiWindowLength := v_SI_SchedulingInfo.si_WindowLength; v_BroadcastIndex := 0; v_SI_SchedulingInfo_v1700 := f_NR_GetSIB1_SiSchedulingInfo_v1700(p_NR_CellId); if (isvalue(v_NR_CellSysInfo.BcchInfo.SIs)) { // SI included in the NR sysinfo combination @sic R5s210361 sic@ v_NoOfSIs := lengthof(v_NR_CellSysInfo.BcchInfo.SIs); v_LengthofSchedulingInfoList := lengthof(v_SI_SchedulingInfo.schedulingInfoList); for (i := 0; i < v_NoOfSIs; i:=i+1) { if ( i < v_LengthofSchedulingInfoList) { //@sic R5-233379 sic@ if (v_SI_SchedulingInfo.schedulingInfoList[i].si_BroadcastStatus== broadcasting) { v_SI_Brodacasted := true; } else { v_SI_Brodacasted := false; } } else { //SI added in the SI-SchedulingInfo-v1700 for (j:=0; j<lengthof(v_SI_SchedulingInfo_v1700.schedulingInfoList2_r17); j:=j+1) { if (valueof(v_SI_SchedulingInfo_v1700.schedulingInfoList2_r17[j].si_WindowPosition_r17) == i+1 ) { //@sic R5s240053 sic@ //Check against i+1 because si_WindowPosition_r17 defined from 1 (si-WindowPosition-r17 INTEGER (1..256)) and j defined from 0 v_SiSchedulList[v_BroadcastIndex].Periodicity := fl_ConvertSiPeriodicity(valueof(v_SI_SchedulingInfo_v1700.schedulingInfoList2_r17[j].si_Periodicity_r17)); if (valueof(v_SI_SchedulingInfo_v1700.schedulingInfoList2_r17[j].si_BroadcastStatus_r17) == broadcasting) { v_SI_Brodacasted := true; } else { v_SI_Brodacasted := false; } } } } if (v_SI_Brodacasted or p_BroadcastAllSI) { //@sic R5-201006 sic@ if ( i < v_LengthofSchedulingInfoList) { //@sic R5s240053 sic@ v_SiSchedulList[v_BroadcastIndex].Periodicity := v_SI_SchedulingInfo.schedulingInfoList[i].si_Periodicity; //@sic R5s230496 sic@ } //@sic R5-237429 sic@ v_SiSchedulList[v_BroadcastIndex].Window[0].SearchSpaceAndDci := cs_NR_SearchSpaceDlDciAssignment_CSSType0A(v_NR_PhysicalParameters, v_RV); //Only one repetition within the SI-window v_SiSchedulList[v_BroadcastIndex].Window[0].SlotOffset := v_SI_SlotOffset; v_BroadcastIndex := v_BroadcastIndex + 1; //@sic R5-201006 sic@ } } } if(isvalue(v_NR_CellSysInfo.BcchInfo.SegmentedSIs)){ //@sic R5s190425 sic@ v_NoOfSIsSegmented := lengthof(v_NR_CellSysInfo.BcchInfo.SegmentedSIs); for (i := 0; i < v_NoOfSIsSegmented; i:=i+1) { v_SegmentedSiPosistionInSIB1 := i + v_BroadcastIndex; v_SegmentedSiSchedulList[i].Periodicity := v_SI_SchedulingInfo.schedulingInfoList[v_SegmentedSiPosistionInSIB1].si_Periodicity; //@sic R5-237429 sic@ v_SegmentedSiSchedulList[i].Window[0].SearchSpaceAndDci := cs_NR_SearchSpaceDlDciAssignment_CSSType0A(v_NR_PhysicalParameters, v_RV); v_SegmentedSiSchedulList[i].Window[0].SlotOffset := v_SI_SlotOffset; } } if(isvalue(v_NR_CellSysInfo.BcchInfo.PosSIs)){ //@sic R5-222377 sic@ // p_BroadcastAllSI not handled inside the function fl_InitSS_PosSI_Scheduling as it is not needed so far: in all PosSI test case all posSIs are broadcasted v_PosSiSchedulInfo := fl_InitSS_PosSI_Scheduling(p_NR_CellId); } v_AllOtherSiSchedul := cs_NR_SiSchedulList_Def(v_SiWindowLength, v_SiSchedulList, v_SegmentedSiSchedulList, v_PosSiSchedulInfo); } } return v_AllOtherSiSchedul; }

After Change:

function fl_NR_InitialiseSiScheduling(NR_CellId_Type p_NR_CellId, boolean p_BroadcastAllSI := false) runs on NR_BASE_PTC return template (omit) NR_AllOtherSiSchedul_Type { var template (omit) NR_AllOtherSiSchedul_Type v_AllOtherSiSchedul := omit; var NR_CellSysInfo_Type v_NR_CellSysInfo := f_NR_CellInfo_GetSysinfo(p_NR_CellId); var SI_SchedulingInfo.si_WindowLength v_SiWindowLength; var integer v_NoOfSIs; var integer v_BroadcastIndex; var integer i; var template (omit) NR_OtherSiSchedulList_Type v_SiSchedulList := omit; //@sic R5s210361 sic@ var template (omit) NR_OtherSiSchedulList_Type v_SegmentedSiSchedulList := omit; var template (omit) PosSiSchedulInfo_Type v_PosSiSchedulInfo := omit; var SI_SchedulingInfo v_SI_SchedulingInfo; var template (omit) SI_SchedulingInfo_v1700 v_SI_SchedulingInfo_v1700; //@sic R5-233379 sic@ var integer v_NoOfSIsSegmented; var NR_RedundancyVersion_Type v_RV := 0; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId); var integer v_SegmentedSiPosistionInSIB1; var integer v_LengthofSchedulingInfoList; //@sic R5-233379 sic@ var integer j :=0; //@sic R5-233379 sic@ var boolean v_SI_Brodacasted:= true; //@sic R5-233379 sic@ var integer v_SI_SlotOffset := f_NR_GetSI_SlotOffset(p_NR_CellId); select (v_NR_CellSysInfo.NR_SysinfoCombination) { case (MibOnly, NR_1) { // WI = 1458048 if (not isvalue(v_NR_CellSysInfo.BcchInfo.SIs)) { v_SiSchedulList := {}; v_SiWindowLength := s80; v_AllOtherSiSchedul := cs_NR_SiSchedulList_Def(v_SiWindowLength, v_SiSchedulList, v_SegmentedSiSchedulList, v_PosSiSchedulInfo); } } //do nothing - no SI case else { //Include SI and/or Segmented SI v_SI_SchedulingInfo := f_NR_GetSIB1_SiSchedulingInfo(p_NR_CellId); v_SiWindowLength := v_SI_SchedulingInfo.si_WindowLength; v_BroadcastIndex := 0; v_SI_SchedulingInfo_v1700 := f_NR_GetSIB1_SiSchedulingInfo_v1700(p_NR_CellId); if (isvalue(v_NR_CellSysInfo.BcchInfo.SIs)) { // SI included in the NR sysinfo combination @sic R5s210361 sic@ v_NoOfSIs := lengthof(v_NR_CellSysInfo.BcchInfo.SIs); v_LengthofSchedulingInfoList := lengthof(v_SI_SchedulingInfo.schedulingInfoList); for (i := 0; i < v_NoOfSIs; i:=i+1) { if ( i < v_LengthofSchedulingInfoList) { //@sic R5-233379 sic@ if (v_SI_SchedulingInfo.schedulingInfoList[i].si_BroadcastStatus== broadcasting) { v_SI_Brodacasted := true; } else { v_SI_Brodacasted := false; } } else { //SI added in the SI-SchedulingInfo-v1700 for (j:=0; j<lengthof(v_SI_SchedulingInfo_v1700.schedulingInfoList2_r17); j:=j+1) { if (valueof(v_SI_SchedulingInfo_v1700.schedulingInfoList2_r17[j].si_WindowPosition_r17) == i+1 ) { //@sic R5s240053 sic@ //Check against i+1 because si_WindowPosition_r17 defined from 1 (si-WindowPosition-r17 INTEGER (1..256)) and j defined from 0 v_SiSchedulList[v_BroadcastIndex].Periodicity := fl_ConvertSiPeriodicity(valueof(v_SI_SchedulingInfo_v1700.schedulingInfoList2_r17[j].si_Periodicity_r17)); if (valueof(v_SI_SchedulingInfo_v1700.schedulingInfoList2_r17[j].si_BroadcastStatus_r17) == broadcasting) { v_SI_Brodacasted := true; } else { v_SI_Brodacasted := false; } } } } if (v_SI_Brodacasted or p_BroadcastAllSI) { //@sic R5-201006 sic@ if ( i < v_LengthofSchedulingInfoList) { //@sic R5s240053 sic@ v_SiSchedulList[v_BroadcastIndex].Periodicity := v_SI_SchedulingInfo.schedulingInfoList[i].si_Periodicity; //@sic R5s230496 sic@ } //@sic R5-237429 sic@ v_SiSchedulList[v_BroadcastIndex].Window[0].SearchSpaceAndDci := cs_NR_SearchSpaceDlDciAssignment_CSSType0A(v_NR_PhysicalParameters, v_RV); //Only one repetition within the SI-window v_SiSchedulList[v_BroadcastIndex].Window[0].SlotOffset := v_SI_SlotOffset; v_BroadcastIndex := v_BroadcastIndex + 1; //@sic R5-201006 sic@ } } } if(isvalue(v_NR_CellSysInfo.BcchInfo.SegmentedSIs)){ //@sic R5s190425 sic@ v_NoOfSIsSegmented := lengthof(v_NR_CellSysInfo.BcchInfo.SegmentedSIs); for (i := 0; i < v_NoOfSIsSegmented; i:=i+1) { v_SegmentedSiPosistionInSIB1 := i + v_BroadcastIndex; v_SegmentedSiSchedulList[i].Periodicity := v_SI_SchedulingInfo.schedulingInfoList[v_SegmentedSiPosistionInSIB1].si_Periodicity; //@sic R5-237429 sic@ v_SegmentedSiSchedulList[i].Window[0].SearchSpaceAndDci := cs_NR_SearchSpaceDlDciAssignment_CSSType0A(v_NR_PhysicalParameters, v_RV); v_SegmentedSiSchedulList[i].Window[0].SlotOffset := v_SI_SlotOffset; } } if(isvalue(v_NR_CellSysInfo.BcchInfo.PosSIs)){ //@sic R5-222377 sic@ // p_BroadcastAllSI not handled inside the function fl_InitSS_PosSI_Scheduling as it is not needed so far: in all PosSI test case all posSIs are broadcasted v_PosSiSchedulInfo := fl_InitSS_PosSI_Scheduling(p_NR_CellId); } v_AllOtherSiSchedul := cs_NR_SiSchedulList_Def(v_SiWindowLength, v_SiSchedulList, v_SegmentedSiSchedulList, v_PosSiSchedulInfo); } } return v_AllOtherSiSchedul; }

  - Change 2

| Function name | f_NR_ModifySysinfo_SIB19() |
| --- | --- |
| Reason for change | According to step 23 and step 28 of Table 8.1.5.14.1.3.2-1 in 38.523-1, TC requires to stop and resume SIB19; but the current TTCN implementation, the function f_NR_ModifySysinfo() is for changing SIBs message and taking the BCCH modification period length into account. It takes more time to stop SIB19. To meet the requirement of TC procedure, SS need to stop broadcasting SIB19 immediately. |
| Summary of change | Add a new function f_NR_ModifySysinfo_SIB19() to stop SIB19 immediately. |
| TTCN module | NR5GC_SysinfoPaging.ttcn |
| MCC160 Comment | Not needed.The issue is handled in R5-256678. Please see MCC160 implementation below. |

Add new function:

// WI = 1458048 // =================================================================================================================== // SYSINFO MODIFICATION for SIB19 // =================================================================================================================== /* * @desc Send the modified SYSINFO to SS * p_BroadcastAllSI = TRUE implies All SI are broadcasted even if their status is set to notBroadcasting * @param p_NR_CellId * @param p_BroadcastAllSI (default value: false) */ function f_NR_ModifySysinfo_SIB19( NR_CellId_Type p_NR_CellId, boolean p_BroadcastAllSI := false ) runs on NR5GC_PTC { var template (omit) NR_BcchToPdschConfig_Type v_NR_BcchToPdschConfig := f_NR_InitialiseBcchToPdschConfig(p_NR_CellId,p_BroadcastAllSI); //@sic R5s200703 sic@ f_NR_SS_ConfigureSysinfo (p_NR_CellId, -, v_NR_BcchToPdschConfig, p_BroadcastAllSI); //@sic R5s220880 sic@ }

  - Change 3

| Function name | f_TC_8_1_5_14_1_NR5GC() |
| --- | --- |
| Reason for change | From step 22 to step 26 in Table 8.1.5.14.1.3.2-1 of 38.523-1, SS start T430(20s) and wait until expiry of T430, in the meantime at step 25, SS sends DL data to UE. That means UE triggers SR to loopback the DL data. With current settings, for NGSO mode,the duration of SR is 10ms * 64 = 640ms, it is too short to meet the requirement of testing. UE send Prach before T430 timeout, So enlarge the duration of SRs for testing. A prose CR will be submitted in coming RAN5 for discussion. |
| Summary of change | Configure the sr_ProhibitTimer to ms64 to avoid UE send Prach early than T430 time out. |
| TTCN module | RRC_Others_NTN_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_8_1_5_14_1_NR5GC() runs on NR5GC_PTC { //NTN /SIB19/T430 with the timer value set to ntn-UlSyncValidityDuration / T430 Expiry var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; //@sic R5-255092 R5s250232 sic@ f_NR5GC_Init(NR_29); //@sic R5-252348 sic@ //@sic R5-255092 R5s250232 sic@ v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.schedulingRequestConfig.schedulingRequestToAddModList[0].sr_TransMax := n64; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); //Set SIB19 acc to Table 8.1.5.14.1.3.3-1 //Set ntn-UlSyncValidityDuration-r17 //@sic R5-255092 R5s250232 sic@ f_NR_CellInfo_SetSIB19_NTN_UlSyncValidityDuration(nr_Cell1,s20); f_NR_CellInfo_SetSIB19_NTN_ephochTime_setTo0(nr_Cell1); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); //The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C. //The UE is in state Switched OFF (state 0N-B) as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1. f_NR5GC_Preamble(nr_Cell1, STATE_OFF_0B); f_NR_TestBody_Set(true); f_TC_8_1_5_14_1_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

After Change:

function f_TC_8_1_5_14_1_NR5GC() runs on NR5GC_PTC { //NTN /SIB19/T430 with the timer value set to ntn-UlSyncValidityDuration / T430 Expiry var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; //@sic R5-255092 R5s250232 sic@ f_NR5GC_Init(NR_29); //@sic R5-252348 sic@ //@sic R5-255092 R5s250232 sic@ v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); // WI = 1458048 v_MAC_CellGroupConfig.schedulingRequestConfig.schedulingRequestToAddModList[0].sr_ProhibitTimer := ms64; v_MAC_CellGroupConfig.schedulingRequestConfig.schedulingRequestToAddModList[0].sr_TransMax := n64; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); //Set SIB19 acc to Table 8.1.5.14.1.3.3-1 //Set ntn-UlSyncValidityDuration-r17 //@sic R5-255092 R5s250232 sic@ f_NR_CellInfo_SetSIB19_NTN_UlSyncValidityDuration(nr_Cell1,s20); f_NR_CellInfo_SetSIB19_NTN_ephochTime_setTo0(nr_Cell1); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); //The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C. //The UE is in state Switched OFF (state 0N-B) as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1. f_NR5GC_Preamble(nr_Cell1, STATE_OFF_0B); f_NR_TestBody_Set(true); f_TC_8_1_5_14_1_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

  - Change 4

| Function name | f_TC_8_1_5_14_1_NR5GC_TestBody() |
| --- | --- |
| Reason for change | - The current TTCN implementation does not consider the scenario abut if the UE requires an external trigger to establish a PDU session(pc_noOf_PDUsSameConnection == 0). - According to step 23 and step 28 of Table 8.1.5.14.1.3.2-1 in 38.523-1, TC requires to stop and resume SIB19; The current TTCN implementation, function f_NR_ModifySysinfo() is for changing SIBs message and taking the BCCH modification period length into account. It takes more time to stop SIB19. To meet the requirement of TC procedure, SS need to stop broadcasting SIB19 immediately. - From step 22 to step 26 in Table 8.1.5.14.1.3.2-1 of 38.523-1, SS start T430(20s) and wait until expiry of T430, in the meantime at step 25, SS sends DL data to UE.UE will trigers SR to loopback DL data. With current TTCN, for NGSO,SS sends the DL data too early, UE reaches the Sr_TransMax(n64) before T430 timeout, then UE sends Prach and TC failed.To meet testing requirement, need to postpone sending DL data (step 25) about 18s to ensure as close as the T430 timeout to avoid UE trigers Prach procedure. - According to the Table 4.4.3.1.2-3 in 38.508-1, for SIB combination of NR-29, SIB1,SIB2 and SIB19 are included, but the implementation of the function f_NR_SetSysinfoCombinationToNR_29() does not include SIB2, replace it with f_NR_ModifySysinfoCombinationToNR_29() to meet the specification requirements. - Resume broadcast SIB19 immediately, same as point 2. |
| Summary of change | - Add function f_NR5GC_RRC_Idle_Steps16_20 () to take care about the scenario on pc_noOf_PDUsSameConnection == 0. - Invoke new function f_NR_ModifySysinfo_SIB19() to stop broadcasting SIB19. - Add new timer t_BeforeStep24 of duration about 18s to postpond sending DL data at step 25. - Invoke f_NR_ModifySysinfoCombinationToNR_29() to include SIB1, SIB2 and SIB19 in schedulingInfoList. - Invoke new function f_NR_ModifySysinfo_SIB19() to resume broadcasting SIB19. |
| TTCN module | RRC_Others_NTN_NR5GC.ttcn |
| MCC160 Comment | - Accepted in principle, but implemented differently. Please see MCC160 implementation below. - 2. Not needed.The issue is handled in R5-256678.Please see MCC160 implementation below - 3. Accepted - 4. Accepted - 5. Not needed.The issue is handled in R5-256678.Please see MCC160 implementation below |

Before Change:

function f_TC_8_1_5_14_1_NR5GC_TestBody() runs on NR5GC_PTC { //@sic R5-255092 R5s250232 sic@ var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var template (value) QosFlow_Identification_Type v_QosFlowId; var integer v_DRBId; var SubFrameTiming_Type v_Timing; var float v_T430fl; timer t_T430 := 20.0; timer t_Watchdog := 2.0; //@siclog "Step 1" siclog@ //Power on UE. f_UT_SwitchOnUE(UT, false); //@siclog "Steps 2-19" siclog@ //Steps 2 to 19a1 of the registration procedure described in TS 38.508-1 [4] table 4.5.2.2-2 with 'connected without release' and Test Loop Function (On) with UE test loop mode B, //Step 2-15 f_NR5GC_Registration(nr_Cell1,TEST_LOOPModeB_ON); //Step 19a1 if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment(nr_Cell1,-,tsc_NR_RbId_SRB1); } //@siclog "Steps 20-21" siclog@ //The SS transmits a CLOSE UE TEST LOOP message to enter the UE test loop mode. The CLOSE UE TEST LOOP is using the associated condition for the test loop. //The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities for the radio bearer(s) have been created and loop back is activated. f_NR5GC_CloseUE_TestLoopModeB (nr_Cell1, '00'O); //@siclog "Step 22" siclog@ //SS stops broadcasting SIB19 f_NR_SetSysinfoCombinationToNR_1(nr_Cell1); //@sic R5s250232 sic@ f_NR_ModifySysinfo(nr_Cell1,false, RRC_CONNECTED); //@siclog "Step 23" siclog@ //@sic R5-255092 R5s250232 sic@ //SS start a timer T1 with value = T430-(((Current SFN- epochsfn)*10)/1000) v_Timing := f_NR_GetCurrentTiming(nr_Cell1); v_T430fl := 20.0 - (int2float(v_Timing.SFN.Number * 10)) / 1000.0; //considering epoch time from SFN=0 and SIB modifiction t_T430.start(v_T430fl); //@siclog "Step 24" siclog@ //SS stops UL periodic grant allocation f_NR_ULGrantConfiguration_Stop(nr_Cell1); //@siclog "Step 25" siclog@ //The SS transmits one IP packet to the UE on the DRB associated with the first PDU session on NR Cell 1. v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(v_DRBId); DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell1, v_QosFlowId, {v_IPData})); //@siclog "Step 26" siclog@ //Wait until expiry of T1. t_T430.timeout; //@sic R5-255092 R5s250232 sic@ //@siclog "Step 26A" siclog@ //SS checks if UE sends Scheduling Request for 2 sec (UE should not send Scheduling Request when UL Sync is lost) f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); t_Watchdog.start (2.0); alt{ [] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?)) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 26A: UE still sending SR while UE suppos to be in UL Sync Lost state "); }; [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 26A: UE stops SR on UL Sync Lost "); } } f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); //@siclog "Step 27" siclog@ //The SS starts the UL periodic grant transmission allowing the UE to return the IP packet as received at step 25. f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_Now, f_NR_GetDefault_ResourceAllocation(f_NR_CellInfo_GetChannelBandwidth(nr_Cell1), f_NR_CellInfo_GetSCS(nr_Cell1)), 5); //@siclog "Step 28" siclog@ //SS resumes broadcasting SIB19. f_NR_SetSysinfoCombinationToNR_29(nr_Cell1); //@sic R5-252348 sic@ //@sic R5-255092 R5s250232 sic@ f_NR_CellInfo_SetSIB19_NTN_UlSyncValidityDuration(nr_Cell1,s20); f_NR_ModifySysinfo(nr_Cell1, false, RRC_CONNECTED); //@siclog "Step 29" siclog@ //Check: Does the UE loop back the IP packet received at step 25 in NR Cell 1? DRB.receive(car_NR_DRB_COMMON_IND_SDAP_SDUList(nr_Cell1, v_QosFlowId, {v_IPData})); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 29: UE loops back the IP packet received at step 25 "); f_NR_ULGrantConfiguration_Start(nr_Cell1,-, -, -, cs_NR_UplinkTimeAlignment_Stop);//@sic R5s250232 sic@ }

After Change:

function f_TC_8_1_5_14_1_NR5GC_TestBody() runs on NR5GC_PTC { //@sic R5-255092 R5s250232 sic@ var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var template (value) QosFlow_Identification_Type v_QosFlowId; var integer v_DRBId; var SubFrameTiming_Type v_Timing; var float v_T430fl; timer t_T430 := 20.0; timer t_Watchdog := 2.0; // WI = 1458048 timer t_BeforeStep24; var float v_T430 := 20.0; var float v_BeforeStep24; //@siclog "Step 1" siclog@ //Power on UE. f_UT_SwitchOnUE(UT, false); //@siclog "Steps 2-19" siclog@ //Steps 2 to 19a1 of the registration procedure described in TS 38.508-1 [4] table 4.5.2.2-2 with 'connected without release' and Test Loop Function (On) with UE test loop mode B, //Step 2-15 f_NR5GC_Registration(nr_Cell1,TEST_LOOPModeB_ON); //Step 19a1 if (pc_noOf_PDUsSameConnection > 0 ){ f_NR5GC_PDUSessionEstablishment(nr_Cell1,-,tsc_NR_RbId_SRB1); }else{ // WI = 1458048 //Steps 16 - 20 f_NR5GC_RRC_Idle_Steps16_20 (nr_Cell1); } //@siclog "Steps 20-21" siclog@ //The SS transmits a CLOSE UE TEST LOOP message to enter the UE test loop mode. The CLOSE UE TEST LOOP is using the associated condition for the test loop. //The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities for the radio bearer(s) have been created and loop back is activated. f_NR5GC_CloseUE_TestLoopModeB (nr_Cell1, '00'O); //@siclog "Step 22" siclog@ //SS stops broadcasting SIB19 f_NR_SetSysinfoCombinationToNR_1(nr_Cell1); //@sic R5s250232 sic@ f_NR_ModifySysinfo_SIB19(nr_Cell1,false); // WI = 1458048 //@siclog "Step 23" siclog@ //@sic R5-255092 R5s250232 sic@ //SS start a timer T1 with value = T430-(((Current SFN- epochsfn)*10)/1000) v_Timing := f_NR_GetCurrentTiming(nr_Cell1); v_T430fl := 20.0 - (int2float(v_Timing.SFN.Number * 10)) / 1000.0; //considering epoch time from SFN=0 and SIB modifiction // WI = 1458048 v_BeforeStep24 := v_T430fl - 2.0; t_T430.start(v_T430fl); // WI = 1458048 t_BeforeStep24.start(v_BeforeStep24); t_BeforeStep24.timeout; //@siclog "Step 24" siclog@ //SS stops UL periodic grant allocation f_NR_ULGrantConfiguration_Stop(nr_Cell1); //@siclog "Step 25" siclog@ //The SS transmits one IP packet to the UE on the DRB associated with the first PDU session on NR Cell 1. v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(v_DRBId); DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell1, v_QosFlowId, {v_IPData})); //@siclog "Step 26" siclog@ //Wait until expiry of T1. t_T430.timeout; //@sic R5-255092 R5s250232 sic@ //@siclog "Step 26A" siclog@ //SS checks if UE sends Scheduling Request for 2 sec (UE should not send Scheduling Request when UL Sync is lost) f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); t_Watchdog.start (2.0); alt{ [] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?)) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 26A: UE still sending SR while UE suppos to be in UL Sync Lost state "); }; [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 26A: UE stops SR on UL Sync Lost "); } } f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); //@siclog "Step 27" siclog@ //The SS starts the UL periodic grant transmission allowing the UE to return the IP packet as received at step 25. f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_Now, f_NR_GetDefault_ResourceAllocation(f_NR_CellInfo_GetChannelBandwidth(nr_Cell1), f_NR_CellInfo_GetSCS(nr_Cell1)), 5); //@siclog "Step 28" siclog@ //SS resumes broadcasting SIB19. f_NR_ModifySysinfoCombinationToNR_29(nr_Cell1); // WI = 1458048 //@sic R5-255092 R5s250232 sic@ f_NR_CellInfo_SetSIB19_NTN_UlSyncValidityDuration(nr_Cell1,s20); f_NR_ModifySysinfo_SIB19(nr_Cell1,false); // WI = 1458048 //@siclog "Step 29" siclog@ //Check: Does the UE loop back the IP packet received at step 25 in NR Cell 1? DRB.receive(car_NR_DRB_COMMON_IND_SDAP_SDUList(nr_Cell1, v_QosFlowId, {v_IPData})); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 29: UE loops back the IP packet received at step 25 "); f_NR_ULGrantConfiguration_Start(nr_Cell1,-, -, -, cs_NR_UplinkTimeAlignment_Stop);//@sic R5s250232 sic@ }

MCC160 implementation:

function f_TC_8_1_5_14_1_NR5GC() runs on NR5GC_PTC { //NTN /SIB19/T430 with the timer value set to ntn-UlSyncValidityDuration / T430 Expiry var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; //@sic R5-255092 R5s250232 sic@ f_NR5GC_Init(NR_29); //@sic R5-252348 sic@ //@sic R5-255092 R5s250232 sic@ v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.schedulingRequestConfig.schedulingRequestToAddModList[0].sr_ProhibitTimer := ms64; //@sic R5-256678 sic@ v_MAC_CellGroupConfig.schedulingRequestConfig.schedulingRequestToAddModList[0].sr_TransMax := n64; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); //Set SIB19 acc to Table 8.1.5.14.1.3.3-1 //Set ntn-UlSyncValidityDuration-r17 //@sic R5-255092 R5s250232 R5-256678 sic@ f_NR_CellInfo_SetSIB19_NTN_UlSyncValidityDuration(nr_Cell1,s20); f_NR_CellInfo_SetSIB19_NTN_ephochTime_setTo0(nr_Cell1); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); //The pre-configured UE location is defined in TS 38.508-1 [4] Clause 4.5C. //The UE is in state Switched OFF (state 0N-B) as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1. f_NR5GC_Preamble(nr_Cell1, STATE_OFF_0B); f_NR_TestBody_Set(true); f_TC_8_1_5_14_1_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); } // ==========================TC 8.1.5.14.1 ===================================== /* * @desc REFERENCE TS 38.523-1 clause 8.1.5.14.1 * @status APPROVED (NR5GC) */ function f_TC_8_1_5_14_1_NR5GC_TestBody() runs on NR5GC_PTC { //@sic R5-255092 R5s250232 sic@ var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var template (value) QosFlow_Identification_Type v_QosFlowId; var integer v_DRBId; var SubFrameTiming_Type v_Timing; var float v_T430fl; timer t_T430 := 20.0; timer t_Watchdog := 2.0; //@sic R5-256678 sic@ timer t_BeforeStep24; var float v_BeforeStep24; //@siclog "Step 1" siclog@ //Power on UE. f_UT_SwitchOnUE(UT, false); //@siclog "Steps 2-19" siclog@ //Steps 2 to 19 of the registration procedure described in TS 38.508-1 [4] table 4.5.2.2-2 with 'connected without release' and Test Loop Function (On) with UE test loop mode B //@siclog "Steps 20-21" siclog@ //The SS transmits a CLOSE UE TEST LOOP message to enter the UE test loop mode. The CLOSE UE TEST LOOP is using the associated condition for the test loop. //The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities for the radio bearer(s) have been created and loop back is activated. f_NR5GC_RRC_IdleState1N_Def(nr_Cell1, TEST_LOOPModeB_ON, noRrcConnectionRelease); //@sic R5-256678 sic@ //@siclog "Step 22" siclog@ //SS stops broadcasting SIB19 //Note : The sysinfo combination is changed to NR-2 f_NR_SetSysinfoCombinationToNR_2(nr_Cell1);//@sic R5-256678 sic@ //@sic R5s250232 sic@ f_NR_ModifySysinfo(nr_Cell1,false, RRC_CONNECTED); //@siclog "Step 23" siclog@ //@sic R5-255092 R5s250232 sic@ //SS start a timer T1 with value = T430-(((Current SFN- epochsfn)*10)/1000) v_Timing := f_NR_GetCurrentTiming(nr_Cell1); v_T430fl := 20.0 - (int2float(v_Timing.SFN.Number * 10)) / 1000.0; //considering epoch time from SFN=0 and SIB modifiction v_BeforeStep24 := v_T430fl - 2.0; //@sic R5-256678 sic@ t_T430.start(v_T430fl); //@sic R5-256678 sic@ //@siclog "Step 23A" siclog@ //SS waits for duration of T1-2s(Note 1) t_BeforeStep24.start(v_BeforeStep24); t_BeforeStep24.timeout; //@siclog "Step 24" siclog@ //SS stops UL periodic grant allocation f_NR_ULGrantConfiguration_Stop(nr_Cell1); //@siclog "Step 25" siclog@ //The SS transmits one IP packet to the UE on the DRB associated with the first PDU session on NR Cell 1. v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession(); v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(v_DRBId); DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell1, v_QosFlowId, {v_IPData})); //@siclog "Step 26" siclog@ //Wait until expiry of T1. t_T430.timeout; //@sic R5-255092 R5s250232 sic@ //@siclog "Step 26A" siclog@ //SS checks if UE sends Scheduling Request for 2 sec (UE should not send Scheduling Request when UL Sync is lost) f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable)); t_Watchdog.start (2.0); alt{ [] SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?)) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 26A: UE still sending SR while UE suppos to be in UL Sync Lost state "); }; [] t_Watchdog.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 26A: UE stops SR on UL Sync Lost "); } } f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable)); //@siclog "Step 27" siclog@ //@sic R5-256678 sic@ //SS resumes broadcasting SIB19. //Note : The sysinfo combination is changed to NR-29 f_NR_SetSysinfoCombinationToNR_29(nr_Cell1); //@sic R5-252348 sic@ //@sic R5-255092 R5s250232 sic@ f_NR_CellInfo_SetSIB19_NTN_UlSyncValidityDuration(nr_Cell1,s20); f_NR_ModifySysinfo(nr_Cell1, false, RRC_CONNECTED); //@siclog "Step 28" siclog@ ////@sic R5-256678 sic@ //The SS starts the UL periodic grant transmission allowing the UE to return the IP packet as received at step 25. f_NR_ULGrantConfiguration_StartPeriodicGrant(nr_Cell1, cs_TimingInfo_Now, f_NR_GetDefault_ResourceAllocation(f_NR_CellInfo_GetChannelBandwidth(nr_Cell1), f_NR_CellInfo_GetSCS(nr_Cell1)), 5); //@siclog "Step 29" siclog@ //Check: Does the UE loop back the IP packet received at step 25 in NR Cell 1? DRB.receive(car_NR_DRB_COMMON_IND_SDAP_SDUList(nr_Cell1, v_QosFlowId, {v_IPData})); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 29: UE loops back the IP packet received at step 25 "); f_NR_ULGrantConfiguration_Start(nr_Cell1,-, -, -, cs_NR_UplinkTimeAlignment_Stop);//@sic R5s250232 sic@ }