3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250454

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3763 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC RRC test case 8.1.2.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2025-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The drB-ToReleaseList is hardcoded to DRB1. It corresponds to the DRB Id of the PDU session established in the preamble which could be IMS. If this is released at Step 13 then it may cause UE to trigger a IMS PDU session establishment which can cause test case to fail. Moreover, in any case, the drB-ToReleaseList is meant to correspond to the DRB corresponding to the PDU session established at 12C as per the test procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed the DRB Id to the one corresponding to the PDU session established at Step 12C. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-1 CR 5159 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact:	Mohit Kanchan
	mohit.kanchan@keysight.com

# Corrections required.

## Change 1

| Function name | function f_TC_8_1_2_1_1_NR5GC_TestBody() runs on NR5GC_PTC |
| --- | --- |
| Reason for change | The drB-ToReleaseList is hardcoded to DRB1. It corresponds to the DRB Id of the PDU session established in the preamble which could be IMS. If this is released at Step 13 then it may cause UE to trigger a IMS PDU session establishment which can cause test case to fail. Moreover, in any case, the drB-ToReleaseList is meant to correspond to the DRB corresponding to the PDU session established at 12C as per the test procedure. |
| Summary of change | Changed the DRB Id to the one corresponding to the PDU session established at Step 12C. |
| TTCN module | RRC_Reconfiguration_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_8_1_2_1_1_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5-197057 R5s200044 R5-216177 R5-217812 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; /***************** Skipping the code *************************************/ // @siclog "Steps 9/10 - Void" siclog@ // Must release and re-configure the SS for SRB2 and DRB1 to re-establish the RLC f_NR_SS_CommonRadioBearerConfig (nr_Cell1, {cs_NR_SS_RadioBearer_Release(tsc_NR_RbId_SRB2)}); f_NR_SS_CommonRadioBearerConfig (nr_Cell1, {cs_NR_SS_RadioBearer_Release(cs_NR_RadioBearerId_DRB(v_DRBId))}); // DRB1 reconfigured during reconfiguration f_NR_SS_CommonRadioBearerConfig(nr_Cell1, {cs_NR_SS_SRB2_Config}); v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(omit, { cs_38508_NRDRB_ToAddMod(v_DRBInfo.SDAP_Config, v_DRBId, cs_38508_PDCP_Config)}); // @sic R5s220029 sic@ // Build RLC-BearerConfig for SRB2 and DRB v_CellGroupConfig := f_NR_GetCellgroupConfig_DRB (nr_Cell1, {v_DRBId}, true); // There should be 2 entries and both of them need to update the re-establishRLC flag v_CellGroupConfig.rlc_BearerToAddModList[0].reestablishRLC := true_; v_CellGroupConfig.rlc_BearerToAddModList[1].reestablishRLC := true_; // @siclog "Steps 11-12" siclog@ // The SS transmits an RRCReconfiguration message to modify SRB and DRB configuration. // Check: Does the UE transmit an RRCReconfigurationComplete message? f_NR_RRCReconfigNewDRBWithSSConfig(nr_Cell1, v_RadioBearerConfig, {v_DRBInfo.SS_Config}, // re-establish DRB in SS with exactly the same config as earlier v_CellGroupConfig, omit, tsc_NR_RbId_SRB1); // @sic R5s190388 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12"); // @sic R5s190936 sic@ // Build NAS command v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionReleaseCommand (int2oct(valueof(v_DRBInfo.PDUSessionId), 1)); // @sic R5s190936, R5s200564 sic@ // Build RRC command v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(omit, omit, {v_DRBId}); v_CellGroupConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_MCG, omit, {v_DRBInfo.RLC_Config.logicalChannelIdentity}); if (not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (nr_Cell1, '00'O); } // @siclog "Step 12A" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 // indicate that the UE is capable of exchanging IP data on DRB#1 established in Step 11? f_NR5GC_CheckDataPath(nr_Cell1, f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]), dedicatedBearer, v_DRBId, "Step 12A"); // @sic R5-197057 R5s200341 Ch. 1 R5s200571 Ch. 4 R5s220779 sic@ // @siclog "Step 12A1/12A4" siclog@ //The SS transmits an OPEN UE TEST LOOP Message. The UE transmits an OPEN UE TEST LOOP COMPLETE Message. // The SS transmits DEACTIVATE TEST MODE message. The UE transmits DEACTIVATE TEST MODE COMPLETE message. if (not pc_IP_Ping) { f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); } // @siclog "Steps 12B - 12C" siclog@ // Cause the UE to request establishment of PDU session. // The PDU session establishment procedure as specified in TS 38.508-1 subclause 4.5A.2 takes place. f_NR5GC_TriggerSinglePDUSession(nr_Cell1, f_NR5GC_MobileInfo_GetNoOfSessions()+1, -, tsc_RandomAPN); // @sic R5s211349 sic@ // @siclog "Steps 13-14" siclog@ // The SS transmits an RRCReconfiguration message with a drb-ToReleaseList and PDU SESSION RELEASE COMMAND to release the PDU session to which the released DRB is associated // Check: Does the UE transmit an RRCReconfigurationComplete message? /***************** Skipping the code *************************************/ }

After Change:

function f_TC_8_1_2_1_1_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5-197057 R5s200044 R5-216177 R5-217812 sic@ var NG_NAS_MSG_Indication_Type v_ReceivedMsg; /***************** Skipping the code *************************************/ // Build NAS command /* v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionReleaseCommand (int2oct(valueof(v_DRBInfo.PDUSessionId), 1)); // @sic R5s190936, R5s200564 sic@ // Build RRC command v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(omit, omit, {v_DRBId}); v_CellGroupConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_MCG, omit, {v_DRBInfo.RLC_Config.logicalChannelIdentity});*/Move this code below if (not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (nr_Cell1, '00'O); } // @siclog "Step 12A" siclog@ // Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 // indicate that the UE is capable of exchanging IP data on DRB#1 established in Step 11? f_NR5GC_CheckDataPath(nr_Cell1, f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]), dedicatedBearer, v_DRBId, "Step 12A"); // @sic R5-197057 R5s200341 Ch. 1 R5s200571 Ch. 4 R5s220779 sic@ // @siclog "Step 12A1/12A4" siclog@ //The SS transmits an OPEN UE TEST LOOP Message. The UE transmits an OPEN UE TEST LOOP COMPLETE Message. // The SS transmits DEACTIVATE TEST MODE message. The UE transmits DEACTIVATE TEST MODE COMPLETE message. if (not pc_IP_Ping) { f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1); } // @siclog "Steps 12B - 12C" siclog@ // Cause the UE to request establishment of PDU session. // The PDU session establishment procedure as specified in TS 38.508-1 subclause 4.5A.2 takes place. f_NR5GC_TriggerSinglePDUSession(nr_Cell1, f_NR5GC_MobileInfo_GetNoOfSessions()+1, -, tsc_RandomAPN); // @sic R5s211349 sic@ v_DRBId := f_NR_GetSessionId_ForLastPDUSession(); // @sic R5s220779 R5-200256 sic@ v_DRBInfo := f_NR_GetDRBInfo_FromId(v_DRBId); // Build NAS command v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionReleaseCommand (int2oct(valueof(v_DRBInfo.PDUSessionId), 1)); // @sic R5s190936, R5s200564 sic@ // Build RRC command v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(omit, omit, {v_DRBId}); v_CellGroupConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_MCG, omit, {v_DRBInfo.RLC_Config.logicalChannelIdentity}); // @siclog "Steps 13-14" siclog@ // The SS transmits an RRCReconfiguration message with a drb-ToReleaseList and PDU SESSION RELEASE COMMAND to release the PDU session to which the released DRB is associated // Check: Does the UE transmit an RRCReconfigurationComplete message? /***************** Skipping the code *************************************/ }