3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250472

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4945 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE SSAC testcase 13.5.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current TTCN implementation calls function f_TC_13_5_1_EUTRAf_RegistrationCSnotAvailable for preamble. This function sets the variable v_ESM_Cause to '00010010'B, which represents cause #18 (CS domain not allowed). This variable is then passed to function f_EUTRA_RRCConnectionReconfigurationWithAttachAcceptAllParams. The problem here is that this cause is sent in the Activate Default EPS Bearer Context REQ message, not in the ATTACH ACCEPT. This is not inline with Prose., and needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the TTCN so that the cause #18 is set in ATTACH ACCEPT message instead of Activate Default EPS Bearer Context REQ message. Please see below |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This change is required in addition to R5s250470 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required.	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact:	Bhise Parikshit
	parikshit.bhise@rohde-schwarz.com

# Corrections required.

## Change 1

| Function name | function f_RegistrationCSnotAvailable |
| --- | --- |
| Reason for change | The current TTCN implementation calls function f_TC_13_5_1_EUTRAf_RegistrationCSnotAvailable for preamble. This function sets the variable v_ESM_Cause to '00010010'B, which represents cause #18 (CS domain not allowed). This variable is then passed to function f_EUTRA_RRCConnectionReconfigurationWithAttachAcceptAllParams. The problem here is that this cause is sent in the Activate Default EPS Bearer Context REQ message, not in the ATTACH ACCEPT. This is not inline with Prose., and needs to be corrected. |
| Summary of change | Added variable cs_EMM_Cause_tv having Type EMM_Cause as an arugment to function f_EUTRA_RRCConnectionReconfigurationWithAttachAcceptAllParams |
| TTCN module | EUTRA_NASCommonFunctions.ttcn |
| MCC160 Comment | Accepted, but the ESM_Cause value should also be set to the default value – omit. |

Before Change:

function f_RegistrationCSnotAvailable(EUTRA_CellId_Type p_CellId, IDLEUPDATED_Type p_Type) runs on EUTRA_PTC { var NAS_MSG_Indication_Type v_NAS_Ind; var APNandPCOs_Type v_APNandPCOs; var ProcedureTransactionIdentifier v_EPS_TI; var template (omit) ProtocolConfigOptions v_PcoFromUE; var template (value) ProtocolConfigOptions v_PcoToUE; var GutiParameters_Type v_GutiParams := f_EUTRA_CellInfo_GetGuti(p_CellId); var template (omit) ESM_Cause v_ESM_Cause := omit; v_NAS_Ind := f_EUTRA_InitialRegistration_Step1_4 (p_CellId, p_Type, NORMAL); v_APNandPCOs := f_EUTRA_InitialRegistration_Step5_13(p_CellId, STATE2_IDLEUPDATE, v_NAS_Ind); v_EPS_TI := v_NAS_Ind.Pdu.PiggybackedPduList[0].Msg.pDN_CONNECTIVITY_REQUEST.procedureTransactionIdentifier; // Use this value in SS responses v_PcoFromUE := v_NAS_Ind.Pdu.PiggybackedPduList[0].Msg.pDN_CONNECTIVITY_REQUEST.protocolConfigurationOptions; //WA#R5s250470 // Get the PCO to be used later (TS 36.508 v.8.2.2 Table 4.7.3-6) if (ispresent(v_APNandPCOs.protocolConfigurationOptions)){ v_PcoFromUE := v_APNandPCOs.protocolConfigurationOptions; } v_PcoToUE := f_GetDefaultProtocolConfigOptions(v_PcoFromUE); if ( match(v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.epsAttachType, cr_EPS_AttachType(tsc_EAttachType_Combined))) { v_ESM_Cause := cs_ESM_Cause_tv('00010010'B); // #18 CS Domain Not Available } f_EUTRA_RRCConnectionReconfigurationWithAttachAcceptAllParams(p_CellId, f_EUTRA_InitialRegistrationEpsBearerId(firstPDN), tsc_RRC_TI_Def, v_EPS_TI, tsc_EAttachType_EpsOnly, f_EUTRA_GetPDNAddress (f_CheckPCOforIPallocationViaNas (v_PcoToUE), f_EUTRA_InitialRegistrationPdnIndex(firstPDN)), f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_GutiParams), cs_TrackingAreaIdListDef_lv, omit, omit, v_ESM_Cause, v_APNandPCOs.accessPointName, f_GetDefaultProtocolConfigOptions (v_PcoToUE), omit); f_EUTRA_InitialRegistration_Step16_18b1(p_CellId); } }

After Change:

function f_RegistrationCSnotAvailable(EUTRA_CellId_Type p_CellId, IDLEUPDATED_Type p_Type) runs on EUTRA_PTC { var NAS_MSG_Indication_Type v_NAS_Ind; var APNandPCOs_Type v_APNandPCOs; var ProcedureTransactionIdentifier v_EPS_TI; var template (omit) ProtocolConfigOptions v_PcoFromUE; var template (value) ProtocolConfigOptions v_PcoToUE; var GutiParameters_Type v_GutiParams := f_EUTRA_CellInfo_GetGuti(p_CellId); var template (omit) ESM_Cause v_ESM_Cause := omit; v_NAS_Ind := f_EUTRA_InitialRegistration_Step1_4 (p_CellId, p_Type, NORMAL); v_APNandPCOs := f_EUTRA_InitialRegistration_Step5_13(p_CellId, STATE2_IDLEUPDATE, v_NAS_Ind); v_EPS_TI := v_NAS_Ind.Pdu.PiggybackedPduList[0].Msg.pDN_CONNECTIVITY_REQUEST.procedureTransactionIdentifier; // Use this value in SS responses v_PcoFromUE := v_NAS_Ind.Pdu.PiggybackedPduList[0].Msg.pDN_CONNECTIVITY_REQUEST.protocolConfigurationOptions; //WA#R5s250470 // Get the PCO to be used later (TS 36.508 v.8.2.2 Table 4.7.3-6) if (ispresent(v_APNandPCOs.protocolConfigurationOptions)){ v_PcoFromUE := v_APNandPCOs.protocolConfigurationOptions; } v_PcoToUE := f_GetDefaultProtocolConfigOptions(v_PcoFromUE); if ( match(v_NAS_Ind.Pdu.Msg.aTTACH_REQUEST.epsAttachType, cr_EPS_AttachType(tsc_EAttachType_Combined))) { v_ESM_Cause := cs_ESM_Cause_tv('00010010'B); // #18 CS Domain Not Available } f_EUTRA_RRCConnectionReconfigurationWithAttachAcceptAllParams(p_CellId, f_EUTRA_InitialRegistrationEpsBearerId(firstPDN), tsc_RRC_TI_Def, v_EPS_TI, tsc_EAttachType_EpsOnly, f_EUTRA_GetPDNAddress (f_CheckPCOforIPallocationViaNas (v_PcoToUE), f_EUTRA_InitialRegistrationPdnIndex(firstPDN)), f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_GutiParams), cs_TrackingAreaIdListDef_lv, omit, omit, v_ESM_Cause, v_APNandPCOs.accessPointName, f_GetDefaultProtocolConfigOptions (v_PcoToUE), omit, -,-,-,-,-,-,-,-, cs_EMM_Cause_tv(tsc_Cause_CSDomainNotAvailable)); //WA#WI1465093 f_EUTRA_InitialRegistration_Step16_18b1(p_CellId); } }