3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250474

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 3773 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of Rel-17 NR NTN test case 6.7.2.1 on NGSO band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions_plus_CT-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add NR5GC test case 6.7.2.1 to the IWD_25wk11 ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to test case 6.7.2.1 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required to testcase 6.7.2.1	4

2.1	Change 1	5

3	Branches executed	7

4	Execution Log Files	8

4.1	Exynos 2500	8

5	References	9

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of NR5GC test case 6.7.2.1 which is part of the NR5GC test suite in iwd-TTCN3-B2024-06_D25wk38.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log file(s) is (are) provided as evidence.

Contact:	Narendra Kalahasti

Narendra.Kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	6.7.2.1.NR5GC

ATS Version:	iwd-TTCN3-B2024-06_D25wk38

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	Exynos 2500

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_6_7_2_1_NR5GC () |
| --- | --- |
| Reason for change | - To avoid radio link failure during SS is configuring RRCresume procedure and to compensate NTN-RTT, need to adjust UE timing to trigger re-establishment procedure. As Cell2 will be configured on test body for RRC resume procedure so SS need time for it just before resume procedure starts for cell2 and f_NR_RRC_InactiveDef function not being called directly for cell1 with target cell2. - To avoid cell1 with unexpcted security changes. As preamble function f_NR5GC_Preamble will configure variable p_InactiveStateFollowedByRRCResume with FollowedByResume, which lead to change on cell1 security |
| Summary of change | - Added function f_NR_CellInfo_SetSIB1_UE_TimersAndConstants with cds_UE_TimersAndConstants_t319 to modify T319 = 2000 ms. - Added ResumeNoSecurity on preamble function to avoid security change on Cell1 |
| TTCN module | NTN_Inactive_NR5GC.ttcn |
| MCC160 Comments | - Accepted, but will be implemented differently, see proposed alternative implementation for change 2 - Accepted |

Before Change:

function f_TC_6_7_2_1_NR5GC() runs on NR5GC_PTC {// NR NTN / Cell reselection, Sintrasearch, Snonintrasearch in RRC_Inactive state / location based measurements //NR Cell 2 and NR Cell 3 use System information combination NR-32 as defined in TS 38.508-1 [4] Table 4.4.3.1.2-1. F_NR5GC_Init(NR_32,-,{nr_Cell2, nr_Cell3}); //NR Cell 1 uses System information combination NR-29 as defined in TS 38.508-1 [4] Table 4.4.3.1.2-1. F_NR_ModifySysinfoCombinationToNR_29(nr_Cell1); //Update SIB2 as per Table 6.7.2.1.3.3-2: SIB2 of NR Cell 1, NR Cell 2 and NR Cell 3 (preamble and all steps, Table 6.7.2.1.3.2-2) f_NR_CellInfo_SetSIB2_S_NonIntraSearchP(nr_Cell1, 5); f_NR_CellInfo_SetSIB2_S_NonIntraSearchP(nr_Cell2, 5); f_NR_CellInfo_SetSIB2_S_NonIntraSearchP(nr_Cell3, 5); f_NR_CellInfo_SetSIB2_CellReselectionPriority(nr_Cell1, 5); f_NR_CellInfo_SetSIB2_CellReselectionPriority(nr_Cell2, 5); f_NR_CellInfo_SetSIB2_CellReselectionPriority(nr_Cell3, 1); f_NR_CellInfo_SetSIB2_S_IntraSearchP(nr_Cell1, 5); f_NR_CellInfo_SetSIB2_S_IntraSearchP(nr_Cell2, 5); f_NR_CellInfo_SetSIB2_S_IntraSearchP(nr_Cell3, 5); //Update SIB4 as per Table 6.7.2.1.3.3-3: SIB4 of NR Cell 2 and NR Cell 3 (preamble and all steps, Table 6.7.2.1.3.2-2) f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell2, 0, 1); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell3, 0, 5); //Update SIB19 as per Table 6.7.2.1.3.3-4: SIB19 of NR Cell 1, NR Cell 2 and NR Cell 3 (preamble and all steps, Table 6.7.2.1.3.2-2) f_NR_CellInfo_SetSIB19_NTN_RefLocation_AndDistanceThresh(nr_Cell1,hex2oct(‘2273BA566355’H),500); f_NR_CellInfo_SetSIB19_NTN_RefLocation_AndDistanceThresh(nr_Cell2,hex2oct(‘2273BA566355’H),500); f_NR_CellInfo_SetSIB19_NTN_RefLocation_AndDistanceThresh(nr_Cell3,hex2oct(‘2273BA566355’H),500); // Create and configure cells f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell2); f_NR_CellConfig_Def(nr_Cell3); // Preamble: The UE is in state 2N-A on NR Cell 1(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-2. F_NR5GC_Preamble(nr_Cell1, STATE_INACTIVE_2A); f_NR_TestBody_Set(true); f_TC_6_7_2_1_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell3, STATE_IDLE_1A); } //end of f_TC_6_7_2_1_NR5GC

After Change:

function f_TC_6_7_2_1_NR5GC() runs on NR5GC_PTC {// NR NTN / Cell reselection, Sintrasearch, Snonintrasearch in RRC_Inactive state / location based measurements //NR Cell 2 and NR Cell 3 use System information combination NR-32 as defined in TS 38.508-1 [4] Table 4.4.3.1.2-1. F_NR5GC_Init(NR_32,-,{nr_Cell2, nr_Cell3}); //NR Cell 1 uses System information combination NR-29 as defined in TS 38.508-1 [4] Table 4.4.3.1.2-1. F_NR_ModifySysinfoCombinationToNR_29(nr_Cell1); f_NR_CellInfo_SetSIB1_UE_TimersAndConstants(nr_Cell1, cds_UE_TimersAndConstants_t319); //Update SIB2 as per Table 6.7.2.1.3.3-2: SIB2 of NR Cell 1, NR Cell 2 and NR Cell 3 (preamble and all steps, Table 6.7.2.1.3.2-2) f_NR_CellInfo_SetSIB2_S_NonIntraSearchP(nr_Cell1, 5); f_NR_CellInfo_SetSIB2_S_NonIntraSearchP(nr_Cell2, 5); f_NR_CellInfo_SetSIB2_S_NonIntraSearchP(nr_Cell3, 5); f_NR_CellInfo_SetSIB2_CellReselectionPriority(nr_Cell1, 5); f_NR_CellInfo_SetSIB2_CellReselectionPriority(nr_Cell2, 5); f_NR_CellInfo_SetSIB2_CellReselectionPriority(nr_Cell3, 1); f_NR_CellInfo_SetSIB2_S_IntraSearchP(nr_Cell1, 5); f_NR_CellInfo_SetSIB2_S_IntraSearchP(nr_Cell2, 5); f_NR_CellInfo_SetSIB2_S_IntraSearchP(nr_Cell3, 5); //Update SIB4 as per Table 6.7.2.1.3.3-3: SIB4 of NR Cell 2 and NR Cell 3 (preamble and all steps, Table 6.7.2.1.3.2-2) f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell2, 0, 1); f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell3, 0, 5); //Update SIB19 as per Table 6.7.2.1.3.3-4: SIB19 of NR Cell 1, NR Cell 2 and NR Cell 3 (preamble and all steps, Table 6.7.2.1.3.2-2) f_NR_CellInfo_SetSIB19_NTN_RefLocation_AndDistanceThresh(nr_Cell1,hex2oct(‘2273BA566355’H),500); f_NR_CellInfo_SetSIB19_NTN_RefLocation_AndDistanceThresh(nr_Cell2,hex2oct(‘2273BA566355’H),500); f_NR_CellInfo_SetSIB19_NTN_RefLocation_AndDistanceThresh(nr_Cell3,hex2oct(‘2273BA566355’H),500); // Create and configure cells f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell2); f_NR_CellConfig_Def(nr_Cell3); // Preamble: The UE is in state 2N-A on NR Cell 1(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-2. F_NR5GC_Preamble(nr_Cell1, STATE_INACTIVE_2A,-, ResumeNoSecurity); f_NR_TestBody_Set(true); f_TC_6_7_2_1_NR5GC_TestBody(); f_NR_TestBody_Set(false); // Switch/Power off UE f_NR_Postamble(nr_Cell3, STATE_IDLE_1A); } //end of f_TC_6_7_2_1_NR5GC

## Change 2

| Function name | f_TC_6_7_2_1_NR5GC_TestBody () |
| --- | --- |
| Reason for change | - Current TTCN is not configuring cell2 for the RACH procedure and Security for the RRC resume procedure as RRC suspend configuration was sent by f_NR5GC_preamble function, |
| Summary of change | - Added security configuration and RACH procedure configuration for cell2 for RRC resume at step1. |
| TTCN module | NTN_Inactive_NR5GC.ttcn |
| MCC 160 Comments | - Accepted, but will be implemented slightly differently to address the timing issue in change 1.2: the configuration will be done before changing the cell power, see the alternative proposal |

Before Change:

function f_TC_6_7_2_1_NR5GC_TestBody() runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; //Table 6.7.2.1.3.2-1: Time instances of cell power level and parameter changes //T0 -> Done in the preamble v_CellPowerList_AtT1:={// NR_Cell1-> -94, NR_Cell2-> -88 cs_NR_CellPower(nr_Cell1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE) }; v_CellPowerList_AtT2:={// NR_Cell1-> Off, NR_Cell2-> -94, NR_Cell3-> -88 cs_NR_CellPower(nr_Cell1,tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell2, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell3, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE) }; //@siclog "Step 1" siclog@ //The SS re-adjusts the SS/PBCH EPRE levels according to row "T1" in table 6.7.2.1.3.2-1. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 2" siclog@ //Check: Does the UE transmit an RRCResumeRequest message on the NR Cell 2? SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); //@siclog "Step 3" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell2); //@siclog "Step 4" siclog@ //The UE transmit an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell2); // @siclog "Step 5" siclog@ // The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. f_NR_RRC_InactiveDef(nr_Cell2, ResumeNoSecurity, -, -, -, -, -, -, nr_Cell3); // @sic R5s250008 sic@ //@siclog "Step 6" siclog@ //The SS re-adjusts the SS/PBCH EPRE levels according to row "T2" in table 6.7.2.1.3.2-1. f_NR_SetCellPowerList(v_CellPowerList_AtT2); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell3); //@siclog "Step 7" siclog@ //Check: Does the UE transmit an RRCResumeRequest message on the NR Cell 3? SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell3, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); //@siclog "Step 8" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell3); //@siclog "Step 9" siclog@ //The UE transmit an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell3); // @siclog "Step 10" siclog@ // The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell3); }

After Change:

function f_TC_6_7_2_1_NR5GC_TestBody() runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell2); var RNTI_Value_Type v_RNTI_Value_Type := f_NR_CellInfo_GetRNTI(nr_Cell2); //Table 6.7.2.1.3.2-1: Time instances of cell power level and parameter changes //T0 -> Done in the preamble v_CellPowerList_AtT1:={// NR_Cell1-> -94, NR_Cell2-> -88 cs_NR_CellPower(nr_Cell1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE) }; v_CellPowerList_AtT2:={// NR_Cell1-> Off, NR_Cell2-> -94, NR_Cell3-> -88 cs_NR_CellPower(nr_Cell1,tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell2, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell3, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE) }; //@siclog "Step 1" siclog@ //The SS re-adjusts the SS/PBCH EPRE levels according to row "T1" in table 6.7.2.1.3.2-1. f_NR_SetCellPowerList(v_CellPowerList_AtT1); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell2,tsc_NR_38508_NextHopChainingCount); f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_RRCResume(v_NR_PhysicalParameters, v_RNTI_Value_Type, tsc_NR_Msg4GrantHO_Def)); //@siclog "Step 2" siclog@ //Check: Does the UE transmit an RRCResumeRequest message on the NR Cell 2? SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); //@siclog "Step 3" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell2); //@siclog "Step 4" siclog@ //The UE transmit an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell2); // @siclog "Step 5" siclog@ // The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. f_NR_RRC_InactiveDef(nr_Cell2, ResumeNoSecurity, -, -, -, -, -, -, nr_Cell3); // @sic R5s250008 sic@ //@siclog "Step 6" siclog@ //The SS re-adjusts the SS/PBCH EPRE levels according to row "T2" in table 6.7.2.1.3.2-1. f_NR_SetCellPowerList(v_CellPowerList_AtT2); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell3); //@siclog "Step 7" siclog@ //Check: Does the UE transmit an RRCResumeRequest message on the NR Cell 3? SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell3, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); //@siclog "Step 8" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell3); //@siclog "Step 9" siclog@ //The UE transmit an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell3); // @siclog "Step 10" siclog@ // The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell3); }

MCC Alternative implementaion:

function f_TC_6_7_2_1_NR5GC_TestBody() runs on NR5GC_PTC { var template (value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template (value) NR_CellPowerList_Type v_CellPowerList_AtT2; var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell2); var RNTI_Value_Type v_RNTI_Value_Type := f_NR_CellInfo_GetRNTI(nr_Cell2); //Table 6.7.2.1.3.2-1: Time instances of cell power level and parameter changes //T0 -> Done in the preamble v_CellPowerList_AtT1:={// NR_Cell1-> -94, NR_Cell2-> -88 cs_NR_CellPower(nr_Cell1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell2, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE) }; v_CellPowerList_AtT2:={// NR_Cell1-> Off, NR_Cell2-> -94, NR_Cell3-> -88 cs_NR_CellPower(nr_Cell1,tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell2, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower(nr_Cell3, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_NonSuitableOffCellSSS_EPRE) }; f_NR_SS_AS_ActivateSecurity_Current(nr_Cell2,tsc_NR_38508_NextHopChainingCount); f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_RRCResume(v_NR_PhysicalParameters, v_RNTI_Value_Type, tsc_NR_Msg4GrantHO_Def)); //@siclog "Step 1" siclog@ //The SS re-adjusts the SS/PBCH EPRE levels according to row "T1" in table 6.7.2.1.3.2-1. f_NR_SetCellPowerList(v_CellPowerList_AtT1); //@siclog "Step 2" siclog@ //Check: Does the UE transmit an RRCResumeRequest message on the NR Cell 2? SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); //@siclog "Step 3" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell2); //@siclog "Step 4" siclog@ //The UE transmit an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell2); // @siclog "Step 5" siclog@ // The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. f_NR_RRC_InactiveDef(nr_Cell2, ResumeNoSecurity, -, -, -, -, -, -, nr_Cell3); // @sic R5s250008 sic@ //@siclog "Step 6" siclog@ //The SS re-adjusts the SS/PBCH EPRE levels according to row "T2" in table 6.7.2.1.3.2-1. f_NR_SetCellPowerList(v_CellPowerList_AtT2); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell3); //@siclog "Step 7" siclog@ //Check: Does the UE transmit an RRCResumeRequest message on the NR Cell 3? SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell3, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7"); //@siclog "Step 8" siclog@ //The SS transmits an RRCResume message. f_NR_SendRRCResume_Def(nr_Cell3); //@siclog "Step 9" siclog@ //The UE transmit an RRCResumeComplete message. f_NR_RRCResumeComplete_Def(nr_Cell3); // @siclog "Step 10" siclog@ // The SS transmits an RRCRelease message f_NR_RRCRelease(nr_Cell3); }

## Change 3

| Function name | cds_UE_TimersAndConstants_t319 () |
| --- | --- |
| Reason for change | - Added new template to increase time duration of t319, So UE will not send unexpected reconfiguration failure for radio link failure. |
| Summary of change | - Added template cds_UE_TimersAndConstants_t319 |
| TTCN module | RRC_Others_NR5GC.ttcn |
| MCC160 Comments | Rejected due to an alternative implementation of change 1.1 |

Added template:

template (value) UE_TimersAndConstants cds_UE_TimersAndConstants_t319 modifies cs_38508_UE_TimersAndConstants := { // UE-TimersAndConstants for tc8.1.5.6.1 /* @status APPROVED (NR5GC) */ t319 := ms2000 };

## Change 4

| Function name | cs_NTN_Config_NGSO_Cell3 () |
| --- | --- |
| Reason for change | - As per 38.508, Table 6.3.4.2-1 Fd Column for 3rd cell has 19.95 as Frequency doppler has been mentioned, Soc accordingly NtnFrequencyDopplerShift has to be updated on TTCN. |
| Summary of change | - Updated template cs_38508_NTN_Config with 199500 |
| TTCN module | NR_NTN_Configuration.ttcn |
| MCC160 Comments | Accepted |

Before Change:

template (value) NR_NTN_Config_Type cs_NTN_Config_NGSO_Cell3 := cs_38508_NTN_Config(cs_NTN_TimingOffset_NGSO_NTNCell2_NTNCell3,-,-195000);

After Change:

template (value) NR_NTN_Config_Type cs_NTN_Config_NGSO_Cell3 := cs_38508_NTN_Config(cs_NTN_TimingOffset_NGSO_NTNCell2_NTNCell3,-,-199500);

# Branches executed

## This NR5GC TC was executed in 5GS Stand Alone mode (Option 2) on NGSO NR Primary Band “n255” using NIA2 Integrity and NEA0 ciphering algorithms on Anritsu 5G Conformance Test platform.

# Execution Log Files

## Samsung Exynos 2500

The Samsung Exynos 2500 UE  passed this test case on Anritsu 5G Protocol Conformance Test platform ME7834NR on NGSO mode. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_6_7_2_1_LOG.html

- PICS/PIXIT parameter file:

TC_6_7_2_1_PIXIT.xml

In the log file the complete test case execution as well as the PICS/PIXIT parameter settings can be seen. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

# References

| [1] | R5s250475: Supporting information for addition of TC 6.7.2.1 on NGSO band |
| --- | --- |