3GPP TSG-RAN5 Meeting #2025-TTCN email	R5s250486

# Online, , 13th Dec 2024 - 31st Dec 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4949 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE RRC test case 8.5.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2025-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Due to the implementation of prose CR R5-255037, the contents of rf_Parameters_v1610 IE is checked as a mandatory field but the sub IEs are optional parameters with expectation of ANYOROMIT value and when UE omits rf_Parameters_v1610, testcase may unfairly fail. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to use ifpresent for rf_Parameters_v1610 IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the testcase. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2024-06_D25wk38 related to the title of this CR.

Contact:	Narendra Kalahasti
                         Narendra.kalahasti@anritsu.com

- Corrections required

  - Change 1

| Template name | cr_UE_EUTRA_Capability_v1540_IEs |
| --- | --- |
| Reason for change | Due to the implementation of prose CR R5-255037, the contents of rf_Parameters_v1610 IE is checked as a mandatory field but the sub IEs are optional parameters with expectation of ANYOROMIT value and when UE omits rf_Parameters_v1610, testcase may unfairly fail. |
| Summary of change | It is proposed to use ifpresent for rf_Parameters_v1610 IE. |
| TTCN module | LTE\8_5\RRC_OtherFunctions.ttcn |
| MCC160 Comment | Accepted |

Before Change:

template UE_EUTRA_Capability_v1540_IEs cr_UE_EUTRA_Capability_v1540_IEs (template UE_EUTRA_Capability_v1630_IEs p_UE_EUTRA_Capability_v1630_IEs := *) := { /* @sic R5s201386 BASELINE MOVING 2020 sic@ */ /* @status APPROVED (LTE) */ phyLayerParameters_v1540 := *, otherParameters_v1540 := ?, fdd_Add_UE_EUTRA_Capabilities_v1540 := *, tdd_Add_UE_EUTRA_Capabilities_v1540 := *, sl_Parameters_v1540 := *, irat_ParametersNR_v1540 := *, nonCriticalExtension := { neighCellSI_AcquisitionParameters_v1550 := *, phyLayerParameters_v1550 := ?, mac_Parameters_v1550 := ?, fdd_Add_UE_EUTRA_Capabilities_v1550 := ?, tdd_Add_UE_EUTRA_Capabilities_v1550 := ?, nonCriticalExtension := { pdcp_ParametersNR_v1560 := ?, irat_ParametersNR_v1560 := ?, appliedCapabilityFilterCommon_r15 := *, fdd_Add_UE_EUTRA_Capabilities_v1560 := ?, tdd_Add_UE_EUTRA_Capabilities_v1560 := ?, nonCriticalExtension := { rf_Parameters_v1570 := *, irat_ParametersNR_v1570 := *, nonCriticalExtension := { neighCellSI_AcquisitionParameters_v15a0 := ?, eutra_5GC_Parameters_r15 := *, fdd_Add_UE_EUTRA_Capabilities_v15a0 := *, tdd_Add_UE_EUTRA_Capabilities_v15a0 := *, nonCriticalExtension := { highSpeedEnhParameters_v1610 := *, neighCellSI_AcquisitionParameters_v1610 := *, mbms_Parameters_v1610 := *, pdcp_Parameters_v1610 := *, mac_Parameters_v1610 := *, phyLayerParameters_v1610 := *, measParameters_v1610 := *, pur_Parameters_r16 := *, eutra_5GC_Parameters_v1610 := *, otherParameters_v1610 := *, dl_DedicatedMessageSegmentation_r16 := *, mmtel_Parameters_v1610 := ?, irat_ParametersNR_v1610 := *, rf_Parameters_v1610 := { supportedBandCombination_v1610 := *, supportedBandCombinationAdd_v1610 := *, supportedBandCombinationReduced_v1610 := * }, mobilityParameters_v1610 := *, ue_BasedNetwPerfMeasParameters_v1610 := ?, sl_Parameters_v1610 := *, fdd_Add_UE_EUTRA_Capabilities_v1610 := *, tdd_Add_UE_EUTRA_Capabilities_v1610 := *, nonCriticalExtension := p_UE_EUTRA_Capability_v1630_IEs // @sic R5s221178 BASELINE MOVING 2022 sic@ } ifpresent // UE-EUTRA-Capability-v1610-IEs } ifpresent // UE-EUTRA-Capability-v15a0-IEs } ifpresent // UE-EUTRA-Capability-v1570-IEs } ifpresent // UE-EUTRA-Capability-v1560-IEs } ifpresent // UE-EUTRA-Capability-v1550-IEs } ifpresent; // UE-EUTRA-Capability-v1540-IEs

After Change:

template UE_EUTRA_Capability_v1540_IEs cr_UE_EUTRA_Capability_v1540_IEs (template UE_EUTRA_Capability_v1630_IEs p_UE_EUTRA_Capability_v1630_IEs := *) := { /* @sic R5s201386 BASELINE MOVING 2020 sic@ */ /* @status APPROVED (LTE) */ phyLayerParameters_v1540 := *, otherParameters_v1540 := ?, fdd_Add_UE_EUTRA_Capabilities_v1540 := *, tdd_Add_UE_EUTRA_Capabilities_v1540 := *, sl_Parameters_v1540 := *, irat_ParametersNR_v1540 := *, nonCriticalExtension := { neighCellSI_AcquisitionParameters_v1550 := *, phyLayerParameters_v1550 := ?, mac_Parameters_v1550 := ?, fdd_Add_UE_EUTRA_Capabilities_v1550 := ?, tdd_Add_UE_EUTRA_Capabilities_v1550 := ?, nonCriticalExtension := { pdcp_ParametersNR_v1560 := ?, irat_ParametersNR_v1560 := ?, appliedCapabilityFilterCommon_r15 := *, fdd_Add_UE_EUTRA_Capabilities_v1560 := ?, tdd_Add_UE_EUTRA_Capabilities_v1560 := ?, nonCriticalExtension := { rf_Parameters_v1570 := *, irat_ParametersNR_v1570 := *, nonCriticalExtension := { neighCellSI_AcquisitionParameters_v15a0 := ?, eutra_5GC_Parameters_r15 := *, fdd_Add_UE_EUTRA_Capabilities_v15a0 := *, tdd_Add_UE_EUTRA_Capabilities_v15a0 := *, nonCriticalExtension := { highSpeedEnhParameters_v1610 := *, neighCellSI_AcquisitionParameters_v1610 := *, mbms_Parameters_v1610 := *, pdcp_Parameters_v1610 := *, mac_Parameters_v1610 := *, phyLayerParameters_v1610 := *, measParameters_v1610 := *, pur_Parameters_r16 := *, eutra_5GC_Parameters_v1610 := *, otherParameters_v1610 := *, dl_DedicatedMessageSegmentation_r16 := *, mmtel_Parameters_v1610 := ?, irat_ParametersNR_v1610 := *, rf_Parameters_v1610 := { supportedBandCombination_v1610 := *, supportedBandCombinationAdd_v1610 := *, supportedBandCombinationReduced_v1610 := * } ifpresent, mobilityParameters_v1610 := *, ue_BasedNetwPerfMeasParameters_v1610 := ?, sl_Parameters_v1610 := *, fdd_Add_UE_EUTRA_Capabilities_v1610 := *, tdd_Add_UE_EUTRA_Capabilities_v1610 := *, nonCriticalExtension := p_UE_EUTRA_Capability_v1630_IEs // @sic R5s221178 BASELINE MOVING 2022 sic@ } ifpresent // UE-EUTRA-Capability-v1610-IEs } ifpresent // UE-EUTRA-Capability-v15a0-IEs } ifpresent // UE-EUTRA-Capability-v1570-IEs } ifpresent // UE-EUTRA-Capability-v1560-IEs } ifpresent // UE-EUTRA-Capability-v1550-IEs } ifpresent; // UE-EUTRA-Capability-v1540-IEs