3GPP TSG-SA5 Meeting #149 	S5-234157

Berlin, Germany, 22 - 26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0059 | rev | - | Current version: | 17.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify service support intent expectation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_ph2 |  |  |  |  |  | Date: |  |  | 2023-05-10 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The naming of object type for edge intent expectation does not reflect its purpose and is too generic. It is not possible to explicitly document intent expectations fro example for Communication Service expectations |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace object type Service Support with Edge Service Support. Table 6.2.2.1.2.1-1 replace Service Support with Edge Service Support Clause 7.2.2 replace ServiceSupportExpectation with EdgeServiceSupportExpectation Clause 7.2.2 replace ServiceSupportExpectationObject with EdgeServiceSupportExpectationObject |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.1.2, 7.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Stage 3 to be updated |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 6.2.2	Scenario specific IntentExpectation definition

#### 6.2.2.1	Scenario specific IntentExpectation definition

##### 6.2.2.1.1	Radio Network Expectation

6.2.2.1.1.1	Definition

Radio Network Expectation is an IntentExpectation which can be used to represent MnS consumer's expectations for radio network (RAN SubNetwork) delivering and performance assurance.

The Radio Network Expectation is defined by utilizing the construct of the generic IntentExpectation <<dataType>> with set of allowed values and concrete dataTypes specified.

Following are the specific allowed values when implemented the IntentExpectation for Radio Network Expectation.

Table 6.2.2.1.1.1-1

| Attribute Name | Allowed Values |
| --- | --- |
| objectType (CM) | RAN SubNetwork |
| objectInstance (CM) | DN of the RAN SubNetwork |

NOTE:	Following are the qualifier description for attribute "objectType" and "objectInstance":

-	In case of the intent expectation is not for a specific RAN SubNetwork instance or/and MnS consumer have no knowledge of the DN of this RAN SubNetwork instance, the attribute "objectType" needs to be specified.

-	In case of the intent expectation is for a specific RAN SubNetwork instance and MnS consumer have the knowledge of the DN of this RAN SubNetwork instance, the attribute "objectInstance" needs to specified.

6.2.2.1.1.2	ObjectContexts

Following provides the concrete ObjectContexts for Radio Network Expectation based on the common structure of ObjectContext. The properties of the attributes in the following table should be same with properties of ObjectContexts defined in clause 6.2.1.3.

Table 6.2.2.1.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| coverageAreaPolygonContext | O | T | T | F | F |
| coverageTACContext | O | T | T | F | F |
| pLMNContext | O | T | T | F | F |
| nRFqBandContext | O | T | T | F | F |
| rATContext | O | T | T | F | F |

6.2.2.1.1.3	ExpectationTargets

Following provides the concrete ExpectationTargets for Radio Network Expectation based on the common structure of ExpectationTarget. The properties of the attributes in the following table should be same with properties of ExpectationTargets defined in clause 6.2.1.3.

Table 6.2.2.1.1.3-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| weakRSRPRatioTarget | O | T | T | F | F |
| lowSINRRatioTarget | O | T | T | F | F |
| aveULRANUEThptTarget | O | T | T | F | F |
| aveDLRANUEthptTarget | O | T | T | F | F |
| lowULRANUEThptRatioTarget | O | T | T | F | F |
| lowDLRANUEThptRatioTarget | O | T | T | F | F |

##### 6.2.2.1.2	Edge Service Support Expectation

6.2.2.1.2.1	Definition

Edge Service Support Expectation is an IntentExpectation which can be used to represent MnS consumer's expectations for edge service deployment.

The Edge Service Support Expectation is defined utilizing the constructs of the generic IntentExpectation <<dataType>> with set of allowed values and concrete dataTypes specified.

Following are the specific allowed values when implemented the IntentExpectation for Edge Service Support Expectation.

Table 6.2.2.1.2.1-1

| Attribute | Allowed Values |
| --- | --- |
| ObjectType (CM) | EdgeServiceSupport |
| objectInstance (CM) | DN of the EdgeServiceSupport |

NOTE:	Following are the qualifier description for attribute "objectType" and "objectInstance":

-	In case of the intent expectation is not for a specific service instance or/and MnS consumer have no knowledge of the DN of this service instance, the attribute "objectType" needs to be specified.

-	In case of the intent expectation is for a specific service instance and MnS consumer have the knowledge of the DN of this service instance, the attribute "objectInstance" needs to be specified.

6.2.2.1.2.2	ObjectContexts

Following provides the concrete ObjectContexts for Edge Service Support Expectation based on the common structure of ObjectContext. The properties of the attributes in the following table should be same with properties of ObjectContexts defined in clause 6.2.1.3.

Table 6.2.2.1.2.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| edgeIdenfiticationIdContext | CM | T | T | F | F |
| edgeIdenfiticationLocContext | CM | T | T | F | F |
| coverageAreaTAContext | CM | T | T | F | F |

NOTE:	Following are the qualifier description for attribute "edgeIdentificationId" and " edgeIdentificationLoc":

-	In case of the Service deployment is needed at a particular edge data network, the attribute " edgeIdentificationId " needs to be specified.

-	In case of the Service deployment is needed at a particular location, the attribute "edgeIdentificationLoc" needs to be specified.

6.2.2.1.2.3	ExpectationTargets

Following provides the concrete ExpectationTargets for Edge Service Support Expectation based on the common structure of ExpectationTarget. The attribute properties defined in the table below should be same with the properties defined for ExpectationTargets in clause 6.2.1.3.

Table 6.2.2.1.2.3-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| dlThptPerUETarget | O | T | T | F | F |
| UlThptPerUETarget | O | T | T | F | F |
| dLLatencyTarget | O | T | T | F | F |
| uLLatencyTarget | O | T | T | F | F |
| maxNumberofUEsTarget | O | T | T | F | F |
| activityFactorTarget | O | T | T | F | F |
| uESpeedTarget | O | T | T | F | F |

6.2.2.1.2.4	ExpectationContexts

Following provides the concrete ExpectationContexts for Service Deployment Expectation based on the common structure of ExpectationContext. The attribute properties defined in the table below should be same with the properties defined for ExpectationContexts in clause 6.2.1.3.

Table 6.2.2.1.2.4-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| serviceStartTimeContext | O | T | T | F | F |
| serviceEndTimeContext | O | T | T | F | F |
| uEMobilityLevelContext | O | T | T | F | F |
| resourceSharingLevelContext | O | T | T | F | F |

2nd Change

### 7.2.2	OpenAPI document "TS28312_IntentNrm.yaml"

openapi: 3.0.1

info:

title: Intent NRM

version: 17.3.0

description: >-

OAS 3.0.1 definition of the Intent NRM

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.312; Intent driven management services for mobile networks

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.312/

paths: {}

components:

schemas:

#-------Definition of concrete IOCs ----------#

SubNetwork-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/SubNetwork-Attr'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/SubNetwork-ncO'

- type: object

properties:

SubNetwork:

$ref: '#/components/schemas/SubNetwork-Multiple'

Intent:

$ref: '#/components/schemas/Intent-Multiple'

Intent-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

userLabel:

type: string

intentExpectations:

type: array

items:

type: object

oneOf:

- $ref: "#/components/schemas/IntentExpectation"

- $ref: "#/components/schemas/RadioNetworkExpectation"

- $ref: "#/components/schemas/EdgeServiceSupportExpectation"

intentContexts:

type: array

items:

$ref: "#/components/schemas/IntentContext"

intentFulfilmentInfo:

$ref: "#/components/schemas/FulfilmentInfo"

#-------Definition of concrete IOCs ----------#

#-------Definition of the IntentExpectation dataType ----------#

IntentExpectation:

description: >-

This data type is the "IntentExpectation" data type without specialisations

type: object

properties:

expectationId:

type: string

expectationVerb:

$ref: "#/components/schemas/ExpectationVerb"

expectationObjects:

type: array

items:

$ref: "#/components/schemas/ExpectationObject"

expectationTargets:

type: array

items:

$ref: "#/components/schemas/ExpectationTarget"

expectationContexts:

type: array

items:

$ref: "#/components/schemas/ExpectationContext"

expectationfulfilmentInfo:

$ref: "#/components/schemas/FulfilmentInfo"

RadioNetworkExpectation:

description: >-

This data type is the "IntentExpectation" data type with specialisations to represent MnS consumer's expectations for radio network  delivering and performance assurance

type: object

properties:

expectationId:

type: string

expectationVerb:

$ref: "#/components/schemas/ExpectationVerb"

expectationObjects:

type: array

items:

$ref: "#/components/schemas/RadioNetworkExpectationObject"

expectationTargets:

type: array

items:

type: object

oneOf:

- $ref: "#/components/schemas/WeakRSRPRatioTarget"

- $ref: "#/components/schemas/LowSINRRatioTarget"

- $ref: "#/components/schemas/AveULRANUEThptTarget"

- $ref: "#/components/schemas/AveDLRANUEThptTarget"

- $ref: "#/components/schemas/LowULRANUEThptRatioTarget"

- $ref: "#/components/schemas/LowDLRANUEThptRatioTarget"

- $ref: "#/components/schemas/ExpectationTarget"

expectationContexts:

type: array

items:

$ref: "#/components/schemas/ExpectationContext"

expectationfulfilmentInfo:

$ref: "#/components/schemas/FulfilmentInfo"

EdgeServiceSupportExpectation:

description: >-

This data type is the "IntentExpectation" data type with specialisations to represent MnS consumer's expectations for service deployment

type: object

properties:

expectationId:

type: string

expectationVerb:

$ref: "#/components/schemas/ExpectationVerb"

expectationObjects:

type: array

items:

$ref: "#/components/schemas/EdgeServiceSupportExpectationObject"

expectationTargets:

type: array

items:

type: object

oneOf:

- $ref: "#/components/schemas/DLThptPerUETarget"

- $ref: "#/components/schemas/ULThptPerUETarget"

- $ref: "#/components/schemas/DLLatencyTarget"

- $ref: "#/components/schemas/ULLatencyTarget"

- $ref: "#/components/schemas/MaxNumberofUEsTarget"

- $ref: "#/components/schemas/ActivityFactorTarget"

- $ref: "#/components/schemas/UESpeedTarget"

- $ref: "#/components/schemas/ExpectationTarget"

expectationContexts:

type: array

items:

type: object

oneOf:

- $ref: "#/components/schemas/ServiceStartTimeContext"

- $ref: "#/components/schemas/ServiceEndTimeContext"

- $ref: "#/components/schemas/UEMobilityLevelContext"

- $ref: "#/components/schemas/ResourceSharingLevelContext"

- $ref: "#/components/schemas/ExpectationContext"

expectationfulfilmentInfo:

$ref: "#/components/schemas/FulfilmentInfo"

#-------Definition of the IntentExpectation dataType ----------#

#-------Definition of the ExpectationObject dataType ----------#

ExpectationObject:

description: >-

This data type is the "ExpectationObject" data type without specialisations

type: object

properties:

objectType:

type: string

enum:

- RAN_SubNetwork  #value for Radio Network Expectation--#

- Edge_Service_Support  #value for Edge Service Support Expectation--#

- TBD    #-This will be added based on defined scenario pecific intent expectation-#

objectInstance:

$ref: “TS28623_ComDefs.yaml#/components/schemas/Dn”

objectContexts:

type: array

items:

$ref: “#/components/schemas/ObjectContext”

RadioNetworkExpectationObject:

description: >-

This data type is the “ExpectationObject” data type with specialisations for RadioNetworkExpectation

type: object

properties:

objectType:

type: string

enum:

- RAN_SubNetwork  #value for Radio Network Expectation--#

objectInstance:

$ref: “TS28623_ComDefs.yaml#/components/schemas/Dn”

objectContexts:

type: array

items:

type: object

oneOf:

- $ref: “#/components/schemas/CoverageAreaPolygonContext”

- $ref: “#/components/schemas/CoverageTACContext”

- $ref: “#/components/schemas/PLMNContext”

- $ref: “#/components/schemas/NRFqBandContext”

- $ref: “#/components/schemas/RATContext”

- $ref: “#/components/schemas/ObjectContext”

EdgeServiceSupportExpectationObject:

description: >-

This data type is the "ExpectationObject" data type with specialisations for EdgeServiceSupportExpectation

type: object

properties:

objectType:

type: string

enum:

- Edge Service_Support  #value for Radio Network Expectation--#

objectInstance:

$ref: "TS28623_ComDefs.yaml#/components/schemas/Dn"

objectContexts:

type: array

items:

type: object

oneOf:

- $ref: "#/components/schemas/EdgeIdenfiticationIdContext"

- $ref: "#/components/schemas/EdgeIdenfiticationLocContext"

- $ref: "#/components/schemas/CoverageAreaTAContext"

- $ref: "#/components/schemas/ObjectContext"

#-------Definition of the ExpectationObject dataType ----------#

#-------Definition of the generic dataType --------------#

Condition:

type: string

enum:

- IS_EQUAL_TO

- IS_LESS_THAN

- IS_GREATER_THAN

- IS_WITHIN_RANGE

- IS_OUTSIDE_RANGE

- IS_ONE_OF

- IS_NOT_ONE_OF

- IS_EQUAL_TO_OR_LESS_THAN

- IS_EQUAL_TO_OR_GREATER_THAN

FulfilStatus:

type: string

enum:

- FULFILLED

- NOT_FULFILLED

NotFulfilledState:

type: string

enum:

- ACKNOWLEDGED

- COMPLIANT

- DEGRADED

- SUSPENDED

- TERMINATED

- FULFILMENTFAILED

FulfilmentInfo:

type: object

properties:

fulfilStatus:

$ref: "#/components/schemas/FulfilStatus"

notFullfilledState:

description: ->

An attribute which is used when FulfilmentInfo is implemented for IntentFulfilmentInfo

$ref: "#/components/schemas/NotFulfilledState"

notFulfilledReasons:

description: ->

An attribute which is used when FulfilmentInfo is implemented for IntentFulfilmentInfo

type: string

ExpectationVerb:

type: string

enum:

- DELIVER

- ENSURE

#-------Definition of the generic dataType --------------#

#-------Definition of the IntentContext dataType --------------#

IntentContext:

description: >-

This data type is the "IntentContext" data type without specialisations

type: object

properties:

contextAttribute:

type: string

contextCondition:

$ref: "#/components/schemas/Condition"

contextValueRange:

type: array

items:

type: number

#-------Definition of the IntentContext dataType --------------#

#-------Definition of the ExpectationTarget dataType----------#

ExpectationTarget:

description: >-

This data type is the "ExpectationTarget" data type without specialisations

type: object

properties:

targetName:

type: string

targetCondition:

$ref: "#/components/schemas/Condition"

targetValueRange:

type: number

targetContexts:

type: array

items:

$ref: "#/components/schemas/TargetContext"

TargetContext:

description: >-

This data type is the "TargetContext" data type without specialisations

type: object

properties:

contextAttribute:

type: string

contextCondition:

$ref: "#/components/schemas/Condition"

contextValueRange:

type: number

WeakRSRPRatioTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for WeakRSRPRatioTarget

type: object

properties:

targetName:

type: string

enum:

- WeakRSRPRatio

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

minimum: 0

maximum: 100

targetContexts:

$ref: "#/components/schemas/WeakRSRPContext"

targetFulfilmentInfo:

$ref: "#/components/schemas/FulfilmentInfo"

WeakRSRPContext:

description: >-

This data type is the "TargetContext" data type with specialisations for WeakRSRPContext

type: object

properties:

contextAttribute:

type: string

enum:

- WeakRSRPThreshold

contextCondition:

type: string

enum:

- IS_LESS_THAN

contextValueRange:

type: number

LowSINRRatioTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for LowSINRatioTarget

type: object

properties:

targetName:

type: string

enum:

- LowSINRRatio

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

minimum: 0

maximum: 100

targetContexts:

$ref: "#/components/schemas/LowSINRContext"

targetFulfilmentInfo:

$ref: "#/components/schemas/FulfilmentInfo"

LowSINRContext:

description: >-

This data type is the "TargetContext" data type with specialisations for LowSINRContext

type: object

properties:

contextAttribute:

type: string

enum:

- LowSINRThreshold

contextCondition:

type: string

enum:

- IS_LESS_THAN

contextValueRange:

type: integer

AveULRANUEThptTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for AveULRANUEThptTarget

type: object

properties:

targetName:

type: string

enum:

- AveULRANUEThpt

targetCondition:

type: string

enum:

- IS_GREATER_THAN

targetValueRange:

type: integer

targetFulfilmentInfo:

$ref: "#/components/schemas/FulfilmentInfo"

AveDLRANUEThptTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for AveDLRANUEThptTarget

type: object

properties:

targetName:

type: string

enum:

- AveDLRANUEThpt

targetCondition:

type: string

enum:

- IS_GREATER_THAN

targetValueRange:

type: integer

targetFulfilmentInfo:

$ref: "#/components/schemas/FulfilmentInfo"

LowULRANUEThptRatioTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for LowULRANUEThptRatioTarget

type: object

properties:

targetName:

type: string

enum:

- LowULRANUEThptRatio

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

minimum: 0

maximum: 100

targetContexts:

$ref: "#/components/schemas/LowULRANUEThptContext"

targetFulfilmentInfo:

$ref: "#/components/schemas/FulfilmentInfo"

LowULRANUEThptContext:

description: >-

This data type is the "TargetContext" data type with specialisations for LowULRANUEThptContext

type: object

properties:

contextAttribute:

type: string

enum:

- LowULRANUEThptThreshold

contextCondition:

type: string

enum:

- Is_less_than

contextValueRange:

type: number

LowDLRANUEThptRatioTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for LowDLRANUEThptRatioTarget

type: object

properties:

targetName:

type: string

enum:

- LowDLRANUEThptRatio

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

minimum: 0

maximum: 100

targetContexts:

$ref: "#/components/schemas/LowDLRANUEThptContext"

targetFulfilmentInfo:

$ref: "#/components/schemas/FulfilmentInfo"

LowDLRANUEThptContext:

description: >-

This data type is the "TargetContext" data type with specialisations for LowDLRANUEThptContext

type: object

properties:

contextAttribute:

type: string

enum:

- LowDLRANUEThptThreshold

contextCondition:

type: string

enum:

- IS_LESS_THAN

contextValueRange:

type: number

DLThptPerUETarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for DLThptPerUETarget

type: object

properties:

targetName:

type: string

enum:

- DlThptPerUE

targetCondition:

type: string

enum:

- IS_GREATER_THAN

targetValueRange:

$ref: "TS28541_SliceNrm.yaml#/components/schemas/XLThpt"

ULThptPerUETarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for ULThptPerUETarget

type: object

properties:

targetName:

type: string

enum:

- UlThptPerUE

targetCondition:

type: string

enum:

- IS_GREATER_THAN

targetValueRange:

$ref: "TS28541_SliceNrm.yaml#/components/schemas/XLThpt"

DLLatencyTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for DLLatencyTarget

type: object

properties:

targetName:

type: string

enum:

- DlLatency

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

ULLatencyTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for ULLatencyTarget

type: object

properties:

targetName:

type: string

enum:

- UlLatency

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

MaxNumberofUEsTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for MaxNumberofUEsTarget

type: object

properties:

targetName:

type: string

enum:

- maxNumberofUEs

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

ActivityFactorTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for ActivityFactorTarget

type: object

properties:

targetName:

type: string

enum:

- activityFactor

targetCondition:

type: string

enum:

- IS_EQUAL_TO

targetValueRange:

type: integer

UESpeedTarget:

description: >-

This data type is the "ExpectationTarget" data type with specialisations for UESpeedTarget

type: object

properties:

targetName:

type: string

enum:

- uESpeed

targetCondition:

type: string

enum:

- IS_LESS_THAN

targetValueRange:

type: integer

#-------Definition of the concrete ExpectationTarget  dataType----------#

#-------Definition of the concrete ObjectTarget dataType----------------#

ObjectContext:

description: >-

This data type is the "ObjectContext" data type without specialisations

type: object

properties:

contextAttribute:

type: string

contextCondition:

$ref: "#/components/schemas/Condition"

contextValueRange:

type: array

items:

type: number

CoverageAreaPolygonContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for CoverageAreaPolygonContext

type: object

properties:

contextAttribute:

type: string

enum:

- CoverageAreaPolygon

contextCondition:

type: string

enum:

- IS_WITHIN_RANGE

contextValueRange:

type: array

items:

$ref: "#/components/schemas/CoverageArea"

CoverageArea:

type: string

CoverageTACContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for CoverageTACContext

type: object

properties:

contextAttribute:

type: string

enum:

- CoverageAreaTac

contextCondition:

type: string

enum:

- IS_WITHIN_RANGE

contextValueRange:

type: array

items:

$ref: "TS28541_NrNrm.yaml#/components/schemas/NrTac"

PLMNContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for PLMNContext

type: object

properties:

contextAttribute:

type: string

enum:

- PLMN

contextCondition:

type: string

enum:

- IS_WITHIN_RANGE

contextValueRange:

type: array

items:

$ref: "TS28623_ComDefs.yaml#/components/schemas/PlmnId"

NRFqBandContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for NRFqBandContext

type: object

properties:

contextAttribute:

type: string

enum:

- NRFqBand

contextCondition:

type: string

enum:

- IS_WITHIN_RANGE

contextValueRange:

type: array

items:

type: string

RATContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for RATContext

type: object

properties:

contextAttribute:

type: string

enum:

- RAT

contextCondition:

type: string

enum:

- IS_WITHIN_RANGE

contextValueRange:

type: array

items:

type: string

enum:

- UTRAN

- EUTRAN

- NR

EdgeIdenfiticationIdContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for EdgeIdenfiticationIdContext

type: object

properties:

contextAttribute:

type: string

enum:

- edgeIdentificationId

contextCondition:

type: string

enum:

- IS_EQUAL_TO

contextValueRange:

type: array

items:

type: string

EdgeIdenfiticationLocContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for EdgeIdenfiticationLocContext

type: object

properties:

contextAttribute:

type: string

enum:

- edgeIdentificationTarget

contextCondition:

type: string

enum:

- IS_EQUAL_TO

contextValueRange:

type: array

items:

type: string

CoverageAreaTAContext:

description: >-

This data type is the "ObjectContext" data type with specialisations for CoverageAreaTAContext

type: object

properties:

contextAttribute:

type: string

enum:

- coverageAreaTA

contextCondition:

type: string

enum:

- IS_WITHIN_RANGE

contextValueRange:

type: array

items:

$ref: "#/components/schemas/CoverageAreaTAList"

CoverageAreaTAList:

type: integer

#-------Definition of the concrete ObjectTarget dataType----------------#

#-------Definition of the concrete ExpectionContext dataType----------------#

ExpectationContext:

description: >-

This data type is the "ExpectationContext" data type without specialisations

type: object

properties:

contextAttribute:

type: string

contextCondition:

$ref: "#/components/schemas/Condition"

contextValueRange:

type: array

items:

type: number

ServiceStartTimeContext:

description: >-

This data type is the "ExpectationContext" data type with specialisations for ServiceStartTimeContext

type: object

properties:

contextAttribute:

type: string

enum:

- ServiceStartTime

contextCondition:

type: string

enum:

- IS_EQUAL_TO

contextValueRange:

type: string

ServiceEndTimeContext:

description: >-

This data type is the "ExpectationContext" data type with specialisations for ServiceEndTimeContext

type: object

properties:

contextAttribute:

type: string

enum:

- ServiceEndTime

contextCondition:

type: string

enum:

- IS_EQUAL_TO

contextValueRange:

type: string

UEMobilityLevelContext:

description: >-

This data type is the "ExpectationContext" data type with specialisations for UEMobilityLevelContext

type: object

properties:

contextAttribute:

type: string

enum:

- UEMobilityLevel

contextCondition:

type: string

enum:

- IS_WITHIN_RANGE

contextValueRange:

type: array

items:

$ref: "TS28541_SliceNrm.yaml#/components/schemas/MobilityLevel"

ResourceSharingLevelContext:

description: >-

This data type is the "ExpectationContext" data type with specialisations for ResourceSharingLevelContext

type: object

properties:

contextAttribute:

type: string

enum:

- ResourceSharingLevel

contextCondition:

type: string

enum:

- IS_WITHIN_RANGE

contextValueRange:

type: array

items:

$ref: "TS28541_SliceNrm.yaml#/components/schemas/SharingLevel"

#-------Definition of the concrete ExpectionContext dataType----------------#

#------Definition of JSON arrays for name-contained IOCs ---------------#

SubNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/SubNetwork-Single'

Intent-Multiple:

type: array

items:

$ref: '#/components/schemas/Intent-Single'

#------Definition of JSON arrays for name-contained IOCs ---------------#

#----- Definitions in TS 28.312 for TS 28.532 --------------------------#

resources-intentNrm:

oneOf:

- $ref: '#/components/schemas/SubNetwork-Single'

- $ref: '#/components/schemas/Intent-Single'

#----- Definitions in TS 28.312 for TS 28.532 --------------------------#