3GPP TSG-SA WG6 Meeting #58	S6-233796

Chicago, IL, USA 13th – 17th November 2023	(revision of S6-233578)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0206 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on modifying the criteria during an ad hoc group call (MCVideo) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2023-11-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Last meeting changes were agreed for the multiple MC systems scenario. This CR aligns changes made for the multiple MC system scenario to the single MC system case: - replace alerting by authorized or requesting - update the pre-conditions - remove alerting from the figure and mention authorised user - clarify that the MC service server continuously checks |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rewording as listed in the reason for change field. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is not conistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.19.2.20, 7.19.2.21, 7.19.3.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

#### 7.19.2.20	Modify ad hoc group call criteria request (MCVideo client – MCVideo server)

Table 7.19.2.20-1 describes the information flow Modify ad hoc group call criteria request from the MCVideo client to the MCVideo server.

Table 7.19.2.20-1: Modify ad hoc group call criteria request

| Information element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the requesting MCVideo user who is authorized to modify the ad hoc group criteria |
| Functional alias | O | The functional alias of the requesting MCVideo user who is authorized to modify the ad hoc group criteria |
| MCVideo ad hoc group ID | M | The MCVideo group ID of ad hoc group call whose participants list needs to be modified |
| Criteria to determine the list of participants | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCVideo server for determining the participants e.g., it can be a location-based criteria to invite participants in a particular area |

#### 7.19.2.21	Modify ad hoc group call criteria response (MCVideo server – MCVideo client)

Table 7.19.2.21-1 describes the information flow Modify ad hoc group call criteria response from the MCVideo server to the MCVideo client.

Table 7.19.2.21-1 Modify Ad hoc group call criteria response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the requesting MCVideo user who is authorized to modify the ad hoc group criteria |
| Functional alias | O | The functional alias of the requesting MCVideo user who is authorized to modify the ad hoc group criteria |
| MCVideo ad hoc group ID | M | The MCVideo group ID of ad hoc group call whose participants list needs to be modified |
| Result | M | Result of the modify ad hoc group call participants request (success of failure) |

# * * * * 2nd change * * * *

##### 7.19.3.1.6	Modification of ad hoc group call criteria by an authorized user

Figure 7.19.3.1.6-1 below illustrates the modification of ad hoc group call criteria procedure by an authorized user.

Pre-conditions:

1.	The MCVideo user at the MCVideo client 1 is authorized to modify the criteria.

2.	Both the MCVideo server and the MCVideo client 1 are aware of the criteria related to the ongoing ad hoc group call.

Figure 7.19.3.1.6-1: Modification of ad hoc group call criteria by the authorized user

1.	The MCVideo user at the MCVideo client 1 is authorized and requests to modify the criteria for determining the list of participants. The MCVideo client 1 sends the modify ad hoc group call criteria request to the MCVideo server which contains an updated criteria to determine the list of participants.

2.	The MCVideo server verifies whether the MCVideo client 1 is authorized to modify the criteria which determines the list of participants during on-going ad hoc group video call. The MCVideo server determines the list of ad hoc group call participants based on the criteria provided and identifies that MCVideo client 3 is to be removed from and MCVideo client 2 is to be added to the group.

3.	The MCVideo server sends modify ad hoc group call criteria response to the MCVideo client 1.

4.	The MCVideo server sends the ad hoc group call leave request to the MCVideo client 3 in order to remove it from the on-going ad hoc group call.

5.	The MCVideo client 3 notifies the user of the ad hoc group call leave request.

6.	The MCVideo client 3 sends the ad hoc group call leave response to the MCVideo server.

7.	The MCVideo server sends the ad hoc group call request towards MCVideo client 2.

NOTE 1:	Steps 7 to 9 can occur at any time following step 3.

8.	The receiving MCVideo client 2 notifies the user about the incoming ad hoc group call.

9.	The MCVideo client 2 accepts the ad hoc group call request and send ad hoc group call responses to the MCVideo server. The response may also contain a functional alias of the responding MCVideo user, which is verified (valid and activated for the user) by the MCVideo server. The MCVideo server considers the MCVideo user as implicitly affiliated to the ad hoc group.

10.	The MCVideo server may notify the initiating MCVideo user of all the users who are added to the on-going ad hoc group call. This notification may be sent to the initiating MCVideo user by the MCVideo server more than once during the call when MCVideo users join or leave the ad hoc group call.

11.	The MCVideo server may notify the participants about the change in the participants list of on-going ad hoc group call.

The MCVideo server continuously checks whether other MCVideo clients meet or if participating MCVideo clients no longer meet the criteria for the ad hoc group emergency call.

NOTE 2:	If the ad hoc group call is associated with an ad hoc group emergency alert and the change of criteria caused the modification of ad hoc group call participant list then the ongoing ad hoc group emergency alert is modified accordingly.

# * * * * End of changes * * * *