3GPP TSG-SA WG6 Meeting #58	S6-233797

Chicago, IL, USA 13th – 17th November 2023	(revision of S6-233579)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0341 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on modifying the criteria during an ad hoc group data communication (MCData) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2023-11-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Last meeting changes were agreed for the multiple MC systems scenario. This CR aligns changes made for the multiple MC system scenario to the single MC system case: - replace alerting by authorized or requesting - update the pre-conditions - remove alerting from the figure and mention authorised user - clarify that the MC service server continuously checks |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rewording as listed in the reason for change field. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is not conistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.17.2.20, 7.17.2.21, 7.17.3.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

#### 7.17.2.20	Modify ad hoc group data session criteria request (MCData client – MCData server)

Table 7.17.2.20-1 describes the information flow Modify ad hoc group data session criteria request from the MCData client to the MCData server.

Table 7.17.2.20-1: Modify ad hoc group data session criteria request

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData ID of the requesting MCData user who is authorized to modify the ad hoc group criteria |
| Functional alias | O | The functional alias of the MCData user who is authorized to modify the ad hoc group criteria |
| MCData ad hoc group ID | M | The MCData group ID of ad hoc group data communication whose participants list needs to be modified |
| Criteria to determine the list of participants | O | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MCData server for determining the participants e.g., it can be a location-based criteria to invite participants in a particular area |

#### 7.17.2.21	Modify ad hoc group data session criteria response (MCData server – MCData client)

Table 7.17.2.21-1 describes the information flow Modify ad hoc group data session criteria response from the MCData server to the MCData client.

Table 7.17.2.21-1 Modify Ad hoc group data session criteria response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The MCData ID of the requesting MCData user who is authorized to modify the ad hoc group criteria |
| Functional alias | O | The functional alias of the requesting MCData user who is authorized to modify the ad hoc group criteria |
| MCData ad hoc group ID | M | The MCData group ID of ad hoc group data communication whose participants list needs to be modified |
| Result | M | Result of the modify ad hoc group data communication participants request (success or failure) |

# * * * * Next change * * * *

##### 7.17.3.1.6	Modification of ad hoc group data communication criteria by an authorized user

Figure 7.17.3.1.6-1 below illustrates the modification of ad hoc group data communication criteria procedure by an authorized user.

Pre-conditions:

1.	The MCData user 1 at the MCData client 1 is authorized to modify the criteria.

2.	Both the MCData server and the MCData client 1 are aware of the criteria related to the ongoing ad hoc group data communication.

Figure 7.17.3.1.6-1: Modification of ad hoc group data communication criteria by an authorized user

1.	The MCData user at the MCData client 1 is authorized and requests to modify the criteria for determining the list of participants. The MCData client 1 sends the modify ad hoc group data session criteria request to the MCData server which contains an updated criteria to determine the list of participants.

2.	The MCVideo server verifies whether the MCVideo client 1 is authorized to modify the criteria which determines the list of participants during on-going ad hoc group data communication. The MCData server determines the list of ad hoc group call participants based on the criteria provided and indentifies that MCData client 3 is to be removed from and MCData client 2 is to be added to the group.

3.	The MCData server sends modify ad hoc group data session criteria response to the MCData client 1.

4.	The MCData server sends the ad hoc group data session leave request to the MCData client 3 in order to remove it from the on-going ad hoc group data communication.

5.	The MCData client 3 notifies the user of the ad hoc group data session leave request.

6.	The MCData client 3 sends the ad hoc group data session leave response to the MCData server.

7.	The MCData server sends the ad hoc group data session request towards MCData client 2.

NOTE 1:	Steps 7 to 9 can occur at any time following step 3.

8.	The receiving MCData client 2 notifies the user about the incoming ad hoc group data communication. The MCData server considers the MCData user as implicitly affiliated to the ad hoc group.

9.	The MCData client 2 accepts the ad hoc group data session request and send ad hoc group data session responses to the MCData server. The response may also contain a functional alias of the responding MCData user, which is verified (valid and activated for the user) by the MCData server.

10.	The MCData server may notify the initiating MCData user of all the users who are added to the on-going ad hoc group data communication. This notification may be sent to the initiating MCData user by the MCData server more than once during the data communication when MCData users join or leave the ad hoc group data communication.

11.	The MCData server may notify the participants about the change in the participants list of on-going ad hoc group data communication.

The MCData server continuously checks whether other MCData clients meet or if participating MCData clients no longer meet the criteria for the ad hoc group emergency call.

NOTE 2:	If the ad hoc group call is associated with an ad hoc group emergency alert and the change of criteria caused the modification of ad hoc group call participant list then the ongoing ad hoc group emergency alert is modified accordingly.

# * * * * End of changes * * * *