3GPP TSG-SA2 Meeting #167	S2-2502669

# Athens, Greece, February 17 – 21, 2025	(revision of S2-2502229)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1236 | rev | 12 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#2: VFL services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Ericsson, LG Electronics, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | See considerations in S2-2500143 Rev. 12 1. For following ENs in Nnwdaf_VFLTraining_Subscribe and Naf_VFLTraining_Subscribe service operations: Editor´s note: addition of Feature ID as input parameter is FFS. It is also FFS whether VFL correlation ID is optional or mandatory And the followng EN in Nnef_VFLTraining_Subscribe service operation: Editor´s note: addition of Feature ID as input parameter is FFS Based on the step 4 of the 6.2H.2.2.1, VFL Server provide determined Feature IDs per VFL client at the start of the training phase. The VFL server determines the final list of samples considering the samples that all selected VFL clients support, if used the Feature ID per VFL client and VFL Interoperability Information to be used for training and provide them to the selected VFL Clients at the start of the training phase, as described in clause 6.2H.2.3.1. Therefore, Feature ID need to be added as optional input paratmer. Also, VFL correlation ID is needed at the first start of the training phase to assign the , the rest of the training, the VFL client can use subscription correlation ID for this VFL correlation ID. 2. For below EN in Naf_VFLTraining_Request and Nnef_VFLTraining_Request service operation: Editor´s note: addition of Feature IDs as output parameter is FFS Based on the step 2 of the 6.2H.2.2.1 agreed in S2-2501347, VFL client need to provide its feature ID to the VFL server as a reponse if availble: The VFL Clients provides the list of supported Feature IDs, which is associated to the VFL Interoperability information, to the VFL Server, if available. Therefore the Feature ID is need to be added as a optional output paramter for these service operations and also for Nnwdaf_VFLTraining_Request service operation. 3. For the second part of of below EN, VFL Correlation ID is also needed for VFL Inference Editor´s note: Parameters of the service operations are FFS and more will be added when procedures and content of services are agreed. Addition of VFL correlation ID as input parameter is FFS |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define new services to support VFL Rev. 12 - Feature ID is included as optional input parameter for Nnwdaf_VFLTraining_Subscribe, Naf_VFLTraining_Subscribe, and Nnef_VFLTraining_Subscribe service operations, and remove related ENs - Feature ID is included as optional output parameter for Nnwdaf_VFLTraining_Request, Nnef_VFLTraining_Request, and Naf_VFLTraining_Request service operation, and remove related EN. - VFL Correlation ID is included as optional input parameter for Nnwdaf_VFLInference_Subscribe service operation, and remove related EN - Add Naf_Inference, Naf_Training, Nnef_Inference, Nnef_Training Services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | VFL cannot be implemted as services are missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1, new 7.X, new 7.Y, new X, new Y |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

7.1	General

Table 7.1-1 illustrates the NWDAF Services.

Table 7.1-1: NF services provided by NWDAF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnwdaf_AnalyticsSubscription | Subscribe | Subscribe / Notify | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF, NRF, SCP, ADRF |
|  | Unsubscribe |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF, NRF, SCP, ADRF |
|  | Notify |  | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, MFAF, LMF, NRF, SCP, ADRF |
|  | Transfer | Request / Response | NWDAF |
| Nnwdaf_AnalyticsInfo | Request | Request / Response | PCF, NSSF, AMF, SMF, NEF, AF, OAM, CEF, NWDAF, DCCF, LMF, NRF, SCP, ADRF |
|  | ContextTransfer | Request / Response | NWDAF |
| Nnwdaf_DataManagement | Subscribe | Subscribe / Notify | NWDAF, DCCF |
|  | Notify |  | NWDAF, DCCF, MFAF, ADRF |
|  | Fetch | Request / Response | NWDAF, DCCF, MFAF, ADRF |
| Nnwdaf_MLModelProvision | Subscribe | Subscribe / Notify | NWDAF, LMF |
|  | Unsubscribe |  | NWDAF, LMF |
|  | Notify |  | NWDAF, LMF |
| Nnwdaf_MLModelInfo | Request | Request / Response | NWDAF, LMF |
| Nnwdaf_MLModelMonitor | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Register | Request / Response | NWDAF |
|  | Request |  | NWDAF |
| Nnwdaf_MLModelTraining | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
| Nnwdaf_MLModelTrainingInfo | Request | Request / Response | NWDAF |
| Nnwdaf_RoamingAnalytics | Subscribe | Subscribe / Notify | H-RE-NWDAF, V-RE-NWDAF |
|  | Unsubscribe |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Notify |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Request | Request / Response | H-RE-NWDAF, V-RE-NWDAF |
| Nnwdaf_RoamingData | Subscribe | Subscribe / Notify | H-RE-NWDAF, V-RE-NWDAF |
|  | Unsubscribe |  | H-RE-NWDAF, V-RE-NWDAF |
|  | Notify |  | H-RE-NWDAF, V-RE-NWDAF |
| Nnwdaf_VFLTraining | Subscribe | Subscribe / Notify | NWDAF, AF, NEF |
|  | Unsubscribe |  | NWDAF, AF, NEF |
|  | Notify |  | NWDAF, AF, NEF |
|  | Request | Request / Response | NWDAF, AF, NEF |
| Nnwdaf_VFLInference | Subscribe | Subscribe / Notify | NWDAF, AF, NEF |
|  | Unsubscribe |  | NWDAF, AF, NEF |
|  | Notify |  | NWDAF, AF, NEF |
|  | Request | Request / Response | NWDAF, AF, NEF |
| NOTE 1: How OAM consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.550 [7] Annex H and out of the scope of this TS. NOTE 2: How CEF consumes Nnwdaf services and which Analytics information is relevant is defined in TS 28.201 [21] and out of the scope of this TS. NOTE 3: The Nnwdaf_MLModelProvision service and the Nnwdaf_MLModelInfo service are provided by an NWDAF containing MTLF and consumed by an NWDAF containing AnLF or provided by an NWDAF containing MTLF supporting FL as a server and consumed by an NWDAF containing MTLF. |  |  |  |

Table 7.1-2 shows the analytics information provided by NWDAF service.

Table 7.1-2: Analytics information provided by NWDAF

| Analytics Information | Request Description | Response Description |
| --- | --- | --- |
| Slice Load level information | Analytics ID: load level information | Load level provided as number of UE registrations and number of PDU sessions for a Network Slice and Network Slice instances as well as resource utilization for Network Slice instances. |
| Observed Service experience information | Analytics ID: Service Experience | Observed Service experience statistics or predictions may be provided for a Network Slice or an Application. They may be derived from an individual UE, a group of UEs or any UE. For slice service experience, they may be derived from an Application, a set of Applications or all Applications on the Network Slice. |
| NF Load information | Analytics ID: NF load information | Load statistics or predictions information for specific NF(s). |
| Network Performance information | Analytics ID: Network Performance | Statistics or predictions on the load in an Area of Interest; in addition, statistics or predictions on the number of UEs that are located in that Area of Interest. |
| UE mobility information | Analytics ID: UE Mobility | Statistics or predictions on UE mobility. When visited AOI(s) is included in the Analytics Filter information, only statistics on UE mobility can be provided. |
| UE Communication information | Analytics ID: UE Communication | Statistics or predictions on UE communication. |
| Expected UE behavioural parameters | Analytics ID: UE Mobility and/or UE Communication | Analytics on UE Mobility and/or UE Communication. |
| UE Abnormal behaviour information | Analytics ID: Abnormal behaviour | List of observed or expected exceptions, with Exception ID, Exception Level and other information, depending on the observed or expected exceptions. |
| End-to-end data volume transfer time | Analytics ID: E2E data volume transfer time | Analytics on E2E data volume transfer time. |
| User Data Congestion information | Analytics ID: User Data Congestion | Statistics or predictions on the user data congestion for transfer over the user plane, for transfer over the control plane, or for both. |
| QoS Sustainability | Analytics ID: QoS Sustainability | For statistics, the information on the location and the time for the QoS change and the threshold(s) that were crossed; or, for predictions, the information on the location and the time when a potential QoS change may occur and what threshold(s) may be crossed. |
| Session Management Congestion Control Experience | Analytics ID: Session Management Congestion Control Experience | Statistics on session management congestion control experience for specific DNN and/or S-NSSAI. |
| Redundant Transmission Experience | Analytics ID: Redundant Transmission Experience | Statistics or predictions aimed at supporting redundant transmission decisions for URLLC services. |
| WLAN performance | Analytics ID: WLAN performance | Statistics or predictions on WLAN performance of UE. |
| Dispersion | Analytics ID: UE Dispersion | Statistics or predictions that identify the location (i.e. areas of interest) or network slice(s) where a UE, or a group of UEs disperse their data volume, or disperse mobility or session management transactions or both. |
| DN Performance | Analytics ID: DN Performance | Statistics or predictions on user plane performance for a specific Edge Computing application. |
| PFD Determination | Analytics ID: PFD Determination | Statistics on PFD information for a known application identifier(s). |
| Movement Behaviour | Analytics ID: Movement Behaviour | Statistics or predictions on movement behaviour for an applicable area. |
| Location Accuracy | Analytics ID: Location Accuracy | Predictions on Location Accuracy. |
| Relative Proximity | Analytics ID: Relative Proximity | Statistics or predictions on Relative Proximity among UEs. |
| PDU Session traffic | Analytics ID: PDU Session traffic | Statistics on whether traffic of UEs via one or multiple PDU sessions is according to the information provided by the service consumer. |
| Signalling Storm | Analytics ID: Signalling storm | Statistics or predictions on controlling signalling storm for network abnormal behaviour mitigation or prevention. |
| QoS and Policy Assistance | Analytics ID: QoS and Policy Assistance | Analytics on predicted QoE for QoS parameter set(s) and candidate QoS parameter sets associated to the corresponding predicted QoE. |

2nd change

7.X	Nnwdaf_VFLTraining Service

7.X.1	General

Service Description: This service is provided by an NWDAF acting as VFL client and enables an NWDAF VFL server, an AF VFL server or an NEF acting on its behalf as consumer to request the NWDAF to participate in VFL preparation and model training as VFL client.

7.X.2	Nnwdaf_VFLTraining_Subscribe service operation

Service operation name: Nnwdaf_VFLTraining_Subscribe

Description: Subscribes to VFL ML Model training information.

Inputs, Required:

For new subscription:

-	Analytics ID as defined in Table 7.1-2;

-	Notification Target Address (+ Notification Correlation ID).

When updating a subscription: Subscription Correlation ID

Inputs, Optional:

-	Analytics filter information

-	maximum response time

-	intermediate training information

-	VFL Correlation ID is added by server at VFL training process start

-	VFL Interoperability Information

-	Feature ID

Outputs Required: When the request is accepted: Subscription Correlation ID (required for management of this subscription). When the request is not accepted, an error response with cause code

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional:

-	client intermediate training results

-	VFL status report to the consumer, according to 6.2H.2.3.1

7.X.3	Nnwdaf_VFLTraining_Unsubscribe service operation

Service operation name: Nnwdaf_VFLTraining_Unsubscribe

Description: Terminate NWDAF VFL ML Model training.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Cause code

7.X.4	Nnwdaf_VFLTraining_Notify service operation

Service operation name: Nnwdaf_VFLTraining_Notify

Description: NWDAF notifies the consumer of client intermediate training result of the local ML model.

Inputs, Required:

-	Notification Correlation Information.

Inputs, Optional:

-	client intermediate training result

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

7.X.5	Nnwdaf_VFLTraining_Request service operation

Service operation name: Nnwdaf_VFLTraining_Request

Editor´s note: Name of this service operation and its necessity and its content is FFS. It is also FFS whether it can be replaced by a subscription request for the preparation.

Description: In preparation of VFL training, requests NWDAF VFL client to check if it can support requirements for VFL.

Inputs, Required:

Inputs, Optional: none

Outputs Required: When the request is accepted:

When the request is not accepted, an error response with cause code

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional:

-	Feature ID

3rd change

7.Y	Nnwdaf_VFLInference Service

7.Y.1	General

Service Description: This service is provided by an NWDAF acting as VFL client and enables an VFL server as consumer to request or subscribe/unsubscribe for a VFL inference.

When the subscription is accepted by the NWDAF containing AnLF, the consumer receives from the NWDAF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription.

Editor´s note: Parameters of the service operations are FFS and more will be added when procedures and content of services are agreed.

7.Y.2	Nnwdaf_VFLInference_Subscribe service operation

Service operation name: Nnwdaf_ VFLInference_Subscribe

Description: Subscribe to VFL inference.

Inputs, Required:

For new subscription:

-	Notification Target Address (+ Notification Correlation ID).

-	Analytics ID

-	VFL Correlation ID

-	Target of VFL inference;

When updating a subscription: Subscription Correlation ID

Inputs, Optional:

-	VFL inference filter

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: client intermediate results

7.Y.3	Nnwdaf_VFLInference_Unsubscribe service operation

Service operation name: Nnwdaf_ VFLInference_Unsubscribe

Description: Unsubscribe to VFL inference.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

7.Y.4	Nnwdaf_VFLInference_Notify service operation

Service operation name: Nnwdaf_VFLInference_Notify

Description: Notify VFL inference result.

Inputs, Required:

- Notification Correlation Information.

Inputs, Optional:

- client intermediate results

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

7.Y.5	Nnwdaf_VFLInference_Request service operation

Service operation name: Nnwdaf_VFLInference_Request

Description: The consumer requests the NWDAF to perform a one-time VFL inference.

Inputs, Required:

-	Target of VFL inference;

-	VFL Correlation ID

-	Analytics ID

Inputs, Optional:

-	VFL inference filter

Outputs, Required: If the request is accepted, then client intermediate results. When the request is not accepted, an error response.

Outputs, Optional: none

4th change

X	AF Services to support network data analytics

X.1	General

Table X.1-1 illustrates the AF Services to support network data analytics.

Table X.1-1: NF services provided by AF to support network data analytics

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Naf_VFLTraining | Subscribe | Subscribe / Notify | NWDAF, NEF |
|  | Unsubscribe |  | NWDAF, NEF |
|  | Notify |  | NWDAF, NEF |
|  | Request | Request / Response | NWDAF, NEF |
| Naf_VFLInference | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Request | Request / Response | NWDAF |
| Naf_Inference | Subscribe | Subscribe / Notify | NWDAF, NEF |
|  | Unsubscribe |  | NWDAF, NEF |
|  | Notify |  | NWDAF, NEF |
|  | Request | Request / Response | NWDAF, NEF |
| Naf_Training | Subscribe | Subscribe / Notify | NWDAF, NEF |
|  | Unsubscribe |  | NWDAF, NEF |
|  | Notify |  | NWDAF, NEF |

X.2	Naf_VFLTraining Service

X.2.1	General

Service Description: This service is provided by an AF acting as VFL client and enables an NWDAF VFL server or an NEF acting on its behalf as consumer to request the AF to participate in VFL model training as VFL client and train a local model as defined in clause 6.2H.2.3.

For VFL, this service may also be used by the consumer (i.e. VFL Server) to prepare the VFL training as described in in clause 6.2H.2.2.

Editor´s note: Parameters of the service operations are FFS and more will be added when procedures and content of services are agreed.

Editor´s note: It is ffs whether this service is also provided by an AF acting as VFL server and enables a consumer to request VFL Training.

X.2.2	Naf_VFLTraining_Subscribe service operation

Service operation name: Naf_VFLTraining_Subscribe

Description: Subscribes to VFL ML Model training with AF as VFL client.

Inputs, Required:

For new subscription:

-	Analytics ID as defined in Table 7.1-2;

-	Notification Target Address (+ Notification Correlation ID).

When updating a subscription: Subscription Correlation ID

Inputs, Optional:

-	Analytics filter information

-	maximum response time

-	intermediate training information

-	VFL Correlation ID is added by server at VFL training process start

-	VFL Interoperability Information

-	Feature ID

Outputs Required: When the request is accepted: Subscription Correlation ID (required for management of this subscription). When the request is not accepted, an error response with cause code

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional:

-	client intermediate training result

X.2.3	Naf_VFLTraining_Unsubscribe service operation

Service operation name: Naf_VFLTraining_Unsubscribe

Description: Terminate AF VFL ML Model training.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Cause code

X.2.4	Naf_VFLTraining_Notify service operation

Service operation name: Naf_VFLTraining_Notify

Description: AF notifies the consumer of client intermediate training result of the local ML mode (for VFL).

Inputs, Required:

-	Notification Correlation Information.

Inputs, Optional:

-	client intermediate training result

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

X.2.5	Naf_VFLTraining_Request service operation

Service operation name: Naf_VFLTraining_Request

Editor´s note: Name of this service operation and its necessity and its content is FFS. It is also FFS whether it can be replaced by a subscription request for the preparation.

Description: In preparation of VFL training, requests NWDAF VFL client to check if it can support requirements for VFL.

Inputs, Required:

-	Analytics ID;

Inputs, Optional: none

Outputs Required: When the request is accepted: list of sample IDs, VFL Interoperability Information. When the request is not accepted, an error response with cause code (e.g. AF does not meet the VFL training requirements

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional:

-	Feature ID

X.3	Naf_VFLInference Service

X.3.1	General

Service Description: This service is provided by AF acting as VFL client and enables an VFL server as consumer to request or subscribe/unsubscribe for a VFL inference

When the subscription is accepted by the AF, the consumer receives from the NWDAF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription.

Editor´s note: Parameters of the service operations are FFS and more will be added when procedures and content of services are agreed.

Editor´s note: It is ffs whether this service is also provided by an AF acting as VFL server to enable a consumer to request Inference

X.3.2	Naf_VFLInference_Subscribe service operation

Service operation name: Naf_ VFLInference_Subscribe

Description: Subscribe to VFL inference.

Inputs, Required:

For new subscription:

-	Notification Target Address (+ Notification Correlation ID).

-	VFL Correlation ID

-	Target of VFL inference;

-	Analytics ID

When updating a subscription: Subscription Correlation ID

Inputs, Optional:

-	VFL inference filter

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: client intermediate results

X.3.3	Naf_VFLInference_Unsubscribe service operation

Service operation name: Naf_ VFLInference_Unsubscribe

Description: Unsubscribe to VFL inference.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

X.3.4	Naf_VFLInference_Notify service operation

Service operation name: Naf_VFLInference_Notify

Description: Notify VFL inference result.

Inputs, Required:

- Notification Correlation Information.

Inputs, Optional:

- client intermediate results

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

X.3.5	Naf_VFLInference_Request service operation

Service operation name: Naf_VFLInference_Request

Description: The consumer requests the AF to perform a one-time VFL inference.

Inputs, Required:

-	Target of VFL inference;

-	VFL Correlation ID

-	Analytics ID

Inputs, Optional:

-	VFL inference filter

Outputs, Required: If the request is accepted, then client intermediate results. When the request is not accepted, an error response.

Outputs, Optional: none

X.4	Naf_Inference Service

X.4.1	General

Service Description: This service is provided by AF acting as VFL server and enables an NWDAF or an NEF acting on its behalf as consumer to request or subscribe/unsubscribe for a VFL inference

Editor´s note: Parameters of the service operations are FFS and more will be added when procedures and content of services are agreed.

X.4.2	Naf_Inference_Subscribe service operation

Service operation name: Naf_ Inference_Subscribe

Description: Subscribe to VFL inference.

Inputs, Required:

For new subscription:

-	Notification Target Address (+ Notification Correlation ID).

-	Analytics ID

-	Target of Analytics Reporting

When updating a subscription: Subscription Correlation ID

Inputs, Optional:

-	Analytics Reporting Information

-	Analytics Filter

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: None

X.4.3	Naf_Inference_Unsubscribe service operation

Service operation name: Naf_ Inference_Unsubscribe

Description: Unsubscribe to VFL inference.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

X.4.4	Naf_Inference_Notify service operation

Service operation name: Naf_Inference_Notify

Description: Notify VFL inference result.

Inputs, Required:

- Notification Correlation Information.

Inputs, Optional:

- VFL inference results

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

X.4.5	Naf_Inference_Request service operation

Service operation name: Naf_Inference_Request

Description: The consumer requests the AF to perform a one-time VFL inference.

Inputs, Required:

-	Analytics ID

-	Target of Analytics Reporting

Inputs, Optional:

-	Analytics Reporting Information

-	Analytics Filter

Outputs, Required: If the request is accepted, then VFL inference results. When the request is not accepted, an error response.

Outputs, Optional: none

X.5	Naf_Training Service

X.5.1	General

Service Description: This service is provided by an AF acting as VFL server and enables an NWDAF or an NEF acting on its behalf as consumer to request the AF to perform model training as defined in clause 6.2H.2.3 under the supervision of the consumer.

Editor´s note: Parameters of the service operations are FFS and more will be added when procedures and content of services are agreed.

X.5.2	Naf_Training_Subscribe service operation

Service operation name: Naf_Training_Subscribe

Description: Subscribes to ML Model training with AF as VFL server.

Inputs, Required:

For new subscription:

-	Analytics ID as defined in Table 7.1-2;

-	Notification Target Address (+ Notification Correlation ID).

When updating a subscription: Subscription Correlation ID

Inputs, Optional:

Outputs Required: When the request is accepted: Subscription Correlation ID (required for management of this subscription). When the request is not accepted, an error response with cause code

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional: none

X.5.3	Naf_Training_Unsubscribe service operation

Service operation name: Naf_Training_Unsubscribe

Description: Terminate AF ML Model training.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Cause code

X.5.4	Naf_Training_Notify service operation

Service operation name: Naf_Training_Notify

Description: AF notifies the consumer of training progress

Inputs, Required:

-	Notification Correlation Information.

Inputs, Optional:

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

5th change

Y	NEF Services to support network data analytics

Y.1	General

Table Y.1-1 illustrates the NEF Services to support network data analytics.

Table Y.1-1: NF services provided by NEF to support network data analytics

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_VFLTraining | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Preparation | Request / Response | NWDAF |
| Nnef_VFLInference | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Request | Request / Response | NWDAF |
| Nnef_VFLNFdiscovery | NwdafDiscovery | Request / Response | AF |
|  | NwdafRelease | Request / Response | AF |
| Nnef_Inference | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |
|  | Request | Request / Response | NWDAF |
| Nnef_Training | Subscribe | Subscribe / Notify | NWDAF |
|  | Unsubscribe |  | NWDAF |
|  | Notify |  | NWDAF |

Y.2	Nnef_VFLTraining Service

Y.2.1	General

Service Description: This service is provided by an NEF on behalf of either an NWDAF or AF acting as VFL client in training process as defined in clause 6.2H.2.3

For VFL, this service may also be used by the consumer (i.e. FL Server) to prepare the VFL training as described in in clause 6.2H.2.2.

Editor´s note: Parameters of the service operations are FFS and more will be added when procedures and content of services are agreed.

Y.2.2	Nnef_VFLTraining_Subscribe service operation

Service operation name: Nnef_VFLTraining_Subscribe

Description: Subscribes to VFL ML Model training with AF as VFL client.

Inputs, Required:

For new subscription:

-	Analytics ID;

-	Notification Target Address (+ Notification Correlation ID).

When updating a subscription: Subscription Correlation ID

-	For NWDAF as VFL client, external NWDAF ID

Inputs, Optional:

Analytics filter information

-	maximum response time

-	intermediate training information

-	VFL Correlation ID is added by server at VFL training process start

-	VFL Interoperability Information

-	Feature ID

Outputs Required: When the request is accepted: Subscription Correlation ID (required for management of this subscription). When the request is not accepted, an error response with cause code

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional:

-	client intermediate training result

Y.2.3	Nnef_VFLTraining_Unsubscribe service operation

Service operation name: Nnef_VFLTraining_Unsubscribe

Description: Terminate AF VFL ML Model training.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Cause code.

Y.2.4	Nnef_VFLTraining_Notify service operation

Service operation name: Nnef_VFLTraining_Notify

Description: NEF notifies the consumer of client intermediate training result of the local ML mode.

Inputs, Required:

-	Notification Correlation Information.

Inputs, Optional:

-	client intermediate training result

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

Y.2.5	Nnef_VFLTraining_Request service operation

Service operation name: Nnef_VFLTraining_Request

Editor´s note: Name of this service operation and its necessity is FFS. It is also FFS whether it can be replaced by a subscription request for the preparation.

Description: In preparation of VFL training, requests NEF to check at untrusted AF acting as VFL client if it can support requirements for VFL.

Inputs, Required:

-	Analytics ID;

-	For NWDAF as VFL client, external NWDAF ID

Inputs, Optional: none

Outputs Required: When the request is accepted: list of sample IDs, VFL Interoperability Information. When the request is not accepted, an error response with cause code (e.g. NWDAF does not meet the VFL training requirements

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional:

-	Feature ID

Y.3	Nnef_VFLInference Service

Y.3.1	General

Service Description: This service is provided by by an NEF on behalf of an AF acting as VFL client and enables an VFL server as consumer to request or subscribe/unsubscribe for a VFL inference.

When the subscription is accepted by the AF, the consumer receives from the NWDAF an identifier (Subscription Correlation ID) allowing to further manage (modify, delete) this subscription.

Editor´s note: Parameters of the service operations are FFS

Y.3.2	Nnef_VFLInference_Subscribe service operation

Service operation name: Nnef_ VFLInference_Subscribe

Description: Subscribe to VFL inference.

Inputs, Required:

For new subscription:

-	Notification Target Address (+ Notification Correlation ID).

-	VFL Correlation ID

-	Target of VFL inference;

When updating a subscription: Subscription Correlation ID

-	For NWDAF as VFL client, external NWDAF ID

Inputs, Optional:

-	VFL inference filter

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: client intermediate results

Y.3.3	Nnef_VFLInference_Unsubscribe service operation

Service operation name: Nnef_ VFLInference_Unsubscribe

Description: Unsubscribe to VFL inference.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

Y.3.4	Nnef_VFLInference_Notify service operation

Inputs, Required:

- Notification Correlation Information.

Inputs, Optional:

- client intermediate results

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

Y.3.5	Nnef_VFLInference_Request service operation

Service operation name: Nnef_VFLInference_Request

Description: The consumer requests the NWDAF to perform a one-time VFL inference.

Inputs, Required:

-	Target of VFL inference;

-	VFL Correlation ID

-	Analytics ID

Inputs, Optional:

-	VFL inference filter

Outputs, Required: If the request is accepted, then client intermediate results. When the request is not accepted, an error response.

Outputs, Optional: none

Y.4	Nnef_VFLNFDiscovery Service

Y.4.1	General

Service Description: This service is provided by by an NEF towards an untrusted AF acting as VFL server to enable the AF to interact with NWDAFs acting as VFL client. It enables the AF to detect NWDAFs as VFL clients as described in in clause 6.2H.2.1.

Editor´s note: Parameters of the service operations are FFS .

Y.4.2	Nnef_VFLNFDiscovery_NwdafDiscovery service operation

Service operation name: Nnef_VFLNFDiscovery_NwdafDiscovery

Description: The consumer requests the NEF to discover an NWDAF that is to act as VFL client.

Inputs, Required:

- Analytics ID

- required NF type (i.e. NWDAF type)

-	VFL capability type (i.e. VFL client),

Inputs, Optional:

-	required feature IDs

-	Time Period of Interest

-	optional Service Area;

Outputs, Required: If the request is accepted, external NWDAF ID. When the request is not accepted, an error response.

Outputs, Optional:

-	none.

Y.4.3	Nnef_VFLNFDiscovery_NwdafRelease service operation

Service operation name: Nnef_VFLNFDiscovery_NwdafDiscovery

Description: The consumer informs the NEF that it mo longer wants .to use an assigned temporary NWDAF ID.

Inputs, Required:

-	external NWDAF ID

Inputs, Optional:

-	None;

Outputs, Required:.

-	None;

Outputs, Optional:

-	none.

Y.5	Nnef_Inference Service

Y.5.1	General

Service Description: This service is provided by an NEF on behalf of an AF acting as VFL server and enables an NWDAF as consumer to request or subscribe/unsubscribe for a VFL inference.

Editor´s note: Parameters of the service operations are FFS

Y.5.2	Nnef_Inference_Subscribe service operation

Service operation name: Nnef_ Inference_Subscribe

Description: Subscribe to VFL inference.

Inputs, Required:

For new subscription:

-	Notification Target Address (+ Notification Correlation ID).

-	Analytics ID

-	Target of Analytics Reporting

When updating a subscription: Subscription Correlation ID

Inputs, Optional:

-	Analytics Reporting Information

-	Analytics Filter

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription). When the subscription is not accepted, an error response.

Outputs, Optional: None

Y.5.3	Nnef_Inference_Unsubscribe service operation

Service operation name: Nnef_ Inference_Unsubscribe

Description: Unsubscribe to VFL inference.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

Y.5.4	Nnef_Inference_Notify service operation

Inputs, Required:

- Notification Correlation Information.

Inputs, Optional:

- VFL inference results

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

Y.5.5	Nnef_Inference_Request service operation

Service operation name: Nnef_Inference_Request

Description: The consumer requests the VFL server to perform a one-time VFL inference.

Inputs, Required:

-	Analytics ID

-	Target of Analytics Reporting

Inputs, Optional:

-	Analytics Reporting Information

-	Analytics Filter

Outputs, Required: If the request is accepted, then VFL inference results. When the request is not accepted, an error response.

Outputs, Optional: none

Y.6	Nnef_Training Service

Y.6.1	General

Service Description: This service is provided by NEF AF acting on behalf of an untzrusted AF as VFL server and enables an NWDAF as consumer to request the AF to perform model training as defined in clause 6.2H.2.3 under the supervision of the consumer.

Editor´s note: Parameters of the service operations are FFS and more will be added when procedures and content of services are agreed.

Y.6.2	Nnef_Training_Subscribe service operation

Service operation name: Nnef_Training_Subscribe

Description: Subscribes to ML Model training with AF as VFL server.

Inputs, Required:

For new subscription:

-	Analytics ID as defined in Table 7.1-2;

-	Notification Target Address (+ Notification Correlation ID).

When updating a subscription: Subscription Correlation ID

Inputs, Optional:

Outputs Required: When the request is accepted: Subscription Correlation ID (required for management of this subscription). When the request is not accepted, an error response with cause code

NOTE:	The detail reasons in the cause code are up to Stage 3.

Outputs, Optional: none

Y.6.3	Nnef_Training_Unsubscribe service operation

Service operation name: Nnef_Training_Unsubscribe

Description: Terminate AF ML Model training.

Inputs, Required: Subscription Correlation ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Cause code

Y.6.4	Nnef_Training_Notify service operation

Service operation name: Nnef_Training_Notify

Description: AF notifies the consumer of training progress

Inputs, Required:

-	Notification Correlation Information.

Inputs, Optional:

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

End of changes