3GPP TSG-WG SA2 Meeting #167	S2-2502550

# Athens, Greece, 17th Feb – 21st Feb, 2025		(revision of S2-2502351)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 5365 | rev | 3 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the UPF event subscription via I-SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When the UPF event subscription involve the I-SMF, the I-SMF may receive the SMF event subscription from SMF. If the related UPF event subscription via SMF is supported by the target UPF and indicates by I-SMF, the SMF subscribes the UPF event from the target UPF. There are no UPF event RELAY capability supported by the I-SMF. Hence the related description is suggested to be updated to avoid confusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the “Relaying of UPF event subscription” to “related subscribed UPF event subscription supported by the target UPF and indicated by I-SMF”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear whether a new SMF capability of RELAYING of UPF event subscription need be defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.4.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 4.15.4.5.2	Information flow for subscription to UPF event exposure service for certain UE(s) via SMF

Figure 4.15.4.5.2-1: Subscription to UPF event exposure service for certain UE(s) via SMF

In the case of a group of UEs, the UPF event consumer (e.g. NWDAF) first issues an Nnrf_NFDiscovery_Request service operation to find the UDM providing the target Group ID and gets the NF profile of the UDM serves this group. Then, NWDAF obtains the list of SUPIs that correspond to the Group ID from UDM using Nudm_SDM_Get

NOTE 1:	It is assumed that all members of a Group ID belong to the same UDM.

Then, for each SUPI:

1.	The UPF event consumer (e.g. NWDAF) invokes Nudm_UECM_Get service operation to retrieve the appropriate SMF by providing UE ID, DNN, S-NSSAI and NF type = SMF.

2.	The UDM provides a Nudm_UECM_Get response with the corresponding SMF.

3.	The UPF event consumer sends the Nsmf_EventExposure Subscription request to the SMF to subscribe to UPF data, including the following information:

-	Notification Target Address (UPF event consumer address), Notification Correlation Information.

-	UPF Event Id.

-	Event Filter Information, i.e. one or more of the following parameters: S-NSSAI, DNN, DNAI, UPF Id, either Application Id(s) or Traffic Filtering Information, Area of Interest, SSID/BSSID.

-	Target of Event Reporting: a UE.

-	Reporting suggestion information.

-	Type of Measurement and Granularity of Measurement.

If the consumer is NWDAF and the analytic filter information includes application server IP address/FQDN, the NWDAF may need to first obtain the DNAI from NEF as described in steps 2 and 3 in Figure 4.15.4.5.3-1.

4.	The SMF selects the PDU session(s) and the UPFs it has to send the request to. The SMF sends the request to the UPF including the UPF event consumer address, UPF Event Id, Notification Correlation Information, Event Filter Information (but only Application Id(s) or Traffic Filtering Information, if received in step 3), Reporting suggestion information, Target of Event Reporting, Type of Measurement and Granularity of Measurement as required. Target of Event Reporting indicates the PDU Session of the UE. The interaction mechanism used between SMF and UPF depends on UPF exposure event and which mechanism applies for each event as described in clause 5.2.26.2.1. For some events, the SMF shall contact UPF (4a) with N4 Session Modification with PFCP (TS 29.244 [69]), for other events (4b) with Nupf_event exposure subscribe request (as defined in clause 5.2.26.2.3).

If an I-SMF is used for this PDU session, and the UPF is controlled by the I-SMF:

- For UPF events for which the SMF-UPF interaction is performed over PFCP (TS 29.244 [69]) (e.g. QoS Monitoring), the corresponding N4 information is sent to I-SMF in Nsmf_PDUSession_Update Request message as described in clause 4.23.5 and I-SMF uses N4 Session Modification (TS 29.244 [69]) towards local UPF (4a).

- For UPF events for which the SMF-UPF interaction is performed using Nupf_EventExposure service, the SMF sends the Nsmf_EventExposure_Subscribe including the UPF event subscription related information to I-SMF, if  the I-SMF indicates that the corresponding UPF event  is supported by the target UPF. I-SMF forwards the subscription to the target UPF, i.e., local UPF (4b).

NOTE 2:	Some events can require SMF interacts with RAN at this stage.

5.	Per Reporting suggestion information (if available), the UPF sends the locally collected UPF data by invoking Nupf_EventExposure_Notify service operation to the UPF event consumer.

# * * * * End of changes * * * *