3GPP TSG-SA2 Meeting #167	 S2-2502666

Athens, Greece, 17th Feb 2025 – 21st Feb 2025 	(revision of S2-2502421)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6064 | rev | 2 | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of regulatory services support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The current text seems to imply that there is a choice between the “sufficient QoS approach” and the dedicated slice for emergency. This sems to give the MWAB the freedom to not use the configured slice. We believe this should not be the case as it makes the behaviour unpredictable and not operator controlled, but subject to MWAB implementation |  |  |  |  |  |  |  |  |
| Summary of change: |  | The text has been modified in the affected clause to clarify the behaviour so it is deterministic and based on the configuration alone. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Possible unpredictable system behaviour |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

### 5.49.8	Support of regulatory service via MWAB

Based on the OAM configurations, an MWAB may support emergency services for UEs connected to the MWAB-gNB.

When the MWAB determines that a UE which connects to the MWAB-gNB initiates an emergency PDU Session, e.g. via AS layer signalling, or based on the ARP of the PDU Session context, the following handling applies:

-	If the HPLMN of MWAB-UE has configured it with specific S-NSSAI and DNN for BH PDU Session(s) to serve emergency PDU Session for UEs accessing this MWAB, the MWAB uses a BH PDU Session associated with this specific S-NSSAI and DNN. The MWAB-UE establishes such PDU session if it was not yet established.

-	If a new BH PDU sessions using this specific S-NSSAI and DNN is established, the emergency PDU Session establishment procedure for the UE(s) served by the MWAB-gNB should not be interrupted by the establishment of the new BH PDU Session. To avoid interruption of the establishment of the emergency PDU Sessions of the UE(s) served by the MWAB-gNB, the MWAB-gNB should complete  the emergency PDU Session establishment by using the existing BH PDU Sessions for N3 traffic and then trigger PDU Session resource modification procedure for the emergency PDU Sessions of the UE(s) to associate the emergency PDU Session N3 interface to the new BH PDU Session to serve emergency PDU Session for UEs accessing this MWAB.

-	If the HPLMN of MWAB-UE has not configured the MWAB-UE with specific S-NSSAI and DNN for BH PDU Session(s) to serve emergency PDU Session(s) for UEs accessing this MWAB, the MWAB reuses the existing BH PDU Session(s) for N2/N3 interface.

-	If there are UEs with emergency services, the MWAB-gNB will not stop operating (e.g. due to de-authorization) as MWAB until it handovers the UEs to other cells as described in clause 5.49.3.

-	Based on configuration for the specific S-NSSAI and DNN for BH PDU Session to serve emergency PDU Sessions, the BH PLMN/SNPN and the MWAB attempt to guarantee the resources of the BH PDU session(s) to serve emergency PDU Session in the BH PLMN/SNPN so that they are not released when MWAB is serving the emergency services.

NOTE:	If the MWAB is configured to support emergency services for the UEs it serves, it is assumed that the MWAB Broadcasted PLMN/SNPN and BH PLMN/SNPN are in the same country.

End of changes