3GPP TSG-SA2 Meeting #167	S2-2502418

Athens, Greece, 17th Feb 2025 – 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6065 | rev | 1 | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of mobility clause |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The clause 5.49.5.1 text point to MWAB-UE specific enhancements when handling UE served by MWAB mobility. This is incorrect and misleading as the text should refer to the UEs served by the MWAB-gNB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The text in first paragraph of clause 5.49.5.1 now correctly refers to MWAB-specific enhancements rather than MWAB UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.49.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This has no overlap with the 23.501 CR5902 in S2-2501319 which affects same clause. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

#### 5.49.5.1	UE mobility when a MWAB-gNB cell is involved

No MWAB-specific enhancements impacting the UE are defined to handle a UE mobility to and from a MWAB-gNB while the UE is in CM-IDLE or CM-CONNECTED modes, irrespective of whether the MWAB is static or the MWAB is moving, i.e.:

-	For UEs in RRC_IDLE and RRC_INACTIVE state, procedure for cell (re) selection as specified in TS 38.304 [50] and TS 23.122 [17] for RRC_IDLE and RRC_INACTIVE is used irrespective of cells are belonging to a MWAB-gNB or a gNB.

-	For UEs in RRC_CONNECTED state, the MWAB-gNB or gNB triggers the handover procedures to the neighbouring cells as specified in TS 38.300 [27] and TS 23.502 [3] irrespective of whether the cells are belonging to a MWAB-gNB or a gNB.

End of changes