3GPP TSG-WG SA2 Meeting #167	S2-2502675

# Athens, Greece, 17th – 21st Feb, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5670 | rev | 6 | Current version: | 19.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#9, Support of Available Data Rate |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, CATT, Tencent, Lenovo, InterDigital, Nokia, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the agreements of KI#9 in TR 23.700-70, available bitrate exposure for GBR QoS flow is supported as described in clause 8.9. The enhancements are added to the corresponding clauses. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Supporting available bitrate exposure for GBR QoS flow is introduced in the related clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Available bitrate exposure for GBR QoS flow for XR service is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.18, 5.37.4, 5.45.1, 5.45.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.8.2.18	QoS Flow related QoS monitoring and reporting

The SMF may configure the UPF to perform QoS monitoring for a QoS Flow and to report the monitoring results with the help of the following parameters provided in the Session Reporting Rule (SRR) described in clause 5.8.5.11:

-	QoS monitoring parameter(s) indicating the subject of the QoS monitoring as defined in clause 5.45;

NOTE:	Clause 5.45 contains specific description about the UPF configuration for QoS monitoring for the respective QoS monitoring parameter.

-	Reporting period indicating the time interval in which a new measurement result and a potential report has to be available. Generally, if no measurement result is available to the UPF within the Reporting period, the UPF shall report a measurement failure; however, for some QoS monitoring parameters (e.g. congestion information, PDV and data rate, available bitrate), the measurement failure report is not applicable.

-	Reporting frequency indicating the type of the reporting as "periodic" or "event triggered":

-	If the Reporting frequency indicates "periodic", the UPF shall send a report each time the reporting period is over.

-	If the Reporting frequency indicates "event triggered", a Reporting threshold for each parameter in the QoS monitoring parameter(s) and a Minimum waiting time are provided as well. The UPF shall send a report when the measurement result matches or exceeds the indicated Reporting threshold. Subsequent reports should not be sent by the UPF during the Minimum waiting time. The UPF shall continue to report a measurement result that matches or exceeds the indicated Reporting Threshold when the Minimum waiting time is over.

NOTE:	As an implementation option, the UPF can be configured to send subsequent report(s) during the Minimum waiting time, e.g. if the UPF determines that this report is considerably different from the previous report.

-	(Optional) Target of the reporting and Indication of direct event notification indicating that the UPF shall send the reports to a different NF than the SMF (e.g. to the NEF/AF or the NWDAF/DCCF/MFAF). The NF is identified by a Notification Target Address and a Notification Correlation ID. The SMF can also indicate that the UPF shall send the reports to both, the NF indicated by the Target of reporting and to the SMF. If so, the UPF shall send the reports to the SMF as well. If the Indication of direct event notification is not provided, the UPF shall send the reports to the SMF.

-	(Optional) Reporting suggestion information as defined in clause 5.8.2.17 applicable to Target of the reporting to reduce the UPF performance impacts.

-	(Optional) Indication of QoS Flow associated with the default QoS Rule (see clause 4.15.4.5.1 of TS 23.502 [3]). The UPF shall forward this indication, that the QoS monitoring report is for the QoS Flow associated with the default QoS Rule, in the Nupf_EventExposure_Notify service operation when sending reports.

The UPF shall send the QoS Monitoring Report as follows:

-	when the UPF sends reports to the SMF, the UPF shall use QoS Monitoring Reports as described in clause 5.8.5.12; and/or

-	When the UPF sends reports to a different NF than the SMF (e.g. the NEF/AF or the NWDAF/DCCF/MFAF), the UPF shall use the Nupf_EventExposure_Notify service operation described in clause 5.2.26.2.2 of TS 23.502 [3].

# * * * * Second change * * * *

5.37.4	Network Exposure of 5GS information

5GS and XR/media services cooperate to provide a better user experience using External Network Exposure.

Based on the AF request, the 5GS can expose the following information based on the QoS Monitoring as defined in clause 5.33.3 and/or clause 5.45:

-	The UL and/or DL congestion information monitoring (see clause 5.45.3).

Based on the PCC rule from PCF, the SMF requests the NG-RAN to report congestion information (i.e. a percentage of congestion level for exposure) via GTP-U header to PSA UPF. This NG-RAN reported congestion information is sent to the PSA UPF in a common information element to support congestion information exposure and to support ECN marking for L4S in PSA UPF as described in clause 5.37.3.3. In the case of congestion information exposure, the PSA UPF interprets the RAN reported information as the percentage of congestion level for exposure and exposes the corresponding UL and/or DL congestion information via Nupf_EventExposure service or via SMF/PCF/NEF as described in clause 5.8.2.18. It can be applied to a Non-GBR or GBR QoS Flow. In the case of ECN marking for L4S in PSA UPF, the PSA UPF interprets the RAN reported information as percentage of packets that UPF uses for ECN marking for L4S as described in clause 5.37.3.3.

-	The UL and/or DL Data rate information (see clause 5.45.4).

Based on the PCC rule from PCF, the SMF requests the PSA UPF to measure and report the information. They may be exposed to the AF directly by PSA UPF via Nupf_EventExposure service or via SMF/PCF/NEF as described in clause 5.8.2.18.

-	The round trip delay for two service data flows considering the UL direction of a service data flow and the DL direction of another service data flow in the same PDU Session.

It is determined based on the QoS Monitoring for packet delay of individual QoS Flows as described in clause 5.33.3. The PCF derives the separate QoS monitoring policies for each direction packet delay (see clause 5.33.3) based on AF request and local policy. The PCF provides the two QoS Monitoring policies in the PCC rules for the service data flows. The PSA UPF reports the delay information per QoS Flow to the SMF. The SMF reports to PCF. The PCF derives round trip delay information based on the two direction's packet delay result for the service data flows and exposes the information to the AF directly or via NEF.

-	The round trip delay for one service data flow.

If the service data flow is mapped to two QoS Flows (i.e. the UL traffic and DL traffic of the service data flow are separated into two QoS flows respectively) in the same PDU Session, similarly to the round trip delay for two service data flows over two QoS flows, the PCF triggers QoS Monitoring for each direction packet delay of individual QoS flows respectively and derives round trip delay based on the two direction QoS flows' packet delay monitored result.

NOTE 1:	How PCF calculates the requested round trip delay for multiple QoS Flows from delays of individual QoS Flows is not specified in this specification.

-	The UL and/or DL available bitrate for a GBR QoS Flow (see clause 5.45.x).

For a GBR QoS Flow, based on the PCC rule from PCF, the SMF requests the NG-RAN to report the UL and/or DL available bitrate. The NG-RAN reports the UL and/or DL available bitrate information to the PSA UPF via the GTP-U header of UL packets.

The SMF further requests the PSA UPF to expose the available bitrate to the AF directly or via local NEF, as described in clause 5.8.2.18.

The AF may provide the Alternative QoS parameter set requirements and Averaging Window to the NEF/PCF for the GBR QoS Flow as specified in clause 4.15.6.6 of TS 23.502 [3].

# * * * * Third change * * * *

5.45	QoS Monitoring

5.45.1	General

QoS monitoring comprises of measurements of QoS monitoring parameters and reports of the measurement result for a QoS Flow and can be enabled based on 3rd party application requests and/or operator policies configured in the PCF. Event Reporting from PCF is specified in clause 6.1.3.18 of TS 23.503 [45] and User Plane QoS Flow related QoS monitoring and reporting in clause 5.8.2.18.

The AF may request measurements for one or more of the following QoS monitoring parameters, which may trigger QoS monitoring for service data flow(s):

-	UL packet delay, DL packet delay, round trip packet delay for a service data flow, see clause 5.45.2.

-	Congestion, see clause 5.45.3.

-	Data Rate, see clause 5.45.4.

-	Packet Delay Variation, see clause 5.37.7.

-	Round trip packet delay considering UL on a service data flow and DL of another service data flow, see clause 5.37.4.

-    Available Bitrate, see clause 5.45.x.

The following AF requested QoS requirements may trigger QoS monitoring for service data flow(s):

-	Round-trip latency requirement, see clause 5.37.6.

The PCF may generate the authorized QoS Monitoring policy for a service data flow based on the QoS Monitoring request received from the AF (as described in clause 6.1.3.21 of TS 23.503 [45]) or based on PCF local policy or configuration reasons, such as PCF awareness of dynamic satellite backhaul connection. The PCF includes the authorized QoS Monitoring policy in the PCC rule and provides it to the SMF.

The QoS monitoring parameter(s) that can be measured by means of QoS monitoring are listed below. The QoS monitoring policy in PCC rule (described in clause 6.3.1 of TS 23.503 [45]) may include the following:

-	UL packet delay, DL packet delay, round trip packet delay, see clause 5.45.2.

-	Congestion, see clause 5.45.3.

-	Data Rate, see clause 5.45.4.

-    Available Bitrate, see clause 5.45.x.

The SMF configures the UPF to perform QoS monitoring for the QoS Flow and to report the monitoring results as described in clause 5.8.2.18 with parameters determined by the SMF based on the authorized QoS Monitoring policy received from the PCF or local configuration or both.

The SMF may also configure NG RAN to measure the QoS monitoring parameters by sending QoS monitoring request based on the authorized QoS Monitoring policy received from the PCF and/or local configuration. The QoS monitoring request to the NG RAN for different parameters is as defined in clause 5.45.2, 5.45.3, and 5.45.x.

Based on configuration, the AMF knows that the QoS monitoring feature is supported by all NG RAN nodes in an area. The AMF shall, when configured, inform the SMF about the support of the QoS monitoring feature by sending the parameter NG RAN QoS monitoring capability during PDU Session establishment, PDU Session modification, Service request and UE mobility procedures. The SMF determines whether QoS monitoring is possible or not for the PDU Session based on whether the NG RAN QoS monitoring capability is received (or not) from the AMF and the QoS monitoring capability of the UPF. QoS monitoring is possible if the SMF has received the NG RAN QoS monitoring capability in the last message received from AMF (i.e. Nsmf_PDUSession_CreateSMContext or Nsmf_PDUSession_UpdateSMContext service operation) and the UPF also supports the QoS monitoring capability. Otherwise, QoS monitoring is not possible for the PDU Session. The SMF may need to notify the PCF when the support for QoS monitoring has changed as described in clause 6.1.3.5 of TS 23.503 [45].

The following clauses describe the QoS monitoring parameters which can be measured and any specific actions or constraints for their measurement.

NOTE:	The QoS monitoring parameter which can be measured are parameters which describe the QoS experienced in the 5GS by the application, i.e. they are not restricted to the 5G QoS Parameters defined in clause 5.7.2.

# * * * * Fourth change * * * *

5.45.x	Available bitrate monitoring

The QoS Monitoring for available bitrate allows the measurement and report of the UL and/or DL available bitrate for a GBR QoS Flow without Alternative QoS Profiles at the NG-RAN. The available bitrate refers to the bitrate that NG-RAN is able to provide for a QoS Flow within a duration. The duration is based on RAN implementation.

NOTE 1:	How the NG-RAN measures the available bitrate is NG-RAN implementation specific.

NOTE 2:	The available bitrate can be between the GFBR and the MFBR. The available bitrate can also be below the GFBR for a QoS Flow with QoS Notification Control enabled without Alternative QoS Profiles.

For the reporting of the available bitrate, periodic reporting is not applied and only threshold-based reporting applies. Based on the PCC rule containing available bitrate as QoS monitoring parameter and Reporting threshold(s) per direction in the QoS Monitoring policy (as described in clause 6.1.3.27.1 of TS 23.503 [45]), the SMF configures the NG-RAN to measure and report the available bitrate by sending QoS monitoring request including the threshold(s). Based on the QoS monitoring request, if the bitrate that NG-RAN is able to provide matches or exceeds indicated Reporting threshold(s), the NG-RAN reports the UL and/or DL available bitrate information of the highest fulfilled threshold to the PSA UPF via the GTP-U header of UL packets.

NOTE 3:	The NG-RAN is expected to immediately report the UL and/or DL available bitrate when it matches or exceeds a lower threshold value and report the UL and/or DL available bitrate when it matches or exceeds a higher threshold value with some hysteresis.

The SMF may configure the PSA UPF to perform QoS monitoring for available bitrate as described in clause 5.8.2.18. For the reporting of the available bitrate via the PSA UPF, only event-based reporting applies in the PSA UPF. The Reporting threshold(s) in the PCC rule are not sent to UPF. In UPF, the event for the reporting is the reception of available bitrate information from NG-RAN. According to the QoS Monitoring request for the available bitrate from the SMF, the PSA UPF monitors and exposes the available bitrate information reported from NG-RAN to the target NF as instructed.

During handover, the QoS Monitoring configuration is provided to the Target NG-RAN per QoS Flow in the QoS Monitoring configuration.

# * * * * End of changes * * * *