3GPP TSG-SA3 Meeting #120	S3-250574

Athens, Greece, 17 - 21 February 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 2103 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | WWC related alignment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As a part of WWC WID, section 7.2.1.2 was added in the last meeting. Few editorial clean up is required |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editorial correction |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7A.2.1.2, A22 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************* Start of Change *************

#### 7A.2.1.2	Re-authentication for UE moving from one TNAP to another TNAP connecting to the same TNGF

When a UE associates with a TNAP in a TNAN connected to a TNGF, if the TNAP and TNGF in TNAN support FT, the TNAP in the TNAN advertises the FT capability as in IEEE 802.11 [80]. During the IKE process, the UE also indicates MOBIKE indication if it supports MOBIKE and TNGF acknowledges it if supported.

NOTE 1: It is assumed that TNGF is aware of the FT capabilities of the TNAP in the TNAN. Whether this is done by configuration or signalling internal to the TNAN is left to operator implementations.

The UE and TNGF perform the following in addition to the procedures specified in clause 7A.2.1.1 of the present document. If the TNAP supports FT, the TNGF shall derive a fresh key to enable re-authentication based on FT procedure, i.e. KFT, from KTNGF as specified in A.22 of the present document and either keep KFT if the TNGF acts as the R0KH or send the KFT to the R0KH otherwise. If the UE supports FT and connects to the TNAN supporting the FT, the UE shall determine to enable re-authentication based on FT procedure and derive an FT key, KFT, from KTNGF as specified in A.22 of the present document.

NOTE 2: Where the entity R0 key holder (R0KH) is located in the TNAN is outside the scope of 3GPP.

The UE and R0KH use KFT as Master PMK (MPMK) to construct the FT key hierarchy as specified in IEEE 802.11 [80] to perform re-authentication and security (re-)establishment while the UE switches between different TNAPs within the same mobility domain (i.e., identified by the MDID specified by IEEE 802.11 [80]) connected to the same TNGF. To perform such a switch, the UE performs the FT procedure as specified in IEEE 802.11 [80]. The R0KH derives a key PMK-R1 from PMK-R0 and provides the key PMK-R1 to the new TNAP in TNAN during the FT procedure.

After the UE switch between TNAPs is complete, if MOBIKE is supported, the UE may send the request to restore the IPSec connectivity, and all IPSec SAs as defined in RFC 4555 [111].

NOTE 3: How to configure the MDID used for FT within the TNGF domain is left to Operator’s implementation and deployment. How the 3GPP-defined TNGF domain and IEEE-defined MDID co-exist/work is outside the scope of the present document.

************* Next  Change *************

# A.22	KTIPSec, KTNAP and KFT derivation function

When deriving a KTIPSec from KTNGF and when deriving a KTNAP or KFT from KTWIF or KTNGF the following parameters shall be used to form the input S to the KDF.

-	FC = 0x84

-	P0 = Usage type distinguisher

-	L0 = length of Usage type distinguisher (i.e. 0x00 0x01)

The values for the Usage type distinguisher are defined in table A.22-1. The values 0x00 and 0x03 to 0xf0 are reserved for future use, and the values 0xf1 to 0xff are reserved for private use.

The Usage type distinguisher shall be set to the value for IPSec (0x01) when deriving KTIPSec. The Usage type distinguisher shall be set to the value for TNAP (0x02) when deriving KTNAP. The Usage type distinguisher shall be set to the value for TNAP Key-refresh using FT (0x03) when deriving KFT.

The input key KEY shall be the 256-bit KTNGF or KTWIF.

Table A.22-1: Usage type distinguishers

| Usage type distinguisher | Value |
| --- | --- |
| IPSec | 0x01 |
| TNAP | 0x02 |
| TNAP Key-refresh based on FT | 0x03 |

************* End of  Change *************