3GPP TSG-SA5 Meeting #159	S5-250156

# Sophia-Antipolis, France, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.550 | CR | 0090 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel19 CR TS28.550 OpenAPI correction related to format of URI query parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When used in query parameters of a URI, Arrays of simple types: they shall be formatted using the OpenAPI "style" keyword set to "form" and the "explode" keyword set to "false". This is to be aligned with TS29.501 clause 5.3.13, as well as recommended by OpenAPI specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the OpenAPI "style" keyword set to "form" and the "explode" keyword set to "false". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It may lead to interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (Normative stage 3 is in Forge) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1592 Commit: 146defbad6f36e8da1bd0835a264d61b7d60b88e |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGE 1 ***

openapi: 3.0.1

info:

title: TS 28.550 Performance Measurement Job Control Service

version: 18.1.0

description: >-

OAS 3.0.1 specification of the Performance Measurement Job Control Service

@ 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.550 Performance assurance

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.550/

servers:

- url: '{MnSRoot}/PerfMeasJobCtrlMnS/{MnSVersion}'

variables:

MnSRoot:

description: See clause 4.4.2 of TS 32.158

default: http://example.com/3GPPManagement

MnSVersion:

description: Version number of the OpenAPI definition

default: XXX

paths:

/measJobs:

post:

summary: Create a measurement job

description: To create a measurement job the representation of the measurement job is POSTed on the /measJobs collection resource.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/measJobCreation-RequestType'

responses:

'201':

description: Success case ("201 Created"). The representation of the newly created measurement job resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/measJobCreation-ResponseType'

'202':

description: Partial success case ("202 Partically created"). The representation of the newly created measurement job resource with unsupported list shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/measJobCreation-ResponseType'

default:

description: Error case.

content:

application/json:

schema:

$ref: '#/components/schemas/error-ResponseType'

get:

summary: Read resources of measurement jobs

description: 'With HTTP GET, resources of measurement jobs are read. The resources to be read are identified with the path component (base resource) and the query component (jobIdList) of the URI. The fields query component allows to select the resource properties to be returned.'

parameters:

- name: jobIdList

in: query

description: This parameter identifies the list of jobId to select the resources from the collection resources identified with the path component of the URI.

required: true

style: form

explode: false

schema:

type: array

items:

type: string

responses:

'200':

description: 'Success case ("200 OK"). The resources identified in the request for retrieval are returned in the response message body. In case the fields query parameter is used, the selected resources are returned.'

content:

application/json:

schema:

$ref: '#/components/schemas/measJobsRetrieval-ResponseType'

default:

description: Error case.

content:

application/json:

schema:

$ref: '#/components/schemas/error-ResponseType'

'/measJobs/{jobId}':

get:

summary: Read resource of a single measurement job

description: 'With HTTP GET, resource of a measurement job is read. The resource to be read is identified with the path component of the URI.'

parameters:

- name: jobId

in: path

description: Identifies the measurement job to be read.

required: true

schema:

$ref: '#/components/schemas/uri-Type'

responses:

'200':

description: 'Success case ("200 OK"). The resource identified in the path for retrieval is returned in the response message body. '

content:

application/json:

schema:

$ref: '#/components/schemas/measJobsRetrieval-ResponseType'

default:

description: Error case.

content:

application/json:

schema:

$ref: '#/components/schemas/error-ResponseType'

delete:

summary: Delete a single measurement job

description: The measurement job is deleted by deleting the corresponding measurement job resource. The resource to be deleted is identified with the path component of the URI.

parameters:

- name: jobId

in: path

description: Identifies the measurement job to be deleted.

required: true

schema:

$ref: '#/components/schemas/uri-Type'

responses:

'204':

description: Success case ("204 No Content"). The measurement job resource has been deleted. The response message body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: '#/components/schemas/error-ResponseType'

components:

schemas:

dateTime-Type:

type: string

format: date-Time

uri-Type:

type: string

measJobCreation-RequestType:

type: object

properties:

iOCName:

type: string

iOCInstanceList:

type: array

items:

$ref: '#/components/schemas/uri-Type'

measurementCategoryList:

type: array

items:

type: string

reportingMethod:

$ref: '#/components/schemas/reportingMethod-Type'

granularityPeriod:

type: integer

reportingPeriod:

type: integer

startTime:

$ref: '#/components/schemas/dateTime-Type'

stopTime:

$ref: '#/components/schemas/dateTime-Type'

schedule:

$ref: '#/components/schemas/schedule-Type'

streamTarget:

type: string

priority:

$ref: '#/components/schemas/priority-Type'

reliability:

type: string

measJobCreation-ResponseType:

type: object

properties:

unsupportedList:

type: array

items:

$ref: '#/components/schemas/unsupportedMeas-Type'

measJobsRetrieval-ResponseType:

type: object

properties:

jobInfoList:

type: array

items:

$ref: '#/components/schemas/measJobInfo-ResourceType'

error-ResponseType:

type: object

properties:

error:

type: object

properties:

errorInfo:

type: string

measJobInfo-ResourceType:

type: object

properties:

href:

$ref: '#/components/schemas/uri-Type'

iOCName:

type: string

iOCInstanceList:

type: array

items:

$ref: '#/components/schemas/uri-Type'

measurementCategoryList:

type: array

items:

type: string

reportingMethod:

$ref: '#/components/schemas/reportingMethod-Type'

granularityPeriod:

type: integer

reportingPeriod:

type: integer

startTime:

$ref: '#/components/schemas/dateTime-Type'

stopTime:

$ref: '#/components/schemas/dateTime-Type'

schedule:

$ref: '#/components/schemas/schedule-Type'

streamTarget:

type: string

priority:

$ref: '#/components/schemas/priority-Type'

reliability:

type: string

schedule-Type:

type: object

properties:

scheduleOption:

$ref: '#/components/schemas/scheduleOption-Type'

dailySchedule:

type: array

items:

$ref: '#/components/schemas/timeInterval-Type'

weeklySchedule:

type: array

items:

$ref: '#/components/schemas/scheduleOfDay-Type'

timeInterval-Type:

type: object

properties:

intervalStart:

type: string

format: Time

intervalEnd:

type: string

format: Time

scheduleOfDay-Type:

type: object

properties:

dayOfWeek:

$ref: '#/components/schemas/dayOfWeek-Type'

intervalsOfDay:

type: array

items:

$ref: '#/components/schemas/timeInterval-Type'

unsupportedMeas-Type:

type: object

properties:

iOCInstance:

$ref: '#/components/schemas/uri-Type'

measurementTypeName:

type: string

reason:

type: string

reportingMethod-Type:

type: string

enum:

- file

- streaming

priority-Type:

type: string

enum:

- low

- medium

- high

scheduleOption-Type:

type: string

enum:

- daily

- weekly

dayOfWeek-Type:

type: string

enum:

- Monday

- Tuesday

- Wednesday

- Thursday

- Friday

- Saturday

- Sunday

*** END OF CHANGE 1 ***