3GPP TSG-SA5 Meeting #159	S5-250884

# Sophia-Antipolis, France, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | 0495 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.623 decouples the ProvMnS schema with supported feature schemas |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei,Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SBMA_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The solution for schema reference enhancement to support flexible extension of the required schema files is documented in TR 28.871 and recommended for normative work. So, it proposes to decouples the ProvMnS schema with supported feature schemas in TS 28.532. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the ProvMnS schema in TS 28.532 to decouple with corresponding features schemas. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3, Following YAML files normatively defined in the forge are updated: - OpenAPI/TS28623_FeatureNrm.yaml |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1552 at commit cd4fe74aad315530905a350b0004aac043f0894d |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

4.3	OpenAPI Definitions

The present clause contains the OpenAPI definitions of the Generic NRM in YAML format.

The Information Service (IS) of the Generic NRM is defined in 3GPP TS 28.622 [4].

Mapping rules to produce the OpenAPI definition based on the IS are defined in 3GPP TS 32.160 [14].

OpenAPI/YAML definitions are specified in 3GPP Forge [17].

Directory: OpenAPI

Files:

TS28623_ComDefs.yaml

TS28623_GenericNrm.yaml

TS28623_PmControlNrm.yaml

TS28623_ThresholdMonitorNrm.yaml

TS28623_SubscriptionControlNrm.yaml

TS28623_MnSRegistryNrm.yaml

TS28623_FileManagementNrm.yaml

TS28623_ManagementDataCollectionNrm.yaml

TS28623_QoEMeasurementCollectionNrm.yaml

TS28623_TraceControlNrm.yaml

TS28623_FeatureNrm.yaml

2nd Change

*** START OF CHANGE 1 ***

*** OpenAPI/TS28623_FeatureNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: NRM Feautres

version: 19.0.0

description: >-

OAS 3.0.1 definition of the Features of NRM

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.623; Generic NRM, NRM Features

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.623/

paths: {}

components:

schemas:

#----- Definitions in TS 28.623 for TS 28.532 --------------------------#

resources-feature:

anyOf:

- $ref: 'TS28104_MdaNrm.yaml#/components/schemas/resources-mdaNrm'

- $ref: 'TS28105_AiMlNrm.yaml#/components/schemas/resources-AiMlNrm'

- $ref: 'TS28111_FaultNrm.yaml#/components/schemas/resources-faultNrm'

- $ref: 'TS28312_IntentNrm.yaml#/components/schemas/resources-intentNrm'

- $ref: 'TS28317_RanScNrm.yaml#/components/schemas/resources-RanScNrm'

- $ref: 'TS28318_DsoNrm.yaml#/components/schemas/resources-DSORecovery'

- $ref: 'TS28319_MsacNrm.yaml#/components/schemas/resources-msacNrm'

- $ref: 'TS28536_CoslaNrm.yaml#/components/schemas/resources-coslaNrm'

- $ref: 'TS28538_EdgeNrm.yaml#/components/schemas/resources-edgeNrm'

- $ref: 'TS28541_NrNrm.yaml#/components/schemas/resources-nrNrm'

- $ref: 'TS28541_5GcNrm.yaml#/components/schemas/resources-5gcNrm'

- $ref: 'TS28541_SliceNrm.yaml#/components/schemas/resources-sliceNrm'

- $ref: 'TS28623_FileManagementNrm.yaml#/components/schemas/resources-fileMgmtNrm'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/resources-genericNrm'

- $ref: 'TS28623_ManagementDataCollectionNrm.yaml#/components/schemas/resources-mgmtDataCollectionNrm'

- $ref: 'TS28623_MnSRegistryNrm.yaml#/components/schemas/resources-mnSRegistryNrm'

- $ref: 'TS28623_PmControlNrm.yaml#/components/schemas/resources-pmControlNrm'

- $ref: 'TS28623_QoEMeasurementCollectionNrm.yaml#/components/schemas/resources-qoEMeasuremetCollectionNrm'

- $ref: 'TS28623_SubscriptionControlNrm.yaml#/components/schemas/resources-subscriptionControlNrm'

- $ref: 'TS28623_ThresholdMonitorNrm.yaml#/components/schemas/resources-thresholdMonitorNrm'

- $ref: 'TS28623_TraceControlNrm.yaml#/components/schemas/resources-traceControlNrm'

#----- Definitions in TS 28.623 for TS 28.532 --------------------------#

<CODE ENDS>

*** END OF CHANGE 2 ***

End of Changes